3GPP TSG RAN WG1 Meeting #104-e		 				R1-210xxxx

E-meeting, January 25th – February 5th, 2021

Agenda item:		7.1

Source:		Moderator (Ericsson)

Title:	 	Moderator summary of Email discussion [104-e-NR-7.1CRs-14]

Document for:	        Discussion and Decision

# Introduction

This document is intended to facilitate exchange of views and discussions for the following assigned email discussion by Mr. Chairman based on the draft CRs [1] and [2] for Rel-15 and Rel-16, respectively.

[104-e-NR-7.1CRs-14] Draft CR on Type-1 HARQ -ACK for PDSCH repetition with different SCSs in DL and UL – Sorour (Ericsson) by Jan 29

Summary of Draft CRs [1], [2]:

Issue: In Clause 9.1.2, TS 38.213, when reporting of Type-1 HARQ-ACK information for a PDSCH reception in case of repetition is described, the slots with PDSCH reception corresponding to a HARQ-ACK are determined from slot  to slot n where, slot n corresponds to the UL slot that is mapped to the last PDSCH reception (with or without repetition) and the time interval on UL for PDSCH reception is stated to be from slot  to slot n. Since the quantity for PDSCH aggregation factor (i.e. NPDSCHrepeat) is measured in DL slot size, in case the SCS of carriers with corresponding PDSCH and PUCCH transmissions is different, it is not correct to state that the first PDSCH transmission is mapped to slot  slot .

Proposed change: The PDSCH aggregation factor is scaled properly with respect to numerologies of PDSCH and PUCCH carriers to determine properly the UL slot that would include the first PDSCH reception.

# Discussions

In order to conclude this discussion, companies are encouraged to share their views on the questions below.

| Question1: Do you agree with the issue and corresponding change discussed in the draft CRs technically? If No, why? Question 2: Can you consider implementing the CRs for Rel-15 and/or Rel-16? Question 3 (in case No to Question 2): Please share your preference on how to proceed, including potential updates of the draft CRs. |  |
| --- | --- |
| Company | Comment |
| Apple | Q1: we agree with the issue. For the draft CRs, we think the floor operation should be ceiling operation instead. Q2: we think this should be implemented in both Rel-15 and Rel-16. Even though the specification is not correct, the intention should be clear for UE implementation. |

# Conclusion

To be updated with the outcome of the email discussion.

# References

| [1] | R1-2101722 | Draft CR on Type-1 HARQ-ACK for PDSCH repetition with different SCSs in DL and UL | Ericsson |
| --- | --- | --- | --- |
| [2] | R1-2101723 | Draft CR on Type-1 HARQ-ACK for PDSCH repetition with different SCSs in DL and UL | Ericsson |