3GPP TSG RAN WG1 #104-e		                                   					       												                 R1-20xxxxx

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	7.2.11

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [104-e-NR-UEFeatures-eMIMO-03]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [104-e-NR-UEFeatures-eMIMO-03] during RAN1 #104-e. According to the Chairman’s Notes:

[104-e-NR-UEFeatures-eMIMO-03] Email discussion/approval of how to interpret FG 16-2c “Simultaneous reception with different Type-D” and whether changes are needed, till 1/29 (Ralf, AT&T)

The following was discussed and agreed during RAN1 #104-e within the scope of [104-e-NR-UEFeatures-eMIMO-03]. All proposals are based on the latest RAN1 UE features list for Rel-16 NR in Error: Reference source not found.

# Summary of email discussion/approval [104-e-NR-UEFeatures-eMIMO-03]

In the latest RAN1 UE features list for Rel. 16 NR, FG 16-2c is defined as follows [1].

| 16-2c | Simultaneous reception with different Type-D | Supports simultaneous reception with different QCL Type-D RSs. |  | Yes | N/A |  | Per band | N/A | FR2 only |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In [2], Samsung observes the following:

For Rel-16 M-TRP features, FG16-2c describes if a UE supports simultaneous reception with different QCL-D. One thing to think about is that support of this FG may be interpreted as supporting any combination of QCL-D with simultaneous reception when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH due to lack of further condition mentioned. However, such interpretation is unrealistic since a UE would unlikely be able to receive different QCL-D unconditionally. Without such consideration, most UE’s would need to decline support of FG16-2c, and this can be a considerable under reporting. Ideally, Rel-16 signaling could have been designed to explicitly acknowledge this aspect. However, given the timing, we would like to propose exploiting the existing Rel-15 FG2-29a described below. 2-29a; Group based beam reporting; Support of beam group RSRP reporting for group of 2 beams; groupBeamReporting; MIMO-ParametersPerBand; n/a; n/a; Optional with capability signalling. In our understanding, one of objectives of group based reporting is to enable information exchange from a UE to gNB regarding which QCL-D can be simultaneously received. Hence, this can also be utilized for support of receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH in principle. Along this line, we provide two alternatives. Alt1: Support of FG16-2c is interpreted as supporting simultaneous reception with different QCL-D without any further condition when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH. Support of FG2-29a and group based beam reporting (if configured) can also be utilized for receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH when a UE does not support FG16-2c. In our view, Alt1 may have a minimal impact in maintaining independency between FG16-2c and FG2-29a. When a UE supports FG16-2c, it supports simultaneous reception with different QCL-D without any further condition when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH irrespective of FG2-29a. When a UE does not support FG16-2c, but supports FG2-29a, receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH with simultaneous different QCL-D can still happen based on group based reporting. Also, this is natural interpretation given lack of clear limitation for applicability of FG2-29a. However, we believe confirmation is beneficial especially given the existence of Rel-16 signaling with potentially similar functionality. Alt2: FG2-29a; FG16-2c; Interpretation for Rel-16 M-TRP operation. Support; Support; A UE supports simultaneous reception with different QCL-D which are in the same group in group based reporting when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH.. Support/not support; Not support; A UE does not support simultaneous reception with different QCL-D.. Not support; Support; A UE supports simultaneous reception with different QCL-D without further condition when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH.. Alt2 introduces more dependency between FG2-29a and FG16-2c while limiting usability of FG16-2c. Hence, we provide a proposal below based on Alt1.

Based on the above, the following is proposed in [2].

Proposal: Support of FG16-2c is interpreted as supporting simultaneous reception with different QCL-D without any further condition when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH. Support of FG2-29a and group based beam reporting (if configured) can also be utilized for receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH when a UE does not support FG16-2c.

Companies are invited to express their views in the table below. Companies supporting the proposal should also provide a detailed text proposal, if necessary, of how the proposal should be implemented in their view.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We first need to clarify for TCI codepoint in DCI, which two TCI states can be configured/indicated. Even if we indicate we support FG16-2c, it does not necessarily mean gNB can indicate any two TCI states in a TCI codepoint since they might come from the same panel This is the same design principle in Rel-15 in which UE needs to pair the group of beam that UE can receive simultaneously which is enabled by FG2-29a So, we need to clarify that gNB can only configure UE to receive PDSCH simultaneously with two TCI that UE reported as group based beam reporting in the past. After this clarification, the rest will be cleaner since for us, these two FGs are the same. |

# Conclusion

…

# References

- R1-2009585, Updated RAN1 UE features list for Rel-16 NR, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2101184, On NR Rel.16 UE features, Samsung