3GPP TSG RAN WG1 #104-e	R1-21xxxxx

e-Meeting, January 25th – February 5th, 2021

Agenda item:		7.2.12

Source:	Moderator (China Telecom)

Title:	Summary of Rel-16 uplink Tx switching

Document for:		Discussion and Decision

# Introduction

In RAN #85 meeting, the following objective was added in the revised WID of “RF requirements for NR frequency range 1” [1].

- Specify UE requirements to allow switching between case 1 and case 2 as below for two uplink carriers case inter-band EN-DC without SUL, inter-band UL CA and standalone SUL for UE supporting maximum two concurrent transmission

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

    - UE RF requirements, e.g., time mask RF requirements and other necessary RF requirements if any

    - The options agreed at RAN4 #92 in R4-1910531 can be considered as starting point

    - Study if there are any impact to interruption and delay requirements, and specify the RRM requirements if needed

    - RAN1 will further study by Dec 2019 if there are any RAN1 potential impacts based on RAN4 LS if any

    - No new TDM pattern will be defined, i.e. scheduling-based switching is assumed.

    - Finalization of RAN4 requirements and approval of RAN4 CRs shall be based on RAN1 LS

    - Strive to minimize RAN1 impact.

    - Strive to achieve no impact to RAN1 E-UTRAN spec

    - Strive to avoid defining location of switching period impacting RAN1 spec

    - Define per band per band combination or per band combination UE capability signaling if needed

Note 1: Only addressing the case of co-located and synchronized network deployment for the two UL carriers

Note 2: Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA

Note 3: The above objectives will not relax the existing requirements specified in Rel-15 38.101-3 for band combinations allowing single uplink transmission

Note 4: The UE is configured with two different uplink carrier frequencies.

This contribution summarizes the maintenance issues for Rel-16 uplink Tx switching.

# Summary of maintenance issues

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: Clarification on the ambiguity issue on SCS in TS 38.214 | R1-2100087 | Suggest to be discussed in RAN1#104-e |
| Issue#2: Align the RRC parameters in TS 38.214 with TS 38.331 | R1-2100117 | Suggest to be discussed in RAN1#104-e |
| Issue#3: Clarification on the Maximum data rate in TS 38.306 | R1-2101445, R1-2101554 | Suggest to be discussed in RAN1#104-e |
| Issue#4: Clarification of Tswitch in TS 38.213 | R1-2101738 | Suggest to be discussed in RAN1#104-e |
| Issue#5: Clarification on UE capability for SRS antenna switching | R1-2101445 | Suggest to be discussed in RAN1#104-e |
| Issue#6: Clarification on the state of Tx chains for SRS antenna switching | R1-2101445 | Suggest to be discussed in RAN1#104-e |
| Issue#7: CA based SRS switching | R1-2101445 | According to the note “Note 4: The UE is configured with two different uplink carrier frequencies.” in the WID, it seems that the scenario of UE configured with 3 UL carrier is out of scope. Suggest not to be discussed in RAN1#104-e. |

| Company | Views |
| --- | --- |

# References

- RP-192282, WID revision: RF requirements for NR frequency range 1 (FR1), Huawei, HiSilicon, China Telecom, RAN #85, Sep. 2019.

- R1-2100087, Remaining issues of Rel-16 UL Tx Switching, ZTE, RAN1#104-e, January 25th – February 5th, 2021.

- R1-2100117, Text Proposals for Tx Switching between Two Uplink Carriers, OPPO, RAN1#104-e, January 25th – February 5th, 2021.

- R1-2101445, Remaining issues for 1Tx-2Tx switching, Qualcomm Incorporated, RAN1#104-e, January 25th – February 5th, 2021.

- R1-2101554, Maintenance for Rel-16 UL Tx Switching, Ericsson, RAN1#104-e, January 25th – February 5th, 2021.

- R1-2101738, Discussion on the remaining problems of supporting Tx switching between two uplink carriers, Huawei, HiSilicon, RAN1#104-e, January 25th – February 5th, 2021.