3GPP TSG RAN WG1 #104-e	R1-21xxxxx

e-Meeting, January 25th – February 5th, 2021

Agenda item:		7.2.12

Source:	Moderator (China Telecom)

Title:	Summary of Rel-16 uplink Tx switching

Document for:		Discussion and Decision

# Introduction

In RAN #85 meeting, the following objective was added in the revised WID of “RF requirements for NR frequency range 1” [1].

- Specify UE requirements to allow switching between case 1 and case 2 as below for two uplink carriers case inter-band EN-DC without SUL, inter-band UL CA and standalone SUL for UE supporting maximum two concurrent transmission

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

    - UE RF requirements, e.g., time mask RF requirements and other necessary RF requirements if any

    - The options agreed at RAN4 #92 in R4-1910531 can be considered as starting point

    - Study if there are any impact to interruption and delay requirements, and specify the RRM requirements if needed

    - RAN1 will further study by Dec 2019 if there are any RAN1 potential impacts based on RAN4 LS if any

    - No new TDM pattern will be defined, i.e. scheduling-based switching is assumed.

    - Finalization of RAN4 requirements and approval of RAN4 CRs shall be based on RAN1 LS

    - Strive to minimize RAN1 impact.

    - Strive to achieve no impact to RAN1 E-UTRAN spec

    - Strive to avoid defining location of switching period impacting RAN1 spec

    - Define per band per band combination or per band combination UE capability signaling if needed

Note 1: Only addressing the case of co-located and synchronized network deployment for the two UL carriers

Note 2: Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA

Note 3: The above objectives will not relax the existing requirements specified in Rel-15 38.101-3 for band combinations allowing single uplink transmission

Note 4: The UE is configured with two different uplink carrier frequencies.

This contribution summarizes the maintenance issues for Rel-16 uplink Tx switching.

# Summary of maintenance issues

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: Clarification on the ambiguity issue on SCS in TS 38.214 | R1-2100087 | Suggest to be discussed in RAN1#104-e |
| Issue#2: Align the RRC parameters in TS 38.214 with TS 38.331 | R1-2100117 | Suggest to be discussed in RAN1#104-e |
| Issue#3: Clarification on the Maximum data rate in TS 38.306 | R1-2101445, R1-2101554 | Suggest to be discussed in RAN1#104-e |
| Issue#4: Clarification of Tswitch in TS 38.213 | R1-2101738 | Suggest to be discussed in RAN1#104-e |
| Issue#5: Clarification on UE capability for SRS antenna switching | R1-2101445 | Suggest to be discussed in RAN1#104-e |
| Issue#6: Clarification on the state of Tx chains for SRS antenna switching | R1-2101445 | Suggest to be discussed in RAN1#104-e |
| Issue#7: CA based SRS switching | R1-2101445 | According to the note “Note 4: The UE is configured with two different uplink carrier frequencies.” in the WID, it seems that the scenario of UE configured with 3 UL carrier is out of scope. Suggest not to be discussed in RAN1#104-e. |

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Regarding Issue#3, it looks unnecessary to change the current general description “cells” into “two cells” as proposed in R1-2101554 because it has been agreed in Rel-17 WI to introduce the third cell, an intra-band cell. It also looks unnecessary to have the change proposed in R1-2101445 because the current text “cell(s)” with plural form have clearly incorporated CA case as the proposal required, i.e. the motivation has been fulfilled already. Regarding issue#5, the current topic is much more general than what is proposed in R1-2101445 which is only specific to 1TyR case. More importantly, there is no explanation in the tdoc why this case should be specially treated. Please proponent provides further clarification, or it should be postponed to next meeting. Considering a limited email thread for this, we feel that 4 issues are already too many for this meeting, and suggest that issue#3, 5 and 7 will not be discussed this meeting, |

# References

- RP-192282, WID revision: RF requirements for NR frequency range 1 (FR1), Huawei, HiSilicon, China Telecom, RAN #85, Sep. 2019.

- R1-2100087, Remaining issues of Rel-16 UL Tx Switching, ZTE, RAN1#104-e, January 25th – February 5th, 2021.

- R1-2100117, Text Proposals for Tx Switching between Two Uplink Carriers, OPPO, RAN1#104-e, January 25th – February 5th, 2021.

- R1-2101445, Remaining issues for 1Tx-2Tx switching, Qualcomm Incorporated, RAN1#104-e, January 25th – February 5th, 2021.

- R1-2101554, Maintenance for Rel-16 UL Tx Switching, Ericsson, RAN1#104-e, January 25th – February 5th, 2021.

- R1-2101738, Discussion on the remaining problems of supporting Tx switching between two uplink carriers, Huawei, HiSilicon, RAN1#104-e, January 25th – February 5th, 2021.