3GPP TSG-RAN WG1 Meeting #104-e	R1-21xxxxx

e-Meeting, January 26th – February 5th, 2021

Title:	[Draft] LS on SL HARQ-ACK reporting to the gNB

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	[RAN WG1], Ericsson

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Ricardo Blasco

E-mail Address:	<ricardo.blasco@ericsson.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

Issue 1

In RAN1#98bis, the following was agreed:

Agreements: For reporting SL HARQ-ACK to the gNB: - For dynamic grant and configured grant type-2 in SL, the Rel-15 procedure and signalling for DL HARQ-ACK are reused for the purpose of selecting PUCCH offset/resource and format in UL. - The configuration for SL is separate from Uu link for a UE - FFS how to indicatae timing of transmission in PUCCH, including whether physical or logical slots are used - For configured grant type-1 in SL, RRC is used to configure PUCCH offset/resource and format in UL (if supported)

Reusing the Rel-15 procedure and signalling used for DL HARQ-ACK reporting includes using the higher-layer parameter sl-N1PUCCH-AN-r16 for determining the PUCCH resource used for SL HARQ-ACK reporting for SL CG type 2 (but only for PSCCH/PSSCH transmissions without a corresponding PDCCH).

However, in RAN1’s understanding the parameter is only configurable for SL CG type 1:

SL-ConfiguredGrantConfig field descriptions

sl-N1PUCCH-AN This field indicates the HARQ resource for PUCCH for sidelink configured grant type 1. The actual PUCCH-Resource is configured in sl-PUCCH-Config and referred to by its ID.

Issue 2

During RAN1#104-e, the following agreement has been made:

Agreements: The parameter pdsch-HARQ-ACK-Codebook is always used for reporting SL HARQ-ACK information.

2. Actions:

To RAN WG2:

- One Issue 1:

  - RAN1 kindly requests RAN2 to clarify whether the parameter is configurable for SL CG type 2 in the current specification.

  - If the parameter is not configurable for SL CG type 2 in the current specification, RAN1 kindly asks RAN2 to introduce a solution that is compatible with the RAN1 agreements, if possible.

  - If introducing such a solution is not possible, RAN1 kindly requests to be informed about this by RAN2.

- One Issue 2:

  - RAN1 kindly requests RAN2 to capture the agreed behavior in their specification.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #104-bis-e	12 Apr – 20 Apr 2021				e-Meeting

TSG RAN WG1 Meeting #105-e	19 May – 27 May 2021				e-Meeting