3GPP TSG RAN WG1 #104-e		R1-2101921

e-Meeting, January 25th – February 5th, 2021

Title:	[Draft] LS on the resource reservation period

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	LG Electronics [RAN WG1]

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Hanbyul Seo

E-mail Address:	<hanbyul.seo@lge.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 discussed how to interpret the resource reservation period field in a SCI format 1-A and reached the following agreements:

RAN1 agrees to adopt Option 1’with the condition that RAN2 confirms RAN2 can do the necessary spec update; If RAN2 informs RAN2 cannot do the necessary spec update, Option 2-3 will be implemented. RAN1 will prepare CRs for these two options and send an LS to RAN2. Proposal: - For UE behaviour for the case when a UE transmits a SCI format 1-A in slot n in a resource pool and “Resource reservation period” in the SCI format 1-A indicates P, down-select one between Option 1’ and Option 2-3: - Option 1: The resource reservation period P’ is calculated following 8.1.7 of 38.214, and P’ is counted in (i.e. the number of slots in the resource pool between slot n and slot n+P’ is always the same as P’ (including slot n+P’ itself)). - Option 1’: Option 1 + updated equation in 8.1.7 (N is the number of slots belonging to the resource pool) - Option 2: The resource reservation period P’ is calculated following 8.1.7 of 38.214 and P’ is counted in (i.e. the number of slots in the resource pool between slot n and slot n+P’ can be less than P’). - Option 2-3: Add “If slot is not in the resource pool, the next slot in the resource pool should be used instead.”

RAN1 notes that the set of slots in the above agreements are defined in 38.214 and copied below:

- denotes the set of slots which can belong to a sidelink resource pool and is defined in Clause 8.

- denotes the set of slots which belongs to the sidelink resource pool and is defined in Clause 8.

2. Actions:

To RAN WG2: RAN1 respectfully asks RAN2 to inform RAN1 ideally during RAN1#104-e whether RAN2 can do the RAN2 specification update necessary to implement Option 1’ above.

3. Dates of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #104-bis-e	12 April – 20 April 2021	eMeeting

TSG RAN WG1 Meeting #105 -e	19 May – 27 May 2021 	eMeeting