| 3GPP TR 36.763 V0.0.3 (2021-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Non-Terrestrial Networks overview and scenarios	10

4.1	Non-Terrestrial Networks overview	10

4.2	Non-Terrestrial Networks reference scenarios	10

5	Radio Layer 1 issues and related solutions	10

5.1	IoT NTN Reference Parameters	10

5.2	Link Budget Analysis	10

5.3	Timing Relationships	10

5.4	UL Synchronization	10

6	User Plane Aspects	10

6.1	Random Access Procedure	10

6.2	MAC Timers	10

6.3	HARQ	11

7	Control Plane Aspects	11

7.1	System Information Enhancements	11

7.2	Idle Mobility Enhancements	11

7.3	Radio Link Failure Enhancements (NB-IoT Only)	11

7.4	Connected Mobility Enhancements (eMTC only)	11

7.5	Tracking Area Update Enhancements	11

8	Recommendations on the way forward	11

8.1	Recommendations from RAN1	11

8.2	Recommendations from RAN2	11

Annex A:	Satellite ephemeris	12

Annex B:	KPI and evaluation assumptions	12

Annex C (Informative): Change history	13

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

TBA

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)"

[3]	3GPP TS 38.821 v16.0.0: " Solutions for NR to support non-terrestrial networks (NTN) (Release 16)"

[4]	TBA

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Availability: % of time during which the RAN is available for the targeted communication. Unavailable communication for shorter period than [Y] ms shall not be counted. The RAN may contain several access network components among which an NTN to achieve multi-connectivity or link aggregation.

Feeder link: Wireless link between NTN Gateway and satellite

Geostationary Earth orbit: Circular orbit at 35,786 km above the Earth's equator and following the direction of the Earth's rotation. An object in such an orbit has an orbital period equal to the Earth's rotational period and thus appears motionless, at a fixed position in the sky, to ground observers.

Low Earth Orbit: Orbit around the Earth with an altitude between 300 km, and 1500 km.

Medium Earth Orbit: region of space around the Earth above low Earth orbit and below geostationary Earth Orbit.

Minimum Elevation angle: minimum angle under which the satellite or UAS platform can be seen by a terminal.

Mobile Services: a radio-communication service between mobile and land stations, or between mobile stations

Mobile Satellite Services: A radio-communication service between mobile earth stations and one or more space stations, or between space stations used by this service; or between mobile earth stations by means of one or more space stations

Non-Geostationary Satellites: Satellites (LEO and MEO) orbiting around the Earth with a period that varies approximately between 1.5 hour and 10 hours. It is necessary to have a constellation of several Non-Geostationary satellites associated with handover mechanisms to ensure a service continuity.

Non-terrestrial networks: Networks, or segments of networks, using an airborne or space-borne vehicle to embark a transmission equipment relay node or base station.

NTN-gateway: an earth station or gateway is located at the surface of Earth, and providing sufficient RF power and RF sensitivity for accessing to the satellite. NTN Gateway is a transport network layer (TNL) node.

On Board processing: digital processing carried out on uplink RF signals aboard a satellite or an aerial.

On board NTN eNB: eNB implemented in the regenerative payload on board a satellite.

On ground NTN eNB: eNB of a transparent satellite payload implemented on ground.

One-way latency: time required to propagate through a telecommunication system from a terminal to the public data network or from the public data network to the terminal. This is especially used for voice and video conference applications.

Regenerative payload: payload that transforms and amplifies an uplink RF signal before transmitting it on the downlink. The transformation of the signal refers to digital processing that may include demodulation, decoding, re-encoding, re-modulation and/or filtering.

Round Trip Delay: time required for a signal to travel from a terminal to the sat-gateway or from the sat-gateway to the terminal and back. This is especially used for web-based applications.

Satellite: a space-borne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO), Medium-Earth Orbit (MEO), or Geostationary Earth Orbit (GEO).

Satellite beam: A beam generated by an antenna on-board a satellite

Service link: Radio link between satellite and UE

Transparent payload: payload that changes the frequency carrier of the uplink RF signal, filters and amplifies it before transmitting it on the downlink

User Connectivity: capability to establish and maintain data / voice / video transfer between networks and Terminals

User Throughput: data rate provided to a terminal

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ECEF	Earth-Centered, Earth-Fixed

EIRP	Equivalent Isotropic Radiated Power

GEO	Geostationary Earth Orbiting

eNB	4G Node B

GW	Gateway

LEO	Low Earth Orbiting

Mbps	Mega bit per second

MS	Mobile Services

MSS	Mobile Satellite Services

NGEO	Non-Geostationary Earth Orbiting

NTN	Non-Terrestrial Network

RAN	Radio Access Network

RTD	Round Trip Delay

SNR	Signal-to-Noise Ratio

TLE	Two-Line Element

Rx	Receiver

UE	User Equipment

# 4	IoT Non-Terrestrial Networks overview and scenarios

## 4.1	IoT Non-Terrestrial Networks overview

--- Start of text proposal ---

A non-terrestrial network refers to a network, or segment of networks using RF resources on board a satellite.

The typical scenario of a non-terrestrial network providing access to user equipment is depicted below:

Figure 4.1-1: Non-terrestrial network typical scenario based on transparent payload

Non-Terrestrial Network typically features the following elements:

-	One or several sat-gateways that connect the Non-Terrestrial Network to a public data network

-	a GEO satellite is fed by one or several sat-gateways which are to enable satellite coverage over the targeted area (e.g. regional or even continental coverage). It is assumed that UE in a cell are served by only one sat-gateway

-	A Non-GEO satellite served successively by one or several sat-gateways at a time. The system ensures service and feeder link continuity between the successive serving sat-gateways with sufficient time duration to proceed with mobility anchoring and hand-over

-	A Feeder link or radio link between a sat-gateway and the satellite

-	A service link or radio link between the user equipment and the satellite.

-	A satellite which implements a transparent payload. The satellite typically generate several beams over a given service area bounded by its field of view. The beam could be either earth fixed beam or earth moving beam for LEO. The footprints of the beams are typically of elliptic shape. The field of view of a satellite depends on the on board antenna design and minimum elevation angle.

-	A transparent payload: Radio Frequency filtering, Frequency conversion and amplification. Hence, the waveform signal repeated by the payload is un-changed;

-	User Equipment are served by the satellite within the targeted service area.

There may be different types of satellites listed here under:

Table 4.1-1: Types of NTN platforms

| Platforms | Altitude range | Orbit | Typical beam footprint size |
| --- | --- | --- | --- |
| Low-Earth Orbit (LEO) satellite | 300 – 1500 km | Circular around the earth | 100 – 1000 km |
| Geostationary Earth Orbit (GEO) satellite | 35 786 km | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | 200 – 3500 km |

Typically

●	GEO satellite are used to provide continental, regional or local service.

●	a constellation of LEO is used to provide services in both Northern and Southern hemispheres. In some case, the constellation can even provide global coverage including polar regions. For the later, this requires appropriate orbit inclination, sufficient beams generated and inter-satellite links.

--- End of text proposal ---

## 4.2	IoT Non-Terrestrial Networks reference scenarios

--- Start of text proposal ---

IoT NTN scenarios A, B, and C are included in the study as shown in Table 4.2-1 below:

| NTN Configurations | Transparent satellite |
| --- | --- |
| GEO based non-terrestrial access network | Scenario A |
| LEO based non-terrestrial access network generating steerable beams (altitude 1200 km and 600km) | Scenario B |
| LEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 1200 km and 600km) | Scenario C |

--- End of text proposal ---

# 5	IoT-NTN Architecture

TBA

# 6	Radio Layer 1 issues and related solutions

## 6.1	IoT NTN Reference Parameters

--- Start of text proposal ---

The IoT NTN reference scenario parameters are listed in Table 5.1-1 below:

| Scenarios | GEO based non-terrestrial access network - scenario A | LEO based non-terrestrial access network -Scenario B & C |
| --- | --- | --- |
| Orbit type | station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point | circular orbiting at low altitude around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Frequency Range (service link) | < 6 GHz (e.g. 2 GHz in S band) |  |
| Device channel Bandwidth (service link) (NOTE 7) | - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz (UL) - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations , including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) |  |
| Payload | Transparent type | Transparent Type |
| Earth-fixed beams | Yes | Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (NOTE 3) | 1000 km (NOTE 2) |
| Min Elevation angle for both sat-gateway and C-IoT device | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and C-IoT device at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | 541.46ms (service and feeder links) | 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links) |
| Max differential delay within a cell | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) (NOTE 6) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s (1200km) |
| C-IoT device motion on the earth | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h |
| C-IoT device antenna types | Omnidirectional antenna with 0 dBi TX antenna gain and 0 dBi RX antenna gain (NOTE 4) |  |
| C-IoT device max Tx power | UE power class 3 with up to 200 mW (23dBm), UE power class 5 with up to 100 mW (20 dBm) |  |
| C-IoT device Noise Figure | Omnidirectional antenna: 7 dB or 9 dB (NOTE 5) |  |
| Service link | 3GPP defined Narrow Band IoT and eMTC |  |

NOTE 1:    Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite.

NOTE 2:   This beam size refers to the Nadir pointing of the satellite.

NOTE 3: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam.

NOTE 4: The use of a Circular polarized antenna is optional.

NOTE 5: Same Noise Figure of 7 dB as in Release 16 TR 38.821 or 9 dB as in Release 12 TR 36.888  for device can be assumed for link budget. The noise figure is device vendor implementation specific.

NOTE 6: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link.

NOTE 7: System bandwidth is FFS

--- End of text proposal ---

## 6.2	Link Budget Analysis

--- Start of text proposal ---

6.2.1 Link Budget Parameters

The following assumptions are agreed for a common set of link budget parameters:

- UE power class (PC5=20 dBm)

- UE Noise Figure (NF=9 dB)

- Channel Bandwidth for NB-IoT and eMTC as was included in IoT NTN reference scenario parameters agreed in RAN1#103e

  - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz (UL)

  - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL)

- Other losses

| Other Losses | GEO (35786 km) | LEO (1200 km) | LEO (600 km) |
| --- | --- | --- | --- |
| Scintillation losses | 2.2 | 2.2 | 2.2 |
| Atmospheric losses | 0.2 | 0.1 | 0.1 |
| Polarization loss | 3 | 3 | 3 |
| Shadow margin | 3 | 3 | 3 |

NOTE 1: With PC3 (23 dBm) there is a 3dB gain compared to the PC5 (20 dBm) assumption on UL.

NOTE 2: With NF=7 dB, there is a 2 dB improvement compare to NF=9 dB on DL.

NOTE 3: Link budgets with other link budget parameters are not excluded from being captured in the TR.

NOTE 4: These parameters are only for the purpose of link budget calculations.

NOTE 5: Atmospheric losses are a function of elevation angle.

Link budget analysis assumes 3 dB polarization loss for DL and 3 dB polarization loss on UL for satellite parameters Set 1, Set 2, Set 3, and Set 4

For the satellite parameter sets Set-3 and Set-4, the 3 dB beam width (HPBW), central beam center elevation and central beam edge elevation in the satellite parameter set(s) to be used in link budget calculations are given in Table 6.2-1 and 6.2-2. These parameters correspond to the satellite parameter Set 3 and Set 4 given in Tables 6.2-3 and 6.2-4 respectively.

Table 6.2-1: Set-3 parameters for link budget analysis

| SET 3 | GEO 35786 km | LEO-600 km | LEO-1200 km |
| --- | --- | --- | --- |
| 3 dB Beam width (HPBW) | 0.735 degree | 22.0631 degree | 22.0631 degree |
| Central beam center elevation | 20.88 degree | 43.78 degree | 46.05 degree |
| Central beam edge elevation | 12.5 degree | 30 degree | 30 degree |
| Central beam edge satellite-UE distance | 40316 km | 1074 km | 1998 km |

Table 6.2-2: Set-4 parameters for link budget analysis

| SET 4 | LEO-600 km |
| --- | --- |
| 3 dB Beam width (HPBW) | 104.7 degree |
| Central beam center elevation | 90 degree |
| Central beam edge elevation | 30 degree |
| Central beam edge satellite-UE distance | 1076 km |

NOTE 1: The 3 dB beam width (HPBW) is already included in satellite parameter set 1 and Set 2 in TR 38.821 Table 6.1.1.1-1 and Table 6.1.1.1-2 respectively. The central beam center elevation for Set-1 and Set-2 is defined as the target elevation angle that is included in in TR 38.821 Table 6.1.3.2-1.   The central beam edge satellite-UE distance can be derived from the central beam edge elevation and does not need to be included.

NOTE 2: Central beam center elevation is the beam center elevation of the central beam in the beam layout.

NOTE 3: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

NOTE 4 In SLS evaluation with a multiple beam layout, the central beam is the serving beam for UEs. The outer beams have beam center elevation that is different from the central beam center elevation.  For the interference modelling, the interference due to the outer beams is determined by using their respective beam center elevations.

NOTE 5: For the multiple-beam satellite cell, the longest beam edge distance will correspond to the minimum beam edge elevation of the most outer beam as illustrated in figure below.

Figure 6.2-1 Illustration of beam layout and elevation angles for IoT NTN

The following satellite set parameters Set-1, Set-2, Set-3, and Set-4 can be used for the for the system level simulator calibration:

- Set 1 satellite parameters (based on TR 38.821, Table 6.1.1.1-1)

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Central beam center elevation |  | 12.5 degrees | 30 degrees | 30 degrees |
| Central beam edge elevation |  | 2.3 degrees | 26.3 degrees | 27.0 degrees |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. NOTE 6: The parameters corresponding to Ka-band for DL and UL in TR 38.821 Table 6.1.1.1-1 were removed. |  |  |  |  |

- Set 2 satellite parameters (based on TR 38.821, Table 6.1.1.1-2)

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Central beam center elevation |  | 12.5 degrees | 30 degrees | 30 degrees |
| Central beam edge elevation |  | [TBC] degrees | 22.2 degrees | 23.8 degrees |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 degrees | 8.8320 degrees | 8.8320 degrees |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. NOTE 6: The parameters corresponding to Ka-band for DL and UL in TR 38.821 Table 6.1.1.1-1 were removed. |  |  |  |  |

- Set 3 satellite parameters (Eutelsat R1-2101146 with central beam edge elevation 12.5 degree for GEO, and 30 degree for LEO-600 km and 1200 km)

Table 6.2-3: Set-3 satellite parameters for system level simulator calibration:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Central beam edge elevation |  | 12.5 degree | 30 degree | 30 degree |
| Central beam center elevation |  | 20.9 degree | 46.05 degree | 43.8 degree |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (NOTE 1) | S-band (i.e. 2 GHz) | 12 m | 0.4m | 0.4 m |
| Satellite EIRP density |  | 59.8 dBW/MHz | 33.7 dBW/MHz | 28.3 dBW/MHz |
| Satellite Tx max Gain |  | 45.7 dBi | 16.2 dBi | 16.2 dBi |
| 3dB beam width (HPBW) |  | 0.7353 degree | 22.1 degree | 22.1 degree |
| Satellite beam diameter (NOTE 2) |  | 459km | 470 km | 234 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (NOTE 1) | S-band (i.e. 2 GHz) | 12 m | 0.4 m | 0.4 m |
| G/T |  | 16.7dB K-1 | -12.8 dB K-1 | -12.8 dB K-1 |
| Satellite Rx max Gain |  | 45.7 dBi | 16.2 dBi | 16.2 dBi |

NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811

NOTE 2: Satellite beam diameter is at Nadir point

NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821

NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

- Set 4 satellite parameters (Thales, Sateliot, Gatehouse R1-2101019)

Table 6.2-4: Set-4 satellite parameters for system level simulator calibration:

| Satellite orbit |  | LEO-600 |
| --- | --- | --- |
| Satellite altitude |  | 600 km |
| Central beam edge elevation |  | 30 degree |
| Central beam center elevation |  | 90 degree |
| Payload characteristics for DL transmissions |  |  |
| Equivalent satellite antenna aperture (NOTE 1) | S-band (i.e. 2 GHz) | 0.097 m |
| Satellite EIRP density |  | 21.45 dBW/MHz |
| Satellite Tx max Gain |  | 11 dBi |
| 3dB beam width (HPBW) |  | 104.7 degree |
| Satellite beam diameter (Note 2) |  | 1700 km |
| Payload characteristics for UL transmissions |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 0.097 m |
| G/T |  | - 18.6 dB·K-1 |
| Satellite Rx max Gain |  | 11 dBi |

NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811

NOTE 2: Satellite beam diameter is at Nadir point

NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821

NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

--- End of text proposal ---

## 6.3	Timing Relationships

--- Start of text proposal ---

The following aspects related to timing relationships enhancements will be studied to check whether enhancement is necessary and beneficial:

For NB-IoT:

- NPDCCH to NPUSCH format 1

- RAR grant to NPUSCH format 1

- NPDSCH to HARQ-ACK on NPUSCH format 2

- NPDCCH order to NPRACH

- Timing advance command activation

- FFS: Other NB-IoT timing relationships

For eMTC:

- MPDCCH to PUSCH

- RAR grant to PUSCH

- PDCCH order to PRACH

- MPDCCH to scheduled uplink SPS

- PUSCH to HARQ-ACK on PUCCH

- CSI reference resource timing

- MPDCCH to aperiodic SRS

- Timing advance command activation

- FFS: Other eMTC timing relationships

Impact of large RTD (which impacts TA) on HD-FDD UL-DL timing relationships

The study will identify IoT-NTN configurations needing activation/de-activation via MAC CE and their timing relationships.

--- End of text proposal ---

## 6.4	Time and Frequency Synchronization

--- Start of text proposal ---

The following aspects related to enhancements to time and frequency synchronization will be studied:

- GNSS measurement window for initial access.

- Potential impact of GNSS Position fix on UE power consumption considering at least the following parameters

  - GNSS power consumption value

  - GNSS position Time To First Fix

- Potential impact of NTN SIB carrying the satellite ephemeris on

  - UE power consumption in NB-IoT and eMTC

  - Accuracy of satellite location tracking

  - RACH congestion

- UE pre-compensation of satellite delay and satellite Doppler shift during long UL transmission on (N-)PUSCH and PRACH in NB-IoT and eMTC.

--- End of text proposal ---

## 6.5	HARQ

--- Start of text proposal ---

For NTN IoT, potential HARQ enhancements need to consider the main characteristics of an IoT device, which are low complexity, low cost, low power consumption and low throughput, and key requirements of IoT services which are extended coverage, delay-tolerant and infrequent data transmissions, and support of massive communications.

The peak throughput of IoT UEs operating over NTN is not expected to be higher than the peak throughput of IoT UEs operating over TN.

The following aspects related to HARQ enhancements will be studied:

Potential benefits and/or drawbacks of

- Increasing the number of HARQ processes on throughput, latency, power consumption and complexity

- Disabling HARQ feedback for NB-IoT

- Disabling HARQ feedback for eMTC.

Necessity, potential benefits and/or drawbacks relation to HARQ operation in IoT NTN

- Any other potential HARQ feedback mechanisms

- Reduced PDCCH monitoring

- Coverage enhancements

- Uplink transmission gaps with multiple HARQ processes

- Maintaining HARQ process continuity in serving cell change

- Multiple Transport Blocks scheduling

- Throughput enhancements

The study will identify whether HARQ stalling happens at least in the GEO satellite scenario.

--- End of text proposal ---

# 7	User Plane Aspects

## 7.1	Random Access Procedure

TBA

## 7.2	MAC Timers

TBA

## 7.3	HARQ

TBA

# 8	Control Plane Aspects

## 8.1	System Information Enhancements

TBA

## 8.2	Idle Mobility Enhancements

TBA

## 8.3	Radio Link Failure Enhancements (NB-IoT Only)

TBA

## 8.4	Connected Mobility Enhancements (eMTC only)

TBA

## 8.5	Tracking Area Update Enhancements

TBA

# 9	Recommendations on the way forward

## 9.1	Recommendations from RAN1

TBA

## 9.2	Recommendations from RAN2

TBA

# Annex A:	Satellite ephemeris

Annex B:	KPI and evaluation assumptions

# B.1	Key Performance Indicators

KPIs defined in TR38.913 are considered.

# B.2	Performance targets for evaluation purposes

# Annex C (Informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-11 | RAN#90e | R3-185304 |  |  |  | Skeleton TR | 0.0.0 |