3GPP TSG RAN WG1 #104-bis-e                   		                             R1-2102810

e-Meeting, April 12th  – April 20th, 2021

Source: 	Fraunhofer IIS, Fraunhofer HHI

Title:	CSI enhancements on Type II PS codebook and multi-TRP

Agenda item:       8.1.4

Document for:	Decision

# Introduction

In RAN1#104-e, the following agreements [1] were made for enhancements on the Rel. 16 Type II PS codebook and CSI reporting for multi-TRP.

Agreement For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, support codebook structure W=W1W2 WfH where - W1 is a free selection matrix, with identity matrix as special configuration - FFS polarization-common/specific selection - Wf is a DFT based compression matrix in which N3 = NCQISubband*R and Mv>=1 - At least one value of Mv>1 is supported - Decide on the value(s) of Mv, e.g. Mv=2, in RAN1# 104bis-e - Working assumption: Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters - FFS candidate value(s) of R, mechanism for configuring/indicating to the UE and/or mechanism for selecting/reporting by UE for Wf - Wf can be turned off by gNB. When turned off, Wf is an all-one vector (FFS; the length of all-one vector) - FFS other signaling/CSI reporting mechanism for trade-off among signaling overhead, UE complexity and UPT gain Agreement For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, - W1 ∈ N^{P×K1} (K1≤P) is a port selection matrix in order to freely select K1 ports out of P CSI-RS ports or K1/2 ports out of P /2 CSI-RS ports - Note that P is the number of CSI-RS ports for port selection (whose value depends on the outcome of the CSI-RS related study). Agreement For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, study following options (or combinations) for CSI-RS configurations associated with Rel-17 PS codebook for supporting low CSI-RS overhead and/or CSI-RS processing complexity considering the impact on UPT performance under realistic CSI-RS measurement: - Option 0: No further CSI-RS enhancement as the baseline - Option 1: Support configuring a lower CSI-RS density per CSI-RS resource, e.g., 0.25 - Option 2: Support configuring one or multiple CSI-RS patterns per CSI-RS resource associated with Rel-17 PS codebook - Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis - Configure UE with two CMR groups with Ks=K1+K2 CMRs. CMR pairs are determined from two CMR groups by following method(s). - K1 and K2 are the number of CMRs in two groups respectively. FFS K1=K2 or different K1/K2. - Note that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses - N CMR pairs are higher-layer configured by selecting from all possible pairs - signalling mechanism can be discussed further, e.g. using a bitmap - FFS: Whether MAC-CE or RRC+MAC CE indication is needed - FFS: how to support NCJT measurement hypotheses in FR2 - Support N=1 and Ks =2, FFS other maximal values of N>1 and Ks>2 - Note: for CPU/resource/port occupation, NCJT hypothesis is considered separately from single TRP hypothesis Agreement - Strive to agree at most one of the following options, if needed - Option 1: Confirm the Working Assumption from RAN1 103e. - Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT - The time of decision is RAN1#105e (May 2021) Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options: - Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis - X = 0, 1, 2 - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups - Support of X=1,2 is UE optional for the UE supporting option 1 - FFS omission of CSI associated with NCJT measurement hypothesis - Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses - FFS how to report recommended measurement hypothesis associated with that CSI report

In this contribution, we provide our views on the performance of Rel. 17 PS codebook and enhancements on multi-TRP CSI reporting.

# Enhancements on Type II PS codebook

For the Rel. 17 PS codebook each port is beamformed with an SD-FD pair i.e., with a spatial beam vi (SD) and a frequency domain (FD) component  fd,i(s) i.e., a delay. For the calculation of the precoder, a single wideband SVD operation is sufficient compared to the Rel.-16 precoder calculation, where an SVD operation is performed per subband; thus reducing the computational complexity of the precoder. The UE then selects a number of coefficients, K, associated with a subset of SD-FD pairs, and reports the same to the gNB. In the following, details regarding the design of Wf and W1 are discussed.

Design of Wf:

Number of delays Mv: The performance of the Rel. 17 PS CB is evaluated for different values of Mv for a single-layer transmission. In addition to the performance of the Rel. 16 eType II PS codebook, the performance of Rel. 16 eType II regular codebook is shown. The simulation parameters are provided in the Table in the appendix. For the Rel. 17 PS CB, the amplitude and phase of the precoder coefficients are quantized with 3 and 4 bits, respectively.

Figure 1 shows the performance gain and feedback overhead of the Rel. 16 eType II PS codebook and Rel. 16 eType II regular codebook for parameter combinations {PC3, PC4, PC5, PC6} [3]. For the sake of convenience, (L,Mv,K) for PC3, PC4 and PC5 are (4,4,8), (4,4,16), (4,4,24), respectively. For PC6, (L,Mv,K) is (4,7,28). The Rel. 17 codebook alternatives are evaluated for 16 and 32 CSI-RS ports, and a single SD-FD pair is mapped to a CSI-RS port. The performance gain is calculated by setting the performance achieved using the Rel. 16 PS codebook - PC3 as a reference. The performance gain and overhead of the Rel. 17 PS codebook for Mv=1  and Mv>1 are shown for the following combinations of (PCSI-RS, Mv,K):

Mv=1: {16,1,8}, {16,1,16},  {32,1,16}, {32,1,24} and {32,1,32},

Mv>1,:{16,2,8}, {16,4,8}, {16,2,16}, {32,2,16}, {32,4,16} and {32,4,24}.

Figure 1: Performance of the Rel. 16 eType II PS, Rel. 16 eType II regular CBs for PC3 - PC6 and the Rel. 17 PS CB for different combinations (PCSI-RS, Mv,K).

From Figure 1, it can be observed that when Mv>1 a significant performance gain  is achieved compared to Mv=1 and the Rel. 16 eType II regular codebook for all parameter combinations. When the channel is perfectly reciprocal, a single delay suffices to achieve a reasonable performance. However, as the channel is not reciprocal in some scenarios [2], Mv>1 may be configured to compensate for the channel non-reciprocity. By doing so, the beamformed delays at the gNB can be correctly adjusted to match the delays of the downlink channel. Moreover, as delay beamforming is already performed at the gNB side, the Rel. 17 PS CB requires a smaller number of delays compared to the Rel. 16 eType II codebook. Therefore, in addition to supporting Mv=1, at least one value for Mv in the range of [2,4] shall be mandatorily supported to achieve significantly enhanced performance compared to Mv=1.

Observation: The Rel. 17 PS CB using Mv>1 delays outperforms the Rel. 16 eType II PS codebook and Rel. 17 PS CB using a single delay.

Proposal: Support at least one value of M in the range of 2,4.

Window size N4 and starting index  of the window Minit: For Rel. 16 Type II codebook, for N3>19, the UE is configured with a window of size 2Mv delays, and the UE freely selects Mv delays, and reports the starting index of the window Minit as well as the selected Mv delays out of the 2Mv delays. However, as mentioned before, as delay beamforming is already performed by the gNB, a small number of delays is only required by the UE to adjust the beamformed delays to match the channel delays. Also, as the dominant channel tap locations of the uplink channel and the downlink channel differ by only a small number [2], such that a smaller window size suffices to achieve a good performance compared to the Rel. 16 eType II regular codebook.

Observation: As the dominant channel tap locations of the uplink and the downlink channel differ by a small number, a small window size suffices to achieve a good performance.

Two cases are considered in the following regarding the window size N4, starting index of the window Minit and the selection of delays. In the first case, the size of the window is gNB configured or fixed to N4 = 2Mv, whereas the starting index of the window Minit is freely selected and reported by the UE. This means that the UE is given full flexibility to select the delays from the entire codebook from a window of size 2Mv. In the second case, the size of the window as well as the starting index Minit of the window is configured by the gNB as N4=Mv and Minit=0, respectively, and all delays are selected by the UE from the configured window of size Mv.

The performance gain and overhead of the Rel. 17 PS codebook are shown in Figure 2 for the following combinations of (PCSI-RS, Mv,K,N4) for the aforementioned two window configurations:

N4=2Mv: 16,2,8,4,16,4,8,8,16,2,16,4,32,2,16,4,32,4,16,8, and 32,4,24,8,

N4=Mv: 16,2,8,2,16,4,8,4,16,2,16,2,32,2,16,2,32,4,16,4, and 32,4,24,4.

When the window size and starting index of the window are configured by the gNB, the UE selects Mv=2 delays from the first N4=2 entries of the codebook. For this case, an FD indicator is not needed. In the case of UE selecting and reporting the starting index Minit of the window, the UE selects Mv=2 delays from  N4=2Mv adjacent entries from the codebook of size N3≫N4. In this case, an FD indicator is further needed to report the selected delays. From Fig. 2, it can be seen that the performance of the Rel. 17 PS CB for both window configurations outperforms the Rel. 16 eType II regular codebook. It can also be inferred that for the Rel. 17 PS CB, reducing the window size to Mv from 2Mv and fixing Minit to zero results in a very small performance loss compared to using a large window size and flexibly selecting Minit. Therefore, for the Rel. 17 PS CB,  the starting index of the window Minit can be fixed to zero, and the window size can be reduced by half compared to the window size supported in Rel. 16 PS CB.

Observation: The Rel. 17 PS CB with fixed starting index of the window (Minit=0) outperforms the Rel. 16 eType II regular codebook for parameter combinations PC3 - PC6.

Observation: No significant performance difference is observed when reducing the window size N4 from 2Mv to Mv.

Observation: No significant performance difference is observed by fixing Minit to zero compared to UE selection and reporting.

Proposal: For the Rel. 17 PS CB, the size of the window is equal to the number of delays Mv.

Proposal: Support fixing or configuring the starting index of window Minit to be zero.

Figure 2: Performance of the Rel. 17 PS CB for different two different window configurations with two different window sizes for different parameter combinations (PCSI-RS, M,K,N4).

Value of R: Figure 3 shows the performance of the Rel. 17 PS CB with increasing values of R for different parameter combinations (PCSI-RS, M,K). It can be observed that when using R=2, the performance of Rel. 17 PS CB improves for all parameter combinations. Further increasing the value of R to 4 results only in a slight improvement in performance. Therefore, based on the simulation results, a value of R=2 can be supported for Rel. 17 PS CB.

Observation: Using R=4 results only in a slight performance improvement  compared to R=2.

Proposal: Support R=2 for Rel. 17 PS codebook.

Figure 3: Performance of the Rel. 17 PS CB for two different value of R and for different parameter combinations (PCSI-RS, M,K).

Design of W1: Although, relaxing the Rel. 16 port selection constraint for the Rel. 17 PS codebook drastically improves the performance, the feedback overhead required to report the selected ports can still be high. Figure 4 shows the performance of the Rel. 17 PS codebook for different parameter combinations for both polarization specific and polarization common selection. As observed from the figure, the performance of the polarization specific and polarization common selections differs only slightly. Therefore, polarization common port selection can be considered as it also results in lower feedback overhead.

Figure 4: Performance of the Rel. 17 PS CB for with polarization specific and polarization common port selection for different parameter combinations (PCSI-RS, M,K).

Observation: Polarization common port selection results in a similar performance but with reduced feedback overhead compared to polarization specific port selection.

Proposal: Support polarization common port selection for W1.

From Figure 1, it is observed that the Rel. 17 PS CB already outperforms both the Rel. 16 eType II PS CB and Rel. 16 eType II Reg. codebook for PCSI-RS≤32 ports with a single SD-FD pair mapping to a CSI-RS port. Therefore, any kind of CSI-RS enhancements seems to be not required for Rel. 17 PS CB.

Observation: The Rel. 17 PS CB outperforms the Rel. 16 eType II PS codebook for PCSI-RS≤32 ports with single SD-FD pair mapping to a CSI-RS port.

Proposal: Support option 0 – no further CSI-RS enhancements.

# CSI enhancements for Multi-TRP

In RAN1#104e, it was agreed that the UE can be configured with two groups with Ks =K1+K2 CMRs, where K1 and K2 are the number of CMRs in the first and second group, respectively. The CMRs in a group belong to the same TRP. Hence, a CMR can be configured only in one group. Moreover, the UE can be configured with N ≥ 1 NZP CSI-RS resource pairs determined from the two groups, and each pair is used for a NCJT measurement hypothesis. A straightforward way to indicate the CMR pairs from the two groups is to use a bitmap of size K1K2, where each bit in the bitmap is associated with a CMR pair and is associated with a NCJT measurement hypothesis.

Proposal: A bitmap of size K1K2 is used to indicate the CMR pairs associated with the N NCJT measurement hypotheses.

It was further agreed that the UE can be configured with either a single CSI for the selected hypothesis among the single-TRP and NCJT measurement hypotheses (option 2) or with X CSIs for X selected single-TRP measurement hypotheses (if configured) and one CSI for a selected NCJT measurement hypothesis. The selected CSI hypotheses are indicated in the CSI report by CRI(s), where for option 2 the CSI report comprises one CRI and for option 1 the CSI report comprises one CRI for the selected NCJT measurement hypothesis and X CRIs (if configured) for the selected single-TRP measurement hypotheses. To uniquely determine the selected single-TRP and/or NCJT hypotheses, a one-to-one mapping between CRI codepoints and CMRs/CMR pairs is needed. The CRI mapping can be higher-layer configured, or it is indirectly defined by mapping the Ks CMRs (that correspond to the Ks single-TRP hypotheses) to Ks CRIs codepoints and the N CMR pairs (that correspond to the N NCJT hypotheses defined by the bitmap) to additional N CRI codepoints.

Proposal: A one-to-one mapping between CRI codepoints and CMRs/CMR pairs is used. Each CRI codepoint is associated with a single-TRP or NCJT measurement hypothesis.

The UCI payload depends on the selected (single-TRP and/or NCJT) hypothesis and can vary a lot. To take into account the different playoad sizes of the UCI, a two-part UCI structure similar to the Rel. 16 UCI structure can be adopted. The UCI comprises two parts, where part 1 has a fixed payload size and indicates the size of part 2. For option 2, UCI part 1 comprises one CRI, two RIs, a wideband CQI and sub-band CQIs. For option 1, UCI part 1 comprises X CRIs, X RIs and X wideband CQIs and the sub-band CQIs for the X selected single-TRP measurement hypotheses (if configured), and one CRI, two RIs and one wideband CQI and the sub-band CQIs for the NCJT measurement hypothesis. The UCI part 2 has a varying payload size and comprises the remaining CSI content.

Proposal: A two-part UCI structure is adopted for option 1 and option 2.

- For option 2, UCI part 1 comprises one CRI, two RIs, a wideband CQI and sub-band CQIs,

- For option 1, UCI part 1 comprises X CRIs, X RIs and X wideband CQIs and the sub-band CQIs for the X selected single-TRP measurement hypotheses (if configured), and one CRI, two RIs, one wideband CQI and the sub-band CQIs for the selected NCJT measurement hypothesis.

# Conclusions

Based on the above discussions, we have the following observations and proposals.

Observation: The Rel. 17 PS CB using Mv>1 delays outperforms the Rel. 16 eType II PS codebook and Rel. 17 PS CB using a single delay.

Proposal: Support at least one value of M in the range of 2,4.

Observation: As the dominant channel tap locations of the uplink and the downlink channel differ by a small number, a small window size suffices to achieve a good performance.

Observation: The Rel. 17 PS CB with fixed starting index of the window (Minit=0) outperforms the Rel. 16 eType II regular codebook for parameter combinations PC3 - PC6.

Observation: No significant performance difference is observed when reducing the window size N4 from 2Mv to Mv.

Observation: No significant performance difference is observed by fixing Minit to zero compared to UE selection and reporting.

Proposal: For the Rel. 17 PS CB, the size of the window is equal to the number of delays Mv.

Proposal: Support fixing or configuring the starting index of window Minit to be zero.

Observation: Using R=4 results only in a slight performance improvement compared to R=2.

Proposal: Support R=2 for Rel. 17 PS codebook.

Observation: Polarization common port selection results in a similar performance with reduced feedback overhead compared to polarization specific port selection.

Proposal: Support polarization common port selection for W1.

Observation: The Rel. 17 PS CB outperforms the Rel. 16 eType II PS codebook for PCSI-RS≤32 ports with single SD-FD pair mapping to a CSI-RS port.

Proposal: Support option 0 – no further CSI-RS enhancements.

Proposal: A bitmap of size K1K2 is used to indicate the CMR pairs associated with the N NCJT measurement hypotheses.

Proposal: A one-to-one mapping between CRI codepoints and CMRs/CMR pairs is used. Each CRI codepoint is associated with a single-TRP or NCJT measurement hypothesis.

Proposal: A two-part UCI structure is adopted for option 1 and option 2.

- For option 2, UCI part 1 comprises one CRI, two RIs, a wideband CQI and sub-band CQIs,

- For option 1, UCI part 1 comprises X CRIs, X RIs and X wideband CQIs and the sub-band CQIs for the X selected single-TRP measurement hypotheses (if configured), and one CRI, two RIs, one wideband CQI and the sub-band CQIs for the selected NCJT measurement hypothesis.

# References

- Chairman Notes, RAN1#104-e, E-meeting, Jan 25th – Feb 05th, 2020.

- 3GPP R1-2101761, On FDD channel reciprocity in real-world scenarios, Fraunhofer IIS, Fraunhofer HHI, RAN1#103-e, E-meeting, 25th January – 05th February 2020.

- 3GPP TS 38.214, Physical layer procedures for data (Rel. 16), V16.0.0, December 2019.

Table 1: Simulation parameters

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL |
| Inter-BS distance |  | 200m |
| Channel model |  | The reciprocity model of DL/UL channel is based on Section 7.6.5 of TR 38.901 with different DL/UL frequency. |
| Antenna setup and port layouts at gNB |  | - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 44dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz for 15kHz as a baseline |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 70% for SU/MU-MIMO with rank adaptation |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-16 PS eTypeII Codebook is the baseline for performance and overhead evaluation. |
| SRS modeling for UL channel estimation |  | SRS periodicity with 5ms/10ms SRS error modeling in Table A.1-2 in 36.897. |
| FDD DL/UL calibration error model at gNB |  | - is the spatial UL channel at gNB side with calibration error - is the ideal spatial UL channel without calibration error - E represents the mismatch of transmission and reception circuits of gNB - ai is the amplitude error - i is the phase error - N is the number of antennas at gNB side With amplitude error (expressed in decibel of ) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. Both amplitude/phase errors are assumed to be constant during a simulation drop at time, and constant either across whole simulation bandwidth or per 4 PRB at frequency. Companies shall report the assumption of error modelling at frequency. |