3GPP TSG- RAN WG1 Meeting #104b-e                                                 R1-2103094

# e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.1.4

Source:	Apple Inc.

Title:	Views on Rel-17 CSI enhancement

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Regarding CSI measurement and reporting enhancement, the following objectives are approved as part of Rel-17 FeMIMO [1]

- Enhancement on CSI measurement and reporting: - Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2 - Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

In the last RAN1 meeting RAN1#104e, the following agreement is reached [2]

Agreement For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, support codebook structure W=W1W2 WfH where - W1 is a free selection matrix, with identity matrix as special configuration - FFS polarization-common/specific selection - Wf is a DFT based compression matrix in which N3 = NCQISubband*R and Mv>=1 - At least one value of Mv>1 is supported - Decide on the value(s) of Mv, e.g. Mv=2, in RAN1# 104bis-e - Working assumption: Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters - FFS candidate value(s) of R, mechanism for configuring/indicating to the UE and/or mechanism for selecting/reporting by UE for Wf - Wf can be turned off by gNB. When turned off, Wf is an all-one vector (FFS; the length of all-one vector) - FFS other signaling/CSI reporting mechanism for trade-off among signaling overhead, UE complexity and UPT gain Agreement For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, - W1 ∈ N^{P×K1} (K1≤P) is a port selection matrix in order to freely select K1 ports out of P CSI-RS ports or K1/2 ports out of P /2 CSI-RS ports - Note that P is the number of CSI-RS ports for port selection (whose value depends on the outcome of the CSI-RS related study). Agreement For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, study following options (or combinations) for CSI-RS configurations associated with Rel-17 PS codebook for supporting low CSI-RS overhead and/or CSI-RS processing complexity considering the impact on UPT performance under realistic CSI-RS measurement: - Option 0: No further CSI-RS enhancement as the baseline - Option 1: Support configuring a lower CSI-RS density per CSI-RS resource, e.g. 0.25 - Option 2: Support configuring one or multiple CSI-RS patterns per CSI-RS resource associated with Rel-17 PS codebook - Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis - Configure UE with two CMR groups with Ks=K1+K2 CMRs. CMR pairs are determined from two CMR groups by following method(s). - K1 and K2 are the number of CMRs in two groups respectively. FFS K1=K2 or different K1/K2. - Note that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses - N CMR pairs are higher-layer configured by selecting from all possible pairs - signalling mechanism can be discussed further, e.g. using a bitmap - FFS: Whether MAC-CE or RRC+MAC CE indication is needed - FFS: how to support NCJT measurement hypotheses in FR2 - Support N=1 and Ks =2, FFS other maximal values of N>1 and Ks>2 - Note: for CPU/resource/port occupation, NCJT hypothesis is considered separately from single TRP hypothesis Agreement - Strive to agree at most one of the following options, if needed - Option 1: Confirm the Working Assumption from RAN1 103e. - Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT - The time of decision is RAN1#105e (May 2021) Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options: - Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis - X = 0, 1, 2 - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups - Support of X=1,2 is UE optional for the UE supporting option 1 - FFS omission of CSI associated with NCJT measurement hypothesis - Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses - FFS how to report recommended measurement hypothesis associated with that CSI report

In this contribution, we provide our views on the Rel-17 FeMIMO CSI enhancement in the following areas

- CSI measurement and reporting enhancement for MTRP

- Type II port selection codebook enhancement utilizing partial DL/UL channel reciprocity

# CSI enhancement for MTRP

Rel-15/16 CSI measurement and reporting adopts the design framework of CMR and IMR, in which CMR is configured for channel measurement, and IMR is configurated for interference measurement. In terms of IMR, there are two types of IMR, i.e., NZP-IMR (a.k.a. NZP-CSI-RS) and ZP-IMR (a.k.a. CSI-IM).

For CSI enhancement for MTRP, the main focus is on the NJCT (Non-Coherent Joint Transmission) scheme, in which, the PDSCH transmitted from two TRP can overlap in frequency. For example, Multi-DCI based Multi-TRP operation and Single-DCI based Multi-TRP operation with SDM mode can be used to schedule explicit NCJT operation to the UE.

Rel-16 solution focused on the PDSCH scheduling enhancement, by introducing 6 different modes, 1 for Multi-DCI based Multi-TRP and 5 for Single-DCI based Multi-TRP. However, CSI enhancement is not allowed in Rel-16. Figure 1 illustrates a simple issue for NCJT. Assume TRP1 transmits one CSI-RS resource set and TRP 2 transmits the other CSI-RS resource set, we have the following observation

- There is no clear definition of CMR and IMR for NCJT

  - When TRP1 transmits its PDSCH, the CSI-RS resource set from TRP 1 is CMR, while CSI-RS resource set from TRP 2 is IMR

  - When TRP2 transmits its PDSCH, the CSI-RS resource set from TRP 2 is CMR, while CSI-RS resource set from TRP 1 is IMR

In other words, for UE to derive the optimal CSI reporting, UE needs to jointly consider the reciprocal CMR/IMR relationship between TRP1 and TRP2. Using PMI as an example, UE needs to jointly solve the following optimization problem, note that the problem formulation is loose and for illustration purpose only

MAXw1,w2CH1*W1H2*W2+CH2*W2H1*W1

Whereas

- H1 and H2 are the channel from TRP1 and TRP2 respectively

- W1 and W2 are the precoding matrix (PMI) of TRP1 and TRP2 respectively

- C∙ is the capacity function

Figure 1 Reciprocal relationship between CMR and IMR for NCJT

From the above analysis, we can see that, for UE to provide the optimal CSI reporting. The following two things are required

- The flexible CSI measurement RS configuration to reflect the reciprocal relationship between CMR and IMR

- The flexible CSI report quantity configuration to allow UE to report joint optimization results in terms of PMI/RI/CRI/LI/CQI.

When considering multi-DCI based multi-TRP, currently, gNB schedules two independent TB from each TRP. Therefore, for a CSI report associated with multi-DCI based Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report

- Two RIs, two PMIs, two LIs and two CQIs

Proposal 1 For a CSI report associated with multi-DCI based Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report

- Two RIs, two PMIs, two LIs and two CQIs

For interference measurement under NCJT, since the transmission from both TRP interfere with each other, it is reasonable that UE should consider the CMR, including RI/PMI decision, from the other TRP as the interference for the intended TRP. In other words, the RI and PMI decision from both TRP should be considered jointly and signals from one TRP should be considered as the interference to the other TRP.

Proposal 2 For interference measurement under NCJT, CMR, including RI/PMI decision, from one TRP should be considered as the interference, i.e. IMR, to the other TRP.

For reporting mechanism, we need to consider the following three cases

- Single TRP operation: Report the best TRP under the assumption that the other TRP is blanked

- Single TRP operation: Report each TRP under the assumption that the other TRP is blanked

- Multiple TRP operation: Report both TRP under NCJT operation

We have the following proposal

Proposal 3 For reporting mechanism, regarding the following three cases

- Single TRP operation: Report the best TRP under the assumption that the other TRP is blanked

- Single TRP operation: Report each TRP under the assumption that the other TRP is blanked

- Multiple TRP operation: Report both TRP under NCJT operation

gNB can configure one or multiple of them

- UE can independently indicate whether UE supports each reporting or not as capability

- The CPU and active RS counting rule needs to be further discussed and clarified

# CSI enhancement with partial DL/UL channel reciprocity

Even for FDD spectrum, it is believed that DL and UL channel still have certain reciprocity, i.e., share similar channel statistic, even though the actual channel can be highly uncorrelated. For DL/UL channel partial reciprocity, it is generally the observation that the following two channel properties can be very similar between DL and UL

- Channel delay and power profile

- Channel angle in terms of AoA/AoD

In general, this is assuming that the DL and UL go through the similar reflection and scattering, and the wavelength of DL and UL signals are also similar even with frequency duplexing separation.

In last meeting, CSI-RS enhancement and potential SD-FD paring concept was discussed. Currently, we support CSI-RS with maximum 32 ports, which can be configured for both reciprocity and non-reciprocity based CSI report. For non-reciprocity based CSI report, gNB needs to sound all the transmit port for UE to feedback the CSI. However, for reciprocity based CSI, gNB already acquires certain DL channel information based on the UL channel estimate, and gNB already precode the CSI-RS with the consideration of the maximum MIMO layers the UE can support, i.e., the maximum Rx ports at the UE side. Normally, the maximum number of Rx ports at the UE side is much smaller than the maximum number of Tx ports at the gNB side, as results, 32 port CSI-RS should be enough for the reciprocity based CSI feedback, i.e., port selection codebook.

On the other side, CSI-RS capability can be an issue for reciprocity based CSI feedback since gNB needs to precode the CSI-RS differently for each user, as different user may experience different channel. There can be a need to increase the multiplexing order, or capability, of CSI-RS by allowing more users to be configured with different CSI-RS with the same amount of REs. After analyzing the CSI-RS configuration allowed by the current specification 38.211, we believe the current CSI-RS already allows very flexible CSI-RS configuration. If there is any enhancement needed, allowing lower density can be a simple and clean solution. In summary, we have the following proposal

Proposal 4 For port selection codebook enhancement,

- Do not introduce SD-FD pairing

- Do not introduce CSI-RS with more than 32 ports

- We can consider CSI-RS enhancement to allow more users to be multiplexed in the same amount of REs, solution can be allowing lower density CSI-RS, e.g., 0.5, 0.25

Delay and power profile determine the frequency selectivity of the channel. To handle the frequency selective channel, Rel-15/16 CSI allows both the Wideband and Subband CSI reporting. For Subband configuration, NW can select one of the two possible subband sizes via RRC for the BWP whose size is greater than or equal to 24 PRB. If NW can obtain the channel frequency selectivity more dynamically via the SRS measurement, more dynamic wideband and subband CSI reporting configuration can be considered

Proposal 5 For CSI enhancement utilizing partial reciprocity of DL/UL channels, more flexible wideband and subband CSI reporting configuration can be considered

Proposal 6 For CSI enhancement utilizing partial reciprocity of DL/UL channels, W1 is polarization common

# Conclusion

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Regarding CSI measurement and reporting enhancement, in this contribution, we provide our views on the following areas

- CSI measurement and reporting enhancement for MTRP

- Type II port selection codebook enhancement utilizing partial DL/UL channel reciprocity

We have the following proposals

Proposal 1 For a CSI report associated with multi-DCI based Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report

- Two RIs, two PMIs, two LIs and two CQIs

Proposal 2 For interference measurement under NCJT, CMR, including RI/PMI decision, from one TRP should be considered as the interference, i.e. IMR, to the other TRP.

Proposal 3 For reporting mechanism, regarding the following three cases

- Single TRP operation: Report the best TRP under the assumption that the other TRP is blanked

- Single TRP operation: Report each TRP under the assumption that the other TRP is blanked

- Multiple TRP operation: Report both TRP under NCJT operation

gNB can configure one or multiple of them

- UE can independently indicate whether UE supports each reporting or not

- The CPU and active RS counting rule needs to be further discussed and clarified

Proposal 4 For port selection codebook enhancement,

- Do not introduce SD-FD pairing

- Do not introduce CSI-RS with more than 32 ports

- We can consider CSI-RS enhancement to allow more users to be multiplexed in the same amount of REs, solution can be allowing lower density CSI-RS, e.g., 0.5, 0.25

Proposal 5 For CSI enhancement utilizing partial reciprocity of DL/UL channels, more flexible wideband and subband CSI reporting configuration can be considered

Proposal 6 For CSI enhancement utilizing partial reciprocity of DL/UL channels, W1 is polarization common

# Reference

- RP-193133, "New WID: Further enhancements on MIMO for NR", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman Notes, 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting, January 25th – February 5th, 2021