3GPP TSG RAN WG1 #104bis-e	R1-21xxxxx

e-Meeting, April 12th – 20th, 2021

Agenda item:		5.1

Source:	Moderator (China Telecom)

Title:	[104b-e-NR-R17-TxSwitching-01] Summary of email discussion on Rel-17 uplink Tx switching

Document for:		Discussion

# Introduction

In RAN #89 e-meeting, a new Rel-17 WID of “RF requirements enhancement for NR frequency range 1 (FR1)” [1] was approved and was revised in RAN #91 e-meeting [2], including following objectives.

- Specify UE requirements to enable Tx switching between different cases across carriers based on SUL and NR inter-band uplink CA for UE supporting maximum two concurrent transmissions

  - Specify UE requirements to enable Tx switching between cases

    - The scenarios include

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - Specify the following RAN4 requirements for above scenarios

      - Length of switching period

      - Time mask RF requirements

      - Uplink interruption and downlink interruption (RRM) requirements, if needed

    - Minimize the impacts on RAN1

      - Update RAN1 uplink switching for carrier aggregation and supplementary uplink

    - Minimize the impacts on RAN2

      - Update the RRC signaling to indicate the switching period location and length

      - Update the UE capabilities

  - Specify UE requirements to enable Tx switching between cases, where 1 carrier on band A and 2 contiguous aggregated carriers on band B, and band A is for SUL or non-SUL and band B is a non-SUL band

    - The scenarios include

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |

and

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - Specify the following RAN4 requirements for above scenarios

      - Length of switching period

      - Time mask RF requirements

      - Uplink interruption and downlink interruption (RRM) requirements, if needed

    - Minimize the impacts on RAN1

      - Update RAN1 uplink switching for carrier aggregation and supplementary uplink

    - Minimize the impacts on RAN2

      - Update the RRC signaling to indicate the switching period location and length

      - Update the UE capabilities

Note 1:  Only addressing the case of co-located and synchronized network deployment for the two UL carriers.

Note 2:  Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA.

Note 3:  The UE is configured with two different uplink carrier frequencies.

An LS was sent by RAN4 [3]. This contribution is a summary of the following email discussion:

[104b-e-NR-R17-TxSwitching-01] Email discussion/approval for RAN1 aspects for RF requirements for NR frequency range 1 (FR1) – Jianchi (China Telecom)

- 1st check point: 4/15

- 2nd check point: 4/19

- Last check point: 4/20

# Email discussion (1st round)

## 2Tx-2Tx switching between two uplink carriers

### Mapping between UL transmission ports and Tx chain

For Rel-16 1Tx-2Tx switching, the mapping between UL transmission ports and Tx chain for SUL is defined in the following table.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |

For Rel-16 1Tx-2Tx switching, two options are supported for UL CA. UE can report via capability signaling which option (between Option 1 and Option 2) is supported.

- Option 1: UE cannot be scheduled or configured with UL transmission on both carriers simultaneously.

- Option 2: UE can be scheduled or configured with UL transmission on both carriers simultaneously.

For UL CA Option 1, the mapping between UL transmission ports and Tx chain is the same as SUL. For UL CA Option 2, the mapping between UL transmission ports and Tx chain for UL CA is defined in the following table.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |

Many companies (R1-2102484, R1-2102583, R1-2102860, R1-2102876, R1-2102931, R1-2103138, R1-2103387) proposed the mapping between UL transmission ports and Tx chain for Rel-16 1Tx-2Tx switching can be extended to support Rel-17 2Tx-2Tx switching.

Proposal 1:

- For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |
| Case 3 | 2T+0T | 2P+0P, 1P+0P |

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal |
| Qualcomm | We are supportive with FL proposal |
| vivo | Support |
| ZTE | We are okay with proposal 1 only if both proposal 1 and proposal 2 can be agreed at the same time. |
| OPPO | We are fine with proposal |
| Huawei, Hisilicon | fine with the proposal |

Proposal 2:

- For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |
| Case 3 | 2T+0T | 2P+0P, 1P+0P |

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal |
| Qualcomm | We are supportive with FL proposal |
| vivo | Support |
| ZTE | We support the above table because the above table can reduce the unnecessary switching to the largest extend. |
| OPPO | We are fine with proposal |
| Huawei, Hisilicon | we are fine with the proposal |

### Switching mechanism

For Rel-16 1Tx-2Tx switching between two uplink carriers, the following agreements have been achieved for SUL.

Agreements:

- For standalone SUL, if UL switching period is configured by RRC

- The switching period is not always applicable on the carrier configured with switching period.

- The switching period is only applicable when the UL transmissions are switched between 1Tx carrier 1 and 2Tx carrier 2.

For Rel-16 1Tx-2Tx switching between two uplink carriers, the following agreements have been achieved for UL CA.

Agreements:

- For inter-band UL CA, if uplink Tx switching is configured:

  - For option 1 of mapping between UL transmission ports and Tx chain, the switching period is only applicable when the UL transmissions are switched between 1Tx carrier 1 and 2Tx carrier 2.

    - Note: 2Tx carrier 2 refers to an UL carrier capable of 2 Tx chains and both 1-port and 2-port UL transmissions.

  - For option 2 of mapping between UL transmission ports and Tx chain

    - The switching period is only applicable in the following cases:

      - If the current state of Tx chains is 1 Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on carrier 2.

      - If the current state of Tx chains is 0 Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port transmission on carrier 1.

    - For other cases, the state of Tx chains of last UL transmission is assumed.

    - Note: No spec change to power configuration and power control.

R1-2102365, R1-2102583, R1-2102860, R1-2102931 proposed the basic principle can be extended to Rel-17 2Tx-2Tx switching.

Proposal 3:

- For standalone SUL, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured,

- The switching period is not always applicable on the carrier configured with switching period.

- The switching period is only applicable when the UL transmissions are switched between carrier 1 and carrier 2.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal. |
| Qualcomm | We are in principle ok with the proposal although the two sub-bullets seem to be redundant. Would suggest deleting the sub-bullet: “The switching period is not always applicable on the carrier configured with switching period.” |
| vivo | Generally fine and we agree with QC that the 1st sub-bullet can be removed. |
| ZTE | From our perspective, proposal 3 and proposal 4 can be combined into one proposal as anyway their description is the same, e.g., Combined proposal 3: - For standalone SUL and UL CA Option1, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured, - The switching period is not always applicable on the carrier configured with switching period. - The switching period is only applicable when the UL transmissions are switched between carrier 1 and carrier 2 regardless of whether the UE has the capability of simultaneousTxSUL-NonSUL or not. Again this combined proposal should be agreed together with proposal 5 related to Option 2. |
| OPPO | We are fine with proposal |
| Huawei, Hisilicon | In our understanding, the key is to reuse the existing mechanism of R16 UL Tx switching for R17 SUL. We suggest not to go through every R16 intermediate agreements, which costs our time to polish new wording but it is still unclear whether the existing mechanism with respect to switching period can be reused. Therefore, we propose, Proposal: For a UE configured with higher layer parameter supplementaryUplink and with 2Tx-2Tx UL Tx switching, the triggering mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 is reused. |

Proposal 4:

- For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured:

- For option 1 of mapping between UL transmission ports and Tx chain,

  - The switching period is not always applicable on the carrier configured with switching period.

  - The switching period is only applicable when the UL transmissions are switched between carrier 1 and carrier 2.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal. |
| Qualcomm | We are in principle ok with the proposal although the two sub-bullets seem to be redundant. Would suggest deleting the sub-bullet: “The switching period is not always applicable on the carrier configured with switching period.” |
| vivo | Generally fine and we agree with QC that the 1st sub-bullet can be removed. |
| ZTE | As we clarified in the above comments, we would prefer to combine proposal 3 and proposal 4 into one proposal. |
| OPPO | We are fine with proposal |
| Huawei, Hisilicon | Similar comments as above, we propose, Proposal: For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching, the triggering mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused. |

Proposal 5:

- For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured:

- For option 2 of mapping between UL transmission ports and Tx chain

  - The switching period is only applicable in the following cases:

    - If the current state of Tx chains is 1Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2.

    - If the current state of Tx chains is 0Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 1.

    - If the current state of Tx chains is 2Tx on carrier 1 and 0Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 2.

  - For other cases, the state of Tx chains of last UL transmission is assumed.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal. |
| Qualcomm | We are in principle ok with the proposal. |
| vivo | Support |
| ZTE | The above proposal seems fine to us. |
| OPPO | We are fine with proposal |
| Huawei, Hisilicon | We prefer to reuse the R16 mechanism for UL CA Option 2. Compared to previous agreement, a note is missing here, please add it back, which is “ - Note: No spec change to power configuration and power control. ” |

The state of Tx chains after Tx switching may not be unique in some cases. For instance, if the current state of Tx chains is 0T+2T and the next UL transmission is 1-port transmission on carrier 1, since 1P+0P can be mapped to either 1T+1T or 2T+0T, then what’s the state of Tx chains after Tx switching? Another example, if the current state of Tx chains is 2T+0T and the next UL transmission is 1-port transmission on carrier 2, since 0P+1P can be mapped to either 1T+1T or 0T+2T, then what’s the state of Tx chains after Tx switching? This issue can be solved by one of the following methods.

- Alt 1: The state of Tx chains after Tx switching is predefined in the specifications.

- Alt 2: The state of Tx chains after Tx switching is indicated by Network.

- Alt 3: The state of Tx chains after Tx switching is determined by UE.

Proposal 6:

- Down select on the following alternatives to address the issue that the state of Tx chains after Tx switching may not be unique.

- Alt 1: The state of Tx chains after Tx switching is predefined in the specifications.

- Alt 2: The state of Tx chains after Tx switching is indicated by Network.

- Alt 3: The state of Tx chains after Tx switching is determined by UE.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We prefer Alt.1 for the case that the state of Tx chains after Tx switching may not be unique so as to ensure the same understanding on the state of Tx chains after Tx switching. |
| Qualcomm | Thanks for China Telecom to bring up this issue. In R16, we define the state switching in the spec without explicit switching signalling. The RAN1 consensus is to implicitly inform UE to switch the UL Tx by network scheduling and configuration. Before we make the selection of above alternatives, we want to better understand whether we could still use the same approach – “implicit switching” in R17. If the implicit switching is still workable, it would not introduce new signalling. The current signalling architecture could be maintained, and the standard efforts would be largely reduced. |
| vivo | We do not prefer alt 2 due to similar reason provided by QC. We are fine to further study between Alt1 and Alt3 |
| ZTE | We understand Alt 1 and Alt 2 here. However, we are not sure how Alt 3 can work because if the state of Tx chains are determined by UE, then network and UE may not be aligned with the next case. We can simply adopt Alt1 by defining the following fixed rule in the spec. If UE is under Case3 (i.e. 2T+0T) and UE is to transmit 0P+1P, UE switches to Case2 (i.e. 0T+2T). If UE is under Case2 (i.e. 0T+2T) and UE is to transmit 1P+0P, UE switches to Case3 (i.e. 2T+0T). |
| OPPO | QC raised a good point. More discussion is needed |

## Uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

### Clarification on RF chain allocation for band B with two intra-band contiguous carriers

R1-2102876 proposed to clarify the PA bandwidth and RF chain allocation for band B with two intra-band contiguous carriers.

- Whether UL switching period and transient period are still needed for the switching between 2 intra-band continuous carriers in band B. Considering UE PA bandwidth is most likely limited to 100M in FR1, it seems UL switching period and transient period are still needed for the switching between 2 intra-band continuous carriers. Further clarification from RAN4 may be needed.

- Whether simultaneous transmission between 2 intra-band carriers need to be considered. Considering the coverage capability in carrier 2 and carrier 3 are similar, when 1 of the 2 carriers is picked for transmission, it can be assumed that 2 Tx chains should be all switched to it. That is to say, there is no 1P + 1P in band B.

Companies are encouraged to provide views on the above issues.

| Company | Views |
| --- | --- |
| CATT | For 1), we need send LS to RAN4 for clarification on whether UL switching period and transient period are needed for the switching between 2 intra-band continuous carriers or not. For 2), in our understanding, in order to support simultaneous transmission between 2 intra-band carriers, 1P + 1P in band B should be considered. |
| Qualcomm | Thank CMCC to bring up this discussion. We believe that the “UE PA bandwidth” is the same as the band itself, if so, switching is not needed because of the PA. For 1) we don’t think there is a need for switching. An intra-band CA capable UE is expected to support simultaneous transmission. For 2) we don’t quite understand the motivation to make the limitation to exclude 1P + 1P. Maybe the proponent can explain more. |
| vivo | We think the proposed issues as above are more suitable for RAN4 discussion. Some clarification from RAN4 would be useful. |
| ZTE | It is fine to clarify this issue. Regarding 1), if it is the common understanding that “UE PA bandwidth” is the same as the band itself, then switching is not needed for this intra-band continuous CA. Regarding 2), the 1P+1P can already be supported in the legacy intra-band continuous CA operation without UL Tx switching, we didn’t see the necessity to preclude it for UL Tx switching case. |
| OPPO | Regarding 1), some clarification/conclusion from RAN4 is needed |
| Huawei, Hisilicon | RAN4 has made agreement in R4-2103235 about this, which is “Clarify that the case with 1T on CC2@Band B and 1T on CC3@Band B is not considered in the WI discussion.” Therefore, 2Tx are available simultaneously on both uplinks at band B for a UE. In light of the RAN4 agreement, our understanding is 1) For the first bullet, there is no switching between two uplinks at band B, thus no switching period. 2) For the second bullet, concurrent uplink transmissions for those two uplinks on band B is supported, regardless 1-port or 2-port transmissions. |

### Clarification on Tx switching between case 1 and case 3 for UL CA option 1 and SUL

R1-2103138 proposed to clarify whether Tx switching between case 1 and case 3 for UL CA option 1 and SUL is included in WID

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 3 | 2T+0T |

Companies are encouraged to provide views on the above issue.

| Company | Views |
| --- | --- |
| CATT | In our understanding, if the current state of Tx chains is 2TX in Band B and the next UL transmission has a 1-port transmission on Band A, there are two solutions for Tx switching. Solution1 is that UE can execute Tx switching from 0T+2T to 1T+1T. Solution2 is that UE can execute Tx switching from 0T+2T to 2T+0T. If solution 1 is adopted, UE needs still another switching period to execute Tx switching from case1 to case 3 when the subsequent UL transmission has a 2-port transmission on carrier 1. If solution 2 is selected, UE does not need to execute Tx switching and UL transmission is not interrupted when the subsequent UL transmission has a 2-port transmission on carrier 1. So we need clarify the motivation of Tx switching between case 1 and case 3 for UL CA option 1 and SUL. |
| Qualcomm | Our understanding is intra band CA of one band is the natural extension of single CC for each band. In this sense, switching between case 1 and case 3 for UL CA option 1 and SUL should be in the scope. Some companies’ view might be that switching of case 1 and case 2 (0T+2T) already cover this case, but our understanding that they are different. The difference is Case 2 (0T+2T) is 0P + yP + zP where y, z = (0, 1, 2) and Case 3 is xP + 0P + 0P where x = (0, 1, 2). Before we get necessary consensus, it will be good to assume these two cases might be with different standard impact. |
| Vivo | Such switching is not captured in the WID. Prefer to handle it in RAN4 first if there is a desire to add the switching between case 1 and case 3. |
| ZTE | If we understand this issue correctly, R1-2103138 proposes to clarify whether switching between “0T+1T” (case1) and “2T+0T” (case3) for UL CA Option1 and SUL. We understand that it may be questionable whether it is needed to support the cases where currently no such band combination exists. We are open to discuss the necessity of this. |
| OPPO | It is not in the WID. |
| Huawei, Hisilicon | The question is unclear. Please clarify what is the definition of Case 1 here. Please note that there is no concurrent transmission for SUL. If it means no UL transmission on band A but only transmission on band B, then it means the UE only apply 1 Tx chain for the transmission on band B while leaving the other Tx chain idle. In this case, only 1-port transmission occurs on band B but no 2-port transmission. Obviously, such 1-port transmission only on band B should be in the scope. |

### Mapping between UL transmission ports and Tx chain

This issue depends on the outcome of section 2.1.1, section 2.2.1 and section 2.2.2.

### Switching mechanism

This issue depends on the outcome of section 2.1.2, section 2.2.1 and section 2.2.2.

## Preparation time

R1-2103387 mentioned the increased number of Tx involved in switching may require different UE implementation from the switching in Rel-16, which may require different switching time by the same UE, it is expected to allow a UE to report different switching time for 2Tx-2Tx switching from that for 1Tx-2Tx switching in the same band pair.

Proposal:

- For UL Tx switching in a band pair of a band combination, the switching time reported by a UE for 2Tx-2Tx switching can be different from that reported by the UE for 1Tx-2Tx switching.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We need clarify why 2Tx-2Tx switching time can be different from that for 1Tx-2Tx switching. |
| Qualcomm | Thanks to bring up this discussion. We think the above statement “increased number of Tx involved in switching may require different UE implementation from the switching in Rel-16, which may require different switching time by the same UE” is not a RAN1 issue. In RAN4 LS (R4-2103234) below, we could not find or derive the observation. If the proponent does see the issue, we would suggest discussing and reach consensus in RAN4 on the highlighted motivation before we make further discussion in RAN1. |
| Vivo | We are open to this proposal, but it seems more suitable for RAN4 to discuss and conclude. |
| ZTE | It seems that RAN4 has concluded that the Rel-16 values of switching time can be reused for 2Tx-2Tx switching. Not sure whether it is RAN4’s common understanding that the same value should be reported for both 1Tx-2Tx and 2Tx-2Tx. Besides, the Rel-16 values of switching time are {35us, 140 us, or 210us}, if UE indicates 35us for 1Tx-2Tx switching and UE can finish 2Tx-2Tx switching let’s say 70us for 2Tx-2Tx, then UE has to indicate 140us for 2Tx-2Tx because RAN4 has concluded that the same candidate values for Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx. This may introduce unnecessary additional switching delay. It is more reasonable to consult with RAN4 on this issue. |
| OPPO | It seems RAN4 should be involved on this issue |
| Huawei, Hisilicon | @CATT, the number of Tx hardware resources in a UE is limited. More Tx involved for a switching, the more Tx hardware resources is required, resulting in different implementation. In our understanding, a UE reporting Y us for 2Tx-2Tx may report X us for 1Tx-2Tx where X<Y. If the UE is not allowed to report a different value for 1Tx-2Tx, then the higher value Y has to be assumed by the gNB when the UE is only configured with 1Tx-2Tx switching, resulting in unnecessary longer UL interruption. @ZTE, The proposal brings no change to the candidate set of switching time {35us, 140us, 210us}, i.e. no new value like 70us, but allow a UE to report different value from the set for different Tx combinations. It seems that we have different understanding on the received RAN4 LS. If companies request for a RAN4 confirm first, then we propose to reply the received RAN4 LS and ask RAN4 the following question, “Question: For UL Tx switching in a band pair of a band combination, whether or not the switching time reported by a UE for 2Tx-2Tx switching can be different from that reported by the UE for 1Tx-2Tx switching.” |

R1-2103387 proposed to differentiate 1Tx-2Tx from 2Tx-2Tx UL Tx switching.

Proposal:

- For a UE configured with UL Tx switching via uplinkTxSwitching, the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources is used to determine the operation mode, i.e. either 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

- 2Tx-2Tx switching mode: when the maximum number is 2 for both uplinks configured with uplinkTxSwitching

- 1Tx-2Tx switching mode: when the maximum number is 1 for any one uplink configured with uplinkTxSwitching

- the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode

  - If the switching time capability value for 1Tx-2Tx switching mode is not reported by the UE, the value reported for 2Tx-2Tx switching mode is applied.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We need clarify why 2Tx-2Tx switching time can be different from that for 1Tx-2Tx switching. |
| Qualcomm | Similar as above comments, we think differentiate “1Tx-2Tx switching mode or 2Tx-2Tx switching mode” would be not a RAN1 issue if the difference is on the switching time by the same UE. We would suggest discussing and reach consensus in RAN4 on this before we make further discussion in RAN1. |
| ZTE | As we discussed in previous proposal, it is better to consult with RAN4 first before we rush into any quick decision on this. |
| Huawei, HiSilicon | The motivation is elaborated in our tdoc R1-2103387. If companies requests for RAN4 confirm first, then we propose to add the following question to the reply LS to RAN4, “Question: If the switching time capability value for 1Tx-2Tx switching is not reported by a UE, but only the value for 2Tx-2Tx switching is reported, then when a gNB configures 2-port transmission on only one of two uplinks (instead of on both of them) for the UE, i.e. a configuration of UL Tx switching requiring only 1Tx-2Tx switching, whether or not the switching time value reported for 2Tx-2Tx switching is applied.” Since Rel-15, for a carrier that a UE is capable of 2Tx transmission, a gNB is allowed to configure no 2-port transmission on the carrier for the UE. This principle gives the gNB freedom to work out the best service strategy for the UE. The same principle is applied to UL Tx switching also. Therefore, we would like to raise the above question here. |

## 1-port transmission via DCI format 0_1 for UL CA option 2

This issue was intensively discussed in Rel-16. Many compromised proposals were discussed by unfortunately no consensus was reached.

The latest two alternatives in RAN1#102e are copied below for reference.

Alt 1: supported by ZTE, Qualcomm

- For UL CA option 2, DCI format 0_1 can be used to schedule a UL transmission on carrier 2 when nrofSRS-Ports is configured as 2 antenna ports and state of Tx chains is 1 Tx on carrier 1 and 1Tx on carrier 2.

  - It’s up to implementation how DCI format 0_1 to be used.

Alt 2: supported by Huawei

- For UL CA option 2

  - Rel-16 uplink full power transmission can be used for codebook based transmission with 2 SRS resources (with one 1-port SRS resource and one 2-port SRS resource) on carrier 2

  - Note: No new uplink full power modes for UL CA Option2

  - Note: If Rel-16 uplink full power mode is not supported by the UE capable of UL CA option 2and configured with one 2-port SRS resource for codebook based operation, 1-port PUSCH is scheduled only by DCI 0_0

  - Note: Rel-16 uplink full power mode is not required as a prerequisite feature for a UE capable of UL-CA Option2.

R1-2103138 raised this issue to accommodate R17 scope and had the following proposal:

Proposal:

- Use the following rule to decide the Tx number(s) on a certain carrier,

- 2 Tx is used for these UL transmissions: PUSCH with TPMI=[ , ], 2-port SRS, 2-port configured grant PUSCH, SRS carrier switching on the paired carrier

- 1 Tx is used for these UL transmissions: PUCCH, SR, PRACH, PUSCH scheduled by DCI 0_0, single port configured grant PUSCH, PUSCH with TPMI=

Companies are encouraged to answer whether it is necessary to continue the discussion in Rel-17. If yes, please provide the views on the above proposal.

| Company | Yes/No |  |
| --- | --- | --- |
| CATT | No |  |
| Qualcomm | Yes | We had lengthy discussion in R16 on this issue but unfortunately, we didn’t get consensus. However, this is indeed an important issue to the product development and deployment. Clear definition of 1 or 2 ports would largely reduce the development and IoT test efforts between infrastructure vendors and chipset/UE vendors. Our proposal in R1-2103138 can be categorized as two parts - Part 1: Using DCI format 0_1 to differentiate antenna ports - 2 Tx is used for these UL transmissions: PUSCH with TPMI=[ , ] - PUSCH with TPMI= - Part 2: Categorize the antenna ports for other cases - 2 Tx is used for these UL transmissions: 2-port SRS, 2-port configured grant PUSCH, SRS carrier switching on the paired carrier - 1 Tx is used for these UL transmissions: PUCCH, SR, PRACH, PUSCH scheduled by DCI 0_0, single port configured grant PUSCH, As far as we recalled, Part 2 is the consensus of most companies and there was no disagreement on this. We would suggest approving Part 2 as R17 agreement. For Part 1, there were two FL proposals in RAN1 – 102e. We are fine with both proposals while FL proposal #1 is with more details and #2 leaves the details to implementation. Among them, we slightly prefer #1. FL proposal #1: A compromised option is proposed in R1-2006333. Option 3: Support 1-port UL transmission indicated by DCI Format 0_1 for UL CA with Tx switching. - If only one 2-port SRS resource is configured for codebook-based transmission on carrier 2, - PUSCH transmission with TPMI= can be supported in Case 1 i.e. when the Tx chain state is 1Tx on carrier 2. - PUSCH transmission with all other TPMIs can only be supported in Case 2 i.e. when the Tx chain state is 2Tx on carrier 2. - If 2 SRS resources (with one 1-port resource and one 2-port resource) are configured for codebook-based transmission on carrier 2, the 1-port SRS resource is used for Case 1 and the 2-port SRS resource is used for Case 2. - Support using the same UE capability 13ignalling as that for UL full Tx power. FL proposal #2 during email discussion - For UL CA option 2, DCI format 0_1 can be used to schedule a UL transmission on carrier 2 when nrofSRS-Ports is configured as 2 antenna ports and state of Tx chains is 1 Tx on carrier 1 and 1Tx on carrier 2. - It’s up to implementation how DCI format 0_1 to be used. |
| ZTE | Yes | We are supportive to continue the discussion in Rel-17.Because strictly speaking, in current Rel-16 spec, the definition of 1-port/2-port transmission is not clearly defined. However, it seems difficult to combine the TPMI issue and SRS carrier switching issue into one proposal. If possible, we would prefer to separate the discussion. Especially for the SRS carrier switching issues, it would be better to clarify it after Rel-16 ongoing discussion for the similar issue. |
| OPPO | No | No need to repeat the same discussion of Rel-16 |
| Huawei, Hisilicon | No | It has significant RAN1 specification impacts, and it is clearly out of scope of Rel-17 WID. |

# Email discussion (2nd round)

FL comments: Proposal 1 and proposal 2 are stable, please refrain from any further comments.

Proposal 1:

- For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |
| Case 3 | 2T+0T | 2P+0P, 1P+0P |

Proposal 2:

- For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |
| Case 3 | 2T+0T | 2P+0P, 1P+0P |

FL comments: Based on the comments, proposal 3/4/5 are combined as the revised proposal 3.

Revised proposal 3:

- For a UE configured with higher layer parameter supplementaryUplink and with 2Tx-2Tx UL Tx switching, the triggering mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 is reused.

- For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching, the triggering mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused.

- For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured:

- For option 2 of mapping between UL transmission ports and Tx chain

  - The switching period is only applicable in the following cases:

    - If the current state of Tx chains is 1Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2.

    - If the current state of Tx chains is 0Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 1.

    - If the current state of Tx chains is 2Tx on carrier 1 and 0Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 2.

  - For other cases, the state of Tx chains of last UL transmission is assumed.

  - Note: No spec change to power configuration and power control.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| ZTE | We are fine with the direction to combine proposal 3/4/5 together. Regarding the first two bullets, we understand the intention of them. However, it seems not appropriate to say “the triggering mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 is reused”. Because in the current specification, we didn’t cover the case of 0P+2P -> 2P+0P switching. It may cause potential ambiguity with the current wording. We would suggest to reuse the wording in previous proposal. Also, the first two bullets can be combined as they are describing the same thing. Regarding the last note for power configuration and power control, we think it can be removed for now. Currently, RAN4 is discussing PC2 support, which may or may not have impact on the power configuration/power control for the issue here. Besides, we didn’t see any contribution discussing the power related issue in this RAN1 meeting, it is more prudent to remove the note for now. Furthermore, the proposal here is discussing the switching mechanism, which is not related the power issue. The power issue can anyway be a separate discussion. Overall, the proposal can be revised as following. Revised proposal 3: - For a UE configured with higher layer parameter supplementaryUplink or configured with UL CA Option1 and if the UE is configured with 2Tx-2Tx UL Tx switching, the switching period is only applicable when the UL transmissions are switched between carrier 1 and carrier 2. - For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured: - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2. - If the current state of Tx chains is 0Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 1. - If the current state of Tx chains is 2Tx on carrier 1 and 0Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 2. - For other cases, the state of Tx chains of last UL transmission is assumed. |
| CATT | We are fine with revised proposal 3 from ZTE |
| Qualcomm | We agree with ZTE’s comments and proposals. In particular, we also think that the note on power control needs to be deleted. Our understanding is that RAN4 is already discussing power control changes for UL Tx switching. |
| Huawei, HiSilicon | Thank for the proposals. With the guidance “minimize RAN1 spec impact” in WID, we would like to suggest to focus on the additional spec impact required by 2Tx-2Tx UL Tx switching and reuse the existing specified mechanism as much as possible. Regarding Proposal 2, two bullets are missing compared to previous agreement, we don’t a reason to remove them yet. Without these two bullets, we would like to know more about the potential spec impact by the proposal. Proposal 2: - For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows. - No new RAN4 impact - No new TDM pattern Number of Tx chains in WID (carrier 1 + carrier 2); Number of antenna ports for UL transmission (carrier 1 + carrier 2). Case 1; 1T+1T; 1P+0P, 1P+1P, 0P+1P. Case 2; 0T+2T; 0P+2P, 0P+1P. Case 3; 2T+0T; 2P+0P, 1P+0P. Regarding Proposal 3, - The current spec for UL Tx switching is much more than whether the switching period only applicable or not, e.g. the exact condition of determination for UL Tx switching. With ZTE proposal, it implies reopening the discussions for all of them except for “only applicable”, which is very time-consuming and thus not necessary. - The triggering mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 has covered the case of “0P+2P”=>”2P+0P” switching. We don’t see additional spec impact for this case. - For S6.1.6.2 of TS 38.214, the additional spec impact for UL CA option 1 can be added for “0+2P” => “2P+0P” Proposal 3-rev: For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching, the triggering mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused with the following add-on “<--Text start--> - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of on any of the two carriers. - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of on any of the two carriers. <--Text end-->“ - Regarding the proposal for UL CA Option 2, we cannot agree the proposal without the last bullet, because it was in the previous agreement and RAN1 spec impact should be minimized as WID requests. Additionally, because the RAN1 spec for UL CA power control is generic to any power class, any introduction of Power Class 2 in RAN4 obviously don’t need any RAN1 spec impact for sure. Could proponents to clarify the reason to revert the previous agreement? We don’t see any RAN4 on-going discussion on power control for this WID. If any, suggest Qualcomm to elaborate it, e.g. the RAN4 AI and tdoc number. Therefore, we support FL proposal 3 or the FL proposal 3 with the above amendment. We are not OK with ZTE’s proposals. |

FL comments: Regarding RF chain allocation for band B with two intra-band contiguous carriers, based on R4-2103235, following conclusion is proposed.

Conclusion:

- For uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B,

  - If the state of Tx chains is 1Tx on Band A and 1Tx on Band B, 1Tx is available simultaneously on both uplink carriers on band B for a UE.

  - If the state of Tx chains is 0Tx on Band A and 2Tx on Band B, 2Tx are available simultaneously on both uplink carriers on band B for a UE.

Companies are encouraged to provide views on the above conclusion.

| Company | Views |
| --- | --- |
| ZTE | We are fine with the proposal. One minor wording change, we suggest to change “available” to “allowed”. |
| CATT | We are fine with FL proposal. |
| Qualcomm | We are fine with the proposal in principle. Ok with either “available” or “allowed”. Our understanding is that the intent with both is that when simultaneous transmission is granted, the UE has the capability to perform simultaneous transmission on both uplink carriers in Band B. |
| Huawei, HiSilicon | Fine with FL proposal. The “available” seems better than “allowed”, because it is about the availability of Tx chain in a UE rather than a restriction of Tx chain. |

FL comments:  Regarding clarification on Tx switching between case 1 and case 3 for UL CA option 1 and SUL, it seems this issue should be clarified in RAN plenary or RAN4.

Conclusion:

- Whether Tx switching between case 1 and case 3 for UL CA option 1 and SUL is included in WID is to be clarified by RAN plenary or RAN4.

Companies are encouraged to provide views on the above conclusion.

| Company | Views |
| --- | --- |
| ZTE | We are ok with the proposed conclusion. |
| CATT | We are fine with FL proposal |
| Qualcomm | We are ok to clarify this in RAN Plenary or RAN4. |
| Huawei, HiSilicon | Not OK with the conclusion. Because it is unclear what should be clarified and what the exact meaning of case 1 here as commented before. This conclusion would be confusing for RAN4 and RAN plenary. |

FL comments: Regarding the preparation time, it seems it’s necessary to consult with RAN4 before further discussion in RAN1.

Proposal 7:

- Send LS to RAN4 asking following questions:

  - Question 1: For UL Tx switching in a band pair of a band combination, whether or not the switching time reported by a UE for 2Tx-2Tx switching can be different from that reported by the UE for 1Tx-2Tx switching.

  - Question 2: If the switching time capability value for 1Tx-2Tx switching is not reported by a UE, but only the value for 2Tx-2Tx switching is reported, then when a gNB configures 2-port transmission on only one of two uplinks (instead of on both of them) for the UE, i.e. a configuration of UL Tx switching requiring only 1Tx-2Tx switching, whether or not the switching time value reported for 2Tx-2Tx switching is applied.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| ZTE | We are ok to send Question 1 to RAN4. But we didn’t see the need of the second question. The second question is purely an issues on how to interpret/report UE capability for UL Tx switching, it is more appropriate to discuss it in RAN1/RAN2 once the all other details are finalized. |
| CATT | For Q1, we are fine with it to RAN4. For Q2, in our understanding, UE will execute 2Tx-2Tx switching for the case 1Tx-2Tx switching is required because UE only report 2Tx-2Tx switching capability to gNB. |
| Qualcomm | We are ok with Question 1. On Question 2, we share similar view as ZTE, the current proposed wording seems not unneeded before RAN4 can give clear answer to Question 1. |
| Huawei, HiSilicon | The question 2 is related to not only RAN1/2 but also RAN4, because it is about the quantity of switching time, which is a RAN4 implementation issue. For example, if RAN4 decides the switching time value for 1Tx-2Tx implementation is larger than that for 2Tx-2Tx, then RAN1 cannot assume the value of 2Tx-2Tx is applicable for 1Tx-2Tx. In our understanding, a confirmation from RAN4 is better since an LS to RAN4 will be sent anyway. @Qualcomm, a cycle of LS request and reply costs at least three meetings, therefore, we prefer to ask RAN4 all relevant questions at one time. |

FL comments: For 1-port transmission via DCI format 0_1 for UL CA option 2, 2 companies (Qualcomm, ZTE) think it is necessary to continue the discussion in Rel-17, while 4 companies (CATT, OPPO, Huawei, Hisilicon) don’t think so.

Conclusion:

- 1-port transmission via DCI format 0_1 for UL CA option 2 is not considered for Rel-17 Tx switching.

Companies are encouraged to provide views on the above conclusion.

| Company | Views |
| --- | --- |
| ZTE | We are NOT ok with this proposal. In Rel-16, for UL CA Option2, in case of 0T+2T, if the previous agreements is not agreed, the only way to schedule 1-port PUSCH on CC2 to is use fallback DCI, which is too restrictive for network. In Rel-17, the issue has greater impact since both carriers can do 2T. We hope companies can consider this and agree to address this issue in Rel-17. |
| CATT | We are fine with this proposal. |
| Qualcomm | We disagree with this proposed conclusion. In R16, it’s unfortunately we didn’t get any conclusion on this important issue and how to categorize one or two ports are quite obscure for many specific channels & cases. Now R17 is with Case 3 where both CCs can be with 2 Tx. Without the categorization of 1 and 2Tx, we expect implementation of R17 will extremely difficult and complicated. We hope companies can agree to and try best to solve this issue in R17 time frame. |
| Huawei, HiSilicon | We don’t agree on ZTE and Qualcomm’s observations as discussed before. The proposal 5 in R1-2103138 is surely out of scope and have significant RAN1 spec impacts. It has been discussed for more than a year, we should conclude no more discussion. We hope companies could respect the WID and the real concerns, at least could accept the following conclusion. Proposed Conclusion: For UL CA Option2, new MIMO scheme, e.g. new UE behavior for PUSCH TPMI, is not considered. |

# References

- RP-202088, New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1) in Rel-17, Huawei, HiSilicon, China Telecom, RAN #89e, Sep. 2020.

- RP-210899, Revised WID: RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon, RAN #91e, Mar. 2021.

- R4-2103234, LS on Rel-17 Tx switching enhancements, RAN4 #98e, Feb. 2021.

- R1-2102365, Discussion on Rel-17 Tx switching enhancement, OPPO, RAN1#104b-e, Apr. 2021.

- R1-2102484, Discussion on Rel-17 UL Tx Switching, ZTE, RAN1#104b-e, Apr. 2021.

- R1-2102583, Discussion on R17 enhancements for UL TX switching, CATT, RAN1#104b-e, Apr. 2021.

- R1-2102860, Discussion on Rel-17 uplink Tx switching, China Telecom, RAN1#104b-e, Apr. 2021.

- R1-2102876, Discussion on Rel-17 UL Tx Switching, CMCC, RAN1#104b-e, Apr. 2021.

- R1-2102931, Discussion on Rel-17 Tx switching enhancements, vivo, RAN1#104b-e, Apr. 2021.

- R1-2103138, Discussion on R17 UL Tx switching, Qualcomm Incorporated, RAN1#104b-e, Apr. 2021.

- R1-2103387, Discussions on enhancements for UL Tx switching, Huawei, HiSilicon, RAN1#104b-e, Apr. 2021.