3GPP TSG RAN WG1 #104bis-e	R1-21xxxxx

e-Meeting, April 12th – 20th, 2021

Agenda item:		5.1

Source:	Moderator (China Telecom)

Title:	Summary of Rel-17 uplink Tx switching

Document for:		Discussion and Decision

# Introduction

In RAN #89 e-meeting, a new Rel-17 WID of “RF requirements enhancement for NR frequency range 1 (FR1)” [1] was approved and was revised in RAN #91 e-meeting [2], including following objectives.

- Specify UE requirements to enable Tx switching between different cases across carriers based on SUL and NR inter-band uplink CA for UE supporting maximum two concurrent transmissions

  - Specify UE requirements to enable Tx switching between cases

    - The scenarios include

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - Specify the following RAN4 requirements for above scenarios

      - Length of switching period

      - Time mask RF requirements

      - Uplink interruption and downlink interruption (RRM) requirements, if needed

    - Minimize the impacts on RAN1

      - Update RAN1 uplink switching for carrier aggregation and supplementary uplink

    - Minimize the impacts on RAN2

      - Update the RRC signaling to indicate the switching period location and length

      - Update the UE capabilities

  - Specify UE requirements to enable Tx switching between cases, where 1 carrier on band A and 2 contiguous aggregated carriers on band B, and band A is for SUL or non-SUL and band B is a non-SUL band

    - The scenarios include

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |

and

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - Specify the following RAN4 requirements for above scenarios

      - Length of switching period

      - Time mask RF requirements

      - Uplink interruption and downlink interruption (RRM) requirements, if needed

    - Minimize the impacts on RAN1

      - Update RAN1 uplink switching for carrier aggregation and supplementary uplink

    - Minimize the impacts on RAN2

      - Update the RRC signaling to indicate the switching period location and length

      - Update the UE capabilities

Note 1:  Only addressing the case of co-located and synchronized network deployment for the two UL carriers.

Note 2:  Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA.

Note 3:  The UE is configured with two different uplink carrier frequencies.

An LS was sent by RAN4 [3]. This contribution summarizes the RAN1 issues to support Rel-17 uplink Tx switching.

# Summary of RAN1 issues

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| - Issue#1: 2Tx-2Tx switching between two uplink carriers - Issue#1.1: Mapping between UL transmission ports and Tx chain - Issue#1.2: Switching mechanism | R1-2102365, R1-2102484, R1-2102583, R1-2102860, R1-2102876, R1-2102931, R1-2103138, R1-2103387 | Suggest to be discussed in RAN1#104bis-e |
| - Issue#2: Uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B - Issue#2.1: Clarification on RF chain allocation for band B with two intra-band contiguous carriers - Issue#2.2: Mapping between UL transmission ports and Tx chain - Issue#2.3: Switching mechanism - Issue#2.4: Clarification on Tx switching between case 1 and case 3 for UL CA option 1 and SUL | R1-2102365, R1-2102484, R1-2102583, R1-2102860, R1-2102876, R1-2102931, R1-2103138, R1-2103387 | Suggest to be discussed in RAN1#104bis-e |
| - Issue#3: Preparation time | R1-2103387 | Suggest to be discussed in RAN1#104bis-e |
| - Issue#4: 1-port transmission via DCI format 0_1 for UL CA option 2 | R1-2103138 | Suggest to be discussed in RAN1#104bis-e |

| Company | Views |
| --- | --- |

# References

- RP-202088, New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1) in Rel-17, Huawei, HiSilicon, China Telecom, RAN #89e, Sep. 2020.

- RP-210899, Revised WID: RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon, RAN #91e, Mar. 2021.

- R4-2103234, LS on Rel-17 Tx switching enhancements, RAN4 #98e, Feb. 2021.

- R1-2102365, Discussion on Rel-17 Tx switching enhancement, OPPO, RAN1#104b-e, Apr. 2021.

- R1-2102484, Discussion on Rel-17 UL Tx Switching, ZTE, RAN1#104b-e, Apr. 2021.

- R1-2102583, Discussion on R17 enhancements for UL TX switching, CATT, RAN1#104b-e, Apr. 2021.

- R1-2102860, Discussion on Rel-17 uplink Tx switching, China Telecom, RAN1#104b-e, Apr. 2021.

- R1-2102876, Discussion on Rel-17 UL Tx Switching, CMCC, RAN1#104b-e, Apr. 2021.

- R1-2102931, Discussion on Rel-17 Tx switching enhancements, vivo, RAN1#104b-e, Apr. 2021.

- R1-2103138, Discussion on R17 UL Tx switching, Qualcomm Incorporated, RAN1#104b-e, Apr. 2021.

- R1-2103387, Discussions on enhancements for UL Tx switching, Huawei, HiSilicon, RAN1#104b-e, Apr. 2021.