3GPP TSG RAN WG1 #104bis-e                                               R1-21xxxxx

e-Meeting, April 12th – 20th, 2021

Title:	[draft] Reply LS on half-duplex operation

Response to:	R1-2100023(R2-2010809)

Release:	Rel-16

Work Item:	TEI16

Source:	Huawei [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Yi WANG

E-mail Address:	<wangyi6@huawei.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	RAN1 CR in R1-21xxxxx

- Overall Description:

RAN1 would like to thank RAN2 on the LS on half-duplex operation in R1-2100023/R2-2010809.

RAN1 has discussed the LS and has the following understanding/agreements:

- RAN1 agrees to use the per-serving-cell configuration of directionalCollisionHandling as currently implemented by RRC, and the collision handling operation is applied to the set of cell(s) configured/enabled by directionalCollisionHandling within the cell group.

- RAN also agrees that the UE does not expect any directional collision among the serving cells that the UE is not capable of simultaneous transmission and reception after the UE applies the directional collision handling within the set of cell(s).

- In addition, RAN1 agrees that

Rel-16 collision handling is applicable to TDD intra-band CA

  - UE can report half-DuplexTDD-CA-SameSCS-r16 for a band combination that is intra-band only.

  - UE can report half-DuplexTDD-CA-SameSCS-r16 in case of mix of intra- and inter-band CA if simultaneousRxTxInterBandCA is not included.

A RAN1 CR for the above is approved and attached. It is up to RAN2 whether the above requires any specification changes.

2. Actions:

To RAN WG2: RAN1 respectfully asks RAN2 to take the above into account in their further work on half-duplex operation.

3. Dates of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #105-e	11 May – 27 May 2021	eMeeting

TSG-RAN WG1 Meeting #106-e	16 Aug. – 27 Aug. 2021	eMeeting