3GPP TSG RAN WG1 Meeting #105-e	R1-2104289

E-meeting, May 10th – May 27th, 2021

Agenda Item:	8.9.2

Source:	Huawei, HiSilicon

Title:	Support of 14-HARQ processes in DL for HD-FDD MTC UEs

Document for:	Discussion and Decision

Introduction

The WID for Rel-17 enhancements for LTE-MTC [1] includes an objective to support 14 HARQ for increasing the peak data rate for HD-FDD MTC UEs.

In RAN1 #104bis-e, the following agreement of HARQ-ACK delay solution was achieved:

Agreement In Rel-17, for the 14 HARQ process feature the HARQ-ACK delay solution will be down-selected in RAN1#105-e from: Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay). - FFS: The expression consisting of different subframe types. - FFS: Signaling Details. Alt-2: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”. - FFS: The percentage of presence of non-BL/CE DL subframes and non-BL/CE UL subframes to be handled. - FFS: HARQ-ACK delay values and length of the HARQ-ACK delay set. - FFS: Signaling Details. The following aspects will be considered towards the down-selection of one of the two alternatives (i.e., Alt-1 or Alt-2) for the HARQ-ACK delay solution: - Total number of bits required in DCI - Scenarios that can be handled, including: (a) different numbers of scheduled HARQ processes per burst (including dynamically switching between more than 10 HARQ processes and 10 or less HARQ processes) (b) different % of invalid subframes for both 10 and 40 SF long bitmaps - Robustness against loss of DCIs - Flexibility - RRC signaling overhead

In this contribution, we discuss the solution of HARQ-ACK delays and signaling methods to support 14 HARQ processes for HD-FDD MTC UEs.

Discussion

HARQ-ACK delay

HARQ-ACK delay values depend on the downlink and uplink bitmaps. An an example, we consider a DL/UL bitmap as shown in Figure 1 with 40% non BL/CE DL subframes and 20% non BL/CE UL subframes. For each 10 subframes, subframe #1, subframe #4, subframe#7 and subframe #9 are non BL/CE DL subframes. Subframe #0 and subframe #1 are non BL/CE UL subframes. Here, the non BL/CE subframe is colored as blue.

Figure 1. The example DL and UL bitmap

According to the agreed PDSCH scheduling delay of 7 and the rule of wasting the least resource for 14 HARQ processes, the timeline of PDSCH transmission with 14 HARQ processes corresponding to bitmap of Figure 1 is shown in Figure 2:

Figure 2. The timeline of PDSCH transmission for 14 HARQ processes

Denote the HARQ processes as D*. Assuming the consecutive HARQ-ACK bundling pattern, D10, D11, D0, D1 HARQ processes are fedback by A0 in subframe #22.

For Alt-1,the HARQ-ACK delay value y for D10, D11, D0 and D1 is 10-13, in which the HARQ-ACK delay subframe type can be expressed as (y-2) DL BL/CE subframes+1 subframe+1 UL BL/CE subframes. For Alt-2, the HARQ-ACK delay values (with absolute subframes) are {22, 20, 19, 17}.

D2, D3, D4 and D5 HARQ processes are fedback by A1 in subframe #23.

For Alt-1,the HARQ-ACK delay value y for D2, D3, D4 and D5 iare 7-13, in which the HARQ-ACK delay subframe type can be expressed as (y-3) DL BL/CE subframes+1 subframe+2 UL BL/CE subframes. For Alt-2, the HARQ-ACK delay values (with absolute subframes) are {17, 15, 13, 11}.

D6, D7, D8 and D9 HARQ processes are fedback by A2 in subframe #24.

For Alt-1,the HARQ-ACK delay value y for D6, D7, D8 and D9 is 4-7, in which the HARQ-ACK delay subframe type can be expressed as (y-4) DL BL/CE subframes+1 subframe+3 UL BL/CE subframes. For Alt-2, the HARQ-ACK delay values (with absolute subframes) are {11, 9, 8, 6}.

Therefore, for Alt-1, no matter what bitmap is configured for the UE, the HARQ-ACK delay values are fixed with the range from 4-13 and can be specified as the following three patterns with totally fixed 12 additional types required  to be indicated :

- (y-2) DL BL/CE subframes+1 subframe+1 UL BL/CE subframes. y ranges from 10-13.

- (y-3) DL BL/CE subframes+1 subframe+2 UL BL/CE subframes. y ranges from 7-10.

- (y-4) DL BL/CE subframes+1 subframe+3 UL BL/CE subframes. y ranges from 4-7.

Proposal 1: For Alt-1, the solution of HARQ-ACK delay values y is:

- (y-2) DL BL/CE subframes+1 subframe+1 UL BL/CE subframes. y ranges from 10-13.

- (y-3) DL BL/CE subframes+1 subframe+2 UL BL/CE subframes. y ranges from 7-10.

- (y-4) DL BL/CE subframes+1 subframe+3 UL BL/CE subframes. y ranges from 4-7.

Consequently, the fixed 4 bits is sufficient to indicate HARQ-ACK delay values in Alt-1 for 14-HARQ processes.

For Alt-2, the HARQ-ACK delay values will be different for different HARQ cycles. For the bitmap in Figure 1, the HARQ-ACK delay values set are {6, 8, 9, 11, 13, 15, 17, 19, 20, 22} with totally 10 HARQ-ACK values in the first HARQ-ACK cycle. However, for the next HARQ-ACK cycle, additional HARQ-ACK delay values occur, which is shown as the following Figure 3:

Figure 3. The second HARQ-ACK cycle of 14 HARQ processes following the first HARQ-ACK cycle in Figure 2

As can be observed, for the second HARQ-ACK cycle, the additional HARQ-ACK delay values set is {4, 7, 10, 21} with 4 additional HARQ-ACK delay values. The two HARQ-ACK process cycles totally contain 14 HARQ-ACK delay values. It can be expected that in the next several HARQ-ACK process cycles, there will be 19 delay values from 4 to 22 totally.

Therefore, 4 bits are not sufficient for the indication of HARQ-ACK delay values even if the non BL/CE UL subframes ratio does not exceed 20%. With different bitmap of reserved subframes, the range of delay values would be even complicated to be determined.

Observation 1: It required 4 bits for the indication of HARQ-ACK delay in Alt-1 and more than 4 bits for the indication of HARQ-ACK delay in Alt-2.

Furthermore, for Alt-1, only 1 bit is needed for RRC to enable 14-HARQ, as there is no need to configure the range of delay values. However, for Alt-2, different HARQ-ACK delay values need to be configured for different bitmaps. Considering 4 bits for each HARQ-ACK delay values, then totally 56 bits are required to configure the 14 HARQ-ACK delay values for the first and second HARQ-ACK cycle in Figure 2 and Figure 3. And even more bits are needed to configure all possible HARQ-ACK delays. Thus, the RRC signalling overhead of Alt2 is much more than Alt-1.

Observation 2: The RRC signalling overhead of Alt-2 is much more than Alt-1.

Considering the scheduling flexibility, RRC configuration and different bitmaps for reserved subframes, Alt-1 is preferred.

Proposal 2: Support Alt-1 for the solution of HARQ-ACK delay values.

Signaling of PDSCH scheduling delay and HARQ-ACK delay

In RAN1#104bis-e, the agreement has been reached that PUCCH repetition is not supported for the 14 HARQ processes. The agreement reached is copied below.

Agreement In Rel-17, for the 14 HARQ processes feature, PUCCH repetition is not supported with HARQ-ACK bundling.

Proposal 3: The 2 spare bits in repetition field for 14HARQ processes can be reused to indicate the PDSCH scheduling delay.

Therefore, the 2-bit of repetition field is considered to be reused to indicate the introduced PDSCH scheduling delay of 7 with two types. It can be observed that the HARQ-ACK delay also depends on the indicated PDSCH scheduling delay. For Alt-1 of HARQ-ACK delay, an example is given in Table 1 to illustrate the relationship between the HARQ-ACK delay value and PDSCH scheduling delay.

For the example in Figure 2, the HARQ process D0-D9 are with HARQ-ACK delay value 4-11 and PDSCH scheduling delay of 2. And the HARQ process D10-D11 are with HARQ-ACK delay value 12 and 13 and PDSCH scheduling delay of 7.

Then, joint encoding of repetition field and HARQ-ACK field in DCI can be employed to indicate the PDSCH scheduling delay and HARQ-ACK delay for 14-HARQ processes.

Table 1. Indication of PDSCH scheduling delay and HARQ-ACK delay

| Repetition field in DCI | HARQ-ACK delay field in DCI | PDSCH scheduling delay | HARQ-ACK delay |
| --- | --- | --- | --- |
| 00 | 000-111 | 2 | 4-11 |
| 01 | 000-011 | 2 | 4-7 (Type 3) |
|  | 100-111 | 2 | 7-10 (Type 2) |
| 10 | 000-010 | 2 | 10-11 (Type 1) |
|  | 011 | 7 (PDSCH Type 2) | 12 (Type 1) |
|  | 110 | 7 (PDSCH Type 1) | 13 (Type 1) |

Proposal 4: Joint encoding of repetition field and HARQ-ACK delay field can be used to indicate the PDSCH scheduling delay and HARQ-ACK delay.

Conclusions

In this contribution, we provide discussions on introducing 14 HARQ for HD-FDD MTC UEs, and the following observation and proposals are given:

Observation 1: It required 4 bits for the indication of HARQ-ACK delay in Alt-1 and more than 4 bits for the indication of HARQ-ACK delay in Alt-2.

Observation 2: The RRC signalling overhead of Alt-2 is much more than Alt-1.

P Proposal 1: For Alt-1, the solution of HARQ-ACK delay values y is:

- (y-2) DL BL/CE subframes+1 subframe+1 UL BL/CE subframes. y ranges from 10-13.

- (y-3) DL BL/CE subframes+1 subframe+2 UL BL/CE subframes. y ranges from 7-10.

- (y-4) DL BL/CE subframes+1 subframe+3 UL BL/CE subframes. y ranges from 4-7.

Proposal 2: Support Alt-1 for the solution of HARQ-ACK delay values.

Proposal 3: The 2 spare bits in repetition field for 14HARQ processes can be reused to indicate the PDSCH scheduling delay.

Proposal 4: Joint encoding of repetition field and HARQ-ACK delay field can be used to indicate the PDSCH scheduling delay and HARQ-ACK delay.

- References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, E-meeting, June 2020.