3GPP TSG RAN WG1 #105-e		                                    R1-2105584

e-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.2.6

Source:	InterDigital, Inc.

Title:	Discussion on channel access mechanisms

Document for:	Discussion and Decision

# Introduction

Channel access mechanisms for operation between 52.6GHz and 71GHz was discussed during the SI [1][2]. During RAN1 #104-e and RAN1 #104bis-e, multiple agreements were reached with respect to channel access. They are provided herein when relevant.

In this contribution we address open issues on channel access and discuss our preferences for channel access for operation up to 71GHz.

# Discussion

The purpose of channel access mechanisms is to ensure that there is fair co-existence between RATs and even within a RAT such as 5G NR. Fair co-existence implies not creating undue interference to other nodes actively receiving transmissions in the channel. However, there is a fine balance between fair co-existence and overly timid channel access that reduces the over-all system performance. 5G NR is designed to handle some level of interference, so the objective of a channel access mechanism need not be to completely avoid interference.

It was shown during the SI [2] that LBT provides benefits for some UEs in some scenarios at 52.6 GHz to 72 GHz. Different channel access mechanisms were studied during the SI and further discussion is required to determine which to specify for Rel-17. In Rel-16, omni-directional LBT was specified. However, the channel characteristics and interference considered in Rel-16 are much different than those experiences above 52GHz. We discuss two mechanisms to improve channel access efficiency. These mechanisms have been shown to have benefits over omni-directional LBT by reducing the effects of hidden and exposed nodes, two issues that are exacerbated when using omni-directional LBT at higher frequencies.

## Directional LBT

Omni-directional LBT suffers from both hidden nodes (where a transmitter doesn’t detect an interfering node that may suffer from the transmission or may negatively impact the receiving node) and exposed nodes (where a transmitter detects a node and determines channel access has failed, when in reality the node would have not suffered from the transmission, nor would it have interfered with the receiver). Omni-directional LBT suffers from hidden node issues given that only the transmitting node performs clear channel assessment prior to a transmission. It suffers from exposed node issues given that the transmitter may detect energy from nodes having no actual negative impact on the receiver and may thus deem the channel busy. The end result of hidden nodes is poor co-existence and reduced BLER performance. Whereas the end result of exposed nodes is increased latency and reduction in spatial reuse. Both of these lead to inefficient use of resources which increase channel occupancy and leads to over-all system inefficiency. Nevertheless, it was determined in R16 that for lower frequency ranges, omni-directional LBT was sufficient to ensure fair co-existence without a large specification effort.

For unlicensed spectrum from 52.6GHz to 71GHz, the hidden node and exposed node issues become more restrictive. Using omni-directional LBT at higher frequencies means that the ED threshold needs to be set in a manner that is overly permissive given that the interference actually affecting the receiver is only a fraction of the total energy detected (e.g. only in the direction of the beam used at the receiver). This further exacerbates the hidden node issue. Otherwise the ED threshold needs to be set in a manner that is overly prohibitive, given that there is no way to know what portion of the total energy detected is in the direction of the associated transmission. This can further exacerbate the exposed node issue.

Observation 1: Omni-directional LBT in unlicensed spectrum from 52.6GHz to 71GHz can under-represent interference in the direction of the associated transmission and over-represent interference in other directions.

At frequencies above 52.6GHz, propagation loss becomes prohibitive even over short distances. Therefore, directional narrowbeam-based transmission is necessary to ensure sufficient power reaches the receiver.

This directional narrowbeam-based approach has an ancillary benefit in that over-all interference is reduced, given that only nodes located within the narrow path of the beam can be victims of interference from the narrowbeam transmission. In both licensed and unlicensed scenario, this can greatly increase the spatial reuse. However, in the unlicensed scenario, while the likelihood of interference to other RATs or uncoordinated nodes is decreased, it is not completely removed. Moreover, when uncoordinated nodes are located within the narrow path of the beam, the uncoordinated nodes may experience more severe interference. Therefore, fair channel access schemes are required to ensure appropriate co-existence while not being too restrictive.

It was argued by some companies during the SI that since narrowbeam transmission reduces the likelihood of interference among transmitter-receiver pairs, LBT may not be required at higher frequencies. The argument says that beam management could enable fair co-existence by ensuring appropriate beam selection in light of neighboring interference. However, in real deployed scenarios, we cannot ensure that all transmitter-receiver pairs maintain their positions and directions on the time scale required to enable efficient beam management. The likelihood and randomness of hidden nodes increases in dynamic scenarios with some level of mobility.

Directional LBT has the benefit of detecting interference only in the path of the narrowbeam while ignoring any other nodes whose transmissions will not cause interference. This reduces the likelihood of exposed nodes caused by omni-directional LBT and the possibility of hidden nodes caused by no LBT. The co-existence thus benefits from buffers being emptied in a timely manner and reducing the likelihood of multiple transmitting nodes competing for the same resources.

The SI [2] showed that the general consensus is that there are gains to using directional LBT in cases of medium to high load and especially for tail UEs, in both UL and DL.

Observation 2: Dynamic scenarios with some level of mobility increases the likelihood of transmitter-receiver pairs interfering with each other even when using narrowbeams.

Observation 3: Directional LBT provides benefits over no LBT at least for medium to high loads and especially for tail UEs, while reducing the drawbacks associated with omni-directional LBT.

Proposal 1: Directional LBT is specified in Rel-17.

At its simplest, the beam used for LBT should match that of an associated transmission (e.g. the transmission that requires the transmitting node to acquire the channel). However, in some cases, the directional LBT beam may have different parameters than that of the associated transmission. For example, in some cases it makes sense for the LBT beam to be wider than that used for the associated transmission. This can be for the case where a node intends to perform multiple transmissions in a COT using different beams. It can also be for the case where a COT is acquired by a first node using a wide beam and then shared with other nodes, possibly each using narrower beams. Therefore, the parameters of the beam used for directional LBT can be determined from the parameters of the beam(s) used for one or more associated transmissions. It was discussed whether 3GPP specifications should define the relationship between the LBT beam and the transmission beam or leave it as implementation. Leaving it as implementation may lead to inefficient COT sharing given that the original beam used for LBT initiating a COT may limit the transmissions/transmitters that may share the COT.

Proposal 2: The relationship between the LBT beam and the transmission beam should be specified.

Proposal 3: A single directional LBT process can be performed on a beam whose parameters are determined from the parameters of the Tx beam of one or more associated transmissions.

## Receiver based LBT

As discussed above, using directional LBT can reduce the likelihood of exposed nodes and can thus improve spatial reuse without negatively impacting co-existence. However, directional LBT can still suffer from exposed nodes, for example, if an interfering node is transmitting in the direction towards the beam used for directional LBT by the transmitting node. Furthermore, hidden nodes may not be resolved by directional LBT.

One of the main flaws of LBT is that it is performed solely at the transmitter. In less directional scenarios with more homogeneous deployments, LBT performs adequately and the impact of hidden nodes is lessened. However, in beam-based scenarios, hidden nodes may be more stealth and undetectable at the transmitter regardless of whether omni-directional or directional LBT are used. That is because the interference from the hidden node may only be in the direction of the receiving node and not the transmitting node.

Observation 4: In a beam-based environment, LBT (omni-directional or directional) can fail to detect hidden nodes if the interference is only in the direction of the receiving node.

In order to mitigate the problem of hidden nodes, it makes sense for the receiver to assess the channel prior to receiving a transmission to make sure it does not suffer from any undue and undetectable (at the transmitter) interference. Moreover, channel assessment at the receiver can further reduce the effect of exposed nodes, given that the receiver can identify if the node is actually interfering with the reception of the transmission or not. During the SI, it was shown that in some medium to high load scenarios, receiver assisted LBT benefits tail UEs [2].

Proposal 4: Receiver based LBT should be considered for both omni-directional and directional LBT.

To support receiver based LBT, we should consider whether to use omni-directional or directional receiver-based LBT, the determination of the beam on which receiver based directional LBT should be performed, the method to perform receiver based LBT and how the receiver can indicate the outcome of receiver based LBT to the transmitter.

Similar arguments for directional LBT can be used for receiver based directional LBT. Therefore, we propose to consider the use of directional LBT. Furthermore, similar to Proposal 3, the parameters of receiver based directional LBT should be determined from the associated transmission to be received.

Proposal 5: Receiver based directional LBT is supported.

Proposal 6: A single receiver based directional LBT process can be performed on a beam whose parameters are determined from the parameters of the Rx beam of one or more associated transmissions.

To implement receiver assisted channel sensing and reporting, the following agreement was reached at RAN1 #104-e:

Agreement:

For receiver to provide assistance, channel sensing and reporting need to be performed. The following set of tools can be considered for further discussion

- Alt 1. Legacy RSSI measurement and reporting with possible enhancements

- Alt 2. AP-CSI report with possible enhancements

- Alt 3. LBT at receiver

  - Alt 3.1 eCCA

  - Alt 3.2 Cat2 LBT

For Alt.1, enhancements are required to handle the directionality of the transmissions. For Alt.2, there could be a new measurement to report. For example, a UE can be configured with resources on which it may perform channel assessment. The UE could then be triggered aperiodically to report a most recent set of channel assessment outcomes. This can reduce the need for signaling and reduce the latency associated with performing receiver based LBT at the moment a transmission is scheduled. Alt. 3 is similar to RTS/CTS and its benefits should be studied when considering the increased latency due to performing receiver based LBT at the moment a transmission is scheduled.

Proposal 7: Enhance legacy RSSI measurements and AP-CSI reporting to enable beam-based receiver assisted channel sensing and reporting.

## Switching between channel access mechanisms with and without LBT

As discussed herein, different LBT mechanisms should be supported: omni-directional, directional, and receiver based. However, the selection of an LBT mechanism (or no LBT altogether) should depend on a combination of deployment, channel characteristics and transmission requirements. In cases where the deployment is very dense and mobile, it makes sense to use directional and receiver based LBT. In cases where the deployment is controlled it makes sense to use the no LBT approach. Furthermore, The LBT mechanism used for channel access may be determined more dynamically. For example, there can be a hierarchical mechanism where a first LBT mechanism (e.g. omni-directional or directional) is used for long-term channel assessment and a second LBT mechanism (e.g. directional or no LBT) is used for immediate channel use.

Furthermore, since LBT (omni, directional and receiver assisted) gains were shown especially for tail UEs, their use should be UE-specific. The gNB should configure a set of channel access mechanisms and indicate a specific one for a UE or for a specific transmission. Given that the gains of directional and receiver assisted LBT are for tail UEs, it is beneficial for over-all performance that the use of directional or receiver assisted LBT be tied to the UE’s geometry. Therefore, a UE could determine what channel access mechanism to use, among a set of configured channel access mechanisms, as a function of measurements such as RSRP. It could also be beneficial to adapt channel access mechanism as a function of previous channel access or transmission success or failure. For example, a UE may require receiver assisted LBT if previous LBT operations were plagued with exposed nodes.

During RAN1 #104b-e, the following agreement was reached:

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode. Down-select between

- Alt 1. Support cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) gNB indication

- Alt 2. Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

- FFS: Whether the indication of the decision on applying LBT mode or no-LBT  mode is per beam (can be different for different UEs in different beams or can be different for different beam pairs between gNB and the UE) or per cell (can be different for different cells for a UE in carrier aggregation)

- FFS: Whether a gNB and its UE(s) can have different mode

- FFS: Whether L1 signalling can be used for both Alt 1 and Alt 2 for gNB indication

Based on the discussion above, Alt.2 is required in order to ensure tail UEs can be configured with LBT without requiring cell-center UEs to be unnecessarily configured with LBT. The indication should be at least per cell and per beam should also be considered, especially for CoMP-like deployments. Given that the configuration is up to gNB, whether a gNB and UE have different modes can be up to implementation. L1 signaling could enable dynamic indication. This can be beneficial for highly mobile scenarios. If per beam indication is specified, L1 signaling could be implicitly associated to the transmission beam.

Proposal 8: The UE receives configuration and indication of the channel access mode (omni-directional, directional, receiver assisted, no LBT) from the gNB using Alt. 2 (either cell specific or UE-specific indication)

Proposal 9: The indication of channel access mode is received per cell and per beam.

Proposal 10: The UE can select a channel access mechanism as a function of measurements (e.g. RSRP) or prior LBT performance.

For UEs configured with LBT in regions where LBT is not mandated, SCS can be used to transmit a greater set of UL channels and signals. For example, short PUCCH can be transmitted using SCS principles.

## LBT Parameters

During RAN1 #104-e, the following was agreed:

Agreement:

The baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS definition of Operating Channel BW

- FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi.

- FFS: EDT when the COT has time varying transmission beams and varying EIRP

If the ED threshold is not modified to account for the beamforming gain, then different UEs with different beamforming gains would perceive the channel occupancy differently. This would be counter to ensuring fair access to the channel. Whether or not LBT succeeds should be determined as a function of whether a victim node would suffer excessive interference. Therefore, it is desirable that LBT performance be normalized such that the transmission parameters would not impact the amount of interference a victim node may be subject to if LBT deems the channel unoccupied. Furthermore, based on Proposal 3, a single LBT operation performed on a sensing beam may be applicable to multiple transmissions each on different transmission beams. Therefore, the EDT should be determined as a function of the sensing beam. Moreover, the operating channel BW used in the EDT formula should depend on the LBT BW of the LBT procedure.

Proposal 11: Adapt EDT to account for beamforming gain of the sensing beam.

Proposal 12: The Operating Channel BW used in the EDT formula is equivalent to the LBT BW.

During RAN1 #104b-e, the LBT BW alternatives were discussed and were down-scoped to the following agreements:

Agreement:

For LBT for single carrier transmission, continue down selection between

- Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

- Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth

For LBT for multi-carrier transmission in intra-band CA, continue down selection between

- Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately

- Alt CA.2. gNB/UE performs single LBT over all CCs

- Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

For single carrier transmission, Alt SC.1 is unnecessary if a transmission is only over a portion of the channel. Such an alternative could be unnecessarily restrictive. Furthermore, it requires that a UE can support LBT operations over all the possible BWP BWs. Therefore, performing LBT only over the transmission BW using a set of LBTs each over a unit of LBT BW (Alt SC.3) is preferable. For smaller transmission BWs it is sufficient to support a single LBT BW, as in Rel-16. However, at higher frequencies, the range of channel BWs (and thus transmission BWs) is increased. Therefore, RAN1 should study if a set of LBT BWs can be specified, to support a wider variety of transmission BWs while limiting the number of LBT operations required.

For multi-carrier transmission in intra-band CA, Alt CA.2 suffers from similar problems as Alt SC.1, in that it requires the support of multiple LBT BWs and can be restrictive when transmission is on a subset of carriers or in a subset of RBs of a channel BW. Alt CA.1 is better since performing LBT on a carrier depends on if there is a transmission in that carrier. However, if Alt CA.1 implies that the LBT is performed over the entire channel BW, then it suffers from similar problems as Alt SC.1. Therefore Alt CA.5 is preferable. Similar to single carrier transmission, a set of LBT BWs can be specified to reduce LBT complexity.

Proposal 13: For single-carrier transmission, support Alt SC.3.

Proposal 14: For multi-carrier transmission, support Alt CA.5.

Proposal 15: Support a set of LBT BWs and LBT is performed in each CC on one or more adjacent LBT BWs that covers at least the transmission BW.

### LBT for COT sharing

It was agreed that when LBT mode is used, time domain multiplexing of DL/UL transmissions in different beams in the same COT is supported. In Rel-16, LBT category 4 must be used to initiate a COT, whereas for COT sharing, LBT category 1 or 2 may be used. Similar behavior should be applied when using directional LBT. However, to ensure fair co-existence, at least once prior to a transmission within a COT, LBT cat 4 covering a transmission beam used in the COT should be performed.

During RAN1 #104-e, the following agreement was reached:

Agreement:

On maximum gap within a COT to allow COT sharing without LBT, down-select from

- Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration

- Alt 2. Define a maximum gap X, such that a later transmission can share the COT without LBT only if the later transmission starts within X from the end of the earlier transmission

  - FFS: Value for X

- Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

  - FFS: Value for Y

  - FFS:  How to define the one-shot LBT

For COT sharing without a need for LBT, it is preferable that a maximum gap is considered (i.e. Alt.2 or Alt. 3). However, the size of a gap depends on the definition of an “earlier” transmission. For directional LBT, the earlier transmission should be one for whom the beam overlaps all of that of the later transmission. If a previous transmission is not for the same beam, then it should not be considered as the earlier transmission and can be considered to be part of the gap.

Proposal 16: When COT sharing, a UE determines what LBT to use based on the gap duration between the upcoming transmission and a previous transmission on the same beam.

For LBT in the case of MU-MIMO (SDM) transmission in a COT or TDM of beams in a COT, the following agreements were reached at RAN1 #104b-e:

Agreement:

For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed in TDM fashion

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Agreement:

Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed one after another in time domain

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

For the case of MU-MIMO (SDM) transmission), at least independent per-beam LBT sensing at the start of the COT should be possible. There may be cases where the beams used in a COT are not adjacent and performing LBT on a single beam that “covers” all beams could lead to exposed node issues in beams that won’t actually be used during the COT. For cases where the beams to be used in the COT are adjacent, a single widebeam LBT may suffice. For SDM, at least Alt-A should be supported given that some UEs may not have the capability to simultaneously sense in different beams. For Alt A-1, clarification is required to determine what completing an eCCA on one beam means. Would there be a set of time periods for each beam to determine when eCCA has failed? Alt A-2 does not seem applicable to the SDM case, when transmissions on different beams are multiplexed in the first slot of a COT. Therefore, we prefer Alt A-3.

For a COT with TDM of beams with beam switching, similar to SDM, at least independent per-beam LBT sensing at the start of the COT should be possible. Furthermore, only supporting Alt-B is not preferred given that some UEs may not have the required capability. Alt A-1 has similar issues as with the SDM case. Alt A-3 is applicable for TDM of beams. Alt A-2 is also applicable for TDM of beams. In such a case, it should be discussed whether CAT4 LBT should be used prior to the first transmission of a beam in a COT. Furthermore, the requirement of the type of LBT (or no LBT at all) prior to a transmission on a beam (e.g. after beam switching) can depend on the size of the gap between two transmissions of a same beam, as in Proposal 16.

Proposal 17: For a COT with MU-MIMO (SDM) transmission, support Alt-3.

Proposal 18: For a COT with TDM of beams with beam switching, support Alt A-2 or A-3.

Proposal 19: Support of Alt B for SDM or TDM of beams can be considered for some UEs.

# Conclusion

The contribution discusses channel access for unlicensed bands up to 71GHz. We provide the following observations and proposals:

Observation 1: Omni-directional LBT in unlicensed spectrum from 52.6GHz to 71GHz can under-represent interference in the direction of the associated transmission and over-represent interference in other directions.

Observation 2: Dynamic scenarios with some level of mobility increases the likelihood of transmitter-receiver pairs interfering with each other even when using narrowbeams.

Observation 3: Directional LBT provides benefits over no LBT at least for medium to high loads and especially for tail UEs, while reducing the drawbacks associated with omni-directional LBT.

Proposal 1: Directional LBT is specified in Rel-17.

Proposal 2: The relationship between the LBT beam and the transmission beam should be specified.

Proposal 3: A single directional LBT process can be performed on a beam whose parameters are determined from the parameters of the Tx beam of one or more associated transmissions.

Observation 4: In a beam-based environment, LBT (omni-directional or directional) can fail to detect hidden nodes if the interference is only in the direction of the receiving node.

Proposal 4: Receiver based LBT should be considered for both omni-directional and directional LBT.

Proposal 5: Receiver based directional LBT is supported.

Proposal 6: A single receiver based directional LBT process can be performed on a beam whose parameters are determined from the parameters of the Rx beam of one or more associated transmissions.

Proposal 7: Enhance legacy RSSI measurements and AP-CSI reporting to enable beam-based receiver assisted channel sensing and reporting.

Proposal 8: The UE receives configuration and indication of the channel access mode (omni-directional, directional, receiver assisted, no LBT) from the gNB using Alt. 2 (either cell specific or UE-specific indication)

Proposal 9: The indication of channel access mode is received per cell and per beam.

Proposal 10: The UE can select a channel access mechanism as a function of measurements (e.g. RSRP) or prior LBT performance.

Proposal 11: Adapt EDT to account for beamforming gain of the sensing beam.

Proposal 12: The Operating Channel BW used in the EDT formula is equivalent to the LBT BW.

Proposal 13: For single-carrier transmission, support Alt SC.3.

Proposal 14: For multi-carrier transmission, support Alt CA.5.

Proposal 15: Support a set of LBT BWs and LBT is performed in each CC on one or more adjacent LBT BWs that covers at least the transmission BW.

Proposal 16: When COT sharing, a UE determines what LBT to use based on the gap duration between the upcoming transmission and a previous transmission on the same beam.

Proposal 17: For a COT with MU-MIMO (SDM) transmission, support Alt-3.

Proposal 18: For a COT with TDM of beams with beam switching, support Alt A-2 or A-3.

Proposal 19: Support of Alt B for SDM or TDM of beams can be considered for some UEs.

# References

RP-193259, “Study on supporting NR from 52.6GHz to 71GHz”

3GPP TR 38.808, “Study on supporting NR from 52.6 GHz to 71 GHz”, v1.0.0

RP-210862, “Extending current NR operation to 71GHz”.