3GPP TSG RAN WG1#105-e	 	 	R1-2105723

e-Meeting, May 10th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on cross-carrier scheduling enhancements for NR DSS

Agenda Item:	8.13.1

Document for: 	Discussion and Decision

- Introduction

At the previous meetings, considerable scheduling patterns and monitored CSS for cross-carrier scheduling from sSCell to P(S)Cell were discussed, and RAN1 made following agreements and conclusion [1][2][3][4].

Agreements:

- Following scheduling combinations are allowed/not allowed when cross-carrier scheduling from an SCell to PCell/PSCell is configured

- self-scheduling on PCell/PSCell is allowed

- cross-carrier scheduling from PCell/PSCell to another SCell is not allowed

- self-scheduling on the ‘SCell used for scheduling PCell/PSCell’ is allowed

- cross-carrier scheduling from the ‘SCell used for scheduling PCell/PSCell’ to another serving cell is allowed

- cross-carrier scheduling from another serving cell to the ‘SCell used for scheduling PCell/PSCell’ is not allowed

- FFS: Search space and DCI format handling for the allowed cases above

Agreements:

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

  - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell/PSCell

Conclusion

- When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

- Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

Agreement

- When CCS from sSCell to PCell/PSCell is configured, UE monitors ‘DCI formats 0_0 and 1_0 in CSS that schedule PDSCH/PUSCH on PCell/PSCell’ only on the PCell/PSCell and not on the sSCell

Here, we discuss considerable monitoring cell (i.e. either P(S)Cell and/or sSCell) for the remaining search space type.

- Monitoring cell for USS (DCI format 0_1, 1_1, 0_2, 1_2)

At the previous meeting, considerable monitoring cell for USS (non-fall back DCI) was discussed and RAN1 made following agreements including some alternatives [2]. In addition, at the last meeting, candidate solutions were discussed and RAN1 made following working assumption [3].

Agreements:

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

    - Alt 2-4:

      - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell

  - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a USS set configured for scheduling of in PCell/PSCell)

Working Assumption

- When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 that schedule PDSCH/PUSCH on PCell/PSCell on PCell/PSCell USS set(s), and/or on sSCell USS set(s)

- The WA to be confirmed after agreements are made on PDCCH BD/CCE handling and PDCCH overbooking handling for CCS from sSCell to PCell/PSCell

- Specs also allow UEs supporting functionality of only Alt-1. Capability signaling details, if any, can be handled during the UE capability discussion for Rel17

- FFS: Whether the UE can monitor PDCCH from both cells in the same slot.

As the working assumption shows, not only Alt 2 but also Alt 1 can be supported depending on UE capability signaling. Here, we discuss Alt 2 candidates in order to solve an FFS in the working assumption.

Firstly, we focus on scheduling flexibility and UE power consumption for monitoring cell. Alt 2-1 has the largest scheduling flexibility, however, more UE power consumption may be required to monitor both P(S)Cell and sSCell simultaneously. Alt.2-3 may also have such issue since UE shall be able to monitor some DCI formats on sSCell and other DCI formats on P(S)Cell simultaneously. On the other hand, Alt 2-2 has benefits on scheduling flexibility and UE power consumption since this alternative does not require UE to monitor any DCI format on both P(S)Cell and sSCell simultaneously. In order to support Alt 2-2, we need to discuss the switching mechanism (e.g. how to indicate monitoring cell for each DCI format). Alt 2-4 also has benefit on UE power consumption, i.e., UE is not required to monitor any DCI format on both P(S)Cell and sSCell simultaneously. In both Alt 2-2 and 2-4, according to available PDCCH capacity on P(S)Cell and sSCell, PDCCH load balancing between P(S)Cell and sSCell for USS can be achieved. Therefore, we prefer Alt 2-2 and/or Alt 2-4.

Secondly, regarding PDCCH BD/CCE limitation, the current specification supports the BD/CCE limitation per slot for P(S)Cell only case and both P(S)Cell and SCell case. Even if the UE can monitor DCI formats on both P(S)Cell and sSCell simultaneously, we think that the current specification would cover the case.

Proposal 1:

- Regarding USS (format 0_1, 1_1, 0_2, 1_2), the following alternatives can be considered for further discussion

  - Alt 2-2:

    - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

      - FFS: Details of switching mechanism

    - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

  - Alt 2-4:

    - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- Conclusion

In this contribution, we discussed on how to support PDCCH of sSCell scheduling PDSCH or PUSCH on P(S)Cell for Rel-17 NR DSS. Based on the discussion, we made following proposal.

Proposal 1:

- Regarding USS (format 0_1, 1_1, 0_2, 1_2), the following alternatives can be considered for further discussion

  - Alt 2-2:

    - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

      - FFS: Details of switching mechanism

    - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

  - Alt 2-4:

    - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

References

- 3GPP RAN1#102-e, Chairman’s Notes, Aug. 2020.

- 3GPP RAN1#103-e, Chairman’s Notes, Nov. 2020.

- 3GPP RAN1#104-e, Chairman’s Notes, Feb. 2021.

- 3GPP RAN1#104b-e, Chairman’s Notes, Apr. 2021.