3GPP TSG RAN WG1 Meeting #106-e	  R1-2106473

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.13.2

Source:	Huawei, HiSilicon

Title:	Discussion on efficient activation/de-activation mechanism for SCells

Document for:	Discussion and Decision

Introduction

In RAN#88-e, a WID on Further Multi-RAT Dual-Connectivity enhancements is approved for Release 17 [1], where an efficient SCG and SCells (de)activation mechanism should be specified based on RAN1 leading:

The objective of this work item is to specify enhancements to MR-DC related scenarios. At least the following topics should be considered in the work: - Support efficient activation/de-activation mechanism for one SCG and SCells - Support for one SCG applies to (NG)EN-DC, and NR-DC [RAN2, RAN3, RAN4] - Support for SCells applies to NR CA, based on RAN1 leading mechanisms [RAN1, RAN2, RAN4] - This objective applies to FR1 and FR2 - Support of conditional PSCell change/addition [RAN2,RAN3, RAN4] - support scenarios which are not addressed in Rel-16 NR mobility WI

In RAN1#105-e meeting, some agreements have been achieved:

Agreement For efficient activation of Scells, the triggered temporary RS is aperiodic. Agreement For efficient activation of a Scell (in known Scell case), at least the number of temporary RS bursts is indicated by a field in new MAC-CE - The number of temporary RS bursts is RRC configurable. - FFS: which field in MAC-CE is used and how this field is associated with the number of bursts - For the purpose of designing temporary RS Scell activation, there is no RAN1 specification impact for the case where the number of indicated temporary RS bursts is smaller than what is expected by the UE Agreement To trigger temporary RS for efficient activation of SCells, the contents of the triggering MAC-CE(s) in a single PDSCH provide at least the following information (explicitly or implicitly): - Whether or not temporary RS is triggered - FFS detailed Information of temporary RS, e.g.: - Resources used for triggered Temporary RS - Triggering time offset of triggered Temporary RS - QCL source for triggered Temporary RS - FFS: Detailed signalling structure of the triggering MAC-CE(s) including the down-selection between the following example options and whether the decision should be made in RAN1 or RAN2 - Opt. 1.1: One new MAC CE for both SCell activation triggering and corresponding temporary RS triggering - Opt. 1.2: One R15/16 SCell activation MAC CE for SCell activation triggering and one new MAC CE (in the same PDSCH) for corresponding temporary RS triggering Agreement For efficient activation of a Scell (in known Scell case), the triggering offset of temporary RS is indicated by a field in new MAC-CE - The candidate value(s) of triggering offset(s) is RRC configurable - FFS: which field in MAC-CE is used and how this field is associated with the value of triggering offset Agreement For the reference slot for triggering offset of temporary RS - Option 2: the last DL slot of the to-be-activated Scell overlapping with slot n+k as defined in 38.213 sub-clause 4.3 - FFS: the earliest slot no earlier than the reference slot for a UE to receive a triggered temporary RS Agreement If a UE measures a temporary RS triggered by a MAC-CE during SCell activation procedure, the measurement is performed within the BWP bandwidth of BWP indicated by firstActiveDownlinkBWP-Id

In this contribution, the discussions above are followed up.

Low latency SCell activation

SCell activation delay constitution

According to TS 38.213, TS 38.133 and TS 38.321, when a UE receives a SCell activation command in a PDSCH in slot n, the UE shall complete SCell activation no earlier than n+k  and no later than n+ [THARQ + Tactivation_time + TCSI_Reporting] as shown in Figure 1, where:

- The value of  k=k1+3∙Nslotsubframe,μ+1 and THARQ is the timing between DL data transmission and acknowledgement corresponding to k1 in k described above.

- Tactivation_time includes the delay of MAC-CE parsing time, RF warm up, AGC settling and frequency/time synchronization; to be specific, take the case as an example that SCell belongs to FR1 is known and SCell measurement cycle is equal to or smaller than 160ms in TS 38.133, Tactivation_time includes

  - 3ms: MAC CE command application, RF warm-up/retuning

  - TFirstSSB:  the time to first SSB indicated by the SMTC after n + THARQ+3ms. The typical SMTC periodicity is 20ms, so TFirstSSB can be 20ms

  - 2ms: SSB processing.

- TCSI_reporting is the delay including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources. Additionally, if the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1, beam management is needed for valid CSI acquisition, which means SCell activation time become more unbearable.

Figure 1 SCell activation procedure

Therefore, reducing Tactivation_time and TCSI_Reporting is the key point to achieve efficient SCell activation/de-activation mechanism.

Scenarios for low latency SCell activation

Based on the discussion in section 2.1, Tactivation_time includes the delay of MAC-CE parsing time, RF warm- up, AGC settling and frequency/time synchronization. The delay of AGC settling is mainly the delay of waiting for and receiving SSB or RS for the information of path loss, coupling loss, RSRP. The delay of frequency/timing synchronization is mainly the delay of waiting for and receiving SSB or RS for frequency/timing offset information.

Two straightforward solutions can be proposed to reduce the Tactivation_time, the first solution is acquiring the required information by an alternative RS, i.e. temporary RS, and another is acquiring the required information from the other activated cells, i.e. the equivalent of such required information is assured by network. The first solution is universally applicable for SCell activation, but the time saving is limited because several ms is needed for the triggering RS offset and processing time. The second solution is applicable for the special scenarios, but more time can be saved. To minimize the activation time, two scenarios can be studied.

If multiple aggregated cells are operated in the same frequency band or adjacent frequency bands, some common properties between to-be-activated SCell and activated cells, as exemplified and analyzed in section 2.3, can be exploited to expedite the SCell activation process. Especially, take continuous 200MHz (100MHz cell 1+100MHz cell 2) in 3500MHz band or continuous 160MHz (100MHz cell 1+60MHz cell 2) in 2600MHz band as an example, these two cells are typically associated with the same hardware, thus those common properties from one activated cell can be used to activate another cell. Therefore, the SCell activation procedure can be simplified and the SCell activation delay can be reduced obviously, as discussed in following subsection.

For other cases, no common properties can be assumed considering different large scale fading loss of different bands or non-co-located deployment. Therefore, among the configured cells for a given UE, there are two possible scenarios as shown in Figure 2.

Figure 2 Two scenarios for CA

- Scenario 1: Some common properties can be derived from active cell. One typical example is that configured cells in the same or adjacent frequency band are co-located and even associated with same hardware or module at the BS side.

- Scenario 2: No common properties can be derived from active cell. This scenario is the general case which can be applied to any CA band combinations and can be applied to non-co-located deployment.

Observation 1: Two scenarios should be considered for low latency SCell activation

- Scenario 1: some common properties can be derived from active cell

- Scenario 2: no common properties can be derived from active cell.

On reducing Tactivation_time

Known SCell and unknown SCell

In RAN4, the delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions, including frequency range, SCell status, SCell measurement cycle configuration, CSI-RS configuration mode and so on. During the discussion on the temporary RS, some conclusions are related to SCell status, i.e. known SCell or unknown SCell. Some companies have concern that the network does not clearly know the known/unknown status of the UE SCell.

- RAN4 conclusion: Different temporary RS bursts are required for known SCell and unknown SCell

In the RAN4 LS reply [2], there are some conclusions on required number of temporary RS burst for SCell activation.

| FR1 | known | MC <= 160ms | 1 burst for AGC and T/F tracking |
| --- | --- | --- | --- |
|  |  | MC > 160ms | 1 burst for AGC, and separate 1 burst for T/F tracking Gaps between RS symbols for AGC and T/F tracking: - Option 1: 2 slots - Option 2: 2 ms |
|  | unknown | Intra-band continuous CA | 1 burst (4 samples) for AGC and 1 burst (4 samples) for T/F tracking |
|  |  | Intra-band non-continuous CA and inter-band CA | It is not a target scenario for temporary RS based SCell activation latency optimization |
| FR2 | known | at least one active serving cell on that FR2 band | 1 burst (4 samples) can be used for T/F tracking |
|  | unknown |  |  |
|  | known | no active serving cell on that FR2 band | 1 burst (4 samples) can be used for T/F tracking |
|  | unknown | no active serving cell on that FR2 band | It is not a target scenario for temporary RS based SCell activation latency optimization |

gNB can indicate the number of temporary RS burst to UE based on frequency range, status of the SCell and SCell measurement cycle. Most of these parameters depend on gNB configuration so that gNB and UE can share the same understanding about the parameter settings. However, the parameter of known/unknown SCell may be an exception.

If gNB and UE have the different understanding on the status of SCell, the SCell activation delay requirement defined in the RAN4 should be invalid for UE, similar scenarios also occur in R15/16 SCell activation procedure. In the last RAN1 meeting, companies has come to a conclusion about this issue, that is, for the purpose of designing temporary RS SCell activation, there is no RAN1 specification impact for the case where the number of indicated temporary RS bursts is smaller than what is expected by the UE.

- Working assumption: For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell

From the working assumption, it can be observed that the QCL assumption is valid only in the case of known SCell. When gNB considers that the UE is unknown, gNB can ignore the assumption no matter UE is known or unknown. Therefore, there is only one case in which the gNB and the UE have different understanding. That is, the gNB considers that the UE is known, and the UE considers itself is unknown. In this case, UE can ignore the QCL assumption.

Observation 2: There is no impact on RAN1 specification if gNB and UE have different understanding on the status of SCell, i.e. known or unknown.

Trigger of temporary RS and SCell activation

In the previous meetings, many candidate options for the trigger of temporary RS and SCell activation are discussed, and finally converged to two options:

- Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s).

- Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding Rel-15/16 A-TRS(s)

For the option 2, it is up to RAN4 whether or not to consider an activation time enhancement without requiring further RAN1 work. For option 1a, detailed signalling structure of the triggering MAC-CE(s) including the down-selection between the following example options and whether the decision should be made in RAN1 or RAN2 should be discussed

- Opt. 1.1: One new MAC CE for both SCell activation triggering and corresponding temporary RS triggering

- Opt. 1.2: One R15/16 SCell activation MAC CE for SCell activation triggering and one new MAC CE (in the same PDSCH) for corresponding temporary RS triggering

When analyzing the pros and cons of the two options, some impacts on RAN2 specifications should be considered, the input of RAN2 views is necessary. The decision can be made in RAN1 or RAN2 as long as the standard impacts of RAN1 and RAN2 are considered.

Opt. 1.1 is preferable, some potential cons for the Opt. 1.2 is provided as following:

- Two MAC CE commands are required, two LCIDs are consumed.

- Usually it is an implementation issue to transmit two MAC-CE either jointly in one PDSCH or separately in two PDSCHs. gNB makes decision depending on available resource and system efficiency. Restriction on two MAC-CEs in one PDSCH may lead system efficiency loss.

- UE which receives the legacy MAC CE needs to check whether the new MAC CE for temporary RS is included or not in the same PDSCH, and then UE follows Rel-17 UE behaviour or legacy UE behaviour accordingly.

- Opt 1.2 still need to define a new MAC-CE, the spec impact is similar as Opt 1.1.

Proposal 1: One new MAC CE should be introduced to trigger both SCell activation and corresponding temporary RS(s).

Temporary RS design

The detailed signaling design of MAC-CE is up to RAN2, e.g. the field and number of bits for each field, but it should be RAN1’s task to decide what contents in the MAC CE are necessary. Some contents had agreed to be contained in the MAC CE, including the number of temporary RS bursts, whether or not temporary RS is triggered, and triggering offset of temporary RS. But which field in MAC-CE is used and how this field is associated with the contents are not clear.

In addition to the agreed contents, a list of potential contents for the signaling is provided below,

For each SCell among multiple to-be-activated SCells,

- Resources used for triggered temporary RS

- Gap between two temporary RS bursts

- QCL resource for triggered temporary RS

It can be found that a lot of contents need to be contained in the MAC CE, if one field is designed for each content, the overhead of the MAC CE is high. It is better to configure the contents expect whether or not temporary RS is triggered by RRC and associate them with a temporary RS ID, then only temporary RS ID needs to be indicated in the MAC CE. The detailed RRC signaling should be decided in the RAN2.

Proposal 2: An ID of temporary RS should be indicated in the MAC CE, the corresponding information associated with the temporary RS ID is configured by RRC.

QCL configuration for TRS

One remaining issue of temporary RS is the QCL configuration for TRS. In current specification [3], for a periodic CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, which is expected to be always configured to a UE in RRC connected mode, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block, or

-	'typeC' with an SS/PBCH block and, when applicable,'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

For an aperiodic CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE shall expect that a TCI-State indicates qcl-Type set to 'typeA' with a periodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, qcl-Type set to 'typeD' with the same periodic CSI-RS resource.

In short, the QCL mechanism of TRS can be observed as follows,

- For a UE in RRC connected mode, periodic TRS must be configured.

- Both kinds of TRS should be configured with a QCL source.

- Triggering-based aperiodic TRS should be QCLed with periodic TRS, which means periodic TRS should be received before the aperiodic TRS.

Because it has been agreed that temporary RS for SCell activation is based on TRS and reuses legacy TRS structure, the QCL mechanism of TRS should be inherited to temporary RS. However, periodic TRS is not active yet during SCell activation, therefore, some straightforward revisions of its QCL mechanism are:

- Temporary RS can be QCLed with other RS(s) during the SCell activation, e.g. SSB.

- QCL relationship between temporary RS and periodic TRS should be retained by sharing the same QCL source.

The QCL of aperiodic TRS should be associated with one effective RS/SSB. In the previous meeting, the working assumption “For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell” has been achieved. In the known SCell case, the UE has acquired coarse synchronization information from a SSB, the QCL source of temporary RS can be SSB so that the UE can receives the temporary RS based on the synchronization information of SSB, the working assumption should be confirmed. In addition to the working assumption, there are still some issues that need to be further studied, e.g. the QCL type, the case of unknown case, other QCL source.

For the QCL type of temporary RS in case of known SCell, it should be similar to the periodic TRS, and following quasi co-location type(s) are expected:

-	'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block.

In case of unknown SCell, no effective information of SSB in the to-be-activated SCell can be achieved when receiving the SCell activation command. There are two candidate methods, one is to receive the SSB, and the other is to refer to the QCL source of other cells. For the first method, the activation method falls back to R16, no temporary RS is required. For the second method, at least in the scenario 1 mentioned above, the QCL source can be SSB/TRS in other active cells. As analyzed in the section 2.2, when two cells are located in same frequency band or adjacent frequency bands, some common properties can be shared. Unknown SCell can be configured a co-located active cell, then UE can acquire the AGC, time/frequency synchronization information from another cell. In addition to existing QCL type, new QCL type associated with AGC can be defined.

As to maintain the QCL relationship between periodic TRS and aperiodic TRS to a certain extent, the source RS for QCL of the temporary RS during the SCell activation and periodic TRS after SCell activation should be configured the same, or the temporary RS is configured as the QCL source of periodic TRS.

Proposal 3: The following working assumption should be confirmed:

- For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell.

Proposal 4: For the QCL type of temporary RS in case of the known SCell, the following quasi co-location type(s) are expected:

- 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block.

Proposal 5: For efficient SCell activation with assistance of temporary RS, a SSB/TRS of another active cell can be indicated as a QCL source for the temporary RS in case of unknown SCell at least in the scenario 1.

Proposal 6: The QCL source RS of a temporary RS during the SCell activation should be same as that of periodic TRS after SCell activation.

Reducing Tactivation_time on two scenarios

As discussed in the section 2.2, two scenarios can be proposed with different solutions to reduce SCell activation time.

For scenario 1, take two continuous cells as an example, if one is active cell and another is inactive SCell, then when the inactive SCell is going to be activated, there is no need for a UE to search SSB or any RS for frequency/time synchronization, path loss, coupling loss, and even RSRP during SCell activation. Because all this information can be derived from the active cell which associates with same BS hardware. Through this way, for the case that the SCell is known FR1 cell and SCell measurement cycle is equal to or smaller than 160ms in TS 38.133, Tactivation_time can be reduced to ‘3ms MAC CE command application’ + ‘200us RF chain reconfiguring’ + ‘200us coarse AGC’ as shown in Figure 3 (i.e., the time of receiving and processing SSB or RS are removed). If PDCCH triggering is used to replace MAC CE triggering for SCell activation, Tactivation_time can be further reduced to ‘X symbol PDCCH decoding’ + ‘200us RF chain reconfiguring’ + ‘200us coarse AGC’.

Figure 3 Optimized SCell activation procedure for scenario 1

Obviously, Tactivation_time is reduced significantly. Therefore, proper UE assumptions on the common properties across serving cells in the scenario 1 is beneficial and necessary to be standardized.

Whether UE can assume shared information of path loss, coupling loss and frequency/timing from other cell depends on network implementation (i.e., whether the two cells share same hardware and are co-located deployment). Thus, network needs to inform the UE about this. To achieve this, one candidate solution is to define cell groups sharing common properties. Network can configure one or some cell groups for the UE. After that, UE assumes some specified properties used for SCell activation can be shared from another cell within a same cell group. The properties contain path loss, coupling loss, RSRP, frequency/timing offset.

Proposal 7: For scenario 1, some common properties derived from activated cell (e.g. frequency/timing synchronization, path loss, coupling loss, RSRP) can be used to reduce Tactivation_time.

Proposal 8: For scenario 1, cell groups sharing common properties (e.g. frequency/timing synchronization, path loss, coupling loss, RSRP) can be configured for a UE.

On reducing TCSI_Reporting

TCSI_Reporting depends on the CSI-RS resource and CSI reporting configuration. For P-CSI-RS reporting, no additional signaling for P-CSI-reporting triggering after SCell configuration which makes SCell activation procedure simpler and little specification impact. If the periodicity of periodic CSI-RS and CSI reporting is long, TCSI_Reporting is obviously long. To reduce it, a short enough periodicity for CSI-RS resource and CSI-RS reporting can be configured. But short periodicity leads to the large resource overhead, an effective time can be defined for the short periodicity CSI-RS and CSI-RS reporting. Short periodicity P-CSI-RS and P-CSI-RS reporting for low latency SCell activation can reduce uncertainty time in acquiring the first available downlink CSI reference resource or uncertainty in acquiring the first available CSI reporting resources in TCSI_Reporting. The existing minimum period of P-CSI-RS resource and reporting is 4 slots, shorter periodicity can be configured, e.g. 1 slot, thus UE can report the CSI in each slot.

Proposal 9: Short periodicity of 1 slot for periodic CSI-RS resource and CSI reporting is supported for low latency of SCell activation.

Normally, gNB activates SCell for a UE only when the UE has data waiting for transmission. In order to improve user’s experience, the most important thing is to start data transmission as soon as possible. For the scenario that SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1, if the PCell/PSCell and the target SCell are in a band pair with independent beam management and the target SCell is unknown to UE, a procedure of beam management is needed to find the suitable RX/TX beam for synchronization and CSI measurement which will increase the latency of SCell activation.

Actually, during the procedure of SCell activation, when gNB receives the beam reporting, i.e. the L1-RSRP report, it implies that UE has completed beam selection and timing synchronization which are necessary conditions for downlink transmission. It means that gNB can start downlink transmission with a conservative or rough MCS on the SCell, and UE can start to monitor PDCCH on the SCell, even though the valid CSI report is not yet reported. Thus the gNB and UE can assume the SCell is activated after the Tactivation_time. After normal CSI report is reported as legacy procedure, the MCS for transmission can be refined for better data rate. So, a possible solution to reduce the latency of SCell activation for FR2 is to remove the time duration to acquire valid CSI report from the SCell activation procedure for the scenario of SCell being activated belongs to FR2 and unknown to UE.

Proposal 10: TCSI_Reporting can be removed from the SCell activation procedure for the scenario that SCell being activated belongs to FR2 and is unknown to UE.

Conclusion

According to the above discussions, we have the following proposals:

Observation 1: Two scenarios should be considered for low latency SCell activation

- Scenario 1: some common properties can be derived from active cell

- Scenario 2: no common properties can be derived from active cell.

Observation 2: There is no impact on RAN1 specification if gNB and UE have different understanding on the status of SCell, i.e. known or unknown.

Proposal 1: One new MAC CE should be introduced to trigger both SCell activation and corresponding temporary RS(s).

Proposal 2: An ID of temporary RS should be indicated in the MAC CE, the corresponding information associated with the temporary RS ID is configured by RRC.

Proposal 3: The following working assumption should be confirmed:

- For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell.

Proposal 4: For the QCL type of temporary RS in case of the known SCell, the following quasi co-location type(s) are expected:

- 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block.

Proposal 5: For efficient SCell activation with assistance of temporary RS, a SSB/TRS of another active cell can be indicated as a QCL source for the temporary RS in case of unknown SCell at least in the scenario 1.

Proposal 6: The QCL source RS of a temporary RS during the SCell activation should be same as that of periodic TRS after SCell activation.

Proposal 7: For scenario 1, some common properties derived from activated cell (e.g. frequency/timing synchronization, path loss, coupling loss, RSRP) can be used to reduce Tactivation_time.

Proposal 8: For scenario 1, cell groups sharing common properties (e.g. frequency/timing synchronization, path loss, coupling loss, RSRP) can be configured for a UE.

Proposal 9: Short periodicity of 1 slot for periodic CSI-RS resource and CSI reporting is supported for low latency of SCell activation.

Proposal 10: TCSI_Reporting can be removed from the SCell activation procedure for the scenario that SCell being activated belongs to FR2 and is unknown to UE.

- References

RP-201040, “Revised WID on Further Multi-RAT Dual-Connectivity enhancements”, TSG RAN Meeting #88e, June 29 - July 3, 2020.

R1-2106427, “Reply LS on temporary RS for efficient SCell activation in NR CA”, TSG RAN1 meeting #106e, August. 16-27, 2021.

TS 38.214: "NR; Physical layer procedures for data".