3GPP TSG RAN WG1 #106-e	R1-2106637

e-Meeting, August 16th – 27th, 2021

Agenda item:		8.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On UL intra-UE multiplexing and prioritization enhancements

Document for:		Discussion and Decision

# Introduction

In this contribution, we present our views on UL intra-UE multiplexing and prioritization. Our discussion is built on top of the discussions, agreements and open issues related to Rel-17 IIoT/URLLC WI. Summary from RAN1#102-e, RAN1#103-e, RAN1#104-e, RAN1#104bis-e, and RAN1#105-e can be found in R1-2007075, R1-2009546, R1-2101842, R1-2103868, and R1-2106063, respectively.

Section 2 covers the issues related to the overlapping CG PUSCH vs. DG PUSCH of different PHY priorities. In Section 3, we further discuss the enhancements related to intra-UE multiplexing/prioritization of traffic with different priorities. Section 4 includes the discussion related to simultaneous PUSCH/PUCCH transmission from different cells.

# CG PUSCH vs DG PUSCH of different PHY priorities

The high-level support of PHY prioritization for the scenarios of high-priority CG PUSCH vs. low-priority DG PUSCH and high-priority DG PUSCH vs. low-priority CG PUSCH was agreed in RAN1#102-e and #103-e, respectively. However, the exact cancellation behaviour is still open for both scenarios. In the following, we provide our view on the cancellation operation for the two scenarios. It should be first noted that here we don’t consider the scenarios where a PUCCH overlaps with at least one of the overlapping PUSCHs and the impact of uplink skipping. Actually, since there are still some related Rel-16 discussions on these aspects, it’s preferable to first wait for the outcome of these discussions.

Observation 2.1: For the scenarios CG PUSCH vs. DG PUSCH of different PHY priorities, the aspects related to handling the cases where a PUCCH overlaps with at least one of the overlapping PUSCHs and the impact of uplink skipping can be discussed after reaching a conclusion on the related Rel-16 discussions.

High-priority CG PUSCH vs. low-priority DG PUSCH:

Recall that handling the collision of overlapping channels with different priorities where the high-priority channel does not have an associated scheduling PDCCH was already specified in Rel-16 NR, except for the scenario CG PUSCH vs. DG PUSCH with different priorities. The related agreement from Rel-16 is that it is up to UE implementation to ensure that the low-priority UL transmission is cancelled, at the latest, before the start of the high-priority UL transmission. Since the scenario here is quite similar to the scenarios discussed in Rel-16, for which the high-priority channel does not have an associated PDCCH, the same way of handling as specified in TS 38.213 could be adopted.

Proposal 2.1: For the scenario high-priority CG PUSCH vs. low-priority DG PUSCH, it is up to UE implementation to ensure that the low-priority DG PUSCH is cancelled, at the latest, from the first symbol that is overlapping with the high-priority CG PUSCH.

High-priority DG PUSCH vs. low-priority CG PUSCH:

The handling of this scenario can be the same as handling of collision between a dynamically scheduled high-priority channel and a low-priority channel, which was specified in Rel-16 (except for DG PUSCH vs. CG PUSCH). Specifically, at least the aspects related to cancellation timeline and minimum processing timeline extension, defined in Rel-16 and captured in TS 38.213, Sec. 9, can be adopted for the scenario high-priority DG PUSCH vs. low-priority CG PUSCH as follows:

- A UE is expected to cancel the overlapping low-priority CG PUSCH by the first overlapping symbol at the latest. Further, a UE expects that the first overlapping symbol of the high-priority DG PUSCH is not earlier than Tproc,2+d1 after the last symbol of the PDCCH scheduling the DG PUSCH.

- The minimum processing timeline for the DG PUSCH should be extended by d2 symbols; d2 is reported as a UE capability. It should be FFS whether this d2 is the same as the Rel-16 UE reported capability, or it should be a different one (i.e. some new d3 with independent UE capability reporting).

Proposal 2.2: The Rel-16 handling of the scenarios where a dynamically scheduled high-priority channel overlaps with a low-priority channel is adopted for the scenario of overlapping between high-priority DG PUSCH and low-priority CG PUSCH.

# 3	Intra-UE multiplexing enhancements for overlapping channels with different priorities

In the following, we will separately discuss the scenarios depending on whether PUSCH is involved or not, i.e. control vs. control and control vs. data.

## 3.1	Control channel vs. control channel

In RAN1#102e, the support of multiplexing was agreed for the following scenarios where a control channel overlaps with another control channel(s):

Agreements: Support multiplexing for following scenarios in R17: - Multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH in R17. - Multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations). - Multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH - …

In the following, we discuss the different scenarios covered in the above agreement, including some issues and open points from previous RAN1 meetings.

### 3.1.1 PUCCH with high-priority HARQ-ACK vs. PUCCH with low-priority HARQ-ACK/UCI

Multiplexing high-priority HARQ-ACK and low-priority HARQ-ACK should be allowed only if this multiplexing does not degrade the high-priority HARQ-ACK performance or if the degradation is acceptable by the network. How to guarantee the reliability and latency of high-priority HARQ-ACK bits should be the main focus when discussing the different aspects of multiplexing for this scenario.

#### 3.1.1.1 Explicit indication for enabling/disabling multiplexing

In RAN1#103-e, the following was agreed on the support of indication for enabling/disabling multiplexing of high-priority and low-priority HARQ-ACKs:

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI.

The aspect related to whether to use RRC-only or to also rely on DCI for the enabling/disabling of multiplexing has been discussed in RAN1#105-e, but without reaching any agreement.

In our view, it is important to allow the network to dynamically control and avoid any potential impact of multiplexing on the high-priority HARQ-ACK from latency or reliability perspective. With such dynamic control, the network can instruct the UE, via the DCI scheduling the high-priority HARQ-ACK, to not multiplex the high- and low-priority HARQ-ACKs, i.e. the low-priority HARQ-ACK is dropped in this case. For instance, if multiplexing would result in selecting a PUCCH resource that is a number of symbols later than the PUCCH resource that would contain high-priority HARQ-ACK only, the latency could be impacted as the high-priority HARQ-ACK transmission will be delayed a number of symbols. The network may find such a delay unacceptable and could thus dynamically disable the multiplexing operation.

Moreover, supporting dynamic indication for multiplexing through DCI signalling would ease the operation and specification effort also in other aspects:

- There may be no need to specify bundling / compression of low-priority HARQ-ACK information as the enabling/disabling of multiplexing could be dynamically indicated. If the combined payload size would become too large and thereby too much impacting the reliability or latency of the high-priority information, the gNB would simply dynamically disable the multiplexing.

- There may be no need to define an extensive set of rules when multiplexing should be possible as this could be left to gNB implementation simplifying the specification effort.

- The handling of multiplexing of sub-slot PUCCH and slot-based PUCCH would be under direct gNB control, as discussed below in Sec. 3.1.3.

Although the focus here is on the scenario where a high-priority HARQ-ACK overlaps with a low-priority HARQ-ACK, the same operation could also be used for the scenarios where a high-priority HARQ-ACK overlaps with a low-priority multiplexed UCIs (such as low-priority SR+HARQ-ACK) if multiplexing is agreed for these scenarios.

Clearly, in a similar way to any new features that get introduced in RAN1 specs, it would be first required to configure the UE with the above operation via RRC. So, we don’t really see anything special that would be needed in that regard.

Proposal 3.1: The gNB dynamically indicates, via an explicit field in the DCI scheduling high-priority HARQ-ACK, whether multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK (or more generally low-priority multiplexed UCIs) is enabled or disabled.

#### 3.1.1.2 Multiplexing of 1-bit high-priority HARQ-ACK and 1-bit low-priority HARQ-ACK

PUCCH resource selection:

In RAN1#104-e, the following agreement was made regarding the PUCCH resource selection for the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK:

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS in case the total number of LP and HP HARQ-ACK bits is 2. - FFS details

In RAN1#104bis-e, the following related proposal has been on the table:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) in case the total number of LP and HP HARQ-ACK bits is 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS: the case where HARQ-ACK is for SPS

This proposal, under which the high-priority PUCCH resource is used for multiplexing, received the support of a large majority of companies in RAN1#104bis-e. And as can be seen from the agreement copied above, this was agreed for the case where the total payload size is more than 2. We don’t see any strong reason why not to also support the same for the case where the total number of HARQ-ACK bits is 2, as also in this case the use of the high-priority PUCCH resource for multiplexing protects the high-priority HARQ-ACK.

It should be noted that in RAN1#105-e, the following agreement on treating the two bits as high-priority bits was reached:

Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority. - Rel-15 design (for PF0 and PF1) is baseline.

Although this proposal may implicitly suggest that the high-priority PUCCH resource is used to carry the two multiplexed bits, it would be preferable to clearly agree on this aspect. We thus propose:

Proposal 3.2: For the scenario where a PUCCH carrying 1-bit high-priority HARQ-ACK overlaps with another PUCCH carrying 1-bit low-priority HARQ-ACK, the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK is done on the high-priority PUCCH resource.

In addition, on the first FFS point in the proposal copied above, the main reason a high-priority PUCCH resource (or more precisely, a PUCCH resource belonging to the PUCCH configuration of high-priority HARQ-ACK) should be used for multiplexing high-priority HARQ-ACK and low-priority HARQ-ACK is to avoid impacting the reliability and latency of high-priority HARQ-ACK. Therefore, we don’t see a motivation for a specific/dedicated configuration of PUCCH resource(s) configured for the multiplexing of high and low priority HARQ-ACK. Therefore, we propose:

Proposal 3.3: Dedicated PUCCH resource configuration for multiplexing of high- and low-priority HARQ-ACK is not supported.

Bit mapping on (high-priority) PUCCH:

Based on the RAN1#105-e’s agreement copied above, the two bits will be treated as high-priority bits. One additional decision that RAN1 will have to take is on the order of the bits, i.e. if the two bits are ordered as [HP HARQ-ACK bit, LP HARQ-ACK bit] or [LP HARQ-ACK bit, HP HARQ-ACK bit] when applying the mapping on the high-priority PUCCH resource. As we do not see any difference between these two options, we suggest here to use the bit order of the two bits to be [HP HARQ-ACK bit, LP HARQ-ACK bit], i.e. the high-priority HARQ-ACK defines the 1st or MSB of the two bits.

Proposal 3.4: For the scenario where a PUCCH carrying high-priority HARQ-ACK overlaps with another PUCCH carrying low-priority HARQ-ACK and the total payload size is two bits, the order of the multiplexed two bits could be [high-priority HARQ-ACK bit, low-priority HARQ-ACK bit].

#### 3.1.1.3 On whether to allow multiplexing in case the high-priority HARQ-ACK does not have a corresponding PDCCH

First, it’s worth recalling that in Rel-16 NR, a UE is not expected to be dynamically scheduled with a PUCCH (or PUSCH) of low priority overlapping with a high priority PUCCH carrying only HARQ-ACK feedback without a corresponding PDCCH; this is captured in TS 38.213, Sec. 9.

When the high-priority HARQ-ACK does not have a corresponding PDCCH (i.e. SPS PDSCH), multiplexing of high-priority HARQ-ACK bits with low-priority HARQ-ACK bits is not preferred. This is because, in this case, there cannot be a dynamic indication/control regarding (i) the (high-priority) PUCCH resource selection, as the PUCCH resource(s) for SPS are only semi-statically configured via RRC as part of the corresponding SPS configuration, and (ii) the enabling/disabling of multiplexing as discussed in Sec 3.1.1.1. Indeed, without such control by the network, the high-priority HARQ-ACK could be impacted if multiplexed with low-priority HARQ-ACK.

Based on the above observations, the following is proposed:

Proposal 3.5: For the scenario where a PUCCH carrying high-priority HARQ-ACK overlaps with another PUCCH carrying low-priority HARQ-ACK: If the high-priority HARQ-ACK does not have a corresponding PDCCH, the multiplexing is not supported.

It’s worth noting that for the case where the high-priority HARQ-ACK does not have a corresponding PDCCH, (i.e. in case of SPS PDSCH(s)), there are two cases depending on whether the low-priority HARQ-ACK has a corresponding PDCCH or not. Specifically:

- Case 1: the low-priority HARQ-ACK does not have a corresponding PDCCH (i.e. only LP and HP HARQ-ACK of SPS PDSCH). For this case, one potential option would be to prioritize the high-priority SPS HARQ-ACK and drop the low-priority SPS HARQ-ACK, i.e. keep the Rel-16 behaviour.

- Case 2: the low-priority HARQ-ACK has a corresponding PDCCH. For this case, we foresee two main alternatives: (i) keep the current Rel-16 rule, i.e. the case is treated as an error case, (ii) simply drop the low-priority HARQ-ACK. We don’t have a strong preference for one alternative over the other one, particularly since a common aspect between these two alternatives is that multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK is not allowed (in case high-priority HARQ-ACK does not have a corresponding PDCCH).

#### 3.1.1.4 On separate encoding of high-priority and low-priority HARQ-ACKs

The discussion here concerns the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK when the total number of low-priority and high-priority HARQ-ACK is greater than 2 bits. The support of separate encoding was agreed in RAN1#104bis-e as follows:

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs. - FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s). - (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N. - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

In RAN1#105-e, the following related agreement was reached:

Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, - For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options: - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit. - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding. - For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1. - FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available.

Next, we discuss some of the open points that are listed under the above agreement.

When HP HARQ-ACK or LP HARQ-ACK is of 1 or 2 bits:

For such cases, Rel-15 repetition and simplex codes can be adopted for the encoding of 1-bit information and 2-bit information, respectively, which is Option 1 in the above agreement. Specifically, in case high-priority/low-priority HARQ-ACK is 1 bit, the existing 1-bit information encoding scheme in TS 38.212 Sec. 5.3.3.1 can be used to separately encode this HARQ-ACK. And in case high-priority/low-priority HARQ-ACK is 2 bits, the existing 2-bit information encoding scheme in TS 38.212 Sec. 5.3.3.2 can be used to separately encode this HARQ-ACK.

Proposal 3.6: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK where the high-priority or low-priority HARQ-ACK is 1-2 bits, adopt the following:

- In case HARQ-ACK is 1 bit: use the existing Rel-15 1-bit information encoding scheme in TS 38.212 Sec. 5.3.3.1 to encode this HARQ-ACK.

- In case HARQ-ACK is 2 bits: use the existing Rel-15 2-bit information encoding scheme in TS 38.212 Sec. 5.3.3.2 to encode this HARQ-ACK.

On dropping CSI:

In our view, CSI (including both part 1 and part 2, if any) should be dropped if the CSI would be multiplexed with a high-priority HARQ-ACK on the same PUCCH. Actually, this would lead to simplifying the related Rel-17 intra-UE specifications work.

Proposal 3.7: Confirm the RAN1#104bis-e Working Assumption to not support multiplexing of CSI (include part 1 and part 2, if any) and high-priority HARQ-ACK on PUCCH and thus to drop the CSI and prioritize the high-priority HARQ-ACK.

In the following, we discuss some of the design details for the multiplexing of high-priority and low-priority HARQ-ACKs given that separate encoding is adopted.

Rate matching output sequence length:

The Rel-15 rate matching operation and formulas for the cases where there is CSI part 2 are defined in TS 38.212. One part of Table 6.3.1.4.1-1 in TS 38.212 is copied below:

Table 3.1: Rate matching output sequence length (Table 6.3.1.4.1-1 in TS 38.212)

| UCI(s) for transmission on a PUCCH | UCI for encoding | Value of |
| --- | --- | --- |
| CSI (CSI of two parts) | CSI part 1 | EUCI=minEtot,OCSI-part1+L/RUCImax/Qm⋅Qm |
|  | CSI part 2 | EUCI=Etot-minEtot,OCSI-part1+L/RUCImax/Qm⋅Qm |
| HARQ-ACK, CSI (CSI of two parts) | HARQ-ACK, CSI part 1 | EUCI=minEtot,OACK+OCSI-part1+L/RUCImax/Qm⋅Qm |
|  | CSI part 2 | EUCI=Etot-minEtot,OACK+OCSI-part1+L/RUCImax/Qm⋅Qm |

The above operation can be leveraged/reused for the scenario where high-priority HARQ-ACK is multiplexed with low-priority HARQ-ACK. Specifically, the following rate matching formulas can be used:

Table 3.2: Rate matching output sequence length (based on Table 6.3.1.4.1-1 in TS 38.212)

| UCI(s) for transmission on a PUCCH | UCI for encoding | Value of |
| --- | --- | --- |
| HP HARQ-ACK, LP HARQ-ACK | HP HARQ-ACK | EUCI=minEtot,OHP ACK+L/RUCI, HPmax/Qm⋅Qm |
|  | LP HARQ-ACK | EUCI=Etot-minEtot,OHP ACK+L/RUCI, HPmax/Qm⋅Qm |

On the formulas provided in Table 3.2, it should be noted that OHP ACK denotes the high-priority HARQ-ACK payload size and RUCI, HPmax denotes the max code rate corresponding to the high-priority PUCCH resource that will be used for the multiplexing of high-priority and low-priority HARQ-ACKs. It’s worth recalling that  is the total rate matching output sequence length given by Table 6.3.1.4-1 in TS 38.212, L is number of CRC bits, Qm is the modulation order.

Proposal 3.8: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK, reuse Rel-15 operation for two UCI parts to determine the rate matching output sequence length for each of the high-priority HARQ-ACK and low-priority HARQ-ACK.

On the determination of maxCodeRate(s):

In RAN1-105-e, it was discussed how the maxCodeRate for each of high-priority HARQ-ACK and low-priority HARQ-ACK should be determined when these HARQ-ACKs are multiplexed. In that regard, two Options have been on the table and proposed by the Moderator (R1-2106063):

- Option 1: maxCodeRate can be separately configured for HP and LP HARQ-ACK in a PUCCH configuration if multiplexed in a PUCCH PF3/4 in R17. - A separate maxCodeRate for LP HARQ-ACK is configured for HP PUCCH format 3 and 4. - Option 2: No enhancement over Rel-16, i.e., maxCodeRate is configured per PF per PUCCH configuration.

In our view, an approach along the lines of Option 2 (as we understand it) should be enough as it would basically leverage the Rel-16 configuration/design. Specifically, we think that the maxCodeRate configured for the selected high-priority PUCCH resource (as part of the PUCCH configuration for high-priority) for multiplexing should be used as maxCodeRate for the high-priority HARQ-ACK and the maxCodeRate configured for the low-priority PUCCH resource (as part of the PUCCH configuration for low-priority), before resolving the overlap, could be used as maxCodeRate for low-priority HARQ-ACK.

It should be noted that when the low-priority PUCCH resource is of Format 0/1 (which corresponds to the case where the low-priority HARQ-ACK payload size is of 1 or 2 bits), there is no associated maxCodeRate. In this case, a simple rule would be to use the maxCodeRate corresponding to the low-priority PUCCH resource that would be determined as if the low-priority HARQ-ACK payload size is 3 bits.

We thus propose:

Proposal 3.9: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK resulting from the overlap between high-priority PUCCH resource and low-priority PUCCH resource:

- the maxCodeRate configured for the selected high-priority PUCCH resource for multiplexing should be used as maxCodeRate for the high-priority HARQ-ACK; and

- the maxCodeRate configured for the low-priority PUCCH resource could be used as maxCodeRate for low-priority HARQ-ACK.

  - If the low-priority PUCCH resource is with Format 0/1, use the maxCodeRate (for low-priority HARQ-ACK) associated to the low-priority PUCCH resource corresponding to a low-priority HARQ-ACK payload size of 3 bits.

Multiplexing of coded UCI bits to PUCCH:

The Rel-15 operation defined in TS 38.212 for PUCCH Format 3/4 can be reused as we don’t see any strong need to open the door for enhancements, especially that the existing operation is already fairly complex.

On the other hand, such ‘multiplexing’ operation was not considered for PUCCH Format 2. In our view, a simple ‘multiplexing’ of coded UCI bits should be adopted for Format 2. One possible approach would be to aggregate the coded high-priority HARQ-ACK bits and the coded low-priority HARQ-ACK bits. And, considering the sequence corresponding to the totality of aggregated coded HARQ-ACK bits, the mapping is done as if there is one UCI to be mapped to PUCCH resource with Format 2, i.e. use this aggregated coded HARQ-ACK bit sequence to follow the procedures described in Sec. 6.3.2.5 of TS 38.211.

Proposal 3.10: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK on PUCCH Format 3/4, adopt the Rel-15 operation of ‘multiplexing of coded UCI bits to PUCCH’ given in TS 38.212 Sec. 6.3.1.6.

Proposal 3.11: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK on PUCCH Format 2, adopt the following approach for mapping the separately coded bits to PUCCH:

- Aggregate the coded high-priority HARQ-ACK bits and the coded low-priority HARQ-ACK bits and apply the procedures described in Sec. 6.3.2.5 of TS 38.211 to this aggregated coded HARQ-ACK bit sequence.

All the above discussions assume that the high-priority PUCCH resource to be used for multiplexing has been selected. Note that this includes the selection/determination of the PUCCH resource set and the number of RBs. This aspect was discussed a bit in RAN1#105-e for the scenario under consideration here. It’s essential that both the UE and gNB have the same understanding regarding the selection of PUCCH resource set and number of RBs; otherwise, the gNB would not be able to correctly decode the multiplexed high-priority and low-priority HARQ-ACK, and this would impact the performance of high-priority bits. Therefore, it’s important to discuss this aspect.

Proposal 3.12: Further discuss the selection of PUCCH resource set and the number of RBs for the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK.

In the next subsection (3.1.1.5), we provide our view on the importance to avoid discrepancy between the UE and gNB on the selection of PUCCH resource set and number of RBs due to missing DCI(s) corresponding to low-priority HARQ-ACK.

#### 3.1.1.5 On how to avoid erroneous selection of PUCCH resource set and number of RBs for the multiplexed high- and low-priority HARQ-ACKs

First, it’s worth recalling that based on the existing procedure of PUCCH resource determination, when a UE has received the dedicated PUCCH resource configuration, the PUCCH resource determination for HARQ-ACK feedback can basically occur in 3 steps:

- UE selects the PUCCH resource set based on the number of UCI bits to be transmitted, i.e. the UCI payload size.

- DL assignment contains PUCCH resource indicator (PRI) field, and UE selects the PUCCH resource from the selected PUCCH resource set based on the PRI value.

- For some of the PUCCH formats (namely, formats 2 and 3), the UE determines the number of RBs used in the transmission based on the UCI payload and configured maximum code rate. The number of RBs is determined to be the smallest number of RBs for which the code rate is below the maximum code rate, capped by the number of RBs configured for the selected PUCCH resource.

Given its target reliability, it’s safe to assume that high-priority HARQ-ACK codebook (CB) size is determined sufficiently reliably. Hence, we focus on the low-priority HARQ-ACK errors in the multiplexing of low-priority and high-priority HARQ-ACK. In case of Type 2 CB, UE missing the last DL assignment(s) causes erroneous low-priority HARQ-ACK codebook size.

In the following, we assume that the PUCCH resource determination for multiplexing high- and low-priority HARQ-ACKs is done based on the existing procedure described above. It can be noticed that in the determination of PUCCH resource to be used, 2 steps out of 3 depend on the UCI payload size. Errors in low-priority HARQ-ACK codebook size determination may thus cause selection of different PUCCH resource set or use of smaller number of RBs for the multiplexed HARQ-ACKs feedback than what the gNB would expect.

Observation 3.1: Errors in low-priority HARQ-ACK codebook size determination may cause selection of different PUCCH resource set or use of smaller number of RBs for the multiplexed high-priority and low-priority HARQ-ACKs feedback than what gNB would expect.

To overcome such UE / gNB discrepancy on the determination of PUCCH resource and number of RBs for UCI containing both high-priority and low-priority HARQ-ACKs, one could rely on gNB blind detection where the gNB tries to detect PUCCH on multiple resources corresponding to different UCI sizes. However, this option increases gNB complexity as well as possibility for detection error. Note that it is desirable for URLLC communications to avoid or mitigate such additional error source and potential additional delay coming from multi-hypotheses PUCCH blind detection.

Another option to overcome the above issue is to allow the PUCCH resource set and/or the number of RBs to be determined at the UE at least partially based on some information indicated by the gNB for multiplexed high-priority and low-priority HARQ-ACKs. In that regard, in RAN1#105-e, the following options have been listed (R1-2106063):

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, further study the problem of ambiguity on LP HARQ-ACK existence or LP HARQ-ACK type-2 codebook size due to DCI mis-detection and the candidate options: - Option 1: Configure a dedicated PUCCH resource for HP+LP in the second PUCCH-Config - Option 2: PRI+x in the HP DCI is used to implicitly determine an extended PUCCH resource - Option 3a: The LP type 2 codebook size is quantized/rounded up to a nearest reference size. FFS reference size granularity. - Option 3b: Configuration of semi-static size reservation for LP HARQ-ACK payload is provided by RRC. LP HARQ-ACK semi-static size reservation is used instead of determined LP HARQ-ACK codebook size when selecting the PUCCH resource set. - Option 4: Additional DCI field in DCI corresponding HP HARQ-ACK or HP PUSCH for determining the number of LP HARQ-ACK bits multiplexed on PUCCH/PUSCH. - Option 5: Provide indication on at least the number of RBs and/or PUCCH resource set index to be used in the PUCCH transmission, where the indication is included in the high-priority DL assignment. - Other solutions are not precluded. - FFS ambiguity cases.

It is not clear how Option 1 would help solving the issue of DCI miss-detection (corresponding to low-priority HARQ-ACK) – unless the intention is to have a single PUCCH resource dedicated for the multiplexing, in which case this would be restrictive. On the other hand, it’s not clear how Option 2 would work at all. Option 3a is somewhat similar to Option 3b, but is slightly more complicated and still has the issue of DCI miss-detection if the miss detection would lead to a change from target reference size to a next smaller reference size. Therefore, Option 3b is clearly preferred over Option 3a. Options 4 and 5 are clear in their operation but they require additional indication in DCI.

Hence, among the above options, we see that the following options could be further considered for the specific indication and/or determination related to the PUCCH resource set index and number of RBs:

- Alt.1 (corresponds to Option 3b): Configuration of semi-static size reservation for low-priority HARQ-ACK payload is provided by RRC

  - Low-priority HARQ-ACK semi-static size reservation is used instead of determined low-priority HARQ-ACK codebook size when selecting the PUCCH resource set.

- Alt.2 (corresponds to Option 4): The indication and determination are defined by combining dynamic indication for enabling/disabling low-priority HARQ-ACK multiplexing and for low-priority HARQ-ACK codebook size.

  - Different values in a DCI field are used to indicate either (i) to not multiplex the low-priority HARQ-ACK or (ii) to multiplex (at least part of) the low priority HARQ-ACK and the corresponding total DAI or CB size for low-priority HARQ-ACK to avoid discrepancy on the low-priority HARQ-ACK codebook size.

- Alt.3 (corresponds to Option 5): The indication and determination are defining the number of RBs and/or PUCCH resource set index to be used in the PUCCH transmission, where the indication is included in the high-priority DL assignment.

  - The UE then uses this indication to check whether there is any discrepancy compared to the PUCCH resource set index and/or number of RBs it has determined, and, if needed, to determine the correct values of these parameters.

Between the above alternatives/options, we have a slight preference towards Option 3b / At.1 as it doesn’t require adding a field in the DCI, while it should be noted that this alternative is less flexible compared to  Option 4 / Alt.2 and Option 5 / Alt.3.

Proposal 3.13: To avoid discrepancy between the UE and the gNB on the determination of PUCCH resource set and number of RBs for UCI containing multiplexed high-priority and low-priority HARQ-ACKs, support Option 3b:

- Option 3b: Configuration of semi-static size reservation for low-priority HARQ-ACK payload is provided by RRC.

Otherwise, as a second preference, support either Option 4 or Option 5:

- Option 4: The indication and determination are defined by combining dynamic indication for enabling/disabling low-priority HARQ-ACK multiplexing and for low-priority HARQ-ACK codebook size.

- Option 5: The indication and determination are defining the number of RBs and/or PUCCH resource set index to be used in the PUCCH transmission, where the indication is included in the high-priority DL assignment.

#### 3.1.1.6 Whether to support multiplexing for all the combinations of HARQ-ACK codebook types and priorities

Considering the different types and priorities of HARQ-ACK codebooks, there are essentially four cases to consider:

- Case a: Multiplexing of low-priority Type-1 HARQ-ACK and high-priority Type-1 HARQ-ACK

- Case b: Multiplexing of low-priority Type-2 HARQ-ACK and high-priority Type-2 HARQ-ACK

- Case c: Multiplexing of high-priority Type-1 HARQ-ACK and low-priority Type-2 HARQ-ACK

- Case d: Multiplexing of high-priority Type-2 HARQ-ACK and low-priority Type-1 HARQ-ACK

In principle, we think that multiplexing could be supported for all four cases. If there would be a need to prioritize some cases for the discussions, in our view Cases (a) and (b) could be discussed first as each one involves overlap of same HARQ-ACK codebook type. Case (c) could also be of interest as, generally speaking, operating with Type-1 HARQ-ACK codebook would typically be better from reliability perspective compared to Type-2 HARQ-ACK codebook (due to DCI miss-detection). On the other hand, Case (d) could be treated with lower priority in terms of discussions.

Observation 3.2: In principle, multiplexing could be supported for all four possible combinations of high-priority HARQ-ACK overlapping with low-priority HARQ-ACK considering Type-1 and Type-2 codebooks. If there would be a need to prioritize some cases for the discussions, the cases involving same HARQ-ACK codebook type (Type-1/Type-2) could be discussed first. The case corresponding to multiplexing high-priority Type-1 HARQ-ACK and low-priority Type-2 HARQ-ACK could also be of interest.

### 3.1.2 PUCCH with high-priority SR vs. PUCCH with low-priority HARQ-ACK

In Rel-16 NR, the following was agreed in RAN1#100b-e regarding the interaction between SR priority and PUCCH configuration priority:

Agreements: - SR priority comes from phy-PriorityIndex-r16 in SchedulingRequestResourceConfig. If not configured, SR is treated as low priority (index 0). - In Rel-16, if a UE is configured with one HARQ codebook, the HARQ-ACK codebook is considered as low priority. If a PUCCH-Config is provided with a subslotLengthFor PUCCH-r16, the PUCCH resource corresponding to any SR or CSI configuration with the same priority as the PUCCH-Config, should be confined within the sub-slot associated to the PUCCH-Config.

Note that the RAN1#100b-e agreement copied above was essentially reached to avoid reverting a previous agreement that SR priority is determined by SchedulingRequestResourceConfig and thus that this priority doesn’t necessarily need to be tied to the priority of the PUCCH configuration in which the PUCCH resource of SR is configured. In this regard, there are two cases that could be of interest for the discussion here:

- Case 1: The PUCCH resource for high-priority SR belongs to the PUCCH configuration of high priority.

- Case 2: The PUCCH resource for high-priority SR belongs to the PUCCH configuration of low priority. Note that this case is valid even if there is a single PUCCH configuration configured.

In the following, for simplicity, our discussion assumes the first case in here, which would be the typical case.

It’s worth recalling that if both HARQ-ACK and SR have the same (high/low) PHY priority, the Rel-16 rule consists of re-using the Rel-15 rule which can be briefly described as follows:

- SR and HARQ-ACK are multiplexed on the HARQ-ACK resource in the following cases: (i) HARQ-ACK is with F0 and SR with F0, and (ii) HARQ-ACK is with F2/F3/F4 and SR with F0/F1.

- For the case where HARQ-ACK is with F1 and SR with F0, HARQ-ACK is prioritized and SR is dropped.

- For the case where HARQ-ACK is with F1 and SR with F1, HARQ-ACK is transmitted on the SR resource if SR is positive whereas HARQ-ACK is transmitted on the HARQ-ACK resource when SR is negative.

The support of multiplexing of high-priority SR and low-priority HARQ-ACK was agreed in RAN1#102-e, at least for some of the PUCCH formats combinations. Reusing the above handling/multiplexing rules for the scenario where high-priority SR overlaps with low-priority HARQ-ACK may impact the reliability of SR mainly because SR and HARQ-ACK will be multiplexed on the PUCCH HARQ-ACK resource (which is here of low priority) for several of the PUCCH format combinations. And this could impact the latency and reliability of SR.

Based on this observation, to allow multiplexing of high-priority SR and low-priority HARQ-ACK without impacting the SR performance, in our view the multiplexed UCI should be transmitted on the PUCCH resource for SR at least when SR is positive.

It’s worth noting that in RAN1#104-e, some ‘agreements’ were made (see FL summary in R1-2101842) regarding the multiplexing of high-priority SR and low-priority HARQ-ACK, but only listing the various options proposed by the companies for the multiplexing without further down-selection. In RAN1#104bis-e, there had been further discussions on these scenarios but without reaching any conclusion or agreement. Some of the listed options for different cases are copied below (see FL summary R1-2103868).

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0 - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - Opt.1d: with a power boost - …

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1 - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - …

When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0 - … - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: No enhancement over Rel-16. - …

In addition, in RAN1#105-e, these scenarios were discussed a bit considering the corresponding listed options but without reaching any outcome.

Based on the above discussion and observations, in the following table we summarize our view on the handling rules that could be used for a high-priority SR overlapping with a low-priority HARQ-ACK.

Table 3.3: Handling rules for high-priority SR overlapping with low-priority HARQ-ACK

|  | LP HARQ-ACK with F0 | LP HARQ-ACK with F1 | LP HARQ-ACK with F2/3/4 |
| --- | --- | --- | --- |
| HP SR with F0 | Opt.1b: Transmit both on the SR resource if SR is positive; and the HARQ-ACK on the HARQ-ACK resource if SR is negative. Opt.1c: Transmit both on the SR resource (regardless whether SR is positive or negative) | Opt.1b: Transmit both on the SR resource if SR is positive; and the HARQ-ACK on the HARQ-ACK resource if SR is negative. Opt.1c: Transmit both on the SR resource (regardless whether SR is positive or negative) | If SR is positive, transmit SR on the SR resource and drop HARQ-ACK; if SR is negative, transmit HARQ-ACK-only on the HARQ-ACK resource. (Rel-16 rule) |
| HP SR with F1 | Opt.3: Transmit HARQ-ACK on the SR resource if SR is positive; and transmit the HARQ-ACK on HARQ-ACK resource when SR is negative. | Transmit HARQ-ACK on the SR resource if SR is positive; and transmit HARQ-ACK on the HARQ-ACK resource when SR is negative. (Rel-15 rule for same priority case) |  |

Proposal 3.14: For the scenario where a PUCCH carrying high-priority SR overlaps with a PUCCH carrying low-priority HARQ-ACK:

- If SR is with F0 and HARQ-ACK is with F0/F1, adopt one of the following options:

  - Opt.1b: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. For negative SR, the UE transmits only HARQ-ACK on the HARQ-ACK resource.

  - Opt.1c: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

- If SR is with F1 and HARQ-ACK is with F0/F1, adopt Opt.3: Transmit HARQ-ACK on the SR resource if SR is positive; and transmit HARQ-ACK on the HARQ-ACK resource when SR is negative.

- If SR is with F0/F1 and HARQ-ACK is with F2/F3/F4: If SR is positive, transmit SR on the SR resource and drop HARQ-ACK; if SR is negative, transmit HARQ-ACK only on the HARQ-ACK resource.

### 3.1.3 Overlapping between high- and low-priority PUCCHs where one is across sub-slots

In Rel-16 NR, it was specified that a UE could be configured with two PUCCH configurations, where e.g. one configuration is slot based and the other one is sub-slot based. Typically, the slot-based configuration is used for low-priority traffic and the sub-slot-based configuration is used for the high-priority traffic; although this would be the typical case, there is no such restriction in the specs. This results in scenarios where at least one low-priority PUCCH that crosses sub-slot boundary overlaps with one or multiple high-priority PUCCHs confined within a sub-slot. An example of such scenarios is illustrated in Figure 3.1.

Figure 3.1: Example illustrating the scenario where a high-priority channel confined in a sub-slot overlaps with a low-priority channel that crosses the sub-slot boundary.

In RAN1#103-e, the high-level support of multiplexing for these scenarios was agreed, but all the related details are still FFS. A rule should be designed to handle these scenarios, where it’s important that such a rule protects and avoids any dropping of the high-priority UCI. In this regard, the following can be observed:

- Multiplexing, when feasible/allowed, should be done only on a high-priority PUCCH resource, because, as previously discussed, using a low-priority PUCCH resource for multiplexing could impact the latency and reliability of the high-priority UCI and may lead to dropping the high-priority HARQ-ACK/UCI in some cases. On the other hand, when using a high-priority PUCCH resource, the reliability is preserved as this resource belongs to the PUCCH configurations with high priority. And the latency is also not impacted since the selected high-priority PUCCH resource for multiplexing would be confined within the sub-slot; e.g. sub-slot #0 in the example of Figure 3.1. It’s worth noting that the use of a high-priority PUCCH resource for multiplexing is so far only explicitly agreed (in RAN1#104-e) for the scenario where a high-priority HARQ-ACK overlaps with a low-priority HARQ-ACK and the total payload size is greater than 2. As discussed in other (sub)sections, we believe that the same principle should also be agreed for a total payload equals to 2 as well as for the scenario where high-priority SR overlapping with low-priority HARQ-ACK.

- The rule for handling the scenarios here should be simple and in line with the rules that will be adopted for the scenarios with more than two high-priority and low-priority overlapping channels.

Based on the above observations, one potential rule for handling the scenarios here, in such a way that the high-priority channel(s) is not impacted, is as follows:

- Multiplexing of low-priority PUCCH and high-priority PUCCH, when feasible, is allowed only if this multiplexing is done on a high-priority PUCCH resource. In addition:

  - The UE does not expect an overlap between the resulting PUCCH resource to be used for multiplexing and another high-priority PUCCH;

  - if the resulting PUCCH resource overlaps with a low-priority PUCCH (instead of a high-priority PUCCH), the low-priority PUCCH can then be simply dropped.

Proposal 3.15: For handling the scenarios where a PUCCH of a given priority crosses the sub-slot boundary of the PUCCH config of another priority and overlaps with a PUCCH of another priority, adopt the following procedure:

- Multiplexing of low-priority PUCCH and high-priority PUCCH, is allowed only if this multiplexing is done on a high-priority PUCCH resource. In addition:

  - UE does not expect an overlap between the resulting PUCCH resource to be used for multiplexing and another high-priority PUCCH;

  - and if the resulting PUCCH resource overlaps with a low-priority PUCCH, the low-priority PUCCH is then dropped.

  - Additional conditions are FFS.

### 3.1.4 More than two overlapping high-priority and low-priority PUCCHs

In addition to the two-channel scenarios, there are scenarios where there could be more than two overlapping channels of different priorities, where here we consider that there is no PUSCH(s) in the group of overlapping channels. Specifically, some of these scenarios are listed below:

- PUCCH with high-priority UCI(s) vs. at least two non-overlapping PUCCHs with low-priority UCI(s)

- PUCCH with low-priority UCI(s) vs. at least two non-overlapping PUCCHs with high-priority UCI(s)

An example of such scenarios is illustrated in Figure 3.2.

Figure 3.2: Example illustrating the scenario with more than two high-priority and low-priority overlapping channels.

In RAN1#103-e, the high-level support of multiplexing for these scenarios was agreed, but all the related details are still FFS. For such scenarios, it should be discussed how to handle the group of overlapping PUCCHs. In general, the handling approach to be designed for these scenarios should be simple so that the processing of high-priority channel(s) is not impacted. Further, the handling approach should not impact the high-priority channels. Note that the discussion here is under the assumption that, the multiplexing (if any) between overlapping channels of the same priority is already done (i.e. for each of high-priority PUCCHs and low-priority PUCCHs); so, we only consider the resulting channels after multiplexing of channels with the same priority.

It should be noted that there had been some discussions on these scenarios in RAN1#105-e, where two main options have been listed (R1-2106063):

For handling the scenarios with more than two overlapping PUCCHs of different priorities, consider the following options: - Option 1: Use R15 multiplexing as baseline, i.e. a single checking/multiplexing step among all channels. - Option 2: Allow a single checking/multiplexing step between channels of different priorities after multiplexing (if any) between overlapping channels of the same priority is already done. - Other options are not excluded. - Note: the candidate cases should be identified first.

Option 1 is not fully clear, especially since it says “Use R15 multiplexing as baseline” but also “i.e. a single checking/multiplexing step among all channels”.

We think that Option 2 is the simpler approach. Specifically, this handling approach should be further clarified as follows:

- Allow a single checking/multiplexing step between channels of different priorities (where the earliest high-priority PUCCH could be considered), where in case of multiplexing:

  - The UE does not expect an overlap between the resulting resource to be used for multiplexing and another high-priority PUCCH;

  - and if the resulting PUCCH resource overlaps with a low-priority PUCCH (instead of a high-priority PUCCH), the low-priority PUCCH can then be simply dropped.

Proposal 3.16: For handling the scenarios with more than two overlapping PUCCHs of different priorities, adopt the following procedure:

- Allow a single checking/multiplexing step between channels of different priorities, where in case multiplexing is feasible:

  - UE does not expect an overlap between the resulting resource to be used for multiplexing and a high-priority PUCCH;

  - and if the resulting PUCCH resource overlaps with a low-priority PUCCH, the low-priority PUCCH is then dropped.

## 3.2	Control channel vs. data channel

The discussion in this section is focused on the scenarios where the overlapping PUSCH(s) and PUCCH(s) are of different priorities.

### 3.2.1 HARQ-ACK vs. PUSCH with different priorities

Supporting multiplexing HARQ-ACK in PUSCH of different priorities was agreed in RAN1#102-e meeting.

RAN1#102 e-meeting agreements (not a full list): Support multiplexing for following scenarios in R17: - Multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). - Multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only) - Multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. - Multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. For the above multiplexing scenarios, - Support separate configurations of at least beta-offset values (FFS for alpha) for multiplexing with different priority combinations. - FFS for other separate configurations. - FFS: value range of beta-offset (e.g. <1). …..

It has been further discussed in all the following up RAN1 e-meetings. In the following we discuss our views on different open issues related to multiplex HARQ-ACK on PUSCH of different priorities.

#### 3.2.1.1 Explicit indication for multiplexing

For multiplexing HARQ-ACK in PUSCH of different priorities, the following agreements were made especially considering supporting gNB of enabling/disabling the multiplexing process:

Agreements (RAN1#103-e meeting): - For HARQ-ACK multiplexing on PUSCH of different priority in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration, beta_offset=0 - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI. Agreements (RAN1#104-e meeting): For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1. - FFS value(s) - FFS to additionally support beta-offset =0 or a value disabling the multiplexing - Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16)

This was discussed in previous RAN1#105-e meeting, while no agreement achieved, and the issue is still open. Similar as discussed in Section 3.1.1.1, it is of importance to enable gNB to have the capability of dynamically enabling/disabling multiplexing HARQ-ACK in PUSCH of different priorities on top of RRC configuration, especially considering that beta-offset is already part of the DCI. In this way the possible intolerable performance degradation in terms of latency and/or reliability on the high-priority channel can be avoided since the multiplexing decision is made by gNB. Taking the overlapping of PUCCH carrying high-priority HARQ-ACK and low-priority DG PUSCH as an example, depending on the potential latency and/or reliability impacts on the high-priority HARQ-ACK feedback, gNB can dynamically determine and inform UE via DCI whether to multiplex high-priority HARQ-ACK in the overlapping low-priority PUSCH or not. The same or similar approach can be used to guarantee the stringent performance requirements of high-priority PUSCH if it is overlapping with a PUCCH carrying low-priority HARQ-ACK bits. For example, if the multiplexed low-priority HARQ-ACK would occupy a large portion of the PUSCH resource and there is a risk that the high-priority PUSCH is not able to be decoded successfully, gNB could disable the multiplexing. In addition, allowing gNB dynamically enabling/disabling multiplexing via DCI signalling would ease the operation and significantly reduce specification effort also in other aspects as well. As one example, there is no need to specify bundling/compression of low-priority HARQ-ACK information.

For the scenarios where DG PUSCH is involved, similar as already specified in TS 38.213, such indication can be carried over DCI scheduling the UL transmission via different values of beta-offset. As one example, when the decision of gNB is not to multiplex, the value of beta-offset can be set to “0”. While in case a positive decision of multiplexing is made, a non-zero beta-offset can be included in DCI scheduling DG PUSCH. However, in case with CG PUSCH, it could be a bit more complicated since the gNB cannot keep the control dynamically by taking into account the potential performance impacts on high-priority channels. Moreover, depending on UE implementation, when UE making multiplexing decision, the UE might not be aware of a later coming PUCCH carrying HARQ-ACK. And therefore, it should be further studied whether to support multiplexing UCI on CG PUSCH of different priorities. Based on this, we propose:

Proposal 3.17: For the scenarios of multiplexing HARQ-ACK bits in DG PUSCH of different priorities, gNB dynamically indicates via beta-offset in the corresponding scheduling DCI whether to multiplex HARQ-ACK in PUSCH of different PHY priority or not (e.g. beta-offset = 0). FFS whether to support multiplexing of HARQ-ACK bits on CG PUSCH of a different PHY priority.

#### 3.2.1.2 Separate configurations of multiplexing parameters (i.e. beta-offset and alpha)

To avoid any potential intolerable performance degradation on the high-priority channels, separate configurations of beta-offset for multiplexing HARQ-ACK in a PUSCH of different priorities were agreed in RAN1#102 e-meeting. For example, when high-priority PUSCH is overlapping with PUCCH carrying HARQ-ACK bits:

- in case the PUCCH carrying low-priority HARQ-ACK, a smaller value of beta-offset (smaller than 1 for example) may be indicated for the purpose of limiting the maximal number of REs occupied by the low-priority HARQ-ACK bits. As discussed above, when gNB determines that it is preferred not to multiplex any low-priority HARQ-ACK in high-priority PUSCH, gNB can indicate a beta-offset value as “0”. In this way, gNB can dynamically indicate whether multiplexing is allowed or not;

- in case the PUCCH carrying high-priority HARQ-ACK, the Rel-16 way of multiplexing with indicated (Rel-15) beta-offset can be reused.

Similarly, for low-priority PUSCH, there could be two sets of beta-offset values for multiplexing high-priority and low-priority HARQ-ACK. With separate configurations, high-priority channels can be better protected in terms of resource usage. In Rel-16, one set of beta-offset values is sufficient for multiplexing HARQ-ACK on PUSCH of the same priority. And hence, in Rel-17 it should be sufficient to specify two additional sets of beta-offset values for multiplexing HARQ-ACK on PUSCH of different priorities. As a result, three sets of beta-offset values are required considering all the multiplexing scenarios including:

- multiplexing of HARQ-ACK and PUSCH of the same priority using the existing set of beta-offset values (i.e. Rel-16 operation);

- multiplexing low-priority HARQ-ACK on high-priority PUSCH using a new dedicated set of beta-offset values;

- multiplexing high-priority HARQ-ACK on low-priority PUSCH using another new dedicated set of beta-offset values.

This issue was discussed in the latest RAN1#104bis and RAN1#105 e-meetings. Based on the Feature Lead summary [R1-2106063], different options are for further discussion (mainly Option 1 and Option 2):

In NR Rel-17, consider the two options below: - Option 1: Up to 3 sets of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases: - Multiplexing HARQ-ACK on the PUSCH with same priority - Multiplexing LP HARQ-ACK on HP PUSCH - Multiplexing HP HARQ-ACK on LP PUSCH - Option 2: Up to 4 sets of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases: - Multiplexing LP HARQ-ACK/UCI on LP PUSCH - Multiplexing LP HARQ-ACK/UCI on HP PUSCH - Multiplexing LP HARQ-ACK on HP PUSCH - Multiplexing HP HARQ-ACK on LP PUSCH - Option 3: Up to 4 groups of beta offset set can be configured to the UE to indicate separate beta_offset values for the following cases: - Group 1: beta Offsets sets for LP UCIs (all UCIs are at LP) over LP PUSCH as in the Rel-16 legacy design - Group 2: beta Offsets sets for HP UCIs (all UCIs are at HP) over HP PUSCH as in the Rel-16 legacy design - Group 3: beta Offsets sets for UCI(s) with at least one UCI at HP over LP PUSCH - Group 4: beta Offsets sets for UCI(s) with at least one UCI at LP over HP PUSCH

We do not see the motivation to specify two different sets of beta-offset values for the case of multiplexing HARQ-ACK on PUSCH of the same priority and three sets of beta-offset values are sufficient.

The beta-offset indicator field in DCI points to either one of the three sets of beta-offset values (if only high-priority or low-priority HARQ-ACK bits to be multiplexed on PUSCH) or two sets of beta-offset values (if both high-priority and low-priority HARQ-ACK bits to be multiplexed on PUSCH) without increasing the corresponding bitwidth in DCI. For the case of both low- and high-priority HARQ-ACK bits are multiplexed on PUSCH, the applicable beta-offset values for multiplexing low- and high-priority HARQ-ACK bits are independently applied for the low-priority HARQ-ACK and high-priority HARQ-ACK information. As already discussed above, in order for the gNB to have the flexibility of enabling/disabling multiplexing HARQ-ACK in PUSCH, it is a preferred way to introduce new value of beta-offset, e.g. value “0” as the indication of no multiplexing.

Based on the discussion above, we propose:

Proposal 3.18: For the scenarios of multiplexing HARQ-ACK bits in PUSCH of different priorities, RAN1 should specify:

- three sets of beta-offset values (i.e. Option 1) for:

  - multiplexing HARQ-ACK bits on the PUSCH with the same priority (specified already);

  - multiplexing low-priority HARQ-ACK bits on high-priority PUSCH;

  - multiplexing high-priority HARQ-ACK bits on low-priority PUSCH;

- for multiplexing of both low-priority and high-priority HARQ-ACK bits, the beta-offset indicator field in the DCI points to the respective two sets of beta-offset values to be applied respectively for low- and high-priority HARQ-ACK.

Another open issue related to the parameters for multiplexing is the configuration of the scaling factor “alpha”, which limits the total number of resource elements assigned to UCI on PUSCH. To be more specific, the issue is whether it is essential to support separate configurations of alpha or not. In principle, separate configurations of alpha can be supported in Rel-17 with the similar arguments as supporting separate configuration of beta-offset. However, since separate configurations of beta-offset is supported, it becomes unclear how much benefits we can get with separately configured alphas. Following the similar procedure specified in Section 6.3.2.4.1.1 of TS 38.212 (rate matching of HARQ-ACK bits on PUSCH), the number of REs occupied by HARQ-ACK bits is determined by both alpha and beta-offset. With the properly configured beta-offset, we do not see the necessity to introduce another set of alpha values to limit the maximum number of REs for HARQ-ACK bits transmission.

Proposal 3.19: For the scenarios of multiplexing HARQ-ACK bits in a PUSCH of different priorities, do not support separate configurations of the scaling factor “alpha”.

#### 3.2.1.3 Multiplexing conditions

The multiplexing conditions, including the multiplexing timeline, between the overlapping HARQ-ACK and PUSCH of different priorities have been discussed in previous RAN1 e-meetings. In RAN1-104 e-meeting, reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline has been agreed as one working assumption.

Working assumption: (RAN1#104-e meeting,): Reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities - FFS whether or not to specify a different behavior than Rel-15 when the timeline requirements are not met

We do not see any technical benefits to specify new multiplexing timeline and hence we support the working assumption. Considering the FFS point, in Rel-15 it is considered as error case when the timeline requirements are not met. In Rel-16, intra-UE PHY prioritization has been specified with the introduction of PHY priority of different channels and related PHY prioritization process in case the overlapping channels are of different PHY priorities. In Rel-17, in case the multiplexing timeline conditions are not fulfilled, it should not be treated as error case anymore, instead the Rel-16 prioritization procedure should be adopted. That is to say, when multiplexing timeline condition is fulfilled, the high-priority/low-priority HARQ-ACK can be multiplexed in low-priority/high-priority PUSCH. When the multiplexing timeline condition is not fulfilled, Rel-16 prioritization rule can be applied, i.e. the transmission of the low-priority channel(s) is cancelled.

On top of the timeline conditions, performance impacts on high-priority channels should be considered as well. However, as discussed earlier, gNB can make the decision of multiplexing or not based on the potential performance impacts on the high-priority channels and send explicitly indication to UE via different values of beta-offset. Therefore, in this way, UE only multiplexes HARQ-ACK bits in the overlapping PUSCH of different priorities when a non-zero beta-offset is received. Otherwise Rel-16 prioritization rule is applied.

Based on the discussions above about multiplexing conditions, we have the following proposal:

Proposal 3.20: For the scenario of the multiplexing between HARQ-ACK and PUSCH with different priorities,

- RAN1 should confirm the working assumption of reusing Rel-15 timeline conditions for multiplexing HARQ-ACK and PUSCH.

- If multiplexing timeline conditions are not met or no multiplexing is indicated from gNB, Rel-16 prioritization rule should be applied (i.e. the transmission of the low-priority channel(s) is cancelled).

#### 3.2.1.4 Separate coding of UCIs with different priorities

Separate coding of multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK was agreed in RAN1#104bis-e meeting:

Agreements (RAN1#104bis-e meeting): For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs. - It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

One aspect from the agreement is about the number of encoding chains for all UCI multiplexing combinations. Our understanding of the intention is to not increase the total number of encoding chains. From Rel-15, different UCIs, i.e. HARQ-ACK, CSI Part 1 and CSI part 2, are separately coded with 3 encoders respectively when they are multiplexing into one PUSCH.

The most challenging scenario is multiplexing UCIs into low-priority PUSCH where the overlapping PUCCHs are supposed to carry all UCIs including high-priority HARQ-ACK, low-priority HARQ-ACK, CSI part 1 and CSI part 2. For this scenario, the issue is that, with separate coding, not all UCIs can be multiplexed on the PUSCH if keeping the total number of encoding chains as 3. During email discussion in RAN1#105 e-meeting, there has been draft proposals with options of potential joint coding of different UCIs, for example low-priority HARQ-ACK bits jointly coded with low-priority CSI part 1. In our view, this should not be the direction for further discussion. With joint coding, significant efforts and time are needed to study all the details for example how to determine the code rate, applicable beta-offset etc. and clearly this is not preferred especially considering the overall Rel-17 timeframe. More importantly, in this example since both UCIs (to be jointed coded) are with low priority, we do not see any big issue to drop for example low-priority CSI. In addition, separate coding is fully aligned with previous RAN1 agreements and a single solution (i.e. separate coding) is much simpler. From coding method point of view, it is worth to point out that the currently specified coding methods in TS 38.212 are appliable to all different UCI sizes, and they are independent of the UCI types. And hence it is not proper to say “coding method for HARQ-ACK” for example as in some discussion round in RAN1#105-e meeting. We do not see any need to discuss different coding methods for UCIs and the coding methods specified in TS 38.212 are sufficient. Based on this, we propose:

Proposal 3.21: For the scenario of the multiplexing between HARQ-ACK and PUSCH with different priorities, RAN1 should not support joint coding of different UCI types, for example low-priority HARQ-ACK and CSI.

In our view, for the scenario of multiplexing high-priority HARQ-ACK, low-priority HARQ-ACK and CSI (both part 1 and part 2) into a low-priority PUSCH, dropping low-priority CSI is a preferred way to go. Different options has been discussed in RAN1#105-e meeting in order to fulfill the requirement on the total number of encoding chains. Considering dropping CSI, the following two options can be discussed further:

- Option 1: Dropping entire CSI (including both part 1 and part2, if exist) if CSI would multiplex into a low-priority PUSCH where both high-priority HARQ-ACK and low-priority HARQ-ACK will be multiplexed.

- Option 2: Dropping CSI part 2 only. Then the 3 encoders can be used for high-priority HARQ-ACK, low-priority HARQ-ACK and CSI part 1 respectively.

Both options are feasible. Considering better alignment with the agreed working assumption from RAN1#104bis-e meeting for the multiplexing on PUCCH,

Agreements (RAN1#104bis-e meeting): (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

Option 1 is slightly preferred. Therefore, we suggest adopting the following proposal:

Proposal 3.22: For the scenario where high-priority HARQ-ACK bits, low-priority HARQ-ACK bits and CSI would be multiplexed into a low-priority PUSCH, drop CSI (including part 1 and part 2, if exist).

Considering the scenario of supporting multiplexing HARQ-ACK into PUSCH of different priorities and with the assumption that CSI carried by PUCCH (treated as low-priority as agreed already in RAN1#98bis during Rel-16 time frame) is not multiplexed into high-priority PUSCH, then the number of encoding chains is not a problem for high-priority PUSCH without CSI since only two encoding chains are needed for separate coding two HARQ-ACKs: one for coding high-priority HARQ-ACK bits and another one for coding low-priority HARQ-ACK bits. On the other side, for the scenario of multiplexing both high-priority HARQ-ACK and low-priority HARQ-ACK into a high-priority PUSCH carrying CSI (both CSI part 1 and CSI part 2 for example), CSI should be treated as high-priority, i.e. the same priority as PUSCH. Following the same principle of separate coding discussed above, low priority HARQ-ACK should be dropped due to the limitation from the number of encoding chains and clearly, we should not drop the high-priority CSI in this scenario.

Observation 3.3: For the scenario where multiplexing both high-priority HARQ-ACK bits and low-priority HARQ-ACK bits into a high-priority PUSCH without CSI, the number of encoding chains is sufficient.

Proposal 3.23: For the scenario where both high-priority HARQ-ACK bits and low-priority HARQ-ACK bits would be multiplexed into a high-priority PUSCH carrying CSI, drop low-priority HARQ-ACK.

In short, when multiplexing UCIs into PUSCH of different priorities, depending on the scenario, either low-priority CSI or low-priority HARQ-ACK will be dropped when the available number of encoding chains is not sufficient. The 3 encoding chains can be used to encode the UCIs to be multiplexed into PUSCH following the same/similar procedure as specified in TS 38.212. As to rate mapping and RE mapping, individual procedure has been specified for HARQ-ACK, CSI part 1 and CSI part 2. Considering the new multiplexing scenarios supported in Rel-17, the same procedure should be reused as a baseline, but can be used for different UCIs. For example, in the scenario of multiplexing both high-priority HARQ-ACK bits and low-priority HARQ-ACK bits into a PUSCH without CSI, the Rel-15 HARQ-ACK process of rate matching and RE mapping can be used for multiplexing high-priority HARQ-ACK based on the corresponding beta-offset. Taking UCI encoded by Polar code as example, for high-priority HARQ-ACK transmission on PUSCH not using repetition type B with UL-SCH, the number of coded modulation symbols per layer for high-priority HARQ-ACK transmission can be determined as follows (based on 6.3.2.4.1.1 of TS 38.212):

QHP_ACK'=minOHP_ACK+LHP_ACK⋅βoffset_HP_ACKPUSCH⋅l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α⋅l=l0Nsymb,allPUSCH-1MscUCIl

where OHP_ACK denotes the number of high-priority HARQ-ACK bits, if OHP_ACK≥360, LHP_ACK=11; otherwise LHP_ACK is the number of CRC bits for high-priority HARQ-ACK determined according to Clause 6.3.1.2.1; and βoffset_HP_ACKPUSCH denotes the beta-offset value for multiplexing high-priority HARQ-ACK, and the rest parameters are the same as in 6.3.2.4.1.1 of TS 38.212. Rel-15 HARQ-ACK RE mapping can be reused for high-priority HARQ-ACK.

Similarly, Rel-15 CSI part 1 process of rate matching and RE mapping can be used for low-priority HARQ-ACK, for example the number of coded modulation symbols per layer for low-priority HARQ-ACK transmission can be determined as follows (based on 6.3.2.4.1.2 in TS 38.212):

QLP_ACK'=minOLP_ACK+LLP_ACK∙βoffset_LP_ACKPUSCH∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=0Nsymb,allPUSCH-1MscUCIl-QHP_ACK'

where OLP_ACK denotes the number of low-priority HARQ-ACK bits, if OLP_ACK≥360, LLP_ACK=11; otherwise LLP_ACK is the number of CRC bits for low-priority HARQ-ACK determined according to Clause 6.3.1.2.1; and βoffset_LP_ACKPUSCH denotes the beta-offset value for multiplexing low-priority HARQ-ACK, and the rest parameters are the same as in 6.3.2.4.1.2 of TS 38.212. Rel-15 CSI part 1 RE mapping can be reused for low-priority HARQ-ACK.

In this way, no need for RAN1 to define new schemes for rate matching and RE matching, but rather applying/reusing the specified procedure for the UCIs to be multiplexed into PUSCH.

Proposal 3.24: For the scenarios where both high-priority HARQ-ACK bits and low-priority HARQ-ACK bits are multiplexed into a PUSCH, reuse Rel-15 rate matching and RE mapping as the baseline.

#### 3.2.1.5 Minimizing impact on the reliability for high-priority UCIs via UL power control

In the sections above, we have discussed different ways of guaranteeing the performance of high-priority UCIs including separately configured beta-offset and separate coding. Another aspect which could be simply enhanced is UL power control. For example, when the high-priority HARQ-ACK is multiplexed with low-priority PUSCH, a different set of power control parameters can be applied comparing to the case without multiplexing high-priority HARQ-ACK. Certainly this is another way to guarantee the required reliability performance of high-priority HARQ-ACK when multiplexed on low-priority PUSCH. And therefore, we propose RAN1 to further study different ways of enhanced PUSCH power control.

Proposal 3.25: For the scenario where multiplexing high-priority HARQ-ACK bits on a low-priority PUSCH, RAN1 to investigate UL power control aspects to guarantee the required reliability of high-priority HARQ-ACK bits.

### 3.2.2 Low-priority PUSCH vs. high-priority SR

In this case, following Rel-16 rule, the low-priority PUSCH is always dropped which will result in unnecessary retransmission. Clearly this is not optimal especially considering that the high-priority SR is of only 1-bit information. Therefore, ways to multiplex the high-priority SR and the low-priority PUSCH should be considered.

One approach would be that gNB always reserves resource in a similar way as multiplexing HARQ-ACK in PUSCH for the high-priority SR transmission (regardless of whether SR is positive or negative) when the high-priority SR resource and the low-priority PUSCH resource collides. While considering 1-bit SR information, another simple way is to carry SR information for example by DM-RS. Therefore, different options can be studied further:

- Option 1: the reserved resource in terms of number of REs for example can be pre-configured via RRC signalling. In this way, the reliability of the high-priority SR can be guaranteed by the properly configured resource elements.

- Option 2: following similar way as multiplexing HARQ-ACK in PUSCH, by introducing a new set of beta_offset for high-priority SR transmission and number of REs for SR transmission is determined by the beta_offset which is defined for multiplexing SR and can be dynamically indicated with DCI or configured beforehand in case with configured PUSCH transmission or the low-priority PUSCH is scheduled by DCI Format 0_0.

- Option 3: using PHY signals to carry the SR information since SR is one-bit information (only positive or negative). One simple example with limited standardization impact is that the information of positive/negative SR can be carried by for example DM-RS. Depending on the detected DM-RS, gNB will learn whether the high-priority SR is positive or negative. Compared to Option 1 and 2, as always with some signalling using different DM-RS, this could lead to error cases (based on incorrect gNB DM-RS identification) potentially impacting the low-priority PUSCH performance.

In case the positive SR is coming too late, Rel-16 rule can be applied, i.e. the low-priority PUSCH is cancelled at latest starting from the first symbol of the PUCCH resource carrying the high-priority SR.

Proposal 3.26: Multiplexing high-priority SR in low-priority PUSCH is supported. FFS detailed ways of carrying high-priority SR information.

### 3.2.3 More than two overlapping channels involving PUSCH

In the discussions above, we have focused our discussion on the scenarios with two overlapping channels. However, there could be cases where after the first step of handling overlapping channels with the same priority, there are more than two overlapping channels of different priorities as also reflected in the FFS point from RAN1#102 e-meeting: FFS: Whether to support multiplexing in case a PUCCH/PUSCH overlaps with more than one PUCCH/PUSCH. Specifically, some of these scenarios are listed below:

- high-priority/low-priority PUSCH vs. at least two non-overlapping PUCCHs carrying low-priority/ high-priority UCI(s);

- PUCCH carrying low-priority/high-priority UCI(s) vs. at least two non-overlapping PUSCHs of different priorities.

RAN1 should discuss how to handle different scenarios with more than two overlapping channels with necessary effort and take into account the discussions of handling the scenarios with two overlapping channels. In addition, different scenarios with more than two overlapping channels involving PUSCH should be handled differently since not all scenarios are equally important or can be supported in the first place.

As one example, let’s focus on potential multiplexing of HP UCI (i.e. HP PUCCH carrying HARQ and/or SR) on PUSCH here (to simplify the discussion as for low-priority UCI and high-priority PUSCH also CSI would need to be considered). Two different cases are to be handled:

- Case 1: low-priority PUSCH overlapping with more than one PUCCH carrying HARQ-ACK, as illustrated in Figure 3.3 (a);

- Case 2: low-priority PUSCH overlapping with one high-priority PUCCH carrying HARQ-ACK and one high-priority PUCCH carrying SR, as illustrated in Figure 3.3 (b).

For case 1, it is noted that even the multiplexing of more than one PUCCH carrying high-priority HARQ-ACK on a high-priority PUSCH is not supported in Rel-16. Clearly the same should apply if we are talking about multiplexing of different priorities. Otherwise, the multiplexing of high-priority HARQ-ACK on low-priority PUSCH would be less restrictive compared to multiplexing on high-priority PUSCH. So generically, the following can be noted:

Proposal 3.27: Multiplexing of more than one PUCCH carrying HARQ-ACK on a PUSCH of different priorities should not be supported, as the multiplexing is also not supported for the scenario where PUSCH and HARQ-ACK are with the same priority.

For case 2, if the multiplexing conditions are fulfilled for both PUCCHs, one carrying high-priority HARQ-ACK and one carrying high-priority SR, the multiplexing on the overlapping low-priority PUSCH in principle could be supported.

Proposal 3.28: Multiplexing of more than one high-priority PUCCH, where one of them is carrying HARQ-ACK and the other one carrying SR, on a low-priority PUSCH could be supported.

(a) two overlapping PUCCHs carrying HARQ-ACK

(b) one overlapping PUCCH carrying HARQ-ACK, one carrying SR

Figure 3.3: Example scenarios with more than two high-priority PUCCHs overlapping with low-priority PUSCH.

Therefore, when discussing the multiplexing on PUSCH overall, RAN1 would need to be specific about the cases to be considered. Specific considerations are needed in terms of the multiplexing timeline, as the UE when making the multiplexing decision may not be aware of a later arriving PUCCH, e.g. carrying another HARQ-ACK CB as discussed for case 1 above. Thus, clearly the handling of multiplexing of different priority UCI on PUSCH is more complex in terms of the needed restrictions and/or exceptions compared to the case of control channel vs. control channel discussed in Sec. 3.1.4.

Moreover, when multiplexing low-priority UCI on a high-priority PUSCH, having restrictions such as partial low-priority UCI dropping (e.g. only UCI of the first overlapping PUCCH is to be multiplexed) may be acceptable, as in case of Rel-16 PHY prioritization all the low-priority UCI would be lost. But in contrast, multiplexing restrictions of high-priority UCI on a low-priority PUSCH potentially leading to the situation of dropping at least part of the high-priority UCI may not be acceptable and clearly against the idea about PHY priority in the first place.

Observation 3.4: Potential multiplexing restrictions or partial UCI dropping for low-priority UCI of more than one overlapping PUCCH multiplexed on an overlapping high-priority PUSCH may be acceptable, whereas partial dropping or multiplexing restrictions of high-priority UCI is not acceptable.

Based on the discussions, we believe that the decision of multiplexing some UCIs on a PUSCH of a different priority needs to consider the case of more than two overlapping channels: first agreeing the support specific multiplexing in general (e.g. for the case of high-priority UCI on sub-slot PUCCH multiplexed on long(er) low-priority PUSCH allocation) and then later on recognizing sever issues, seems to be counterproductive here.

Proposal 3.29: RAN1 needs to take the cases of more than two overlapping channels (involving at least one PUSCH) of different priorities into account when deciding whether to support certain multiplexing enhancements in first place. This is specifically important when considering the support of multiplexing high-priority UCI on low-priority PUSCH.

# 4	Rel-17 PHY prioritization to enable at least simultaneous PUCCH/PUSCH transmission on different cells

As indicated in [R1-2007075] and [R1-2009546], supporting simultaneous PUCCH/PUSCH transmissions on different cells is one way to reduce the potential cancellation of low-priority channels when they are overlapping with high-priority channels from a different cell. From the previous RAN1 e-meetings, we have the following agreements:

Agreements (RAN1#102-e meeting): Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA. - FFS how to trigger this function. - FFS for intra-band CA. Agreements (RAN1#104-e meeting): Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group - FFS: dynamic indication

Looking at the intention of the agreements and some of the discussions during RAN1#104-e, we are a bit wondering what the group overall tries to achieve and how to implement this in the end. Specifically, our assumption here is that we try to prevent unnecessary at least dropping of low-priority UCI due to overlap with high priority transmissions on a different serving cell. But this may be only Nokia’s intention, as at least the support of simultaneous PUSCH / PUCCH of the same PHY priority as proposed by some companies is unclear to us in how this would improve anything related to URLLC (neither in terms of LP channel / information dropping, nor in terms of reliability or latency).

Proposal 4.1: RAN1 to clarify the intention of the support of simultaneous PUCCH / PUSCH, namely what to improve in terms of e.g. LP channel or information dropping, latency, reliability, efficiency, or the like.

For the discussion, it would be worth to first review why actually low-priority UL transmissions are cancelled and how the order to the processing of LP and HP UL channels is actually happening. At RAN1#104-e, as part of the maintenance discussions in [104-e-NR-L1enh-URLLC-04] (the RAN1 CR to Sec.9 of 38.213 in R1-2102229) – with the following final PHY prioritization operation has been agreed:

# 9 UE procedure for reporting control information If a UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG. - When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively. - When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG. *** Void*** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in Clauses 9.2.5 and 9.2.6 - Any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in Clause 11.1 - the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assumingd2,1=0 [6, TS 38.214], based onμ andN2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability *** void ***

So basically, the following is happening sequentially (using the related colour coding):

- Step 1: The UE performs PHY multiplexing decision for low priority channels – and determines the final LP UL transmissions

- Step 2:

  - The UE determines the overlapping of the resulting LP channel with HP PUCCH / PUSCH before and/or after HP UL channel multiplexing across all serving cells of a MCG/SCG.

  - Based on the overlapping determination, the UE cancels an overlapping LP PUCCH and if overlapping with a HP PUCCH cancels overlapping PUSCH(s) across all serving cells of the MCG or SCG

Let’s therefore, look at some simple examples shown in Figure 4.1 where two cases are demonstrated having the scenarios that we try in the end to support simultaneous PUCCH / PUSCH transmission and to not drop the low-priority UCI there. We don’t at this point want to discuss intra-band versus inter-band operation here – so at least for SCell on a different band (inter-band CA) this should be supported.

Figure 4.1: Examples for simultaneous PUCCH / PUSCH transmission.

Looking at the PHY prioritization operation, to support the simultaneous PUCCH / PUSCH operation of different priorities for Case A and Case B, the Rel-16 overlapping determination and related cancelation behaviour would need to be changed, i.e. the cancelation would be conditional on the overlap being on different serving cells (of different bands).

Observation 4.1: For the scenario of only having PUCCH for a certain priority, the support of simultaneous PUCCH/PUSCH transmission of different PHY priorities on different serving cells at least for inter-band CA requires changes to the overlapping determination and related cancelation behaviour of Rel-16 PHY prioritization operation.

Moreover, let’s look at some more example cases namely 3 overlapping channels in across different priorities in Figure 4.2.

Figure 4.2: Examples for simultaneous PUCCH / PUSCH when considering 3 overlapping channels.

Obviously, when considering more than 2 overlapping channels, the needed changes to support simultaneous PUSCH/PUSCH are quickly even more evident. Looking at the different examples cases here the following can be note:

- Case C: Based on the Rel-16 PHY prioritization operation, the multiplexing of the LP channels is done before the PHY prioritization – so the remaining channel would be the LP (CG) PUSCH on SCell. If the intention is to support the LP PUCCH transmission on parallel to the HP (CG) PUSCH (enhancements to enable CG and DG PUSCH combination in Rel-17), the question is if we (a) change the Rel-16 PHY prioritization behavior to not do the LP multiplexing before the PHY prioritization (i.e. do the LP PUSCH cancelation before the multiplexing) as well as the overlapping determination and related cancelation behaviour, (b) alternatively enable simultaneous PUSCH/PUCCH of the same priority (i.e. no LP UCI multiplexing on PUSCH) in addition to the change for the Rel-16 PHY prioritization operation to the overlapping determination and related cancelation behaviour or (c) add the simultaneous PUSCH/PUCCH operation at the end of the PHY prioritization – i.e. as a last step: if the UCI has been cancelled re-generate the PUCCH for transmission (which is somehow against the overall processing flow, as the argument for the current operation has been that it may be easier /quicker to cancel compared to prepare for a new UL transmission later on).

Looking at this example, one might say that (b) of using or supporting simultaneous PUSCH/PUCCH of the same priority (in this case LP) would have the least specs impact to support this case – compared to the more substantial changes needed for (a) and (c). Having said that, using simultaneous PUSCH/PUCCH will create issues for other cases, such as the next case D

- Case D: Based on the Rel-16 operation, the resulting LP channel would be the LP PUSCH (with UCI multiplexed) which is transmitted on the SCell on parallel to the HP PUSCH on Pcell. When now operating with simultaneous PUCCH / PUSCH of the same priority before the PHY prioritization (as discussed for a potential solution (b) for Case C), the HP PUSCH would cancel the LP PUCCH and compared to the Rel-16 operation, the LP UCI would be lost. Therefore, operating with simultaneous PUSCH/PUCCH operation for the same PHY priority clearly will not solve all the issues – but on the contrary: as having a larger number of channels transmitted (i.e. PUCCH on Pcell on top of PUSCH on SCells), the amount of LP channel cancelation due to the PHY prioritization will even increase.

- Case E: For case E the final HP channel to be transmitted would be the HP PUSCH (incl. UCI on SCell with the LP PUCCH to be cancelled). To enable simultaneous LP PUCCH / HP PUSCH transmission (incl. the HP UCI), the changes are needed in the order of overlapping determination and HP multiplexing (only applicable after the HP multiplexing) – and again the overlapping determination and related cancelation behaviour.
But we would like to note here, that in case simultaneous PUSCH/PUCCH of the same priority would be operated for this example case, the UE would be transmitting simultaneously HP PUSCH and HP PUCCH – and the LP UCI / PUCCH would be dropped as in Rel-16. Thus, the support of simultaneous PUSCH / PUCCH of the same priority in this case will not allow for any improvement in terms of LP UCI dropping at all (but on the contrary, no parallel LP transmission would be possible potentially also affecting LP PUSCH transmissions).

- Case F: Based on the Rel-16 operation, the resulting LP channel being the PUSCH including the multiplexed UCI, this after the LP multiplexing results in the same situation before the prioritization situation as Case B of Figure 4.1 – and changes to  the overlapping determination and related cancelation behaviour would be needed. 
But similar as for Case D & E, supporting simultaneous PUSCH / PUCCH of the same PHY priority again will lead to the case of the LP UCI / PUCCH dropping – as parallel PUCCH transmissions on a single serving cell (or any parallel UL transmissions on a serving cell) seems to be out of scope.

Observation 4.2: When considering more than two overlapping channels, the support of simultaneous PUCCH/PUSCH transmissions of different PHY priorities on different serving cells at least for inter-band CA to reduce the LP channel dropping in Rel-17 requires changes to all logical steps of the Rel-16 PHY prioritization operation, namely (i) the order of LP multiplexing and PHY prioritization, (ii) the order of PHY prioritization and HP UL multiplexing, (ii) the overlapping determination and (iv) the related cancelation behaviour of Rel-16 PHY prioritization operation.

Last but not least on the support, we think that the support of simultaneous PUSCH/PUCCH transmission of the same PHY priority is not in scope of the WI objective, as the WI objective clearly describes that only multiplexing enhancements of different PHY priorities are in focus:

3. Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]: - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH. - Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

Observation 4.3: The support of simultaneous PUSCH/PUCCH transmission of the same PHY priority would increase low-priority information and channel dropping when taking the PHY prioritization into account. Moreover, multiplexing enhancements of the same PHY priority are not in scope of the WI objective.

We really think RAN1 should clarify what ‘simultaneous PUCCH / PUSCH’ here means, and to which extend this is to be supported in conjunction with the PHY prioritization operation, i.e. which cases are to be supported and therefore, how much we want to change the PHY prioritization operation.

Based on the discussion above, the following related proposals are made:

Proposal 4.2: RAN1 to discuss and clarify, for which cases simultaneous PUCCH / PUSCH operation of different priorities should be supported with respect to the required changes to the PHY prioritization operation / behavior, including at least:

- Are changes to the processing order of LP multiplexing and PHY prioritization in scope?

- Are changes to the processing order of PHY prioritization and HP UL channel multiplexing in scope?

- Is the intention to support case-specific optimized solutions to enable simultaneous PUSCH/PUCCH of different PHY priorities for all possible overlapping cases or instead a generic enhancement of the PHY prioritization framework / procedure (with limitations for some cases)?

Proposal 4.3: The simultaneous transmission of PUCCH and PUSCH on different serving cells is applicable only for the case when PUCCH and PUSCH are of different PHY priority.

We really think that related discussions in RAN1 on the points raised in Proposals 4.1 and 4.2 are essential for the group to be able to move forward (to see what other cases in addition could be supported / where restrictions are needed). On the current discussion level during RAN1 meetings of what to support and what not (without having clarity), the following related proposals are still brought forward based on our earlier contribution in R1-2100729:

Proposal 4.4: For intra-band CA, simultaneous transmission of PUCCH and PUSCH on different cells is not supported.

Proposal 4.5: For UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission over different cells can be triggered via higher layer signalling (e.g. RRC signalling).

# 5	Conclusions

In this contribution, firstly we discussed the identified issues related to overlapping CG-PUSCH and DG-PUSCH with different priorities in Sec. 2 and proposed the following:

Observation 2.1: For the scenarios CG PUSCH vs. DG PUSCH of different PHY priorities, the aspects related to handling the cases where a PUCCH overlaps with at least one of the overlapping PUSCHs and the impact of uplink skipping can be discussed after reaching a conclusion on the related Rel-16 discussions.

Proposal 2.1: For the scenario high-priority CG PUSCH vs. low-priority DG PUSCH, it is up to UE implementation to ensure that the low-priority DG PUSCH is cancelled, at the latest, from the first symbol that is overlapping with the high-priority CG PUSCH.

Proposal 2.2: The Rel-16 handling of the scenarios where a dynamically scheduled high-priority channel overlaps with a low-priority channel is adopted for the scenario of overlapping between high-priority DG PUSCH and low-priority CG PUSCH.

For intra-UE multiplexing of overlapping channels of different priority on PUCCH (i.e. PUCCH versus PUCCH) in Sec 3.1, we have the following observations and proposals:

Proposal 3.1: The gNB dynamically indicates, via an explicit field in the DCI scheduling high-priority HARQ-ACK, whether multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK (or more generally low-priority multiplexed UCIs) is enabled or disabled.

Proposal 3.2: For the scenario where a PUCCH carrying 1-bit high-priority HARQ-ACK overlaps with another PUCCH carrying 1-bit low-priority HARQ-ACK, the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK is done on the high-priority PUCCH resource.

Proposal 3.3: Dedicated PUCCH resource configuration for multiplexing of high- and low-priority HARQ-ACK is not supported.

Proposal 3.4: For the scenario where a PUCCH carrying high-priority HARQ-ACK overlaps with another PUCCH carrying low-priority HARQ-ACK and the total payload size is two bits, the order of the multiplexed two bits could be [high-priority HARQ-ACK bit, low-priority HARQ-ACK bit].

Proposal 3.5: For the scenario where a PUCCH carrying high-priority HARQ-ACK overlaps with another PUCCH carrying low-priority HARQ-ACK: If the high-priority HARQ-ACK does not have a corresponding PDCCH, the multiplexing is not supported.

Proposal 3.6: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK where the high-priority or low-priority HARQ-ACK is 1-2 bits, adopt the following:

- In case HARQ-ACK is 1 bit: use the existing Rel-15 1-bit information encoding scheme in TS 38.212 Sec. 5.3.3.1 to encode this HARQ-ACK.

- In case HARQ-ACK is 2 bits: use the existing Rel-15 2-bit information encoding scheme in TS 38.212 Sec. 5.3.3.2 to encode this HARQ-ACK.

Proposal 3.7: Confirm the RAN1#104bis-e Working Assumption to not support multiplexing of CSI (include part 1 and part 2, if any) and high-priority HARQ-ACK on PUCCH and thus to drop the CSI and prioritize the high-priority HARQ-ACK.

Proposal 3.8: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK, reuse Rel-15 operation for two UCI parts to determine the rate matching output sequence length for each of the high-priority HARQ-ACK and low-priority HARQ-ACK.

Proposal 3.9: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK resulting from the overlap between high-priority PUCCH resource and low-priority PUCCH resource:

- the maxCodeRate configured for the selected high-priority PUCCH resource for multiplexing should be used as maxCodeRate for the high-priority HARQ-ACK; and

- the maxCodeRate configured for the low-priority PUCCH resource could be used as maxCodeRate for low-priority HARQ-ACK.

  - If the low-priority PUCCH resource is with Format 0/1, use the maxCodeRate (for low-priority HARQ-ACK) associated to the low-priority PUCCH resource corresponding to a low-priority HARQ-ACK payload size of 3 bits.

Proposal 3.10: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK on PUCCH Format 3/4, adopt the Rel-15 operation of ‘multiplexing of coded UCI bits to PUCCH’ given in TS 38.212 Sec. 6.3.1.6.

Proposal 3.11: For the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK on PUCCH Format 2, adopt the following approach for mapping the separately coded bits to PUCCH:

- Aggregate the coded high-priority HARQ-ACK bits and the coded low-priority HARQ-ACK bits and apply the procedures described in Sec. 6.3.2.5 of TS 38.211 to this aggregated coded HARQ-ACK bit sequence.

Proposal 3.12: Further discuss the selection of PUCCH resource set and the number of RBs for the multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK.

Observation 3.1: Errors in low-priority HARQ-ACK codebook size determination may cause selection of different PUCCH resource set or use of smaller number of RBs for the multiplexed high-priority and low-priority HARQ-ACKs feedback than what gNB would expect.

Proposal 3.13: To avoid discrepancy between the UE and the gNB on the determination of PUCCH resource set and number of RBs for UCI containing multiplexed high-priority and low-priority HARQ-ACKs, support Option 3b:

- Option 3b: Configuration of semi-static size reservation for low-priority HARQ-ACK payload is provided by RRC.

Otherwise, as a second preference, support either Option 4 or Option 5:

- Option 4: The indication and determination are defined by combining dynamic indication for enabling/disabling low-priority HARQ-ACK multiplexing and for low-priority HARQ-ACK codebook size.

- Option 5: The indication and determination are defining the number of RBs and/or PUCCH resource set index to be used in the PUCCH transmission, where the indication is included in the high-priority DL assignment.

Observation 3.2: In principle, multiplexing could be supported for all four possible combinations of high-priority HARQ-ACK overlapping with low-priority HARQ-ACK considering Type-1 and Type-2 codebooks. If there would be a need to prioritize some cases for the discussions, the cases involving same HARQ-ACK codebook type (Type-1/Type-2) could be discussed first. The case corresponding to multiplexing high-priority Type-1 HARQ-ACK and low-priority Type-2 HARQ-ACK could also be of interest.

Proposal 3.14: For the scenario where a PUCCH carrying high-priority SR overlaps with a PUCCH carrying low-priority HARQ-ACK:

- If SR is with F0 and HARQ-ACK is with F0/F1, adopt one of the following options:

  - Opt.1b: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. For negative SR, the UE transmits only HARQ-ACK on the HARQ-ACK resource.

  - Opt.1c: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

- If SR is with F1 and HARQ-ACK is with F0/F1, adopt Opt.3: Transmit HARQ-ACK on the SR resource if SR is positive; and transmit HARQ-ACK on the HARQ-ACK resource when SR is negative.

- If SR is with F0/F1 and HARQ-ACK is with F2/F3/F4: If SR is positive, transmit SR on the SR resource and drop HARQ-ACK; if SR is negative, transmit HARQ-ACK only on the HARQ-ACK resource.

Proposal 3.15: For handling the scenarios where a PUCCH of a given priority crosses the sub-slot boundary of the PUCCH config of another priority and overlaps with a PUCCH of another priority, adopt the following procedure:

- Multiplexing of low-priority PUCCH and high-priority PUCCH, is allowed only if this multiplexing is done on a high-priority PUCCH resource. In addition:

  - UE does not expect an overlap between the resulting PUCCH resource to be used for multiplexing and another high-priority PUCCH;

  - and if the resulting PUCCH resource overlaps with a low-priority PUCCH, the low-priority PUCCH is then dropped.

  - Additional conditions are FFS.

Proposal 3.16: For handling the scenarios with more than two overlapping PUCCHs of different priorities, adopt the following procedure:

- Allow a single checking/multiplexing step between channels of different priorities, where in case multiplexing is feasible:

  - UE does not expect an overlap between the resulting resource to be used for multiplexing and a high-priority PUCCH;

  - and if the resulting PUCCH resource overlaps with a low-priority PUCCH, the low-priority PUCCH is then dropped.

For intra-UE multiplexing of overlapping channels of different priority on PUSCH (i.e. PUCCH versus PUSCH) in Sec 3.2, we have the following observations and proposals:

Proposal 3.17: For the scenarios of multiplexing HARQ-ACK bits in DG PUSCH of different priorities, gNB dynamically indicates via beta-offset in the corresponding scheduling DCI whether to multiplex HARQ-ACK in PUSCH of different PHY priority or not (e.g. beta-offset = 0). FFS whether to support multiplexing of HARQ-ACK bits on CG PUSCH of a different PHY priority.

Proposal 3.18: For the scenarios of multiplexing HARQ-ACK bits in PUSCH of different priorities, RAN1 should specify:

- three sets of beta-offset values (i.e. Option 1) for:

  - multiplexing HARQ-ACK bits on the PUSCH with the same priority (specified already);

  - multiplexing low-priority HARQ-ACK bits on high-priority PUSCH;

  - multiplexing high-priority HARQ-ACK bits on low-priority PUSCH;

- for multiplexing of both low-priority and high-priority HARQ-ACK bits, the beta-offset indicator field in the DCI points to the respective two sets of beta-offset values to be applied respectively for low- and high-priority HARQ-ACK.

Proposal 3.19: For the scenarios of multiplexing HARQ-ACK bits in a PUSCH of different priorities, do not support separate configurations of the scaling factor “alpha”.

Proposal 3.20: For the scenario of the multiplexing between HARQ-ACK and PUSCH with different priorities,

- RAN1 should confirm the working assumption of reusing Rel-15 timeline conditions for multiplexing HARQ-ACK and PUSCH.

- If multiplexing timeline conditions are not met or no multiplexing is indicated from gNB, Rel-16 prioritization rule should be applied (i.e. the transmission of the low-priority channel(s) is cancelled).

Proposal 3.21: For the scenario of the multiplexing between HARQ-ACK and PUSCH with different priorities, RAN1 should not support joint coding of different UCI types, for example low-priority HARQ-ACK and CSI.

Proposal 3.22: For the scenario where high-priority HARQ-ACK bits, low-priority HARQ-ACK bits and CSI would be multiplexed into a low-priority PUSCH, drop CSI (including part 1 and part 2, if exist).

Observation 3.3: For the scenario where multiplexing both high-priority HARQ-ACK bits and low-priority HARQ-ACK bits into a high-priority PUSCH without CSI, the number of encoding chains is sufficient.

Proposal 3.23: For the scenario where both high-priority HARQ-ACK bits and low-priority HARQ-ACK bits would be multiplexed into a high-priority PUSCH carrying CSI, drop low-priority HARQ-ACK.

Proposal 3.24: For the scenarios where both high-priority HARQ-ACK bits and low-priority HARQ-ACK bits are multiplexed into a PUSCH, reuse Rel-15 rate matching and RE mapping as the baseline.

Proposal 3.25: For the scenario where multiplexing high-priority HARQ-ACK bits on a low-priority PUSCH, RAN1 to investigate UL power control aspects to guarantee the required reliability of high-priority HARQ-ACK bits.

Proposal 3.26: Multiplexing high-priority SR in low-priority PUSCH is supported. FFS detailed ways of carrying high-priority SR information.

Proposal 3.27: Multiplexing of more than one PUCCH carrying HARQ-ACK on a PUSCH of different priorities should not be supported, as the multiplexing is also not supported for the scenario where PUSCH and HARQ-ACK are with the same priority.

Proposal 3.28: Multiplexing of more than one high-priority PUCCH, where one of them is carrying HARQ-ACK and the other one carrying SR, on a low-priority PUSCH could be supported.

Observation 3.4: Potential multiplexing restrictions or partial UCI dropping for low-priority UCI of more than one overlapping PUCCH multiplexed on an overlapping high-priority PUSCH may be acceptable, whereas partial dropping or multiplexing restrictions of high-priority UCI is not acceptable.

Proposal 3.29: RAN1 needs to take the cases of more than two overlapping channels (involving at least one PUSCH) of different priorities into account when deciding whether to support certain multiplexing enhancements in first place. This is specifically important when considering the support of multiplexing high-priority UCI on low-priority PUSCH.

Based on the discussions on simultaneous PUCCH/PUSCH transmission on different cells in Sec. 4, we propose:

Proposal 4.1: RAN1 to clarify the intention of the support of simultaneous PUCCH / PUSCH, namely what to improve in terms of e.g. LP channel or information dropping, latency, reliability, efficiency, or the like.

Observation 4.1: For the scenario of only having PUCCH for a certain priority, the support of simultaneous PUCCH/PUSCH transmission of different PHY priorities on different serving cells at least for inter-band CA requires changes to the overlapping determination and related cancelation behaviour of Rel-16 PHY prioritization operation.

Observation 4.2: When considering more than two overlapping channels, the support of simultaneous PUCCH/PUSCH transmissions of different PHY priorities on different serving cells at least for inter-band CA to reduce the LP channel dropping in Rel-17 requires changes to all logical steps of the Rel-16 PHY prioritization operation, namely (i) the order of LP multiplexing and PHY prioritization, (ii) the order of PHY prioritization and HP UL multiplexing, (ii) the overlapping determination and (iv) the related cancelation behaviour of Rel-16 PHY prioritization operation.

Observation 4.3: The support of simultaneous PUSCH/PUCCH transmission of the same PHY priority would increase low-priority information and channel dropping when taking the PHY prioritization into account. Moreover, multiplexing enhancements of the same PHY priority are not in scope of the WI objective.

Proposal 4.2: RAN1 to discuss and clarify, for which cases simultaneous PUCCH / PUSCH operation of different priorities should be supported with respect to the required changes to the PHY prioritization operation / behavior, including at least:

- Are changes to the processing order of LP multiplexing and PHY prioritization in scope?

- Are changes to the processing order of PHY prioritization and HP UL channel multiplexing in scope?

- Is the intention to support case-specific optimized solutions to enable simultaneous PUSCH/PUCCH of different PHY priorities for all possible overlapping cases or instead a generic enhancement of the PHY prioritization framework / procedure (with limitations for some cases)?

Proposal 4.3: The simultaneous transmission of PUCCH and PUSCH on different serving cells is applicable only for the case when PUCCH and PUSCH are of different PHY priority.

Proposal 4.4: For intra-band CA, simultaneous transmission of PUCCH and PUSCH on different cells is not supported.

Proposal 4.5: For UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission over different cells can be triggered via higher layer signalling (e.g. RRC signalling).