3GPP TSG RAN WG1 #106-e		R1-2108210

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussion on mode 2 enhancements

Agenda Item:	8.11.1.2

Document for:	Discussion and Decision

- Introduction

NR SL supports various use cases, including V2X services, critical D2D communication and commercial D2D communication. For some use cases, low latency and extremely high reliability are expected. To meet such high QoS requirements, the WID [1] on NR sidelink enhancement was approved as following. In this contribution, we provide further discussion on mode 2 enhancements.

2. Resource allocation enhancement: - Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#89), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following until RAN#88. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The study scope after RAN#88 is to be decided in RAN#88. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

- Discussion

Based on the investigation of the Rel-16 mode 2 resource selection mechanism, it is observed that resource collision, half-duplex constraint and simultaneous transmission capability are the main barriers for achieving higher transmission reliability. These issues can be addressed with the help of inter-UE coordination schemes as analysis in companion paper [2]. In the following, the details of inter-UE coordination schemes will be discussed.

  - Generalization of mode 2 enhancement

Agreement: - Support the following schemes of inter-UE coordination in Mode 2: - Inter-UE Coordination Scheme 1: - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information - FFS condition(s) in which Scheme 1 is used - Inter-UE Coordination Scheme 2: - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict - FFS condition(s) in which Scheme 2 is used

In this section, general solutions will be described based on the solution categories as agreed in RAN1#104bis-e, i.e., inter-UE coordination scheme 1 and inter-UE coordination scheme 2.

Inter-UE coordination scheme 1

In inter-UE coordination scheme 1, UE-A can send to UE-B the set of resources preferred for UE-B’s transmission.

As one approach of scheme 1, the UE-A as a RX UE, identifies low interference resources and informs all or part of them to the TX UE (i.e., UE-B) before its resource selection, then the TX UE uses the recommended low interference resources for its transmission to increase the reliability by, e.g., avoiding resource collision due to hidden node issue. Besides, the UE-A can also take into account its SL/UL transmission occasions when determining the recommended resources, e.g., when there is UL transmission overlapped with a given SL slot, the UE-A can preclude this SL slot from the recommended resource to avoid half duplex issue. Moreover, the UE-A can also preclude the PSSCH resources whose associated PSFCH occasion is located in this SL slot to avoid PSFCH TX and UL TX overlap.

Alternatively, scheme 1 can also be used for resource coordination among a leading-UE and member-UEs. When a member-UE (as UE-B) performs SL transmission, it can trigger the leading-UE (as UE-A) to assign suitable transmission resources, to achieve centralized resource coordination among member-UEs in a UE group. In this approach, UE-B can either use the recommended resources for transmissions to UE-A, or for transmission to a third UE (e.g., another UE-B) as shown in Figure 1. However, to reduce specification effort, RAN1 should strive for a common solution framework regardless the destination UE is UE-A or not.

Figure 1 leading-UE and member-UE structure

In RAN1#104bis-e e-meeting, it was discussed that whether the set of recommended resources is termed as ‘preferred resources for UE-B’s transmission’ or ‘non-preferred resources for UE-B’s transmission’. As in the discussed approach of scheme 1, there would be lots of troublesome resources for UE-A’s reception and the amount of troublesome resources may vary in time, large signaling overhead would be incurred if all of the ‘non-preferred resources for UE-B’s transmission’ are recommended from UE-A to UE-B. However, UE-A can only selects some of the ‘preferred resources for UE-B’s transmission’ as recommended resources to reduce the signaling overhead of the coordination information. Therefore, it is suggested to term the recommended resources as ‘preferred resources for UE-B’s transmission’.

Proposal 1: In inter-UE coordination scheme 1, UE-A (RX UE/leading-UE) sends to UE-B (TX UE) a set of resources preferred for UE-B’s transmission to assist its resource selection.

Inter-UE coordination scheme 2

In inter-UE coordination scheme 2, UE-A detects the presence of resource conflict based on decoding result of UE-B’s SCI, then, UE-A can indicate the resource conflict to UE-B to trigger its resource re-selection. It is assumed that UE-B is TX UE and UE-A is the intended RX UE for scheme 2.

As one approach of scheme 2, the UE-A detects any potential resource conflict, and notifies the UE-B to trigger resource reselection. Noted that such conflict may happen not only due to another NR SL transmission, but also due to (potential) transmission of other link (such as UL) or other RAT (such as LTE SL).

Proposal 2: In inter-UE coordination scheme 2, UE-A (RX UE) detects potential/expected resource conflict of TX UE’s transmission and triggers resource reselection of UE-B (TX UE).

As another approach of scheme 2, the UE-A identifies (potential) resource conflict by detecting a number of PSCCH/PSSCH decoding failure or mis-detection, i.e., consecutive packet conflict, and send NACK/DTX to UE-B. When UE-B detects multiple NACK/DTX for a given SL grant, UE-B understands that the failure is incurred by consecutive resource conflict, and then triggers resource re-selection to recover the transmissions.

Proposal 3: In inter-UE coordination scheme 2, UE-B (TX UE) performs resource re-selection based on multiple detected NACK/DTX feedback from UE-A (RX UE).

  - Condition to be UE-A/UE-B

Agreements: - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination: - Details include applicable scenario(s)/inter-UE coordination scheme(s) - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc. - Including the possibility of being subject to certain conditions and/or capability

In RAN1#104bis-e e-meeting, it has been agreed to further study how to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination.

For scheme 1, before using the enhanced resource selection procedure, UE-B needs to know whether there is a UE-A or not to provide the coordination information, so the relationship of UE-A and UE-B should be pre-determined. This relationship can be determined by high layer (e.g., based on handshake between UE-A and UE-B) and informed to PHY layer. After determination of the relationship of UE-A and UE-B, the inter-UE coordination scheme 1 as described in section 2.1 can be applied.

Proposal 4: For scheme 1, the relationship between UE-A and UE-B is pre-determined by high layer.

For scheme 2, it is believed that UE-A is an intended receiver of UE-B; otherwise, UE-B may receive plenty of conflict indications. Consequently, it is challenging for UE-B to handle the multiple conflict indications, especially some indications may be from evildoers. Regarding determination of UE-B, it is preferred to restrict UE-B as a TX UE who performs unicast transmission or groupcast transmission, because it is not realistic for a TX UE to resolve the detected/expected resource conflict for each intended receiver of a broadcast transmission.

Proposal 5: For scheme 2, UE-B can only be a UE performing unicast or groupcast transmission, while UE-A is the intended receiver of the transmission.

  - Determination of ‘a set of resource’

FL’s proposal: - For inter-UE coordination in Mode 2, consider at least one of the following information (with details FFS) - For Scheme 1 to determine by UE-A the set of resources preferred and/or non-preferred for UE-B’s transmission: - Other UEs’ reserved resources based on UE-A’s sensing result and/or coordination information (e.g., non-preferred resource set) received from other UEs - Coordination information (e.g., preferred resource set) received from other UEs - Information on UE-B’s traffic requirements (e.g., conveyed via triggering information from UE-B, if any) - Location information on UE-B and other UEs - Subset or all of UE-A’s NR SL resources selected for its transmission(s) of TB(s) - UE-A’s scheduled/configured resources for UL - LTE SL transmission and/or reception of UE-A - Resource set selected by UE-A for other UE-Bs’ transmissions - PSFCH transmission and/or reception - UE-A’s candidate resource set based on UE-A’s sensing - UE-B’s ability to use coordination information - Etc. - For Scheme 2 to determine by UE-A the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI: - Other UEs’ reserved resources and/or existing transmission (i.e. used resources) based on UE-A’s sensing result (e.g., measurement, information extracted from SCI) - Information on UE-B’s traffic requirements - Location information on UE-B and other UEs - Subset or all of UE-A’s NR SL resources selected for its transmission(s) of TB(s) - UE-A’s scheduled/configured resources for UL - LTE SL transmission and/or reception of UE-A - PSFCH transmission and/or reception - UE-B’s ability to use coordination information - Etc.

The issue of how to determine ‘a set of resources’ was discussed but not resolved in the previous meeting. In the following, the details of resource set determination are discussed to address this issue.

Inter-UE coordination scheme 1

As discussed in section 2.1, in one approach of scheme 1, UE-B is TX UE and UE-A is an intended RX UE of UE-B. UE-A recommends resources to UE-B to avoid potential resource conflict at UE-A, thus to improve transmission reliability and resource efficiency. As discussed in companion paper [2], there would be multiple scenarios of resource conflicts, due to diverse causes, e.g., resource collision, half duplex conflict, TX/TX conflict.

Some typical conflict cases of NR SL transmission/reception are illustrated in Figure 2, where the types of resource conflict are listed as following.

- PSSCH resource collision

- PSSCH TX/RX conflict

- PSFCH TX/RX conflict

Figure 2 conflict of NR SL transmission/reception at UE-A

Additionally, some conflict cases of NR SL transmission/reception and UL transmission are illustrated in Figure 3, where the types of resource conflicts are listed as following.

- PSSCH RX and UL TX conflict

- PSFCH TX and UL TX conflict

Figure 3 Conflict of NR SL transmission/reception and UL transmission at UE-A

Last but not least, conflict cases of NR SL transmission/reception and LTE SL transmission/reception are illustrated in Figure 4, and the types of resource conflicts are listed as following.

- NR PSSCH RX and LTE PSSCH TX conflict

- NR PSFCH TX and LTE PSSCH RX conflict

- NR PSFCH TX and LTE PSSCH TX conflict

Figure 4 Conflict of NR SL transmission/reception and LTE SL transmission/reception at UE-A

If the listed resource conflicts occur, the PSSCH transmission from UE-B to UE-A or PSFCH feedback from UE-A to UE-B might be delayed or dropped, consequently decreasing the data reception reliability and increasing the system load and interference level. To avoid the listed resource conflict issues at UE-A, UE-A should carefully recommend preferred resources for UE-B’s transmission.

- To avoid resource collision incurred by hidden node issue, UE-A should identify low interference resources based on sensing, and select recommended resources only within the low interference resource. Regarding the ‘low interference resource’ identification, legacy candidate resource set identification procedure can be the baseline. Assume that UE-A acquires several parameters for the associated TB transmission of UE-B, e.g., remaining PDB, sub-channel size, etc., then UE-A can identify y% candidate resources with lowest measured RSRP from time window bounded by the remaining PDB. The y% candidate resources is taken as ‘low interference resource’.

- To avoid resource conflicts due to half-duplex constraint and simultaneous transmission constraint, the recommended time resources (including PSSCH occasion and associated PSFCH occasion) should not be overlapped with UL TX, LTE PSSCH TX or RX, NR PSSCH TX or PSFCH RX of UE-A. Since the operation only reflects the desire of UE-A, the operation may be left to UE-A’s implementation. Thus, it is reasonable to give some freedom for UE-A to decide the coordination information. However, it is also beneficial to restrict the UE-A’s implementation, e.g., preclude the UL slot(s) to avoid impact to UL transmission.

In another approach of scheme 1, UE-A is a leading-UE and UE-B is a member-UE. UE-A as coordinator can coordinate transmission resources of multiple UE-Bs to avoid resource collision between different UE-Bs. For the determination of ‘a set of resources’ by UE-A, it is believed that common framework as discussed above can be reused. Regarding how UE-A to determine resource to different UE-Bs, it is preferred to let the UE-A to handle this with some necessary restriction to guarantee the system performance, e.g., avoiding a large number of resources to be assigned to a given UE-B; or orthogonal resources to be assigned to different UE-Bs.

Proposal 6: For scheme 1, UE-A determines preferred resources for UE-B’s transmission from low interference resources identified by UE-A based on sensing.
-	Legacy candidate resource set identification procedure is the baseline for determination of the low interference resources, assuming the associated PDB/sub-channel size is known by UE-A.

Proposal 7: For scheme 1, it is up to UE-A’s implementation to select recommended resources from the low interference resources.
-	FFS any additional restriction for the resource selection.

Inter-UE coordination scheme 2

In one approach of scheme 2, UE-A should detect the potential/expected resource conflict. In this case, the types of resource conflicts need to be defined, so that UE-A determines ‘a set of resource’ as the resources incurring the defined resource conflicts. The types of resource conflict as listed in scheme 1 can be starting point for further discussion.

Regarding PSSCH resource collision as in figure 1, hidden node is the main reason incurring the resource conflict. The collided resource which is not sensed by UE-B (TX UE) can be sensed by UE-A (RX UE), thus the sensing result of UE-A can be used to assist UE-B to determine transmission resource. When UE-A detects potential resource collision between the reserved resources of UE-B and another UE, it can trigger resource re-selection of UE-B to resolve the collision. For such solution, RAN1 needs to study the criteria to define a resource collision between UE-B and the other UE, e.g., the RSRP measured on the UE-B’s reserved resource is above a RSRP-threshold.

For the resource overlap of PSSCH TX/RX or PSFCH TX/RX overlap as in Figure 1, it is believed that inter-UE coordination scheme 2 is redundant in such case. Instead, UE-A can perform resource re-selection by itself to resolve the conflict, since the conflict is incurred due to UE-A’s PSSCH transmission.

For the resource conflicts between NR PSSCH/PSFCH and another interface/RAT as shown in Figure 3 or Figure 4., either TX/RX of NR PSSCH/PSFCH or TX/RX of another RAT is dropped as specified in Rel-16 NR SL. To protect transmissions of both NR SL and other RAT, it is straightforward to trigger UE-B to reselect its NR SL resources before occurrence of the conflict.

Proposal 8: For scheme 2, the expected/potential resource conflict at UE-A includes,
-	PSSCH resource collision on the resource reserved by UE-B, FFS definition of resource collision.
-	Overlap between UE-A’s UL/LTE resource and PSSCH resource reserved by UE-B.
-	Overlap between UE-A’s UL/LTE resource and resource carrying PSFCH feedback to UE-B.

In another approach of scheme 2, RX UE as UE-A sends NACK/DTX to trigger resource reselection of TX UE (UE-B). Hence, ‘a set of resource’ is PSCCH/PSSCH transmission resource where PSCCH/PSSCH decoding failure occurs.

Proposal 9: For scheme 2, the detected resource conflict refers to the resource conveying PSCCH/PSSCH transmission from UE-B to UE-A, on which PSCCH/PSSCH decoding failure occurs.

  - Enhancement on resource selection procedure

Agreement: - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission - For scheme 1: - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information - For scheme 2: - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

In RAN1#104bis-e e-meeting, how UE-B to use the coordination information has been discussed and several options have been identified. In this section, further discussion based on the agreements are made.

For scheme 1, the coordination information comes from intended receiver of UE-B or leading-UE. Considering performance of the transmission from UE-B to UE-A or the performance of a whole UE group, smart UE implementation should allow UE-B to select transmission resources from the recommended resources. Especially for the case when UE-B does not perform sensing, UE-B can select transmission resource only based on the coordination information.

Proposal 10: For scheme 1, UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information.

Moreover, there would be different scenarios for inter-UE coordination. In some scenarios, the advanced UEs may need to consider both sensing information and coordination information in transmission resource selection. For example, when there are UEs performing sensing based resource selection in the resource pool, the resources recommended by UE-A may incur severe interference to proximity-UEs of UE-B, since the determination of the recommended resource does not consider interference situation around UE-B. To guarantee performance of the TB transmission of proximity-UEs of UE-B, UE-B needs to jointly consider UE-B’s sensing result and coordination information for the transmission resource selection. Regarding how to combine UE-B’s sensing result and coordination information, enhancement of mode 2 procedure based on coordination information can be considered, e.g., UE-B selects resource from the union of the set of identified candidate resources and the recommended resources, and UE-B can prioritize the resource in the intersection of UE-B’s candidate resource set and recommended resources.

Proposal 11: For scheme 1, UE-B’s resource(s) to be used for its transmission resource (re)-selection is(are) based on both UE-B’s sensing result (if available) and the received coordination information.
-	 UE-B selects transmission resource(s) from the union of its set of identified candidate resources and the recommended resources.
-	Prioritize resources in the intersection of the set of candidate resources identified by UE-B and the recommended resources.

  - Signaling aspect

    - Trigger of coordination information transmission

Regarding transmission of inter-UE coordination information, periodic transmission or UE-B triggered transmission or event-triggered transmission can be considered.

The periodic coordination information transmission is applicable for scheme 1. For example, RX UE/header UE can provide coordination information to TX UE periodically, and the coordination information is mainly used for transmission of periodic traffic.

UE-B triggered coordination information transmission can be considered scheme 1. For example, when the UE-B has packet transmission or when the UE-B detects NACK/DTX, it can trigger the UE-A to feedback coordination information. In this case, the signaling to convey the triggering information needs to be specified.

Event-triggered coordination information transmission can be considered for scheme 2. When resource conflict or transmission decoding failure occurs at UE-A, UE-A as RX UE can send the coordination information.

Proposal 12: Support periodic and UE-B triggered coordination information transmission for scheme 1.
-	FFS when UE-B triggers the coordination information transmission.

Proposal 13: Support event-triggered coordination information transmission for scheme 2, FFS the event definition.

    - Container

For scheme 1, the size of coordination information can be up to tens of or even hundreds of bits to convey the information such as time-frequency location of recommended resource, interference level on the recommended resource, UE ID information, etc. Hence, PSSCH is the proper channel to deliver such signaling due to its large capacity. Regarding the signaling, it is preferred to use 2nd stage SCI or MAC CE for coordination information transmission, since the processing delay of 2nd stage SCI and MAC CE is small enough, which can be used for both dynamic and semi-static coordination information transmission.

Moreover, for UE-B triggered coordination information transmission, UE-B needs to send additional triggering signaling. The triggering signaling can be up to several bits or bytes to convey the information such as amount of data of UE-B’s transmission, priority information of UE-B’s transmission, etc. Considering the similarity between the coordination information and triggering signaling, 2nd stage SCI or MAC CE is preferred as container of the triggering signaling.

Proposal 14: MAC CE or 2nd stage SCI is used to deliver coordination information for scheme 1.

Proposal 15: For UE-B triggered coordination information transmission in scheme 1, MAC CE or 2nd stage SCI is used to convey the trigger.

For scheme 2, it seems a 1-bit flag is enough to trigger resource reselection. From perspective of signaling overhead reduction, PSFCH or a new PSFCH-like channel is more proper to deliver such signaling.

Proposal 16: PSFCH or PSFCH-like channel is used to indicate the presence of resource conflict in scheme 2.

    - Signaling transmission timeline

For scheme 1, the timeline for trigger and coordination information transmission is illustrated in Figure 5. When UE-B has TB transmission at time instant n, UE-Bs send trigger signaling within an interval bounded by n+alpha to request coordination information from UE-A. After receiving the request, UE-A determines and sends coordination information to UE-B within another interval bounded by m+beta, where m is the time instant of the trigger signaling. In order to deliver the coordination information in time and to guarantee the system performance in case of signaling transmission, RAN1 should define timeline/resource selection procedure for the signaling delivery.

Figure 5 signaling transmission timeline of scheme 1

Proposal 17: For scheme 1, the timeline for trigger signaling and coordination information generation/transmission should be specified.
-	The trigger signaling is generated by UE-B when TB is arrived.
-	The coordination information is generated when receiving the trigger signaling from UE-B.

-	The trigger signaling and coordination information are transmitted within pre-determined delay budgets.

For scheme 2, the timing for the coordination information should be defined as well. As illustrated in Figure 6, when UE-B is performing transmission to UE-A, UE-A detects resource collision on the UE-B’s reserved resource at time instant m, and then sends indication to trigger UE-B to reselect the collided resources after m. If PSFCH-like channel is used to indicate the presence of resource conflict, to simplify the system design, the association relationship between the conflict indicator and the conflicted resource can be fixed, similarly as the PSSCH and PSFCH association in Rel-16, The potential difference is that the conflict indicator can be located before the conflicted resource as shown by the red arrow in Figure 6.

Figure 6 coordination information transmission timeline of scheme 2

Proposal 18: For scheme 2, the timing to send the PSFCH-like channel (carrying coordination information) is implicitly associated with the time location of the potential conflicted PSSCH resource.
-	Association between PSSCH occasion and PSFCH occasion can be starting point. 
-	The PSFCH-like channel is transmitted before the potential conflicted PSSCH resource.

  - Evaluation result

Based on the discussed solutions of inter-UE coordination, system level simulation has been performed to evaluate the performance of scheme 1 and scheme 2. In this section, the related results are provided.

Inter-UE coordination scheme 1

In the simulation, the coordination information includes preferred resource for UE-B transmission. When UE-A determines recommended resource, the resources not preferred for UE-B’s transmission are precluded, including resource with high-interference, resource occupied by UE-A for its UL/SL transmission, etc.

Moreover, both hierarchical topology and non-hierarchical topology are simulated. Regarding hierarchical topology, a leading-UE (e.g., RSU) is assumed in a given UE group, and the leading-UE can communicate with the UEs in the UE group. When the leading-UE as UE-A performs reception from multiple UE-Bs, it applies the inter-UE coordination scheme 1 and sends the ‘recommended resource set’ to individual UE-B in the UE group. Regarding non-hierarchical topology, multiple unicast UE pairs are distributed on the roads, each RX UE can send the ‘recommended resource set’ to the associated TX UE.

The general simulation assumptions can be found in Annex I of Table 1. The evaluation results with periodic and aperiodic traffic are shown in Figure 7 - Figure 8 for hierarchical topology, and in Figure 9 - Figure 10 for non-hierarchical topology.

| Figure 7 PRR Performance of hierarchical topology with periodic traffic | Figure 8 PRR Performance of hierarchical topology with aperiodic traffic |
| --- | --- |
| Figure 9 PRR Performance of non-hierarchical topology with periodic traffic | Figure 10 Performance of non-hierarchical topology with aperiodic traffic |

According to the evaluation results above, it is observed that the performance of inter-UE coordination scheme 1 outperform the legacy mode 2 resource selection scheme with higher transmission reliability in both hierarchical topology and non-hierarchical topology. The PRR performance of the inter-UE coordination scheme 1 outperforms mode 2 resource selection by up to 5%-7% and 2%-4% in hierarchical topology and non-hierarchical topology, respectively.

Observation 1: In the case of hierarchical topology, scheme 1 outperforms Rel-16 mode 2 resource selection scheme in the sense of higher transmission reliability, i.e., ~5%-7% PRR improvement assuming 150m communication range.

Observation 2: In the case of non-hierarchical topology, scheme 1 outperforms Rel-16 mode 2 resource selection scheme in the sense of higher transmission reliability, i.e., ~2%-4% PRR improvement assuming 150m communication range.

Inter-UE coordination scheme 2

In the simulation, multiple unicast UE pairs are distributed on the roads, RX UE as UE-A should always monitor the potential/expected resource conflict of the associated TX UE, and trigger resource reselection of the associated TX UE in case of the potential/expected resource conflict.

Two types of potential/expected resource conflicts are simulated. The first type of potential/expected resource conflict is NR PSSCH resource collision, e.g., when RX UE detects potential resource collision between the reserved resources of its TX UE and another UE, it can trigger resource re-selection of the TX UE to resolve the collision. The second type is the resource conflicts between NR PSSCH RX and UL TX from RX UE perspective, e.g., when RX UE detects potential resource conflicts between NR PSSCH RX and UL TX, it can trigger resource re-selection of the TX UE.

The general simulation assumptions can be found in Annex I of Table 2. The evaluation results with periodic and aperiodic traffic are shown in Figure 13 - Figure 14 for the case of NR PSSCH RX and UL TX resource conflict.

| Figure 11 PRR Performance in PSSCH resource collision scenario with periodic traffic | Figure 12 PRR Performance in PSSCH resource collision scenario with aperiodic traffic |
| --- | --- |
| Figure 13 PRR Performance in UL and SL resource collision scenario with periodic traffic | Figure 14 PRR Performance in UL and SL resource collision scenario with aperiodic traffic |

From the evaluation results, it is observed that inter-UE coordination scheme 2 can improve the transmission reliability compared with the legacy mode 2 resource selection. However, the performance gain depends on the detailed solution design for scheme 2. When resource re-selection is triggered by potential/expected NR PSSCH resource collision, the performance gain scheme 2 is less than 0.5%; nevertheless, when resource re-selection is triggered by potential/expected resource conflict between NR PSSCH RX and UL TX, ~1%-2% PRR improvement can be observed from scheme 2 assuming 150m communication range. Thus, it is proposed to further study which sub-scheme is to be support for scheme 2.

Observation 3: For scheme 2, when resource re-selection is triggered by potential/expected resource conflict from RX UE perspective, only slight performance gain, i.e., less than 0.5% PRR improvement, is observed from scheme 2 over Rel-16 mode 2 resource selection scheme.

Observation 4: For scheme 2, when resource re-selection is triggered by potential/expected resource conflict between NR PSSCH RX and UL TX from RX UE perspective, scheme 2 outperforms Rel-16 mode 2 resource selection scheme in the sense of higher transmission reliability, i.e., ~1%-2% PRR improvement assuming 150m communication range.

Proposal 19: RAN1 further studies the sub-scheme(s) to be support for scheme 2.

- Conclusion

This contribution focus on inter-UE coordination mechanism with the following observations and proposals:

Proposal 1: In inter-UE coordination scheme 1, UE-A (RX UE/leading-UE) sends to UE-B (TX UE) a set of resources preferred for UE-B’s transmission to assist its resource selection.

Proposal 2: In inter-UE coordination scheme 2, UE-A (RX UE) detects potential/expected resource conflict of TX UE’s transmission and triggers resource reselection of UE-B (TX UE).

Proposal 3: In inter-UE coordination scheme 2, UE-B (TX UE) performs resource re-selection based on multiple detected NACK/DTX feedback from UE-A (RX UE).

Proposal 4: For scheme 1, the relationship between UE-A and UE-B is pre-determined by high layer.

Proposal 5: For scheme 2, UE-B can only be a UE performing unicast or groupcast transmission, while UE-A is the intended receiver of the transmission.

Proposal 6: For scheme 1, UE-A determines preferred resources for UE-B’s transmission from low interference resources identified by UE-A based on sensing.
-	Legacy candidate resource set identification procedure is the baseline for determination of the low interference resources, assuming the associated PDB/sub-channel size is known by UE-A.

Proposal 7: For scheme 1, it is up to UE-A’s implementation to select recommended resources from the low interference resources.
-	FFS any additional restriction for the resource selection.

Proposal 8: For scheme 2, the expected/potential resource conflict at UE-A includes,
-	PSSCH resource collision on the resource reserved by UE-B, FFS definition of resource collision.
-	Overlap between UE-A’s UL/LTE resource and PSSCH resource reserved by UE-B.
-	Overlap between UE-A’s UL/LTE resource and resource carrying PSFCH feedback to UE-B.

Proposal 9: For scheme 2, the detected resource conflict refers to the resource conveying PSCCH/PSSCH transmission from UE-B to UE-A, on which PSCCH/PSSCH decoding failure occurs.

Proposal 10: For scheme 1, UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information.

Proposal 11: For scheme 1, UE-B’s resource(s) to be used for its transmission resource (re)-selection is(are) based on both UE-B’s sensing result (if available) and the received coordination information.
-	 UE-B selects transmission resource(s) from the union of its set of identified candidate resources and the recommended resources.
-	Prioritize resources in the intersection of the set of candidate resources identified by UE-B and the recommended resources.

Proposal 12: Support periodic and UE-B triggered coordination information transmission for scheme 1.
-	FFS when UE-B triggers the coordination information transmission.

Proposal 13: Support event-triggered coordination information transmission for scheme 2, FFS the event definition.

Proposal 14: MAC CE or 2nd stage SCI is used to deliver coordination information for scheme 1.

Proposal 15: For UE-B triggered coordination information transmission in scheme 1, MAC CE or 2nd stage SCI is used to convey the trigger.

Proposal 16: PSFCH or PSFCH-like channel is used to indicate the presence of resource conflict in scheme 2.

Proposal 17: For scheme 1, the timeline for trigger signaling and coordination information generation/transmission should be specified.
-	The trigger signaling is generated by UE-B when TB is arrived.
-	The coordination information is generated when receiving the trigger signaling from UE-B.

-	The trigger signaling and coordination information are transmitted within pre-determined delay budgets.

Proposal 18: For scheme 2, the timing to send the PSFCH-like channel (carrying coordination information) is implicitly associated with the time location of the potential conflicted PSSCH resource.
-	Association between PSSCH occasion and PSFCH occasion can be starting point. 
-	The PSFCH-like channel is transmitted before the potential conflicted PSSCH resource.

Observation 1: In the case of hierarchical topology, scheme 1 outperforms Rel-16 mode 2 resource selection scheme in the sense of higher transmission reliability, i.e., ~5%-7% PRR improvement assuming 150m communication range.

Observation 2: In the case of non-hierarchical topology, scheme 1 outperforms Rel-16 mode 2 resource selection scheme in the sense of higher transmission reliability, i.e., ~2%-4% PRR improvement assuming 150m communication range.

Observation 3: For scheme 2, when resource re-selection is triggered by potential/expected resource conflict from RX UE perspective, only slight performance gain, i.e., less than 0.5% PRR improvement, is observed from scheme 2 over Rel-16 mode 2 resource selection scheme.

Observation 4: For scheme 2, when resource re-selection is triggered by potential/expected resource conflict between NR PSSCH RX and UL TX from RX UE perspective, scheme 2 outperforms Rel-16 mode 2 resource selection scheme in the sense of higher transmission reliability, i.e., ~1%-2% PRR improvement assuming 150m communication range.

Proposal 19: RAN1 further studies the sub-scheme(s) to be support for scheme 2.

- Reference

- RP-193257, “New WID on NR sidelink enhancement”, RAN#86, Spain, December 2019.

- R1-2101911, “Discussion on mode 2 enhancements”, RAN1#104e, e-meeting, January, 2021.

# Annex I

Table 1 System level simulation assumption for scheme 1 scenario

| Parameter | value |
| --- | --- |
| Deployment | Urban scenario |
| UE type | Vehicle UE |
| Communication type | Unicast |
| Carrier frequency | 6GHz |
| Bandwidth | 40MHz |
| Subcarrier spacing | 30KHz |
| Traffic parameter | Traffic type: Periodic traffic and Aperiodic traffic For Periodic traffic: Traffic load: Medium Intensity Packet arrival interval: 50ms Packet latency requirement: 50ms Packet size: 800 or 1200byte For Aperiodic traffic: Traffic load: Medium Intensity Packet arrival interval: 50ms+exp(50)ms Packet latency requirement: 50ms Packet size: random size from 200 to 2000byte with 200byte step |
| Resource allocation | Mode 2 scheme in Rel-16 and inter-UE coordination scheme |
| Portion of slots for UL transmission | 20% |
| Max HARQ retransmission time | 4 |
| Coordination information transmission | Realistic transmission |

Table 2 System level simulation assumption for scheme 2 scenario

| Parameter | value |
| --- | --- |
| Deployment | Urban scenario |
| UE type | Vehicle UE |
| Communication type | Unicast |
| Carrier frequency | 6GHz |
| Bandwidth | 40MHz |
| Subcarrier spacing | 30KHz |
| Traffic parameter | Traffic type: Periodic traffic and Aperiodic traffic For Periodic traffic: Traffic load: Medium Intensity Packet arrival interval: 50ms Packet latency requirement: 50ms Packet size: 800 or 1200byte For Aperiodic traffic: Traffic load: Medium Intensity Packet arrival interval: 50ms+exp(50)ms Packet latency requirement: 50ms Packet size: random size from 200 to 2000byte with 200byte step |
| Resource allocation | Mode 2 scheme in Rel-16 and inter-UE coordination scheme |
| Portion of slots for UL transmission | 0 |
| Max HARQ retransmission time | 4 |
| Resource reselection trigger condition | RX detects that the reserved resource for TX UE transmission is conflicted with the resource reserved by other UEs or with the UL transmission of itself. |