3GPP TSG RAN WG1 #106bis-e	R1- 2108956

e-Meeting, October 11th – 19th, 2021

Source:	vivo

Title:	Further discussion on SRS enhancement

Agenda Item:	8.1.3

Document for:	Discussion and Decision

Introductions

In RAN#86 meeting, WID [1] on further enhancement of NR MIMO was approved which includes enhancement on SRS, which is copied below for reference.

Enhancement on SRS, targeting both FR1 and FR2:

a.	Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction

b.	Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8})

c.	Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

From the scope in the WID, SRS enhancement for both FR1 and FR2 can be considered in following four aspects:

- Aperiodic SRS triggering;

- Usage reduction;

- SRS switching extension;

- SRS capacity and/or coverage enhancement.

Aperiodic SRS triggering

Agreement

A given aperiodic SRS resource set is transmitted in the (t+1)-th available slot counting from a reference slot, where t is indicated from DCI, or RRC (if only one value of t is configured in RRC), and the candidate values of t at least include 0. Adopt at least one of the following options for the reference slot.

- Opt. 1: Reference slot is the slot with the triggering DCI.

- Opt. 2: Reference slot is the slot indicated by the legacy triggering offset.

- FFS the detailed definition of “available slot” considering UE processing complexity and timeline to determine available slot, potential co-existence with collision handling, etc., e.g.,

  - Based on only RRC configuration, “available slot” is the slot satisfying: there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set

- FFS explicit or implicit indication of t

- FFS whether updating candidate triggering offsets in MAC CE may be beneficial

The following agreements on aperiodic SRS enhancement were made in RAN1#103-e

Agreement

Further study whether and if needed, how to achieve further enhancements on aperiodic SRS triggering and resource management based on repurposing unused fields in DCI format 0_1/0_2 without data and without CSI. Consider the following examples

- CAT A: Time-domain parameters

  - A-1: Indication of available slot position, i.e., the t values

  - A-2: Indication of slot offset

  - A-3: Indication of SRS symbol-level offset

  - A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting

- CAT B: Frequency-domain parameters

  - B-1: Indication of a group of CCs for SRS transmission

  - B-2: Indication of frequency domain resource in a BWP for SRS transmission

  - B-3: Indication of whether DL/UL BWP is applied for SRS transmission

- CAT C: Power control parameters

  - C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’

    - FFS impact on power control, impact from triggering a group of CCs for SRS

  - C-2: Indication of open loop power control parameter e.g., p0.

- CAT D: Spatial-domain parameters, i.e., indication of SRS port and beamforming

- CAT E: Extend the number of DCI codepoints for aperiodic SRS trigger states

- Other examples are not precluded

Agreement

A list of t values is configured in RRC for each SRS resource set. Adopt at least one of the following for DCI indication of t.

- In DCI format 0_1/0_2 without data and without CSI request,

  - Alt 1-1: Reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH

  - Alt 1-2: Re-purpose unused DCI field to indicate t

  - Alt 1-3: t is indicated by a configurable DCI field, where the DCI field may contain bits from unused fields and additional bits configured by gNB

    - FFS design details with other potential field(s)

  - FFS: whether t can be slot offset

- In DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH

  - Alt 2-1: t is indicated by adding a new configurable DCI field

  - Alt 2-2: t is indicated without adding DCI payload

- Note: The size of DCI payload does not change dynamically

- Note: RAN1 should strive for unified solution for different DCI formats.

- FFS: The number of RRC configured t values per SRS resource set and DCI bit field size.

Agreement

Support at least DCI 0_1 and 0_2 to trigger aperiodic SRS without data and without CSI.

- FFS whether/how to re-purpose the unused fields, e.g., the triggering offset(s) and the frequency resources for triggering A-SRS on one or more component carriers, SFI-index, etc.

- FFS UL/DL DCI with data for aperiodic SRS

- FFS group common DCI

In RAN1 # 104-e meeting, further agreements were made as below:

Agreement

Support Opt. 2: Reference slot is the slot indicated by the legacy triggering offset.

- If DCI is transmitted in slot n, and k is the legacy triggering offset, reference slot is slot n+k.

- Note: the legacy triggering offset can be 0, if slotOffset is absent.

In RAN1 # 106-e meeting, further agreement was made for ‘t’ values as below:

Agreement

Confirm the following WA:

For DCI indication of “t” in Rel-17 SRS triggering offset enhancement

- For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request

  - t is indicated by adding a new configurable DCI field (up to 2 bits)

    - Applies only when there are multiple   candidate values of t configured

  - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

In this section, we will discuss aperiodic SRS triggering enhancement, such as slot offset indication in case of scheduled DCI and non-scheduled DCI, collision handling and group common DCI enhancement.

## Slot offset indication

Rel-15 supports aperiodic SRS triggering by UL DCI format 0-1 and 0-2 without scheduling PUSCH but with non-zeros CSI request indicator for triggering aperiodic CSI measurement only. UL DCI used for scheduling data or not is determined by UL-SCH indicator in its bit field. In previous meeting [3], it was agreed that aperiodic SRS can be triggered by UL DCI format 0-1 and 0-2 without scheduling PUSCH and with all-zeros CSI request indicator, which represents the UL DCI is used only for triggering aperiodic SRS. Therefore, for UL DCI, the function of triggering aperiodic SRS only or triggering both of SRS and CSI measurement is controlled by configuring a specific value in UL-SCH and CSI request indicator.

Besides, SRS request field can also be included in DL DCI with no limitation on configuring all-zeros FDRA indicator and non-zeros SRS request field for triggering aperiodic SRS only in current specification. For example, a typical DL heavy use case with two component carriers, it is possible that PUSCH-config is only configured on the PCell but not on the SCell. Such kind of configuration not only saves RRC signaling overhead but also alleviates the gNB from constructing DCI format 0_1 and 0_2 for dynamic scheduling. Thus, under such scenarios, SRS triggering with non-scheduled DCI format 1_1 and 1_2 should also be supported since DCI format 0_1 and 0_2 may not be present if PUSCH_config is not configured.

Support to trigger aperiodic SRS by non-scheduled DCI format 1-1 and 1-2 with minimal specification impact for SRS triggering enhancement.

In RAN1#104 meeting, following alternatives were agreed in the case of non-scheduled DCI,

- Alt 1-1: Reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH

- Alt 1-2: Re-purpose unused DCI field to indicate t

- Alt 1-3: t is indicated by a configurable DCI field, where the DCI field may contain bits from unused fields and additional bits configured by gNB

  - FFS design details with other potential field(s)

  - FFS: whether t can be slot offset

From our understanding, the indication method of Alt 1-1 is based on available slot offset indication scheme without any additional new fields in DCI, i.e. reuse SRS request field as we discussed above. Aperiodic SRS triggering and available slot offset indication are achieved with SRS request field which increases complexity in aperiodic SRS resource set configuration. For Alt 1-2, some unused field is used for indicating slot offset in a DCI format, while in Alt 1-3 slot offset is indicated by some unused fields in a DCI format and a configurable slot offset field together. Thus, we prefer Alt 1-2 and Alt 1-3.

Furthermore, many unused fields can be re-purposed not only for slot offset indication of aperiodic SRS but also for other purposes, such as modifying SRS bandwidth, comb size value, comb offset value, and etc. 5 categories of use cases were listed for repurposing unused fields in DCI format 0-1 and 0-2 without data and without CSI in RAN1#104 meeting.

Figure 2-1: Slot offset indication by non-scheduled DCI

It was agreed that reference slot is the slot indicated by the legacy triggering offset and dynamic available slot offset indicated in DCI is better solution with minimal DCI payload increase.  However, there are several unused fields which can be repurposed to indicate slot offset directly rather than available slot offset concept in the above two scenarios. Thus, with non-scheduling DCI more flexible aperiodic SRS triggering with lower UE processing complexity can be realized. For example, as shown in Figure 2-1, an aperiodic SRS is transmitted in slot n + 3 with slot offset = 1 indicated in DCI; or the SRS transmission occurs on slot n + 6 as 4 slots offset indicated in DCI with addition of 2 slot offset configured by RRC. Thus, we suggest to support direct slot offset indication for aperiodic SRS triggering using non-scheduling DCI rather than available slot offset indication when the new DCI field is absent.

Support direct slot offset indication by repurposing unused fields for aperiodic SRS triggering when the new DCI field is absent for non-scheduling DCI

Besides, group common DCI 2-3 can be used for triggering a group of CCs for aperiodic SRS transmission for carrier switching or closed loop power control value in Rel-15/16, and as discussed in section 2.4 that some unused bits or configurations can be re-interpreted for non-carrier switching purpose for DCI format 2-3 with minimum specification impact. Therefore, no need to enhance power control parameter and frequency domain parameter for non-scheduling DCI, i.e. CAT B and CAT C. According to above discussion, we have the following proposals,

Do not support power control parameter and frequency domain parameter enhancement in non-scheduling DCI.

## Collison handling

Figure 2-2: Collision with different offset indication

If more than one aperiodic SRS sets are triggered at the same time by one DCI with same available slot offset indication, the SRS resources in these sets should be configured in non-overlapping symbol(s) if the SRS resources are transmitted in same slot, which is guaranteed by network configuration. However, when available valid slot offset is indicated for each aperiodic SRS resource set triggered by one DCI, it is possible that transmission of 2 or more SRS resource sets are delayed such that SRS resources in multiple sets fall in one slot. For example, as depicted in Figure 2-2, legacy slot offset is configured as 0, and available slot offset is equal to 2 and 1, which indicates the 2nd available slot and the 1st available slot after slot n + legacy slot offset (‘0’), for the SRS resource set 1 and SRS resource set 2, respectively. If the set of symbols in slot n+1 are configured as flexible and indicated as DL + UL, by definition slot n+1 and slot n+2 are available slots thus according to available slot offset the UE is supposed to transmit SRS set 1 in slot n+2. By contrast, slot n+2 is the first available slot for SRS set 2 since the set of DL symbols in slot n+1 is overlapped with the symbols for SRS set 1 transmission. Thus, SRS set 1 and SRS set 2 are overlapping on symbol level, even if different available slot offset is configured.

Even with different available slot offset indication for two aperiodic SRS resource sets triggered by one or more DCI, collision between two SRS can happen.

Figure 2-3: Collision with two SRS on different CCs configured with different offset indication

In UL CA, for SRS for antenna switching + SRS for codebook/non-codebook/BM/antenna switching case, the simultaneous transmission of SRS on different CCs depends on UE capability. Thus, in UL CA, collision between two SRS from different carriers is possible when simultaneous transmission of those two SRS on different CCs is not supported. Taking figure 2-3 as an example, SRS set 1 is transmitted on slot n+2 on CC1 and SRS set 2 is also transmitted on slot n+2 on CC2 even with different available slot offset indication between two SRS.

SRS transmission collision may happen for two aperiodic SRS resource sets triggered in case of UL CA, even with different available slot offset indication, if simultaneous transmission of SRS on difference CCs is not supported.

To address above scenarios, some principle of priority to handle collision in an available slot should be considered.

Although priority can be based on Usage or the order of triggering DCI, it cannot solve collision if multiple sets configured with same usage or simultaneously triggered by multiple DCI are overlapping in time domain across CCs. Thus, as single dropping rule is not sufficient, we propose a combined dropping rule, including usage, order of triggering DCI, CC ID and set ID, to handle collision.

Consider a combined dropping rule, including usage, order of triggering DCI, CC ID and set ID, to resolve SRS collision when more than one SRS resource sets are triggered by one or more DCI on same CC or different CCs.

## Group common DCI

The purpose of flexible triggering of aperiodic SRS transmission is to avoid PDCCH congestion for scheduling multiple UEs or triggering multiple SRS. Other than available slot mechanism discussed above, a group common signaling, such as DCI format 2-3, can be considered for triggering multiple SRS for more than one UEs or more than one CCs. In NR Rel-15, group common DCI format 2-3 is used for SRS carrier switching and/or SRS power control. To enable more flexible aperiodic SRS triggering and reduce DCI overhead, DCI format 2-3 can be enhanced with minimum specification impact on current SRS carrier switching mechanism.

There are two types of indication schemes, i.e. type A and type B, in current carrier switching method. For carrier switching type A, each block in DCI format 2-3 includes one SRS request field and at least one TPC field corresponding to at least one CC. The function of SRS request is used to select one CC set from at most 4 CC sets rather than triggering aperiodic SRS. Specifically, the codepoint of SRS request 00 represents no CC set triggered, while codepoint 01, 10 and 11 choose the 1st CC set, 2nd CC set and 3rd CC which are corresponding to CC set index 0,1 and 2 respectively. As at most 4 CC sets can be configured in SRS carrier switching, the 4th CC sets could be configured as non-carrier switching if it is configured by network. Thus, an extra indictor can be used to select the 4th CC set for non-carrier switching.

For carrier switching type B, each block in DCI format 2-3 maps with one CC and includes an SRS request field and a TPC field. By contrast, the SRS request field is used for triggering aperiodic SRS directly. Since it is limited to triggering SRS resource set with antenna switching usage only, the SRS request field can be reduced to 1 bit as an indicator for non-triggering or triggering common for carrier switching type B. Therefore, the remaining bit in SRS request field can be re-interpreted for other purposes.

Based on above analysis, some unused bits or configurations can be re-interpreted for non-carrier switching purpose for DCI format 2-3 with both of type A and type B structures.

DCI format 2-3 can be enhanced with minimum specification impact on current SRS carrier switching mechanism to achieve more flexible aperiodic SRS triggering and reduce probability of PDCCH congestion.

Support re-interpretation of unused bits or configurations in DCI format 2-3 for triggering multiple SRS transmissions on different CCs and/or different UEs.

Overhead and Usage reduction

Agreement

For SRS overhead reduction, study reusing same resources among multiple usages, at least for “codebook” and “antenna switching”. Study aspects include

  - Whether implementation approach based on legacy SRS configuration is sufficient

If not, and if there are benefits other than RRC overhead reduction, study further on the case that antenna switching and PUSCH have different number of Tx antennas, whether UL BWP for different SRS usages is the same or different, whether and how to ensure UE to use same virtualization, the set of applicable usages, UE implementation complexity and overhead, etc.

In RAN1#102-e meeting [2], following agreement for usage reduction was made:

## SRS for codebook used for DL CSI acquisition

In Rel-15, gNB configures SRS set with usage ‘antenna switching’ for DL CSI acquisition and SRS set with usage ‘codebook’ for UL CSI acquisition separately. For example, for a UE with 2 Tx and 2 Rx antennas, gNB still has to configure separate SRS sets for 2T2R antenna switching (actually there is no antenna switching operation at the UE) and 2 Tx codebook-based operation. Although the specification does not prevent the SRS resource in these two SRS resource sets having same SRS resource ID, this implies that gNB always has to configure two SRS resource sets with same time domain behavior related to same SRS resource.

Besides, when these two sets are configured as aperiodic with different slot offset configuration, UE will transmit both in different time locations if triggered separately or if configured with different slot offset. Consequently, it causes unnecessary signaling overhead in both of RRC configuration and/or DCI triggering as well as UL transmission resources.

Therefore, current implementation approach for different usage sharing scheme is not sufficient. And for the case of antenna switching with xTxR, the SRS resource set with usage ‘codebook’ or ‘antenna switching’ can be used for either DL CSI acquisition or UL CSI acquisition. In consideration of different antenna virtualization implementation in different usage, we thus propose at least SRS resource set with usage of codebook can be used for DL CSI acquisition in the case of xTxR.

SRS resource set for ‘codebook’ or ‘antenna switching’ should be configured separately.

For the case of two SRS resource sets related to same SRS resource, same time domain behavior should be configured across these two shared SRS resource sets.

UE may transmit an SRS resource twice at different time locations for acquiring DL CSI and UL CSI.

Current implementation approach for different usage sharing scheme is not sufficient.

At least support SRS resource set with usage of codebook to be used for DL CSI acquisition in the case of xTxR.

Figure 3-1: Normal resource sharing scheme for codebook and antenna switching

Furthermore, SRS resource set configured for xTyR with periodic or aperiodic, one of the SRS resources can be used for UL CSI acquisition where SRS with usage ‘codebook’ is not configured. For example, a UE supporting 2T4R capability is configured with SRS resource set with two 2-port SRS resources and usage ‘antenna switching’ and/or one 2-port SRS resource with usage ‘codebook’ as shown in Figure 3-1. Current spec allows overlapping configuration of SRS resources for ‘codebook’ and ‘antenna switching’, and triggering of both SRS sets simultaneously. However, when gNB triggers SRS resource set for ‘antenna switching’ only, one of SRS resources (e.g. SRS resource 0) in antenna switching set can be used for UL CSI acquisition, that means gNB derives CSI for PUSCH scheduling based on resource 0 and the UE shall transmit PUSCH using same antenna(s) used for transmission of resource 0. This gives flexibility in SRS configuration and aperiodic triggering while reducing RRC configuration and PDCCH overhead.

Figure 3-2: Resource sharing for codebook and antenna switching with different time domain behavior

Above example of SRS resource sharing is limited by same time domain behavior across shared SRS sets. However, similar approach can be utilized for SRS resource sharing in the case of SRS sets configured with different time domain behavior. Similarly, take an example of 2T4R capable UE, gNB configures 2-port periodic SRS with usage ‘codebook’, and SRS set with usage ‘antenna switching’ can be configured with one 2-port SRS resource on different symbols than the SRS for ‘codebook’ and preserving at least one symbol gap in between as shown in Figure 3-2. In this case gNB can aperiodically trigger one SRS resource for ‘antenna switching’, UE transmits SRS resource for ‘antenna switching’ using different antennas than those used to transmit SRS for ‘codebook’.

More specifically, one SRS resource can be reused for two different usage of codebook and antenna switching with different time domain behavior, and number of SRS resources in the set of enhanced antenna switching is less than the number of SRS resource specified in Rel-15 antenna switching set. Antenna switching SRS can be triggered aperiodically on the same slot when periodic SRS for codebook is transmitted. We thus propose,

Support resource sharing scheme for usage ‘antenna switching’ and ‘codebook’ configured with different time domain behavior.

## SRS for antenna switching used for UL CSI acquisition

Similarly, when SRS for ‘antenna switching’ can be naturally utilized for UL CSI acquisition, in this case at least same antenna virtualization shall be maintained by UE for transmission of SRS which is being shared for usage ‘codebook’ and transmission of subsequent PUSCH. Although there are multiple SRS resources in a set for ‘antenna switching’, when gNB schedules PUSCH a TPMI is indicated without SRI which is implicitly corresponding to one of the SRS resources in the set.

At least for the case of same antenna virtualization between antenna switching and codebook-based transmission, considering one of antenna switching resource can be used for UL CSI acquisition.

Above discussion is mainly focused on SRS resource sharing for xT codebook and xTyR antenna switching, whose value of T is equal. Below, we analyze a case of SRS resource sharing with different value of transmitting port for codebook and antenna switching. For example, a UE supports 2Tx for codebook and 1T4R for antenna switching, for such a UE gNB configures one 2-port SRS resource and four 1-port SRS resources that are configured on different symbols or different slots if only last 6 symbols in a slot can be used for SRS resource. Now the question is whether the four 1-port SRS resources can be shared with 2-port SRS resource. For 2Tx UE configured with 2-port SRS resource, both ports are configured on same symbol with shared power between two Tx (or two ports), i.e. if total transmission power is P then each Tx transmission power is P/2.

Figure 3-3: 2Tx codebook and 1T4R antenna switching resource sharing

If one of the SRS resources for 1T4R antenna switching is shared with 2-port SRS, one possible SRS resource configuration is as shown in Figure 3-3. 2-port SRS is transmitted using 2 Tx antennas, and 1-port SRS resource is transmitted using 1 Tx antenna. From RAN1 perspective, it is unclear whether specific UE hardware design for Tx antennas switching is considered for such SRS resource sharing. Further clarification and discussion are needed on viability and usefulness of such type of SRS resource sharing.

Whether SRS for 2 Tx codebook-based operation and SRS for 1T4R antenna switching can share resource needs further discussion.

## SRS for codebook and BM

In NR Rel-15/16, only one SRS resource set can be configured with up to two SRS resources with usage ‘codebook’, and one of purpose of configuring two SRS resources could be to support transmission of SRS resources with different spatial relation information which means SRS resources with different spatial relation information within a set can be used to indicate different UL beams. However, two UL beams may be sufficient for FR1 deployment. More beams, such as 4 or 8 beams, are needed for efficient operation in FR2, thus increasing maximum number of SRS resources within a set with usage ‘codebook’ can be considered in Rel-17.

Since spatial relation information is semi-statically configured on SRS resource level and large number of SRS resources within a set will increase DCI overhead. Similar to TCI states in DCI, MAC CE activation of subset of SRS resources can be considered as well as updating spatial relation information. A UE only transmits activated SRS resources within a set thus limiting the size of SRI field in DCI. In our view, at least the current size of SRI field should be extended to support dynamic beam indication in UL.

Allow extending the number of SRS resources in an SRS resource set with usage of ‘codebook’ and resource sharing between codebook and beam management for more flexible UL beam indication.

SRS switching extension

Agreement

Support 4T6R SRS antenna switching in Rel-17.

Agreement

- On the presence of guard symbols in Rel-17 for SRS antenna switching, down-select one of the following

  - Alt 1-0: Guard symbols are always-on, which is same as Rel-15

  - Alt 1-1: Guard symbols are configurable subject to UE capability

- On whether to introduce guard symbols between SRS resource sets for antenna switching, down-select one of the following

  - Alt 2-0: Do not introduce guard symbols between SRS resource sets, i.e., guard symbols only appears between SRS resources in a resource set

  - Alt 2-1: Introduce guard symbols between two sets mapped to consecutive slots

- Note: Rel-15 guard period symbols are supported if none of the above enhancements is agreed

Agreement

For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS.

- Note: the two SP-SRS resource sets are not activated at the same time

- For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS

- Applies for all supported xTyR where y<=8

- For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources.

This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS

Agreement

For aperiodic xTyR antenna switching SRS, where xTyR is from {1T6R, 1T8R, 2T6R, 2T8R, 4T8R}, support all the non-zero integer values N<=N_max except N=1 for 1T8R

- For each xTyR configuration, UE does not expect multiple SRS resource sets are configured or triggered in one slot

- UE does not expect that the OFDM symbols contained in one SRS resource set exceed UE capability on which OFDM symbols can be used for SRS taking guard period into account

In RAN1 # 106-e meeting, further agreements were made as below:

## SRS switching enhancement for up to 8 antennas

In this section, we will discuss guard period enhancement required for antenna switching. Usage with antenna switching is different from other usage cases above in the sense that a Tx chain should be switched from one antenna to another antenna, and, hence, would require a specific configuration between two antenna switching SRS resources.

In current specification, the guard symbol/period is defined between the SRS resources within a set in current 38.214[7]. Only last 6 symbols in a slot can be configured for SRS, and the number of SRS resource set is restricted to 1 for periodic antenna switching SRS and up to 2 with different slot offset configuration for aperiodic antenna switching SRS. A minimal gap between two SRS resource sets with antenna switching usage is equal to 8 symbols which certainly fulfills the minimum guard period restriction. As a consequence, no need to define guard period between two antenna switching SRS resource sets in Rel-15/16.

- Figure 4-1: zero symbol gap between two SRS resource in adjacent slots

RAN1 agreed that no further guard period enhancement is needed for multiple antenna switching sets in same slot as it was agreed UE does not expect multiple SRS resource sets are configured or triggered in one slot in previous meeting. However, for the case of 2 sets in adjacent slots, as depicted in Figure 4-1, the minimal gap between the two sets is equal to 0 which does not meet the capability requirement for antenna switching, if a resource in the first set is configured on the last symbol within the first slot and another resource in the second set is configured on the first symbol within the second slot. Thus, the guard period should be enhanced to support 2 SRS sets for antenna switching located in adjacent slots, i.e. Alt 2-1.

Guard period should be enhanced to support 2 SRS sets for antenna switching located in adjacent slots, i.e. Alt 2-1.

Symbols between two antenna switching SRS resources in an SRS resource set is defined as guard symbols and is not allowed to transmit any other signals. Furthermore, if the original guard period definition is applied across SRS sets with antenna switch usage, many OFDM symbols will be wasted and, thus, would deteriorate spectral efficiency. It is better to make the guard period configurable according to UE capability. In addition, if one antenna switching resource transmitted on same antenna with PUSCH or other SRS resources for UL purpose, it is not necessary to configure guard symbols between the antenna switching set and UL signals. We, thus, propose,

Support guard period configurable for antenna switching resource, i.e. Alt 1-1.

Guard period can be absent before or after an antenna switching resource if the resource and other UL signals are transmitted on same physical antenna.

Lastly, we discuss flexible antenna switching issue. Some companies propose MAC CE based flexible antenna switching resources activation and deactivation, and gNB indicated antenna switching configuration should not be beyond supported configurations indicated by UE capability signaling. We are open to discuss or support this feature. However, from UE’s perspective, time slots for antenna switching resource(s) activation, i.e. required time for turning ON Rx front end, may be different across realistic UEs due to various capabilities. By contrast, for reducing the number of Rx front end may not impact the performance, deactivation time slots can be zero. Thus, a new capability should be introduced at least for activation of Rx front end when dynamically indicated number of Rx antennas is larger than previous configuration. For example, a UE reports antenna switching capability as {1t1r, 1t2r, 1t4r} and for some reason (e.g. power saving) only 2 Rx antennas are active, for this UE when dynamically indicated with 1t4r antenna switching, additional RF activation time is required.

Introduce a new capability to report activation time (slots or symbols) if MAC CE based flexible antenna switching scheme is supported.

Figure 4-2: Two effective timelines for flexible antenna switching if Y < X

Figure 4-3: Two effective timelines for flexible antenna switching if Y > X

An effective timeline has been specified after a MAC CE signaling received, i.e. slot n + 3ms where the UE would transmit a HARQ-ACK information in slot n corresponding to the MAC CE. Let the timeline related to MAC CE as slot n + X and the second timeline related to UE activating RF front ends for flexible antenna switching as slot n + Y, where the UE transmits a HARQ-ACK information in slot n corresponding to a MAC CE for flexible antenna switching activation, X means the MAC CE effective time slots, and Y is antenna switching resources activation time slots. If Y < X, as depicted in figure 4-2, it seems ok when only applying original MAC CE timeline, i.e. slot n + X. However, if Y > X shown in figure 4-3, applying the same mechanism for a MAC CE effective timeline may not be suitable for the MAC CE used for flexible antenna switching as activation processing is not completed. Therefore, if MAC CE based flexible antenna switching resources activation and deactivation is supported, it should be clarified that which timeline should be applied to a UE when two timelines occur in same UE, i.e. timeline related to MAC CE and timeline related to antenna switching resources activation capability. To solve above timeline issue, instead of 3ms max {3ms, Y} should be applied, which represents slot n + max {3ms, Y}, can be a simple solution for effective timeline determination for MAC CE based flexible antenna switching scheme. We, thus, propose,

For activation processing, support enhancement on effective timeline for MAC CE based flexible antenna switching scheme as slot n + max {3ms, Y}, where the UE transmits a HARQ-ACK information in slot n corresponding to a MAC CE for flexible antenna switching activation, and Y is antenna switching resources activation time related to UE capability report.

## SRS antenna switching enhancement for FR2/MTRP

In FR2, concept of antenna panel is mostly used, where a panel may represent an integrated module including at least transceiver and antenna(s). Assuming dual-polarized antenna array structure, it can be envisioned there are at least two antenna ports per panel. And, it can also be assumed that the number of TX antenna port are equal to RX antenna port number per panel, hence antenna switching may only take place across panels. However, intra-panel switching may also occur between two polarizations within a panel if hardware design so, where the switching across two polarizations would be faster than switching across panels. Besides, based on MPUE assumption 1 and assumption 3, i.e. whether only one panel is active at a time or multiple panels are active simultaneously but only one panel is used for transmission, the distinction between these two assumptions will lead to different panel switching period requirements. Both MPUE 1 and MPUE 3 should be considered in SRS antenna switching enhancement in FR2.

In FR2, it should be further studied whether to support intra-panel switching.

The impact of MPUE assumption should be considered into SRS antenna switching enhancement.

Another problem is that if single SRS resource set group is configured for DL CSI acquisition in MTRP scenario, it represents same power control parameter and/or spatial relation information shared across TRPs. The SRS resource set group can be defined as current configurations in specification for Rx antennas less than or equal to 4 and enhancement for Rx antennas up to 8 in antenna switching case with single TRP.

As spatial relation information is not configured in SRS resource, SRS resource may be received in multiple TRPs by gNB due to realistic UE antenna structures especially in FR1. It shall increase interference in other TRPs if power control parameters are configured and calculated based on the worst TRP, while DL CSI may not be obtained in some TRPs if the relative parameters are provided based on the best TRP.

In FR2, although it has been defined that UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set with antenna switching usage, relationship of spatial information between SRS resource sets within a group is not specified. Therefore, enhancement is needed to obtain full DL CSI that is configured with same spatial information in SRS resources within an SRS group. Thus, some TRPs cannot receive an SRS resource set group correctly if spatial relation configured in the group is pointed to the rest of TRPs.

Therefore, based on the agreement in MTRP scenario, the number of codebook and non-codebook sets is doubled for transmitting multiple SRS resources in two sets with different beam and power control configuration. Similar enhancement, i.e. the number of SRS resource set group doubled, should be agreed in antenna switching with MTRP scenario especially for FR2 case. Furthermore, the total number of SRS resource set for DL CSI acquisition increases rapidly with the number of SRS resource set group increasing. Since many parameters in an SRS resource or SRS resource set should be configured with same values within an SRS resource set group, the majority of parameters, including spatial relation information, P0, alpha, pathloss RS and etc., are duplicated which increases signaling overhead. Hence, it is preferred that configuration mechanism should be optimized at least in antenna switching scenario.

Enhancement is needed to clarify association of spatial relation information between SRS resource sets within an SRS resource set group for obtaining full DL CSI.

The number of SRS resource set group should be doubled for antenna switching in MTRP scenario.

Since the total number of SRS resource set for DL CSI acquisition increases rapidly with the number of SRS resource set group increasing, consider optimization of configuration mechanism in MTRP scenario for reducing overhead.

SRS capacity and/or coverage enhancement

Agreement

Study the maximum number of cyclic shifts for Comb-8 in Rel-17, with the following alternatives as starting points

- Alt 1: The maximum number of CSs for Comb-8 is 6

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

Agreement

For RPFS SRS in Rel-17, adopt one of the following alternatives for sequence generation, where no new sequence length other than the ones supported in the current spec is introduced (to be decided in RAN1#105-e)

- Alt 1: Generate length-12PFmSRS, BSRSComb ZC sequence

- Alt 2: Truncate from legacy length-12⋅mSRS, BSRSComb sequence according to the location of RPFS SRS

Agreement

- For RPFS in Rel-17, support PF = {2, 4}.

- FFS  3, 8, 12, 16 or fractional numbers

- Support at least one of the following alternatives (to be decided in RAN1#105-e)

  - Alt 1: 1PFmSRS, BSRS is an integer value

  - Alt 2: 1PFmSRS, BSRS is an integer value with minimum value 4

  - Alt 3:  is a multiple of 4

  - Alt 4: Round 1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2

Following agreement for partial frequency sounding was made in RAN1#104bis-e,

Agreement

For Comb-8 SRS in Rel-17, down-select one of the following in RAN1#106bis-e

- Alt 1: The maximum number of CSs for Comb-8 is 6

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

Agreement

For RPFS SRS sequence generation, support

- Alt 1: Generate length-12PFmSRS, BSRSComb ZC sequence.

Agreement

Support start RB location (Noffset) hopping in different SRS frequency hopping periods for RPFS and at least periodic/semi-persistent SRS, where Noffset Noffset  is the start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs.

- For a given SRS transmission occasion, Noffset=kF+khopping mod PFPFmSRS, BSRS , where khopping is same for all SRS occasions within a legacy FH period but changes across legacy FH periods, kF and PF are at least configured by RRC signaling (kF = {0, 1, …, PF-1}).

- Support at least one pattern for khopping in time domain, FFS detailed pattern

- Note: the legacy FH period is the period to sound the full SRS hopping bandwidth across the different subbands of mSRS, BSRS RBs each.

- This start RB location hopping is enabled or disabled by RRC signaling.

- FFS whether MAC CE or DCI can be additionally used

- When this start RB location hopping is disabled,khopping khopping is fixed to be 0 for all SRS symbols

- This start RB location hopping is UE optional.

FFS whether start RB location hopping is also applicable on SRS occasion(s) within one FH period (e.g., when R>1) and/or on aperiodic SRS, if so, how

Agreement

For RB-level partial frequency sounding (RPFS) in Rel-17

- The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kFPFmSRS, BSRS, where kF = {0, …, PF-1}

  - FFS support start RB location (Noffset) hopping in different SRS occasions, symbols or frequency hopping periods, and if supported, detailed hopping pattern

- Support to determine PF and Noffset at least via RRC configuration per SRS resource.

  - FFS whether to introduce DCI and/or MAC CE in addition

In RAN1 # 106-e meeting, further agreements were made as below

## RB-level partial sounding

- Before discussing hopping issues on partial frequency sounding, sequence length determination of an SRS resource should be clarified firstly. In 38.211 [6], Low-PAPR sequence generation type 1 is used to generate sequence of an SRS resource, and there are two categories, one is for base sequence length equal or larger than 36 and another is for sequence length smaller than 36 with specific length, i.e. 6,12,18,24 and 30. Thus, in theory, following sequence length of an SRS resource are supported to {6,12,18,24,30,36,37,38,….} without regarding to 4*N RBs bandwidth configurations.

- Partial frequency sounding in RB level and comb 8 as enhancement of SRS are already agreed. For comb 8 without enabling partial frequency sounding, the minimum bandwidth is 4 RBs which gives the SRS sequence length equal to 6, and other bandwidth configurations can also fulfill current type 1 sequence generator. Thus, there is no issue on sequence for enabling comb 8 feature only in the system. However, it causes mismatch between SRS sequence length after partial frequency sounding in RB level with or without comb 8 function and sequence length supported in current specification. For example, if an SRS resource is configured with 4 RBs bandwidth, PF value = 2 and comb value = 8 or configured with 4 RBs bandwidth, PF value = 4 and comb value = 4, sequence length of the SRS resource is equal to 3 which is shorter than the minimum supported length. Such cases cannot be configured based on the agreement that no new sequence length is introduced. In other words, UE is expected no new SRS sequence length configured by gNB implementation, which may increase SRS resource configuration limitations.

- Similarly, there are many configurations where SRS bandwidth corresponding to mSRS,b  is not divisable by PF. Specifically, almost in all SRS configurations, SRS bandwidth is equal to 4 when BSRS=3, which can be divisible by PF. However, for other BSRS values, i.e. BSRS=0,1 or 2, many configurations with SRS bandwidth size are not divisable by PF. gNB implementation can avoid odd combinations of BSRS and PF values, it brings complexity at gNB configuration and less flexbile in terms of SRS configuration, and it doesn’t make sense to only support BSRS=3.

Although gNB implementation can prevent short sequence length and mismatched sequence length, it decreases SRS resource configuration flexibility.

gNB implementation can avoid odd combinations of SRS configuration and PF value(s), however a rule to handle scenarios where SRS bandwidth size is not devisable by PF is necessary.

- Although gNB configuration can prevent an SRS resource sequence length less than the minimum supported length, odd number of RBs may occur in some configuration combinations, such as 20 RBs bandwidth with PF = 4 or 40 RBs bandwidth with PF = 8 corresponding to 5 RBs’ partial bandwidth. A partial bandwidth with odd number of RBs is hard to achieve frequency resources multiplexing between different UE capabilities or different SRS resources, which may decrease SRS capacity. In below Figure 5-1, 20 RBs bandwidth configured with 4 UEs in subplot a), UE 1 to UE 3 has 5 RBs bandwidth after enabling partial frequency sounding with PF = 4, while UE 4 is Rel-15 UE configured with 4 RBs bandwidth or applying different PF value. Thus, 4 UEs are multiplexed in this 20 RBs bandwidth but with 1 RB frequency resource wastage. If each UE keeps even number of RBs bandwidth no matter what capabilities are supported in subplot b), it achieves RB level frequency resource multiplexing without wastage of any RBs across 5 UEs.

- odd RB bandwidth                                               b) even RB bandwidth

- Figure 5-1: Multiplexing with different UE capabilities for partial frequency sounding feature

- Furthermore, the formula of the sounding reference signal sequence length is shown as below,

Msc,bSRS=mSRS,bNscRBKTC

- , where mSRS,b represents an SRS resource bandwidth in an OFDM symbol and KTC is comb value. The SRS sequence length of 5RBs partial bandwidth is equal to 30, 15 and 7.5 corresponding to comb value equal to 2, 4 and 8 respectively based on above formula. From sequence level, only comb value equal to 2 can be configured for 5 RBs SRS which decreases SRS capacity in subcarrier level. Thus, we have following observation,

Odd number of RBs bandwidth may deteriorate SRS multiplexing capacity in both of RB level and subcarrier level.

- From above discussion on partial frequency sounding in RB level, the issues on sequence length, partial SRS bandwidth with remainder and odd number of RBs should be considered. Consequently, partial frequency sounding may decrease SRS resource configuration flexibility, which doesn’t bring benefit to the network in terms of flexible configuration and multiplexing rendering the feature of partial frequency sounding useless. Furthermore, not consistent with legacy SRS bandwidth configuration which bandwidth is in multiples of 4 RBs, it is better to keep an SRS bandwidth after partial frequency feature equal to 4*N RBs or rounded to 4*N RBs. Thus, a unified solution should be further discussed to reap benefits of partial frequency sounding, e.g. equal to 4*N RBs or rounded to 4*N RBs.

A unified solution should be supported to address the issue of SRS bandwidth indivisible in partial frequency sounding, i.e. support Alt 3 or Alt 4.

- Back to the previous agreements, different candidate PF values may be used for different SRS bandwidth. For example, small PF value is corresponding to SRS bandwidth less than 32 RBs and large PF value can be used for SRS bandwidth larger than 32 RBs. We believe PF value = 12 is sufficient for large bandwidth configurations, and we do not see a need to introduce fractional numbers for partial frequency sounding. And for increasing flexibility, we suggest supporting PF values which can be used to all SRS configurations.

- Furthermore, one of the main purposes of partial frequency sounding is to shorter frequency hopping cycle by configuring larger bandwidth in an SRS hopping subband and as well increasing accuracy in CSI acquisition by RE level power boosting, while partial frequency sounding can be achieved by re-configuration PF value in non-frequency hopping case. And, for better multiplexing with other UEs, it is no necessary to introduce DCI or MAC CE updating PF value as well as Noffset. We, thus, propose,

Support extra PF value = {3,8,12} for all CSRS and BSRS for flexible configuration, and partial frequency sounding applicable to frequency hopping only.

No need to introduce DCI or MAC CE updating PF value as well as Noffset.

a) SRS hopping period-based                                      b) SRS symbol-based

- Figure 5-2: partial frequency index hopping

- Although partial frequency sounding in RB level brings power boosting gain, channel estimation accuracy may deteriorate significantly in some RBs, especially for large PF value. Similar with SRS hopping mechanism, partial frequency index of an SRS can be changed per SRS hopping period for acquiring full channel estimation information. For example, in Figure 5-2, an SRS bandwidth in an OFDM symbol is divided by PF value = 3, and 4 times frequency hopping is needed in an SRS hopping period. Since partial frequency index of an SRS is changed to 1 after the 1st SRS hopping period in subplot a) or after each SRS transmission in subplot b), full CSI can be obtained from both partial frequency index hopping mechanism with long term SRS hopping. However, SRS symbol-based mechanism seems not beneficial and may lead to some resource wastage as same start RB hopping applied for an SRS hopping frequency subband in adjacent transmissions which correspondingly represents more start RB hopping times in comparison with SRS hopping period-based scheme.

- In addition, some companies suggested start RB hopping can be applicable on SRS transmission occasion within an SRS hopping period, which represents each of the antenna ports of the SRS resource in each slot is mapped to the different set of subcarriers within each pair of R adjacent OFDM symbol if R > 1. It has serious impacts on the definition of SRS repetition scheme in current specification and may decrease repetition performance due to unaligned SRS frequency resource. Hence, for a harmonic approach, it is better to support SRS hopping period-based scheme only.

There is no need to expand start RB hopping feature on SRS transmission occasion level.

- Lastly, signaling mechanism to determine the location of partial SRS is related to the feature of start RB hopping. If we support period-based scheme, hopping pattern could be determined in one of two ways:

- Scheme 1: pre-defined hopping order

- Scheme 2: directly hopping pattern indication, including pre-defined hopping pattern and flexible hopping pattern indication

For the first option, since there are many configuration combinations between SRS bandwidth and PF value, which correspond to different hopping patterns, it may impossible to use scheme with pre-defined hopping patterns. To remedy this issue, a pre-defined hopping order can be introduced, for example, Noffset,current = Noffset,previous+1 or Noffset,current = Noffset,previous-1. Thus, UE can be configured with Noffset once and following partial frequency location shall be determined automatically.

For the last option, although hopping pattern can be dynamically indicated by DCI signaling which is the most flexible, it will lead to unacceptable overhead increase. We believe semi-statistic indication or pre-configuration can be an optimal SRS partial frequency hopping signaling method. Besides option 2 can achieve more flexible hopping pattern indication which is one of the greatest strengths in comparison with pre-defined hopping order scheme. Thus, for increasing flexibility, there is no need to restrict hopping pattern indication as pre-defined hopping pattern if semi-statistic indication by MAC CE signaling or pre-configuration by RRC signaling is supported.

Support pre-defined hopping order for simplification start RB hopping, and it can be considered to support flexible hopping pattern indication for increasing flexibility if semi-statistic indication or pre-configuration is supported.

## Comb 8

- According to 38.211[6] for comb 8 SRS enhancement for positioning, maximum number of cyclic shifts nSRScs,max is equal to 6. Due to only 1 port SRS used for positioning, there is no issue on cyclic shift assignment for different SRS ports in an SRS resource. In RAN1#104bis-e, two alternatives of number of CSs for Comb-8 were agreed as following,

- Alt 1: The maximum number of CSs for Comb-8 is 6

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

- For Alt 2, from our perspective, it is difficult to support more than 6 CSs in the case of Comb-8 in a real channel, thus we don’t see the benefit on capacity aspect if the number of cyclic shifts is 12. In addition, some companies who support alt 2 have concerns on the cyclic shift allocation/mapping across 4 ports SRS for Alt 1, however, Alt 1 can be supported with minimum specification impact if the inherent issue of Alt 1, which is to determine 6 cyclic shifts mapping into 4 ports SRS with comb 8 feature, is solved.

- For Alt 1, note that 4 ports SRS with comb value equal to 8 cannot be precluded based on the agreement from Rel-17 SRS capacity and coverage enhancement, and we have explained above that the inherent issue for Alt 1 is to determine those 6 cyclic shifts allocated to 4 ports SRS as well as following formula enhancement.

,

- , where  nSRScs ∈ 0,1,...,nSRScs,max -1 represents comb-offset configured in an SRS resource.

One possible solution is to modify the formula with a floor function for preventing remainder in cyclic shift assignment. But it may cause non-continuous cyclic shift mapping into 4 ports SRS. For example, 4 port SRS resource with comb value equal to 8, and with a floor function in above mapping formula, cyclic shift index is equal to 0, 1,3 and 4 for SRS port index from 1000 to 1003 respectively if  nSRScs=0. Cyclic shift index 2 is not used due to non-continuous mapping (or called value jumping). Furthermore, multiplexing complexity increases as well.

Another solution is to combine FDM and cyclic shift allocation for 4 ports SRS with comb 8. Those ports are divided into two groups, and each group has two SRS ports with different cyclic shift value but with same frequency location, i.e. same comb offset value, while two port groups use same cyclic shift values and different comb offset value. Therefore, 6 cyclic shifts can be mapped into 4 ports SRS without value jumping after combining FDM scheme. Besides, different port grouping method may lead to different formula enhancement when specified. Two possible grouping methods are shown as below,

1st grouping method:

- group 1 = {1000,1001}, and group 2 = {1002,1003}):

2nd grouping method:

- group 1 = {1000,1002}, and group 2 = {1001,1003}

Some minor difference regarding to cyclic shift allocation formula can be found between above two grouping methods. Although we are open to determine which grouping method as well as relative allocation formula, since the 2nd grouping method had been used for comb 4 if  specified in 38.211[6], we slightly prefer to 2nd group method and relative formula enhancement for comb 8 cyclic shift allocation. Thus, we have following observations and proposal,

We don’t see the benefit on capacity aspect if the number of cyclic shifts is 12.

For positioning, due to only 1 port SRS is used, there is no issue on cyclic shift assignment for different SRS ports in an SRS resource.

A simple floor function in the formula leads to non-continuous cyclic shift index mapping to ports as well as increases multiplexing complexity.

Different port grouping method may lead to different formula enhancement.

Support the maximum number of CSs for Comb-8 is 6, i.e. Alt 1, and support following formula enhancement on cyclic shift allocation and grouping method.

Conclusions

In this contribution, we discuss some issues on SRS enhancement and have the following observations:

- Even with different available slot offset indication for two aperiodic SRS resource sets triggered by one or more DCI, collision between two SRS can happen.

- SRS transmission collision may happen for two aperiodic SRS resource sets triggered in case of UL CA, even with different available slot offset indication, if simultaneous transmission of SRS on difference CCs is not supported.

- DCI format 2-3 can be enhanced with minimum specification impact on current SRS carrier switching mechanism to achieve more flexible aperiodic SRS triggering and reduce probability of PDCCH congestion.

- SRS resource set for ‘codebook’ or ‘antenna switching’ should be configured separately.

- For the case of two SRS resource sets related to same SRS resource, same time domain behavior should be configured across these two shared SRS resource sets.

- UE may transmit an SRS resource twice at different time locations for acquiring DL CSI and UL CSI.

- Current implementation approach for different usage sharing scheme is not sufficient.

- Although gNB implementation can prevent short sequence length and mismatched sequence length, it decreases SRS resource configuration flexibility.

- gNB implementation can avoid odd combinations of SRS configuration and PF value(s), however a rule to handle scenarios where SRS bandwidth size is not devisable by PF is necessary.

- Odd number of RBs bandwidth may deteriorate SRS multiplexing capacity in both of RB level and subcarrier level.

- We don’t see the benefit on capacity aspect if the number of cyclic shifts is 12.

- For positioning, due to only 1 port SRS is used, there is no issue on cyclic shift assignment for different SRS ports in an SRS resource.

- A simple floor function in the formula leads to non-continuous cyclic shift index mapping to ports as well as increases multiplexing complexity.

- Different port grouping method may lead to different formula enhancement.

- and proposals:

- Support to trigger aperiodic SRS by non-scheduled DCI format 1-1 and 1-2 with minimal specification impact for SRS triggering enhancement.

- Support direct slot offset indication by repurposing unused fields for aperiodic SRS triggering when the new DCI field is absent for non-scheduling DCI.

- Do not support power control parameter and frequency domain parameter enhancement in non-scheduling DCI.

- Consider a combined dropping rule, including usage, order of triggering DCI, CC ID and set ID, to resolve SRS collision when more than one SRS resource sets are triggered by one or more DCI on same CC or different CCs.

- Support re-interpretation of unused bits or configurations in DCI format 2-3 for triggering multiple SRS transmissions on different CCs and/or different UEs.

- At least support SRS resource set with usage of codebook to be used for DL CSI acquisition in the case of xTxR.

- Support resource sharing scheme for usage ‘antenna switching’ and ‘codebook’ configured with different time domain behavior.

- At least for the case of same antenna virtualization between antenna switching and codebook-based transmission, considering one of antenna switching resource can be used for UL CSI acquisition.

- Whether SRS for 2 Tx codebook-based operation and SRS for 1T4R antenna switching can share resource needs further discussion.

- Allow extending the number of SRS resources in an SRS resource set with usage of ‘codebook’ and resource sharing between codebook and beam management for more flexible UL beam indication.

- Guard period should be enhanced to support 2 SRS sets for antenna switching located in adjacent slots, i.e. Alt 2-1.

- Support guard period configurable for antenna switching resource, i.e. Alt 1-1.

- Guard period can be absent before or after an antenna switching resource if the resource and other UL signals are transmitted on same physical antenna.

- Introduce a new capability to report activation time (slots or symbols) if MAC CE based flexible antenna switching scheme is supported.

- For activation processing, support enhancement on effective timeline for MAC CE based flexible antenna switching scheme as slot n + max {3ms, Y}, where the UE transmits a HARQ-ACK information in slot n corresponding to a MAC CE for flexible antenna switching activation, and Y is antenna switching resources activation time related to UE capability report.

- In FR2, it should be further studied whether to support intra-panel switching.

- The impact of MPUE assumption should be considered into SRS antenna switching enhancement.

- Enhancement is needed to clarify association of spatial relation information between SRS resource sets within an SRS resource set group for obtaining full DL CSI.

- The number of SRS resource set group should be doubled for antenna switching in MTRP scenario.

- Since the total number of SRS resource set for DL CSI acquisition increases rapidly with the number of SRS resource set group increasing, consider optimization of configuration mechanism in MTRP scenario for reducing overhead.

- A unified solution should be supported to address the issue of SRS bandwidth indivisible in partial frequency sounding, i.e. support Alt 3 or Alt 4.

- Support extra PF value = {3,8,12} for all CSRS and BSRS for flexible configuration, and partial frequency sounding applicable to frequency hopping only.

- No need to introduce DCI or MAC CE updating PF value as well as Noffset.

- There is no need to expand start RB hopping feature on SRS transmission occasion level.

- Support pre-defined hopping order for simplification start RB hopping, and it can be considered to support flexible hopping pattern indication for increasing flexibility if semi-statistic indication or pre-configuration is supported.

Support the maximum number of CSs for Comb-8 is 6, i.e. Alt 1, and support following formula enhancement on cyclic shift allocation and grouping method.

References

RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung, RAN#86.

Chairman notes, 3GPP TSG RAN WG1 Meeting #102-e, Meeting

Chairman notes, 3GPP TSG RAN WG1 Meeting #103-e, Meeting

Chairman notes, 3GPP TSG RAN WG1 Meeting #104-e, Meeting

Chairman notes, 3GPP TSG RAN WG1 Meeting #104bis-e, Meeting

3GPP TS 38.211 V16.5.0 (2021-03)

3GPP TS 38.214 V16.5.0 (2021-03)