3GPP TSG RAN WG1 Meeting #107-e	R1-2110849

e-Meeting, November 11th – November 19th, 2021

Agenda Item:	8.16.16

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for DL 1024QAM for NR FR1

Document for:	Discussion and Decision

- Introduction

In RAN1#106bis-e meeting, the UE features of DL 1024QAM for NR FR1 was discussed [1]. This contribution provides views on the remaining issues.

- Discussion

The pre-requisite feature groups for 1024-QAM should be pdsch-256QAM-FR1 instead of 1-4, as there are different “1-4” in different sections in 38.822: In section 4.1 of layer 1 UE features, it’s the CSI-RS based RRM measurement with associated SS-block. In section 4.2, it’s “Out of order delivery”. And in section 4.3, which is more related with 1024-QAM feature, there are two fields names for 256-QAM, one for FR1 and the other for FR2. Therefore, to be clear, the requisite feature groups should be “pdsch-256QAM-FR1” or simply a plain description as “256QAM for PDSCH for FR1”.

In addition, the prerequisite feature groups for other feature groups of 1024-QAM should be the basic feature (36-1).

Proposal 1: The prerequisite feature groups for FG 36-1 should be pdsch-256QAM-FR1, and the prerequisite feature groups for other feature groups of 1024-QAM should be the basic feature, i.e. FG 36-1.

For the type of feature groups, as it has been clearly stated in the WID that the capability for 1024-QAM is a per-band capability, it should be Per Band.

Proposal 2: The feature groups for 1024-QAM are per band capabilities.

Therefore, we have the following proposal for the UE feature table:

Proposal 3: Adopt the following UE features for 1024-QAM:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_DL1024QAM_FR1 | 36-1 | 1024QAM for PDSCH for FR1 | Support 1024QAM for PDSCH for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214. | 1-4 [pdsch-256QAM-FR1] | Yes | N/A | No support of 1024 QAM for PDSCH | Per Band | N/A | No Applicable only to FR1 | N/A | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability | Optional with capability signalling |
| 36. NR_DL1024QAM_FR1 | 36-2 | scalingFactor for 1024QAM | Indicates the scaling factor to be applied to the band in the max data rate calculation as defined in 4.1.2 when support of 1024-QAM is signalled for the band. | 36-1 | Yes | N/A |  | Per Band | N/A | Applicable only to FR1 | N/A | Supported values [0.4, 0.75, 0.8, 1.0] | Optional with capability signaling |

- Conclusion

In this contribution, we give further considerations on views on the UE feature for NR DL 1024-QAM. The following proposals are made.

Proposal 1: The prerequisite feature groups for FG 36-1 should be pdsch-256QAM-FR1, and the prerequisite feature groups for other feature groups of 1024QAM should be the basic feature, i.e. FG 36-1.

Proposal 2: The feature groups for 1024-QAM are per band capabilities.

Proposal 3: Adopt the following UE features for 1024-QAM:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_DL1024QAM_FR1 | 36-1 | 1024QAM for PDSCH for FR1 | Support 1024QAM for PDSCH for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214. | 1-4 [pdsch-256QAM-FR1] | Yes | N/A | No support of 1024 QAM for PDSCH | Per Band | N/A | No Applicable only to FR1 | N/A | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability | Optional with capability signalling |
| 36. NR_DL1024QAM_FR1 | 36-2 | scalingFactor for 1024QAM | Indicates the scaling factor to be applied to the band in the max data rate calculation as defined in 4.1.2 when support of 1024-QAM is signalled for the band. | 36-1 | Yes | N/A |  | Per Band | N/A | Applicable only to FR1 | N/A | Supported values [0.4, 0.75, 0.8, 1.0] | Optional with capability signaling |

- Reference

- R1-2109919, Summary of UE features for DL 1024QAM for NR FR1, Moderator (AT&T), RAN1#106bis-e, E-meeting, Oct., 2021.