3GPP TSG RAN WG1 #109-e			     R1-2204385

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussions on potential solutions for SL positioning

Agenda Item:	9.5.1.3

Document for: 	Discussion and Decision

- Introduction

In positioning study item, sidelink positioning is included in the scope [1]. In this contribution, we share our views on potential solutions for SL positioning.

- Discussions

  - SL-positioning method

In Uu positioning, several mechanisms are supported, e.g. TDOA, RTT, AOA/D. Let us use the following illustration as an example. UE-X is a UE acquiring its own positioning-related information. UE-Y1/Y2/Y3 are surrounding UEs of UE-X. To obtain positioning-related information at UE-X, UE-X and/or UE-Ys transmit reference signal each other for SL-positioning and measure some information.

One important note to discuss SL-positioning method would be that there is a small misalignment among UE-X and UE-Ys as illustrated below. Even when they are synchronized by using the same sync source (GNSS/eNB/gNB/SyncRefUE), the timing is a bit different due to different propagation delay of the received sync source. The difference represented as ∆tsync will be within CP duration, then communication each other is still possible. However, the difference leads to large positioning error. The error should be removed appropriately.

Fig. 1: Assumed situation and a small time-misalignment among UEs.

Observation 1:

- In SL, there is a small time-misalignment among UEs even when they are synchronized based on the same sync source. For SL-positioning, the time-misalignment needs to be considered.

For TDOA/RTT, ∆t in the above illustration is what the UE needs to obtain to calculate positioning information. Here, in our view, at least RTT can be supported as mechanisms of SL-positioning in consideration of the existence of ∆tsync.

In RTT mechanism, for example UE-X transmits reference signal to UE-Y. UE-Y receives the reference signal and then transmits reference signal to UE-X. After that, UE-X receives the reference signal from UE-Y. In addition, UE-Y reports RX-TX time difference (= Tdiff). This is illustrated in the following figure, where the time difference is considered. RTT measured at UE-X is given as the time duration between starting timing of reference signal transmission and starting timing of reference signal reception. By using parameters shown in the figure, the RTT can be written as follows:

RTT= ∆t+Tdiff+∆t

Therefore, in RTT mechanism, ∆t can be obtained at UE-X as RTT-Tdiff/2. In other words, the formula does not include ∆tsync and thus there is no impact from the time-misalignment between UE-X and UE-Y.

Fig. 2: RTT mechanism.

Observation 2:

- It seems that RTT measured at UE does not include the time-misalignment.

Proposal 1:

- For SL-positioning, at least RTT mechanism is supported.

Meanwhile in TDOA mechanism, one-way propagation delay is measured. For example, UE-X transmits reference signal. UE-Y receives the reference signal and then measures time-difference from a reference timing. UE-Y reports the time-difference to UE-X and positioning information is calculated at UE-X. However, understanding of the reference timing is not common among UEs due to the time-misalignment between UE-X and UE-Y. This issue is illustrated in the following figure. The measured value would include ∆tsync and the value is not the same among UE-Ys; hence accurate positioning information cannot be obtained from the reported information.

Fig. 3: TDOA mechanism.

Observation 3:

- It seems that time difference from a reference timing measured at UE includes the time-misalignment.

- The amount of the time-misalignment is different among links. It would be difficult to remove the time-misalignment at any UE.

Proposal 2:

- For SL-positioning, study whether TDOA mechanism can be supported or not, in consideration of time-misalignment among synchronizing UEs.

  - SL-positioning procedure

For realizing supported SL-positioning method (e.g. RTT), we are considering the following procedure options. It is noted that definitions of UE-X and UE-Ys are the same; UE-X is a UE that tries obtaining its own location, and UE-Ys are surrounding UEs. In addition, let us use UE-A as a UE that needs UE-B’s position-related information.

- Option 1: Triggering by UE that needs its own position

In this option, UE-X starts SL-positioning method. As the 1st step, UE-X transmits some information/signal to UE-Ys. UE-Ys receive it from UE-X, and UE-Ys send corresponding information/signal to UE-X as the 2nd step. After UE-X receives them from UE-Ys, UE-X calculates its own location. This option would be the most basic procedure.

Which information/signal is transmitted from UE-X/UE-Ys are dependent on SL-positioning methods. If RTT is used for example, UE-X transmits reference signal (let’s call SL-PRS) to UE-Ys and a kind of request. UE-Ys send SL-PRS with information on time gap between the reception and the transmission.

- Option 2: Triggering by UE that needs other UE’s position

In this option, UE-A would like to obtain not its own position but UE-B’s location. Then at the 1st step, UE-A will transmit a request of position report to UE-B. As the 2nd step, UE-B performs acquisition of its own location e.g. by using Option 1 as UE-X. UE-B reports the acquired information to UE-A as the last step. This option would be necessary if this work item considers a situation where a UE needs location information of other UE.

Fig. 4: Possible SL-positioning procedure.

Proposal 3:

- Support the following SL-positioning procedure to obtain its own location.

  - Step 1: UE that requires its own location transmits information and/or signal to surrounding UEs

  - Step 2: The surrounding UEs receive the information and/or signal and transmit corresponding information and/or signal to the UE

Proposal 4:

- Study whether a case where a UE requires other UE’s location is considered in Rel-18 SL positioning or not.

In both options, it might be possible to replace UE to gNB and thus a part of measurement for supported SL-positioning method is performed from communication between UE and gNB (i.e. by using Uu signaling instead of SL signaling). This direction is illustrated as below. Whether such a flexible mechanism can be supported or not should be discussed.

Fig. 5: Possible SL-positioning procedure with Uu measurement.

Proposal 5:

- Study whether SL-positioning can use Uu measurement or not.

  - If supported, some UE in SL-positioning method can be replaced to gNB.

In addition, combination with Uu positioning should be discussed. For example, if UE that needs its own location or other UE’s position is in coverage of gNB, it might be possible to obtain the required information from gNB and in this case there would be no need to perform SL positioning since gNB can do Uu-positioning or have location information already. In other words, after checking possibility of location acquisition at Uu link, SL-positioning procedure is performed. For this direction, the following procedure options can be considered.

- Option A: Requesting position information by UE that needs its own position

In this option, firstly UE-X transmits a request of its own location information to gNB. Then gNB performs Uu-based positioning with UE-X if possible/necessary, as the 2nd step. After that, gNB reports the acquired information to UE-X.

Meanwhile, if gNB cannot perform it, gNB sends a kind of rejection to UE-X instead of the above 2nd step. When UE-X received the rejection information, then UE-X triggers to perform SL-positioning as Option 1/1’.

- Option B: Requesting position information by UE that needs other UE’s position

In this option, UE-A transmits a request of UE-B’s location information to gNB at the 1st step. gNB performs Uu-based positioning with UE-B if possible/necessary. Then gNB reports the acquired information to UE-A. As in Option A, if rejection information is received at UE-A from gNB, UE-A triggers to perform SL-positioning as Option 2/2’.

Fig. 6: Acquisition of required location information without SL-positioning.

Observation 4:

- In in-coverage scenario, Uu positioning might also be available and thus it might be possible to acquire required location information from Uu positioning.

Proposal 6:

- Study availability of Uu positioning instead of SL-positioning in use cases assumed for SL-positioning.

  - If available, study priority order between Uu positioning and SL positioning and detailed procedure.

Besides, the following aspects have to be studied.

Proposal 7:

- For SL-positioning,

  - Study details on configuration/indication/report

  - Study measurement UE determination

  - Study cast-type to be used in SL-positioning method

  - Reference signal for SL-positioning

For any SL positioning method, SL-PRS would be necessary to measure required information like time/angle/etc. In SL, it is hard to support independent signal transmission due to transient period/AGC/sensing/etc. From this reason, PSCCH/PSSCH are allocated not in symbol/PRB-level but in slot/subchannel-level, S-SSB uses dedicated slots, and PSFCH is put at the end of PSFCH-slot except for gap symbol, which is always TDMed with PSCCH/PSSSCH. If SL-PRS is transmitted as independent signal, whole SL structure needs to be modified. This would lead to quite large RAN1 workload. Instead, we believe that it is better to support SL-PRS like other SL-RS (DM-RS/PT-RS/CSI-RS); i.e., SL-PRS is multiplexed on PSCCH/PSSCH transmission. PSCCH is a channel blind-decoded. PSSCH would be straightforward choice to multiplex SL-PRS.

Fig. 7: An example of SL-PRS multiplexing on PSSCH.

Observation 5:

- Supporting SL-PRS as independent signal would lead to significant modification of SL structure.

Proposal 8:

- Support SL-positioning RS multiplexed on PSSCH.

For SL-PRS multiplexing on PSSCH, the following alternatives can be considered.

- Alt 1: Define SL-PRS as a new reference signal

For SL-positioning measurement, there would be optimal mapping of time-domain location/granularity and frequency-domain location/granularity. Alt 1 can support the optimal mapping while the detail and how to configure/indicate shall be discussed.

For example, if new RS is introduced, Rel-18 SL-positioning UE will support the reception. However, Rel-16/17 SL UE cannot receive. Especially in broadcast/groupcast, SL-PRS mapping on PSSCH should be studied well e.g. puncturing mapping instead of rate-matching mapping. Note that SL-positioning method/procedure might support broadcast/groupcast transmission as well as unicast transmission, might not. It is dependent on discussions of method/procedure

- Alt 2: Define SL-PRS as a kind of existing reference signal

DM-RS/CSI-RS are supported as reference signal that can be multiplexed on PSSCH. If DM-RS is used for SL positioning measurement, keeping the existing DM-RS mechanism would be fine. DM-RS pattern is (pre-)configured and indicated via SCI 1-A. If CSI-RS is used, the existing CSI-RS mechanism can be baseline; but CSI-RS is available only in unicast link. Discussion progress on method/procedure is necessary to discuss whether/how to support CSI-RS as SL-PRS and whether/how to enhance.

Proposal 9:

- Study the following alternatives for SL-PRS.

  - Alt 1: Define SL-PRS as a new reference signal.

    - Configuration/Indication/Cast-type/Mapping resource/Mapping procedure (rate-matching vs puncturing)/Sequence

  - Alt 2: Define SL-PRS as an existing reference signal.

    - Configuration/Indication/Cast-type/Mapping resource modification

- Conclusion

In this contribution, we discussed potential solutions for SL positioning. Observations/Proposals are summarized as following:

Observation 1:

- In SL, there is a small time-misalignment among UEs even when they are synchronized based on the same sync source. For SL-positioning, the time-misalignment needs to be considered.

Observation 2:

- It seems that RTT measured at UE does not include the time-misalignment.

Proposal 1:

- For SL-positioning, at least RTT mechanism is supported.

Observation 3:

- It seems that time difference from a reference timing measured at UE includes the time-misalignment.

- The amount of the time-misalignment is different among links. It would be difficult to remove the time-misalignment at any UE.

Proposal 2:

- For SL-positioning, study whether TDOA mechanism can be supported or not, in consideration of time-misalignment among synchronizing UEs.

Proposal 3:

- Support the following SL-positioning procedure to obtain its own location.

  - Step 1: UE that requires its own location transmits information and/or signal to surrounding UEs

  - Step 2: The surrounding UEs receive the information and/or signal and transmit corresponding information and/or signal to the UE

Proposal 4:

- Study whether a case where a UE requires other UE’s location is considered in Rel-18 SL positioning or not.

Proposal 5:

- Study whether SL-positioning can use Uu measurement or not.

  - If supported, some UE in SL-positioning method can be replaced to gNB.

Observation 4:

- In in-coverage scenario, Uu positioning might also be available and thus it might be possible to acquire required location information from Uu positioning.

Proposal 6:

- Study availability of Uu positioning instead of SL-positioning in use cases assumed for SL-positioning.

  - If available, study priority order between Uu positioning and SL positioning and detailed procedure.

Proposal 7:

- For SL-positioning,

  - Study details on configuration/indication/report

  - Study measurement UE determination

  - Study cast-type to be used in SL-positioning method

Observation 5:

- Supporting SL-PRS as independent signal would lead to significant modification of SL structure.

Proposal 8:

- Support SL-positioning RS multiplexed on PSSCH.

Proposal 9:

- Study the following alternatives for SL-PRS.

  - Alt 1: Define SL-PRS as a new reference signal.

    - Configuration/Indication/Cast-type/Mapping resource/Mapping procedure (rate-matching vs puncturing)/Sequence

  - Alt 2: Define SL-PRS as an existing reference signal.

    - Configuration/Indication/Cast-type/Mapping resource modification

References

- RP-213588	, Revised SID on Study on expanded and improved NR positioning, Intel (Email discussion moderator)