3GPP TSG-RAN WG1 Meeting #111 	R1-2212935

Toulouse, France, November 14 – 18, 2022

Agenda Item:	9.7.1

Source:	Moderator (Huawei)

Title:	FL summary#3 for NW Energy Savings

Document for:	Discussion and decision

Introduction

This summary contains simulation results submitted to RAN1#111 meeting for the study of network energy savings for NR (referring to a template for collection of companies input, in Template_NW_EnSav_v19_Samsung_CEWiT.xlsx), and provides observations/conclusion for discussion and decision. To ease the TR work, the document uses the same structure as (draft) TR 38.864 (v0.4.0). Note, since the summary is not intended for email discussion, companies are not required to have input for the draft Tables/Texts/Observations.

Companies can search ‘Consensus’ for the relevant draft TPs.

6	Techniques to improve network energy savings

Editor's note: simulation results to be captured under this section.

Editor's note: RAN2 and RAN3 related aspect to be provided by using separate sections like 6.X when applicable.

##### === draftTP0: start ===

Various techniques in time, frequency, spatial and power domains are studied. Companies’ simulation results as well as evaluation assumption details are gathered in [ref.]. In this document, results as well as some notable assumptions and setting are explicitly present in relevant tables. Also, the categorization of techniques in terms of technical domain and results presentation/tabulation are for study/evaluation purpose. This does not preclude to further merge or combine certain techniques.

=== end ===

FL: when checking the simulation results, it appears to be quite challenging to capture all the details reported from companies. FL considers it might be sufficient from study point of view to capture the critical results and notable settings/evaluation assumptions while leave other detailed referable from the excel or companies’ contributions. Therefore, for different schemes, the table may show differently. Company can request to clarify/add details for some results into corresponding tables.

Also, the numbering of techniques, i.e. A-1, A-3, A-4 etc. shall be re-ordered when capturing into TR in consecutive order. For discussion purpose the current TP use the same numbering as those in FL summary of agenda item 9.7.2.

| Company | Comments |
| --- | --- |
| Ericsson1 | As a general comment, for the techniques that preclude legacy UEs from accessing the carrier, following should be added for clarity in the observation. Gains from the technique are only available when Rel-15/16/17 UEs are not using the carrier on which this technique is enabled. |
| FL | Offline consensus to include |

6.1	Techniques in time domain

6.1.1	Technique A-1 Adapting transmission/reception of common channels/signals

6.1.1.1	Description of technique

6.1.1.2	Analysis of performance and impacts

##### draftTP A-1-1: start

The following table captures the simulation results for the schemes that use simplified version of SSB, such as only PSS or only PSS and SSS without PBCH, or PSS and SSS with partial PBCH.

Table 6.1.1-x: BS energy savings by simplified SSB

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/access delay/latency/UE power consumption | Reference configuration | Baseline configuration/assumption |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC [R1-2211692] | SSB and SIB1 repetition period 40ms, for other 20ms occasions, only PSS and SSS are transmitted. | cat.2 | Zero | 15.7% | / | Set 1 | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |
|  | SSB and SIB1 repetition period 40ms, for other 20ms occasions, only PSS and SSS are transmitted. | cat.1 | Zero | 28.3% | / |  | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |
| vivo [R1-2211018, R1-2212541] | SSB structure adaptation including light SSB (ES scheme: 20ms light-SSB and SIB1, 20ms RACH listening only PSS and SSS for light-SSB) | Cat1 | Zero | 0.9% | 0% | Set 1 | Baseline scheme: 20ms SSB and SIB1, 20ms RACH listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 20ms UEWUS listening only PSS and SSS for light-SSB) | Cat1 | Zero | 1.2% | 0% | Set 1 | Baseline scheme: 160ms SSB, 20ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 80ms UEWUS listening only PSS and SSS for light-SSB) | Cat1 | Zero | 2.4% | 0% |  | Baseline scheme: 160ms SSB, 80ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 160ms UEWUS listening only PSS and SSS for light-SSB) | Cat1 | Zero | 4.4% | 0% |  | Baseline scheme: 160ms SSB, 160ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 20ms light-SSB and SIB1, 20ms RACH listening only PSS and SSS for light-SSB) | Cat2 | Zero | 0.7% | 0% |  | Baseline scheme: 20ms SSB and SIB1, 20ms RACH listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 20 UEWUS listening only PSS and SSS for light-SSB) | Cat2 | Zero | 0.8% | 0% |  | Baseline scheme: 160ms SSB, 20ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 80ms UEWUS listening only PSS and SSS for light-SSB) | Cat2 | Zero | 0.8% | 0% |  | Baseline scheme: 160ms SSB, 80ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 160ms UEWUS listening only PSS and SSS for light-SSB) | Cat2 | Zero | 0.8% | 0% |  | Baseline scheme: 160ms SSB, 160ms UEWUS listening |
| CEWiT [R1-2212429] | simplified SSB with repetition period 20ms, only PSS and SSS with partial PBCH are transmitted in simplified SSB | Cat.1 | Zero | 2.4% |  | Set 1 | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |

Based on the simulation results, at empty load, one result shows that BS energy saving gain can be achieved by 15.7%-28.3% with only PSS and SSS transmitted from SSB, and half-reduced SIB1 transmission One company show that the gain from light SSB only ranges from 0.8% to 4.4%, which slightly increases as the listening periodicity of WUS from UE becomes larger. One result shows that simplified SSB with PSS, SSS and partial PBCH, for empty load and Set 1 reference configuration, 2.4% BS energy savings can be achieved.

No impact on UPT was observed due to empty load.

=== end ===

| Company | Comments |
| --- | --- |
| Intel | Suggested revisions: On UE power consumption, no negative impact is observed as identified by one source (vivo) |
| QCOM1 | The “light SSB” in our analysis is for on-demand SSB/SIB1 transmission triggered by cell WUS. We can view “light SSB” as discovery RS. Hence, we would like to capture it in A-6 as well. |
| vivo | Based on our understanding, the energy saving gain in (CMCC and Qualcomm)’s results are not purely from light version of SSB. For example, in CMCC’s results, the enhanced scheme 40ms SSB/SIB1 period compared to 20ms in baseline scheme in addition to light SSB. In Qualcomm’s result, the energy saving gain may mainly come from wake up scheme in addition to light SSB. Then the observation is better to be updated as: Based on the simulation results, at empty load, two companies (CMCC, Qualcomm) show that 15.7%-28.3% gain is achieved by light version of SSB and SSB&SIB1 period adaptation/WUS triggering on demand SIB1. Note that the energy saving gain is not purely from light version of SSB. One company (vivo) show that the gain from light SSB only ranges from 0.8% to 4.4%, which slightly increases as the listening periodicity of WUS from UE becomes larger. Besides, we suggest to remove the following sentence: |
| FL | To modify the description for the results from CMCC and [move QC results to A-6] To add CEWiT No impact on UPT due to empty load ¦clarify the combined technique(s) that were in the results |
| CMCC | We are ok with FL’s modification. In our results, when light version of SSB without PBCH is applied for some occasions, SIB1 is also omitted since no PBCH to inform the PDCCH for that occasion. |

##### draftTP A-1-3: start

The following table captures the simulation results for the schemes by which transmission occasion of one or more common signals/channels, which is SIB1 and SSB based on the submitted results, can be skipped.

Table 6.1.1-x: BS energy savings by skipping one or more common signals/channels

| Company | ES scheme | Load scenario | ES gain (%) |  | BS Category/Reference configuration/Baseline configuration/assumption | Other evaluation methodology/assumption details/other performance impact |
| --- | --- | --- | --- | --- | --- | --- |
| OPPO [R1-2211458] | Transmission occasion of SIB1 with 24 RBs for 20 ms periodicity is skipped | low load(RU-10%) | 2.6% | Cat 1 Set 1 SIB1 with 24 RBs for 20 ms periodicity, SSB with 20 RBs for 20 ms periodicity |  | SLS FTP3 (0.5MB as packet size, 200ms as mean inter-arrival time) UPT/Access delay/Latency: almost similar with the baseline |
|  |  | low load(RU-0.2%) | 3.9% |  |  | SLS IM (0.1MB as packet size, 2s as mean inter-arrival time) UPT/Access delay/Latency: almost similar with the baseline |
| Samsung [R1-2212543] | the number of SSB adaptation | Medium load: 42 % RU | 1.9%, 2.8%, 3.3% | Cat 1 Set 1 8 SSBs for FR1 and ssb-periodicity = 20 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 3.1%, 6.1%, 7.6% |  |  |  |
|  |  | Low load: 7.5 % RU | 5.5%, 11.0%, 13.8% |  |  |  |
|  |  | Low load: 2 % RU | 7.2%, 14.3%, 17.9% |  |  |  |
|  |  | Medium load: 42 % RU | 2.0%, 3.0%, 3.5% | Cat 2 Set 1 8 SSBs for FR1 and ssb-periodicity = 20 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 2.8%, 4.2%, 4.9% |  |  |  |
|  |  | Low load: 7.5 % RU | 4.4%, 6.6%, 7.7% |  |  |  |
|  |  | Low load: 2 % RU | 5.3%, 7.9%, 9.2% |  |  |  |
|  |  | Medium load: 42 % RU | 3.3%, 5.0%, 5.8%, 6.3%, 6.5%, 6.6% | Cat 1 Set 3 64 SSBs for FR2 and ssb-periodicity = 20 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 4.3%, 17.6%, 19.5%, 20.5%, 21.0%, 21.3% |  |  |  |
|  |  | Low load: 7.5 % RU | 7.1%, 13.6%, 16.9%, 18.5%, 19.3%, 19.7% |  |  |  |
|  |  | Low load: 2 % RU | 8.3%, 15.9%, 19.7%, 21.6%, 22.6%, 23.0% |  |  |  |
|  |  | Medium load: 42 % RU | 4.0%, 5.9%, 6.9%, 7.4%, 7.7%, 7.8% | Cat 2 Set 3 64 SSBs for FR2 and ssb-periodicity = 20 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 5.4%, 8.1%, 9.5%, 10.2%, 10.5%, 10.7% |  |  |  |
|  |  | Low load: 7.5 % RU | 8.2%, 12.2%, 14.3%, 15.3%, 15.8%, 16.1% |  |  |  |
|  |  | Low load: 2 % RU | 9.6%, 14.4%, 16.8%, 18.0%, 18.6%, 18.9% |  |  |  |
|  |  | Medium load: 42 % RU | 1.1%, 2.2%, 2.7% | Cat 1 Set 1 8 SSBs for FR1 and ssb-periodicity = 40 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.6%, 3.2%, 4.0% |  |  |  |
|  |  | Low load: 7.5 % RU | 3.1%, 6.1%, 7.7% |  |  |  |
|  |  | Low load: 2 % RU | 4.2%, 8.3%, 10.3% |  |  |  |
|  |  | Medium load: 42 % RU | 1.0%, 1.6%, 1.8% | Cat 2 Set 1 8 SSBs for FR1 and ssb-periodicity = 40 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.5%, 2.2%, 2.5% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.3%, 3.5%, 4.0% |  |  |  |
|  |  | Low load: 2 % RU | 2.8%, 4.2%, 4.9% |  |  |  |
|  |  | Medium load: 42 % RU | 1.9%, 3.6%, 4.4%, 4.8%, 5.0%, 5.2% | Cat 1 Set 3 64 SSBs for FR2 and ssb-periodicity = 40 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 2.6%, 5.0%, 6.1%, 6.7%, 7.0%, 7.2% |  |  |  |
|  |  | Low load: 7.5 % RU | 4.0%, 7.6%, 9.4%, 10.4%, 10.8%, 11.0% |  |  |  |
|  |  | Low load: 2 % RU | 4.8%, 9.2%, 11.4%, 12.5%, 13.1%, 13.3% |  |  |  |
|  |  | Medium load: 42 % RU | 2.1%, 3.1%, 3.6%, 3.9%, 4.0%, 4.1% | Cat 2 Set 3 64 SSBs for FR2 and ssb-periodicity = 40 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 2.9%, 4.3%, 5.1%, 5.4%, 5.6%, 5.7% |  |  |  |
|  |  | Low load: 7.5 % RU | 4.4%, 6.7%, 7.8%, 8.3%, 8.6%, 8.8% |  |  |  |
|  |  | Low load: 2 % RU | 5.4%, 8.1%, 9.4%, 10.1%, 10.4%, 10.6% |  |  |  |
|  |  | Medium load: 42 % RU | 0.6%, 1.1%, 1.4% | Cat 1 Set 1 8 SSBs for FR1 and ssb-periodicity = 80 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.0%, 2.3%, 3.0% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.5%, 5.9%, 7.6% |  |  |  |
|  |  | Low load: 2 % RU | 4.5%, 10.7%, 13.8% |  |  |  |
|  |  | Medium load: 42 % RU | 0.5%, 0.8%, 0.9% | Cat 2 Set 1 8 SSBs for FR1 and ssb-periodicity = 80 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.7%, 1.1%, 1.3% |  |  |  |
|  |  | Low load: 7.5 % RU | 1.2%, 1.8%, 2.1% |  |  |  |
|  |  | Low load: 2 % RU | 1.4%, 2.1%, 2.5% |  |  |  |
|  |  | Medium load: 42 % RU | 1.0%, 1.8%, 2.3%, 2.5%, 2.6%, 2.6% | Cat 1 Set 3 64 SSBs for FR2 and ssb-periodicity = 80 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.3%, 2.6%, 3.2%, 3.5%, 3.7%, 3.7% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.1%, 4.1%, 5.0%, 5.5%, 5.8%, 5.9% |  |  |  |
|  |  | Low load: 2 % RU | 2.6%, 5.0%, 6.2%, 6.8%, 7.1%, 7.2% |  |  |  |
|  |  | Medium load: 42 % RU | 1.1%, 1.6%, 1.8%, 2.0%, 2.0%, 2.1% | Cat 2 Set 3 64 SSBs for FR2 and ssb-periodicity = 80 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.5%, 2.2%, 2.6%, 2.8%, 2.9%, 2.9% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.3%, 3.5%, 4.1%, 4.4%, 4.5%, 4.6% |  |  |  |
|  |  | Low load: 2 % RU | 2.9%, 4.3%, 5.0%, 5.4%, 5.5%, 5.6% |  |  |  |
|  |  | Medium load: 42 % RU | 0.3%, 0.6%, 0.7% | Cat 1 Set 1 8 SSBs for FR1 and ssb-periodicity = 160 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.5%, 1.3%, 1.6% |  |  |  |
|  |  | Low load: 7.5 % RU | 1.5%, 3.5%, 4.5% |  |  |  |
|  |  | Low load: 2 % RU | 3.3%, 7.8%, 10.1% |  |  |  |
|  |  | Medium load: 42 % RU | 0.3%, 0.4%, 0.5% | Cat 2 Set 1 8 SSBs for FR1 and ssb-periodicity = 160 |  | FTP3 model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.4%, 0.6%, 0.7% |  |  |  |
|  |  | Low load: 7.5 % RU | 0.6%, 0.9%, 1.0% |  |  |  |
|  |  | Low load: 2 % RU | 0.7%, 1.1%, 1.3% |  |  |  |
|  |  | Medium load: 42 % RU | 0.5%, 0.9%, 1.1%, 1.3%, 1.3%, 1.3% | Cat 1 Set 3 64 SSBs for FR2 and ssb-periodicity = 160 |  | FTP3 model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.7%, 1.3%, 1.6%, 1.8%, 1.9%, 1.9% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.6%, 7.1%, 9.3%, 10.4%, 11.0%, 11.2% |  |  |  |
|  |  | Low load: 2 % RU | 6.0%, 16.0%, 21.0%, 23.5%, 24.8%, 25.4% |  |  |  |
|  |  | Medium load: 42 % RU | 0.5%, 0.8%, 0.9%, 1.0%, 1.0%, 1.1% | Cat 2 Set 3 64 SSBs for FR2 and ssb-periodicity = 160 |  | FTP3 model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.8%, 1.1%, 1.3%, 1.4%, 1.5%, 1.5% |  |  |  |
|  |  | Low load: 7.5 % RU | 1.2%, 1.8%, 2.1%, 2.2%, 2.3%, 2.3% |  |  |  |
|  |  | Low load: 2 % RU | 1.5%, 2.2%, 2.6%, 2.8%, 2.8%, 2.9% |  |  |  |

Based on the results, it is observed that

- One company observed that statically skipping certain SIB1 transmission occasions under Set 1 reference configuration for BS Category 1 can achieve energy saving gain by 2.6%~3.9% compared to the baseline of 20 ms SSB&SIB1 repetition periodicity at low load. No impact to UPT was observed. There is no random access procedure modelled in the simulation, therefore the impact on access delay/latency is not shown.

- One company observed that static adaptation of number of SSB can achieve energy saving gain by 0.3%~25.4% at different scenarios with FTP3 model. The gain generally increases when the traffic load becomes lighter while decreases as the SSB periodicity becomes larger. For a same traffic load and SSB periodicity, the gain increases as the number of SSB can be reduced. For FR2 with larger number of SSB for baseline, there is generally larger gain observed than FR1. Due to reduced number of SSB, access delay is increased. Performance of dynamic adaptation of SSB numbers is not provided. There is no random access procedure modelled in the simulation, therefore the impact on access delay/latency is not shown.

=== end ===

| Company | Comments |
| --- | --- |
| QCOM1 | Based on the results, it is observed that - One company observed that statically skipping certain SIB1 transmission occasions under Set 1 reference configuration for BS Category 1 can achieve energy saving gain by 2.6%~3.9% compared to the baseline of 20 ms SSB&SIB1 repetition periodicity at low load; No impact to UPT was observed. - One company observed that static adaptation of number of SSB can achieve energy saving gain by 0.3%~25.4% at different scenarios with FTP3 model. The gain generally increases when the traffic load becomes lighter while decreases as the SSB periodicity becomes larger. For a same traffic load and SSB periodicity, the gain increases as the number of SSB can be reduced. For FR2 with larger number of SSB for baseline, there is generally larger gain observed than FR1. Performance of dynamic adaptation of SSB numbers is not provided. On UPT/access delay/latency, - skipping SIB1 transmission does not observed negative impact. On UE power consumption, []. |
| vivo | We agree with Qualcomm’s update. |
| FL | Adopt QC revisions There is no random access procedure modelled in the simulation for OPPO results. Therefore the impact on latency is not shown. Add the latency impact to SS result [Impact on access latency to the network is not captured]. For SS results, impact on UE mobility is not investigated. The impact by not being able to request system information update is not investigated. |
| Samsung | Regarding the UE mobility and system information update impact, we think most of techniques in time domain also could be investigated. Therefore, we would like to add the UE mobility and system information update impact on all techniques in time domain or remove it simply. |

##### draftTP A-1-5: start

The following show the BS energy savings by configuration/adaptation of longer periodicity of common signals and/or uplink random access opportunities.

Table 6.1.1-x: BS energy savings by adapting SSB/SIB1 periodicities

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/Access delay/latency/UE power consumption, etc. | Baseline configuration/assumption/Other notable setting |
| --- | --- | --- | --- | --- | --- | --- |
| CMCC [R1-2211692] | SSB periodicity 20ms, SIB repetition period 40ms. | cat.2 | Zero | 13.7% |  | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |
|  | SSB and SIB1 repetition period 40ms. |  |  | 17.6% |  |  |
|  | SSB periodicity 20ms, SIB repetition period 40ms. | cat.1 | Zero | 25.7% |  |  |
|  | SSB and SIB1 repetition period 40ms. |  |  | 28.7% |  |  |
| vivo [R1-2211018, R1-2212541] | Period adaptation of common signals and channels (ES scheme: 160ms SSB and SIB1, 160ms RACH listening) | Cat1 | Zero | 78.8% | UE power consumption: 0% | Baseline scheme: 20ms SSB and SIB1, 20ms RACH listening |
|  |  | Cat2 |  | 16.6% | UE power consumption: 0% |  |
| NOKIA/NSB [R1-2211097] | SSB/SIB1/RO monitoring period= 160ms | Cat 2 | Zero, Low, Light, Medium | 48.4%, 44.3%, 43.7%, 39.9% | UPT: 0 Mbps, 83 Mbps, 70 Mbps, 55 Mbps | SSB/SIB1/random-access occasion (RO) monitoring periodicity @ 20ms UEs are initially in RRC_idle state |
|  | SSB/SIB1/RO monitoring period= 640ms |  | Zero, Low, Light, Medium | 53.6%, 49.0%, 48.8%, 46.1% | UPT: 0 Mbps, 29 Mbps, 27 Mbps, 25 Mbps |  |
|  | SSB/SIB1/RO monitoring period= 1280ms |  | Zero, Low, Light, Medium | 83.6%, 51.3%, 51.7%, 50.6% | UPT: 0 Mbps, 11.2 Mbps, 11 Mbps, 10.5 Mbps |  |
| Spreadtrum [R1-2211241] | Prolonging the periodicity of SSB/SIB1/paging: 1) SSB burst periodicity is 160ms, and SIB1 repetition periodicity is 160ms. 2) PF periodicity at gNB side is 160ms (T=1280ms, N=8). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. | Cat 1 | Zero | Set 1- Set 3: 23.8%, 19.6%, 16.3% |  | 1) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. 2) PF periodicity at gNB side is 20ms (T=1280ms, N=64). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. |
|  |  | Cat 2 |  | Set 1- Set 3: 9.3%, 8.3%, 9.4% |  |  |
|  | Transmission window of SSB/SIB1/paging: 1) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. 2) PF periodicity at gNB side is 20ms (T=1280ms, N=64). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. | Cat 1 |  | Set 1- Set 3: 23.8%, 19.6%, 16.3% |  | 1) The transmission window periodicity is 1280ms, and the transmission window duration is 160ms. 2) SSB burst periodicity is 20ms within the transmission window, and SIB1 repetition periodicity is 20ms within the transmission window. 3) PF periodicity at gNB side is 160ms (T=1280ms, N=8) within the transmission window. 4) gNB can enter light sleep for Cat 1, and can enter both light sleep and micro sleep for Cat 2 (at the tail of the transmission window). |
|  |  | Cat 2 |  | Set 1- Set 3: 51.5%, 47.3%, 20.6% |  |  |
| Intel [R1-2212563] | Increasing the common channel/signal periodicity | Cat 1 | Low | 40.1% | UPT: 819.66 Mbps Avg EE* (baseline): 5.10 Avg EE (ES scheme): 9.17 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity. EE* is defined as cell throughput (in Mbps) / average power consumption (in relative power), and averaged from all BS. |
|  |  |  |  | 45.0% | UPT: 819.66 Mbps Avg EE (baseline): 5.10 Avg. EE (ES scheme): 10.60 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  |  | Light | 14.6% | UPT: 611.45Mbps Avg EE (baseline): 2.66 Avg. EE (ES scheme): 3.31 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 16.8% | UPT: 611.45Mbps Avg EE (baseline): 2.66 Avg. EE (ES scheme): 3.46 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  |  | Medium | 6.2% | UPT: 457.92Mbps Avg EE (baseline): 1.50 Avg. EE (ES scheme): 1.63 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 7.1% | UPT: 457.92Mbps Avg EE (baseline): 1.50 Avg. EE (ES scheme): 1.65 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  | Cat2 | Low | 8.2% | UPT: 819.66Mbps Avg EE (baseline): 35.82 Avg. EE (ES scheme): 39.23 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 10.9% | UPT: 819.66Mbps Avg EE (baseline): 35.82 Avg. EE (ES scheme): 40.09 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
|  |  |  | Light | 5.1% | UPT: 611.45Mbps Avg EE (baseline): 20.75 Avg. EE (ES scheme): 22.00 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 5.8% | UPT: 611.45Mbps Avg EE (baseline): 20.75 Avg. EE (ES scheme): 22.19 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
|  |  |  | Medium | 3.0% | UPT: 457.92Mbps Avg EE (baseline): 12.44 Avg. EE (ES scheme): 12.89 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 3.4% | UPT: 457.92Mbps Avg EE (baseline): 12.44 Avg. EE (ES scheme): 12.96 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms. ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
| CATT [R1-2211210] | Adaptation of common signals and channels | Cat 1 | Zero load | 10.2%, 72.7%, 84.8% |  | Baseline: 20ms SSB; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Low load | 3.4%, 18.8%, 19.7% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Light load | 1.9%, 5.2%, 5.6% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Medium load | 1.3%, 2.2%, 2.6% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
| Fujitsu [R1-2211085] | SSB/SIB1 period= 40ms | Cat2 | Zero, low, light, medium | 17.9%, 13.7%, 11.1%, 8.6% |  | Baseline scheme: 20 ms SSB/SIB1 period |
|  | SSB/SIB1 period= 80ms |  | Zero, low, light, medium | 26.8%, 20.6%, 16.7%, 12.8% |  |  |
|  | SSB/SIB1 period= 160ms |  | Zero, low, light, medium | 31.4%, 24.1%, 19.4%, 15.0% |  |  |
|  | SSB/SIB1 period= 40ms |  | Zero, low, light, medium | 18.3%, 12.6%, 9.4%, 6.9% |  |  |
|  | SSB/SIB1 period= 80ms |  | Zero, low, light, medium | 27.4%, 18.8%, 14.1%, 10.4% |  |  |
|  | SSB/SIB1 period= 160ms |  | Zero, low, light, medium | 32.0%, 22.0%, 16.5%, 12.1% |  |  |
| Ericsson [R1-2212154] | 40ms SSB+SIB1 | Cat1 | Zero | 0.9% |  | Baseline scheme: 20ms SSB + 160ms SIB1 ES: one SSB. Energy calculation: per symbol energy consumption is modeled. |
|  | 80ms SSB+SIB1 |  |  | 48.5% |  |  |
|  | 160ms SSB+SIB1 |  |  | 72.6% |  |  |
|  | 40ms SSB+SIB1 |  |  | -6.2% |  | Baseline scheme: 20ms SSB + 160ms SIB1 ES: Four SSBs. Energy calculation: per symbol energy consumption is modeled. |
|  | 80ms SSB+SIB1 |  |  | 43.8% |  |  |
|  | 160ms SSB+SIB1 |  |  | 70.5% |  |  |
| Qualcomm [R1-2212128] | Adaptation of Common Signals and Channels | Category 1 | No Load | 13.9% | Access delay/latency: additiona 20 ms; UE power consumption increment: 99% | Note: "SSB period of 40 ms" without any network traffic either in DL or UL. Therefore, there are no statistics for UPT, latency, etc.. |

Based on the results with static configurations from 9 sources, it can be observed that longer SSB/SIB1 periodicity can bring BS with significant energy savings in most cases, compared to a selected baseline, for both BS Categories, under all reference configurations. When other configurations/settings are the same, the saving gain generally increase as the periodicity becomes larger, and decrease as the traffic load increases or the number of SSBs increases. Particularly, there are two companies providing results with SSB periodicity larger than 160ms which is the maximum value that is currently supported, i.e., being 640ms and 1280ms, and observed that together with longer SIB1/RACH/RO monitoring periodicities, then depending on the traffic load, the BS energy saving gain can be 53.6%~7.1% and 83.6%~3.4%, respectively, compared to a baseline with 20ms SSB periodicity.

The scheme does not affect the UPT for empty load case. When traffic occurs and load increases, the UPT also significantly decreases. The latency/access delay/UE power consumption increases proportionally as the periodicity of SSB/SIB increases compared to a corresponding baseline.

Performance of dynamic SSB/SIB1 periodicity adaptation is not provided.

=== end ===

| Company | Comments |
| --- | --- |
| QCOM1 | Suggest updating as follows: Based on the results with static configurations, it can be observed that longer SSB/SIB1 periodicity can bring BS with significant energy savings in most cases, compared to a selected baseline, for both BS Categories, under all reference configurations ... The scheme does not affect the UPT for empty load case. When traffic occurs and load increases, the UPT also significantly decreases. The latency/access delay/UE power consumption increases proportionally as the periodicity of SSB/SIB increases compared to a corresponding baseline. Performance of dynamic SSB/SIB1 periodicity is not provided. The UE power consumption is [also negatively impacted]. |
| FL | Adopt the revisions to the table and to the texts as ¦QC suggested |
| LG Electronics | Observations show that saving gain decreases as the number of SSB increases, but there seems to be no result of the company that adjusted the number of SSBs. Therefore, we suggest the following modification: When other configurations/settings are the same, the saving gain generally increase as the periodicity becomes larger, and decrease as the traffic load increases or the number of SSBs increases. |
| Spreadtrum | Our second scheme in technique A-1-5 should be moved to technique A-1-7, since our second scheme is similar to the scheme of Intel in technique A-1-7, and it seems the schemes here mean the extending SSB/SIB1 directly. |
| Offline | Based on the results with static configurations from 9 sources, … |

##### draftTP A-1-7: start

The following show the BS energy savings by configuration/adaptation of transmission patterns of common signals, i.e. Paging or SSB based on the submitted results.

Table 6.1.1-x: BS energy savings by adapting Paging/SSB transmission patterns

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/access delay/latency/UE power consumption | Reference configuration | Baseline configuration/assumption | Other evaluation methodology/assumption details/notable settings |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intel [R1-2212563] | Enhanced Paging by increasing the number of consecutive POs within a PF by factor of M while reducing PF density by a factor of M. This keeps the total number of POs same within the DRX cycle. | Cat1 | Zero, Paging load 2% | 21.2% |  | Set 1 | Paging Parameters: N = T/4; Ns = 4; Enh. Paging†: N = T/4; Ns = 4; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 80 msec periodicity; Number of SSB: 1 Paging load is the average load per simulation run time. Paging events were randomly generated. Same as below results. The value of T is larger than 160ms. |
|  |  |  | Zero, Paging load 0.2% | 4.0% |  |  | Paging Parameters: N = T/16; Ns = 2; Enh. Paging†: N = T/16; Ns = 2; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 80 msec periodicity; Number of SSB: 1 |
|  |  |  | Zero, Paging load 2% | 42.3% |  |  | Paging Parameters: N = T/4; Ns = 4; Enh. Paging†: N = T/4; Ns = 4; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 1; |
|  |  |  | Zero, Paging load 0.2% | 6.7% |  |  | Paging Parameters: N = T/16; Ns = 2; Enh. Paging†: N = T/16; Ns = 2; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 1; |
|  |  |  | Zero, Paging load 3.6% | 18.9% |  |  | Paging Parameters: N = T/4; Ns = 4; Enh. Paging†: N = T/4; Ns = 4; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 80 msec periodicity; Number of SSB: 4; SSB and SIB1 contained in same slot. 1 SSB per slot along with SIB1 to maximize SSB/SIB1 packing; |
|  |  |  | Zero, Paging load 0.5% | 0.2% |  |  | Paging Parameters: N = T/16; Ns = 2; Enh. Paging†: N = T/16; Ns = 2; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 80 msec periodicity; Number of SSB: 4; SSB and SIB1 contained in same slot. 1 SSB per slot along with SIB1 to maximize SSB/SIB1 packing; |
|  |  |  | Zero, Paging load 3.6% | 26.4% |  |  | Paging Parameters: N = T/4; Ns = 4; Enh. Paging†: N = T/4; Ns = 4; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 4; SSB and SIB1 contained in same slot. 1 SSB per slot along with SIB1 to maximize SSB/SIB1 packing; |
|  |  |  | Zero, Paging load 0.5% | 0.3% |  |  | Paging Parameters: N = T/16; Ns = 2; Enh. Paging†: N = T/16; Ns = 2; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 4; SSB and SIB1 contained in same slot. 1 SSB per slot along with SIB1 to maximize SSB/SIB1 packing; |
| Qualcomm [R1-2212128] | Adaptation of Common Signals and Channels | Category 1 | No Load | 10.3% | UE power consumption: -4% | FR2 Set 3 |  | "Compact SSB" without any network traffic either in DL or UL. Therefore, there are no statistics for UPT, latency, etc.. |

Based on the results,

- One company observed that for BS category 1 and at empty load case, statically adapting paging configuration could provide BS energy savings by 0.3%~6.7% when paging load (resource used for paging) is 0.2%~0.5%, and the gain can be up to 42.3% when paging load is increased up to 3.6%. The gain could also increases as the number of SSB increases. Performance of dynamically adapting paging configurations is not provided. The above energy saving gains were achieved with SSB periodicity of 80ms or 160ms.

- One company observed that having compact SSB (i.e., no time gap between consecutive SSBs) could provide 10.3% network energy saving for BS category 1 and at empty load case in FR2 when SSB periodicity is 20ms. Furthermore, UE power saving can be improved by 4%.

=== end ===

| Company | Comments |
| --- | --- |
| QCOM1 | Suggest updating the text as follows: Based on the results, - One company observed that statically adapting paging configuration could provide 0.3%~42.3% network energy saving for BS category 1 and at empty load case. The gain generally increases as paging load increases and decreases when the number of SSB increases. Performance of dynamically adapting paging configurations is not provided. - One company observed that having compact SSB (i.e., no time gap between consecutive SSBs) could provide 10.3% network energy saving for BS category 1 and at empty load case in FR2 when SSB periodicity is 20ms. Furthermore, UE power saving can be improved by 4%. On UPT/access delay/latency, [] On UE power consumption, adaptation of SSB has negative impact. |
| FL | Adopt the above and add one note that gNB allocation of S-TMSI to concentrate the PF/POs where paging needs to be transmitted is not considered. [in the most probable scenarios, i.e. zero load in the system and low paging load, the network energy savings range in the order of 0.3 to ~6.7%] |
| LG Electronics | For the last texts in the observation, it is not clear which company's results are related to the number of SSB increase. Therefore, we suggest the following modification: The gain generally increases as paging load increases or SSB/SIB/PRACH periodicity increases, but decreases when the number of SSB increases. Based on the discussion for Intel’s proposal during offline session, the following also can be captured. In Intel’s evaluation results for PFs/POs adaptation, it was assumed that paging load was generated only for Rel-18 NES UEs. |
| Intel | After giving more thought, gNB allocation of S-TMSI to concentrate the PF/PO cannot be really consider a benchmark since not really the same. Since this effectively increase the paging load probability for a given PO. Comparison of PO concertation cannot be really compared with what we have compared as we are simply comparing the different PF/PO positions for a given paging load. Even if careful selection of S-TMSI is possible, this can in theory apply to both legacy and enhance paging scheme. Given that this aspect is more relevant for RAN2, we don’t think it will be such a good idea to capture the note “that gNB allocation of S-TMSI to concentrate the PF/POs where paging needs to be transmitted is not considered”. This might be mis-leading, as mentioned the concentration of PO is simply evaluating a different paging load, which we have done in our evaluations. So this might not be accurate. As for the text in bracket [in the most probable scenarios, i.e. zero load in the system and low paging load, the network energy savings range in the order of 0.3 to ~6.7%] not sure how the “most probably scenario” was determined by the commenter. In our evaluations, we simply try to provide a range a paging loads to provide insights for various ranges. Its not clear to us how “most probable scenario” was determined. I think moderator’s original text or text Qualcomm seems to capture things objectively as the evaluation shows. We suggest not to capture subjective aspects, as it simply open the door for further discussion which may not be useful. LG’s comment seemed to be generally ok. Instead of “… was generated only for Rel-18 NES UEs”, it might be better to state “… was generated with UEs that support paging enhancements”. |
| Spreadtrum | Our second scheme in technique A-1-5 should be moved in here. Our second scheme in technique A-1-5 means “Transmission window of SSB/SIB1/paging”. It is similar to Intel’s scheme here. The difference we assume SSB/SIB1 is also confined in the “paging transmission window” and kept 20ms periodicity within the “paging transmission window”. In this way, UE does not need to wake up too early before PO to process SSB, so the power consumption of paging reception is not impacted negatively. Therefore, we suggest the following revision base on QC’s version. Based on the results, - One company observed that statically adapting paging configuration could provide 0.3%~42.3% network energy saving for BS category 1 and at empty load case. The gain generally increases as paging load increases and decreases when the number of SSB increases. Performance of dynamically adapting paging configurations is not provided. - One company observed that having compact SSB (i.e., no time gap between consecutive SSBs) could provide 10.3% network energy saving for BS category 1 and at empty load case in FR2 when SSB periodicity is 20ms. Furthermore, UE power saving can be improved by 4%. - One company observed that using a transmission window for SSB/SIB1/PO (20ms periodicity within the window) and extending the periodicity of the transmission window to extend SSB/SIB1/PO periodicity (160ms in average) could provide 16.3%~23.8% network energy saving for BS category 1 and 20.6%~51.5% network energy saving for BS category 2 at empty load case. |

##### draftTP A-1-8: start

The following show the BS energy savings by dynamically adapting common signals, i.e. RACH based on the submitted results.

Table 6.1.1-x: BS energy savings by dynamically adapting RACH periodicity/occasions

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/access delay/latency/UE power consumption | Reference configuration | Baseline configuration/assumption/notable settings | Other evaluation methodology/assumption details |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson [R1-2212154] | PRACH periodicity= 20ms | Cat1 | Zero | 14.4% | Access delay/latency: 10ms increase | Set 1 | Baseline scheme: 20 ms SSB, 40ms SIB1 period, 10ms PRACH periodicity. Per symbol energy consumption is modeled. ES scheme: adapting PRACH periodicity for energy efficiency via dynamic PRACH occasions adaptation. Note separate evaluation performed for different PRACH periodicities (i.e. no switching between these settings). | 1 SSB |
|  | PRACH periodicity= 40ms |  |  | 20.9% | Access delay/latency: 30ms increase |  |  |  |
|  | PRACH periodicity= 80ms |  |  | 22.2% | Access delay/latency: 70ms increase |  |  |  |
|  | PRACH periodicity= 20ms |  |  | 17.3% | Access delay/latency: 10ms increase |  |  | four SSBs |
|  | PRACH periodicity= 40ms |  |  | 23.9% | Access delay/latency: 30ms increase |  |  |  |
|  | PRACH periodicity= 80ms |  |  | 24.9% | Access delay/latency: 70ms increase |  |  |  |

Based on the results with multiple static RACH occasion configurations, it is one company observed that dynamic adaptation of RACH occasions can achieve BS energy savings by 14.4%~24.9% for BS Category 1 at empty load case under FR1 TDD compared to 10ms RACH periodicity without adaptation. The gain generally increases as PRACH periodicity increases for the same number of SSBs. Performance of dynamic RACH configuration is not provided.

On UPT/access delay/latency, this scheme increases access delay/latency from 10ms to 70ms, proportional to the increased PRACH periodicity.

=== end ===

| Company | Comments |
| --- | --- |
| QCOM1 | Suggest the following update: “Based on the results with static RACH occasion configurations, it is one company observed that dynamic adaptation of RACH occasions can achieve BS energy savings by 14.4%~24.9% for BS Category 1 at empty load case under FR1 TDD compared to 10ms RACH periodicity without adaptation. The gain generally increases as adapted PRACH periodicity increases, and also increases as the number of SSBs increases. Performance of dynamic RACH configuration is not provided.” |
| Ericsson1 | - Suggest adding the following in the last column of the table. Note: PRACH resources configured via SIB1 are used to support Rel-15/16/17/18 UEs. Dynamic adapted RACH resources are additionally available for Rel-18 UEs. - Suggest following changes on top of QC updates “Based on the results with multiple static RACH occasion configurations, it is one company observed that dynamic adaptation of RACH occasions can achieve BS energy savings by 14.4%~24.9% for BS Category 1 at empty load case under FR1 TDD compared to 10ms RACH periodicity without adaptation. The gain generally increases as adapted PRACH periodicity increases, and also increases as the number of SSBs increases. Performance of dynamic RACH configuration is not provided.” |
| LG Electronics | We are fine with the second changes in Ericsson’s comments while the first change seems not necessary but if needed it can be captured in 6.X.Y.3. |

##### === (Consensus) draftTP A-1-9: start ===

The following show the BS energy savings by scheduling of SIB1 by SSB, without PDCCH for SIB1, with repetition period 20ms.

Table 6.1.1-x: BS energy savings by scheduling of SIB1 by SSB

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT Access delay/latency UE power consumption Other KPI(s), if any | Reference configuration | Baseline configuration/assumption | Traffic model | Other evaluation methodology/assumption details - Part 1 (other than power modeling aspects) | Other evaluation methodology/assumption details - Part 2 (power modeling aspects) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CEWiT+B417:R417 [R1-2212429, R1-2212765] | Scheduling of SIB1 by SSB, without PDCCH for SIB1, with repetition period 20ms | Cat.1 | Zero | 4.8% |  | Set 1 | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |  | numerical analysis | •SIB1: PDCCH: 3 symbols; PDSCH: 12 OFDM symbols including DMRS. •ղ=1, A=0.4. •Time unit for power model is ms. power consumption is calculated in a 20ms long period |
| CEWiT [R1-2212429, [R1-2212765] | Scheduling of SIB1 by SSB, without PDCCH for SIB1, 4 beams, with repetition period 20ms | Cat.1 | Zero | 11.4% |  | Set 1 | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |  | numerical analysis | •SIB1: PDCCH: 3 symbols; PDSCH: 12 OFDM symbols including DMRS. •ղ=1, A=0.4. •Time unit for power model is ms. power consumption is calculated in a 20ms long period |
| CEWiT [R1-2212429, R1-2212765] | Scheduling of SIB1 by SSB, without PDCCH for SIB1, 8 beams, with repetition period 20ms | Cat.1 | Zero | 14.8% |  | Set 1 | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |  | numerical analysis | •SIB1: PDCCH: 3 symbols; PDSCH: 12 OFDM symbols including DMRS. •ղ=1, A=0.4. •Time unit for power model is ms. power consumption is calculated in a 20ms long period |

It is observed on one source that using SSB to schedule SIB1 can obtain 4.8%~14.8% BS energy savings for Set 1 reference configuration for BS Category 1, compared to SSB/SIB1 periodicity of 20 ms for both.

=== end ===

| Company | Comments |
| --- | --- |
| QCOM1 | Suggest the following update: It is One company observed that using SSB to schedule SIB1 |
| CEWiT | We suggest to updatethe observation as follows: It is observed that using SSB to schedule SIB1 can obtain 4.8 to 14.8% BS energy savings for Set 1 reference configuration for BS Category 1, compared to SSB/SIB1 periodicity of 20 ms for both. |
| Offline | Please see the above revisions |

6.1.1.3	Specification impacts

6.1.2	Technique A-3 XX

6.1.2.1	Description of technique

6.1.2.2	Analysis of performance and impacts

##### === (Consensus) draftTP A-3: start ===

The following capture the results for waking up gNB triggered by UE wake up signal (WUS).

Table 6.1.1-x: BS energy savings by UE wake up signal (WUS)

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT | Access delay/latency | UE power consumption | Reference configuration | Baseline configuration/assumption | Traffic model | Other evaluation methodology/assumption details/notable settings |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MTK [R1-2212259] | UE_can_wake_up_gNB | Cat 1 | Low | 49.3% | 0.00% | 0.00% | 0.07% | Set 1 | All 21 cells active | VoIP | SLS; DRX (40, 4, 10); 9 out of 21 cells remian active. BS power consumption value is sum of 21 cells. |
|  |  | Cat 2 |  | 51.9% | 0.00% | 0.00% | 0.07% |  |  |  |  |
| ZTE, Sanechips [R1-2211903] | UE WUS is used to wake up a gNB in an energy saving state without DL transmission including SSB/SIB1 | 1 | low | 7.4% 19.6% 23.8% | 0.66% 2.59% 5.04% |  |  | Set 1 | no WUS, cell is in a normal state with {20ms/40ms} SSB/SIB periodicity | FTP3 | UE mobility. slot-level; Pstatic=P3, η(s_f,s_p )=1; time-domain scaling for SSB; time and frequency domain scaling for SIB. WUS period=20ms/80ms/160ms for each load. |
|  |  |  | light | 4.9% 12.7% 15.5% | 0.11% 0.43% 0.86% |  |  |  |  |  |  |
|  |  | 2 | low | 6.2% 6.4% 6.5% | 0.66% 2.59% 5.04% |  |  |  | no WUS, cell is in a normal state with {20ms/40ms} SSB/SIB periodicity |  |  |
|  |  |  | light | 4.5% 4.6% 4.7% | 0.11% 0.43% 0.86% |  |  |  |  |  |  |
| vivo [R1-2211018, R1-2212541] | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) | Cat 1 | 0% | 29.7% 66.6% 80.7% |  |  | 0.00% 0.00% 0.00% | Set 1 | legacy BS, where all cells are always in the normal mode. Normal mode: 20ms SSB and SIB1, 20ms RACH listening | NaN NaN NaN | SLS No UE DRX 100% detection reliability |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 0.002% | 27.3% 60.4% 72.8% | 0.8% 15.5% 21.7% | 5.68% 38.73% 39.53% | 0.00% 0.00% 0.00% |  |  | FTP3, mean packet interval of 10s, packet size of 100bytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 20.55% 20.81% 20.49% | 0.8% 4.3% 6.0% | 3.4% 4.5% 8.6% | 9.70% 20.72% 32.51% | 0.98% 1.46% 1.66% |  |  | FTP3, mean packet interval of 200ms, packet size of 0.5Mbytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 41.79% 41.17% 41.35% | -2.4% 0.3% 0.1% | 2.7% 6.0% 7.2% | 8.95% 14.55% 20.53% | 1.13% 1.51% 1.97% |  |  |  |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 0% | 32.1% 69.6% 83.7% |  |  | 0.00% 0.00% 0.00% |  |  | NaN NaN NaN |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 0.002% | 29.4% 63.3% 75.6% | 0.8% 16.5% 24.2% | 4.17% 38.05% 39.53% | 0.00% 0.00% 0.00% |  |  | FTP3, mean packet interval of 10s, packet size of 100bytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 20.71% 20.51% 20.66% | -0.1% 6.4% 6.6% | 3.9% 7.0% 8.7% | 11.04% 20.31% 29.07% | 1.08% 1.33% 1.65% |  |  | FTP3, mean packet interval of 200ms, packet size of 0.5Mbytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 41.74% 41.91% 42.07% | -2.2% -0.7% -0.6% | 1.3% 6.6% 7.5% | 10.32% 16.58% 18.21% | 1.13% 1.63% 1.84% |  |  |  |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) | Cat 2 | 0% | 19.1% 19.4% 19.4% |  |  | 0.00% 0.00% 0.00% |  |  | NaN NaN NaN |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 0.002% | 18.1% 18.3% 18.3% | 0.76% 5.40% 11.79% | 0.58% 8.98% 20.16% | 0.00% 0.00% 0.00% |  |  | FTP3, mean packet interval of 10s, packet size of 100bytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 20.58% 20.28% 20.76% | 0.5% 1.0% -0.4% | 0.69% 1.02% 2.88% | 7.93% 9.93% 17.27% | 0.64% 0.56% 0.99% |  |  | FTP3, mean packet interval of 200ms, packet size of 0.5Mbytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 41.46% 41.22% 41.04% | -2.4% -2.1% -1.8% | 0.05% 0.30% 0.45% | 5.94% 10.07% 11.62% | 0.99% 1.10% 0.96% |  |  |  |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 0% | 20.3% 20.6% 20.6% |  |  | 0.00% 0.00% 0.00% |  |  | NaN NaN NaN |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 0.002% | 19.2% 19.4% 19.5% | 0.85% 4.17% 10.53% | 2.63% 9.83% 20.86% | 0.00% 0.00% 0.00% |  |  | FTP3, mean packet interval of 10s, packet size of 100bytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 20.55% 20.61% 21.26% | 0.5% 0.3% -1.0% | 0.36% 0.61% 2.21% | 8.30% 10.14% 17.32% | 0.71% 0.73% 1.11% |  |  | FTP3, mean packet interval of 200ms, packet size of 0.5Mbytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 42.05% 41.38% 41.74% | -3.1% -2.3% -2.9% | 0.10% 0.21% 0.36% | 7.48% 9.16% 10.22% | 1.26% 0.98% 1.04% |  |  |  |  |
| NOKIA/NSB [R1-2211097] | Wake up of gNB triggered by UE wake up signal (WUS) @ 20ms | Cat 2 | Low | 45.6% | 13,01 Mbps |  |  | Set 1 | SSBs/SIB1s/RO monitoring @ 20ms default periodicity UEs are initially in RRC_idle state | UL - IM | SLS+Post-processing |
|  | Wake up of gNB triggered by UE wake up signal (WUS) @ 160ms |  |  | 51.9% | 6,08 Mbps |  |  |  |  |  |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS) @ 640ms |  |  | 52.5% | 2,15 Mbps |  |  |  |  |  |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS) @ 1280ms |  |  | 66.7% | 1,16 Mbps |  |  |  |  |  |  |
| Samsung [R1-2212543] | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy | Cat 1 | Low | 70.3% 64.0% 57.2% |  | 0.00% 0.00% 0.00% |  | Set 2 | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 90. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @15 ms WUS periodicy |  |  | 76.4% 69.4% 61.8% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 80.1% 73.7% 66.7% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 55. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @15 ms WUS periodicy |  |  | 83.7% 76.5% 68.8% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 92.8% 86.2% 79.0% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 10. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @15 ms WUS periodicy |  |  | 93.0% 85.7% 77.8% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 45.0% 39.2% 32.8% |  | -29.56% -28.9% -29.41% |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 90. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 39.1% 32.6% 25.7% |  | -45.37% -45.15% -45.51% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 67.1% 60.2% 52.7% |  | -22.44% -22.85% -22.79% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 64.7% 58.5% 51.8% |  | -29.56% -28.9% -29.41% |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 55. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 60.8% 54.1% 46.7% |  | -45.37% -45.15% -45.51% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 78.0% 70.9% 63.2% |  | -22.44% -22.85% -22.79% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 90.0% 83.4% 76.3% |  | -29.56% -28.9% -29.41% |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 10. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 88.9% 81.6% 73.8% |  | -45.37% -45.15% -45.51% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 92.0% 84.7% 76.8% |  | -22.44% -22.85% -22.79% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
| Qualcomm [R1-2212128] | Wake up of gNB triggered by UE two symbol wake up signal (WUS) | Category 1 | No Load | 18.7% |  |  |  | FR2 Set 3 | "light SSB" combined with UL WUS and on demand SIB 1 |  |  |

For UE WUS signal triggering SSB/SIB1/RACH for RRC IDLE/INACTIVE/CONNECTED mode, based on results from 4 sources, it is observed that, with UE WUS signal triggering a BS of 100% detection assumption,

- With C-DRX, at low load, one source observed about 50% network energy savings with marginal UE power increment, without UPT loss observed. The scheduling delay when switching to a new gNB is not modelled.

- For the evaluations with assumption of RRC_IDLE/INACTIVE mode without C-DRX,

  - without DL transmission including DL common signals before gNB reception of WUS, with WUS period of 20ms, 80ms and 160ms, at zero or low load, the network energy savings could be 7.4%~32.1% (6.2%~45.6%), 19.6%~69.6% (6.4%~51.9%), 23.8%~ 83.7% (6.5%~52.5%) respectively by using Category 1 (Category 2) BS power model. The savings can increase as the WUS period increases, and decrease as the traffic load increases. When WUS period is 20ms, marginal UPT loss, access delay/latency increment and UE power consumption increment are observed. The UPT loss and access delay/latency increases as WUS periodicity increases, while there is marginal UE power consumption increment.

  - With sparse SSB of 160ms periodicity transmitted before gNB reception of WUS, at zero or low load, 27.3%~29.7% (18.1%~19.1%), 60.4%~66.6% (18.3%~19.4%), 72.8%~80.7% (18.3%~19.4%) network energy savings can be achieved with WUS period of 20ms, 80ms and 160ms respectively by using Category 1 (Category 2) BS power model. When WUS period is 20ms, marginal UPT loss, access delay/latency increment and UE power consumption increment are observed. The UPT loss and access delay/latency increases as WUS periodicity increases, while there is marginal UE power consumption increment.

- Note: gNB coordination for WUS reception is assumed. Resource configuration for WUS is not specifically modelled, while one source assumes the configuration of WUS can be obtained from a camping cell. For the case of no DL transmission, also gNB synchronization is further assumed.

- Note: For evaluation results from 2 source, it is assumed that UE achieves timing for the UL WUS transmission from the other cell. For evaluation results from 2 source, it is assumed that UE achieves synchronization with the gNB targeted for energy saving utilizing discovery signal from the same cell, and one source assumed the discovery signal contains PSS only and its use is to help the UE to get synchronized and to be able to transmit an uplink trigger signal. The differentiation of multiple gNBs which has detected the WUS is not modelled.

  - The detection of WUS is assumed to be ideal. False triggering for detection of targeting gNB is not considered.

For UE WUS signal triggering gNB to wake up in case of uplink traffic arrival, for RRC_CONNECTED without C-DRX and without DL common signals/DL transmission other than PDCCH carrying UL grant, with the assumption of a separate receiver used and 100% detection assumption, at low load, 1 source observed that,

- With WUS detection power of 10, 55 or, with 90 which has the same active UL power

  - When the WUS periodicity is same as the baseline of SR periodicity, 77.8%~93%, 66.7%~92.8% or 57.2%~76.4% network energy savings could be achieved respectively;

  - When the WUS periodicity is smaller than the SR periodicity of the baseline, 76.3%~92%, 46.7%~78% or 25.7%~67.1% network energy savings could be achieved respectively;

  - For each case, the gain generally increases as the WUS periodicity increases and decreases as the traffic load increases. The gain could also increase as the gNB detection power decreases.

  - There is latency reduction observed, which could increase as the periodicity of WUS decreases. The gain can be up to 45%.

- The assumption is that gNB needs to wake up to detect SR but can detect WUS during sleep state. gNB is assumed to be in a state such that the main UL receiver is still in deep sleep when detecting wake-up signal and gNB is able to wake up from deep sleep to active in one slot after WUS detection. The WUS receiver is assumed to be active only when detection of WUS signal and becomes 0 power in other time.

When combined with a light version of SSB and on demand SIB1, one source observed 18.7% network energy savings at low load for FR2, assuming the light version of SSB contains PSS only and its use is to help the UE to get synchronized and to be able to transmit an uplink trigger signal.

=== end ===

| Company | Comments |
| --- | --- |
| Nokia/Nsb | We have the following re-wording proposal: “On UPT/access delay/latency, compared to a baseline where the cell is operating with SSB/SIB/RACH monitoring, the scheme has some UPT loss or increased latency for UEs initially in RRC IDLE/INACTIVE state observed with longer periodicity of WuS as compared to RACH in legacy operation, and it is reduced as traffic load increases.” |
| Samsung | It seems there is some misalignment regarding the understanding of ‘Access delay/latency’, for clarification, our results for ‘Access delay/latency’ are latency reduction instead of latency increase. We suggest to change ‘Access delay/latency’ to ‘Access delay/latency reduction’ or ‘Access delay/latency increase’ to avoid misunderstanding. There are two options for A-3 and the observation for Option 2 is not captured, we suggest to the following update. On UPT/access delay/latency, - compared to a baseline where the cell is operating with SSB/SIB/RACH monitoring, the scheme has some UPT loss or increased latency, which is reduced as traffic load increases. - compared to a baseline where the cell has to wake up to monitor SR, the scheme has up to ~45% latency reduction gain which increases as the periodicity of WUS decreases. |
| CATT | We would like to add “Notes” on the observation - Note1: UE does not synchronize with the gNB in the energy saving state when it transmit the WUS to wake up the gNB - The gNB in the energy saving state will have large detection window for WUS detection due to UE is synchronizing with its serving cell only. The detection of low-power WUS is assumed to be ideal without co-channel interference. |
| QCOM1 | Suggest the following update: On UE power consumption, two companies showed small negative impact is observed. @Samsung: Why do you think the WUS is needed for SR? Do you assume that WUR at gNB is a separate receiver from the current (main) receiver? [Samsung] In our simulation, we assume WUS detection does not impact gNB state transition to enable gNB staying longer in the sleep mode. A possible implementation could be using a separate receiver. |
| ZTE，Sanechips | More clarifications about our simulations are added in red for Technique#A-3. (The excel document has also been updated in V24). |
| vivo | One clarification on our reporting results: all our reported results are (baseline-enhance)/baseline. So negative value for latency/UE power means increased latency/UE power. If positive value is defined as increased latency/UE power, please change our reported negative value for latency/UE power to positive value. Besides, it seems evaluation scheme by Samsung is quite different with others. We suggest to separate the observation for it. |
| LG Electronics | We support adding NOTE suggested by CATT. @ Samsung, Regarding the evaluation results, could you please clarify what is the assumption for common signal/channel transmission (e.g., SSB) before receiving the WUS transmitted by the UE? In addition, the ES gains depend on the periodicity of SR and WUS, and it seems necessary to clarify the procedural differences between SR and WUS. [Samsung] The evaluation does not consider DL common signal/channel transmission. The main difference between WUS and SR is that gNB needs to wake up to receive SR but can receive WUS during sleep/inactive mode. |
| Samsung2 | Updated our evaluation results for latency reduction gain in red to align with FL’s understanding. Negative values mean latency reduction percentages. The note from CATT seems only apply to Option 1, it should not be taken as a general note. @QCOM, LG, please find our reply inline. |
| Spreadtrum2 | In our view, the UE can send UL request to the coverage cell, and the coverage cell can trigger the capacity cell to open. It is more logical. The UE sending the UL request to a discovered cell without fine DL sync even without UL sync seems too unreliable, and thus the capacity cell should perform the “non-coherent” blind detection of UL signal, which is power consumed at gNB. Moreover, UL interference should be controllable in the whole cellular network. gNB working on a heavy UL interference environment is not power efficient also. |
| vivo2 | Regarding the evaluation result table, - We have updated our result for latency and UE power consumption by changing to positive value with change remark to align with FL’s understanding. - It seems that there may be multiple values for the same row. However, it is not clear that the multiple values are corresponding to which case. It is better to add the following note for such row, .e.g. Note: Multiple values in the same row are corresponding to multiple cases with different WUS period. - As discussed online, there will be two different evaluation scheme. To make it more clear that which evaluation result is corresponding to which scheme, we prefer to split this table into two tables like what has been done for A-1. Regarding the observation, - For the title of two schemes discussed online For UE WUS signal triggering SSB/SIB1/RACH for RRC_idle/inactive/connected mode, based on results from XX sources, it is observed that, with UE WUS signal triggering a BS of 100% detection assumption, with or without UE C-DRX, For UE WUS signal triggering SR monitoring for RRC connected mode, based on results from XX sources, it is observed that, with UE WUS signal triggering a BS of 100% detection assumption, with or without UE C-DRX, - For the category of evaluation results, we suggest to describe the observation for different WUS period, e.g. If WUS period is 20ms, ES gain is X%-X%; |
| ZTE, Sanechips | Our simulation assumptions for the baseline have been updated. |

6.1.2.3	Specification impacts

6.1.2	Technique A-4 XX

6.1.2.1	Description of technique

6.1.2.2	Analysis of performance and impacts

##### === (Consensus) draftTP A-4: start ===

The following captures the results for adaptation of UE DTX/DRX.

Table 6.1.1-x: BS energy savings by adaptation of UE DTX/DRX

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT | Access delay/latency/UE power consumption/Other KPI(s), if any | a) Reference configuration b) Baseline configuration/assumption c) Traffic model d) Other evaluation methodology/assumption details/notable settings |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MTK [R1-2212259] | DRX_offset_alignment | Cat 1 | Low | 29.8% | 0.91% | Access delay/latency: 0.92% UE power consumption: 2.17% | Set 1 Random DRX offset (granularity = 5 ms) VoIP SLS; DRX (40, 4, 10); DRX offset aligned to 0 |  |  |  |
|  |  | Cat 2 |  | 13.7% | 0.91% |  |  |  |  |  |
| OPPO [R1-2211458] | DRX align | Cat 1 | low load(RU-9.3%) | 4.7% | 361.08Mbps(-15.5%) | Access delay/latency: 78.03ms(+50%) | Set 1 UE-specific DRX, SSB with 20 RBs for 20 ms periodicity |  | FTP (0.5MB as packet size, 200ms as mean inter-arrival time) | SLS, C-DRX config: FTP (160,100,8), DRX align |
|  |  |  | low load(RU-0.15%) | 6.7% | 85.91Mbps(-8.7%) | Access delay/latency: 143.55ms(+3.83%) |  |  | IM (0.1MB as packet size, 2s as mean inter-arrival time) | SLS, C-DRX config: IM (320,80,10), DRX align |
|  | DRX align and dropping SSB outside UE active time |  | low load(RU-9.3%) | 14.4% | 361.08Mbps(-15.5%) | Access delay/latency: 78.03ms(+50%) |  |  | FTP (0.5MB as packet size, 200ms as mean inter-arrival time) | SLS, C-DRX config: FTP (160,100,8), DRX align and dropping SSB outside UE active time |
|  |  |  | low load(RU-0.15%) | 70.1% | 85.91Mbps(-8.7%) | Access delay/latency: 143.55ms(+3.83%) |  |  | IM (0.1MB as packet size, 2s as mean inter-arrival time) | SLS, C-DRX config: IM (320,80,10), DRX align and dropping SSB outside UE active time |
| ZTE, Sanechips [R1-2211903] | DRX alignment | 1 | low | 0.3% 0.9% | 5% 1.30% | unfinished packet ratio=(total number of unfinished packet for baseline-total number of unfinished packet for enhanced)/total number of unfinished packet for baseline: 50%, 54.5% for each BS caterogy | Set 1 UE-specific CDRX FTP3 CDRX pattern for FTP3 CDRX alignment in a cell |  |  |  |
|  |  | 2 |  | 0.2% 0.4% | 5% 1.30% |  |  |  |  |  |
| Spreadtrum [R1-2211241] | traffic concentration (in a transmission window | Cat 1 | Low | 37.8% 34.9% 30.9% |  |  | a) For each BS Category: Set 1, Set 2, Set 3 b)- 1) There are 5% load (UE specific data) in 40 slots every 20ms. The load is frequency multiplexed with SSB burst and SIB1 in 2 slots every 20ms. 2) Scaling: Sf≈0.21 in 2 slots every 20ms, and Sf≈0.05 in 38 slots every 20ms c)-1) The load is concentrated in first 10ms. There are 10% load (UE specific data) in the first 20 slots every 20ms, zero load in the last 20 slots every 20ms. The load is frequency multiplexed with SSB burst and SIB1 in 2 slots every 20ms. 2) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. 3) Scaling: Sf≈0.26 in 2 slots every 20ms, and Sf≈0.1 in 18 slots every 20ms d)- 1) 160ms duration in total. 2) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. Two SSBs and the corresponding SIB1 share a slot. SSB burst and SIB1 take 40 PRBs. 3) PF periodicity at gNB side is 20ms (T=1280ms, N=64). Paging is transmitted in another slot every PF assuming one PO is effective in each PF. Paging takes 40 PRBs. 4) Scaling: Sa=1, Sp=1, P_static=P3. Numerial evaluation resutls. |  |  |  |
|  |  | Cat 2 |  | 31.1% 27.7% 29.2% |  |  |  |  |  |  |
|  | Offload between cells (the offloaded cell is turned off) | Cat 1 | Low | 57.7% 53.5% 47.9% |  |  | a) For each BS Category: Set 1, Set 2, Set 3 b)-1) Cell #1 and cell #2: There are 5% load (UE specific data) in 40 slots every 20ms. The load is frequency multiplexed with SSB burst and SIB1 in 2 slots every 20ms. 2) Scaling: Sf≈0.21 in 2 slots every 20ms, and Sf≈0.05 in 38 slots every 20ms c)-1) The load in cell #1 is shifted to cell #2. 1.1) Cell #1: There are zero load. There are only SSB burst and SIB1 in 2 slots every 20ms. gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. 1.2) Cell #2: There are 10% load (UE specific data) every 20ms. The load is frequency multiplexed with SSB burst and SIB1 in 2 slots every 20ms. 2) Scaling: 2.1) Cell #1: Sf≈0.16; 2.2) Cell #2: Sf≈0.26 in 2 slots every 20ms, and Sf≈0.1 in 38 slots every 20ms d)-1) 160ms duration in total. 2) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. Two SSBs and the corresponding SIB1 share a slot. SSB burst and SIB1 take 40 PRBs. 3) PF periodicity at gNB side is 20ms (T=1280ms, N=64). Paging is transmitted in another slot every PF assuming one PO is effective in each PF. Paging takes 40 PRBs. 4) Scaling: Sa=1, Sp=1, P_static=P3. Numerial evaluation results. |  |  |  |
|  |  | Cat 2 |  | 46.5% 44.3% 46.6% |  |  |  |  |  |  |
| Intel [R1-2212563] | Enhanced C-DRX | Cat1 | Light | 2.8% | Baseline: 122.3 Mbps ES: 86.4 Mbps | Avg EE (baseline): 5.20 Avg EE (ES): 4.82 | a)Set1 c) FTP3 d) SLS CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 1; Slot-level model For scaling: A = 0.4; η(s_f,s_p )=1 for any sf, sp; | Baseline DRX Parameters: DRX Cycle: 80 msec; ON duration 4ms, Inactivity Timer: 40msec For Enh C-DRX, cycle is 80ms and gNB is active for 20ms. |  |  |
|  |  |  | Medium | 29.7% | Baseline: 93.2 Mbps ES: 29.6 Mbps | Avg EE (baseline): 1.87 Avg EE (ES): 2.33 |  |  |  |  |
|  |  |  | Low | 2.3% | Baseline: 111.2 Mbps ES: 186.5 Mbps | Avg EE (baseline): 8.81 Avg EE (ES): 9.37 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Light | 2.3% | Baseline: 98.1 Mbps ES: 66.6 Mbps | Avg EE (baseline): 5.31 Avg EE (ES): 4.66 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 40ms |  |  |
|  |  |  | Light | 2.6% | Baseline: 98.1 Mbps ES: 164.3 Mbps | Avg EE (baseline): 5.31 Avg EE (ES): 5.31 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Medium | 30.9% | Baseline: 75.0 Mbps ES: 28.2 Mbps | Avg EE (baseline):1.97 Avg EE (ES): 2.54 |  | Baseline DRX Parameters: DRX Cycle: 160 msec; Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 40ms |  |  |
|  |  |  | Medium | 4.8% | Baseline: 75.0 Mbps ES: 116.6 Mbps | Avg EE (baseline):1.97 Avg EE (ES): 2.04 |  | Baseline DRX Parameters: DRX Cycle: 160 msec; Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
| CATT [R1-2211210] | Adaptation of DTX/DRX | Cat 1 | Low load | 71.4% |  |  | Set1 | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; | FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);DTX configuration: gNB starting offset of DTX on locate before UE DRX on duration in order to support UE wakeup;SSB periodicity: 20ms; CSI-RS/TRS periodicity: 10ms. |
|  |  |  | Light load | 62.6% |  |  |  |  | FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes |  |
|  |  |  | Medium load | 47.8% |  |  |  |  | FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes |  |

Based on 6 sources results, semi-static UE C-DRX alignment achieves BS energy savings gain by 0.2%~71.4% depending on the traffic, UE DRX configurations, and the assumed baseline e.g. random DRX offset per UE, or gNB is always ON to provide service to the UE. At low or light traffic load cases, 4 sources show that the gain can be 14.4%~71.4%, while 3 sources show less than 6.7% gain, depending on whether BS and UE active duration are aligned or not; at medium load case, 2 sources show network energy saving gain can be 4.8%~47.8%.  According to one source, dropping SSB outside UE active time can achieve the energy savings by 14.4%~70.1% and it is assumed that the UE active durations are aligned and the potential impact on synchronization and UE measurement outside the UE duration is not considered.

On UPT, one result shows there is marginal negative impact while one result shows it can be up to 15.5%. Also, one result shows that the impact on UPT varies: when the UE DRX cycle is 160ms and gNB active time is 80ms, the UPT is increased while in other configurations, there can be large UPT loss.

On access delay/latency, one result shows marginal increment while one result shows the increment can be up to 50%. Also, about 50% unfinished packet ratio is observed from one source compared to the baseline without UE C-DRX alignment during the evaluation period. The increments are related to the DRX configuration.

Additionally, one source shows that at low and medium load, the average EE is increased up to 28.93% when UE DRX alignment is assumed, whereas for light load case, average EE decreases up to 12.24% when UE DRX alignment is assumed.

=== end ===

| Company | Comments |
| --- | --- |
| OPPO | According to our evaluation results, when SSB burst periodicity is 20ms, compared with the baseline of UE DRX alignment, dropping SSB outside UE active time can provide additional energy saving gain of 9.7% (i.e. from 4.7% to 14.4% for FTP model) and 63.4% (i.e. from 6.7% to 70.1% for IM model) for FTP and IM model respectively, which is not negligible. |
| Intel | It seems our results are copied multiple times. It can be corrected. We suggest following revision on the observation. It is not clear how observation in last sentence is supported and we suggest to remove it. Based on the results, DTX/DRX adaptation achieves BS energy savings gain by 0.2%~71.4% depending on the traffic and UE DRX configurations. At low or light traffic load cases, four results (MediaTek, OPPO, Spreadtrum, CATT) show that the gain can be larger than 14.4% [include range] and becomes larger when traffic further increases, while three results (OPPO, ZTE, Intel) show less than 6.7% gain [include range] gain at low and light load cases. For medium load cases, X (company1, company2,..) results show [include range] gain. Particularly, dropping SSB outside UE active time can further increase the energy savings, and with baseline of UE DRX alignment, the gain can be marginal, less than 1%. |
| QCOM1 | Is the observation for C-DRX enhancements or for explicitly introducing DTX/DRX at gNB? It would be good to have clarification on this. |
| ZTE，Sanechips | More clarifications about our simulations are added in red for Technique#A-4. (The excel document has also been updated in V24). |
| vivo | Share the same clarification question as QCOM indicates Based on our understanding, most of the evaluation results are for enhanced C-DRX alignement. If this is true, we suggest the following revision: Based on the results, enhanced UE DRX alignement achieves BS energy savings gain by 0.2%~71.4% depending on the traffic and UE DRX configurations. At low or light traffic load cases, four results (MediaTek, OPPO, Spreadtrum, CATT) show that the gain can be larger than 14.4% and becomes larger when traffic further increases, while three results (OPPO, ZTE, Intel) show less than 6.7% gain at low and light load cases. Particularly, dropping SSB outside UE active time can further increase the energy savings, and with baseline of UE DRX alignment, the gain can be marginal, less than 1%. |
| LG Electronics | We agree with OPPO that according to OPPO’s results the effects of dropping SSB outside UE active time is not negligible. |
| Spreadtrum | We suggest: Based on the results, DTX/DRX adaptation (potentially needs UE C-DRX aligned to gNB DTX) achieves… |
| MTK2 | After the online session this morning, we would suggest a simpler modification for including the generic dependency for the difference in companies’ results: Based on the results, DTX/DRX adaptation achieves BS energy savings gain by 0.2%~71.4% depending on the traffic and UE DRX configurations. At low or light traffic load cases, four results (MediaTek, OPPO, Spreadtrum, CATT) show that the gain can be 14.4%- 71.4% and becomes larger when traffic further increases, while three results (OPPO, ZTE, Intel) show less than 6.7% gain at low and light load cases, depending on whether SSB activity is jointly considered or not. On UPT, one result shows there is marginal negative impact while one result shows it can be up to 15.5%. Also one result shows that the impact on UPT varies: when the UE DRX cycle is 160ms and gNB active time is 80ms, the UPT is increased while in other configurations, there can be large UPT loss. On access delay/latency, one result shows marginal increment while one result shows the increment can be up to 50%. Also, about 50% unfinished packet ratio is observed from one result. The increments are related to the DRX configuration and packet delay budget evaluated. Additionally, one result shows that at low/light traffic load, the average EE may be negatively impacted while at medium load, the average EE is slightly increased. |

6.1.2.3	Specification impacts

6.1.2	Technique A-6 XX

6.1.2.1	Description of technique

6.1.2.2	Analysis of performance and impacts

##### === (Consensus) draftTP A-6: start ===

The following capture the results for adaptation of SSB and/or SIB1.

Table 6.1.1-x: BS energy savings by adaptation of SSB and/or SIB1

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT | Access delay/latency/UE power consumption/Other KPI(s), if any | Reference configuration | Baseline configuration/assumption | Traffic model/Other evaluation methodology/assumption details/notable settings |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CATT [R1-2211210] | Adaptation of SSB/SIB1 | Cat 1 | Low load | 22.0% |  |  | set1 | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; | FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes. SLS;Cell OFF:Without normal SSB/SIB/CSI-RS transmission within Cell off duration;On demand SSB transmission is trigger by neighbour cell with 300ms transmission duration and 20ms SSB. For the case with DRX, DTX configuration: gNB starting offset of DTX on locate before UE DRX on duration in order to support UE wakeup; A=0.4; η(s_f, s_p)=1. |
|  |  |  |  | 43.4% |  |  |  | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms;DTX configuration: gNB starting offset of DTX on locate before UE DRX on duration in order to support UE wakeup; |  |
| Ericsson [R1-2212154] | 20ms Discovery signal (4 symbols) + no SIB1 | Cat1 | Zero | 2.6% / 5.9% |  |  | Set 1 | Baseline scheme: 20ms SSB + 160ms SIB1 | one SSB/ four SSBs Energy calculation: per symbol energy consumption is modeled. According to Rel-15 specification, SIB1 can be transmitted with variable transmission repetition periodicity within a 160 ms period, including one SIB1 PDSCH transmission every 160ms or even sparser. |
| Qualcomm [R1-2212128] | on-demand SIB1 | Cat 1 | Empty load | 5.8% / 7.7% / 8.6% |  |  | Set 1 | Baseline: 20ms periodicity for SSB/SIB1/RO, one beam Enhanced: 20%/10%/5% SIB1 Tx rate, C-WUS with 20ms periodicity |  |
|  |  |  |  | 32.1% / 36.6% / 38.8% |  |  |  | Baseline: 20ms periodicity for SSB/SIB1/RO, 8 beams Enhanced: 20%/10%/5% SIB1 Tx rate, C-WUS with 20ms periodicity |  |

One source shows that with a 4-symbol Discover signal (DRS), and without SIB1 transmission and for on-demand SIB1, 2.6% and 5.9% energy savings can be achieved for one SSB and four SSB respectively, at empty load with baseline of 20ms SSB/160ms SIB1 periodicity.

One source shows on-demand SSB can achieve BS energy savings by 22.0%/43.4% at low load compared to a baseline of 20ms SSB/SIB1 periodicity with or without gNB DTX configuration.

One source is provided with on-demand SIB1 at empty load with baseline of 20ms SSB/SIB1 periodicity, 5.8%~8.6% BS energy savings can be achieved at SIB1 transmission rate of 20%~5% for one SSB beam, and the gains can increase to 32.1%~38.8% for 8 beams case for a same SIB1 transmission rate range.

Performance impact of on demand SSB/SIB is not provided.

=== end ===

| Company | Comments |
| --- | --- |
| ZTE，Sanechips | More clarifications about our simulations are added in red, update (typos are fixed when filling the excel document) on our evaluation results in red for Technique#A-6. (The excel document has also been updated in V24). Whether it is CA based framework, the details need to be further discussed. We think it is too early for such a conclusion at this phase. Therefore, we suggest to remove the following sentence. For the results other than the above, the observation is to be made together with techniques evaluation for B-1, since company results assume multiple carriers are available at the gNB but not necessarily CA from UE point of view. |
| LG Electronics | Since the Ericsson’s results show the ES gain by the on-demand SIB1, it is better to rephrase the observation as below: One result (Ericsson) shows that with on-demand SIB1 a 4-symbol Discover signal (DRS), and without SIB1 transmission, 2.6% and 5.9% energy savings can be achieved for one SSB and four SSB respectively, at empty load. |

6.1.2.3	Specification impacts

6.1.z	Impacts on network interfaces

6.1.aa	Cell DTX/DRX

6.1.aa.1	Description of technique

6.1.aa.2	Analysis of performance and impacts

6.1.aa.3	Specification impacts

6.1.aa.4	Higher layer procedures

Cell DTX/DRX is applied to at least UEs in RRC_CONNECTED state. A periodic Cell DTX/DRX (i.e., active and non-active periods) can be configured by gNB via RRC signalling. Below examples on Cell DTX/DRX behaviour during non-active periods are assumed to be possible options, and the UE behaviour/impact will be studied:

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

- Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

- Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The study will focus on UE behavior when at any point in time the cell activates a single DTX/DRX configuration.

Editor's note: FFS if multiple sets of Cell DRX/DTX configuration are allowed.

The Cell DTX mode/configuration can also be indicated to the UE via dynamic L1/L2 signalling. The dynamic L1/L2 signalling at least supports UE dedicated indication. Whether UE group common signalling is also supported will be further studied.

It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs. The alignment mechanism will be studied.

Editor's note: FFS details of alignment, including UE transmission/reception behavior during DTX.

6.1.aa.5	Assistance information from UE side

Editor's note: will be updated once more agreements are made.

#### 6.1.aa.6	Impacts on network interfaces

Editor's note: will be updated once more agreements are made.

The cell DTX/DRX information is considered necessary to be exchanged and coordinated between neighbour gNBs. The gNB can use the received cell DTX/DTX information to determine its own cell DTX/DRX configuration for network energy saving purpose.

Editor’s note: The details of cell DTX/DRX is finally up to RAN1 and RAN2.

6.2	Techniques in frequency domain

6.2.1	Technique B-1 YY

6.2.1.1	Description of technique

6.2.1.2	Analysis of performance and impacts

##### === (Consensus) draftTP B-1: start ===

The following capture the results by multi-carrier energy savings enhancements.

Table 6.1.1-x: BS energy savings by multi-carrier enhancements

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | KPI | Baseline configuration/assumption |
| --- | --- | --- | --- | --- | --- | --- |
| Huawei，HiSilicon [R1-2210858] | Inter-band SSB-less on Scell | Cat 2 | 0% load(zero) | 14.4% |  | 4 SSB beams with 20ms period, 20RB 2 SSB per slot, and 4 symbols for each SSB, when the SSB is transmitted on a carrier |
|  |  |  | 10% load(low) | 9.3% |  |  |
|  |  |  | 20% load(light) | 7.4% |  |  |
|  |  |  | 30% load(medium) | 5.7% |  |  |
| ZTE, Sanechips [R1-2211903] | SSB-less SCell | 1 | zero load | 97.4% |  | SSB20ms for baseline; set 1; |
|  |  |  |  | 93.9% |  | SSB80ms for baseline; set 1; |
|  |  |  |  | 88.4% |  | SSB160ms for baseline; set 1; |
|  |  | 2 |  | 83.8% |  | SSB20ms for baseline; set 1; |
|  |  |  |  | 82.4% |  | SSB80ms for baseline; set 1; |
|  |  |  |  | 82.1% |  | SSB160ms for baseline; set 1; |
|  |  | 1 |  | 97.3% |  | SSB20ms for baseline; set 2; |
|  |  |  |  | 93.8% |  | SSB80ms for baseline; set 2; |
|  |  |  |  | 88.3% |  | SSB160ms for baseline; set 2; |
|  |  | 2 |  | 82.1% |  | SSB20ms for baseline; set 2; |
|  |  |  |  | 80.7% |  | SSB80ms for baseline; set 2; |
|  |  |  |  | 80.4% |  | SSB160ms for baseline; set 2; |
|  | SSB-less SCell with DL traffic | 1 | low | 58.4% | UPT:801.79, SSB-less UPT：812.57 UPT gain: 1.3%; SCell activation delay reduced by 6ms | SSB20ms for baseline; set 1; with DL traffic; SCell activation delay =12 ms |
|  |  |  |  | 35.2% | UPT:804.41, SSB-less UPT：812.57 UPT gain: 1.0%; Scell activation delay reduced by 6ms | SSB80ms for baseline; set 1; with DL traffic; Scell activation delay =12 ms |
|  |  |  |  | 21.2% | UPT:804.54, SSB-less UPT：812.57 UPT gain: 1.0% Scell activation delay reduced by 6ms | SSB160ms for baseline; set 1; with DL traffic; Scell activation delay =12 ms |
|  |  | 2 |  | 15.2% | UPT:801.79, SSB-less UPT：812.57 UPT gain: 1.3%; Scell activation delay reduced by 6ms | SSB20ms for baseline; set 1; with DL traffic; Scell activation delay =12 ms |
|  |  |  |  | 7.4% | UPT:804.41, SSB-less UPT：812.57 UPT gain: 1.0%; Scell activation delay reduced by 6ms | SSB80ms for baseline; set 1; with DL traffic; Scell activation delay =12 ms |
|  |  |  |  | 6.1% | UPT:804.54, SSB-less UPT：812.57 UPT gain: 1.0%; Scell activation delay reduced by 6ms | SSB160ms for baseline; set 1; with DL traffic; Scell activation delay =12 ms |
|  |  | 1 |  | 72.7% | UPT:115.80, SSB-less UPT：119.41 UPT gain: 3.1%; Scell activation delay reduced by 6ms | SSB20ms for baseline; set 2; with DL traffic; Scell activation delay =12 ms |
|  |  |  |  | 51.7% | UPT:118.20, SSB-less UPT：119.41 UPT gain: 1.0%; Scell activation delay reduced by 6ms | SSB80ms for baseline; set 2; with DL traffic; Scell activation delay =12 ms |
|  |  |  |  | 34.9% | UPT:118.70, SSB-less UPT：119.41 UPT gain: 0.6%; Scell activation delay reduced by 6ms | SSB160ms for baseline; set 2; with DL traffic; Scell activation delay =12 ms |
|  |  | 2 |  | 24.9% | UPT:115.80, SSB-less UPT：119.41 UPT gain: 3.1%; Scell activation delay reduced by 6ms | SSB20ms for baseline; set 2; with DL traffic; Scell activation delay =12 ms |
|  |  |  |  | 16.9% | UPT:118.20, SSB-less UPT：119.41 UPT gain: 1.0%; Scell activation delay reduced by 6ms | SSB80ms for baseline; set 2; with DL traffic; Scell activation delay =12 ms |
|  |  |  |  | 15.5% | UPT:118.70, SSB-less UPT：119.41 UPT gain: 0.6% Scell activation delay reduced by 6ms | SSB160ms for baseline; set 2; with DL traffic; Scell activation delay =12 ms |
|  | SSB-less Scell with UL traffic | 2 | low | 39.4% | Scell activation delay reduced by 6ms | SSB20ms for baseline; set1; with UL traffic; Scell activation delay =12 ms |
|  |  |  |  | 22.4% |  | SSB80ms for baseline; set1; with UL traffic; Scell activation delay =12 ms |
|  |  |  |  | 18.7% |  | SSB160ms for baseline; set1; with UL traffic; Scell activation delay =12 ms |
| Vivo [R1-2211018 R1-2212541] | Inter-band CA with SSB-less carriers/Scell (ES scheme: CC 1: 20ms SSB and SIB1(with 48 PRB), 20ms RACH listening; CC 2: neither transmission nor reception) | Cat 1 | 0% | 14.7% | UE power consumption: 0% | Baseline scheme: CC 1: 20ms SSB and SIB1(with 48 PRB), 20ms RACH listening; CC 2: only 20ms SSB |
|  |  | Cat 2 | 0% | 5.1% |  |  |
| Intel [R1-2212563] | inter-band SSB-less Scell | Cat1 | Low | 3.0% | UPT: 1639.3 Mbps;Avg EE (baseline): 6.56; Avg EE (ES): 6.81 | Baseline: CC# 2 (Scell): 160 msec SSB, no SIB1/PRACH, ES: CC# 2 (Scell): no SSB/SIB1/PRACH, |
|  |  | Cat1 | Light | 1.0% | UPT:1222.9 Mbps;Avg EE (baseline): 2.96; Avg EE (ES): 3.00 |  |
|  |  | Cat1 | Medium | 0.3% | UPT: 915.8Mbps;Avg EE (baseline): 1.57; Avg EE (ES): 1.57 |  |
| MTK [R1-2212259] | Scell_w/o_SIB1 | Cat 1 | Light | 2.3% | UPT: 0.00%; Access delay/latency: 0%; UE power consumption: 0% | Scell has SSB and SIB1 |
|  |  | Cat 2 |  | 1.1% |  |  |
|  | Scell_w/o_SSB_SIB1 | Cat 1 |  | 7.9% |  |  |
|  |  | Cat 2 |  | 1.3% |  |  |
| CMCC [R1-2211692] | Scell with simplified SSB: Scell with only PSS/SSS, with 20ms periodicity. Pcell with normal SSB, SIB1 and also SIB information for Scell. | Cat.2 | Zero | 5.7% | N/A | Baseline: normal SSB on Scell. Pcell with normal SSB, SIB1 and also SIB1 information for Scell. |
|  |  | Cat.1 |  | 10.5% |  |  |
| Vivo [R1-2211018 R1-2212541] | SSB/SIB-less carrier operation with assistance of anchor carrier (ES scheme: CC 1: 20ms SSB and SIB1(with 72 PRB), 20ms RACH listening; CC 2: only 20ms RACH listening) | Cat 1 | 0% | 14.8% |  | Baseline scheme: CC 1: 20ms SSB and SIB1(with 48 PRB), 20ms RACH listening; CC 2: 20ms SSB and SIB1(with 48 PRB), 20ms RACH listening |
|  |  | Cat 2 | 0% | 9.1% |  |  |
| CATT [R1-2211210] | Multi-carrier energy savings enhancements | Cat 1 | Low load | 25.7% |  | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms;CSI-RS/TRS 10ms;Rel-17 Scell activation/deactivation; |
|  |  |  | Light load | 24.1% |  |  |
|  |  |  | Medium load | 15.5% |  |  |
|  |  |  | Low load | 30.3% |  |  |
|  |  |  | Light load | 29.1% |  |  |
|  |  |  | Medium load | 20.3% |  |  |
| Qualcomm [R1-2212128] | Dynamic UE-group Pcell switching | Cat 1 | Medium (39% RU for 1 CC; 22% RU across 2 CCs) | 37.5% | UPT: -14% | Assumption: Number of Ues changes from 25 to 20 Baseline: Keep 2 CCs activated Enhancement: deactivate 1 CC and keep 1CC activated |

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT | Access delay/latency/UE power consumption/Other KPI(s), if any | Reference configuration | Baseline configuration/assumption | Traffic model/Other evaluation methodology/assumption details/notable settings |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC [R1-2211692] | SSB/SIB1-less scheme: gNB has 2 co-deployed CCs, both of them are available for UE with single carrier operation to access, but only CC1 has normal SSB and SIB1 with default 20ms transmission period. CC2 only has PSS/SSS for synchronization. | cat.2 | Zero | 31.4% | / | CC1 carries SIB1 of CC2, the power consumption of CC1 increases 1.73% for FDM SIB of both CC. | set 1 | Baseline scheme: gNB has 2 co-deployed CCs, both of them are available for UE with single carrier operation to access, so both CC1 and CC2 has SSB and SIB1 with default 20ms transmission period. As shown in Figure.5 (a). | numerical analysis. •SIB1: -Baseline: for both CC1 and CC2, PDCCH: 2 symbols, 48RB; PDSCH: 12RBs, 12 OFDM symbols including DMRS. -SSB/SIB1-less scheme: no SIB1 on CC2, but CC1 carries SIB1 for CC2, so the TBS will be doubled. The number of PDSCH PRBs is 24 RBs, 12OFDM symbols. PDCCH still occupies 2 OFDM symbols, 48 PRBs. •SSB1 and SSB are transmitted in different slots, e.g. value in Table 13-11 is assumed to be 5ms. •ղ=1, A=0.4. •Time unit for power model is slot. power consumption is calculated in a 40ms long period |
|  |  | cat.1 |  | 56.5% | / | CC1 carries SIB1 of CC2, the power consumption of CC1 increases 1.41% for FDM SIB of both CC. |  |  |  |
| Huawei, HiSilicon [R1-2210858] | SIB-less on ES CC | Cat 2 | 0% load(zero) | 33.6% | N/A | N/A | Set 2 | 4 SIB1 with 20ms period,20RB | FTP3 IM. NO C-DRX; Subband based CSI-feedback in every 5 slots. slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |
|  |  |  | 10% load(low) | 26.2% | N/A | N/A |  |  |  |
|  |  |  | 20% load(light) | 19.0% | N/A | N/A |  |  |  |
|  |  |  | 30% load(medium) | 16.0% | N/A | N/A |  |  |  |
|  | dual SIB on Anchor CC |  | 0% load(zero) | -7.5% | N/A | N/A |  |  |  |
|  |  |  | 10% load(low) | -6.7% | N/A | N/A |  |  |  |
|  |  |  | 20% load(light) | -6.1% | N/A | N/A |  |  |  |
|  |  |  | 30% load(medium) | -5.5% | N/A | N/A |  |  |  |
|  | SIB-less on ES CC |  | 0% load(zero) | 20.2% | N/A | N/A |  | 4 SIB1 with 40ms period,20RB |  |
|  |  |  |  | 11.2% | N/A | N/A |  | 4 SIB1 with 80ms period,20RB |  |
|  |  |  |  | 5.9% | N/A | N/A |  | 4 SIB1 with 160ms period,20RB |  |
|  |  |  | 10% load(low) | 15.7% | N/A | N/A |  | 4 SIB1 with 40ms period,20RB |  |
|  |  |  |  | 9.3% | N/A | N/A |  | 4 SIB1 with 80ms period,20RB |  |
|  |  |  |  | 4.0% | N/A | N/A |  | 4 SIB1 with 160ms period,20RB |  |
| ZTE, Sanechips [R1-2211903] | (SSB and SIB)-less cell | 1 | zero | 97.9% 95.4% 91.1% |  |  | Set 1 | Baseline: SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: (SSB+SIB1)-less for non-anchor cell | FTP3 for Set 1. IM for Set 2. slot-level; Pstatic=P3, η(s_f,s_p )=1; time-domain scaling for SSB; time and frequency domain scaling for SIB. For the multiplexing pattern of two SIBs in the anchor cell (when applicable), TDM is considered in the evaluations. |
|  |  |  | low | 64.3% 43.6% 28.0% |  |  |  |  |  |
|  | SIB-less cell |  | zero | 19.3% 24.6% 23.5% |  |  |  | Baseline: SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: SIB1-less for non-anchor cell |  |
|  |  |  | low | 15.5% 13.6% 8.8% |  |  |  |  |  |
|  | anchor cell with dual SIB transmission |  | zero | -14.1% -18.9% -18.1% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | Baseline: SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  |  | low | -11.6% -10.5% -6.8% |  |  |  |  |  |
|  | (SSB and SIB)-less cell |  | zero | 98.4% 96.2% 92.6% |  |  | Set 2 | Baseline: SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  | low | 80.3% 59.4% 42.4% |  |  |  |  |  |
|  | SIB-less cell |  | zero | 40.7% 38.4% 37.0% |  |  |  | Baseline: SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: SIB1-less for non-anchor cell |  |
|  |  |  | low | 28.0% 16.0% 11.5% |  |  |  |  |  |
|  | anchor cell with dual SIB transmission |  | zero | -17.6% -12.4% -12.0% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | Baseline: SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  |  | low | -17.8% -10.8% -7.7% |  |  |  |  |  |
|  | (SSB and SIB)-less cell | 2 | zero | 85.8% 83.6% 82.8% |  |  | Set 1 | Baseline: SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  | low | 24.5% 13.4% 9.4% |  |  |  |  |  |
|  | SIB-less cell |  | zero | 12.1% 7.0% 3.7% |  |  |  | Baseline: SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: SIB1-less for non-anchor cell |  |
|  |  |  | low | 10.8% 6.2% 3.3% |  |  |  |  |  |
|  | anchor cell with dual SIB transmission |  | zero | -8.0% -4.6% -2.4% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | Baseline: SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  |  | low | -7.5% -4.3% -2.3% |  |  |  |  |  |
|  | (SSB and SIB)-less cell |  | zero | 87.5% 82.6% 81.4% |  |  | Set 2 | Baseline: SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  | low | 42.9% 23.6% 19.1% |  |  |  |  |  |
|  | SIB-less cell |  | zero | 28.2% 9.8% 5.3% |  |  |  | Baseline: SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: SIB1-less for non-anchor cell |  |
|  |  |  | low | 23.9% 8.0% 4.3% |  |  |  |  |  |
|  | anchor cell with dual SIB transmission |  | zero | -14.1% -4.9% -2.6% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | Baseline: SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Enhanced: (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  |  | low | -13.8% -4.6% -2.5% |  |  |  |  |  |
| NOKIA/NSB [R1-2211097] | SSB-less at 20 ms period of RO | CAT2 | Unloaded/low/light/Medium | 27.5% 26.1% 26.0% 22.7% | / 135 Mbps 105 Mbps 74 Mbps |  | SET 1 | Intra-band/collocated cells with non-CA case, consisting of: * Coverage cell with 20 ms periodicity of SSB/SIB1 Tx and RO monitoring * Capacity cell with 20 ms periodicity of SSB/SIB1 Tx and RO monitoring UEs initially in RRC Idle state. | DL-FTP3. SLS+Post-processing |
|  | SSB-less at 160 ms period of RO |  | Unloaded/low/light/Medium | 51.8% 48.2% 47.3% 43.2% | / 85 Mbps 72 Mbps 56 Mbps |  |  |  |  |
|  | SIB1-less at 20 ms period of RO |  | Unloaded/low/light/Medium | 43.1% 40.5% 40.1% 36.5% | / 132 Mbps 104 Mbps 73 Mbps |  |  |  |  |
|  | SIB1-less at 160 ms period of RO |  | Unloaded/low/light/Medium | 53.8% 51.2% 47.3% 43.2% | / 84 Mbps 72 Mbps 56 Mbps |  |  |  |  |
|  | SSB&SIB1-less at 20 ms period of RO |  | Unloaded/low/light/Medium | 52.7% 50.6% 52.7% 44.4% | / 135 Mbps 105 Mbps 74 Mbps |  |  |  |  |
|  | SSB&SIB1-less at 160 ms period of RO |  | Unloaded/low/light/Medium | 55.1% 53.1% 54.9% 46.8% | / 85 Mbps 72 Mbps 56 Mbps |  |  |  |  |
| CATT [R1-2211210] | Adaptation of SSB/SIB1 | Cat 1 | Low load | 22.0% |  |  | set1 | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; | FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes. SLS;Cell OFF:Without normal SSB/SIB/CSI-RS transmission within Cell off duration;On demand SSB transmission is trigger by neighbour cell with 300ms transmission duration and 20ms SSB. For the case with DRX, DTX configuration: gNB starting offset of DTX on locate before UE DRX on duration in order to support UE wakeup; A=0.4; η(s_f, s_p)=1. |
|  |  |  |  | 43.4% |  |  |  | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms;DTX configuration: gNB starting offset of DTX on locate before UE DRX on duration in order to support UE wakeup; |  |
| Fujitsu [R1-2211085] | SSB&SIB-less | Cat2 | Zero low light medium | 34.5% 27.0% 21.7% 16.7% |  |  | Set 1 | Baseline scheme: 20 ms SSB/SIB1 period | BS goes into mico-sleep on symbolc w/o TX/RX simplified SSB which contains SSS and PSS is transmitted with periodicity of 160 ms No UE DRX. A=0.4, η=1 |
|  |  |  | Zero low light medium | 36.0% 24.7% 18.4% 13.5% |  |  | Set 2 |  |  |
| Ericsson [R1-2212154] | 20ms Discovery signal (4 symbols) + no SIB1 | Cat1 | Zero | 2.6% / 5.9% |  |  | Set 1 | Baseline scheme: 20ms SSB + 160ms SIB1 | one SSB/ four SSBs Energy calculation: per symbol energy consumption is modeled. According to Rel-15 specification, SIB1 can be transmitted with variable transmission repetition periodicity within a 160 ms period, including one SIB1 PDSCH transmission every 160ms or even sparser. |
| Qualcomm [R1-2212128] | on-demand SIB1 | Cat 1 | Empty load | 5.8% / 7.7% / 8.6% |  |  | Set 1 | Baseline: 20ms periodicity for SSB/SIB1/RO, one beam Enhanced: 20%/10%/5% SIB1 Tx rate, C-WUS with 20ms periodicity |  |
|  |  |  |  | 32.1% / 36.6% / 38.8% |  |  |  | Baseline: 20ms periodicity for SSB/SIB1/RO, 8 beams Enhanced: 20%/10%/5% SIB1 Tx rate, C-WUS with 20ms periodicity |  |

Observation includes the results for techniques that are also evaluated under #A-6. The following is observed.

In general, for SSB and/or SIB saved from one carrier of two carriers, 8 resources observed BS energy savings gain, by 5.1%~97.4% for empty load, 3.0%~58.4% for low load, and 1.0%~7.9% for light load, 0.3%~5.7% for medium load. When traffic load is low, network may turn off SCell for energy saving. The results are for FR1 only.

With one of two carriers having simplified SSB and no SIB1, one result shows BS energy saving gain can be achieved by 31.4%~56.5% compared with a baseline of both carriers having SSB and SIB1 periodicity of 20ms; the same source company result also show that with CA configured where SIB1 is already carried by PCell, compared with normal SSB on SCell, the gain of simplified SSB on SCell can be 5.7%~10.5%.

With SIB-less only from one of two carriers and SSB is still transmitted,

- one result shows that 33.6%~16.0% BS energy saving gain can be achieved compared with a carrier has 20ms SIB1 periodicity and both SSB and SIB1 are transmitted, and the gain decreases as the traffic load increases,. Meanwhile, the SIB1 carried on another carrier increase the energy of that carrier by 7.5%~5.5%, resulting a total saving across two carries by 26.1%~10.5%. The gain decreases to 4.0% when the baseline SIB1 periodicity increases to 160ms;

- one result shows BS energy saving gain can be 3.3%~40.7% compared with baseline of SSB+SIB periodicity of {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; meanwhile, the SIB1 carried on another carrier increase the energy of that carrier by 2.0%~17.8%, resulting a total saving across two carries by 1.3%~22.9%;

- one result shows at different loads, compared to baseline of 20 ms SSB/SIB1 periodicity, that BS energy savings can be achieved by 53.8%~36.5% with RO periodicity of 20ms and 160ms;

- also one result show less than 2.3% BS energy savings when compared with a baseline of SCell having SIB1.

With SSB-less only from one of two carriers, for CA in which case the SIB is already saved from SCell, with assumption that UE is able to acquire sync from a carrier from another band,

- at different loads, compared to a baseline of 20 ms SSB/SIB1 periodicity, one result shows BS energy savings by 27.5%~22.7% and 51.8%~43.2% when the RO periodicity is 20ms and 160ms respectively;

- two results show that BS energy savings can be 5.1%~14.7% at different loads, compared to a baseline of 20ms SSB periodicity;

- one result shows with the same load, BS energy saving gain can be 6.1%~15.2% for DL traffic, and 18.7%~39.4% for UL traffic compared with baseline of SSB periodicity of {20ms, 80ms, 160ms}. The BS energy saving gain from SSB-less cell with UL traffic is 12.6%~24.2% larger than SSB-less cell with DL traffic;

- one result also shows that when the baseline Scell SSB periodicity is 160ms, only 0.3%~3% BS energy savings can be achieved, and one another shows BS energy savings by less than 7.9% if compared with Scell having SIB1.

- UE measurement is based on SSB(s) transmitted in the other carrier of the two carriers

- Number of SSB

With both SSB-less and SIB1-less from one of two carriers, with assumption that UE is able to acquire sync from a carrier from another band,

- compared to baseline of 20 ms SSB/SIB1 periodicity on both carriers, one result shows BS energy savings by 55.1%~44.4% with RO periodicity of 20ms~160ms at different loads, and one result shows 9.1%~14.8% energy savings at empty load if an anchor carrier carries additional SIB1 for another carrier;

- at different loads, compared to baseline of 20 ms SSB/SIB1 periodicity, one result shows BS energy savings by 36.0%~13.5% when combined with simplified SSB (i.e. PSS and SSS only);

- one result shows that with baseline of SSB+SIB periodicity of {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell, BS energy savings can be 9.4%~97.9% if an anchor carrier carries the SSB and SIB1 for another carrier depending on the traffic load. Meanwhile, the SIB1 carried on another carrier increase the energy of that carrier by 2.0%~17.8%, resulting a total saving across two carries by 7.4%~80.1%.

- UE measurement and condition

- Number of SSB

For source results where SSB is not transmitted in SCell, performance impact(s) due to lack of AGC and cell measurement results before SCell access and activation is not provided.

For source results where SSB is not transmitted in neighbour cell, mobility performance impact(s) due to SSB-less operation in neighbour cell(s) is not provided.

In most results for SSB and/or SIB saved from one carrier of two carriers, the UPT is not negatively impacted while one result shows slightly increased UPT. One source shows that the SCell activation delay can also be reduced to 6ms from the baseline.

No negative impact observed on UE power consumption for the above schemes.

Additionally, SSB-less SCell for CA can slightly improve the average EE, reported by one result.

One company showed that UE-group Pcell switching together with Scell dormancy could provide network energy saving by up to 37.5% for two-CC CA scenario with FR1 Set 1. However, UPT degrades by 14% if one Scell goes to dormant state.

=== end ===

| Company | Comments |
| --- | --- |
| Intel | General comment: Some other observations have captured company references. Is there any plan to apply this consistently for all the TPs? |
| QCOM | We suggest making the following observation for dynamic UE-group Pcell switching: One company showed that UE-group Pcell switching together with Scell dormancy could provide network energy saving by up to 37.5% for two-CC CA scenario with FR1 Set 1. However, UPT degrades by 14% if one Scell goes to dormant state. |
| ZTE，Sanechips | More clarifications about our evaluation results are added in red for Technique#B-1. Regarding the observation, the following is suggested. With SIB-less only from one of two carriers, - one result shows BS energy saving gain can be 3.3%~40.7% compared with baseline of SSB+SIB periodicity of {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell; Meanwhile, the SIB1 carried on another carrier increase the energy of that carrier by 2.0%~17.8%, resulting a total saving across two carries by 1.3%~22.9%. - [ZTE comment] the above observation in red is added based on our simulation results. With SSB-less only from one of two carriers, for CA in which case the SIB is already saved from Scell, - One result shows with the same RU load, BS energy saving gain can be 6.1%~15.2% for DL traffic, BS energy saving gain can be 18.7%~39.4% for UL traffic compared with baseline of SSB periodicity of {20ms, 80ms, 160ms}. The BS energy saving gain from SSB-less cell with UL traffic is 12.6%~24.2% larger than SSB-less cell with DL traffic. - [ZTE comment] the above observation in red is added based on our simulation results. With both SSB-less and SIB1-less from one of two carriers, for non-CA - one result shows that with baseline of SSB+SIB periodicity of {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell, BS energy savings can be 9.4%~97.9% if an anchor carrier carries the SSB and SIB1 for another carrier without additional/separate SIB1 depending on the traffic load. Meanwhile, the SIB1 carried on another carrier increase the energy of that carrier by 2.0%~17.8%, resulting a total saving across two carries by 7.4%~80.1%. and the savings can be 2.0%~18.9% if the SIB1 for another carrier is TDMed transmitted on the anchor carrier. - [ZTE comment] the above observation in red is updated to be symmetric with the observations with SIB-less. Moreover, it is too early to conclude whether the framework is based on CA or not. |
| Ericsson1 | Suggest adding the following in the observation. It is assumed in the evaluations that UE is able to acquire sync from a carrier from another band for the evaluations. |
| Vivo | Suggest the following revision: With both SSB-less and SIB1-less from one of two carriers, for non-CA compared to baseline of 20 ms SSB/SIB1 periodicity on both carriers, one result shows BS energy savings by 55.1%~44.4% with RO periodicity of 20ms~160ms at different loads, and one result shows 9.1%~14.8% energy savings at empty load if an anchor carrier carries additional SIB1 for another carrier; |
| CMCC | The following description can be modified, With one of two carriers having simplified SSB and no SIB1, one result shows BS energy saving gain can be achieved by 31.4%~56.5% compared with a baseline of both carriers having SSB and SIB1 periodicity of 20ms; the same source company result also show that with CA configured by which SIB1 is saved from Scell,where SIB1 is already carried by Pcell, compared with normal SSB on Scell, the gain of simplified SSB on Scell can be achieved by 5.7%~10.5%. |
| MTK2 | For the results table, we suggest to include the following revision for MTK results to avoid the confusion (UPT/latency/UE power consumption actually represent the increment w.r.t. the baseline): Increments in UPT: 0.00%; Access delay/latency: 0%; UE power consumption: 0% For the observation, the following sentence is too generic, and we suggest to capture the dependency between NW ES gain and traffic load conditions: In general, for SSB and/or SIB saved from one carrier of two carriers, NW ES gain is 5.1% - 97.4% for empty load, 3.0% - 58.4% for low load, 1.0% - 7.9% for light load, 0.3% for medium load. When traffic load is low, NW may turn off Scell for energy saving. For the 1st sentence in the following paragraph, we cannot find the results in the table. For 2nd sentence, we think it is included in the above revised paragraph. In this regard, we suggest to remove the whole paragraph: |
| ZTE,Sanechips | More clarifications about our evaluations are added. |

6.2.1.3	Specification impacts

6.2.2	Technique B-2 YYY

6.2.2.1	Description of technique

6.2.2.2	Analysis of performance and impacts

##### === (Consensus) draftTP B-2: start ===

The following capture the results for semi-statically configured bandwidth part of UEs within a carrier. The evaluation is performed with different traffic, e.g. medium traffic to light traffic for Set 1, and low traffic to very low traffic for Set 3, and the reduced BW of 80 MHz is applied as NES mode compared with baseline BW of 100 MHz.

Table 6.1.1-x: BS energy savings by BWP adaptation within carrier

| Company | BS Category | Load scenario | ES gain (%) | KPI | Reference configuration | Baseline configuration/assumption |
| --- | --- | --- | --- | --- | --- | --- |
| Samsung [R1-2212543] | Cat 1 | Baseline traffic: 42.8 % RU Reduced traffic: 28.47 % RU | 38.2% | UPT: 6.05%; Packet latency: 6.44%; Scheduling latency: No increase | Set1 | Baseline: full 100MHz with 55 dBm NES mode: 80 MHz with 54 dBm |
|  | Cat 2 |  | 27.8% |  |  |  |
|  | Cat 1 | Baseline traffic: 7.5 % RU Reduced traffic: 2.75 % RU | 52.2% | UPT: 14.67%; Packet latency: 17.2%; Scheduling latency: No increase |  |  |
|  | Cat 2 |  | 17.6% |  |  |  |
|  | Cat 1 | Baseline traffic: 32.1 % Reduced traffic: 25.7 % | 17.4% | UPT: 28.24%; Packet latency: 39.4%; Scheduling latency: No increase | Set3 | Baseline: full 100MHz with 49 dBm NES mode: 80 MHz with 48 dBm |
|  | Cat 2 |  | 17.8% |  |  |  |

One source observed BS energy savings by 17.4%~52.2% at the expense of UPT loss by 28.4%~14.47%, and packet latency increases by 6.44%~39.4% when traffic is reduced compared to corresponding baseline. BWP switching delay is not modelled.

On scheduling latency, no negative impact is not observed due to very low traffic applied to the large bandwidth of 80MHz.

=== end ===

| Company | Comments |
| --- | --- |
| Nokia/Nsb | There is some in-consistency compared with Tdoc description. And clearly there should have latency impact and UPT loss with bandwidth reduction. From Samsung’s Tdoc R1-2212056, following is what we can see the “loss/negative impact” Observation 5: Adapting bandwidth achieves up to 52.2 % energy saving gain at FR1 and 17.79 % energy saving gain at FR2 in results. Observation 6: Adapting bandwidth shows up to 19.8 % UPT loss and packet latency increase 24.7 % at FR1 and 28.24 % UPT loss and packet latency increase 39.4 % at FR2. |
| Samsung | @ Nokia, Thanks for your comments. and packet latency increase by 6.44%~39.4% From our point of view, BW adaptation should be applied under low traffic scenario. So, we evaluated the dynamic BW adaptation in the scenario that traffic was changed. So, according to change the traffic loads, operating BW also would be changed, and gNB consumes less power with acceptable UPT loss and latency increases. We think it’s difficult to show the effectiveness of energy saving gain without assumption on changing traffic loads. Regarding the observation, it should be revised as below: One result observes BS energy savings by 17.4%~52.2% at the expense of UPT loss by 28.4%~14.47% when traffic is reduced compared to corresponding baseline. On packet latency/scheduling latency, no negative impact is observed. |
| QCOM1 | @Samsung: Are the results based on dynamically updating BWP? It seems that the results are based on static configurations of BWP. If this is the case, we suggest updating as follows: The following capture the results for dynamic static adaptation of bandwidth part of UE(s) within a carrier. |
| vivo | Actually, we are not quite clear of the enhanced scheme for each row. Could @Samsung clarify a bit on how the evaluation is done? |
| Samsung | @QC, We assumed that bandwidth was adapted dynamically without BWP transition time when traffic load was changed. We don’t think whether it is dynamic configuration or static configuration. @vivo, We assume the scenario including traffic changes, like medium traffic to light traffic and low traffic to very low traffic. With the scenario, we apply reduced BW as NES mode under reduced traffic scenario, and baseline was applied over baseline traffic. Energy consumption normalized slot was calculated for each baseline and NES mode. As mentioned in previous comment, we’d like to show that BW adaptation can be considered when traffic load was reduced. Under same traffic scenario, we observed that there is no ESG by adapting BW. |

6.2.2.3	Specification impacts

6.2.3	Technique B-3 YYY

6.2.2.1	Description of technique

6.2.2.2	Analysis of performance and impacts

##### === (Consensus) draftTP B-3: start ===

The following captures the results for dynamic /(semi)-static adaptation of bandwidth of active BWP.

Table 6.1.1-x: BS energy savings by BW adaptation within BWP

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | KPI | Baseline configuration/assumption |
| --- | --- | --- | --- | --- | --- | --- |
| OPPO [R1-2211458] | adaptation of bandwidth of active BWP of UEs | Cat 1 | low load(RU-10%) | 1.4% | UPT: 554.74Mbps(-46.8%); Access delay/latency: 9.35ms(+86.3%) | system BW of 100MHz, 64T: (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8) |
|  |  |  | low load(RU-0.2%) | 1.3% | UPT: 513.43Mbps(-52%); Access delay/latency: 1.78ms(+48.3%) |  |
| Intel [R1-2212563] | intra-carrier BWP adaptation |  | Low | -20.6% | UPT: Baseline (819.7Mbps), ES (346.8 Mbps); Avg EE (baseline): 5.10; Avg EE (ES): 1.87 | Baseline: Full BW ES: 50% BW |
|  |  |  |  | -75.4% | UPT: Baseline (819.7Mbps), ES (99.4 Mbps); Avg EE (baseline): 5.10; Avg EE (ES): 0.54 | Baseline: Full BW ES: 25% BW |
|  |  |  | Light | -45.9% | UPT: Baseline (611.5Mbps), ES (155.2Mbps); Avg EE (baseline): 2.66; Avg EE (ES): 0.69 | Baseline: Full BW ES: 50% BW |
|  |  |  |  | -61.8% | UPT: Baseline (611.5Mbps), ES (25.7Mbps); Avg EE (baseline): 2.66 Avg EE (ES): 0.26 | Baseline: Full BW ES: 25% BW |
|  |  |  | Medium | -27.6% | UPT: Baseline (457.9Mbps), ES (50.5Mbps); Avg EE (baseline): 1.50 Avg EE (ES): 0.44 | Baseline: Full BW ES: 50% BW |
|  |  |  |  | -13.5% | UPT: Baseline (457.9Mbps), ES (12.3Mbps); Avg EE (baseline): 1.50; Avg EE (ES): 0.44 | Baseline: Full BW ES: 25% BW |
| CEWiT {R1-xxxx} | Dynamic adaptation of bandwidth of active BWP of UEs with dynamic indication | Cat 1 | Medium | 1.75% |  | Baseline: Full BW of 100MHz, 32 ports ES: 50% BW |

3 sources show different observations. One source  show small BW energy saving gain by 1.3%/1.4% at the expense of about 50% UPT loss and increased access delay/latency by 48.3%/86.3%. One source shows a BW energy saving gain of 1.7%. One source shows BS power consumption increases with BWP size reduction in a carrier and negative energy saving gain in the range -13.5%~ -75.4% is observed, together with significantly reduced UPT, and additionally reduced average EE.

=== end ===

| Company | Comments |
| --- | --- |
| Nokia/Nsb | There is some in-consistency compared with Tdoc description. Based on the observation from Intel (R1-2212563), there is no NW ES gains: Observation 5: - For Category 1 BS power model, - No power saving gains were observed from reducing the operating bandwidth in any load scenarios. - Severe throughput loss was observed along with increase in overall power consumption when operating bandwidth is reduced. This is due to reduction of maximum throughput of the UEs and this resulting in longer time to complete the transmissions to the UEs. The longer activity in time domain resulted in higher power consumption compared to the power reduction from using less bandwidth (and transmit power). |
| Intel | Suggest to revise TP as follows: Two results show different observations. One result (OPPO) show small BW energy saving gain by 1.3%/1.4% at the expense of about 50% UPT loss and increased access delay/latency by 48.3%/86.3%. One result (Intel) shows that BS power consumption increases with BWP size reduction in a carrier and negative energy saving gain in the range -13.5%~ -75.4% is observed, also together with significantly reduced UPT, and additionally reduced average EE. |
| QCOM1 | @OPPO and Intel: Are the results based on dynamically updating BW? It seems that the results are based on static BW configurations. If this is the case, we should update the text as follows: The following captures the results for dynamic static adaptation of bandwidth of active BWP. |
| Intel | OPPO’s suggestion seems ok. Yes, the evaluated case was with (semi)-static adaptation. |
| OPPO | Thanks for QC and Intel’s comments. Yes, our evaluation result is based on static adaptation of bandwidth of active BWP. |
| CEWiT | We think that dynamically adapting the BW and indicating the same to the UEs dynamically will minimize throuput loss while providing limited energy saving. On the contrary, static adaptation of BW will increase the throuput loss while improving the ES gain. |
| CEWiT | We suggest to update the obervation for dynamic adaptation of BW as follows: Two results show different observations. One result (OPPO) show small BW energy saving gain by 1.3%/1.4% at the expense of about 50% UPT loss and increased access delay/latency by 48.3%/86.3%. One result (CEWiT) show a BW energy saving gain of 1.7%. One result (Intel) shows BS energy is increased by 13.5%~75.4%, also together with significantly reduced UPT, and additionally reduced average EE. |

6.2.1.3	Specification impacts

6.2.z	Impacts on network interfaces

6.3	Techniques in spatial domain

6.3.1	Technique C-1 ZZ

6.3.1.1	Description of technique

6.3.1.2	Analysis of performance and impacts

##### === (Consensus) draftTP C-1: start ===

The following capture the results for dynamic adaptation of spatial elements.

Table 6.1.1-x: BS energy savings by adaptation of spatial elements

| Company | ES scheme | BS Category &Reference configuration | Load scenario | ES gain (%) | UPT/latency/UE power/ Other KPIs | Baseline configuration/assumption | Evaluation methodology/assumption details/traffic model |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MTK [R1-2212259] | #TxRU_32 | Cat 1, Set 1 | Light | 15.8% | UPT loss:4.54%; latency increase:4.76%; UE power increase:3.48% | BS #TxRU 64 | SLS; DRX (160, 8, 100); FTP3 traffic |
|  |  | Cat 2, Set 1 |  | 15.8% | UPT loss:4.54%; latency increase:4.76%; UE power increase:3.48% |  |  |
|  | #TxRU_16 | Cat 1, Set 1 |  | 19.2% | UPT loss:16.92%; latency increase:20.36%; UE power increase:14.70% |  |  |
|  |  | Cat 2, Set 1 |  | 22.1% | UPT loss:16.92%; latency increase:20.36%; UE power increase:14.70% |  |  |
|  | #TxRU_8 | Cat 1, Set 1 |  | 22.1% | UPT loss:47.48%; latency increase:90.42%; UE power increase:47.94% |  |  |
|  |  | Cat 2, Set 1 |  | 24.9% | UPT loss:47.48%; latency increase:90.42%; UE power increase:47.94% |  |  |
|  | #TxRU_32 | Cat 1, Set 1 | Medium | 25.3% | UPT loss:6.39%; latency increase:6.83%; UE power increase:4.20% |  |  |
|  |  | Cat 2, Set 1 |  | 26.6% | UPT loss:6.39%; latency increase:6.83%; UE power increase:4.20% |  |  |
|  | #TxRU_16 | Cat 1, Set 1 |  | 31.4% | UPT loss:44.78%; latency increase:81.08%; UE power increase:32.85% |  |  |
|  |  | Cat 2, Set 1 |  | 36.7% | UPT loss:44.78%; latency increase:81.08%; UE power increase:32.85% |  |  |
|  | #TxRU_8 | Cat 1, Set 1 |  | 36.0% | UPT loss:87.08%; latency increase:647.07%; UE power increase:79.99% |  |  |
|  |  | Cat 2, Set 1 |  | 45.2% | UPT loss:87.08%; latency increase:647.07%; UE power increase:79.99% |  |  |
|  | #TxRU_32_PDSCH_PowOffset_-3dB | Cat 1, Set 1 | Light | 18.8% | UPT loss:9.06%; latency increase:9.96%; UE power increase:7.62% | BS #TxRU 64; PDSCH power offset 0 dB | SLS; DRX (160, 8, 100); FTP3 traffic model; Single value η (=1) |
|  |  | Cat 2, Set 1 |  | 19.7% | UPT loss:9.06%; latency increase:9.96%; UE power increase:7.62% |  |  |
| OPPO [R1-2211458] | Dynamic adaptation of spatial elements. | Cat 1,Set 1 | low load(RU-10%) | 22.1% | UPT: 550Mbps(-47.2%); latency: 12.41ms(+147%) | system BW of 100MHz, 64T: (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8) | SLS, 8T: (M, N, P, Mg, Ng, MP, NP,) = (4, 2, 2, 1, 1, 2, 2) is used for evaluation; FTP3 traffic model; A = 0.4 and η=1 |
|  |  |  | low load(RU-0.2%) | 13.7% | UPT: 782.56Mbps(-21.2%); latency:1.79ms(+49.1%) |  | SLS, 8T: (M, N, P, Mg, Ng, MP, NP,) = (4, 2, 2, 1, 1, 2, 2) is used for ES evaluation,; FTP3 IM traffic model; A = 0.4 and η=1 |
| Huawei,HiSilicon [R1-2210858] | Dynamic TRX adaption with Multiple CSIs | Cat 2,Set 1 | 10% load(low) | 7.7% | 0% UPT loss | Dynamic TRX adaption with Single 64T CSI; | NO C-DRX; Subband based CSI-feedback in every 5 slots; FTP3 IM traffic model; A=0.4; η=1, 0.76 |
|  |  |  |  | 4.0% | 5% UPT loss |  |  |
|  |  |  |  | 3.4% | 10% UPT loss |  |  |
|  |  |  | 30% load(medium) | 13.0% | 0% UPT loss |  |  |
|  |  |  |  | 11.3% | 5% UPT loss |  |  |
|  |  |  |  | 9.6% | 10% UPT loss |  |  |
|  |  |  | 10% load(low) | 7.5% | 0% UPT loss |  | C-DRX with (cycle, on-duration, inactivity timer) = (320, 10, 80) ms; Subband based CSI-feedback in every 5 slots; FTP3 IM traffic model; A=0.4; η=1, 0.76 |
|  |  |  | 30% load(medium) | 10.9% | 0% UPT loss |  |  |
|  |  | Cat 2,Set 2 | 10% load(low) | 7.5% | 0% UPT loss |  | NO C-DRX; Subband based CSI-feedback in every 5 slots; FTP3 IM traffic model; A=0.4; η=1, 0.76) |
|  |  |  |  | 13.2% | 5% UPT loss |  |  |
|  |  |  |  | 10.2% | 10% UPT loss |  |  |
|  |  |  | 30% load(medium) | 10.3% | 0% UPT loss |  |  |
|  |  |  |  | 19.2% | 5% UPT loss |  |  |
|  |  |  |  | 14.8% | 10% UPT loss |  |  |
| ZTE,Sanechips [R1-2211903] | TxRU reduction 48TxRU | Cat 2, Set 1 | Low load(RU=8.8%) | 7.8% | 1.5% UPT loss | Baseline: 64TxRU | FTP3: 20K packet size;η=1 |
|  | TxRU reduction 32TxRU |  | Low load(RU=8.8%) | 15.5% | 4.47% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | Low load(RU=8.8%) | 23.5% | 11.06% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | light load(RU=20%) | 10.8% | 1.5% UPT loss |  |  |
|  | TxRU reduction 32TxRU |  | light load(RU=20%) | 21.7% | 7.06% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | light load(RU=20%) | 33.7% | 15.31% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | medium load(RU=32%) | 12.5% | 3.34% UPT loss |  |  |
|  | TxRU reduction 32TxRU |  | medium load(RU=32%) | 24.6% | 10.44% UPT loss |  |  |
|  | Dynamic TxRUs adaptation via multi-CSI |  | Low load(RU=8.8%) | 27.1% | 0.9% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 28.7% | 1.5% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 31.3% | 7% UPT loss |  |  |
|  |  |  | medium load(RU=32%) | 23.8% | 1.17% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | Low load(RU=10%) | 5.6% | 6.89% UPT loss |  | FTP3: 0.1M packet size,η=1 |
|  | TxRU reduction 32TxRU |  | Low load(RU=10%) | 11.0% | 18.39% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | light load(RU=20%) | 9.1% | 6.32% UPT loss |  |  |
|  | TxRU reduction 32TxRU |  | light load(RU=20%) | 18.6% | 14.88% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | Medium load(RU=40%) | 11.8% | 8.01% UPT loss |  |  |
|  | TxRU reduction 32TxRU |  | Medium load(RU=40%) | 25.0% | 20.88% UPT loss |  |  |
|  | Dynamic TxRUs adaptation via multi-CSI |  | Low load(RU=10%) | 7.6% | 3.1% UPT loss |  |  |
|  |  |  | Low load(RU=10%) | 11.1% | 5.04% UPT loss |  |  |
|  |  |  | Low load(RU=10%) | 12.7% | 6.03% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 13.8% | 2.52% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 16.3% | 4.13% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 18.7% | 5.15% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 21.1% | 6.96% UPT loss |  |  |
|  |  |  | Medium load(RU=40%) | 15.7% | 2.89% UPT loss |  |  |
|  |  |  | Medium load(RU=40%) | 17.1% | 4.16% UPT loss |  |  |
|  | TxRU reduction 24TxRU | Cat 2, Set 2 | Low load(RU=5%) | 4.8% | 2.03% UPT loss | Baseline: 32TxRU | FTP3: 20K packet size,η=1 |
|  | TxRU reduction 16TxRU |  | Low load(RU=5%) | 9.6% | 5.61% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | Low load(RU=5%) | 14.8% | 12.5% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | Low load(RU=11%) | 8.0% | 3.07% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | Low load(RU=11%) | 15.9% | 9.75% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | Low load(RU=11%) | 25.3% | 19.36% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | light load(RU=20%) | 9.6% | 5.19% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | light load(RU=20%) | 19.7% | 12.87% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | light load(RU=20%) | 32.1% | 23.931% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | Low load(RU=5%) | 7.9% | 0.42% UPT loss |  | FTP3: 4K packet size, η=1 |
|  | TxRU reduction 16TxRU |  | Low load(RU=5%) | 15.8% | 1.72% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | Low load(RU=5%) | 24.3% | 3.54% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | Low load(RU=13%) | 11.2% | 0.67% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | Low load(RU=13%) | 22.7% | 1.5% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | Low load(RU=13%) | 35.0% | 3.84% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | light load(RU=28%) | 14.0% | 1.86% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | light load(RU=28%) | 27.8% | 6.16% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | light load(RU=28%) | 43.4% | 14.15% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | Medium load(RU=48%) | 14.3% | 5.07% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | Medium load(RU=48%) | 29.4% | 14.63% UPT loss |  |  |
|  | Dynamic TxRUs adaptation via multi-CSI |  | Low load(RU=5%) | 18.1% | 0.62% UPT loss |  |  |
|  |  |  | Low load(RU=13%) | 23.7% | 0.16% UPT loss |  |  |
|  |  |  | light load(RU=28%) | 19.4% | 0.74% UPT loss |  |  |
|  |  |  | Medium load(RU=48%) | 13.7% | 1.01% UPT loss |  |  |
| Vivo [R1-2211018 , R1-2212541] | Dynamic antenna port adaptation (antenna ports are dynamically adapted (between 64 ports and 8 ports) according to the cell traffic load, in every slot) | Cat 1, Set 1 | 12.38% | 9.4% | UPT loss:0.36%; latency increase: 0.08%; UE power increase: 0.02% | Baseline: antenna ports are always 64 | SLS; No UE DRX; FTP3 traffic model,A=0.4, η=1 |
|  |  |  | 12.57% | 9.4% | UPT loss:1.98%; latency increase: 2.20%; UE power increase: 0.04% |  |  |
|  |  |  | 15.31% | 6.8% | UPT loss:12.26%; latency increase: 14.20%; UE power increase: 1.35% |  |  |
|  |  | Cat 2, Set 1 | 12.52% | 8.1% | UPT loss:2.12%; latency increase: 2.35%; UE power increase: 0.17% |  |  |
|  |  |  | 13.16% | 8.1% | UPT loss:6.48%; latency increase:8.25%; UE power increase:0.45% |  |  |
|  |  |  | 16.42% | 6.7% | UPT loss:18.50%; latency increase:38.22%; UE power increase:1.96% |  |  |
|  | Dynamic antenna port adaptation (between 64 ports and 8 ports) with multi-CSI | Cat 1, Set 1 | 15.33% | 12.8% | UPT loss:0.02%; latency increase: 0.05%; UE power increase: 0.01% | Baseline: antenna ports are always 64 | SLS; No UE DRX; FTP3 traffic model,A=0.4, η=1 |
| NOKIA/NSB [R1-2211097] | Reduced number of TX to 32 | Cat 2, Set 1 | Low | 27.6% | UPT: 163,26 Mbps | Single cell operation as per SET1 (64 TRX). UEs are initially in RRC_CONNECTED state | SLS+Post-processing; FTP3 traffic model; A=0,4; Single value η (=1) |
|  |  |  | Light | 28.5% | UPT: 117,64 Mbps |  |  |
|  |  |  | Medium | 29.5% | UPT: 75,47 Mbps |  |  |
| Intel [R1-2212563] | Antenna port adaptation | Cat 1, Set 1 | Low | 19.1% | UPT Baseline: 819.7Mbps UPT ES: 731.1Mbps; Avg EE (baseline): 5.11 Avg EE (ES): 5.46 | Baseline: 64Tx (fixed) ES: 32Tx (fixed) | SLS No C-DRX used for UEs; CSI feedback based on SRS; FTP3 traffic model; A = 0.4; η(s_f,s_p )=1 for any sf, sp; |
|  |  |  |  | 27.3% | UPT Baseline: 819.7Mbps UPT ES: 585.5Mbps; Avg EE (baseline): 5.11 Avg EE (ES): 4.81 | Baseline: 64Tx (fixed) ES: 16Tx (fixed) |  |
|  |  |  |  | 4.5% | UPT Baseline: 819.7Mbps UPT ES: 801.8Mbps; Avg EE (baseline): 5.11 Avg EE (ES): 5.07 | Baseline: 64Tx (fixed) ES: variable |  |
|  |  |  | Light | 25.7% | UPT Baseline: 611.5Mbps UPT ES: 539.8Mbps; Avg EE (baseline): 2.67 Avg EE (ES): 3.11 | Baseline: 64Tx (fixed) ES: 32Tx (fixed) |  |
|  |  |  |  | 35.7% | UPT Baseline: 611.5Mbps UPT ES: 400.3Mbps; Avg EE (baseline): 2.67 Avg EE (ES): 2.73 | Baseline: 64Tx (fixed) ES: 16Tx (fixed) |  |
|  |  |  |  | 1.9% | UPT Baseline: 611.5Mbps UPT ES: 606.7Mbps; Avg EE (baseline): 2.67 Avg EE (ES): 2.71 | Baseline: 64Tx (fixed) ES: variable |  |
|  |  |  | Medium | 29.6% | UPT Baseline: 457.9Mbps UPT ES: 389.3Mbps; Avg EE (baseline): 1.5 Avg EE (ES): 1.84 | Baseline: 64Tx (fixed) ES: 32Tx (fixed) |  |
|  |  |  |  | 41.8% | UPT Baseline: 457.9Mbps UPT ES: 243.9Mbps; Avg EE (baseline): 1.5 Avg EE (ES): 1.67 | Baseline: 64Tx (fixed) ES: 16Tx (fixed) |  |
|  |  |  |  | 0.0% | UPT Baseline: 457.9Mbps UPT ES: 457.8Mbps; Avg EE (baseline): 1.5 Avg EE (ES): 1.50 | Baseline: 64Tx (fixed) ES: variable |  |
| CATT [R1-2211210] | Dynamic adaptation of spatial elements | Cat 1, Set 1 | Low load | 6.8% | UPT loss:0.32% | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms;TxRU= 64. | SLS; (cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms;CSI-RS/TRS 10ms;dynamic spatial antenna adaptation: gNB dynamic adaptation of the number of TxRU from 64TxRU to 32 TxRU; FTP3 traffic model; A=0.4; η(s_f, s_p)=1. |
|  |  |  | Light load | 12.2% | UPT loss:0.62% |  |  |
|  |  |  | Medium load | 18.0% | UPT loss:3.8% |  |  |
|  |  |  | Low load | 6.9% | UPT loss:1.5% | SLS; SSB periodicity 20ms;CSI-RS/TRS 10ms;TxRU= 64. | SLS; No DRX; SSB periodicity 20ms;CSI-RS/TRS 10ms;dynamic spatial antenna adaptation: gNB dynamic adaptation of the number of TxRU from 64TxRU to 32 TxRU; A=0.4; η(s_f, s_p)=1. |
|  |  |  | Light load | 12.3% | UPT loss:1.6% |  |  |
|  |  |  | Medium load | 19.6% | UPT loss:1.8% |  |  |
| Fujitsu [R1-2211085] | Dynamic TxRU adaptation | Cat 2, Set 1 | low | 24.1% | 5.7% average UPT loss | BS #TxRU=64 | CSI feedback period = 20ms, feedback delay = 4ms, immediate antenna adaptation delay gNB dynamically turns out half of the TxRUs if the DL data in the buffer is expected to be transmitted in the next slot; FTP3 traffic model; A=0.4; Single value η (=1) |
|  |  |  | light | 18.6% | 4.1% average UPT loss |  |  |
|  |  |  | medium | 12.0% | 2.3% average UPT loss |  |  |
|  |  | Cat 2, Set 2 | low | 26.5% | 0.6% average UPT loss | BS #TxRU=32 |  |
|  |  |  | light | 20.0% | 0.5% average UPT loss |  |  |
|  |  |  | medium | 12.8% | 0.3% average UPT loss |  |  |
| Ericsson [R1-2212154] | BS #TxRU 32 | Cat1, Set 1 | Low | 15.0% | UPT loss of 1% for 95-%, UPT loss of 2% for 50-% UPT loss of 8% for 5-% | BS #TxRU 64 | 1 SSB Single value η (=1) FTP3 traffic model Dynamic switching applied, i.e. adapting number of antennas for energy efficiency in durations when only users in good channel condition are scheduled. Note separate evaluation performed for different number of antennas (i.e. no switching between these settings). |
|  |  |  | Light | 21.2% | UPT loss of 3% for 95-%, UPT loss of 6% for 50-% UPT loss of 12% for 5-% |  |  |
|  |  |  | Medium | 22.4% | UPT loss of 3% for 95-%, UPT loss of 14% for 50-% UPT loss of 22% for 5-% |  |  |
|  | BS #TxRU 16 |  | Low | 21.4% | UPT loss of 3% for 95-%, UPT loss of 5% for 50-% UPT loss of 14% for 5-% |  |  |
|  |  |  | Light | 31.6% | UPT loss of 6% for 95-%, UPT loss of 15% for 50-% UPT loss of 44% for 5-% |  |  |
|  |  |  | Medium | 36.6% | UPT loss of 8% for 95-%, UPT loss of 25% for 50-% UPT loss of 33% for 5-% |  |  |
| Qualcomm [R1-2212128] | #TxRU reduction (64 to 32) | Cat 1, Set 1 | Low | 29.4% | UPT loss at 50%tile: 31%; DL SINR loss at 5% tile: 4.5dB | BS #TxRU 64 | FTP3 traffic model |
|  |  |  | Light | 28.6% | UPT loss at 50%tile: 30%; DL SINR loss at 5% tile: 6.5dB |  |  |
| Samsung [R1-2212543] | #TxRU reduction (64 to 32) | Cat 1, Set 1 | Medium | 28.82% | UPT loss: 12.97%; latency increase: 16.69% Baseline traffic: 27.87 % RU Changed traffic: 32.28 % RU | BS #TxRU 64 | FR1, Port adaptation from 64 to 32 TxRU FTP3 traffic model |
|  |  |  | Light | 25.3% | UPT loss: 14.70%; latency increase: 16.84%; Baseline traffic: 14.21 % RU Changed traffic: 16.94 % RU |  |  |
|  |  |  | Low | 19.47% | UPT loss: 19.19%; latency increase: 17.46% Baseline traffic: 3.48 % RU Changed traffic: 4.05 % RU |  |  |
|  |  |  | Low | 10.93% | UPT loss: 25.12%; latency increase: 16.66% Baseline traffic: 1.29 % RU Changed traffic: 1.56 % RU |  |  |
|  | #TxRU reduction (32 to 16) | Cat 1, Set 3 | Medium | 31.9% | UPT loss: 10.37%; latency increase: 4.58% Baseline traffic: 21.69 % RU Changed traffic: 22.56 % RU | BS #TxRU 32 | FR2, Port adaptation from 32 to 16 TxRU FTP3 traffic model |
|  |  |  | Low | 26.8% | UPT loss: 13.42%; latency increase: 15.54% Baseline traffic: 7.23 % RU Changed traffic: 7.5 % RU |  |  |
|  | #TxRU reduction (32 to 8) | Cat 1, Set 3 | Medium | 48.2% | UPT loss: 7.6%; latency increase: 12.47% Baseline traffic: 21.69 % RU Changed traffic: 23.31 % RU | BS #TxRU 32 | FR2, Port adaptation from 32 to 8 TxRU FTP3 traffic model |
|  |  |  | Low | 40.5% | UPT loss: 13.7%; latency increase: 26.4% Baseline traffic: 7.23 % RU Changed traffic: 7.76 % RU |  |  |

12 sources observed that BS energy savings can be achieved, at all loads for different sets of reference configurations with FTP3 for FTP3 IM traffic models, with or without UE C-DRX configuration. The gain depends on whether there is multiple CSI report assistance, the number of antenna ports that can be adapted, the load scenarios, and UPT loss. No performance analysis was provided for broadcast and common channels with dynamic antenna adaptation.

With dynamic/semi-static adaptation of spatial elements,

- One source shows that BS energy saving for UE specific PDSCH for FR1 can be achieved by 3.4%~19.2% with dynamic adaptation and multi-CSI, compared to dynamic adaptation of spatial elements with single CSI report. The UPT loss was observed by less than 10%.

- 2 source shows that the gain for UE specific PDSCH for FR1 can be 7.6%~31.3% with dynamic adaptation and multi-CSI, compared with no adaptation, with UPT loss of 0.02%~7%.

- 2 sources show that the gain can be 6.7%~26.5% with dynamic adaptation without multi-CSI, compared to no adaptation, with UPT loss of 0.3%~18.5%.

- 9 source show that the gain can be 4.8%~48.2% with static adaptation without multi-CSI, compared to no adaptation, with UPT loss of 0.02%~87.08%. One source observed that the downlink coverage is reduced by 4.5dB – 6.5dB when reducing the number of TxRUs from 64 to 32 in Set 1 FR1 configuration.

- One source shows that when dynamic antenna adaptation is variably changed, the gain is reduced to a range of 0~4.5% with 0.02%~2.18% UPT loss.

- One source shows BS energy saving can be 18.8%~19.7% that additional gain can be obtained when this scheme is combined with PDSCH power offset. The UPT loss is observed by 9.06%.

- More number of elements are reduced, more gain can be generally obtained.

On latency, there is negative impact observed in three sources and the increment becomes larger as the number of reduced antenna ports becomes larger.

On UE power consumption, 2 sources show that there is increase by up to 79.99% % (@ MTK: if additional power due to obtaining/sending additional CSIs is modeled in the evaluation).

Additionally, one result shows that the average EE can be generally increased except for the low load case where number of antenna is reduced from 64 to 16, and the case of antenna number variably changing.

=== end ===

| Company | Comments |
| --- | --- |
| Samsung | Based on our updates, I revised the observation as below: Based on the results, it is observed that BS energy savings can be achieved by 3.4%~48.2% at all loads for different sets of reference configurations with FTP3 for FTP3 IM traffic models, with or without UE C-DRX configuration, compared to no dynamic antenna adaption. The gain depends on whether there is multiple CSI report assistance, and the number of antenna ports that can be adapted. More number of elements are reduced, more gain can be generally obtained. One result reports that when dynamic antenna adaptation is variably changed, the gain is reduced to a range of 0~4.5%. One result reports that additional gain can be obtained when this scheme is combined with PDSCH power offset. |
| Fujitsu | Regarding the factors that affect the energy saving gain, aside from the multiple CSI report assistance and number of antenna ports, there are other impact factors, for example, adaptation of spatial elements is conducted in semi-static or dynamic manner, the periodicity of CSI feedback, etc. Thus, we don’t think the conclusion can be drawn that the energy saving gain depends on whether there is multiple CSI report assistance, and the number of antenna ports that can be adapted. Similarly, whether multiple CSI report assistance is available or not is not the only factor affects UPT performance. We suggest the following modifications: Based on the results, it is observed that BS energy savings can be achieved by 3.4%~49.4% at all loads for different sets of reference configurations with FTP3 for FTP3 IM traffic models, with or without UE C-DRX configuration, compared to no dynamic antenna adaption. The gain depends on whether there is multiple CSI report assistance, and the number of antenna ports that can be adapted. More number of elements are reduced, more gain can be generally obtained. One result reports shows that when dynamic antenna adaptation is variably changed, the gain is reduced to a range of 0~4.5%. One result reports shows that additional gain can be obtained when this scheme is combined with PDSCH power offset. On UPT impact, When there is no multiple CSI report assistance, one result shows that the BS energy saving gain comes at the expense of UPT loss which can be up to 87.08% by reducing the number of TxRU from 64 to 8. Two sets of results (Huawei/HiSilicon, ZTE/Sanechips) show that dynamically adapting the number of TxRUs with When there is multiple CSI report assistance, two results show that the gain remains in the range of 3.4%~31.3% with UPT loss mitigated/reduced to less than 10% or no UPT loss. |
| QCOM1 | The results/observations should be classified into two categories: (1) Observations with dynamic configuration of spatial elements (e.g., results from Huawei, ZTE) (2) Observations with static configuration of spatial elements. The dynamic configuration is based on multiple CSI feedback corresponding to multiple configurations, which are only suitable for UE-specific PDSCH. For broadcast channels/signals, only semi-static configuration is applicable since there is no CSI procedure associated with these channels/signals. |
| ZTE, Sanechips | Some modifications on our results for Technique#C-1. |
| Ericsson1 | Suggest adding the following in last column of the table for Ericsson results. Evaluation results show potential benefit achievable via dynamic switching i.e. adapting number of antennas for energy efficiency in durations when only users in good channel condition are scheduled. Note separate evaluation performed for different number of antennas (i.e. no switching between these settings). |
| LG Electronics | We support the modified observation by Fujitsu. |
| vivo | Sorry for late to add simulation results of dynamic port adaptation with multi-CSI, which has been inserted in the above table and uploaded to the draft inbox with the updated excel table. We recommend updating the observation to the following revised version in red, with the reasons marked in blue： Based on the results, it is observed that BS energy savings can be achieved by 3.4%~49.4% at all loads for different sets of reference configurations with FTP3 for FTP3 IM traffic models, with or without UE C-DRX configuration, compared to no dynamic antenna adaption. Based on the above results, it is observed that dynamic spatial elements adaptation can achieve X%-Y% energy saving gain compared with no dynamic adaptation. It can also be observed that the semi-static adaptation of spatial elements can achieve A%-B% energy saving gain compared with no dynamic adaptation. The gain depends on whether there is multiple CSI report assistance, and the number of antenna ports that can be adapted. More number of elements are reduced, more gain can be generally obtained. One result reports that when dynamic antenna adaptation is variably changed, the gain is reduced to a range of 0~4.5%. One result reports that additional gain can be obtained when this scheme is combined with PDSCH power offset. [vivo: We support the same view with QC. The simulation results for semi-static shutdown and dynamic shutdown antennas should be summarized separately] When there is no multiple CSI report assistance, the BS energy saving gain comes at the expense of UPT loss which can be up to 87.08%. When there is multiple CSI report assistance, the gain remains in the range of 3.4%~31.3% with UPT loss mitigated/reduced to less than 10% or no UPT loss. On latency, there is negative impact observed in three results and the increment becomes larger as the number of reduced antenna ports becomes larger. There is also one result showing that the latency can be slightly reduced. On UE power consumption, one two result shows that there is increase and one result shows there can be slight UE power savings. [vivo: One clarification on our reporting results: all our reported results are (baseline-enhance)/baseline. So negative value for latency/UE power means increased latency/UE power. We already update it in the above table] |

6.3.1.3	Specification impacts

6.3.2	Technique C-2 ZZZ

6.3.2.1	Description of technique

6.3.2.2	Analysis of performance and impacts

##### === (Consensus) draftTP C-2: start ===

The following capture the results for TRP muting in multi-TRP operation.

Table 6.1.1-x: BS energy savings by TRP muting in multi-TRP operation

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/latency/UE power consumption/ Other KPIs | Baseline configuration/assumption | Evaluation methodology/assumption details |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NOKIA/NSB [R1-2211097] | Semi-static reduced number of TRPs | Cat 2, Set 1 | Low | 38.8% | UPT loss: -14.49% | 2 TRPs are assumed. UEs are initially in RRC_CONNECTED state. | SLS+Post-processing, FTP3 traffic model; A=0,4; Single value η (=1). 70% of the P_Static among TRPs |
|  |  |  | Light | 37.2% | UPT loss: -14.14% |  |  |
|  |  |  | Medium | 36.9% | UPT loss: -7.27% |  |  |
| CATT [R1-2211210] | Dynamic TRP muting/adaptation in multi-TRP operation | Cat 1, Set 1 | Low load | 28.4% | UE power: 50.6; UE ESG:12.7% | M-TRP configuration: One cell is configured with 2TRPs; Both of TRP are activated. | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms; CSI-RS/TRS 10ms; TRP OFF: 160ms SSB/CSI-RS transmission. When TRP is activated, additional CSI-RS/TRS is transmitted before data scheduling. FTP3 traffic model; A=0.4; η(s_f, s_p)=1. |
|  |  |  | Light load | 28.7% | UE power: 50.6; UE ESG:12.7% |  |  |
|  |  |  | Medium load | 19.7% | UE power: 51.6; UE ESG:12.4% |  |  |
| Qualcomm [R1-2212128] | Semi-static TRP reduction (2 to 1) | Cat 1, Set 1 | Low | 41.6% | UPT loss at 50%-tile: 16% | 2 TRPs, each with 64 TxRUs | FTP3 traffic model |
|  |  |  | Light | 39.0% | UPT loss at 50%-tile: 22% |  |  |

For two TRP configuration case at different loads,

- with semi-static TRP reduction, BS energy saving gain can be achieved by 36.9%~41.6% compared to no TRP reduction, with UPT loss of 7.27%~22%;

- with dynamic TRP reduction, compared to no TRP reduction, BS energy saving gain can be achieved by 19.7%~28.7%, without reported UPT impact. It assumes two TRP is transmitting CSI-RS and

For the BS energy saving gain around 19.7%~28.7%, it is also observed from one source that UE power savings can be achieved by about 12%.

=== end ===

| Company | Comments |
| --- | --- |

6.3.2.3	Specification impacts

6.3.z	Impacts on network interfaces

6.4	Techniques in power domain

6.4.1	Technique D-1 WW

6.4.1.1	Description of technique

6.4.1.2	Analysis of performance and impacts

##### === (Consensus) draftTP D-1: start ===

The following capture the results for adaptation of transmission power of signals and channels.

Table 6.1.1-x: BS energy savings by dynamic transmission power adaptation

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | Baseline configuration/assumption | Other KPI |
| --- | --- | --- | --- | --- | --- | --- |
| MTK [R1-2212259] | PDSCH_PowOffset_-3dB | Cat 1 | Light | 8.7% | Baseline：PDSCH power offset 0 dB ES scheme：PDSCH power offset -3/-6/-9 dB reference configuration：Set1 FTP3，DRX (160, 8, 100)，Single value η (=1) | UPT:2.03%,UE power comsumption:1.80%,ltency:2.07% |
|  | PDSCH_PowOffset_-6dB |  |  | 11.1% |  | UPT:5.66%,UE power comsumption:4.47%,latency:6.00% |
|  | PDSCH_PowOffset_-9dB |  |  | 9.0% |  | UPT:10.80%,UE power comsumption:9.17%,latency:12.11% |
|  | PDSCH_PowOffset_-3dB |  | Medium | 13.9% |  | UPT:3.28%,UE power comsumption:2.46%,latency:3.39% |
|  | PDSCH_PowOffset_-6dB |  |  | 18.7% |  | UPT:8.97%,UE power comsumption:6.80%,latency:9.85% |
|  | PDSCH_PowOffset_-9dB |  |  | 17.7% |  | UPT:19.49%,UE power comsumption:14.78%,latency:24.21% |
|  | PDSCH_PowOffset_-3dB | Cat 2 | Light | 8.7% |  | UPT:2.03%,UE power comsumption:1.80%,latency:2.07% |
|  | PDSCH_PowOffset_-6dB |  |  | 11.8% |  | UPT:5.66%,UE power comsumption:4.47%,.latency:6.00% |
|  | PDSCH_PowOffset_-9dB |  |  | 11.2% |  | UPT:10.80%,UE power comsumption:9.17%,latency:12.11% |
|  | PDSCH_PowOffset_-3dB |  | Medium | 14.7% |  | UPT:3.28%,UE power comsumption:2.46%,latency:3.39% |
|  | PDSCH_PowOffset_-6dB |  |  | 20.6% |  | UPT:8.97%,UE power comsumption:6.80%,latency:9.85% |
|  | PDSCH_PowOffset_-9dB |  |  | 21.0% |  | UPT:19.49%,UE power comsumption:14.78%,latency:24.21% |
|  | #TxRU_32_PDSCH_PowOffset_-3dB | Cat 1 | Light | 18.8% | Baseline：PDSCH power offset 0 dB,BS #TxRU 64 ES scheme：PDSCH power offset -3 dB, BS #TxRU 32 reference configuration：Set1 FTP3，DRX (160, 8, 100)，Single value η (=1) | UPT:9.06%,UE power comsumption:7.62%,latency:9.96% |
|  |  | Cat 2 |  | 19.7% |  | UPT:9.06%,UE power comsumption:7.62%,latency:9.96% |
| Huawei，HiSilicon [R1-2210858] | Dynamic Power back-off with Multiple CSIs | Cat 2 | 30% load(medium) | 5.3% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set1 FTP3 IM, NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) | UPT: 0% loss |
|  |  |  |  | 7.3% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set1 VoIP, NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |  |
|  |  |  |  | 6.9% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set2 VoIP, NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |  |
|  |  |  |  | 11.5% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set2 VoIP, 4T for Set 2,NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |  |
|  |  |  |  | 5.3% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set2 FTP3 IM, 4T for Set 2, NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |  |
| ZTE,Sanechips [R1-2211903] | PDSCH PSD reduction 53.75dBm | Cat 2 | Low load(RU=10%) | 2.3% | Baseline: 55dBm reference configuration:Set 1: FR1 TDD, FTP3: 20K packet size, slot-level,Pstatic=P3, η=1 | 0.56% UPT loss |
|  |  |  | light load(RU=20%) | 4.4% |  | 1.82% UPT loss |
|  |  |  | Medium load(RU=31%) | 6.0% |  | 3.8% UPT loss |
|  | PDSCH PSD reduction 52dBm |  | Low load(RU=10%) | 6.4% |  | 1.26% UPT loss |
|  |  |  | light load(RU=20%) | 10.1% |  | 1.83% UPT loss |
|  |  |  | Medium load(RU=31%) | 12.9% |  | 2.38% UPT loss |
|  |  |  | Low load(RU=4.7%) | 3.9% | Baseline: 55dBm reference configuration:Set 1: FR1 TDD, FTP3: 0.5M packet size, slot-level,Pstatic=P3, η=1 | 5.74% UPT loss |
|  |  |  | Low load(RU=9.6%) | 7.0% |  | 4.32% UPT loss |
|  |  |  | light load(RU=23.5%) | 12.2% |  | 5.48% UPT loss |
|  |  |  | Medium load(RU=38.4%) | 16.6% |  | 9.81% UPT loss |
|  | PDSCH PSD reduction 47.75dBm |  | Low load(RU=13%) | 5.9% | Baseline: 49dBm reference configuration:Set 2: FR1 FDD, FTP3: 0.1M packet size, slot-level,Pstatic=P3, η=1 | 0.64% UPT loss |
|  |  |  | light load(RU=29%) | 8.6% |  | 0.05% UPT loss |
|  | PDSCH PSD reduction 46dBm |  | Low load(RU=13%) | 11.8% |  | 1.56% UPT loss |
|  |  |  | light load(RU=29%) | 17.0% |  | 0.75% UPT loss |
|  | Dynamic PDSCH PSD adaptation via multi-CSI |  | Low load(RU=10%) | 12.1% | Baseline: 55dBm reference configuration:Set 1: FR1 TDD, FTP3: 20K packet size, slot-level,Pstatic=P3, η=1 | 0.38% UPT loss |
|  |  |  | light load(RU=20%) | 16.6% |  | 0.35% UPT loss |
|  |  |  | Medium load(RU=31%) | 23.8% |  | 1.17% UPT loss |
|  |  |  | Medium load(RU=31%) | 16.4% |  | 0.27% UPT loss |
| NOKIA/NSB [R1-2211097] | Reduced DL transmit power by 3dB | Cat 2 | Low | 9.8% | Baseline: MaximumTx power of 49 dBm.UEs are initially in RRC_CONNECTED state reference configuration:Set 2, DL-FTP3, A=0,4; Single value η (=1) SLS | UPT:144 Mbps |
|  | Reduced DL transmit power by 6dB |  |  | 12.2% |  | UPT:134 Mbps |
|  | Reduced DL transmit power by 9dB |  |  | 13.4% |  | UPT:119 Mbps |
|  | Reduced DL transmit power by 3dB |  | Light | 15.4% |  | UPT:104 Mbps |
|  | Reduced DL transmit power by 6dB |  |  | 19.7% |  | UPT:97 Mbps |
|  | Reduced DL transmit power by 9dB |  |  | 17.9% |  | UPT:88 Mbps |
|  | Reduced DL transmit power by 3dB |  | Medium | 16.2% |  | UPT:76 Mbps |
|  | Reduced DL transmit power by 6dB |  |  | 23.4% |  | UPT:70 Mbps |
|  | Reduced DL transmit power by 9dB |  |  | 24.7% |  | UPT:63 Mbps |
| DCM [R1-2211994] | PDSCH_PowOffset_-6dB | Cat 1 | Light | 6.6% | Baseline:PDSCH power offset 0 dB reference configuration:Set 1,FTP3, A=0.4, Single value η (=1) | 1.00% UPT loss |
|  | PDSCH_PowOffset_-12dB |  |  | 8.3% |  | 2.60% UPT loss |
|  | PDSCH_PowOffset_-18dB |  |  | 8.7% |  | 5.70% UPT loss |
|  | PDSCH_PowOffset_-6dB |  | Medium | 15.5% |  | 2.50% UPT loss |
|  | PDSCH_PowOffset_-12dB |  |  | 19.5% |  | 2.00% UPT loss |
|  | PDSCH_PowOffset_-18dB |  |  | 20.4% |  | 5.80% UPT loss |
|  | PDSCH_PowOffset_-6dB |  | High | 24.1% |  | -3.10% UPT loss |
|  | PDSCH_PowOffset_-12dB |  |  | 29.9% |  | -0.70% UPT loss |
|  | PDSCH_PowOffset_-18dB |  |  | 31.4% |  | 0% UPT loss |
| Intel [R1-2212563] | Transmit Power Adaptation/ -12dB power | Cat1 | Low | 19.2% | Baseline: Full power SLS; No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; No paging overhead; 1 SSB beam; SSB/PRACH periodicity: 20msec; SIB1 periodicity: 40msec; Slot level model For scaling: A = 0.4; η(s_f,s_p )=1 for any sf, sp; | Baseline: 819.7 Mbps ES: 746 Mbps Avg EE (Baseline): 5.10 Avg EE (ES) : 5.43 |
|  |  |  | Light | 28.2% |  | Baseline: 611.5 Mbps ES: 567.5 Mbps Avg EE (Baseline): 2.66 Avg EE (ES) : 3.25 |
|  |  |  | Medium | 34.3% |  | Baseline: 457.9 Mbps ES: 415.1 Mbps Avg EE (Baseline): 1.5 Avg EE (ES) : 2.03 |
|  | Transmit Power Adaptation/ -6dB power |  | Low | 17.6% |  | Baseline: 819.7 Mbps ES: 798.5 Mbps Avg EE (Baseline): 5.10 Avg EE (ES) : 5.83 |
|  |  |  | Light | 25.4% |  | Baseline: 611.5 Mbps ES: 604.8 Mbps Avg EE (Baseline): 2.66 Avg EE (ES) : 3.41 |
|  |  |  | Medium | 30.0% |  | Baseline: 457.9 Mbps ES: 450.7 Mbps Avg EE (Baseline): 1.5 Avg EE (ES) : 2.06 |
| CATT [R1-2211210] | Adaptation of transmission power of signals and channels | Cat 1 | Low load | 3.9% | Baseline:SSB periodicity 20ms;CSI-RS/TRS 10ms;Transmission power:55dBm; reference configuration:Set 1, FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes; A=0.4; η(s_f, s_p)=1. | UPTloss:1.6% |
|  |  |  | Light load | 7.7% |  | UPTloss:1.8% |
|  |  |  | Medium load | 9.1% |  | UPTloss:2.8% |
|  |  |  | Low load | 4.1% | Baseline: SSB periodicity 20ms;CSI-RS/TRS 10ms;(DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);Power domain adaptation; reference configuration:Set 1, FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes; A=0.4; η(s_f, s_p)=1. | UPTloss:1.9% |
|  |  |  | Light load | 7.7% |  | UPTloss:3.9% |
|  |  |  | Medium load | 11.2% |  | UPTloss:3.1% |
| Ericsson [R1-2212154] | Tx power adaptation (reduction up to 12 dB) | Cat1 | Low | 20.9% | Baseline: BS Tx power 55 dBm reference configuration:Set 1,FTP3 1 SSB Single value η (=1). For ES scheme: dynamic switching applied, i.e. adapting DL Tx power for energy efficiency in durations when only users in good channel condition are scheduled. Note separate evaluation performed for different power settings (i.e. no switching between these settings) | UPT loss of 1% for 95-% UE, UPT loss of 3% for 50-% UE UPT loss of 22% for 5-% UE |
|  |  |  | Light | 40.5% |  | UPT loss of 1% for 95-% UE, UPT loss of 8% for 50-% UE UPT loss of 36% for 5-% UE |
|  |  |  | Medium | 47.6% |  | UPT loss of 0% for 95-% UE, UPT loss of 9% for 50-% UE UPT loss of 13% for 5-% UE |
|  | Tx power adaptation (reduction up to 6 dB) |  | Low | 17.7% |  | UPT loss of 1% for 95-% UE, UPT loss of 1% for 50-% UE UPT loss of 2% for 5-% UE |
|  |  |  | Light | 33.0% |  | UPT loss of 0% for 95-% UE, UPT loss of 4% for 50-% UE UPT loss of 7% for 5-% UE |
|  |  |  | Medium | 38.5% |  | UPT loss of 2% for 95-% UE, UPT loss of 9% for 50-% UE UPT loss of 7% for 5-% UE |
| Samsung [R1-2212543] | Transmission power adaptation | Cat 1 | Reference traffic: 7.5 % RU Reduced traffic: 4.4 % RU | 51.5% | Baseline: BS Tx power 55 dBm reference configuration:Set 1,FTP3 a static part of power for BS: P_3 A: 0.4 For eta, If two values of η(s_f,s_p ) are used for evaluation,η(s_f,s_p ) = 0.76 if s_f*s_p <0.5; otherwise, η(s_f,s_p )=1. 46.5 and 5.2 relative power per a SSB for Cat 1 and Cat 2 | UPT:10.40%, Packet latency: 24.7% Scheduling latency: No increase |
|  |  | Cat 2 |  | 17.5% |  | UPT:10.40%, Packet latency: 24.7% Scheduling latency: No increase |
| Qualcomm [R1-2212128] | Tx power reduction (55dBm to 52dBm) | Cat 1 | Low | 13.1% | Baseline: BS BS #TxRU 64 with 55dBm Tx power reference configuration:Set 1,FTP3 | UPT lsss at 50%-tile: 10%, DL SINR loss at 5% tile: 4dB |
|  |  |  | Light | 6.6% |  | UPT loss at 50%-tile: 16%, DL SINR loss at 5% tile: 3dB |

With transmission power reduction on PDSCH, 10 sources show that it can achieve BS energy savings gain at all load cases for both Set 1 FR1 TDD and Set 2 FR1 FDD including 4Tx BS antenna configuration, for both BS categories with FTP3 or FTP3 IM model, with or without UE C-DRX configuration.

With dynamic power reduction assisted by multi-CSI,

- one source observed BS energy saving gain by 5.3%~11.5%, compared to dynamic power reduction without multi-CSI report; one source observed BS energy savings by 12.1%~23.8%, compared to no power reduction baseline;

- The gain generally increases when the traffic load increases;

- The UPT loss is less than 1.17%.

- No performance analysis was provided for broadcast and common channels with dynamic downlink transmission power adaptation.

With semi-static power reduction of 3~18dB in 6 sources and two other sources, compared to a baseline without power reduction, network energy saving can be achieved by 3.9%~51.6%.

- The gain can increase as the traffic load increases in most cases while one source observed a reduced gain, for BS category 1 with power reduction of 3 dB;

- The UPT loss is observed from 2.03%~19.49%.

One source observed that the latency can be increased by up to 24.21% when the power reduction level is up to 9 dB; one source observed that packet latency can be increased by 24.7% while scheduling delay is not increased.

On UE power consumption, one source shows that less than 10% increment is observed in most cases.

One source also observed that when combined with spatial element reduction, in the case of 3 dB power reduction, the network energy savings can be further increased by about 10%, while together with UPT loss/UE power consumption increase/latency increase of 9.06%/7.62%/9.96% respectively.

One source shows this scheme can increase the average EE and one source observed that the downlink coverage is reduced by 3dB – 4dB when reducing the downlink transmission power from 55dBm to 52dBm in Set 1 FR1 configuration.

=== end ===

| Company | Comments |
| --- | --- |
| CATT | The results are quite consistent from most companies in the range 2.3% - 20% except results from a couple of companies with 20-51.5% gain. We should have the range of energy saving gain from the common results and outline the reason for the techniques with extreme high power saving gain. |
| QCOM1 | The results/observations should be classified into two categories: (1) Observations with dynamic power configuration (2) Observations with static power configuration. The dynamic configuration is based on multiple CSI feedback corresponding to multiple configurations, which are only suitable for UE-specific PDSCH. For broadcast channels/signals, only semi-static configuration is applicable since there is no CSI procedure associated with these channels/signals. |
| ZTE, Sanechips | Some modifications on our results for Technique#D-1. |
| Ericsson1 | Suggest adding the following in the second last column for Ericsson results in the table. Evaluation results show potential benefit achievable via dynamic switching i.e. adapting DL Tx power for energy efficiency in durations when only users in good channel condition are scheduled. Note separate evaluation performed for different power settings (i.e. no switching between these settings). |

6.4.1.3	Specification impacts

6.4.2	Technique D-3 WWW

6.4.2.1	Description of technique

6.4.2.2	Analysis of performance and impacts

##### === (Consensus) draftTP D-3: start ===

The following capture the results by Channel Aware Tone reservation.

Table 6.1.1-x: BS energy savings by Channel Aware Tone reservation

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | Baseline configuration/assumption | Other KPI | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm [R1-2212129] | PA Backoff reduction of 1-3dB due to PAPR reduction from Channel Aware Tone reservation | Cat 1 |  | 9.5% | Set 3 | UPT:0.00% latency: 0% UE power consumption: 0% | Evaluation showing utilization of PAPR reduction, where the PAPR reduction is used to reduce the backoff PA attribute (Pmax) while maintaining the TX power and signal EVM The Backoff also compensates for the tones used for the TR signal, thus no UPT loss occurs Comparing Channel Aware Tone Reservation to Transparent Tone Reservation Note: η was calculated corresponding to the backoff reduction as was provided by a formula (referencing Tdoc R1-220996) |
|  |  |  |  | 2.1% |  |  |  |
|  |  |  |  | 4.4% | Set 1 |  |  |
|  |  |  |  | 2.1% |  |  |  |

One source observed that channel aware tone reservation can achieve PA back-off reduction of 1-3 dB which leads to 2.1%~9.5% BS energy saving gains depending on configurations, compared with transparent tone reservation. Note RAN1 has no power consumption scaling model for this NW ES scheme.

On UPT/latency, no negative impact is observed.

No impact on UE power consumption.

=== end ===

| Company | Comments |
| --- | --- |
| QCOM1 | support |
| MTK 2 | Suggested revision as follows: One company observed that channel aware tone reservation can achieve PA back-off reduction of 1-3 dB which leads to 2.1%~9.5% BS energy saving gains depending on configurations, compared with transparent tone reservation. Note RAN1 has no power consumption scaling model for this NW ES scheme. |
| QCOM3 | @MTK 2: We agree with the first suggested change. However, we don’t agree with the added note since we follow the agreed power model in our evaluation. Following the offline discussion, in order to address the need to explain how the evaluations are done considering the power model, we therefore propose the following: One source observed that channel aware tone reservation can achieve PA back-off reduction of 1-3 dB which leads to 2.1%~9.5% BS energy saving gains depending on configurations, compared with transparent tone reservation. For the sake of simplicity and progress, PAE scaling factor (η) =1 was agreed to be used in the power consumption scaling model (optionally - an additional value taken from a formula, was used for power reduction > 3dB). However, this doesn’t capture the PAE increase with increased transmit power and reduced backoff. Therefore, η was calculated corresponding to the backoff reduction as was provided by said formula (referencing Tdoc R1-220996) On UPT/latency, no negative impact is observed. No impact on UE power consumption. |

6.4.2.3	Specification impacts

6.4.3	Technique D-5 adaptation of transceiver processing algorithm

6.4.3.1	Description of technique

6.4.3.2	Analysis of performance and impacts

##### ===(Consensus) draftTP D-5: start ===

The following capture the results for UE post-distortion.

Table 6.1.1-x: BS energy savings by [UE post-distortion]

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | Baseline configuration/assumption | Other KPI | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm [R1-2212129] | UE post-distortion | Cat 1 |  | 16.1% | Set 3 | UPT:0.00% latency: 0% | Evaluation showing utilization of PAPR reduction, where the PAPR reduction is used to reduce the backoff PA attribute (Pmax) while maintaining the TX power and signal EVM The Backoff also compensates for the tones used for the TR signal, thus no UPT loss occurs Processing and power consumption of the UE depends on the UE receiver’s design for DPoD. Note: η was calculated corresponding to the backoff reduction as was provided by a formula (referencing Tdoc R1-220996) |

One source observed that UE post-distortion can achieve BS energy saving by 16.1% for Set 3 reference configuration. Note RAN1 has no power consumption scaling model for this NW ES scheme.

On UPT or latency, there is no negative impact observed.

The impact on UE power consumption depends on UE receiver’s design for DPoD.

=== end ===

| Company | Comments |
| --- | --- |
| QCOM1 | support |
| MTK2 | Suggested revision: One company observed that UE post-distortion can achieve BS energy saving by 16.1% for Set 3 reference configuration. Note RAN1 has no power consumption scaling model for this NW ES scheme. |
| QCOM3 | @MTK 2: We agree with the first suggested change. However, we don’t agree with the added note since we follow the agreed power model in our evaluation. Following the offline discussion, in order to address the need to explain how the evaluations are done considering the power model, we therefore propose the following: One source observed that UE post-distortion can achieve BS energy saving by 16.1% for Set 3 reference configuration. For the sake of simplicity and progress, PAE scaling factor (η) =1 was agreed to be used in the power consumption scaling model (optionally - an additional value taken from a formula, was used for power reduction > 3dB). However, this doesn’t capture the PAE increase with increased transmit power and reduced backoff. Therefore, η was calculated corresponding to the backoff reduction as was provided by said formula (referencing Tdoc R1-220996) On UPT/latency, no negative impact is observed. The impact on UE power consumption depends on UE receiver’s design for DPoD. |

6.4.2.3	Specification impacts

6.4.z	Impacts on network interfaces

6.4.4	Technique D-2 WWW

6.4.4.1	Description of technique

6.4.4.2	Analysis of performance and impacts

##### === (Consensus) draftTP D-2: start ===

The following capture the results by Over the air DPD.

Table 6.1.1-x: BS energy savings by Over the air DPD

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | Baseline configuration/assumption | Other KPI | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm [R1-2212745] | Over the air DPD (DPD-OTA) | Cat 1 |  | 8.9% | Set 3 | UPT:0.00% latency: 0% | Evaluation showing utilization of PAPR reduction, where the PAPR reduction is used to reduce the backoff PA attribute (Pmax) while maintaining the TX power and signal EVM Note: η was calculated corresponding to the backoff reduction as was provided by a formula (referencing Tdoc R1-220996) |

One source observed that DPD-OTA can achieve BS energy saving by 8.9% for Set 3 reference configuration. Note RAN1 has no power consumption scaling model for this NW ES scheme.

On UPT/latency, no negative impact is observed.

Additional UE power consumption is considered to be negligible due to the low report periodicity expected.

=== end ===

| Company | Comments |
| --- | --- |
| QCOM1 | support |
| LG Electronics | It seems necessary to clarify - If Qualcomm considered increased gNB power consumption to perform digital pre-distortion at gNB side. If Qualcomm considered additional UE power consumption to report over-the-air assistance information. |
| MTK2 | Suggested revision: One company observed that DPD-OTA can achieve BS energy saving by 8.9% for Set 3 reference configuration. Note RAN1 has no power consumption scaling model for this NW ES scheme. |
| QCOM2 | @LGE: - In our view DPD is typically implemented on gNB side to improve the PA efficiency. We took DPD as part of baseline BS power model. - The periodicity of report is very low (around ~1sec); therefore, we didn’t consider additional UE power consumption for reporting OTA assistance information, since this additional UE power consumption is negligible |
| QCOM3 | @MTK 2: We agree with the first suggested change. However, we don’t agree with the added note since we follow the agreed power model in our evaluation. Following the offline discussion, in order to address the need to explain how the evaluations are done considering the power model, we therefore propose the following: One source observed that DPD-OTA can achieve BS energy saving by 8.9% for Set 3 reference configuration. For the sake of simplicity and progress, PAE scaling factor (η) =1 was agreed to be used in the power consumption scaling model (optionally - an additional value taken from a formula, was used for power reduction > 3dB). However, this doesn’t capture the PAE increase with increased transmit power and reduced backoff. Therefore, η was calculated corresponding to the backoff reduction as was provided by said formula (referencing Tdoc R1-220996) On UPT/latency, no negative impact is observed. The impact of reporting over-the-air assistance information on UE power consumption is negligible due to the low report periodicity expected |

6.5	Other energy saving aspects and techniques

Editor's note: placeholder.

6.x	Higher layer aspects for network energy savings

Editor's note: This section includes common aspects of higher layers deduced from the above candidate directions.

7	Conclusions

##### === (non-consensus) draftTPX: start ===

Based on the evaluation and analysis for techniques in time and frequency domain, the following can be recommended:

- In the DL, dynamic indication that enables configuration change/update for a group of UEs on DRX/DTX

- In the UL, an UL signal that can support wake up gNB that is targeting energy saving by a longer (than currently supported) SSB periodicity or operating with SSB- and/or SIB-less, where

  - for SSB-less, where UE synchronization for current cell is acquired from other cell with SSB transmission;

  - for SIB1-less, where a UE acquires system information for the current cell from other cell;

Based on the evaluation and analysis for techniques in spatial domain, the following can be recommended:

- dynamically adapting spatial elements such as the number of active transceiver chains or the number of active antenna panels at gNB in transmitting and/or receiving channels and signals, by enhancing the related configuration/indication to the UE (e.g. on spatial element patterns for potential reduction of spatial element reduction) and/or enhancing the UE feedback (e.g. CSI report) to assist NW energy saving operation.

Based on the evaluation and analysis for techniques in power domain, the following can be recommended:

- dynamically adapting the transmission power or PSD of downlink signals and channels, by enhancing the related configuration to the UE (e.g. considering power offsets that account for potential power adaptation) and/or enhancing the UE feedback (e.g. CSI report) to assist NW energy saving operation.

  - Enhancements for updating the power offset values between various signals and channels, e.g. PDSCH to CSI-RS using lower layer signalling can be considered.

=== end ===

| Company | Comments |
| --- | --- |