3GPP TSG RAN WG1 #112                                                                                  R1-2300152

Athens, Greece, February 27th - March 3rd, 2023

Source:	New H3C

Title:	Discussions on increased number of orthogonal DMRS ports

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

- Introduction

In the RAN1 #111 meeting, increased number of orthogonal DMRS ports in Rel.18 NR MIMO evolution was discussed. Some agreements were made as below [1].

Agreement

For FD-OCC length 4 for PDSCH/PUSCH, select the following:

- Opt.1-1 (Walsh matrix) for PDSCH

- Opt.1-2 (Cyclic shift) for PUSCH

Agreement

- For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, all of the following port combinations can be indicated:

  - Cat. 1) Legacy port indexes (eType 1: p=0~7, eType 2: p=0~11)

  - Cat. 2) New port indexes (eType 1: p=8~15, eType 2: p=12~23)

  - Cat. 3) Legacy port indexes and New port indexes at least within a CDM group at least for maxLength=1 (eType 1: up to 4 ports from {0, 1, 8, 9} and/or up to 4 ports from {2, 3, 10, 11}, eType 2: up to 4 ports from {0, 1, 12, 13} and/or up to 4 ports from {2, 3, 14, 15} and/or up to 4 ports from {4, 5, 16, 17}) at least for S-TRP case,

    - For up to 4 ranks, only one CDM group is used per UE. For larger than 4 ranks, more than one CDM groups can be used per UE.

- FFS: Whether to increase the size of antenna ports field in DCI format 1_1/1_2, or introduce new DCI field for antenna ports indication, or not.

- FFS: Whether the new antenna port(s) table is specified or not.

- FFS: MU restrictions for certain entries. e.g., DMRS ports = {0,2}, or {8,10}, etc.

- FFS: Cat.3 for M-TRP case.

- Note: DMRS port index for PDSCH is determined by p +1000

Agreement

- For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS.

  - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following:

    - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook.

  - For Rel.18 eType1/eType2 DMRS ports,

    - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details).

    - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked.

  - FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group.

  - FFS: One or more than one DMRS port combination(s) for each rank and TPMI

  - Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17.

  - FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports.

Agreement

For length 2 TD-OCC (across consecutive DMRS symbols, if any) for DMRS of PDSCH/PUSCH for Rel.18 eType 1/2 DMRS, support Opt.1:

- Opt.1:

| TD-OCC index | Wt(0) | Wt(1) |
| --- | --- | --- |
| 0 | +1 | +1 |
| 1 | +1 | -1 |

Agreement

- For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured.

- Down select from the following in RAN1#112:

  - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2.

    - FFS: Association with the CW with the higher MCS.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW with the higher MCS |
| 1 | 2nd scheduled DMRS port the CW with the higher MCS |
| 2 | 3rd scheduled DMRS port the CW with the higher MCS |
| 3 | 4th scheduled DMRS port the CW with the higher MCS |

      - Alt.2: The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port |
| 4 | 5th scheduled DMRS port |
| 5 | 6th scheduled DMRS port |
| 6 | 7th scheduled DMRS port |
| 7 | 8th scheduled DMRS port |

Agreement

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

  - FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | [1] | [0] | Cat. 1 | [0] | [2] | [0,1,2,3,8] | [Rank 5-8 with one DMRS symbol] |
| 1 | [1] | [1] |  | [1] | [2] | [0,1,2,3,8,10] |  |
| 2 | [1] | [0,1] |  | [2] | [2] | [0,1,2,3,8,9,10] |  |
| 3 | 2 | 0 |  | [3] | [2] | [0,1,2,3,8,9,10,11] |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | [2] | [0-2] |  |  |  |  |  |
| 10 | [2] | [0-3] |  |  |  |  |  |
| 11 | [2] | [0,2] |  |  |  |  |  |
| 12 | [1] | [8] | Cat.2 |  |  |  |  |
| 13 | [1] | [9] |  |  |  |  |  |
| 14 | [1] | [8,9] |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | [2] | [8-10] |  |  |  |  |  |
| 22 | [2] | [8-11] |  |  |  |  |  |
| 23 | [2] | [8, 10], [9, 11] |  |  |  |  |  |
| 24 | [1] | [0,1,8] | Cat.3 |  |  |  |  |
| 25 | [1] | [0,1,8,9] |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

This contribution focuses on further discussion of increased number of orthogonal DMRS ports.

- Discussion

Based on the agreement in last meeting[1], we have 2 options on one port PTRS to be configured for full-coherent PUSCH with rank 5-8 as follows:

Agreement - For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured. - Down select from the following in RAN1#112: - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with the CW with the higher MCS. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS. - Alt.2: The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port. 1; 2nd scheduled DMRS port. 2; 3rd scheduled DMRS port. 3; 4th scheduled DMRS port. 4; 5th scheduled DMRS port. 5; 6th scheduled DMRS port. 6; 7th scheduled DMRS port. 7; 8th scheduled DMRS port.

Compared with Opt 1, Option 2 aligns with legacy design. It isn’t necessary to consider association with the CW with the higher MCS and complexity of implementation isn’t increased. So we support Option 2 as baseline.

Proposal 1: For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured. The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212.

In terms of  the agreement in last meeting[1], we have indication  table on Rel.18 DMRS ports for PDSCH as follows:

Agreement - For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case. Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes. 0; [1]; [0]; Cat. 1; [0]; [2]; [0,1,2,3,8]; [Rank 5-8 with one DMRS symbol]. 1; [1]; [1]; [1]; [2]; [0,1,2,3,8,10]. 2; [1]; [0,1]; [2]; [2]; [0,1,2,3,8,9,10]. 3; 2; 0; [3]; [2]; [0,1,2,3,8,9,10,11]. 4; 2; 1. 5; 2; 2. 6; 2; 3. 7; 2; 0,1. 8; 2; 2,3. 9; [2]; [0-2]. 10; [2]; [0-3]. 11; [2]; [0,2]. 12; [1]; [8]; Cat.2. 13; [1]; [9]. 14; [1]; [8,9]. 15; 2; 8. 16; 2; 9. 17; 2; 10. 18; 2; 11. 19; 2; 8,9. 20; 2; 10,11. 21; [2]; [8-10]. 22; [2]; [8-11]. 23; [2]; [8, 10], [9, 11]. 24; [1]; [0,1,8]; Cat.3. 25; [1]; [0,1,8,9]. 26; 2; 0,1,8. 27; 2; 0,1,8,9. 28; 2; 2,3,10. 29; 2; 2,3,10,11.

From technical perspective, above combinations on DMRS CMD group and DMRS ports are very flexible to cover different scenarios so we suggest removing square brackets to support all of combination in the table. M-TRP can reuse the table of S-TRP because current combinations on DMRS CMD group and DMRS ports in the table of S-TRP can cover M-TRP case.

Proposal 2: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, below table is used for S-TRP and M-TRP case.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | 1 | 0 | Cat. 1 | 0 | 2 | 0,1,2,3,8 | Rank 5-8 with one DMRS symbol |
| 1 | 1 | 1 |  | 1 | 2 | 0,1,2,3,8,10 |  |
| 2 | 1 | 0,1 |  | 2 | 2 | 0,1,2,3,8,9,10 |  |
| 3 | 2 | 0 |  | 3 | 2 | 0,1,2,3,8,9,10,11 |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | 2 | 0-2 |  |  |  |  |  |
| 10 | 2 | 0-3 |  |  |  |  |  |
| 11 | 2 | 0,2 |  |  |  |  |  |
| 12 | 1 | 8 | Cat.2 |  |  |  |  |
| 13 | 1 | 9 |  |  |  |  |  |
| 14 | 1 | 8,9 |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | 2 | 8-10 |  |  |  |  |  |
| 22 | 2 | 8-11 |  |  |  |  |  |
| 23 | 2 | 8, 10 |  |  |  |  |  |
| 24 | 1 | 0,1,8 | Cat.3 |  |  |  |  |
| 25 | 1 | 0,1,8,9 |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

Conclusions

In this contribution, we discussed about increased number of orthogonal DMRS ports. The proposals are as follows:

Proposal 1: For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured. The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212.

Proposal 2: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, below table is used for S-TRP and M-TRP case.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | 1 | 0 | Cat. 1 | 0 | 2 | 0,1,2,3,8 | Rank 5-8 with one DMRS symbol |
| 1 | 1 | 1 |  | 1 | 2 | 0,1,2,3,8,10 |  |
| 2 | 1 | 0,1 |  | 2 | 2 | 0,1,2,3,8,9,10 |  |
| 3 | 2 | 0 |  | 3 | 2 | 0,1,2,3,8,9,10,11 |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | 2 | 0-2 |  |  |  |  |  |
| 10 | 2 | 0-3 |  |  |  |  |  |
| 11 | 2 | 0,2 |  |  |  |  |  |
| 12 | 1 | 8 | Cat.2 |  |  |  |  |
| 13 | 1 | 9 |  |  |  |  |  |
| 14 | 1 | 8,9 |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | 2 | 8-10 |  |  |  |  |  |
| 22 | 2 | 8-11 |  |  |  |  |  |
| 23 | 2 | 8, 10 |  |  |  |  |  |
| 24 | 1 | 0,1,8 | Cat.3 |  |  |  |  |
| 25 | 1 | 0,1,8,9 |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

- Reference

- Chairman’s Notes, RAN1#111, final, November 14th – 18th, 2022

- R1-2212760, FL summary on DMRS#3, Moderator (NTT DOCOMO)