3GPP TSG-RAN WG1 Meeting #112	Tdoc R1-2300908

Athens, Greece, 27th February – 3rd March, 2023

Agenda Item:	9.3.2

Source:	Ericsson

Title:	Sub-band non-overlapping full duplex

Document for:	Discussion, Decision

# 1	Introduction

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

In this paper we treat the highlighted parts of following SID objective:

In the first part of the paper we discuss SBFD solutions and their impact on legacy operation. It is expected that when SBFD is deployed, both legacy UEs and SBFD aware UEs would co-exist in the same cell for the foreseeable future. Hence, it is important to study how SBFD can be deployed in a way that would satisfy both of the following goals jointly:

- Enable benefits for SBFD aware UEs in terms of improved UL coverage and/or improved UL latency, and

- Allow legacy UEs to operate in a cell where SBFD is deployed in a transparent manner.

We discuss various aspects of a baseline SBFD solution that satisfies these goals jointly. We focus mainly on the configuration and operation of a serving cell with SBFD from both the perspective of legacy UEs and SBFD aware UEs.

  - 9.3	Study on evolution of NR duplex operation

Please refer to RP-222110 for detailed scope of the SI.

    - 9.3.2	Subband non-overlapping full duplex

Including study on possible solutions, feasibility, and impact to legacy operation assuming co-existence in co-channel and adjacent channels.

According to the agenda for RAN1#111, feasibility of sub-band full duplex (SBFD) should be discussed under AI 9.3.2:

Thus, in the second part of this paper we provide a feasibility analysis of SBFD, focusing on the feasibility of self-interference mitigation for both FR1 and FR2.

# 2	SBFD Solutions Including Impact to Legacy Operation

It is expected that when subband full duplex (SBFD) is deployed, both legacy UEs and SBFD aware UEs would co-exist in the same cell for the foreseeable future. Hence, it is important to study how SBFD can be deployed in a way that would satisfy both of the following goals jointly:

- Enable benefits for SBFD capable UEs in terms of improved UL coverage and/or improved UL latency

- For convenience of presentation, we refer to "SBFD aware UEs" as "Rel-18+ UEs" where the ‘+’ indicates that normative work on SBFD may occur in a later release.

- Allow legacy UEs (Rel-17 and prior) to operate in a cell where SBFD is deployed without mandating that new Rel-18+ features related to SBFD are implemented in the UE.

In this part of this paper, we discuss various aspects of a baseline SBFD solution that satisfies these goals jointly. We focus on configuration and operation of a serving cell with SBFD from both a Rel-18+ and legacy UE perspective. In the discussion we assume that the baseline solution comprises Alt-4 described in the below agreements. Indeed, Alt-4 was agreed as baseline in RAN1#110bis-e. Hence legacy UEs should operate according to existing specifications, and new UE behaviors can be introduced for Rel-18+ UEs. In this paper we discuss such new UE behaviors, especially in the area of resource allocation.

Agreement

Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state.

- SBFD operation Alt 1:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side.

- SBFD operation Alt 2:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs

- SBFD operation Alt 3:

  - Only time location of subbands for SBFD operation is known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

UE capability discussion is held in work item phase.

Agreement

For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline.

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Throughout this paper, we assume that an UL subband can be configured in symbols indicated as 'D' or ‘F’ by TDD-UL-DL-ConfigCommon, e.g., with frequency domain pattern D-U-D. We do not consider configuration of a DL subband in symbols indicated as ‘U’. The rationale is the following:

- The premise for SBFD operation is that it enables improved UL coverage and/or latency by enabling more UL opportunities compared to the baseline time domain TDD pattern. Hence, it is not motivated to configure symbols configured as 'U' by TDD-UL-DL-ConfigCommon for SBFD operation, e.g., with frequency domain pattern U-D-U.

- SBFD operation in symbols configured as 'U' by TDD-UL-DL-ConfigCommon can create strong gNB-gNB cross-link interference to a legacy (static TDD) gNB configured with the same baseline time domain TDD pattern, hence such a configuration should be avoided.

Do not support SBFD operation in symbols configured as 'U' by TDD-UL-DL-ConfigCommon.

## 2.1	Carrier and BWP Configuration

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Agreement

For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

Agreement

For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline.

Agreement

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

Agreement

For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required.

- FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

With the above baseline SBFD solution in mind, we note the following prior agreements relating to carrier configuration.

These agreements say that semi-static configuration of the subband time and frequency locations is the baseline, and that the subband configuration across different SBFD symbols is the same. Furthermore, subband time locations are configured within a period, and explicit configuration of the frequency location of at least the UL subband is required. We first discuss the case of an UL subband in symbols configured as 'D' by TDD-UL-DL-ConfigCommon. We view this as the baseline, since configuration of symbols with a pre-defined direction is the norm for most, if not all commercial TDD deployments. Following this, we discuss the case of an UL subband in symbols configured as 'F' by TDD-UL-DL-ConfigCommon.

### 2.1.1	UL Subband in Symbols Configured as 'D' by TDD-UL-DL-ConfigCommon

Figure 1 illustrates a carrier configured for SBFD operation viewed from a system perspective, e.g., with time domain pattern D-D-D-D-U indicated by TDD-UL-DL-ConfigCommon.

Figure 1: Time/frequency domain pattern for SBFD from a system perspective where an UL subband is configured in symbols configured as ‘D’ by TDD-UL-DL-ConfigCommon.

From a UE perspective, a 'D' slot in the above time domain pattern would appear as shown in Error! Reference source not found.. A legacy UE would see the slot as configured for DL reception only, but the gNB would need to avoid scheduling/configuring DL transmission in the middle RBs for this UE. A legacy UE would be unaware that these RBs can be used for UL by Rel-18+ UEs. This puts constraints on the configuration and scheduling of DL for legacy UEs, e.g., some signals/channels need to be scheduled/configured in only one of the 'D' subbands if non-contiguous frequency domain resource allocation (FDRA) is not supported.

Figure 2: Time/frequency domain pattern from a UE perspective (legacy and Rel-18+ UEs) within one of the 'D' slots.

For Rel-18+ UEs, the UE understands that a 'D' slot in the above pattern can be used for either DL reception or UL transmission (but not simultaneously). While the gNB would still need to avoid scheduling/configuring DL in the middle RBs like for legacy UEs, new behavior for certain signals/channels can be specified for making use of both 'D' subbands through enhancements to the FDRA procedures.

For Rel-18+ UEs, i.e., UEs that are SBFD-aware, the following agreement regarding ‘D’ symbols was made last meeting:

Agreement

For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

  - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

- DL receptions within DL subband(s) are allowed in the symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

For example, in a ‘D’ symbol configured with an UL subband, this symbol can only be used as D-U-D (or D-U, if configured that way). In other words, the direction of each subband is semi-static, and cannot be changed without an RRC re-configuration.

### 2.1.2	UL Subband in Symbols Configured as 'F' by TDD-UL-DL-ConfigCommon

In prior meetings, there has been a lot of discussion about allowing an UL subband to be configured in symbols configured as 'F' in TDD-UL-DL-ConfigCommon, e.g., as shown in Error! Reference source not found..

Figure 3: Time/frequency domain pattern for SBFD from a system perspective where an UL subband is configured in symbols configured as ‘F’ by TDD-UL-DL-ConfigCommon.

The motivation for this is so that legacy UEs (those that actually support ‘F’ symbols) can be scheduled/configured UL in an SBFD symbols, at least within the RBs occupied by the UL subband indicated to Rel-18+ UEs. This can be done in a transparent manner, i.e., the legacy UEs do not need to be aware that an UL subband is configured for Rel-18+ UEs.

Figure 4: Time/frequency domain pattern from a UE perspective (legacy and Rel-18+ UEs) within one of the 'F' slots.

Agreement

For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs,

Option 1:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

- FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol

Option 2:

- UL transmissions within UL subband are allowed in the symbol

- The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same

  - FFS: SBFD aware UE behaviours

  - FFS: Whether or not signalling of guardband(s) is needed

- FFS: Whether or not the symbol can be converted to a DL-only symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

The following agreement was made last meeting regarding SBFD operation in symbols configured as ‘F’:

In this agreement, Option 1 (without the FFS bullet) is harmonized to the agreement shown in the previous section for symbols configured as ‘D’ by TDD-UL-DL-ConfigCommon. What Option 1 offers on top is that not only Rel-18+ (SBFD aware) UEs can be scheduled in an UL subband; legacy UEs can be scheduled too. This offers legacy UEs more UL opportunities since the symbol is configured as ‘F’ and can thus be dynamically indicated as ‘U’ for those UEs. We have a strong preference that Option 1 is the baseline, due to the harmonized operation with ‘D’ symbols. We think that Option 2 is extremely complicated, and as we will show through evaluation later in this section it does not appear to bring benefits compared to operating the system in a dynamic TDD fashion, i.e., without subbands.

RAN1 to conclude that Option 1 is the baseline, and Option 2 is not further pursued.

For Option 1, there is an FFS on whether to allow DL receptions outside the DL subband(s) (i.e., within the UL subband). However, we think that this defies the principle of configuring an UL subband in the first place. If an UL subband is configured, then it should be used only for UL transmissions. This is the same principle as configuring a symbol as either 'D' or 'U' in TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated. Such symbols are only used for downlink or only used for uplink, respectively. If it is desired to use the UL subband for DL, then the symbol can always be re-configured semi-statically as 'D', i.e., without an UL subband. Alternatively, if it is desired to have the flexibility to determine the direction dynamically (as in dynamic TDD), then the symbol can be configured as 'F' using existing specifications. In fact, below we show that there is no performance advantage in allowing DL receptions outside the DL subband(s).

For Option 1, only UL transmissions are allowed within an UL subband. In other words, DL receptions outside DL subband(s) are not allowed.

Option 2 leads to very dynamic operation, and because of this we call it “dynamic SBFD” or dSBFD. According to the agreement including the FFS on conversion to a DL-only symbol, the gNB can dynamically select between the following three ‘modes’ for a symbol configured as ‘F’:

- Full UL, i.e., UL-only

- Full DL, i.e., DL-only

- Mixed direction, i.e., SBFD operation with D-U-D configuration

Some companies have proposed Option 2 with the motivation that a static partition of the carrier into ‘D’ and ‘U’ subbands can lead to unused resources depending on the traffic pattern, particularly in the DL. The hope is that by allowing a symbol to be more flexible, this wastage can be avoided. However, as we will show, this kind of dynamic operation can be very well supported through already existing mechanisms, i.e., dynamic TDD (dTDD), which dynamically selects between 1. and 2. only. As we show below, the ability to also dynamically select a mixed direction slot, i.e., 3., does not bring tangible benefits in terms of performance.

To demonstrate this, we evaluate the following five systems, all with a protected UL-only slot as the 5th slot in the pattern that is free of CLI. This is to ensure that essential UL control channels carrying HARQ-ACK for operation of the DL can be reliably detected.

- Baseline: Static TDD (sTDD)

  - Fixed D-D-D-D-U pattern

- Static subband full duplex (SBFD)

  - X-X-X-X-U pattern where X = mixed direction slot with D-U-D configuration

- Dynamic SBFD (dSBFD, Option 1)

  - Dynamic selection between modes 2., and 3 in the first four slots

    - Selection performed per slot based on available traffic in the buffers

  - This is Option 1 in the above agreement including the FFS bullet on allowing DL outside DL subbands

- Dynamic SBFD (dSBFD, Option 2)

  - Dynamic selection between modes 1., 2., and 3 in first four slots

    - Selection performed per slot based on available traffic in the buffers

  - This is Option 2 in the above agreement including the FFS bullet on conversion to DL-only symbol

- Dynamic TDD (dTDD)

  - Dynamic selection between modes 1. and 2. in first four slots

    - Selection performed per slot based on available traffic in the buffers

  - No subbands are configured

We consider a simplified, yet illustrative, scenario in which no guard symbols are configured, and no symbols need to be reserved as ‘D’ for the DL control channel (PDCCH) that schedules both DL and UL traffic channels. These simplifications demonstrate the theoretical potential of dynamic operation (dSBFD Options 1/2 and dTDD). Our primary interest is to determine the relative performance differences between these dynamic modes of operation, so in that sense, the simplifying assumptions do not matter.

The evaluation scenario is Urban Macro with single operator (Case 1) – please see Appendix A for more details on the simulation assumptions. For the sTDD and SBFD systems (static systems), all sectors of all sites have the same pattern. In contrast, for dSBFD Options 1/2 and dTDD (dynamic systems), there is no co-ordination of patterns between sectors/sites other than the protected UL slot which is used in all sectors/sites.

Figure 5 shows the cell-edge (5th percentile) user throughput obtained from system simulation of all 5 systems at 3 load levels: low, medium, and high. This metric is important since it illustrates whether or not UL coverage improvement can be obtained for a given system compared to the baseline static TDD system.

Figure 5: Cell-edge user throughput (DL and UL) for Dynamic SBFD (Options 1 and 2) vs. Dynamic TDD

In our evaluations, we find that at low load it is rare that there is both DL and UL traffic in the buffers of the scheduled user at the same time. Hence, for the dynamic systems (dSBFD Options 1/2 and dTDD), the scheduler most often chooses the transmission direction as ‘D’ or ‘U’. As the load increases, however, it is more likely that there is DL and UL traffic in the buffers simultaneously. Hence, for the dynamic systems, the scheduler needs to make a decision on which direction to impose on the slot. For dTDD ‘D’ is chosen if there is simultaneous DL and UL traffic. For dSBFD Options 1/2, it is natural to choose the slot as mixed direction (D-U-D). Additionally, we find that at medium and high load most of the UL traffic is steered to the protected UL-only slot (5th slot in the pattern). This is due to worsening gNB-gNB CLI in the first 4 slots as the load increases making the UL SINR poor in these slots in contrast to the “clean” protected slot.

We make the following observations from the cell-edge throughput results in Figure 5:

- Low load

  - DL Performance

    - The dynamic systems, dSBFD Options 1/2 and dTDD. all offer improved DL throughput compared to (static) SBFD, but still perform no better than the baseline static TDD. This was the motivation for some companies for investigating dSBFD as a way to avoid a loss in DL performance that occurs for static SBFD due to a static partition of resources where the UL resources may go wasted if there is DL traffic, but no UL traffic to serve. Clearly, the intended goal is achieved in the DL; however, dynamic TDD still outperforms dSBFD Options 1/2.

  - UL Performance

    - The dynamic systems, dSBFD Options 1/2 and dTDD, do not perform as well in the UL as (static) SBFD due to less controlled gNB-gNB CLI which negatively affects the first 4 slots. Despite this, all 4 systems perform better than the baseline static TDD system, illustrating a coverage benefit.

    - While dSBFD Options 1/2 have the ability to select a mixed direction (D-U-D) in addition to selecting D (Option 1) or between D and U (Option 2), it can be seen that this ability brings little benefit; dTDD performs almost as well as dSBFD in the UL. The reason for this is that at low load it is rare that there is both DL and UL traffic in the buffers of the scheduled user at the same time; hence it is sufficient to select only between U and D directions as is done in the dTDD system.

- Medium/High Load

  - DL Performance

    - dTDD achieves the same performance as the baseline static TDD system due to the fact that most of the UL traffic is “steered” to the protected UL only slot, and the first 4 slots effectively become DL-only slots most of the time. In other words the “effective” pattern is most often the same as the D-D-D-D-U static TDD pattern.

    - In contrast, both SBFD and dSBFD Options 1/2 exhibit significant losses compare to the baseline static TDD system due to a deficiency in DL resources since the first 4 slots effectively become mixed direction D-U-D slots. dSBFD achieves roughly the same performance as (static) SBFD since the two systems end up having the same “effective” pattern most of the time.

  - UL Performance

    - Since the most of the UL traffic is “steered” to the protected UL-only slot, all systems end up with roughly the same performance in the UL

To obtain additional insight, Figure 6 shows the median (50th percentile) user throughput. Similar performance trends as described above occur, except that for the UL at low load, the dynamic systems (dSBFD Options 1/2 and dTDD) have less loss vs. (static) SBFD than for the cell-edge users. This is because the gNB-gNB CLI is less dominant for users achieving the median (50th percentile) throughput compared to the cell-edge users.

Figure 6: Median user throughput (DL and UL) for Dynamic SBFD (Options 1 and 2) vs. Dynamic TDD

In summary, we make the following observation

Dynamic SBFD (dSBFD) Options 1 and 2 do not offer a performance advantage compared to the considerably simpler Dynamic TDD (dTDD) system.

For this reason we think that it is not necessary to further pursue Option 2 or the FFS bullet in Option 1 in the agreement from last meeting as we state in Proposal 2 and Proposal 3. If one wants to operate a  system in a dynamic fashion, then conventional  dynamic TDD is sufficient. There is no need to incur the additional complexity from dynamic SBFD.

As a final note, we observe that both the agreement from last meeting for ‘D’ symbols (without the FFS bullet) and Option 1 of the agreement from last meeting for ‘F’ symbols, map to Option 1 in the following agreement from RAN1#109. Hence we see no need to further pursue Options 2, 3, and 4.

Agreement

For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol

- Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

RAN1 to conclude that Options 2, 3, and 4 in the RAN1#109 agreement are no longer pursued.

### 2.1.3 Configuration of Subband Time Locations

We note that Figure 1 is in-line with the prior agreement that says that subband time locations are semi-statically configured within a period. In our view it is natural that the period is defined by the parameter dl-UL-Transmiossionperiodicity within the TDD-UL-DL-ConfigCommon IE:

TDD-UL-DL-ConfigCommon ::=          SEQUENCE {

referenceSubcarrierSpacing          SubcarrierSpacing,

pattern1                            TDD-UL-DL-Pattern,

pattern2                            TDD-UL-DL-Pattern                             OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::=               SEQUENCE {

dl-UL-TransmissionPeriodicity       ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10},

nrofDownlinkSlots                   INTEGER (0..maxNrofSlots),

nrofDownlinkSymbols                 INTEGER (0..maxNrofSymbols-1),

nrofUplinkSlots                     INTEGER (0..maxNrofSlots),

nrofUplinkSymbols                   INTEGER (0..maxNrofSymbols-1),

...,

[[

dl-UL-TransmissionPeriodicity-v1530     ENUMERATED {ms3, ms4}                      OPTIONAL -- Need R

]]

}

Assuming 30 kHz SCS, for example, the period Figure 1 is 2.5 ms, i.e., dl-UL-Transmiossionperiodicity = ms2p5.

For semi-static configuration of SBFD subband time locations within a period, the periodicity is configured by the existing DL-UL transmission periodicity parameter defined in TDD-UL-DL-ConfigCommon

According to prior agreements, explicit indication of the frequency location of at least the UL subband is required. Such configuration is only needed for Rel-18+ UEs, and this can be handled by introducing new signaling to indicate frequency domain (FD) information. The signaling can be cell-common, since the most common mode of operation would be that the subband locations are the same for all UEs. If there is a need to have slightly different guardband sizes for different UEs, then dedicated signaling could be introduced to indicate the frequency domain information. This could be new signaling or enhancement of existing signaling, e.g., extension of TDD-DL-UL-ConfigDedicated.

### 2.1.4 Configuration of Subband Frequency Locations

One approach for indicating the FD information for Rel-18+ UEs is to introduce the notion of RB sets with guard bands in between to allow for filter rolloff at the gNB and/or UE. Such RB sets and guardbands are very similar to what was specified in Rel-16 for NR-U (see 38.214 Clause 7); however, more flexibility is needed to adjust the size of the RB sets for SBFD operation. Figure 7 shows two options for achieving such configuration. In Option 1, the starting RB and number of RBs is provided for each RB set as well as the transmission direction 'D' or 'U'. The guardband sizes can be implicitly determined as whatever RBs are not indicated to be part of an RB set. In Option 2, it is recognized that it is only necessary to indicate the size and position of the 'U' RB set and the size of the guardbands. The remaining RBs are then implicitly determined to be 'D' RB sets. Option 2 is probably more efficient from an overhead point of view. Hence we propose the following which covers both a D-U-D configuration and a D-U or U-D configuration.

Within an SBFD symbol, a single UL subband is configured as an RB set, and one or two guardband sizes are configured in terms of a number of RBs. The guardband RBs are contiguous and adjacent to the UL RB set.

Figure 7: Options for RB set and guard band configuration in SBFD symbols.

Figure 8 shows an example where TDD-UL-DL-ConfigCommon configures the time domain pattern for all UEs as D-D-D-D-U with 2 symbol guard for DL-UL transition. The diagram also illustrates the indication of the frequency domain pattern (RB sets) for Rel-18+ UEs.

Figure 8: Exemplary carrier configuration in which TDD-UL-DL-ConfigCommon configures the underlying time domain pattern for all UEs, and new/extended signaling configures the frequency domain pattern (RB sets) for Rel-18+ UEs within the 'D' slots/symbols.

### 2.1.5	Dynamic Indication of UL Subband Size / Frequency Location

We do not see a motivation for dynamic indication of the UL subband size and/or frequency location since this is something decided at deployment time, or at most would be something that would be updated on a slow basis if deployment conditions change.

From base station implementation point of view, dynamic indication is undesirable since it would imply frequent re-configuration of analog filters with different passband bandwidths which is not attractive from an implementation point of view. Depending on the transmit power class of the gNB, such analog filters are needed in SBFD symbols to suppress the DL subbands in order to avoid saturating the LNA and/or ADC in the UL receive chain of the gNB. If the UL subband size or frequency location is changed, then a different analog filter is needed. This is problematic considering modern gNB architectures that are required to handle multiple bands and multiple carriers within a band where SBFD can be configured on an arbitrary combination of carriers. The number of switchable analog filters becomes prohibitive, and is not even feasible in particular receiver architectures that are used increasingly in the industry, e.g., direct RF sampling receivers (see Section 3.8).

From UE implementation point of view, dynamic indication is equally undesirable or infeasible. As discussed in Section 2.4 below, many UL physical signal transmissions (for example PUCCH frequency hopping) have strong dependence on knowing the UL subband size. As the Rel-15 NR latency discussion and the difficulty of supporting out-of-order UL transmissions revealed, it takes time for the UE to prepare the UL PDU and UL physical signals. It is unrealistic to interrupt UE preparation of UL signals if a new subband size/frequency location is dynamically signaled from the network.

Dynamic indication of the size and/or frequency location of the UL subband in SBFD symbols is not supported.

### 2.1.6	BWP Configuration

Agreement

For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

In RAN1#110bis-e, the following agreement was made regarding BWP configuration

We support this agreement since it is in-line with current specifications and seems sufficient for SBFD operation. Further, we don't believe that there is a need to further discuss potentially misaligned center frequencies as it doesn't seem to bring any benefits and would likely negatively impact UE complexity.

For new (Rel-18+) UEs supporting SBFD operation within a TDD carrier, do not support misaligned center frequencies between DL and UL BWPs.

## 2.2	UL/DL Time Alignment Aspects

In NR, the UE determines the total timing advance (TA) value from the sum of (1) a UE-specific timing advance command (absolute or relative), and (2) a cell-specific timing offset defined as NTA,offset∙Tc. For the case of a TDD carrier in FR1 (not co-existing with LTE), the default value of NTA,offset is defined in Table 7.1.2-2 in 38.133 as 25,600. Using Tc defined in 38.211, the resulting cell-specific timing offset is approximately 13 µs. Comparing this to the OFDM symbol duration for 30 kHz SCS, the offset is roughly 1/3 of an OFDM symbol. Since the UE-specific TA is meant to compensate for the two-way propagation delay from gNB-to-UE, the UL and DL frame timing observed at the gNB ends up being misaligned by the cell-specific offset NTA,offset∙Tc. The purpose of this misalignment is to facilitate a time gap for the gNB when switching from receiving UL to transmitting DL. This timing relationship is illustrated in Figure 9.

Figure 9: UL/DL timing from gNB perspective showing misalignment by NTA,offset∙Tc. The UL receiver FFT window is assumed to be placed 1/2 way through CP.

In legacy NR operation, the gNB does not transmit and receive at the same time, hence the timing misalignment does not affect UL performance. However, for SBFD operation where the UE simultaneously transmits and receives, the UL-DL timing misalignment can have an impact, depending on the scenario. Several companies have raised this issue in prior meetings in both RAN1 and RAN4 (e.g., [4], [5], [6], [7], [8]). Furthermore, RAN4 has agreed

R4-2217464 Section 4:

Agreement:

- For the BS self-interference issue related to timing and SCS of D and U for both BS and UE:

  - RAN4 understanding is that this issue will be studied in RAN1.

  - RAN4 has not evaluated the timing and SCS impacts to BS SI. RAN4 for now assumes they are negligible and do not impact RAN4 requirement work.

  - RAN4 will not consider this issue in the BS SI feasibility study if no request from RAN1.

that RAN1 shall study this issue:

The effect is summarized as follows. Since the timing offset is a significant fraction of the OFDM symbol (roughly 1/3 for 30 kHz SCS), and since the UL receiver FFT window is typically placed somewhere near the midpoint of the CP to give margin for delay spread, the Rx FFT window ends up spanning the boundary between two OFDM symbols of the simultaneous DL transmission (see illustration in Figure 9). Since the simultaneous DL transmission leaks into the UL receiver (self-interference), the UL Rx FFT window ends up capturing the discontinuity, causing a spectrum re-growth effect.

This is illustrated by the red curve in Figure 10 which shows the self-interference power in the Rx FFT window for the case of timing misalignment by 13 µs. The reference blue curve is for the case of no misalignment where the Rx FFT window does not cross a symbol boundary. In this graph, we consider the case of no transmit side impairments, i.e., a perfectly linear PA in the transmitter. Clearly the misalignment creates a dramatic rise in interference received in the 20 MHz UL subband between the two 40 MHz DL subbands compared to the case of no misalignment.

Figure 10: Power in Rx FFT window with no Tx impairments and no UL subband selection filter

A similar evaluation is performed in [8] demonstrating the effect of timing misalignment leading to elevated interference in the UL subband. Based on the intuition that timing misalignment causes such a degradation, several companies have suggested studying schemes whereby the UE alters its behavior such that it applies zero cell-specific timing offset during SBFD slots, i.e., it assumes NTA,offset=0, and then applies the legacy timing offset, e.g., NTA,offset=25,600, during legacy UL-only slots. It goes without saying that such altered behavior would only be possible for Rel-18+ UEs. If an UL subband is configured in symbols indicated as ‘F’ by TDD-UL-DL-ConfigCommon, thus allowing legacy UEs to be scheduled in the UL subband of these symbols, then the UL reception for such UEs could be compromised by the elevated self-interference if the gNB schedules simultaneous DL (to another UE) in these symbols.

Setting aside the legacy UE issue for now, it is important to further study whether there is merit to using the adaptive NTA,offset scheme described above. To this end, we have done further evaluations to investigate the impact of misalignment when two practical aspects are considered. The first aspect is appropriate modeling transmit impairments due to residual non-linearities in the transmit chain. The second aspect is modeling of a UL subband selection filter used in the gNB receive chain which acts to suppress the interference from the DL subbands. Such filtering may occur in the analog domain, or the digital domain prior to the FFT, or both.

In Figure 11 we show the power of the self-interference in the Rx FFT window in the presence of Tx side impairments that modify the spectrum of the DL transmit signal. The impairments are captured by modelling of the net effect of crest factor reduction (CFR) + digital pre-distortion (DPD) + non-linear power amplifier (PA) used in each transmit chain. We describe this net effect model in in Section 2.1 of our companion contribution [1]. The key aspect is that the net effect of CFR, DPD, and PA results in adjacent channel leakage levels that satisfy the RAN4 requirement of 45 dB ACLR. In contrast to Figure 10, one can see a very much reduced gap (now only 10-15 dB) between the red curve (misalignment case) and the reference blue curve (no misalignment) within the 20 MHz UL subband. Clearly the Tx side impairments partially mask the misalignment effect.

Figure 11: Power in Rx FFT window with Tx impairments, but no UL subband selection filter

Finally, in Figure 12, we show the power of the self-interference in the Rx FFT window in the presence of both Tx side impairments and an UL subband selection filter. The power spectra in this figure have been smoothened using a sliding average of 12 subcarriers to make the comparatively small differences between curves clearer. The UL subband filter has a passband roughly equal to the UL subband bandwidth with a stop band attenuation of 25 dB that partially suppresses the DL subbands. Compared to Figure 11, one can see a very much reduced gap (now 1 dB on average) between the red curve (misalignment case) and the reference blue curve (no misalignment). Clearly the combined effect of Tx side impairments and UL subband selection filter come very close to fully masking the misalignment effect.

Figure 12: Power in Rx FFT window with Tx impairments and with UL subband selection filter

Based on the above discussion and results we propose the following regarding further study in RAN1:

RAN1 should model practical transmitter and receiver components to enable a realistic assessment of the effect of UL/DL timing misalignment at the gNB.

## 2.3	DL Signals and Channels

In this section we discuss configuration/scheduling of DL signals and channels for both legacy and Rel-18+ UEs. We discuss CONNECTED and IDLE modes separately.

### 2.3.1	IDLE Mode

In the baseline SBFD solution, we assume that in IDLE mode, all UEs (legacy and Rel-18+) are not aware that certain slots/symbols are used for SBFD operation. Such information is only made available to Rel-18+ UEs by dedicated signaling once the UE enters CONNECTED mode.

UEs in IDLE mode are not aware of whether or not symbols/slots are used for SBFD operation.

We now analyze the feasibility of this proposal. During initial access, the UE receives the following signals/channels, all of which are restricted in current specifications to have contiguous frequency domain resource allocation (FDRA):

- SS/PBCH Block

- PDCCH in Type0-CSS

- RMSI PDSCH

Hence, in an SBFD system, the gNB must ensure that these signals "fit" within one of the 'D' subbands in SBFD slots/symbols. This puts a constraint on the maximum size of the 'U' subband for a given carrier bandwidth. The constraint is determined by investigating the bandwidth requirements of CORESET0 for various configurations. Here we analyse the maximum size of the 'U' RB set under the following assumptions:

- Symmetric D-U-D allocation ('D' subbands have equal bandwidth)

- Same subcarrier spacing for SS/PBCH block and CORESET0

  - 15 or 30 kHz SCS for FR1

  - 120 kHz SCS for FR2

- Guard bands between RB sets

  - FR1: 10/5 RBs for 15/30 kHz SCS

  - FR2: 3 RBs for 120 kHz SCS

Table 1 shows the maximum size of the UL RB set as a percentage of the total number of RBs in the transmission bandwidth under the constraint that both SSB and CORESET0 must fit into one of the 'D' subbands. The table can be summarized as follows, where "feasible" means that the maximum UL allocation corresponds to greater than 20% of the total number RBs:

- FR1 with 15 kHz SCS:

  - 40 MHz channel bandwidth is sufficient for feasibility of 24 and 48 RB CORESET0, but not 96 RB

- FR1 with 30 kHz SCS:

  - At least 100 MHz channel bandwidth is required for feasibility of 48 RB CORESET0

  - 40 MHz bandwidth is sufficient for feasibility of 24 RB CORESET0

- FR2 with 120 kHz SCS:

  - At least 200 MHz channel bandwidth is required for feasibility of both 48 and 24 RB CORESET0

    - For 48 RB CORESET0 with Mux Pattern 1, the UL subband can be no larger than about 20%

  - At least 400 MHz channel bandwidth is required for feasibility of 48 RB CORESET0 + Mux Pattern 2

Table 1: Maximum % of RBs that can be configured for 'U' RB set for FR1 and FR2

| Frequency Range | SCS | Channel BW | Transmission BW (RBs) | Mux Pattern | CORESET0 Size (RBs) | Maximum UL Allocation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 kHz | 40 MHz | 216 | 1 | 24 | 68% |
|  |  |  |  |  | 48 | 45% |
|  |  |  |  |  | 96 | 1% |
|  | 30 kHz | 40 MHz | 106 | 1 | 24 | 43% |
|  |  |  |  |  | 48 | 0% |
|  |  | 100 MHz | 273 | 1 | 24 | 78% |
|  |  |  |  |  | 48 | 60% |
| 2 | 120 kHz | 100 MHz | 66 | 1 | 24 | 15% |
|  |  |  |  |  | 48 | 0% |
|  |  |  |  | 2 | 24 | 0% |
|  |  |  |  |  | 48 | 0% |
|  |  | 200 MHz | 132 | 1 | 24 | 58% |
|  |  |  |  |  | 48 | 21% |
|  |  |  |  | 2 | 24 | 27% |
|  |  |  |  |  | 48 | 0% |
|  |  | 400 MHz | 264 | 1 | 24 | 79% |
|  |  |  |  |  | 48 | 61% |
|  |  |  |  | 2 | 24 | 64% |
|  |  |  |  |  | 48 | 45% |

For both legacy and new (Rel-18+) UEs in IDLE mode, it is feasible that DL reception  (i.e., during initial access) can occur in only one of the 'D' subbands within SBFD slots/symbols configured as D-U-D. By "feasible," it is meant that assuming a certain minimum channel bandwidth, CORESET0 "fits" within a single 'D' subband in a symmetric D-U-D configuration in either FR1 or FR2. For 48 RB CORESET0, at least 40/100/200 MHz channel bandwidth is required for 15/30/120 kHz SCS.

### 2.3.2	CONNECTED Mode

#### Legacy UEs

A legacy UE sees the whole carrier bandwidth as either 'D' or 'U' based on the cell-specifically indicated time domain TDD pattern TDD-UL-DL-ConfigCommon. In CONNECTED mode, if the active DL BWP spans the whole carrier, both 'D' subbands can be used for certain DL signals/channels that support non-contiguous frequency domain resource allocation (FDRA). These include the following:

- PDSCH scheduled by DCI 1_1 and DCI 2_1

  - DCI 1_1 and DCI 2_1 support Type0 FDRA in which a bitmap indicates the scheduled RBGs which can be non-contiguous

- PDCCH in a search space associated with a CORESET other than CORESET0

  - The RRC parameter frequencyDomainResources within a CORESET configuration is a bitmap used to indicate groups of 6 contiguous RBs, and the groups can be non-contiguous

For other signals/channels that support only contiguous FDRA then only one of the two 'D' subbands within the DL BWP can be used for legacy UEs.  These include the following

- SS/PBCH block

- PDSCH scheduled by DCI 1_0

  - DCI 1_0 only supports Type1 FDRA in which the start PRB and number of contiguous RBs are indicated

- CSI-RS

For legacy UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets, while reception of others is restricted to a single 'D' RB set.

#### Rel-18+ UEs

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Agreement

Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

Agreement

Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

Agreement

Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following:

- RBG for PDSCH RA type 0

- CSI reporting configuration

- CSI-RS resource configuration

- PRG of PDSCH

In contrast to legacy UEs, if a Rel-18+ UE has an active DL BWP in CONNECTED mode than spans the whole carrier, then it is within scope of the SI to consider enhancements for certain signals/channels to support non-contiguous FDRA according to the following agreements:

In current specifications, CSI-RS is limited to only contiguous FDRA. To enable non-contiguous FDRA, and thus make better use of both 'D' subbands, several possible enhancements exist.

One solution is to make use of the RRC configuration of RB sets as an implicit indication of RBs that are not available for DL transmission. This is illustrated in Figure 7 where based on either option, the 'U' RB set and guardbands are not available. Thus, a Rel-18+ UE can be configured with a CSI-RS with contiguous FDRA (as in current specifications) that spans all RB sets of the carrier. The UE would understand that the RBs that intersect with the 'U' RB set and the guardbands are not available. This effectively results in a CSI-RS resource  with non-contiguous allocation in both of the 'D' RB sets. Similar handling is specified for Rel-16 NR-U with respect to RB set availability, but the Rel-16 behavior is to drop the whole CSI-RS if any RB set is unavailable (due to failed LBT). For SBFD, the UE would only drop the RBs that overlap the unavailable RBs, rather than the whole resource.

Another solution is to extend the current CSI-RS resource definition to explicitly configure non-contiguous FDRA in the two non-contiguous 'D' RB sets. This could be achieved by defining two starting PRB indices and use the existing length indicator to enable equal number of CSI-RS PRBs in each 'D' RB set. Alternatively, for even more flexibility, two starting PRB indices and two lengths could be defined to allow for configuration of potentially different number of CSI-RS PRBs in each 'D' RB set. The same solution could be envisioned for extending CLI-RSSI measurement resources, since the current resource definition consists of a start PRB and a length.

For new (Rel-18+) UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets according to current specifications, while reception of others is restricted to a single 'D' RB set unless enhancements are made.

For new (Rel-18+) UEs, it is beneficial to support enhancements to CSI-RS resource configuration and CLI-RSSI measurement resource configuration to allow non-contiguous frequency domain resource allocation, i.e., in both 'D' RB sets.

### 2.3.3	SS/PBCH Block Handling for SBFD

Agreement

Study whether SBFD operation in SSB symbols is supported or not.

In RAN1#110bis-e the following agreement was made:

The agreement is to study whether or not SBFD operation is allowed in SS/PBCH symbols or not, i.e., whether or not simultaneous DL and UL is allowed in SS/PBCH block symbols. This is different than whether or not an SBFD symbol is allowed to overlap an SS/PBCH block symbol. If the latter occurs, then one could still disallow SBFD operation in SS/PBCH block symbols by introducing a restriction in the spec that would disallow UL to be scheduled/configured in SS/PBCH block symbols.

Agreement

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

Indeed, we think it is beneficial to introduce a restriction in the spec which would disallow SBFD operation in SS/PBCH block symbols since SS/PBCH block is a crucial signal for proper system operation. Hence, it should be "protected" from UE-UE CLI. One way of achieving this is to ensure that SS/PBCH block is always contained in symbols that are not configured with an UL subband, i.e., DL-only symbols. However, this would require that the period used for the explicit configuration of SBFD subband time locations (see following agreement) is at least as long as the time duration of the SS/PBCH burst and additionally evenly divides SS/PBCH block burst period (20 ms). This would be needed so that DL-only symbols could always be configured to overlap SS/PBCH symbols. We think this is unnecessarily inflexible, and results in a period that is longer than it needs to be.

A more flexible and compact approach is to define a period for the subband time locations to be equal to the periodicity parameter dl-UL-Transmiossionperiodicity defined within the TDD-UL-DL-ConfigCommon IE (see Proposal 5 and related discussion). For example, for a D-D-D-D-U time domain pattern, the periodicity would be 5 slots (2.5 ms / 0.625 ms at 30 / 120 kHz SCS). Additionally, a restriction can be specified such that the UE shall not transmit in the UL in a symbol which overlaps with an SS/PBCH block. Since the SS/PBCH block is cell-common, this effectively creates a gNB restriction that prevents SBFD operation in SS/PBCH block symbols. Based on this we propose the following:

If SBFD symbol overlaps an SS/PBCH block symbol, a UE shall not transmit in the UL subband of this symbol.

## 2.4	UL Signals and Channels

### 2.4.1	IDLE Mode

In the baseline SBFD solution, we assume that in IDLE mode, all UEs (legacy and Rel-18+) are not aware that certain slots/symbols are configured for SBFD operation. Such information is only made available to Rel-18+ UES by dedicated signaling once the UE enters CONNECTED mode. Hence during initial access, UL transmissions by all UEs (legacy and Rel-18+) would occur within the UL-only slot(s) indicated by TDD-UL-DL-ConfigCommon. These transmissions include

- PRACH

- Msg3 PUSCH

- Cell-specifically configured PUCCH

For both legacy and new (Rel-18+) UEs in IDLE mode, UL transmissions (i.e., during initial access) should occur only within UL-only slots.

### 2.4.2	CONNECTED Mode

#### Legacy UEs

A legacy UE in CONNECTED mode is not aware that certain slots/symbols are configured for SBFD operation, same as for IDLE mode. Such a UE sees the whole carrier bandwidth as either 'D', 'U', or ‘F’. Hence legacy UEs can be configured/scheduled in symbols configured as ‘U’ or ‘F’ by TDD-UL-DL-ConfigCommon. If an UL subband is configured in an ‘F’ symbol, the gNB will restrict the configuration/scheduling to only the UL subband; however, the UE does not need to be aware of this. Such UL transmissions can be configured/scheduled as per current specifications without changes. These signals include

- CG- and DG-PUSCH

- PUCCH resources configured by dedicated signaling

- SRS

- PRACH

In contrast to the DL where there is a need to restrict certain signals to only a single subband, no such constraints exist for legacy signals/channels where in both cases mentioned above, UL resources are inherently contiguous.

For legacy UEs in CONNECTED mode, UL transmissions can occur in symbols configured as ‘U’ or ‘F’ by TDD-UL-DL-ConfigCommon as per current specifications without changes.

#### Rel-18+ UEs

In CONNECTED mode, a Rel-18+ UE can be configured/scheduled UL transmissions in either SBFD slots or UL-only slots as shown in Figure 9.

Figure 13: Available RBs for UL transmission within the UL BWP for a Rel-18+ UE

In both cases, the RBs available for UL transmissions are contiguous. Hence in certain cases, current specifications which support contiguous FDRA can be reused for configuration/scheduling of the above list of UL signals/channels. These cases include:

- Case 1: Single/multi-slot UL transmissions in only SBFD slots

- Case 2: Single/multi-slot UL transmissions in only UL-only slots

- Case 3a: Multi-slot UL transmissions (e.g., PUSCH/PUCCH with repetition) spanning both SBFD and UL-only slots in which the indicated frequency domain resources are fully contained within the bandwidth of the UL subband, regardless of whether the slots are SBFD or UL-only.

Agreement

Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols

- Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols

- Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition

- Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

- Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols

Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered.

Examples of potential enhancements include:

- Resource allocation in frequency domain including frequency hopping

- Resource allocation in time domain

- Power domain

- Spatial domain

FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

Where there is a need for enhancement of current specifications is Case 3b defined to be the same as Case 3a except the indicated frequency domain resources are not fully contained within the bandwidth of the UL subband. This is an important case to study since it includes PUSCH and PUCCH with repetition which is the natural mechanism to realize the UL coverage enhancement potential of SBFD. This study fits within the following agreement from RAN1#111:

Figure 14 shows an example of Case 3b in which a multi-slot PUCCH transmission is configured with inter-slot frequency hopping and NPUCCHrepeat=10 repetitions. A 5-slot TDD UL/DL pattern is assumed in which the first 4 slots consist of SBFD symbols, and the 5th slot consists of UL-only symbols. Hence the multi-slot PUCCH spans two cycles of the TDD UL/DL pattern. According to the current specifications, the first and second hop PRB indices are configured by the RRC parameters startingPRB and secondHopPRB which are valid for UL-only slots. The example shows hopping from one edge of the BWP to the other. Since the UL subband spans only a portion of the UL BWP, a mechanism is needed to indicate the PRB indices for the 1st and 2nd hops within SBFD slots. One simple mechanism could be to configure an RB offset relative to startingPRB and secondHopPRB to ensure that the frequency hopping is contained within the UL subband during SBFD slots.

Figure 14: Example of a multi-slot PUCCH transmission spanning both SBFD and UL-only slots configured with frequency hopping and NPUCCHrepeat repetitions.

Figure 15 shows another example of Case 3b for PUSCH with repetition Type A with 10 repetitions. For UL-only slots, the FDRA field in the scheduling DCI indicates both a number of contiguous RBs with starting PRB index RBFDRAstart near the lower end of the BWP, and a frequency hopping offset RBoffset for inter-slot frequency hopping such that the 2nd hop starting PRB index is near the upper end of the BWP. Just like for PUCCH, a mechanism is needed to indicate the starting PRB indices for the 1st and 2nd hops within SBFD slots. It would be undesirable from a coverage perspective to add a 2nd FDRA field to DCI for the SBFD slots. Hence, a more practical approach could be to re-interpret the bits in the existing FDRA field. For example, the bits of the FDRA field could be interpreted for UL-only slots with respect to the start of the UL BWP (as in the current spec), and then re-interpreted in SBFD slots in such a way that they are relative to the start of the UL subband. If frequency hopping is configured, the pre-configured list of RB offsets (RBoffset values) could be extended or a 2nd list defined such that the existing 1 or 2 bits in DCI used to indicate the RB offset could do so for both UL-only and SBFD slots.

Figure 15: Example of a multi-slot PUSCH transmission spanning both SBFD and UL-only slots configured with inter-slot frequency hopping and 10 repetitions.

Based on the above discussion, it is clear that it is needed to study enhancements to frequency domain resource allocation and frequency hopping indication for both PUSCH and PUCCH configured with repetitions that span both SBFD and UL-only slots.

For new (Rel-18+) UEs in CONNECTED mode, study enhancements to frequency domain resource allocation and frequency hopping mechanisms for PUSCH and PUCCH configured with repetition in order to allow repetitions to occur in both SBFD and UL-only slots.

# 3	Feasibility Analysis of SBFD in FR1

As mentioned previously, feasibility of SBFD is a RAN1 SID objective and is studied in this agenda item. In the feasibility analysis we present in this section, we show that feasibility is a function of the class of the gNB. RAN4 defines power limits for local area (LA) and medium range (MR) base-station (BS) classes, as listed below. For wide area (WA) BS class, we consider the power limit agreed in the system evaluation assumptions for this SI.

- LA BS with 24 dBm power

- MR BS with 38 dBm power

- WA BS with 53 dBm power

In RAN4 specifications, the reference receiver sensitivity levels depend on the BS class. Taking 20 MHz with 30 kHz sub-carrier spacing as a representative UL bandwidth for SBFD feasibility analysis while assuming a 40-20-40 DL-UL-DL SBFD subband configuration, the reference sensitivity levels are as follows:

- LA BS Prefsens is -87.6 dBm

- MR BS Prefsens is -90.6 dBm

- WA BS Prefsens is -95.6 dBm

Since link-budgets for SBFD are fundamentally dependent on the gNB class and reference sensitivity, we propose the following:

TR 38.858 should contain feasibility analysis for multiple BS classes with different transmit power limits and reference sensitivity levels. For example, the following three representative BS classes defined in RAN4 can be used for FR1: Local Area (LA), medium range (MR), and wide area (WA).

For the feasibility analysis for the LA and MR BS classes, use the RAN4 defined power limits 24 and 38 dBm, respectively. For WA, use the 53 dBm power limit agreed for system simulations in this study item. For all BS classes use the corresponding RAN4 defined reference sensitivity levels: -87.6 dBm, -90.6 dBm, and -95.6 dBm for LA, MR, and WA, respectively.

In a SBFD network, there are two types of UL resources:

- A first type of UL resource resides in UL-only slots (e.g., 100 MHz in 1 out of every 5 slots assuming a D-D-D-D-U pattern)

- A second type of UL resource resides in UL subbands of mixed direction (SBFD) slots (e.g., 20 MHz  subband in 4 out of every 5 slots)

Because of the TX and RX nonlinearities and mutual coupling as listed in the following, the second type of UL resource is subject to interference leaked from the DL subbands:

- Adjacent subband leakage due to nonlinearities and other impairments in TX radio frontend. Modeling details discussed in Section 2.1 of [1].

The TX radio Analog/ RF frontend contains several nonlinear devices or operations. For instance, when the signal power is driven to approach the limit of the power amplifier, the amplification response becomes increasingly nonlinear. Mechanisms such as digital predistortion (DPD) and crest factor reduction (CFR) are typically implemented in the BS TX chains to mitigate such nonlinearity and maximize PA efficiency, respectively. RAN4 BS specifications require BS implementation to limit such adjacent channel leakage ratio to no more than -45 dBc for FR1 and can be lower for FR2.

- Spatial coupling and isolation between TX and RX arrays. Modeling details discussed Section 2.2 of [1].

To achieve sufficient isolation between the transmit and receive signals, the gNB antenna array needs to be partitioned such that a part of the array can transmit whilst the other section receives, with sufficient isolation mechanism implemented between the two subarrays. Such partitioning of the BS array implies either 3 dB beamforming gain losses, due to reduced antenna area, or at least doubling of physical TX array sizes.

- Adjacent subband distortion due to nonlinearity in RX radio frontend. Modeling details discussed in Section 2.4.1 of [1].

Practical components of the RX radio Analog/ RF frontend also exhibit nonlinear responses. For instance, high gain low noise amplifier (LNA) may output excessive IM3 distortion to the adjacent UL subband (i.e., spectrum regrowth) when the input signal power exceeds its designed operation range due to self-interference from the adjacent DL subbands.

Down conversion phase noise may also introduce interference from the DL subband to the UL subband, thus affecting feasibility and cost of digital cancellation solutions. Further details are discussed in Section 2.4.2 of [1].

To further mitigate self-interference in the digital domain, a prerequisite is to ensure the desired cell-edge UL signals are not swamped or erased by the automatic gain control or due to the lack of enough analog to digital converter bit width. Analog filtering may need to be introduced to suppress the power in the DL subbands, to protect the LNA, ADC, or both.

In the following, we will use these modeling results to provide a high-level discussion on the implementation feasibility of SBFD operations. To further substantiate the discussion of SBFD implementation challenges and potential solutions, we consider the three representative BS classes (LA, MR, and WA) with power limits and reference sensitivity levels listed above.

The interference caused by the interplay of SBFD transmission and TX and/or RX nonlinearities will add to the sensitivity/noise floor. Given an interference to noise ratio of INR, the signal to interference and noise ratio (SINR) is reduced by

Desensitization≜10log101+INR

This desensitization-INR relationship is illustrated in Figure 16.

Figure 16: Relationship between interference to noise ratio (INR) and desensitization of the receiver.

It can be observed that, depending on the system operation objectives, maintaining the total interference due to SBFD operation (residual self-interference, IMD due to Rx non-linearities, interference due to reciprocal mixing of phase noise, ADC quantization noise, etc.) at or near the noise floor is most likely not sufficient. This is because at this 0 dB INR point, the system still suffers a 3 dB desensitization even assuming an SBFD antenna array designed to have the same gain as a conventional array (implying at least double area). This very much reduces, or even eliminates, the potential coverage or latency improvements promised by SBFD. To avoid such reduction, the total interference needs to be maintained at a level below the noise floor. However, due to the logarithmic relationship shown above, lower desensitization requires much more reduction in the INR. For instance,

- Desensitization can be reduced to 2 dB if the interference is maintained at a level 2 dB below the sensitivity/noise floor.

- To further reduce the desensitization to 1 dB, the interference needs to be maintained at a level 6 dB below the sensitivity/noise floor.

We observe that in many regulatory discussions, INR of -10 dB is used as a norm.

To avoid excessive receiver desensitization, the total interference due to SBFD operation from all sources (residual self-interference, IMD due to Rx non-linearities, interference due to reciprocal mixing of phase noise, ADC quantization noise, etc.) needs to be maintained well below the noise/reference sensitivity level for the gNB receiver.

In the following sub-sections we discuss various sources of interference and determine the interference level compared to the defined RAN4 REFSENS values. This analysis allows us to make observations on feasibility for the 3 different BS classes.

## 3.1	Self-interference from direct TX to RX leakage

Assuming

- the gNB TX is designed to provide a 45 dBc leakage ratio from the DL subband to the UL subband (see details in Section 2.1 of [1]),

- 80 dB spatial isolation is feasible for WA and MR BS classes (70 dB antenna isolation + 10 dB Tx beam nulling)

- 75 dB spatial isolation is feasible for LA BS class (70 dB antenna isolation + 5 dB Tx beam nulling)

The direct self-interference leakage from the TX frontend can be estimated in the following table.

Table 2 Direct TX self-leakage to UL subband assuming 80/75 dB spatial isolation.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| C | DL subband to UL subband leakage ratio | -45 | -45 | -45 |
| D | Assumed spatial isolation | -80 | -80 | -75 |
| E | TX leakage power in UL subband = B+C+D | -72 | -87 | -96 |

It can be observed that

LA BS class

Direct TX self-leakage does not cause appreciable performance degradation (i.e., desensitization) if 75 dB spatial isolation is achievable for these small BS since the interference level is well below REFSENS. Given the small antenna arrays for the local area BS class, the isolation mechanism between TX and RX arrays can take up a substantial portion of an SBFD LA BS dimensions.

MR BS class

Direct TX self-leakage alone can cause ~5 dB of desensitization / coverage loss even with 80 dB spatial isolation (INR = 3.6 dB). Two possible mitigation solutions can be considered.

- The TX RF frontend linearity is improved (e.g., by increasing the DPD complexity) to further reduce the DL subband to UL subband leakage ratio. For instance, if the DL subband to UL subband leakage ratio is improved by 5 dB, desensitization can be reduced to 2.4 dB. If the DL subband to UL subband leakage ratio is improved by 10 dB, desensitization can be reduced to 0.9 dB. The complexity of this solution scales with the number of TX RF frontend chains.

- Digital cancellation of leaked interference can be considered, if the RX analog/ RF frontends are not saturated, e.g., due to the power level in the DL subbands.

WA BS class

Direct TX self-leakage alone can cause substantial desensitization / coverage loss even with 80 dB spatial isolation since the interference level is significantly higher than the noise level (INR = 23.6). Two possible mitigation solutions can be considered.

- The TX RF frontend linearity is improved to further reduce the DL subband to UL subband leakage ratio by more than 20 dB. The feasibility, complexity, cost and power consumption of this solution scales with the number of TX RF frontend chains and can become questionable for modern massive MIMO BSs.

- Digital cancellation of leaked interference can be considered, if the RX analog/ RF frontends are not saturated, e.g., due to the power level in the DL subbands. However, as discussed in Section 2.5 of [1], the complexity of this solution scales with the product of the numbers of the TX and RX frontend chains which can become questionable for modern massive MIMO BSs.

## 3.2	Spectrum regrowth due to gNB Rx non-linearity

In Section 2.4.1 of our companion paper [1], we made an estimate of typical IIP3dB of BS receiver low noise amplifier (LNA) based on RAN4 requirement and typical improvements need to avoid receiver desensitization. Assuming a certain spatial isolation, one can estimate the distortion power to the UL subband when SBFD is deployed using typical existing BS LNA as in the following table. Since there are DL subbands on the two sides of a UL subband, we make a simplifying assumption that all the IM3 power calculated from one blocker falls into the UL subband.

Table 3 Example IM3 distortion in the UL subband caused by SBFD DL subbands assuming 80/75 dB spatial isolation and typical existing BS LNA components.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| D | Assumed spatial isolation | -80 | -80 | -75 |
| F | TX power in DL subbands = B+D | -27 | -42 | -51 |
| G | Assumed IIP3dB | -22.6 | -17.6 | -14.6 |
| H | Gain normalized IM3 power in UL subband =3*F-2*G | -35.9 | -90.9 | -123.8 |

One can observe that

LA BS class

With an assumption of 75 dB spatial isolation, DL subband signal likely does not cause excessive IM3 distortions to the UL subband when SBFD is deployed using typical existing LNA for the LA BS class. For these BS, the IM3 powers in the UL subband are well below the REFSENS level.

MR BS class

With the assumed 80 dB spatial isolation and IIP3dB for the LNA, DL subband signal can leak IM3 distortions to the UL subband when SBFD is deployed and causes approximately 3 dB desensitization to the UL subband resources. One possible solution is to improve the IIP3dB performance of the LNA. For instance, if the IIP3dB is improved by 5 dB, then the gain normalized IM3 power would be around 10 dB below the receiver sensitivity level, which means the desensitization is reduced to 0.4 dB.

WA BS class

Due to the much higher transmit power and more stringent REFSENS requirements for the WA BS class, SBFD cannot be deployed using typical existing LNA for the WA BS class. The resulting IM3 distortion to the UL subband is approximately 60 dB above the REFSENS level. Alternative implementation solutions are needed.

- One possibility is to consider an LNA implementation with an IIP3dB of at least 10 dBm, which can reduce the IM3 induced desensitization to 1 dB. However, such large implementation improvements to the LNA come with substantially higher energy consumption and larger physical dimensions. Both of which make the integration into the antenna array much more challenging considering the radio unit size and heat management limits.

- Another possibility is to consider RF filtering before the LNA. However, this may not be practical for carrier frequencies of interest due to a very high requirement on the Q-factor of the filter caused by very small guard PRBs between the DL and the UL subbands. For a system operating with both UL only slots and SBFD mixed direction slots, the RF filtering also need to be tunable. Insertion/switch losses of the RF filters can introduce additional sources of desensitization.

- A third option is to consider removing these distortions using digital cancellation solutions, if the RX analog/ RF frontends are not saturated, e.g., due to the power level in the DL subbands; however, as can be seen in the above table the required cancellation level is very high and may not be practical.

## 3.3	Reciprocal mixing of phase noise

In Section 2.4.2 of our companion paper [1], we make estimates of the interference power from the DL subbands to the UL subband caused by reciprocal mixing of phase noise using the phase noise models discussed in RAN4 Rel-17. It can be observed that the interference power from the DL subbands to the UL subband is around -60 to -70 dBc depending on the implementation of the BS. The impact to the SBFD system can be estimated in the following table:

Table 4 Example reciprocal mixing distortion in the UL subband caused by SBFD DL subbands assuming 80 dB spatial isolation and example phase noise models. (All powers are gain normalized.)

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| D | Assumed spatial isolation | -80 | -80 | -75 |
| F | TX power in DL subbands = B+D | -27 | -42 | -51 |
| I | Interference from reciprocal mixing of phase noise | -69.7 | -64.7 | -60.3 |
| J | Gain normalized interference power in UL = F+I | -96.7 | -106.7 | -111.3 |

It can be observed that

LA BS class

BS receiver phase noise is not an issue for the LA BS class since the ICI power is well below the noise level, and thus does not result in appreciable desensitization.

MR BS class

For the MR BS class, receiver phase noise may cause marginal desensitization of around 0.1 dB to the UL subband resources.

WA BS class

Due to the much higher transmit power and more stringent REFSENS requirements for the WA BS class, the mixer phase noise can cause >2.5 dB desensitization to the UL subband resources if no mitigation solutions are further deployed. Alternative implementation solutions are needed.

- One possibility is to improve down-conversion LO quality. With a 10 dB improvement in phase quality, the desensitization can be brought down to 0.3 dB. However, such improvement in phase quality may require much increased implementation complexity and operating energy consumption.

- Another option is to investigate whether these reciprocal mixing distortions can be removed using digital cancellation solutions, if the RX analog/ RF frontends are not saturated, e.g., due to the power level in the DL subbands.

## 3.4	Impact on analog to digital converter (ADC) and additional filtering

The analog to digital converters (ADC) for a BS are designed to handle a dynamic range covering weak UL signals and the potential adjacent channel blockers without losing the standard required sensitivity. RAN4 spec TS 38.104 further stipulates BS performance requirements for in-band narrowband blocker, adjacent channel selectivity, and in-band general blocker.

For allowance of 6 dB desensitization, the RAN4 test powers for these blockers or adjacent channels are listed in the following table.

Table 5 RAN4 blocker and adjacent channel selectivity test interference power levels.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
|  | RAN4 in-band narrowband blocker power | -49 | -44 | -41 |
|  | RAN4 adjacent channel interfering signal power | -52 | -47 | -44 |
|  | RAN4 in-band general blocker power | -43 | -38 | -35 |

Assuming an antenna array spatial isolation of 80/75 dB is feasible for the different BS classes, the direct leakage from the TX frontend in the DL subbands can be estimated in the following table.

Table 6 Example interfering powers in SBFD DL subbands assuming 80/75 dB spatial isolation. (All powers are gain normalized.)

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| D | Assumed spatial isolation | -80 | -80 | -75 |
| F | TX power in DL subbands = B+D | -27 | -42 | -51 |

Comparing the interfering powers in DL subbands calculated in Table 6 and these RAN4 test powers, the following observations can be drawn for the different BS classes.

LA BS class

For local area BS class, the interfering powers in the DL subbands in a SBFD system are lower than those in the current RAN4 performance requirement specs. Hence, the use of SBFD transmission schemes likely does not affect the typical ADC implementations in these BS.

MR BS class

For the medium range, the interfering powers in the DL subbands in a SBFD system are in the same range as those in the current RAN4 performance requirement specs. Hence, the use of SBFD transmission schemes may cause some desensitization with the typical ADC implementations in these BS. To avoid such desensitization, solutions such as those considered below for the wide area BS may also need to be adopted for the medium range BS.

WA BS class

For the wide area BS class, however, the interfering powers in the DL subbands in a SBFD system can be 22 dB or more higher than current RAN4 blocker performance test powers – which may result in the RX analog/ RF frontends getting saturated if they are not handled suitably. To handle such enlarged dynamic range than typical wide area BS implementations, alternative solutions need to be considered to avoid losing sensitivity.

- One possibility is to introduce analog filtering before the ADC. The implementation challenge and feasibility of this approach depends on the base station receiver architecture choices. There are various receiver structures used for BS depending on the deployment as well as capabilities such as beam forming for massive MIMO type of BS or support of multiple carriers or bands. We provide a high-level survey of different base station receiver architectures and examine their compatibility with SBFD operations in Section 3.8 below. It was observed that the implementation complexity and difficulty of sharp analog filtering that is suitable separating DL and UL subbands with a few RBs of guard frequencies in a SBFD carrier depends strongly on the operating frequency at which such analog filtering takes place.

  - For the homodyne type receivers, such analog filtering can be implemented at or near zero Hz. For a single SBFD carrier, such analog filter may even take the shape of a low pass filter, which makes the implementation easier. However, to support

    - multiple SBFD carriers or

    - when the SBFD carrier needs to be away from zero Hz

band pass filters will need to be implemented with increased complexity. If a set of UL subband bandwidths need to be supported, and/or different a set of carrier combinations with/without SBFD need to be supported, a potentially large network of switchable analog filter components needs to be designed which further increases complexity.

  - For the heterodyne type receivers, analog filtering at the IF to separate DL and UL subbands of a SBFD carrier presents substantially increased implementation challenges.

  - For the direct RF sampling type receiver, the feasibility of sharp filtering at RF to separate DL and UL subbands is of a SBFD carrier is highly questionable considering requirements on very high Q values and low insertion loss.

- Another possibility is to increase the bit width of the ADC. However, such solutions come with steep increases in implementation costs as well as energy consumption. As a rule of thumb, every additional ADC bit width doubles the power consumption. Note further that ADC power consumption is already a significant component of typical BS receivers. Larger ADC bit width would also require high bandwidth connections between the radio units and the baseband units.

- A combination of the above two types of solutions. However, if the desired DL subband power suppression is not completely achieved in the analog filtering stage, it is necessary to not only increase the ADC bit width but also increase the suppression at the digital channel filtering stage, which may have its own additional complexity implications.

## 3.5	Example digital self-interference cancellation solution for FR1 homodyne/heterodyne type base station receivers

In modern FR1 MIMO base station, TX beamforming is performed digitally in the frequency domain to direct the beams at different PRBs to different directions for the scheduled UEs allocated to different PRBs. In fact, even for a UE, different beamforming weights for different PRBs are also needed to align with the UE’s frequency domain channel responses. Such frequency-selective beamforming weights manifest in the time domain as if the signals have been through multi-tap channels already. With multi-layer SU-MIMO and MU-MIMO, the baseband signal into the TX chain is the superposition of more than one beamformed signals targeting the multiple co-scheduled UEs. The superposition of these signals then goes through the transmitter chains with the components discussed in Section 2.1 of the companion contribution [1]where many of the components introduce further multi-tap and nonlinear responses.

An initial example of digital self-interference cancellation solution for FR1 homodyne/heterodyne type base station receivers is discussed in Section 2.5 of the companion contribution [1]. On the RX side, as discussed in the previous sections, analog subband filters (shown in the figure as a red “AF” box in Figure 17) are needed to suppress the leaked TX powers in the DL subbands. Depending on the receiver architectures and the SBFD use cases, low pass or band pass filters are needed. Such filters need to be much sharper than normal anti-aliasing filters after down conversion since there is only a small guard frequency gap between the DL and UL subbands.

The digital cancellation fabric takes the analog coupled signal from each of the TX chains and apply the same analog subband filtering as those in the RX chains before digitization. For a 100 MHz carrier, the sampling rate is at 122.88 MHz. Hence, one time sample corresponds to 2.44 m. To cover potential strong reflectors within even a small radius, the adaptive filters need to keep track of several taps if no over sampling is used. In reality, delays of the reflected signals are generally not aligned with the sampling grid perfectly. Over-sampling is likely needed to obtain adequate cancellation performance. The filter lengths will scale with the over-sampling rate accordingly. The analog filters also add to the effective lengths of the overall channel responses.

The digital cancellation fabric takes inputs from NT TX chains and keeps a memory of the most recent L input values for each of these inputs. The fabric applies one set of filter weights on these NTL values to produce one cancellation signal to be used by one RX chain. To serve NR RX chains, NR sets of filter weights are needed. Effectively, for each new sampling time, the cancellation fabric multiplies the stored inputs from the TX chains by a NR×NTL matrix to obtain NR cancellation samples. To obtain the NR×NTL filter weights, the digital cancellation fabric needs inputs from the NR RX chains as well.

Figure 17: An initial generic example of digital self-interference cancellation implementation for FR1 homodyne/heterodyne receivers. The red “AF” box represents an analog subband filter for the SBFD operations.

For the three representative BS classes introduced in previous sections, we can also make some assumptions on the typical antenna array sizes as follows:

- LA BS with (Mg, Ng, M, N, P)  = (1,1,1,2,2)

- MR BS with (Mg, Ng, M, N, P) = (1,1,2,4,2)

- WA BS with (Mg, Ng, M, N, P)  = (1,1,4,8,2)

where M, N indicate numbers of subarrays and each subarray contains 3 antenna elements. That is, the typical numbers of TX chains are NT=M∙N∙P= 4, 16, and 64 for the LA, MR and WA BS classes, respectively. For SBFD system implemented to achieve “same gain”, then the number of the RX chains equals that of the TX chain: NR=NT. The complexity of the digital cancellation scales with NT2L. That is, the complexity of digital self-interference cancellation for the three BS classes is scaling with a factor of 16, 256, and 4096 for the LA, MR and WA BS classes, respectively. It can be observed that digital cancellation solutions are much more feasible for lower power BS with small coverage than for high power BS with massive MIMO capabilities.

Digital self-interference cancellation solutions are more feasible for lower power BS with small coverage than for high power BS with massive MIMO capabilities.

## 3.6	Feasibility observations for a single-carrier, single-sector BS

From the above analysis of the different sources of interference due to SBFD operations, we can make some general observations on implementation feasibility/challenges for the different BS classes with different max power levels as well as antenna array sizes for the case of single-carrier, single-sector BSs.

LA BS class

For the low power BS class, such as the representative LA BS class, the above identified interference powers are generally below the REFSENS level with enough margin if spatial  isolation of 75 dB is achieved. It is likely that a single isolated lower power LA BS can operate in the SBFD mode using existing hardware components and without the need of digital self-interference cancellation.

A low power BS such as the representative local area (LA) BS class, can likely operate in SBFD mode in FR1 using existing hardware components without digital self-IC if spatial isolation of 75 dB is achieved.

MR BS class

For the medium power BS class, such as the representative MR BS class, we identified in the above that several interference powers are at a level similar level to the REFSENS level. Without addressing these interference powers, the reception of the SBFD UL subbands can be desensitized by several dB. For instance, the direct TX leakage into the RX UL subbands has a power 3.6 dB higher than the REFSENS, which will result in 4.5 dB desensitization to these UL subband resources by this interference source alone. That is, no coverage gain can be obtained even if the UL signals are repeated in three UL subband slots. To obtain the coverage benefits of operating in the SBFD mode, two different interference mitigation approaches can be considered.

MR BS Interference Mitigation solution A (Hardware component upgrades)

The interference powers can be reduced by upgrading several components of the representative MR BS class above those typically used in current static TDD MR BS.

- The TX linearity need to be improved (e.g., by increasing the DPD complexity) to reduce the DL subband to UL subband leakage ratio by 10 dB. This will reduce the desensitization of these UL subband resources by the direct leakage alone to 0.9 dB.

- The RX LNA linearity needs to be improved. For instance, if the LNA IIP3dB is improved by 5 dB, then the desensitization of these UL subband resources by the IM3 alone to 0.4 dB. Such increase of LNA IIP3dB usually requires in the size, energy consumption and heat dissipation of the LNA.

- The ADC bit width may need to increase by 1 bit, which, by rule of thumb, will double the power consumption of the ADC. Alternatively, analog filtering needs to be introduced.

With these hardware component upgrades to the MR BS, the total residual (gain normalized) interference power is around -94.3 dBm. Compared to REFSENS of -90.6 dBm, the desensitization of the UL subband resources is 1.5 dB. With this approach, a 3 dB coverage gain would require the UL signals to be repeated in three UL subband slots, which will substantially reduce the improvement in latency.

Note further that this cost, complexity, energy consumption and heat dissipation issues of this approach scale linearly with the number of TX/RX chains.

MR BS Interference Mitigation Solution B (Digital self-interference cancellation)

Digital self-interference cancellation has been proposed by several sources as a potential solution to address the direct TX to Rx leakage. However, as discussed in the previous sections, there are additional sources of interference.

- As shown in the generic digital cancellation solution in Figure 17, replicas of the TX RF signals with all their nonlinear and multi-tap responses can be taken from the TX chains. Several sources have opined that the direct TX leakage power can be reduced by 10 – 15 dB via digital cancellation. However, the complexity scaling and the cancellation performance in outdoor environments with strong reflectors needs further study.

- The RX LNA nonlinearity causes additional IM3 leakage from the received DL subbands into the UL subbands. To remove these IM3 powers, the digital cancellation solutions need to estimate the filter coefficients to generate the needed cancellation signals. The complexity scaling and the cancellation performance of the RX IM3 need further study.

- The ADC bit width may need to increase by 1 bit and possibly by 2 bits to obtain better digital cancellation performance. Either increases of ADC bit width will increase its power consumption substantially.

If the digital self-interference cancellation solution manages to suppress all interference powers by 10 dB, then the total residual (gain normalized) interference power is around -95.5 dBm. Compared to REFSENS of -90.6 dBm, the desensitization of the UL subband resources is 1.2 dB. With this approach, a 3 dB coverage gain can be achieved if the UL signals are repeated in three UL subband slots, which will substantially reduce the improvement in latency. If the digital cancellation solution manages to suppress all interference powers by 15 dB, then the desensitization of the UL subband resources is reduced to 0.4 dB.

Note further that this cost, complexity, energy consumption and heat dissipation issues of this approach scale quadratically with the number of TX/RX chains.

It is, of course, possible to consider combinations of these two basic approaches. For instance, if RX IM3 interference estimation proves unreliable, it may be necessary to upgrade the RX LNA even when digital cancellation solution is adopted in the BS RX. With any of these approaches to address interferences for a MR BS, it can be observed the RX chains suitable for SBFD operation require more or better hardware, higher energy consumption, and higher heat dissipation than those suitable for conventional static TDD operation.

For a medium power BS such as the representative medium range (MR) BS class for FR1, in a single-carrier, single-sector deployment, interference mitigation solutions such as (1) hardware component upgrades to avoid spectrum regrowth caused by RX non-linearities, or (2) digital self-interference cancellation, or combination of (1) and (2), are needed even with 80 dB spatial isolation.

For mitigating interference due to SBFD operation, the cost, complexity, energy consumption, and heat dissipation scales (1) linearly with the number of TX/RX chains for mitigation through hardware component upgrades, and (2) quadratically with the number of TX/RX chains for mitigation through digital self-interference cancellation.

WA BS class

For the high-power BS class, such as the representative WA BS class, we identified in the above that the interference powers from each source are far above what current typical WA BS hardware is designed for. Either of the mitigation approaches discussed in the above for the MR BS class appear infeasible:

- For the hardware component upgrade approach, the required improvements in the hardware are far above the specs of current typical WA BS components. For instance, the TX linearity needs to be improved such that the DL subband to UL subband leakage ratio is reduced by 30 dB. The LNA IIP3dB needs to be improved by more than 20 dB. The phase noise powers need to be reduced by >10 dB. Sharp analog filters need to be introduced or the ADC bit width needs to increase by several bits. Any, and, particularly in combinations, of these can cause substantial cost, complexity, energy consumption, or heat management issues.

- All the potential issues with digital self-interference cancellation solutions discussed in the above for the MR BS class scale up exponentially for the WA BS class because of the much higher degrees of cancellation needed as well as the quadratic scaling with the massive MIMO antenna array sizes. Also, the possibility of the RX analog/ RF frontends getting saturated, e.g., due to the power level in the DL subbands, is much higher in the WA BS case than in the MR BS case.

If these hardware implementation issues were to be overcome, one can also observe that, similar to the case for the MR BS class, the RX chains suitable for WA BS SBFD operation require more or better hardware, higher energy consumption, and higher heat dissipation than those suitable for conventional static TDD operation.  Such complexity, cost and power consumption scaling may not be justifiable considering the existence of much simpler alternative system operation approaches with similar or better UL performance improvements such as that discussed in Section 3.9 below.

For a high-power BS, such as the representative wide area (WA) BS class in FR1, interference powers due to SBFD operation are far above what current typical WA BS hardware is designed for even with 80 dB spatial isolation, even in a single-carrier, single-sector deployment. Either of the SBFD related interference mitigation approaches discussed for the medium range (MR) BS class would result in excessive cost, complexity, energy consumption, and heat management.

## 3.7	Consideration of multi-sector base station sites

Sectorized site deployments are typically used to provide full coverage in the horizontal plane. This means that up to three antennas will be mounted close together on the mast as illustrated by the photos of representative WA and MR commercial sites in Figure 18. As can be seen in these site deployments, the edge-to-edge distance dH between antennas in different sectors can be quite small, e.g., on the order of 40 cm.

Figure 18: Representative WA and MR commercial sectorized sites showing that the edge-to-edge distance dH between antennas in different sectors can be quite small.

In Section 2.2.3 of [1] we provide inter-sector isolation results for a simplified 3-sector site including vertically separated Tx and Rx panels as would occur for a deployment with antennas built for SBFD operation (see Figure 19 below copied from [1]). We find that the isolation varies significantly depending on the azimuth and elevation beam steering directions, as well as the frequency within the band. The variation range of the panel-to-Rx-port coupling magnitude is -87 to -67 dB with -72 dB being typical. We also observe that other realistic effects that are not modeled in our simulations (for simplicity reasons) are likely to degrade isolation further, for example:

- re-radiation from the electronics on the backside of the antennas

- practical site aspects such as equipment and other metallic objects in between sectors that can act as PIM sources

- presence of sub-arrays (the norm in AAS products), that result in higher side lobes and grating lobes, thus reducing achievable isolation.

- presence of radomes

The above effects are not necessarily solved simply by moving antennas further apart.

- Top view of site

- Side view of site

Figure 19: Simplified 3-sector site setup for FR1 considered for inter-sector EM simulations (see [1])

Based on the simulation results from Section 2.2.3 of [1] we choose a panel-to-Rx-port inter-sector isolation value of 72 dB in the following analysis, which in itself may still be optimistic. At this level, inter-sector self-interference causes challenges to the WA and MR BS classes as shown in the following tables for the following interference sources:

- The inter-sector direct TX leakage to UL subband

- IM3 interference in UL subbands induced by the inter-sector received power in DL subbands

- Reciprocal mixing interference in UL subbands induced by the inter-sector received power in DL subbands

Note that an LA BS is not typically used in a three-sector setup, so we omit the inter-sector analysis for this BS type. Note that the inter-sector antenna leakage values in the following tables include a factor of 3 dB to account for the factor that inter-sector interference is received from two co-site sectors in a 3-sector deployment

Table 7 Inter-sector direct TX leakage to UL subband

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| C | DL subband to UL subband leakage ratio | -45 | -45 | -45 |
| K | Assumed inter-sector antenna leakage | -72 | -72 | N/A |
| L | Inter-sector received power in UL subband = B+C+K | -64 | -79 | N/A |

Table 8 Example IM3 distortion in the UL subband induced by inter-sector received power in DL subbands assuming typical existing BS LNA components.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| K | Assumed inter-sector antenna leakage | -72 | -72 | N/A |
| M | Inter-sector received power in DL subbands = B+K | -19 | -34 | N/A |
| G | Assumed IIP3dB | -22.6 | -17.6 | N/A |
| N | Gain normalized IM3 in UL subband = 3*M-2*G | -11.8 | -66.8 | N/A |

Table 9 Example reciprocal mixing distortion in the UL subband induced by inter-sector received power in DL subbands assuming example phase noise models. (All powers are gain normalized.)

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| K | Assumed inter-sector antenna leakage | -72 | -72 | N/A |
| M | Inter-sector received power in DL subbands = B+K+3 | -19 | -34 | N/A |
| I | Interference from reciprocal mixing of phase noise | -69.7 | -64.7 | N/A |
| O | Gain normalized interference power in UL = M+I | -88.7 | -98.7 | N/A |

It can be observed that the receivers for the MR and WA BS classes can be severely affected by strong interference powers leaked from adjacent sectors.

- From Table 7, one can see that for addressing inter-sector direct leakage into the UL subband, improving Tx linearity alone is not a viable solution given the amount of improvement needed for both WA and MR BS classes.

- From Table 8, one can see that the inter-sector received power in the DL subbands is either far above (WA BS) or not sufficiently below (MR BS) the assumed IIP3 of the LNA which is based on current requirements for BS RX chains. The input power levels create excessive inter-modulation distortion in the receive chain.

  - For the WA BS, if the LNA would be hypothetically upgraded to handle the strong interference leaked from other sectors, the required IIP3 that would be needed to reduce the IM3 in the UL subband to 6 dB below the sensitivity level (i.e., 1 dB desense), would be approximately 22 dBm which is infeasibly high (45 dB upgrade compared to existing BS requirements). This would create large challenges from a power consumption and heat management perspective in an AAS product with a large number of antenna elements.

- From Table 9, one can see that reciprocal mixing interference in the UL subband induced by inter-sector received power in the DL subbands is significantly above the RX sensitivity levels for WA BS.

- For the WA BS class, given the component failures before or at the digitization stages (e.g., due to saturation of the LNAs or ADCs), digital cancellation of inter-sector interference powers is not viable solution.

- For the MR BS class, whether or not inter-sector digital interference cancellation is feasible needs further study/discussion.

With inter-sector panel-to-Rx-port isolation of 72 dB, the receivers for WA and MR BS classes suffer from strong interference powers leaked from adjacent sectors far above the current BS operation specs.

In prior meetings there was some discussion about the possibility of installing "isolation enhancing" material between sectors of the same operator. Following this logic, then would also need to consider installing isolation enhancing matrix also between antennas for different operators with antennas mounted on a tower/pole at different heights. Figure 20 below shows an example of a co-sited multi-operator deployment. Deployments with multiple co-sited operators are the norm due to zoning, regulatory and economic considerations. We note that typical co-location isolation requirement for current BS equipment is -30 dBc.

Figure 20: Example of multiple BS installed at proximity. See further examples in https://en.wikipedia.org/wiki/Cell_site.

We see no precedent in operators’ current site solutions for installation of isolation enhancing material, e.g., to improve coexistence. Even if such material existed, we see that installation of such material would face significant practical challenges:

- Severe space constraints on a macro tower already today, and only getting worse in the future due to site capacity upgrades (addition of new radios covering new bands),

- Challenges in obtaining site permits taking into account requirements on site aesthetics,

  - This is especially true for street macros/micro deployments,

- Wind load considerations,

- Requirements to leave space between antennas for accessibility for maintenance engineers.

Installation of isolation enhancing material between sectors of one operator and/or between antennas of different operators with antennas at different heights on a tower/pole would face significant practical challenges, and does not appear feasible.

## 3.8	Consideration of modern gNB receiver architectures for multi-carrier and multi-band use cases

There are various receiver structures used for BS depending on the deployment as well as capabilities such as beam forming for massive MIMO type of BS. However, the need to support ever large RF bandwidths for multi-carriers and multi-bands use cases has, in particular, been causing BS implementation to trend toward certain architecture choices. In this section, we survey major BS architectural choices and examine their compatibility with the SBFD operation requirements. In particular, the need to handle large dynamic ranges and the resulting required DL subband filtering to protect the ADC in the receiver chain (as discussed in Section 3.4 in the above) can have important implications to the BS receiver architectures.

Super heterodyne RX

The super heterodyne was for a long time the major receiver architecture for base station applications and it can use one or more stages of frequency down conversion. The sampling of the analog data is in the real domain and occurs at an intermediate frequency. Being at an intermediate frequency makes it more difficult to integrate and usually requires sharper anti-aliasing filtering compared to a Homodyne receiver. This is due to the mixer output having significant spurious components at the output that are generated from the products of the input frequencies and the LO (Local Oscillator) frequency. This also makes the need of quite good filtering at RF to attenuate out-of-band interferences that could be mixed down into the wanted channel in the same way. Note that sufficient filtering at RF level requires significant guard and high Q resonators.

The super heterodyne is good in performance for RF bandwidth up to around 200MHz and therefore a suitable choice for single band base-stations at lower frequencies. However, for many bands above 3 GHz, the needed maximum RF bandwidth is most often larger than 200 MHz, e.g., Band n77 (3300 – 4200) has bandwidth of 900 MHz.

Due to limited guard between DL sub-band and UL sub-band and limitation in SAW filter technology, it would be difficult to mitigate the DL sub-band powers using filters in the IF domain.

Homodyne RX

The homodyne architecture has been applied for a long time in the handsets. The homodyne receiver is using a down conversion to zero Hz with IQ-demodulation and the spectrum is then sampled by the ADC in the complex domain. This puts stringent requirements on keeping phase and amplitude perfectly matched between the I- and Q-branches to avoid quadrature errors, particularly for multi-carrier use cases.

The benefit of the Homodyne is that larger bandwidth can be sampled. Furthermore, operating at a low frequency, fairly good filters can be more easily implemented on the same chip as the converters and mixers, resulting in more integrated components. The Homodyne architecture works well for up to a few hundred MHz in RF bandwidth and can be used for multi-band receiver applications as well as smaller single band radios. When going above 400-500MHz RF-BW it starts to become much difficult to have a good compensation for the IQ-errors that might make other solutions more suitable from an energy consumption point of view.

For SBFD, homodyne receiver would possibly allow for some analogue filtering at baseband to protect the ADC at the expense of increased complexity. This approach would work for a single carrier receiver and will limit the flexibility of using different UL sub-band sizes unless complex switchable filter banks are incorporated. For multi-carrier/multi-band use cases, the analog filtering implementation complexity may increase substantially and requires further study.

Direct RF-sampling RX

The direct RF-sampling architecture is starting to be used in AAS/massive MIMO radios but less frequently seen in classic/non-AAS radios, due to the performance difference needed. The ADC is sampling the spectrum at RF without any down conversions and by that in the real domain. Many of the chipset suppliers are selecting this architecture for their future roadmaps as direct RF sampling structures provides the possibility to design very wideband receivers to support multi-carrier/multi-band use cases.

To protect the LNA and/or ADC, filtering needs to be done at RF depending on the BS class. At RF, the bandwidth of the transition from passband to stop band relative to the carrier frequency becomes much smaller than when the filter is implemented at IF or baseband. Thus, to get same type of filter response it requires very large Q-values for the filters translating to large filter size. This poses significant challenges to integration on the same chip as the ADC and significant challenges integrating a solution into an AAS radio with a large number of antennas. Further complicating this is the need for a network of switchable analog filter components which increases complexity and creates extra insertion loss, affecting coverage.

Based on the above survey, we can observe the industry trend toward supporting ever wider RF bandwidths for multi-carrier/multi-band receivers for at least higher power base stations. Such need to support multiple carriers and/or bands in a single base station is driven by technological, economical, and operational considerations. Integration allows lower overall power consumption than discrete components. Smaller deployment footprints reduce mitigation needs for wind loads and other installation issues. Both can be beneficial in reducing maintenance expenses and site rents.

In summary, the implementation complexity and difficulty of sharp analog filtering that is suitable for separating DL and UL subbands with a few RBs of guard frequencies in a SBFD carrier depends strongly on the operating frequency at which such analog filtering takes place.

- For the homodyne type receivers, such analog filtering can be implemented at or near zero Hz. For a single SBFD carrier, such analog filter may even take the shape of a low pass filter, which makes the implementation easier. To support other cases (multiple SBFD carriers or when the SBFD carrier needs to be away from zero Hz), band pass filters will need to be implemented with increased complexity. If a set of UL subband bandwidths need to be supported, a network of switchable analog filter components needs to be designed with further complexity.

- For the heterodyne type receivers, analog filtering at the IF to separate DL and UL subbands of a SBFD carrier presents substantially increased implementation challenges.

- For the direct RF sampling type receiver, the feasibility of sharp filtering at the IF to separate DL and UL subbands of a SBFD carrier is highly questionable due to requirements on very high Q value and low insertion loss.

The implementation complexity and difficulty of sharp analog filtering that is suitable separating DL and UL subbands with a few RBs of guard frequencies in a SBFD carrier depends strongly on the operating frequency at which such analog filtering takes place. Such analog filtering is more feasible with homodyne type receivers but becomes increasingly infeasible with the heterodyne or direct RF sampling type receivers.

Operators generally utilize a portfolio of carriers and frequency bands to create high-quality services. Similar to the need of sharing BS sites, due to zoning, regulatory and economic considerations, many base stations serve multiple carriers and/or multiple frequency bands. With each additional carriers and/or bands, the total output powers from the BS scale accordingly. Some of the implementation issues and challenges discussed in the previous scale linearly as well but some may scale nonlinearly. For instance, when the amplifier is shared by multiple carriers and/or bands, signals/activities from multiple sources affect the leakage models of the TX signals to the RX, which can have significant impact on the complexity and feasibility of digital self-interference cancelation.

RAN1/4 to study solutions, performance and applicability for SBFD operations in multi-carrier and/or multi-band base stations.

## 3.9	Alternative approaches

While there is a significant discussion on the potential for coverage and latency improvement offered by SBFD, we think it is important to not forget that alternative approaches exist for achieving such coverage and latency improvements. In this section, we discuss one such alternative.

For conventional static TDD deployments, operators need to coordinate the TDD UL/DL configuration to avoid interference between the different operators’ networks, both for carriers in the same band and even for carriers in nearby bands. That is, other than a new frequency band without potential interference issues with other bands, a prevalent TDD UL/DL configuration structure is usually imposed for a geographical area and an operating frequency range. In this high-level discussion, we use the DDDDU pattern as an example of this prevalent TDD UL/DL structure. We further use 100 MHz carrier bandwidth as an example for the discussion.

Prevalent static TDD pattern: DDDDU

  - There are 3*14+12 = 54 DL OFDM symbols in a TDD UL/DL cycle of five slots considering a 2 symbol guard for DL to UL switching in the 4th slot

  - There are 14 UL OFDM symbols in the UL-only slot

The SBFD scheme can be considered as an opportunistic TDD operation mode that appropriates some DL resources for UL coverage, throughput and latency improvements:

SBFD operation mode: MMMMU with 40-20-40 MHz subbands (M stands for mixed-direction slots)

  - There are 2*106/273 * (3*14 + 12) = 41.9 equivalent DL OFDM symbols in a TDD UL/DL cycle of five slots.

  - There are 51/273 * (3*14 + 12) + 14 = 24.1 equivalent UL OFDM symbols

The new UL subband resources in the mixed-direction slots are opportunistic in the sense that UL transmissions in these new resources may or may not succeed because of potential collisions with self-interference or interference from other BSs within the same or other operator networks. This point is amply demonstrated by the single network system evaluation results presented in Appendix B.3 and the two network system evaluation results presented in Appendix B.4 of the companion contribution [1]. It can be generally observed that:

- When the SBFD MMMMU network is operating in isolation

  - The SBFD network provides UL coverage and user throughput gains at the expense of DL user throughputs at low traffic loads relative to a static TDD network using the DDDDU configuration.

  - As traffic load increases, the UL coverage and user throughput gains diminish and disappear completely at high traffic loads because of self-interference and inter-sector interference. However, the loss of DL throughputs remain since DL resources have been appropriated for UL even though the UL transmissions are not always successful.

- When the SBFD MMMMU network is operating with a static TDD DDDDU network in the adjacent channel

  - The SBFD network provides UL coverage and user throughput gains when the SBFD network has low traffic loads and the adjacent network also has low traffic loads.

  - When the traffic loads of the adjacent network increase, the UL coverage and user throughput gains vanish even when the traffic loads of the SBFD network remain low.

If the purpose is to introduce opportunistic UL resources for UL coverage, throughput and latency improvements, there may be simpler alternatives that do not require complicated hardware and do not suffer from self-interference or inter-sector interference of its own network. For instance, one could consider operating a different static TDD pattern than the prevalent static TDD pattern:

Opportunistic static TDD mode: DUDDU

  - There are 3*14 = 42 DL OFDM symbols in a TDD UL/DL cycle of five slots.

  - There are 2*12 = 24 UL OFDM symbols (i.e., guard periods are at the beginning of the two UL slots).

It can be observed this opportunistic static TDD network has almost identical overall DL and UL resource appropriations as those of the SBFD network. This static TDD operation mode is opportunistic because the newly introduced UL slot may be interfered by adjacent channel operators using the prevalent TDD pattern. The main difference from SBFD is this is a static TDD operation mode and hence it does not suffer from self-interference and inter-sector and inter-site interference of its own network. As a result, it is demonstrated in Appendix B.3 of the companion contribution [1]:

- When the opportunistic static TDD DUDDU network is operating in isolation

  - The network provides similar UL coverage and user throughput gains at the expense of DL user throughputs at low traffic loads as those provided by the SBFD network.

  - As traffic loads increase, the UL coverage and user throughput gains only degrade moderately and thus become better than those achieved by the SBFD network.

  - The UL latency reduction obtained by this opportunistic static TDD network is comparable to that achieved by the SBFD network.

For UL coverage, throughput, and latency improvements, RAN1 should study alternatives to SBFD operation, namely opportunistic TDD operation without configuration of UL subbands. Such alternatives do not require complicated gNB hardware, and do not suffer from self-interference, inter-sector interference of its own network.

## 3.10	Feasibility conclusions for FR1

Based on the extensive analysis in the previous sections, we draw conclusions on the feasibility of SBFD for different BS classes based on consideration of at least the following:

- Self-interference from direct Tx-to-Rx leakage within a gNB antenna array

- Spectral regrowth due to non-linearities in the gNB Rx chains and requirements on hardware component upgrades

- Interference due to reciprocal mixing of phase noise

- Complexity of digital self-interference cancellation considering scaling with the number of TX/Rx chains

- Analog/ RF filtering required to protect ADC and/or LNA

- Inter-sector interference

- Multi-carrier, multi-band aspects of modern gNB receiver architectures

- Size, power consumption, and heat management issues

For high power BS (e.g., WA BS class) in FR1, the feasibility of SBFD in a modern multi-carrier, multi-band, AAS architecture is highly questionable given the excessive requirements on (1) hardware component upgrades which translate to significant issues on product size, power consumption, and heat management, and (2) site solutions for mitigation of co-site interference. Alternative approaches can be considered for WA deployments that do not rely on simultaneous transmission and reception at the gNB, e.g., opportunistic TDD operation with equivalent amount of UL resources as SBFD.

For low and medium power BS (e.g., LA and MR BS classes) in FR1, assuming 80 and 75 dB spatial isolation is achieved, respectively, the feasibility of SBFD appears more likely at least for single-sector, single-carrier deployments for which there are lower demands on hardware component upgrades and mitigation of self-interference compared to WA BS class. Further study under realistic assumptions is needed in the area of multi-sector, multi-carrier, and multi-band deployments.

# 4	Feasibility Analysis of SBFD in FR2

In this section we provide a feasibility analysis for FR2. Similar to the FR1 analyze multiple BS power levels:

- 40 dBm BS

- 30 dBm BS

- 23 dBm BS

Agreement

For SLS evaluation purposes only, Alt 1/2/4 (SBFD UL subband is about 20% of the channel bandwidth) and SBFD Subband configuration#1 with {DUD} pattern, the following is assumed:

- For FR1

  - Baseline: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5>

  - Optional: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <106, 51, 5>

- For FR2

  - Baseline: 100MHz channel bandwidth and 120kHz SCS (66 PRB) < ND, NU, NG > = <25, 14, 1>

  - Optional: 200MHz channel bandwidth and 120kHz SCS (132 PRB): < ND, NU, NG > = <47, 32, 3>

- Other values of < ND, NU, NG > are not precluded and can be reported by companies.

In all cases, we assume a noise figure of 10 dB and a 100 MHz carrier. We assume the same size UL subband as has been agreed for system level simulations in the following agreement, i.e., 14 RBs which translates to 20.16 MHz, assuming 120 kHz SCS:

For all three BS power levels, we assume that the noise floor in the UL subband bandwidth is given by -174 dBm/Hz + 10*log10(20.16e6) + 10 = -91.0 dBm.

TR 38.858 should contain feasibility analysis for multiple BS power levels. For example, the following three representative power levels can be used for FR2: 40 dBm, 30 dBm, 23 dBm.

## 4.1	Self-interference from direct TX to RX leakage

Assuming

- the gNB TX is designed to provide a 28 dBc leakage ratio from the DL subband to the UL subband according to RAN4 specifications for FR2

- and 85 dB spatial isolation is feasible for all three BS power levels (80 dB antenna isolation + 5 dB Tx beam nulling)

The direct self-interference leakage from the TX frontend can be estimated in the following table.

Table 10 Direct TX self-leakage to UL subband assuming 85 dB spatial isolation.

|  |  | 40 dBm BS | 30 dBm BS | 23 dBm BS |
| --- | --- | --- | --- | --- |
| A | Noise Floor | -91.0 | -91.0 | -91.0 |
| B | TX power | 40 | 30 | 23 |
| C | DL subband to UL subband leakage ratio | -28 | -28 | -28 |
| D | Assumed spatial isolation | -85 | -85 | -85 |
| E | TX leakage power in UL subband = B+C+D | -73.0 | -83.0 | -90.0 |
|  | Interference-to-noise ratio (INR) = E - A | 18.0 | 8.0 | 1.0 |

It can be observed that

23 dBm BS power level

Direct TX self-leakage does not cause appreciable performance degradation (i.e., desensitization) if 85 dB spatial isolation is achievable for these small BS; the interference level is quite close to the noise floor.

30 dBm BS power level

If unmitigated, direct TX self-leakage alone can cause 8.6 dB of desensitization / coverage loss even with 85 dB spatial isolation (INR = 8.0 dB). Two possible mitigation solutions can be considered.

- The TX RF frontend linearity is improved (e.g., by increasing the DPD complexity) to further reduce the DL subband to UL subband leakage ratio. The complexity, cost, and feasibility of this solution at FR2 requires further analysis.

- Depending on the architecture, e.g., number of digital ports, digital cancellation/suppression of leaked interference can be considered, as long as the RF frontends are not saturated, e.g., due to the power level in the DL subbands.

40 dBm BS power level

Direct TX self-leakage alone causes approximately 18 dB desensitization / coverage loss even with 85 dB spatial isolation since the interference level is significantly higher than the noise level (INR = 18 dB). This would require solutions in the following two areas.

- The TX RF frontend linearity is improved to further reduce the DL subband to UL subband leakage ratio by more than 20 dB. The feasibility, complexity, cost and power consumption of this solution is highly questionable for FR2.

- Depending on the architecture, e.g., number of digital ports, digital cancellation/suppression of leaked interference can be considered, if the RX analog/ RF frontends are not saturated, e.g., due to the power level in the DL subbands.

## 4.2	Spectrum regrowth due to gNB Rx non-linearity

In Section 2.4.1 of our companion paper [1], we remark that a reasonable assumption on IIP3 of BS receiver low noise amplifier (LNA) for FR2 is -35 dBm. Assuming a spatial isolation of 85 dB, one can estimate the distortion power to the UL subband when SBFD is deployed as in the following table. Since there are DL subbands on the two sides of a UL subband, we make a simplifying assumption that all the IM3 power calculated from one blocker falls into the UL subband.

Table 11 Example IM3 distortion in the UL subband caused by SBFD DL subbands assuming 85 dB spatial isolation and -35 dBm IIP3 for the BS LNA.

|  |  | 40 dBm BS | 30 dBm BS | 23 dBm BS |
| --- | --- | --- | --- | --- |
| A | Noise Floor | -91.0 | -91.0 | -91.0 |
| B | TX power | 40 | 30 | 23 |
| D | Assumed spatial isolation | -85 | -85 | -85 |
| F | TX power in DL subbands = B+D | -45.0 | -55.0 | -62.0 |
| G | Assumed IIP3dB | -35 | -35 | -35 |
| H | Gain normalized IM3 power in UL subband =3*F-2*G | -65.0 | -95.0 | -116.0 |
|  | Interference-to-noise ratio (INR) = H - A | 26.0 | -4.0 | -25.0 |

One can observe that

23 dBm and 30 dBm BS power levels

With an assumption of 85 dB spatial isolation, the DL subbands do not cause significant IMD to the UL subband when SBFD is deployed using typical LNA. For these BSs, the IM3 powers in the UL subband are below the noise floor, and desensitization of the receiver due to IM3 is small

40 dBm BS power level

Due to the higher transmit power for this BS power level, the resulting IMD distortion to the UL subband is 26 dB above the noise floor resulting in very large desense. To reduce this to a reasonable level, a very large upgrade of the IIP3dB would be needed (at least 15 dB improvement). The feasibility and cost of such a solution is questionable at FR2.

## 4.3	Reciprocal mixing of phase noise

In Section 2.4.2 of our companion paper [1], we make estimates of the interference power from the DL subbands to the UL subband caused by reciprocal mixing of phase noise using the phase noise models discussed in RAN4 Rel-17. It can be observed that the interference power from the DL subbands to the UL subband is around -45 to -55 dBc depending on the implementation of the BS. The impact to the SBFD system can be estimated in the following table:

Table 12 Example reciprocal mixing distortion in the UL subband caused by SBFD DL subbands assuming 85 dB spatial isolation and example phase noise models. (All powers are gain normalized.)

|  |  | 40 dBm BS | 30 dBm BS | 23 dBm BS |
| --- | --- | --- | --- | --- |
| A | Noise Floor | -91.0 | -91.0 | -91.0 |
| B | TX power | 40 | 30 | 23 |
| D | Assumed spatial isolation | -85 | -85 | -85 |
| F | TX power in DL subbands = B+D | -45.0 | -55.0 | -62.0 |
| I | Interference from reciprocal mixing of phase noise | -44.0 | -49.5 | -54.5 |
| J | Gain normalized interference power in UL = F+I | -89.0 | -104.5 | -116.5 |
|  | Interference-to-noise ratio (INR) = J - A | 2.0 | -13.5 | -25.5 |

It can be observed that

23 dBm and 30 BS power levels

BS receiver phase noise is not an issue for the 23 dBm BS power level since the ICI power is well below the noise level, and thus does not result in appreciable desensitization.

40 dBm BS power level

Due to the higher transmit power for this BS power level, the mixer phase noise is 2 dB above the noise floor resulting in approximately 4 dB desense. To reduce this, two possibilities are the following:

- Improve the down-conversion LO quality. With a 10 dB improvement in phase quality, the densensitization can be reduced to 0.5 dB; however, such an improvement may require increased implementation complexity and operating energy consumption

- Another option is to investigate whether these reciprocal mixing distortions can be removed using digital cancellation solutions, if the RX analog/ RF frontends are not saturated, e.g., due to the power level in the DL subbands.

## 4.4	Feasibility observations for a single-carrier, single-sector BS

From the above analysis of the different sources of interference due to SBFD operations, we can make some general observations on implementation feasibility/challenges for the different BS power levels with different max power levels as well as antenna array sizes for the case of single-carrier, single-sector BSs.

23 dBm BS power level

For a low power BS power level, such as 23 dBm, the above identified interference powers are generally below the noise floor with enough margin if spatial isolation of 85 dB is achieved. It is likely that a single isolated lower power BS can operate in the SBFD mode using existing hardware components and without the need of digital self-interference cancellation.

A low power BS such as 23 dBm, can likely operate in SBFD mode in FR2 using existing hardware components without digital self-IC if spatial isolation of 85 dB is achieved.

30 dBm BS power level

For the 30 dBm BS power level, we identified in the above that interference due to direct Tx-to-Rx leakage is several dB above the noise floor. Without addressing this interference power, the reception of the SBFD UL subbands would be desensitized by an unacceptable amount. To reduce the desensitization and realize the coverage benefits of operating in the SBFD mode, two different interference mitigation approaches can be considered:

- The TX RF frontend linearity is improved (e.g., by increasing the DPD complexity) to further reduce the DL subband to UL subband leakage ratio. The complexity, cost, and feasibility of this solution at FR2 requires further analysis.

- Depending on the architecture, digital cancellation/suppression of leaked interference can be considered, as long as the RF frontends are not saturated, e.g., due to the power level in the DL subbands.

For a 30 dBm BS in FR2, in a single-carrier, single-sector deployment, interference mitigation solutions such as (1) improved Tx linearity, (2) hardware component upgrades to avoid spectrum regrowth caused by RX non-linearities, or (3) digital self-interference cancellation, or combination of all three are needed to realize the coverage benefits of SBFD if 85 dB spatial isolation is achieved.

40 dBm BS power level

For the 40 dBm BS power level, we identified in the above that both the interference power due to direct Tx-to-Rx leakage and that interference due to the non-linear LNA are significantly higher than the noise floor. These two effects together result in unacceptable receiver desensitization erasing potential coverage benefits of SBFD unless they are addressed. To reduce the desensitization, the following mitigation approaches would be needed:

- Hardware component upgrade

  - The TX RF frontend linearity is improved to further reduce the DL subband to UL subband leakage ratio by more than 20 dB. The feasibility, complexity, cost and power consumption of this solution is questionable for FR2.

  - The Rx frontend linearity is significantly improved to reduce the IM3 that falls into the UL subband. The feasibility and cost of such a solution is questionable.

- Digital self-interference cancellation/suppression

  - Digital cancellation of leaked interference can be considered, if the RX analog/ RF frontends are not saturated, e.g., due to the power level in the DL subbands.

For 40 dBm BS power level in FR2, interference powers due to SBFD operation are significantly higher than the noise floor even with 85 dB spatial isolation, even in a single-carrier, single-sector deployment. Significant hardware component upgrades and/or digital self-interference cancellation/suppression would be needed to mitigate the interference. The the cost and power consumption needs to be carefully considered.

## 4.5	Consideration of multi-sector base station sites

In Section 2.2.4 of [1] we provide inter-sector isolation results for a simplified 3-sector site including vertically separated Tx and Rx panels as would occur for a deployment with antennas built for SBFD operation (see Figure 21 copied from [1] below). We find that the isolation varies significantly depending on the azimuth and elevation beam steering directions, as well as the frequency within the band. The variation range of the panel-to-Rx-port coupling magnitude is -72 to -95 dB with -85 dB being typical. We also observe that other realistic effects that are not modeled in our simulations (for simplicity reasons) are likely to degrade isolation further, for example:

- re-radiation from the electronics on the backside of the antennas

- practical site aspects such as equipment and other metallic objects in between sectors that can act as PIM sources

- presence of sub-arrays (the norm in AAS products), that result in higher side lobes and grating lobes, thus reducing achievable isolation.

- presence of radomes

The above effects are not necessarily solved simply by moving antennas further apart.

Figure 21: Simplified 3-sector site setup for FR2 considered for inter-sector EM simulations (see [1])

Based on the simulation results from Section 2.2.4 of [1] we choose a panel-to-Rx-port inter-sector isolation value of 85 dB in the following analysis, which in itself may still be optimistic. At this level, inter-sector self-interference causes challenges to the 40 dBm and 30 dBm BS power levels as shown in the following tables for the following interference sources:

- The inter-sector direct TX leakage to UL subband

- IM3 interference in UL subbands induced by the inter-sector received power in DL subbands

Note that an LA BS is not typically used in a three-sector setup, so we omit the inter-sector analysis for this BS type. Note that the inter-sector antenna leakage values in the following tables include a factor of 3 dB to account for the factor that inter-sector interference is received from two co-site sectors in a 3-sector deployment

Table 13 Inter-sector direct TX leakage to UL subband

|  |  | 40 dBm BS | 30 dBm BS | 23 dBm BS |
| --- | --- | --- | --- | --- |
| A | Noise Floor | -91.0 | -91.0 | -91.0 |
| B | TX power | 40 | 30 | 23 |
| C | DL subband to UL subband leakage ratio | -28 | -28 | -28 |
| K | Assumed inter-sector antenna leakage | -85 | -85 | N/A |
| L | Inter-sector received power in UL subband = B+C+K | -73 | -83 | N/A |
|  | Interference-to-noise ratio (INR) = L - A | 18 | 8 | N/A |

Table 14 Example IM3 distortion in the UL subband induced by inter-sector received power in DL subbands assuming typical existing BS LNA components.

|  |  | 40 dBm BS | 30 dBm BS | 23 dBm BS |
| --- | --- | --- | --- | --- |
| A | Noise Floor | -91.0 | -91.0 | -91.0 |
| B | TX power | 40 | 30 | 23 |
| K | Assumed inter-sector antenna leakage | -85 | -85 | N/A |
| M | Inter-sector received power in DL subbands = B+K | -45 | -55 | N/A |
| G | Assumed IIP3dB | -35 | -35 | N/A |
| N | Gain normalized IM3 in UL subband = 3*M-2*G | -65.0 | -95.0 | N/A |
|  | Interference-to-noise ratio (INR) = N - A | 26.0 | -4.0 | N/A |

Table 15 Example reciprocal mixing distortion in the UL subband induced by inter-sector received power in DL subbands assuming example phase noise models. (All powers are gain normalized.)

|  |  | 40 dBm BS | 30 dBm BS | 23 dBm BS |
| --- | --- | --- | --- | --- |
| A | Noise Floor | -91.0 | -91.0 | -91.0 |
| B | TX power | 40 | 30 | 23 |
| K | Assumed inter-sector antenna leakage | -85 | -85 | N/A |
| M | Inter-sector received power in DL subbands = B+K | -45 | -55 | N/A |
| I | Interference from reciprocal mixing of phase noise | -44.0 | -49.5 | -54.5 |
| O | Gain normalized interference power in UL = M+I | -89.0 | -104.5 | N/A |
|  | Interference-to-noise ratio (INR) = O - A | 2.0 | -13.5 | N/A |

It can be observed that both the receivers for the 30 dBm and 40 dBm BS power levels are affected by strong interference powers leaked from adjacent sectors.

- From Table 13, one can see that the 40 dBm BS power level suffers from strong interference due to inter-sector direct leakage into the UL subband creating large desensitization. The desensitization for the 30 dBm BS power level is less severe, but still significant. For the 40 dBm BS power level, improving Tx linearity alone is not a viable solution given the amount of improvement needed.

- From Table 14, one can see that for the 40 dBm BS power level, the intermodulation distortion created by the LNA in the receive chain is significantly above the noise floor, creating significant desensitization. In order to reduce the desensitization to below 1 dB, roughly 15 dB improvement in the LNA IIP3 would be needed. This would create challenges from a power consumption and heat management perspective for an FR2 radio that typically includes a very large number of antenna elements. Very little improvement would be needed for the 30 dBm BS power level.

- From Table 15, one can see that for the 40 dBm BS power level, reciprocal mixing interference in the UL subband induced by inter-sector received power in the DL subbands is 2 dB above the noise floor, several dB improvement in the phase quality of the LO used for downconversion. No improvement is needed for the 30 dBm BS power level.

With inter-sector panel-to-Rx-port isolation of 85 dB for FR2, the receiver for 40 dBm BS power level suffers from strong interference power leaked from adjacent sectors creating very large desensitization. For the 30 dBm BS power level, the desensitization is less severe, but still significant.

## 4.6	Feasibility conclusions for FR2

Based on the extensive analysis in the previous sections, we draw conclusions on the feasibility of SBFD for different BS power levels based on consideration of at least the following:

- Self-interference from direct Tx-to-Rx leakage within a gNB antenna array

- Spectral regrowth due to non-linearities in the gNB Rx chains and requirements on hardware component upgrades

- Interference due to reciprocal mixing of phase noise

- Inter-sector interference

- Size, power consumption, and heat management issues

For 40 dBm BS power level in FR2, the feasibility of SBFD is questionable given requirements on significant hardware upgrades which translate to issues on power consumption, heat management, and cost.

For 30 dBm and 23 dBm BS power levels in FR2, assuming 85 dB spatial isolation is achieved, the feasibility of SBFD appears more promising at least for single-sector, single-carrier deployments. For 30 dBm BS power level, small improvements may be needed for Tx linearity and/or modest requirements are needed for digital self interference cancellation/supression.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Dynamic SBFD (dSBFD) Options 1 and 2 do not offer a performance advantage compared to the considerably simpler Dynamic TDD (dTDD) system.

Observation 2	For both legacy and new (Rel-18+) UEs in IDLE mode, it is feasible that DL reception  (i.e., during initial access) can occur in only one of the 'D' subbands within SBFD slots/symbols configured as D-U-D. By "feasible," it is meant that assuming a certain minimum channel bandwidth, CORESET0 "fits" within a single 'D' subband in a symmetric D-U-D configuration in either FR1 or FR2. For 48 RB CORESET0, at least 40/100/200 MHz channel bandwidth is required for 15/30/120 kHz SCS.

Observation 3	For legacy UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets, while reception of others is restricted to a single 'D' RB set.

Observation 4	For new (Rel-18+) UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets according to current specifications, while reception of others is restricted to a single 'D' RB set unless enhancements are made.

Observation 5	For both legacy and new (Rel-18+) UEs in IDLE mode, UL transmissions (i.e., during initial access) should occur only within UL-only slots.

Observation 6	For legacy UEs in CONNECTED mode, UL transmissions can occur in symbols configured as ‘U’ or ‘F’ by TDD-UL-DL-ConfigCommon as per current specifications without changes.

Observation 7	To avoid excessive receiver desensitization, the total interference due to SBFD operation from all sources (residual self-interference, IMD due to Rx non-linearities, interference due to reciprocal mixing of phase noise, ADC quantization noise, etc.) needs to be maintained well below the noise/reference sensitivity level for the gNB receiver.

Observation 8	Digital self-interference cancellation solutions are more feasible for lower power BS with small coverage than for high power BS with massive MIMO capabilities.

Observation 9	A low power BS such as the representative local area (LA) BS class, can likely operate in SBFD mode in FR1 using existing hardware components without digital self-IC if spatial isolation of 75 dB is achieved.

Observation 10	For a medium power BS such as the representative medium range (MR) BS class for FR1, in a single-carrier, single-sector deployment, interference mitigation solutions such as (1) hardware component upgrades to avoid spectrum regrowth caused by RX non-linearities, or (2) digital self-interference cancellation, or combination of (1) and (2), are needed even with 80 dB spatial isolation.

Observation 11	For mitigating interference due to SBFD operation, the cost, complexity, energy consumption, and heat dissipation scales (1) linearly with the number of TX/RX chains for mitigation through hardware component upgrades, and (2) quadratically with the number of TX/RX chains for mitigation through digital self-interference cancellation.

Observation 12	For a high-power BS, such as the representative wide area (WA) BS class in FR1, interference powers due to SBFD operation are far above what current typical WA BS hardware is designed for even with 80 dB spatial isolation, even in a single-carrier, single-sector deployment. Either of the SBFD related interference mitigation approaches discussed for the medium range (MR) BS class would result in excessive cost, complexity, energy consumption, and heat management.

Observation 13	With inter-sector panel-to-Rx-port isolation of 72 dB, the receivers for WA and MR BS classes suffer from strong interference powers leaked from adjacent sectors far above the current BS operation specs.

Observation 14	Installation of isolation enhancing material between sectors of one operator and/or between antennas of different operators with antennas at different heights on a tower/pole would face significant practical challenges, and does not appear feasible.

Observation 15	The implementation complexity and difficulty of sharp analog filtering that is suitable separating DL and UL subbands with a few RBs of guard frequencies in a SBFD carrier depends strongly on the operating frequency at which such analog filtering takes place. Such analog filtering is more feasible with homodyne type receivers but becomes increasingly infeasible with the heterodyne or direct RF sampling type receivers.

Observation 16	For high power BS (e.g., WA BS class) in FR1, the feasibility of SBFD in a modern multi-carrier, multi-band, AAS architecture is highly questionable given the excessive requirements on (1) hardware component upgrades which translate to significant issues on product size, power consumption, and heat management, and (2) site solutions for mitigation of co-site interference. Alternative approaches can be considered for WA deployments that do not rely on simultaneous transmission and reception at the gNB, e.g., opportunistic TDD operation with equivalent amount of UL resources as SBFD.

Observation 17	For low and medium power BS (e.g., LA and MR BS classes) in FR1, assuming 80 and 75 dB spatial isolation is achieved, respectively, the feasibility of SBFD appears more likely at least for single-sector, single-carrier deployments for which there are lower demands on hardware component upgrades and mitigation of self-interference compared to WA BS class. Further study under realistic assumptions is needed in the area of multi-sector, multi-carrier, and multi-band deployments.

Observation 18	A low power BS such as 23 dBm, can likely operate in SBFD mode in FR2 using existing hardware components without digital self-IC if spatial isolation of 85 dB is achieved.

Observation 19	For a 30 dBm BS in FR2, in a single-carrier, single-sector deployment, interference mitigation solutions such as (1) improved Tx linearity, (2) hardware component upgrades to avoid spectrum regrowth caused by RX non-linearities, or (3) digital self-interference cancellation, or combination of all three are needed to realize the coverage benefits of SBFD if 85 dB spatial isolation is achieved.

Observation 20	For 40 dBm BS power level in FR2, interference powers due to SBFD operation are significantly higher than the noise floor even with 85 dB spatial isolation, even in a single-carrier, single-sector deployment. Significant hardware component upgrades and/or digital self-interference cancellation/suppression would be needed to mitigate the interference. The the cost and power consumption needs to be carefully considered.

Observation 21	With inter-sector panel-to-Rx-port isolation of 85 dB for FR2, the receiver for 40 dBm BS power level suffers from strong interference power leaked from adjacent sectors creating very large desensitization. For the 30 dBm BS power level, the desensitization is less severe, but still significant.

Observation 22	For 40 dBm BS power level in FR2, the feasibility of SBFD is questionable given requirements on significant hardware upgrades which translate to issues on power consumption, heat management, and cost.

Observation 23	For 30 dBm and 23 dBm BS power levels in FR2, assuming 85 dB spatial isolation is achieved, the feasibility of SBFD appears more promising at least for single-sector, single-carrier deployments. For 30 dBm BS power level, small improvements may be needed for Tx linearity and/or modest requirements are needed for digital self interference cancellation/supression.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not support SBFD operation in symbols configured as 'U' by TDD-UL-DL-ConfigCommon.

Proposal 2	RAN1 to conclude that Option 1 is the baseline, and Option 2 is not further pursued.

Proposal 3	For Option 1, only UL transmissions are allowed within an UL subband. In other words, DL receptions outside DL subband(s) are not allowed.

Proposal 4	RAN1 to conclude that Options 2, 3, and 4 in the RAN1#109 agreement are no longer pursued.

Proposal 5	For semi-static configuration of SBFD subband time locations within a period, the periodicity is configured by the existing DL-UL transmission periodicity parameter defined in TDD-UL-DL-ConfigCommon

Proposal 6	Within an SBFD symbol, a single UL subband is configured as an RB set, and one or two guardband sizes are configured in terms of a number of RBs. The guardband RBs are contiguous and adjacent to the UL RB set.

Proposal 7	Dynamic indication of the size and/or frequency location of the UL subband in SBFD symbols is not supported.

Proposal 8	For new (Rel-18+) UEs supporting SBFD operation within a TDD carrier, do not support misaligned center frequencies between DL and UL BWPs.

Proposal 9	RAN1 should model practical transmitter and receiver components to enable a realistic assessment of the effect of UL/DL timing misalignment at the gNB.

Proposal 10	UEs in IDLE mode are not aware of whether or not symbols/slots are used for SBFD operation.

Proposal 11	For new (Rel-18+) UEs, it is beneficial to support enhancements to CSI-RS resource configuration and CLI-RSSI measurement resource configuration to allow non-contiguous frequency domain resource allocation, i.e., in both 'D' RB sets.

Proposal 12	If SBFD symbol overlaps an SS/PBCH block symbol, a UE shall not transmit in the UL subband of this symbol.

Proposal 13	For new (Rel-18+) UEs in CONNECTED mode, study enhancements to frequency domain resource allocation and frequency hopping mechanisms for PUSCH and PUCCH configured with repetition in order to allow repetitions to occur in both SBFD and UL-only slots.

Proposal 14	TR 38.858 should contain feasibility analysis for multiple BS classes with different transmit power limits and reference sensitivity levels. For example, the following three representative BS classes defined in RAN4 can be used for FR1: Local Area (LA), medium range (MR), and wide area (WA).

Proposal 15	For the feasibility analysis for the LA and MR BS classes, use the RAN4 defined power limits 24 and 38 dBm, respectively. For WA, use the 53 dBm power limit agreed for system simulations in this study item. For all BS classes use the corresponding RAN4 defined reference sensitivity levels: -87.6 dBm, -90.6 dBm, and -95.6 dBm for LA, MR, and WA, respectively.

Proposal 16	RAN1/4 to study solutions, performance and applicability for SBFD operations in multi-carrier and/or multi-band base stations.

Proposal 17	For UL coverage, throughput, and latency improvements, RAN1 should study alternatives to SBFD operation, namely opportunistic TDD operation without configuration of UL subbands. Such alternatives do not require complicated gNB hardware, and do not suffer from self-interference, inter-sector interference of its own network.

Proposal 18	TR 38.858 should contain feasibility analysis for multiple BS power levels. For example, the following three representative power levels can be used for FR2: 40 dBm, 30 dBm, 23 dBm.

# References

R1-2300907, "Evaluation of NR duplex evolution," Ericsson, RAN1#112, February 2023.

TR 38.858, " Study on Evolution of NR Duplex Operation," v.0.0.2, May 2022.

R1-2211943, "Potential enhancements of dynamic TDD," Ericsson, RAN1#111, November 2022.

R1-2210042, “On subband non-overlapping full duplex for NR,” Nokia, RAN#110bis-e, October 2022.

R1-2205897, “Discussion on subband non-overlapping full duplex,” Huawei, RAN#110, August 2022.

R1-2206170, “Views on Subband non-overlapping Full Duplex,” Fujitsu, RAN1#110, August 2022.

R1-2207364, “Study on Subband non-overlapping full duplex,” LG Electronics, RAN1#110, August 2022.

R4-2215384, “Further discussion of the interference modelling for duplex evolution SLS,” CATT, RAN4#104-e, August 2022.

# Appendix A: Parameters for system level evaluations

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Scenario | UMa, Hexagonal layout, 7 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 500 m |
|  | Carrier Frequency | 4 GHz |
|  | Duplex Type | sTDD, SBFD, dSBFD, dTDD |
|  | Channel bandwidth | 100 MHz |
|  | Available resource blocks | 273 |
|  | Guard periods | None |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 840 (7 sites * 3 sectors/site * 40 per site (20 UL + 20 DL)) 7000 |
|  | Channel model | gNB-UE: UMa gNB-gNB: 75% if 2D distance < ISD; otherwise according to gNB-UE LOS probability in TR 38.901. UE-UE: UMi |
|  | UE to BS min 2D distance | 25 m |
| BS | (Mg, Ng, M, N, P), where M, N indicate sub-array number | (1,1,4,8,2) (same antenna gain) (1,1,4,4,2) (same antenna area) |
|  | Sub-array configuration | 3x1 |
|  | Max gNB Tx Power (per polarization) | 53 dBm |
|  | (dv, dh) | (0.7λ, 0.5λ) |
|  | Antenna element gain | 6.4 dBi |
|  | Antenna element and sub-array model | R4-2109872, Table 1-2: Macro urban |
|  | Subarray electrical downtilt | 3 deg |
|  | Mechanical downtilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Noise figure | 5 dB |
|  | Max modulation | 256 QAM |
|  | BS height | 25 m |
|  | Panel HW assumptions | Same antenna gain for all systems, i.e., double area for SBFD and dSBFD |
| UE | UE antenna | 1TX 2RX |
|  | Antenna model | Isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
|  | UE power control | Sec. 9.1 TR36.942 |
|  | SNR target | 16 dB |
|  | Noise figure | 9 dB |
|  | Max modulation | 64 QAM |
|  | UE distribution outdoor:indoor | 20% outdoor 80% indoor |
| Traffic | Traffic model | FTP3, 0.5 Mbytes for DL and 0.125 Mbytes for UL |
|  | Target Resource utilization | <10%, 25-35%, 55% |
| Spatial isolation | Tx è Rx within a sector | 80 dB |
|  | Tx è Rx between sectors of the same site | 90 dB |