3GPP TSG RAN WG1 #112  	                                 R1-2301022

Athens, Greece, February 27th – March 3rd, 2023

Source: 	CMCC

Title:	Discussion on disabling of HARQ feedback for IoT NTN

Agenda item:	9.11.3

Document for:	Discussion & Decision

- Introduction

During last RAN1 meeting, the respective configurations of disabling of HARQ feedback in IoT NTN have been discussed, and some progress were achieved [1].

In this contribution, we will further discuss and share our views on disabling of HARQ feedback for IoT NTN, including the detailed configuration and other enhancements on disabling of HARQ feedback for IoT NTN.

- Discussion

  - 2.1 Configuration of disabling HARQ feedback

Disabling of HARQ feedback is beneficial to throughput improvement and latency reduction. In addition, disabling of HARQ feedback may especially bring more benefits to the GEO network scenarios (e.g. GEO with RTT about 500 ms) due to the transmission waiting time is reduced.

For the disabling of HARQ feedback in IoT NTN, one key issue is the configuration of enabling/disabling HARQ feedback for both eMTC and NB IoT NTN. Several options from different perspectives were discussed during previous RAN1 meetings, and the working assumption on reusing RRC signalling for CE Mode A and using two merged options for NB-IoT NTN and eMTC NTN for CE Mode B were achieved in the last RAN1 meeting [1].

RAN1#110 Agreement For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options: - Option 1: per HARQ process via UE specific RRC signaling. - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field). - Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS) - Option 6: combinations of some options above. RAN1#110 Agreement For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options: - Option 1: per HARQ process via UE specific RRC signaling - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field) - Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS) - Option 6: combinations of some options above RAN1#110b-e Agreement For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select ONE from the following options at RAN1#111: - Option 6a-1: Support RRC signaling configured between Option 1 and Option 3 - Option 6a-4: Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission RAN1#111 Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission - Additional RRC signaling to enable Option 3 - If the bitmap for option 1 is not present and if option 3 is configured then the DCI directly indicates HARQ enable/disable. Option 3 can also be configured when the bitmap for option 1 is configured. - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes or only applied to semi-statically HARQ disabled processes or only applied to semi-statically HARQ enabled processes. - FFS #2: whether/how to support Option 3 overriding default configuration for corresponding transmission for multiple TBs scheduled by single DCI For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Considering the configuration of enabling/disabling HARQ feedback, since there are concerns on MAC CE activation issue and other important message for NB IoT NTN and eMTC NTN for CE Mode B with very limited number of HARQ process, especially for single HARQ process, it is recommended that the DCI indication method should also be taken into consideration to provide more flexibility, which can enable the HARQ feedback for important RRC signalling and MAC CE message, and RRC configuration is considered as the default configuration.

For both NB IoT NTN and eMTC NTN UE, the bitmap to indicate the configuration per HARQ process in NR NTN can be reused for RRC-based mechanism. Moreover, for NB-IoT NTN and eMTC NTN for CE Mode B, if additional RRC signalling indicates to enable DCI to override the semi-static configuration of enabling/disabling HARQ feedback, the issue and detailed configuration of DCI-based overridden mechanism needs to be discussed.

From our perspective, DCI-based mechanism can be enabled to support dynamic enabling/disabling configuration for the HARQ process. Considering that disabling of HARQ feedback is beneficial to throughput improvement and latency reduction, it is reasonable to also apply the DCI-based overridden mechanism on semi-statically HARQ enabled process, which can provide more flexibility and avoid the latency caused by RRC reconfiguration.

Proposal 1. For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, DCI-based overridden mechanism can be applied to both semi-statically HARQ enabled and disabled processes.

For the detailed configuration of indication in DCI to indicate the enabled/disabled HARQ feedback, we prefer not to introduce additional DCI field to to minimize the standard impact. Considering the limited number of HARQ process of NB-IoT NTN and eMTC NTN for CE Mode B, i.e., one or two HARQ process, it is reasonable to reinterpret the existing DCI filed, such as HARQ-ACK resource field in DCI format N1 to indicate enabling/disabling of HARQ feedback for NB IoT NTN. If the RRC signalling indicates to enable the DCI-based mechanism, then one of the value in the existing HARQ-ACK resource field can be predefined to indicate enabling/disabling of HARQ feedback for the HARQ process. Otherwise, the DCI field is used as the legacy manner.

Proposal 2. When DCI-based overridden mechanism is enabled by RRC signalling, the existing DCI field, such as the indicator in HARQ-ACK resource field, can be reinterpreted to indicate the enabling/disabling of HARQ feedback for the HARQ process.

  - 2.2 (N)PDSCH/(N)PDCCH scheduling restriction

  - The NPDSCH scheduling restriction in NB-IoT NTN was discussed during previous RAN1 meetings, and it is agreed that for a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH in a period of Y=12(ms) from the end of reception of the NPDSCH.

  - The PDSCH/PDCCH scheduling restriction in eMTC NTN also needs to be studied by considering eMTC UE characteristics. The PDSCH scheduling restriction has been discussed in NR NTN and several agreements for NTN were achieved as follows [3].

- RAN1#104-e Agreement For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. - Working assumption: X = T_proc,1 - FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0 - Note: The TB of the two PDSCHs can be either same or different RAN1#105-e Agreement Confirm the previous working assumption for X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH for the given HARQ process.

  - Considering the PDSCH/MPDCCH scheduling restriction for eMTC NTN UEs, from our perspective, the restriction of receiving another PDCCH after the last PDSCH agreed in Rel-17 NR NTN can be considered as baseline, and it is clarified that the minimum gap between the end of PDSCH and the start of corresponding HARQ-ACK is 3 ms for eMTC, the 3 ms gap value can be considered for the restriction in eMTC NTN. Therefore, the restriction can be imposed to eMTC UE in NTN not to receive another MPDCCH carrying a DCI scheduling a PDSCH for a given HARQ process that starts until 3(ms) after the end of the reception of the last PDSCH for that HARQ process.

  - Proposal 3. For a DL HARQ process with disabled HARQ feedback in eMTC, UE is not expected to receive another MPDCCH carrying a DCI scheduling a PDSCH for a given HARQ process that starts until 3 ms after the end of the reception of the last PDSCH for that HARQ process.

  - 2.3 SPS PDSCH

- The configuration of HARQ feedback enabling/disabling for SPS PDSCH also needs to be discussed for IoT NTN. This issue also has been discussed in Rel-17 NR NTN, and the agreed configuration of SPS PDSCH in Rel-17 NR NTN are shown as following [3].

RAN1#107-e Agreement HARQ feedback for SPS activation may be additionally enabled by the network by RRC configuration. - If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of whether HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation - Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation, - FFS between Alt1 and Alt2 - [Alt-1: UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process - Alt-2: UE follows the feedback-enabled/disabled configuration of the SPS PDSCH] RAN1#108-e Agreement - For HARQ feedback of each SPS PDSCH, UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process, except for the first SPS PDSCH after activation if HARQ feedback for SPS activation is additionally enabled.

- Considering the configuration of SPS PDSCH transmission in IoT NTN, it is preferred to reuse the agreed configuration of SPS PDSCH in Rel-17 NR NTN. That is the per HARQ process configuration can be used.

Proposal 4. For HARQ feedback of each SPS PDSCH in IoT NTN, UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process, except for the first SPS PDSCH after activation if HARQ feedback for SPS activation is additionally enabled.

- Conclusions

In this contribution, we share our views on the issues of disabling of HARQ feedback for IoT NTN. The observations and proposals are summarized as follows:

Proposal 1. For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, DCI-based overridden mechanism can be applied to both semi-statically HARQ enabled and disabled processes.

Proposal 2. When DCI-based overridden mechanism is enabled by RRC signalling, the existing DCI field, such as the indicator in HARQ-ACK resource field, can be reinterpreted to indicate the enabling/disabling of HARQ feedback for the HARQ process.

  - Proposal 3. For a DL HARQ process with disabled HARQ feedback in eMTC, UE is not expected to receive another MPDCCH carrying a DCI scheduling a PDSCH for a given HARQ process that starts until 3 ms after the end of the reception of the last PDSCH for that HARQ process.

Proposal 4. For HARQ feedback of each SPS PDSCH in IoT NTN, UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process, except for the first SPS PDSCH after activation if HARQ feedback for SPS activation is additionally enabled.

- References

- Chairman's Notes RAN1#111.

- Chairman's Notes RAN1#110bis-e.

- R1-2202910, “3GPP TSG-RAN WG1 Agreements under 8.4 up to eMeeting RAN1#108-e”, February 21th - March 3rd, 2022.