3GPP TSG-RAN WG1 Meeting #112	R1-2301081

# Athens, Greece, February 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | draft | rev | _ | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the availability of a soft RB set in an RB set group in TS38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2023-2-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are inconsistent descriptions related to “RB set” in Table 14-3, for example, the content related to “RB set” is described as “soft symbols or soft RB sets in an RB set group” in some places, but “soft symbols or RB set group” in other places. In addition, using “RB set group” is not accurate because one RB set group may consist of hard/soft/NA RB sets, but it is unnecessary to consider Hard/NA RB sets in an RB set group in Table 14-3. Therefore, we suggest changing “RB set group” in “soft symbols or RB set group” as “soft RB sets in an RB set group”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The “RB set group” in “soft symbols or RB set group” in Table 14-3 is modified to “ soft RB sets in an RB set group”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The “soft symbols or RB set group” in Table 14-3 causes misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

14 Integrated access-backhaul operation

<Unchanged parts are omitted>

An AI index field value in a DCI format 2_5 indicates to an IAB-DU a soft symbol or a soft RB set in an RB set group availability in each slot for a number of slots starting from the earliest slot of the IAB-DU which overlaps in time with the slot of the IAB-MT where the IAB-MT detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace. The AI index field includes maxlog2maxAIindex+1,1 bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol or a soft RB set in an RB set group in a slot is identified by a corresponding value resourceAvailability as provided in Table 14-3.

Table 14-3: Mapping between values of resourceAvailability elements and types of soft symbol or soft RB set availability in a slot

| Value | Indication |
| --- | --- |
| 0 | No indication of availability for soft symbols or soft RB sets in an RB set group |
| 1 | DL soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for UL and Flexible soft symbols or soft RB sets in an RB set group |
| 2 | UL soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for DL and Flexible soft symbols or soft RB sets in an RB set group |
| 3 | DL and UL soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for Flexible soft symbols or soft RB sets in an RB set group |
| 4 | Flexible soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for DL and UL soft symbols or soft RB sets in an RB set group |
| 5 | DL and Flexible soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for UL soft symbols or soft RB sets in an RB set group |
| 6 | UL and Flexible soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for DL soft symbols or soft RB sets in an RB set group |
| 7 | DL, UL, and Flexible soft symbols or soft RB sets in an RB set group are indicated available |

<Unchanged parts are omitted>