3GPP TSG RAN WG1 #112		                                   					       												                  R1-2301221

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	8.16.2

Source:	Moderator (AT&T)

Title:	Summary of UE features topics 2

Document for: 	Discussion/Decision

# Introduction

This document presents the summary of email discussion [112-R17-UE_features_2] during RAN1 #112.

[112-R17-UE_features_2] To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc – Ralf (AT&T)

The following was discussed during RAN1 #112 within the scope of [112-R17-UE_features_2]. All proposals are based on the latest RAN1 NR UE features lists for Rel-17 in [1].

# Summary of Contributions Submitted to RAN1 #112

The following is the moderator’s summary of contributions submitted to RAN1 #112 in this agenda item.

## NR_FeMIMO

Void

## NR_ext_to_71GHz

Void

## NR_NTN_solutions

Void

## IoT over NTN

Void

## NR_IAB_enh

Void

## NR_DSS

Void

## LTE_NR_DC_enh2

Void

## NR_pos_enh

| Company | Summary |
| --- | --- |
| Samsung [2] | In Rel-16, a UE can perform timing measurements for up to two additional paths in addition to the first path. In Rel-17, reporting for multiple paths was extended by adding UE capabilities to support timing reporting for K>2 additional paths (27-13 and 27-14). In addition, it was also agreed to report RSRPP measurement for first path and additional paths. However, based on current UE capability list and signaling, reporting RSRPP for 1 or 2 additional paths is not possible. With regards to FG 27-13 and FG 27-14, the candidate number of additional path reporting for DL-TDOA and Multi-RTT is {4, 6, 8} for timing are assumed to be the same for RSRPP report. In Rel-16, the additional path timing report for K=1 or 2 was supported for DL-TDOA and multi-RTT. The signaling for supporting additional paths is provided in IE additionalPathsReport-r16 as shown in the table below (a capability defined by RAN2), and the number of additional paths K=1, 2 for timing report is provided in IE nr-AdditionalPathList-r16 by the UE to the LMF. Features; Index; Feature group; Components; Field name in TS 37.355 [9]; Parent IE in TS 37.355 [9]; Mandatory/Optional. 16. NR_pos-Core; 16-1; Additional paths reporting; Indicates whether the UE supports additional paths reporting for Multi-RTT or DL-TDOA; additionalPathsReport-r16; NR-Multi-RTT-ProvideCapabilities-r16 or NR-DL-TDOA-ProvideCapabilities-r16 LPP; Optional with capability signalling. However, the RSRPP reporting was introduced in Rel-17, and there is no RSRPP reporting in Rel-16. Thus, currently, we have the following for Rel-17 UEs: - Ability to report timing measurements for {1, 2, 4, 6, 8} additional paths, with 1 and 2 paths being supported by Rel-16 capabilities, and {4, 6, 8} paths being supported by Component 1 in 27-13 and 27-14 - Ability to report RSRPP for {4, 6, 8} additional paths with indication given by Component 2 in 27-13 and 27-14, where the maximum number of additional paths for RSRPP measurement is not the same as that for timing measurement It is not possible to report RSRPP measurement for 1 or 2 additional paths. Thus, there is a mismatch on the number of additional paths between the timing reporting and RSRPP reporting. However, RAN1 has reached the agreement on support RSRPP for all paths as following: Agreement: Support reporting the path RSRP for the first path and for additional paths as part of DL-TDOA, UL-TDOA, and multi-RTT reporting enhancements. · FFS: Support introducing a request from the LMF to the UE/TRP when the path-RSRP for additional paths is desired to be reported. · FFS: Support of path RSRP for additional paths as part of DL-AoD. Thus, we think such support is missing from current specification and UE capability report, and it ends up with not supporting the UE implementation that wants to keep the Rel-16 capability (e.g., up to 2 additional paths) but with the new RSRPP report. As in current signaling in TS37.355: additionalPathsExtSupport-r17 ENUMERATED { n4, n6, n8 } OPTIONAL, additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-DL-TDOA-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-DL-TDOA-MeasurementCapability also applies to the additional paths.. Similar description appears for Multi-RTT as well. The support of the first path RSRP applies to the additional paths but here the additional paths only contain the value of n4, n6, n8. In addition, if the reported max value of the additional paths is different between timing report and the RSRPP report, we think adding one note could solve the issue, i.e., constraining the two numbers to be the same. Thus, we can avoid dealing with the error case where different values have been picked by UE for the reported max value of the additional paths for timing and RSRPP reporting. Proposal 1: Support of RSRPP reporting for K=1 and 2 additional paths for DL-TDOA and Multi-RTT. 27. NR_pos_enh; 27-13; Additional path reporting for UE-assisted DL-TDOA; 1. Support of additional detected path timing reporting for K>2 additional paths for UE-assisted DL-TDOA 2. Support of RSRPP reporting for additional paths if UE supports FG 27-13a; Additional path reporting for UE-assisted DL-TDOA is not supported; Per UE; Component 1 candidate values: {4, 6, 8} Component 2 candidate values: {1, 2, 4, 6, 8} Note: the reported value of the additional paths should be same for the timing report and the RSRPP report. Need for location server to know if the feature is supported.; Optional with capability signaling.. 27. NR_pos_enh; 27-14; Additional path reporting for Multi-RTT; 1. Support of additional detected path timing reporting for K>2 additional paths for Multi-RTT 2. Support of RSRPP reporting for additional paths if UE supports FG 27-14a; Additional path reporting for Multi-RTT is not supported; Per UE; Component 1 candidate values: {4, 6, 8} Component 2 candidate values: {1, 2, 4, 6, 8} Note: the reported value of the additional paths should be same for the timing report and the RSRPP report. Need for location server to know if the feature is supported.; Optional with capability signaling.. For DL-TDOA additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-DL-TDOA-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-DL-TDOA-MeasurementCapability also applies to the additional paths (including the number of additional paths supported in Rel-16 and Rel-17).. For Multi-RTT additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-Multi-RTT-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-Multi-RTT-MeasurementCapability also applies to the additional paths (including the number of additional paths supported in Rel-16 and Rel-17).. |

## NR_DL1024QAM_FR1

Void

# Discussion Items during RAN1 #112

After review of contributions submitted to RAN1 #112 in this agenda item, the following topics were identified by the moderator for discussion during RAN1 #112.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

## NR_FeMIMO

Void

## NR_ext_to_71GHz

Void

## NR_NTN_solutions

Void

## IoT over NTN

Void

## NR_IAB_enh

Void

## NR_DSS

Void

## LTE_NR_DC_enh2

Void

## NR_pos_enh

After review of contributions submitted to RAN1 #112 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal:

- Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-13 | Additional path reporting for UE-assisted DL-TDOA | 1. Support of additional detected path timing reporting for K>2 additional paths for UE-assisted DL-TDOA 2. Support of RSRPP reporting for additional paths if UE supports FG 27-13a |  | No |  | Additional path reporting for UE-assisted DL-TDOA is not supported | Per UE | No | No | No | Component 1 candidate values: {4, 6, 8} Component 2 candidate values: {1, 2, 4, 6, 8} Note: the reported value of the additional paths should be same for the timing report and the RSRPP report. Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-14 | Additional path reporting for Multi-RTT | 1. Support of additional detected path timing reporting for K>2 additional paths for Multi-RTT 2. Support of RSRPP reporting for additional paths if UE supports FG 27-14a |  | No |  | Additional path reporting for Multi-RTT is not supported | Per UE | No | No | No | Component 1 candidate values: {4, 6, 8} Component 2 candidate values: {1, 2, 4, 6, 8} Note: the reported value of the additional paths should be same for the timing report and the RSRPP report. Need for location server to know if the feature is supported. | Optional with capability signaling. |

- Include the following TPs (changes highlighted in chromatic fonts) into the LS to RAN2 that accompanies every updated RAN1 UE features list for Rel-17 NR

  - For DL-TDOA

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-DL-TDOA-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-DL-TDOA-MeasurementCapability also applies to the additional paths (including the number of additional paths supported in Rel-16 and Rel-17).

  - For Multi-RTT

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-Multi-RTT-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-Multi-RTT-MeasurementCapability also applies to the additional paths (including the number of additional paths supported in Rel-16 and Rel-17).

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We think the issue is valid. To make things clearer, it is better to have some clarification. From our view, for a UE supporting Rel-16 FG 16-1 and Rel-17 FG 27-13a or 14a, it is naturally that UE can support RSRPP measurement for the K=1 or 2 additional paths. This is similar as the relationship between 27-13 and 13a. Hence, we think it is sufficient to send an LS to RAN2, such as the following. There is no need to introduce new components for FG 27-13 and 27-14 Proposal: Send an LS to RAN2 that UE supporting Rel-16 FG 16-1 and Rel-17 FG 27-13a/14a should support RSRPP reporting for the K=1 or 2 additional paths. |
| Huawei, HiSilicon | We acknowledge that without any clarification, the current specification does not support UE to indicate its capability of only supporting RSRPP reporting for up to 2 additional paths. In general, we do not think this change is needed, because the request message from LMF is not well optimized for the related UE capabilities. If the change is necessary, we do not think that the change proposed by SS is backward compatible both from ASN.1 or from function perspective. The change from ZTE suggests to re-purpose of first path RSRPP reporting (27-13a/27-14a) as indication of support of up to 2 additional path RSRPP reporting, which we do not believe is backward compatible. Therefore, if such a change is needed, we prefer to add another FG. 27. NR_pos_enh; 27-13b; Up to 2 additional paths RSRPP reporting for UE-assisted DL-TDOA; Support of RSRPP reporting of up to 2 additional paths for UE-assisted DL-TDOA. Note: UE supporting this feature shall not support 27-13.; 16-1. 27. NR_pos_enh; 27-14b; Up to 2 additional paths RSRPP reporting for Multi-RTT; Support of RSRPP reporting of up to 2 additional paths for Multi-RTT. Note: UE supporting this feature shall not support 27-14.; 16-1. |
| Qualcomm | We prefer to not make such change at this point. We should also avoid non-backward compatible changes to the UE capabilities as HW/HiSi pointed out. Either a new FG or SS proposal could be made BC from ASN1 perspective eventually, so we can accept a change if majority wants and assuming RAN2 makes sure It is being implemented using an BC change. |
| CATT | It seems that no need to make the change. If the majority wants to make the things clear, we can live with the new FGs from HW above. |

## NR_DL1024QAM_FR1

Void

# Summary of Agreements

This Section summarizes the final agreements in RAN1 #112 in this agenda item [3].

Agreement:

- Send an LS to RAN2 that UEs supporting Rel-16 FG 16-1 and Rel-17 FGs 27-13a/14a should support RSRPP reporting for K=1 or 2 additional paths.

- From RAN1 perspective, no ASN.1 impact is foreseen.

# References

- R1-2212895, Updated RAN1 UE features list for Rel-17 NR after RAN1 #111, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2301243, Discussions on Rel-17 UE features for NR positioning on additional path reporting, Samsung

- R1-2302008, Session Notes of AI 8.16.2, Ad-Hoc Chair (AT&T)