3GPP TSG RAN WG1 #112		R1- 2301466

Athens, Greece, 27 February – 03 March 2023

Agenda item:	9.2.2.1

Source:		Southeast University (SEU)

Title:	Evaluation of AI/ML based methods for CSI feedback enhancement

Document for:	Discussion and Decision

1.Introduction

In RAN 94-e meeting, new study item on AI/ML for NR Air Interface was approved, the initial set of use cases were identified [1]:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

A DL-based time-correlation aided implicit CSI feedback network named ImplicitNet-LSTM was proposed in [2] to evaluate AI/ML for CSI feedback enhancement in spatial-frequency-time domain.

In this paper, we further provide the evaluation result for ImplicitNet-LSTM.

2.Simulation Results

Based on the discussion in [2], we conduct a numerical simulation to evaluate the performance of the proposed DL-based implicit feedback architecture.

The dataset used is generated by QuaDRiGa. Initial position of UE is randomly generated in a circle with a radius of 100m from BS, and motion track of UE is straight line motion in random direction from the initial position. Speed of UE is set to 10m/s, 20m/s, 35m/s.

Table 1: Simulation assumption of dataset

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2.1GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| Antennas | 32T4R |
| Rank | 1 |
| Subband | 12 |

First matrix’s feedback bits is 8，the others are 4，hence the total feedback overhead is (8+3*4)/4*12=60

Datasets with speed of 10m/s, 20m/s and 35m/s are used for testing separately, and the results are recorded as 𝜌10，𝜌20，𝜌35.

Table 2: Recover performance of ImplicitNet-LSTM on UMi-NLOS Dataset

| Schemes | Feedback Bits | ρ10 | ρ20 | ρ35 |
| --- | --- | --- | --- | --- |
| ImplicitNet-LSTM | 60 | 0.918 | 0.888 | 0.887 |
| ImplicitNet | 48 | 0.850 | 0.831 | 0.830 |
|  | 96 | 0.879 | 0.861 | 0.860 |
|  | 144 | 0.903 | 0.888 | 0.886 |
|  | 192 | 0.917 | 0.906 | 0.904 |

Table 3: Recover performance of ImplicitNet-LSTM on UMa-NLOS Dataset

| Schemes | Feedback Bits | ρ10 | ρ20 | ρ35 |
| --- | --- | --- | --- | --- |
| ImplicitNet-LSTM | 60 | 0.916 | 0.886 | 0.886 |
| ImplicitNet | 48 | 0.852 | 0.834 | 0.836 |
|  | 96 | 0.877 | 0.865 | 0.863 |
|  | 144 | 0.916 | 0.889 | 0.887 |
|  | 192 | 0.918 | 0.907 | 0.903 |

Table 4: Recover performance of ImplicitNet-LSTM on UMi-LOS Dataset

| Schemes | Feedback Bits | ρ10 | ρ20 | ρ35 |
| --- | --- | --- | --- | --- |
| ImplicitNet-LSTM | 60 | 0.981 | 0.960 | 0.958 |
| ImplicitNet | 48 | 0.942 | 0.923 | 0.921 |
|  | 96 | 0.956 | 0.938 | 0.933 |
|  | 144 | 0.969 | 0.946 | 0.946 |
|  | 192 | 0.979 | 0.951 | 0.950 |

Observation 1: In implicit feedback, time correlation can be used to improve network performance.

Observation 2: ImplicitNet-LSTM still outperform ImplicitNet when the feedback overhead is approximately reduced by 58%.

Observation 3: ImplicitNet-LSTM outperforms well on all three datasets.

Proposal #1: Framework performance on should be evaluated on datasets with different scenario and configuration.

Moreover, the effect of training sample number are evaluated. The dataset with the speed of 20m/s and channel model UMi-NLOS is adopted as an example.

Table 5: Effect of training sample number

| Sample Numbers(k) Framework | 300 | 600 | 900 | 1200 | 1500 | 1800 | 2100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ImplicitNet-LSTM (60 feedback bits) | 0.839 | 0.857 | 0.867 | 0.879 | 0.888 | 0.888 | 0.889 |
| ImplicitNet (144 feedback bits) | 0.870 | 0.878 | 0.886 | 0.888 | 0.889 | 0.888 | 0.888 |

Observation 4: More training samples are required to achieve the best performance for ImplicitNet-LSTM.

Proposal #2: The scale of training datasets should be regarded as an important indicator to evaluate the framework performance.

4.Conclusion

In this contribution, the performance of ImplicitNet-LSTM is evaluated.

- Proposal 1: Framework performance on should be evaluated on datasets with different scenario and configuration.

- Proposal 2: The scale of training datasets should be regarded as an important indicator to evaluate the framework performance.

5.Reference

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R1-2207475 “Evaluation on AI/ML for CSI feedback enhancement in spatial-frequency-time domain”  SEU