3GPP TSG-RAN WG1 #112	 R1- 2301677

Athens, Greece, February 27th-march 3rd, 2023

Agenda Item:	8.16.1

Source:	Ericsson

Title:	UE capabilities for NR MBS

Document for:	Discussion, Decision

- Introduction

This contribution provides our view on the list of UE features for NR MBS in release 17,  based on the  updated RAN1 UE features list for Rel-17 NR captured in in R1-2212895 [1].

- UE Features for NR MBS

### Number of GRNTIs for FG 33-2

when FG 33-2 is signaled, the components do not state how many GRNTIs may be used.  In broadcast, a component exists and states a single GRNTI is supported for broadcast reception. We propose to use similar wording for 33-2.

add the following component to FG 33-2:

- One G-RNTI per UE is supported for multicast reception

## FGs for HARQ support (33-3-x)

### Consequence if FG 33-3-3a/b is not supported by the UE

The two FGs details have been agreed during RAN1#111 but the column for consequence if not supported need to be clarified. Our view is that at least one of the two FGs have to be supported in order to support multiplexing of unicast and multicast in the same type-2 codebook. If one of the  feature is not supported, what differs is how type 1 is supported.

Therefore we propose the following for the description of the UE behaviour in case the FGs are not supported:

For the UE behaviour when FG 33-3-3a or FG 33-3-3b is not supported:

  - If 33-3-3a is not supported, FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH are not supported

  - If FG 33-3-3b is not supported, Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH is not supported

  - If neither are supported, multiplexing of type-1 unicast feedback  and type 1 multicast feedback in a type1 codebook is not supported, and multiplexing of type-2 unicast feedback  and type 2 multicast feedback in a type2 codebook is not supported

  - If either are supported, muxing unicast and multicast in type 2  HARQ feedback is supported.

## FGs for SPS (33-5-x and 33-9-x)

During the past meetings, we discussed including additional components in FG for SPS 33-5-1 as follow:

High priority proposal 2-11-2:

- Components of FG 33-5-1 are revised as

- Add a component “Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast”

- Add a component “Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast”

- Add a component “ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI”

The last component is controversial for some companies, with the argument that HARQ for SPS is captured in FG 33-5-1a. however, HARQ for the SPS activation and release and HARQ for SPS PDSCH transmissions are separately specified. For SPS activation and release, HARQ is not optional. The following agreement was struck at RAN1#107e:

Agreement For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported.

We therefore support that the basic FG for SPS must support HARQ response for activation and release, and support the original proposal by the UE feature FL. Similarly, FG 33-5-3 for SPS in SCell should be modified accordingly.

  - Components of FG 33-5-1 are revised as

      - Add a component “Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast”

      - Add a component “Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast”

      - Add a component “ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI”

  - In component 1 in 33-5-1, remove the following text: [SPS group-common PDSCH activation, and SPS release PDCCH]

  - Components of FG 33-5-3 are revised as

      - Add a component “Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast”

      - Add a component “Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast”

      - Add a component “ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI”

  - In component 1 in 33-5-1, remove the following text: [SPS group-common PDSCH activation, and SPS release PDCCH]

Regarding the reporting of FGs 33-5-1e/f/g/i, and 33-5-2 we propose following the template of 33-5-1h and use per UE with FDD/TDD , licensed unlicensed, NTN/TN and FR1/2 differentiation

FG 33-5-1e/f/g/i and 33-5-2 are reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation.

For the support of SPS and NACK only (FG 33-5-1f and 1j),  we discussed adding the clarifications that multiple TBs HARQ feedback  is transmitted by transforming NACK only to ACK NACK, transmitted over PUCCH or over PUSCH.  With think such description is necessary to avoid confusion between the FGs in 33-5-1f and 33-5-1j.  additionally, there were some comments that NACK only+ SPS should be limited to 2TBs, but that only applies when the HARQ bits are mapped to a single PUCCH resource in our view, i.e. for 33-5-1j.

In FG 33-5-1f, add the following components:

  - One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits

  - One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI

In FG 33-5-1j, add the following components:

  - Single TB with NACK-only feedback transmitted in PUCCH

  - Up to 2TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits

Finally, in 33-9-1,  the discussion in RAN1#111 also remained open with choices between reporting per UE (alt1), BC(alt3), or per UE with differentiation for FDD/TDD, NTN, licensed/unlicensed, and FR1/FR2 (alt2).  In our view, Alt2 is a good compromised considering the pre-requisites.

FG 33-9-1 is reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation.

## FGs for separate MBS PUCCH (33.8.x)

During RAN1#111, we discussed whether to add FG 33-4 as prerequisite for 33-8-1. In our view, 33-8-1 is the capability for separate PUCCH configuration for multicast ACK NACK feedback, i.e., the capability signals the UE ability to receive a configuration in pucch-ConfigurationListMulticast1. Therefore, FG 33-4-1 should not be a prerequisite for 33-8-1. However, the description of 33-8-1 could be extended to include NACK-only mode 1, since the same kind of PUCCH resource is required for both ACK NACK and NACK only mode 1. Alternatively, a separate capability for the configuration of a distinct PUCCH resource for NACK only in pucch-ConfigurationListMulticast2 when moreThanOneNackOnlyMode is not configured can be configured.

Regarding the configuration of a separate PUCCH for NACK only mode 1 feedback, select either of the following:

  - In FG 33-8-1, the first  component is modified as follow: Support of a PUCCH config for multicast HARQ ACK or NACK only feedback, separate from that of unicast configurations.

    - Note: NACK only refers to the mode supported as described in FG 33-4.

    - Prerequisite include FG 33-4

  - A new FG 33.8.1a is introduced as follow:

| 33-8-1a | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast with NACK only feedback with ACK NACK transforming | Support of a PUCCH-Config for multicast NACK only feedback, separate from that of unicast configurations, where feedback, with multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits | 33-4 | Yes |  |  | [Per band ] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Regarding the signalling of the capability, we think that per band would be enough, since the pre-requisite capability already gives per BC granularity.

In FG 33-8-1, the signalling is per BC, and the pre-requisite FG is kept to 33-2a.

# Conclusion

We have proposed to update the MBS UE feature list with the following:

Proposal 1	add the following component to FG 33-2:

One G-RNTI per UE is supported for multicast reception

Proposal 2	For the UE behaviour when FG 33-3-3a or FG 33-3-3b is not supported:

-	If 33-3-3a is not supported, FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH are not supported

-	If FG 33-3-3b is not supported, Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH is not supported

-	If neither are supported, multiplexing of type-1 unicast feedback  and type 1 multicast feedback in a type1 codebook is not supported, and multiplexing of type-2 unicast feedback  and type 2 multicast feedback in a type2 codebook is not supported

-	If either are supported, muxing unicast and multicast in type 2  HARQ feedback is supported.

Proposal 3

a.	Components of FG 33-5-1 are revised as

▪	Add a component “Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast”

▪	Add a component “Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast”

b.	In component 1 in 33-5-1, remove the following text: [SPS group-common PDSCH activation, and SPS release PDCCH]

Proposal 4

a.	Components of FG 33-5-3 are revised as

▪	Add a component “Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast”

▪	Add a component “Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast”

b.	In component 1 in 33-5-1, remove the following text: [SPS group-common PDSCH activation, and SPS release PDCCH]

Proposal 5	FG 33-5-1e/f/g/i and 33-5-2 are reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation.

Proposal 6	In FG 33-5-1f, add the following components:

a.	One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits

b.	One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI

Proposal 7	In FG 33-5-1j, add the following components:

a.	Single TB with NACK-only feedback transmitted in PUCCH

b.	Up to 2TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits

Proposal 8	FG 33-9-1 is reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation.

Proposal 9	Regarding the configuration of a separate PUCCH for NACK only mode 1 feedback, select either of the following:

a.	In FG 33-8-1, the first  component is modified as follow: Support of a PUCCH config for multicast HARQ ACK or NACK only feedback, separate from that of unicast configurations.

i.	Note: NACK only refers to the mode supported as described in FG 33-4.

ii.	Prerequisite include FG 33-4

b.	A new FG 33.8.1a is introduced as follow:

Proposal 10	In FG 33-8-1, the signalling is per BC, and the pre-requisite FG is kept to 33-2a.

- References

[1] R1-2212895, Updated RAN1 UE features list for Rel-17 NR after RAN1#111, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#111.