3GPP TSG RAN WG1 #114			R1-2307295

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.9.3

Source:	Apple

Title:	On HARQ Feedback Disabling for IoT NTN

Document for:	Discussion/Decision

# Introduction

In the IoT NTN work item, one of the RAN1 objectives is disabling of HARQ feedback. In RAN1 #113 meeting [1], [2], the following has been agreed on the topic of disabling HARQ feedback for IoT NTN.

Working assumption

For DCI-based direct indication in single TB scheduled by DCI,

- Indication by reusing/reinterpreting HARQ-ACK related field in DCI

    - For eMTC CE mode B, one state of “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource.

      - FFS: detailed state, and whether this state is different across different UEs

    - For NBIoT, one state of “HARQ-ACK resource” field in DCI format N1 is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource.

      - FFS: detailed state, and whether this state is different across different UEs

- If reusing/reinterpreting HARQ-ACK related field in DCI is also used for DCI overriding scheme, the interpretation of the state can be different than for DCI-based direct indication.

Agreement

For single TB scheduled by DCI,

- Working assumption 1 DCI based overridden indication is applied to both semi-statically HARQ disabled and enabled processes

  - For DCI based overridden indication, adopt indication by reusing/reinterpreting HARQ-ACK related field in DCI

    - For eMTC CE mode B, “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled.

      - HARQ feedback disabled is reversed to enabled in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is maintained as disabled.

      - HARQ feedback enabled is maintained in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is reversed to disabled.

        - FFS: detailed state A, and whether this state A is different across different UEs

    - For NBIoT, “HARQ-ACK resource” field in DCI format N1 is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled.

      - The same DCI indication functionality as eMTC is adopted.

- Working assumption 2 For Option 1 + Option 3 DCI based overridden mechanism, for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the NBIoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process (or monitoring any NPDCCH for the case of single HARQ process configuration).

- Send an LS to RAN2 with the following contents:

  - RAN1 respectfully ask RAN2 for the feasibility of Working assumption 2 (taking into account potential RAN2 spec impact).

Agreement

For the RRC configuration of DCI solution enabling/disabling of HARQ feedback for NB-IoT and LTE-MTC in CE Mode B, the RRC configuration is UE-specific.

Agreement

for NB-IoT and LTE-MTC in CE Mode B, if multiple TBs is configured, for DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduled by single DCI, the same indication is applied to all scheduled TBs, i.e. HARQ is enabled or disabled for all TBs.

In this contribution, we provide our views on HARQ disabling for IoT NTN.

# Discussion

## Indication/configuration of disabling HARQ feedback for multiple TBs scheduled by single DCI

It was working assumption [1] that for an NB-IoT UE or an eMTC CE Mode B UE, both DCI-based direct indication and DCI-based overridden indication reuse or reinterpret HARQ-ACK related field in DCI. Specifically, one state in “HARQ-ACK resource offset” field of DCI format 6-1B (for eMTC CE Mode B) and one state in “HARQ-ACK resource” field of DCI format N1 (for NB-IoT) are used to indicate the HARQ feedback disabling. It is open whether this state is different across different UEs. In our view, the HARQ-ACK resource offset field or HARQ-ACK resource field is configured in cell-specific way (i.e., Table 10.1.2.1-1, Table 16.4.2-1 or Table 16.4.2-2 of [3]). Hence, it is straightforward to config a common state for all UEs.

Proposal 1: For NB-IoT NTN and eMTC NTN for CE Mode B, the state in “HARQ-ACK resource” field in DCI format N1 and the state in “HARQ-ACK resource offset” field in DCI format 6-1B to indicate HARQ feedback disabling are common for all UEs.

## HARQ feedback for single DCI scheduling multiple TBs

In IoT, it is supported that a single DCI scheduling multiple TBs. If a subset of these multiple TBs are associated with HARQ processes configured with feedback disabling, then it simply assumes ACK for those TBs in the feedback. This applies to both the case with HARQ bundling and the case without HARQ bundling for a unified design and less specification impact.

Proposal 2: For both NB-IoT and eMTC NTN, if a single DCI schedules multiple TBs, then ACK is assumed/reported for the downlink transmission with feedback disabled HARQ processes. This applies to both the case with HARQ bundling and the case without HARQ bundling.

# Conclusion

In this contribution, we provided our views on HARQ feedback disabling for IoT NTN. Our proposals are as follows:

Proposal 1: For NB-IoT NTN and eMTC NTN for CE Mode B, the state in “HARQ-ACK resource” field in DCI format N1 and the state in “HARQ-ACK resource offset” field in DCI format 6-1B to indicate HARQ feedback disabling are common for all UEs.

Proposal 2: For both NB-IoT and eMTC NTN, if a single DCI schedules multiple TBs, then ACK is assumed/reported for the downlink transmission with feedback disabled HARQ processes. This applies to both the case with HARQ bundling and the case without HARQ bundling.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 #113 Meeting, May 2023.

- R1-2306021, “FLS#2 on disabling of HARQ feedback for IoT NTN,” May 2023.

- 3GPP TS36.213, Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures, v17.5.0, Mar. 2023.