3GPP TSG RAN WG1 #114			   R1-2308335

Toulouse, France, August 21st – 25th, 2023

Agenda item:	7.2

Source: 	Moderator (ZTE)

Title: 	Moderator Summary #1 for Maintenance on Rel-17 Multi-Beam

Document for:	Discussion and Decision

## Introduction

The following in Section 2 and Section 3 is assigned for discussion on maintenance on normal and editorial issues of Rel-17 Multi-Beam. Please provide your comments in corresponding sections.

## Summary of normal issues

### Issue 1-1 Correction to power Control parameters for unified TCI framework (R1-2307450)

| Reason for change: | In RAN1#112bis-e, RRC parameter mismatch of p0 for PUCCH/SRS in Rel-17 unified TCI framework is corrected in Clause 7. For the editors: - The following text proposals are provided to improve the clarity of the RAN1 specifications. Please include them in the Rel-17 alignment CRs - Proposal-1 in Section 4 in R1-2304061 is endorsed as in alignment CR in TS 38.213 - Proposal-2 in Section 4 in R1-2304061 is endorsed as in alignment CR in TS 38.213 - Proposal-3 in Section 4 in R1-2304061 is endorsed as in alignment CR in TS 38.214 R1-2304230 CR on value range mismatch of p0 for SRS in Rel-17 unified TCI framework Moderator (ZTE), NTT DOCOMO, INC., Ericsson, ZTE, Samsung However, there are still wrong RRC parameters in Clause 6. The value range of P0 for PUCCH in p0AlphaSetforPUCCH and P0 for SRS in p0AlphaSetforSRS is (-16..15) dB, and it fits for the configuration of “differential target power”, e.g., PO_UE_PUCCH,b,f,cqu for PUCCH and PO_UE_SRS,b,f,cqs for SRS. |
| --- | --- |
| Summary of change: | Change RRC parameter name of P0 for PUCCH and SRS in Clause 6 from PO_PUCCH,b,f,cqu to PO_UE_PUCCH(qu) and from PO_PUCCH,b,f,cqu to PO_UE_SRS,b,f,cqs. FL-note: There is a typo of above PO_PUCCH,b,f,cqu should be PO_SRS,b,f,cqs: |
| Consequences if not approved: | Unintended transmit power is used for PUCCH and SRS after BFR in case unified TCI is configured. |

Due to above, the following draft CR is provided in R1-2307450:

----------------------------------------------------------------------------------------------

6	Link recovery procedures

*** Unchanged parts are omitted ***

If a UE is provided dl-OrJointTCI-StateList or ul-TCI-StateList indicating a unified TCI state for the PCell or the PSCell [6, TS 38.214], after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE

-	if SSB-MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH, using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and the PUCCH, using a same spatial domain filter as for the last PRACH transmission using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the value ofPO_PUCCH,b,f,cquPO_UE_PUCCHqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the values ofPO_SRS,b,f,cqsPO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

*** Unchanged parts are omitted ***

If a UE is provided dl-OrJointTCI-StateList or ul-TCI-StateList indicating a unified TCI state for the PCell or the PSCell and the UE provides BFR MAC CE in Msg3 or MsgA of contention based random access procedure, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in [11, TS 38.321], the UE

-	if SSB-MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and PUCCH, using a same spatial domain filter as for the last PRACH transmission using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the value of PO_PUCCH,b,f,cquPO_UE_PUCCHqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the values ofPO_SRS,b,f,cqsPO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

*** Unchanged parts are omitted ***

If a UE is provided dl-OrJointTCI-StateList or ul-TCI-StateList indicating a unified TCI state, after 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	if SSB -MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, on the SCell (s) indicated by the MAC CE, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and PUCCH, using a same spatial domain filter as the one corresponding to qnew, if any, and using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the corresponding SCell

-	the value of PO_PUCCH,b,f,cquPO_UE_PUCCHqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the corresponding SCell

-	the values ofPO_SRS,b,f,cqsPO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the corresponding Scell

*** Unchanged parts are omitted ***

----------------------------------------------------------------------------------------------

Please provide company’s view in the table below.

| - Company | - Comment |
| --- | --- |
| - Mod_V00 | FL note: Above is to handle left-over RRC parameter mismatch of p0 for PUCCH/SRS in Rel-17 unified TCI framework, although progress in the endorsed CR R1-2304230 and alignment CR was made in RAN1#112-bis-e. - Please provide your views for this issue, and then do you have any further views on the draft CR, if identifying the essentiality from your side. |
| - vivo | - fine |
| - Docomo | - Support. Since both parameters (P0_xx and P0_UE_xx) exist in TS38.213, we believe it is necessary to correct the parameter name to avoid miss understanding. |
| - Nokia | - Support as editorial. |
| - Ericsson | - Support as editorial |
| - Huawei, Hisilicon | - Support as editorial |
| - CATT | - Fine |
| - LG | - Fine with editorial issue |
| - Mod_V09 | FL note: Let’s try to go with the following as in an alignment CR, although, in my initial thought, a normal CR may be much more suitable considering that the update may be not straightforward - Proposal-1: To endorse the following as in alignment CR in TS 38.213. ---------------------------------------------------------------------------------------------- 6 Link recovery procedures *** Unchanged parts are omitted *** If a UE is provided dl-OrJointTCI-StateList or ul-TCI-StateList indicating a unified TCI state for the PCell or the PSCell [6, TS 38.214], after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE - if SSB-MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH, using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any - transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and the PUCCH, using a same spatial domain filter as for the last PRACH transmission using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1 - the RS index qd=qnew for obtaining the downlink pathloss estimate - the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell - the value ofPO_PUCCH,b,f,cquPO_UE_PUCCHqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell - the values ofPO_SRS,b,f,cqsPO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the PCell or the PSCell *** Unchanged parts are omitted *** If a UE is provided dl-OrJointTCI-StateList or ul-TCI-StateList indicating a unified TCI state for the PCell or the PSCell and the UE provides BFR MAC CE in Msg3 or MsgA of contention based random access procedure, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in [11, TS 38.321], the UE - if SSB-MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any - transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and PUCCH, using a same spatial domain filter as for the last PRACH transmission using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1 - the RS index qd=qnew for obtaining the downlink pathloss estimate - the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell - the value of PO_PUCCH,b,f,cquPO_UE_PUCCHqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell - the values ofPO_SRS,b,f,cqsPO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the PCell or the PSCell *** Unchanged parts are omitted *** If a UE is provided dl-OrJointTCI-StateList or ul-TCI-StateList indicating a unified TCI state, after 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE - if SSB -MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, on the SCell (s) indicated by the MAC CE, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any - transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and PUCCH, using a same spatial domain filter as the one corresponding to qnew, if any, and using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1 - the RS index qd=qnew for obtaining the downlink pathloss estimate - the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the corresponding SCell - the value of PO_PUCCH,b,f,cquPO_UE_PUCCHqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the corresponding SCell - the values ofPO_SRS,b,f,cqsPO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the corresponding Scell *** Unchanged parts are omitted *** ---------------------------------------------------------------------------------------------- - . |

## Summary of editorial (E) issues

Companies are to share their inputs on the editorial CR for the following issues herein.

### Issue 2-1

- R1-2307038	Correction to power Control parameters for unified TCI framework	CATT

Table 1 Companies’ inputs

| - Company | - Comment |
| --- | --- |
| - Mod_V00 | - Above seems fine, although, alternatively, it may be better if we only use ‘being associated with’ for all cases of PL-RS and UL-PC setting. It seems that ‘being included in’ can be covered by ‘being associated with’ without any ambiguities, and then efforts on differentiating ‘being included in’ or ‘being associated with’ in RRC level may NOT be necessary. - Please provide your views as follows. |
| - vivo | fine with the part of correction on pathlossRS-Id, but it is unnecessary to modify the part of power control parameter settings(P0, alpha, closed loop index). As the parameter ul-powercontrol-r17 included in the BWP-UL-dedicated is not associated with the indicated TCI state or TCI-UL-State. And there is no ambiguity for the original description of “associated with”. |
| - Docomo | - We are fine with the part of correction on pathlossRS-Id. - For the other part, we don’t think it is essential correction. We think the current spec. has no ambiguity. |
| - OPPO | - The part of deleting “associated with” is fine since we did agree that path loss is included in a TCI state. - but the parts of adding “or included” are not needed because it would just add some abundant description and furthermore, the description in original agreement is “TCI state … associated with power control parameters”. |
| - Nokia | - Neither change is essential! |
| - Ericsson | - Not essential. There is no risk of misunderstanding. |
| - Huawei, Hisilicon | - (P0, alpha, closed loop index): Do not change. - pathlossReferenceRS-Id-r17: Fine to change but not essential |
| - CATT | - Support the changes. |
| - LG | - Fine with the correction on pathlossRS-Id |
| - Mod_V09 | - FL note: Majority companies seem to be fine with the update for psthlossRS-Id, and please review the following FL proposal. - Proposal-2: To endorse the following as in alignment CR in TS 38.213. ---------------------------------------------------------------------------------------------- 7 Uplink Power control *** Unchanged parts are omitted *** In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State and for an indicated TCI-State or TCI-UL-State as described in [6, TS 38.214] - in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS - in clause 7.1.1, if p0AlphaSetforPUSCH is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State - in clause 7.2.1, if p0AlphaSetforPUCCH is provided, the values of PO_UE_PUCCHqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH associated with the indicated TCI-State or TCI-UL-State - in clause 7.3.1, if p0AlphaSetforSRS is provided, - if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set *** Unchanged parts are omitted *** ---------------------------------------------------------------------------------------------- |

## Conclusion

# References

| 1 | R1-2307038 | Correction to power Control parameters for unified TCI framework | CATT |
| --- | --- | --- | --- |
| 2 | R1-2307450 | Draft CR on correction of RRC parameters of p0 for PUCCH/SRS in Rel-17 unified TCI framework | NTT DOCOMO, INC. |