3GPP TSG RAN WG1 #118			R1-2406091

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:		9.4.2.1

Source:	China Telecom

Title:	Discussion on general aspects of physical layer design for Ambient IoT

Document for:		Discussion

Introduction

The study item on solutions for Ambient IoT (Internet of Things) was approved in RAN#102 meeting [1] and revised in RAN#103 meeting [2]. In last RAN1#117 meeting [3], the following agreements were achieved for general aspects of physical layer design for Ambient IoT as follows:

Agreement Study the following regarding CP location/length determination for Method Type 1: - Alt 1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt 2: duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Companies are encouraged to clarify the CP removal method used and implementation aspects for the device - Evaluations are encouraged to be performed for a small value of M, e.g. 4 and a large value of M, e.g. 24, at least by comparison to the case where the CP length of each OFDM symbol is known by device - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement Study the following options regarding subcarrier orthogonality for Method Type 2: - Alt 1: Method Type 2 retains subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol) - Alt 1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same - FFS: whether this alternative applies if CP length is longer than the chip duration - Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP - Other potential methods are not precluded - Alt 2: Method Type 2 does not retain subcarrier orthogonality - Proponents to bring further details to RAN1#118 - Evaluations and discussions are encouraged to be performed for a small value of M, e.g. M = 4 and a large value of M, e.g. M = 24. - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement The study assumes the following bit to chip mapping for Manchester encoding: - bit 0→chips{10}, bit 1→chips{01} - FFS: Variant of the above for CP handling

In this contribution, we further discuss the remaining issues on general aspects of physical layer design for Ambient IoT.

Discussion on general aspects of physical layer design for Ambient IoT

  - Waveform

CP handling for R2D

In last RAN1#117 meeting, the following agreements on CP handling for R2D were achieved [3].

Agreement Study the following regarding CP location/length determination for Method Type 1: - Alt 1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt 2: duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Companies are encouraged to clarify the CP removal method used and implementation aspects for the device - Evaluations are encouraged to be performed for a small value of M, e.g. 4 and a large value of M, e.g. 24, at least by comparison to the case where the CP length of each OFDM symbol is known by device - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement Study the following options regarding subcarrier orthogonality for Method Type 2: - Alt 1: Method Type 2 retains subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol) - Alt 1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same - FFS: whether this alternative applies if CP length is longer than the chip duration - Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP - Other potential methods are not precluded - Alt 2: Method Type 2 does not retain subcarrier orthogonality - Proponents to bring further details to RAN1#118 - Evaluations and discussions are encouraged to be performed for a small value of M, e.g. M = 4 and a large value of M, e.g. M = 24. - Companies should report the values of SFO, and SFO detection methods used in evaluations

Regarding the two method types for potential down-selection, the key point is how to remove CP at device side. The reader and device implementation complexities should be studied further. Maybe no CP can be also supported. For method type 1, considering the different CP lengths in different OFDM symbol index supported in NR system (see details in TS 38.211), it requires A-IoT devices to know detailed OFDM symbol index and then to determine the CP location. The CP length in first OFDM symbol is larger than the other OFDM symbols in a slot. Hence, it needs A-IoT devices to know the exact OFDM symbol index, which would bring more implementation complexity and unexpected mistakes due to large initial sampling frequency offset (SFO). However, for method type 2, it obviously increases additional complexity for reader and device implementation. How to ensure CP insertion of OFDM-based waveform will not introduce false rising/falling edge is still unclear. It gives more restrictions on CP insertion.

In our perspective, if CP is supported, the removal of CP should be specified with considering reader and device complexity. Otherwise, we support no CP for R2D. There are two common methods to determine the CP location and then remove CP at device side. One is to count samples from the acquired timing clock, which is simple to implement and has been well used in RFID system. The other is by comparing the interval between adjacent transition edges. The above two methods can be further considered with potential down-selection. Hence, we have the following proposal.

Proposal 1: If CP is supported for A-IoT R2D, the removal of CP can be considered for potential down-selection as follows:

- Method 1: By counting samples from the acquired timing clock as used in RFID system.

- Method 2: By comparing the interval between adjacent transition edges.

- The impact on reader and device complexity.

- FFS: more details if any

  - Modulation

D2R modulation

During the discussion in last RAN1 meeting [4], most companies support to study OOK and BPSK. Some companies propose to prioritize OOK and a few companies propose to prioritize BPSK including study phase shaping. For BFSK, some companies mentioned different BFSK in their papers while similar number of companies propose to deprioritize or not study BFSK. The following proposal was made with no consensus [4].

Proposal 3.2.1(I): - OOK and Binary PSK are the basic D2R modulation for all devices. - FFS: Whether/how pulse shaping of Binary PSK and impact to devices - Strive to identify one variant of Binary FSK for all devices among the followings - Variant 1: Frequency offset being a function of symbol rate - Variant 2: MSK - Variant 3: GFSK - Variant 4: GMSK - Variant 5: Deprioritize/not study further

We support to study OOK and BPSK first and propose to deprioritize BFSK, until the necessity and benefit is identified. For A-IoT UL study, it has possibility to conduct non-OFDM waveform. Considering the constraints of low device capability, we think the D2R modulation mechanisms should be simple enough. BFSK is more complex with different variants and should have lower priority. Thus, we support to study OOK and BPSK as baseline for all devices. Other baseband modulation mechanisms (e.g., BFSK with different variants) are deprioritized during Rel-19 SI phase. Hence, we have the following proposal.

Proposal 2: For D2R modulation, study OOK and Binary PSK as baseline for all devices.

- FFS: Whether/how pulse shaping of Binary PSK and impact to devices

Proposal 3: Other baseband modulation mechanisms (e.g., BFSK with different variants) are deprioritized during Rel-19 SI phase.

The value(s) of M for R2D modulation

For A-IoT DL study, it can reuse LP-WUS waveform design such as OOK-1/4 as much as possible. The definitions in TR 38.869 can be a starting point. Generally, OOK-4 with M-chip has an additional precoding procedure based on OOK-1, which brings acceptable complexity for gNB implementation and has no impact on the receiving process for A-IoT devices. Regarding the value of M, we prefer M=4 as baseline. The other value of M can be further studied, if necessary. The larger value of M than 8 might be too complicated for A-IoT devices with ultra-low complexity. Hence, we have the following proposal.

Proposal 4: For A-IoT DL study, support OOK-4 with 4-chip per OFDM symbol transmission as baseline.

- FFS: M=1, 2, 8

  - CRC

In RAN1 #116-bis meeting, the following agreement on CRC was achieved.

Agreement Study - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PRDCH - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PDRCH - FFS: details when different CRC lengths or no CRC may be used - FFS: other 6 bits and 16 bits CRC with different polynomials than from TS 38.212

In our perspective, using 6 bits and 16 bits CRC with polynomials from TS 38.212 as baseline is enough, no need to have other different CRC polynomials as used in RFID. As a common understanding, the link performance is significantly impacted by CRC lengths, not CRC polynomials. Hence, we suggest deleting the last FFS. With no doubt, the proper length of CRC is closely related to the transmitted data size. The transmitted data size can be one or more candidate values to save calculation memory and avoid more complexity. The detailed value(s) can be further studied. Hence, we have the following proposals.

Proposal 5: No need to consider other 6 bits and 16 bits CRC with different polynomials than from TS 38.212.

Proposal 6: The transmitted data size can be one or multiple candidate values.

- FFS: detailed values.

For the details when different CRC lengths or no CRC may be used, some companies discussed about the design aspects. The key point is the used CRC length depends on the transmitted data size before CRC. In last RAN1#117 meeting, the following proposal was made with no consensus [4].

Proposal 4.1a(I): For PRDCH/PDRCH transmissions with CRC, the used CRC length depends on the number of bits Z before CRC, i.e. CRC-6 for Z<=X bits, while CRC-16 for Z > X bits - Option 1: X = 16 - Option 2: X = 24 Note: This does not preclude PRDCH/PDRCH transmissions also without CRC.

We generally support FL proposal. CRC-6 can be used for the smaller number of transmitted data bits while CRC-16 can be used for the larger number of transmitted data bits. However, the detailed value of the threshold of transmitted data bits can be further study. In addition, no CRC can be also supported for short messages or command to save the CRC overhead. Hence, we think it is not precluded PRDCH/PDRCH transmissions without CRC.

Proposal 7: For PRDCH/PDRCH transmissions with CRC, the used CRC length depends on the number of transmitted data bits Z before CRC, i.e. CRC-6 for Z<=X bits, while CRC-16 for Z > X bits.

- FFS: detailed value of X.

- Note: This does not preclude PRDCH/PDRCH transmissions without CRC.

  - Bandwidths

R2D bandwidth

In RAN1 #116-bis meeting, the agreement on the transmission bandwidth has been made for down-selection.

Agreement For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is ≤ [12] PRBs and is down-selected among: - Alt 1: Including 180 kHz, 360 kHz, and FFS other values - Alt 2: Integer multiple(s) of 180 kHz (FFS: what integer(s)) - Alt 3: Integer multiple(s) of the subcarrier spacing (FFS: what integer(s))

The supported maximum bandwidth is 5MHz, 10MHz, 20MHz or 100MHz in FR1 for legacy LTE/NR UEs and (e)RedCap UEs. For NB-IoT and eMTC, the transmission bandwidth is 1RB (180KHz) and 6RB (1.08MHz), respectively. The transmission bandwidth size significantly affects the device performance and complexity. With the constraints of A-IoT device capability and low performance requirements, it can reuse 1RB (180KHz) transmission bandwidth as a starting point. Regarding the three alternatives in the above agreement, we prefer Alt2. We think integer multiple(s) of 180 kHz is needed, which have good alignment with NR system and can avoid additional boundary alignment. Further study on whether it is necessary to configure other values for A-IoT devices is deprioritized.

Proposal 8: Support the transmission bandwidth of 1RB (180kHz) for R2D as a starting point.

- Integer multiple(s) of 180 kHz (FFS: what integer(s)).

D2R bandwidth

In last RAN1#117 meeting, the heated discussion on R2D bandwidth has been made while with no consensus [4].

Proposal 3.8.1b(I): For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier wave, it can be: - Alt 1: An integer number of PRBs - Alt 2: An integer multiple of SCS NOTE: Carrier wave is internal or external to device as appropriate.

We support FL proposal. The transmitted D2R bandwidth can be smaller than one PRB in some cases, which depends on the data rate. Hence, an integer number of PRBs or SCS can be proposed for further study. In addition, it may face more complications if the potential multi-single tone CW is considered. When defining the value of D2R bandwidth, we can assume single-tone CW as baseline. Thus, we have the following proposal.

Proposal 9: For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier wave, it can be:

- Alt 1: An integer number of PRBs

- Alt 2: An integer multiple of SCS

Conclusions

In this contribution, we have the following proposals:

Proposal 1: If CP is supported for A-IoT R2D, the removal of CP can be considered for potential down-selection as follows:

- Method 1: By counting samples from the acquired timing clock as used in RFID system.

- Method 2: By comparing the interval between adjacent transition edges.

- The impact on reader and device complexity.

- FFS: more details if any

Proposal 2: For D2R modulation, study OOK and Binary PSK as baseline for all devices.

- FFS: Whether/how pulse shaping of Binary PSK and impact to devices

Proposal 3: Other baseband modulation mechanisms (e.g., BFSK with different variants) are deprioritized during Rel-19 SI phase.

Proposal 4: For A-IoT DL study, support OOK-4 with 4-chip per OFDM symbol transmission as baseline.

- FFS: M=1, 2, 8

Proposal 5: No need to consider other 6 bits and 16 bits CRC with different polynomials than from TS 38.212.

Proposal 6: The transmitted data size can be one or multiple candidate values.

- FFS: detailed values.

Proposal 7: For PRDCH/PDRCH transmissions with CRC, the used CRC length depends on the number of transmitted data bits Z before CRC, i.e. CRC-6 for Z<=X bits, while CRC-16 for Z > X bits.

- FFS: detailed value of X.

- Note: This does not preclude PRDCH/PDRCH transmissions without CRC.

Proposal 8: Support the transmission bandwidth of 1RB (180kHz) for R2D as a starting point.

- Integer multiple(s) of 180 kHz (FFS: what integer(s)).

Proposal 9: For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier wave, it can be:

- Alt 1: An integer number of PRBs

- Alt 2: An integer multiple of SCS

References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #102, Edinburgh, UK, December 11th -15th, 2023.

- RP-240826, revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024.

- 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

- R1-2405441, Final Feature Lead Summary for 9.4.2.1 Ambient IoT – General aspects of physical layer design, 3GPP TSG RAN1 meeting #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.