3GPP TSG RAN WG1 #118                                       	      R1-2406303

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.9.1

Source:	Xiaomi

Title:                     Measurements related enhancements for LTM

Document for:	Discussion and Decision

Introduction

The WI for R19 NR mobility enhancements Phase 4 was revised in RAN-104 meeting, and the detailed objectives [1] are as follows:

The detailed objectives are as follows:

RAN1:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

While in this contribution, we will discuss the event and the CSI-RS measurement for LTM procedures.

Event(s) for LTM

RAN1-116bis Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

RAN2-126 Agreement

·       Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-        Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-        Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-        Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-        Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

·         FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

·         Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

·         RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

·         For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In RAN1-116bis meeting[2], the following agreements were archived on events for UE initiated beam report. And in RAN2-126 meeting[3], the following agreements were archived on events for LTM.

During the discussion of MIMO agenda, the Event 2 was agreed for UE initiated/event triggered beam report so far. And in the discussion in RAN2 for Rel-19 LTM, Event LTM2, LTM3, LTM4 and LTM 5 were agreed. We prefer to support the RAN2’s conclusion.

Proposal 1: Support Event LTM2, LTM3, LTM4 and LTM5 in RAN2’s agreement for LTM.

But since both intra-frequency and inter-frequency can be considered, in addition to the beam specific offset and cell specific offset, we think that a frequency specific offset is also needed to be considered in Event-LTM3, LTM4 and LTM5 at least if the frequency of candidate cell is different from that of serving cell. Take Event LTM3 as an example:

Event LTM3 for LTM: Mn+Ofn+Obn+Ocn-Hys>Mc+Ofc+Obc+Occ+Threshold

It means that for inter-frequency LTM beam report triggering of F1, an offset with Ofn value#1 can be configured. While for inter-frequency LTM beam report triggering of F2, an offset with Ofn value#2 can be configured.

Proposal 2: A frequency specific offset can be configured in each LTM event for inter-frequency LTM

Regarding the definition of the beams of the serving cell in the Event LTM2~5, we support indicated TCI state at least. With indicated TCI state as the beams of serving cell, Event LTM 3 and 5 can be used to update the indicated TCI state. In addition, we also think the update of the activated TCI state is also important can be reduced the latency of indicated TCI state switching. So we suggest to consider the worst activated TCI state as the beams of serving cell, which can be used to trigger the report for activated TCI state update for LCM.

Proposal 3: Support indicated TCI state and the worst activated TCI state as the beams of serving cell in Event for LTM.

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

Regarding the filtering, there were also some agreements in MIMO agenda. We prefer to reuse those mechanism.

It means that for Event LTM 3~5, If within a time window (which is configurable), the number of the Event instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. And the M can be configured to 1, the length of the time window can be configured to 0.

Proposal 4: Support to reuse the filtering mechanism in UEI agenda with a configurable time window and a configurable number M for LTM.

During the discussion in inter-cell beam management, CSI-RS resource was supported for beam measurement of neighboring cells. Thus for LTM, we prefer to support CSI-RS resource with the SSB of the candidate cell as the QCL source. And the RO and preamble of the CSI-RS resource can be decided by the QCL source SSB or configured by gNB.

Proposal 5: Support CSI-RS resource with SSB of candidate cell as QCL source for beam measurement of LTM.

Content of Beam reports for LTM

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

    - FFS: candidate value of ‘N’.

  - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams

- FFS: Option-1/1a/1b/2.

- Above applies at least for the single CC case

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported.

- RRC can enable or disable whether current beam is always reported

    - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported

      - Note: The reported current beam is NOT counted in the N reported beams.

    - When disabled by RRC, N beams are reported.

Refer to Rel-18 LTM, NW configures the number of reported candidate cells L and the number of reported beams for each candidate cell M. In this case, the number of beams in the beam report is fixed. And according to the agreements of Rel-19 UEI beam report in RAN1-117 meeting[4], the number of reported beams is also configured by gNB.

Regarding to the beam report for Event-triggered LTM, it is straightforward to support gNB to configure the number of reported beams. And at least one of the reported beams of one of the reported candidate cells should satisfy the condition of the Event. Take this as the baseline, we need to consider is there any necessary enhancement? For example, for each reported candidate cell, at least one reported beam should satisfy the Event. Or for the reported cell with at least one reported beam satisfying the Event, the number of reported beams should be more than the reported candidate cell without any reported beam satisfying the Event.

Proposal 6: Regarding the number of reported beams triggered by Event for LTM, support following two Alts:

- Alt 1: gNB configures the number of reported candidate cells and the number of reported beams for each candidate cell.

- Alt 2: gNB configures the total number of reported beams and the number of reported candidate cells. But UE decide the number of reported beams for each candidate cell.

Proposal 7: At least one reported beam for LTM should satisfy the condition of the Event.

In addition, since there are multiple events were agreed for LTM, it is straightforward to report the event ID in the beam report to provide some information to gNB.

Proposal 8: Report Event ID in the beam report for Event-triggered beam report for LTM.

UL signaling medium/container considering the event-driven beam report for LTM

For Rel-18 LTM, the maximum number of reported candidate cell is 4. The maximum number of reported beams per cell is also 4. And the candidate value of M*L can be {1,2,3,4, 6, 8, 9, 12, 16} according to UE capability. It means UCI can support up to 16 reported beams in one report instance. It is sufficient and not necessary to use MAC CE. So we support to use UCI for beam report of LTM.

Proposal 9: Support UCI for Event-triggered beam report for LTM.

During the discussion in UEI agenda in MIMO session, Mode A and Mode B were agreed as the UL signaling/container for UE-initiated/event-driven beam reporting. we prefer to reuse the medium/container of UEI beam report for event-triggered beam report of LTM.

Proposal 10: Reuse the UL signaling medium /container Mode A and Mode B of UEI beam report in MIMO session for event-triggered beam report of LTM.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - FFS: Request format, e.g., SR or a new UCI type.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - FFS: Notification format, e.g., SR or a new UCI type.

  - Step 2: UE transmits the beam report in the second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

FFS: Whether UE receives acknowledge information with response to each step for all modes

For above procedures, cross-CC beam reporting is supported for both modes.

- FFS: Details.

Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH.

- The UL-grant DCI format at least comprises DCI format 0_1/0_2.

  - FFS: DCI format 0_3

- FFS: How to trigger the UEI beam report by the UL-grant DCI format

- FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

    - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report

      - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report.

    - The DL-grant DCI format at least comprises DCI format 1_1/1_2.

      - FFS: DCI format 1_3

Conclusion

In this contribution, we discuss about the event-triggered measurement reporting for LTM. Based on above discusses, we provide the following proposals.

Proposal 1: Support Event LTM2, LTM3, LTM4 and LTM5 in RAN2’s agreement for LTM.

Proposal 2: A frequency specific offset can be configured in each LTM event for inter-frequency LTM

Proposal 3: Support indicated TCI state and the worst activated TCI state as the beams of serving cell in Event for LTM.

Proposal 4: Support to reuse the filtering mechanism in UEI agenda with a configurable time window and a configurable number M for LTM.

Proposal 5: Support CSI-RS resource with SSB of candidate cell as QCL source for beam measurement of LTM.

Proposal 6: Regarding the number of reported beams triggered by Event for LTM, support following two Alts:

- Alt 1: gNB configures the number of reported candidate cells and the number of reported beams for each candidate cell.

- Alt 2: gNB configures the total number of reported beams and the number of reported candidate cells. But UE decide the number of reported beams for each candidate cell.

Proposal 7: At least one reported beam for LTM should satisfy the condition of the Event.

Proposal 8: Report Event ID in the beam report for Event-triggered beam report for LTM.

Proposal 9: Support UCI for Event-triggered beam report for LTM.

Proposal 10: Reuse the UL signaling medium /container Mode A and Mode B of UEI beam report in MIMO session for event-triggered beam report of LTM.

References

- RP-241515-Revised-WID-R19-Mobility-Enhancements

- Chairman notes of 3GPP RAN1-116bis meeting

- Chairman notes of 3GPP RAN2-126 meeting

- Chairman notes of 3GPP RAN1-117 meeting