3GPP TSG RAN WG1 #118                                                                                  R1-2406357

Maastricht, NL, August 19th – 23th, 2024

Source:	CATT, CICTCI

Title:	Further study on AI/ML for other aspects

Agenda Item:	9.1.3.3

Document for:	Discussion and Decision

Introduction

In Rel-18, RAN1 studied the life cycle management (LCM) of AI/ML model/functionality, evaluated the performance gain in three selected use cases, i.e. CSI feedback, beam management, positioning, and also analyzed the potential specification impact. The valuable outcome is captured in TR 38.843 [1]. As the continuation of Rel-18 RAN1 SI, a new WI on support AI/ML in air interface was approved in Rel-19. Nevertheless, some controversial issues related to LCM require further study [2], including:

- Model identification

- Model transfer/delivery

- CN/OAM/OTT collection of UE-sided model training data

Additionally, some other issues are identified during previous study, for instance [4]:

- UE-side additional condition, in the study of MI-Option2

- Post-deployment performance monitoring (similar to monitoring performance of inactive model), in the study of CSI compression

In this contribution, we share our views on these incomplete study issues. It is recognized that most of the mentioned issues are RAN2-lead. Meanwhile, RAN1 is responsible on the necessity, requirement and feasibility (from RAN1 perspective), which need to be carefully investigated.

Model identification

General views on model-ID-based LCM

Model identification has been exhaustively discussed in both RAN1 and RAN2 during Rel-18 SI. It is a key component and the prerequisite of model-ID-based LCM. Unfortunately, previous study focused too much on ‘how to identify a model’ rather than ‘what’s the benefit of model-ID-based LCM (including model identification) than functionality-based LCM’. That’s also the main reason why it is still not part of normative work.

In previous meeting, we provided our views on the necessity, feasibility and benefit of model identification [5]. The largest merit of model identification seems to be a potential solution to the consistency of NW-side additional condition. However, it is worthy to note that NW-side additional condition is just a necessary but insufficient condition of good AI/ML performance. Other solutions are also identified and may even be better, such as improving generalization by implementation, or by performance monitoring which can address all unidentified reasons of performance degradation.

For neatness, we only leave our final observation here. One can find more details on analysis in our previous companion paper [5] if being interested in.

Observation 1: Generalization capability and performance monitoring are considerable alternative solutions to address/alleviate additional condition consistency issue and provide minimum guaranteed performance.

Analysis of different MI-Options

Following the discussion in Section 2.1, we provide detailed analysis of each model identification options in this section.

Model identification type A

Type A model identification purely relies on offline inter-vendor engineering coordination. It is somehow not fully aligned with the spirit of standardization, i.e. each vendor can individually develop its product and the products co-work well when they are put together. It is argued that the specification impact is minor, e.g. only model ID, but the profit is questionable since an individual vendor does not know how to use such model ID. More importantly, the exchanged meta information and procedure of offline model identification are totally out of 3GPP. RAN1 cannot justify the feasibility or effectiveness of model identification type A. RAN1 cannot sign a blank check that model identification type A. If model identification is pursued in Rel-19, RAN1 should focus on online model identification, i.e. type B.

Proposal 1: Offline model identification, i.e. type A, is out of 3GPP and cannot be justified by RAN1.

MI-Option1

In RAN1#116bis, for MI-Option 1, RAN1agreed on an AI-Example1 for further study [3].

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

Though the agreed AI-Example1 provides a good picture for understanding, a few questions are to be answered.

Additional benefit compared to functionality-based LCM with associated ID

MI-Option 1 is one alternative to ensure the consistency of NW-side additional condition for UE-sided model. The inner logic of MI-Option 1 is that NW realizes the NW-side additional condition of UE-sided models by their corresponding associated ID(s) at data collection phase, and then properly manages the LCM of UE-sided models by comparing the associated ID(s) at data collection phase and those of current (inference) phase. But to enable model-level indication at NW-side, model ID has to be specified.

Another alternative is that UE controls its models, by requiring NW to provide associated ID(s) at both data collection phase and current (inference) phase. This alternative is known as functionality-based LCM + associated ID(s) interaction. This has been captured in the note of the RAN1#116bis agreement above. The inner logic of this alternative is that UE realizes the current NW-side additional condition by the associated ID(s) informed by the NW, and then properly manages the LCM of UE-sided models by comparing the associated ID(s) at data collection phase and those of current (inference) phase. Note that model ID is not necessarily to be specified since the model-level LCM is within UE device and transparent to network.

From procedure wise, these two alternatives are almost the same at step A, B and C in AI-Example1. The only difference is that:

- For AI-Example1, in step D, UE needs to report model related information (including associated IDs at data collection phase) of its models to NW. Model ID will be assigned or determined for each model.

  - After step D, NW manages UE-sided models by model-level LCM command using model IDs.

- For functionality-based LCM, in step D, NW indicates current associated ID to UE. No model ID is needed.

  - After step D, UE manages UE-sided models. NW management is kept in functionality-level.

The difference of these two alternatives is illustrated in Figure 1.

Figure 1 Difference between AI-Example1 and functionality-based LCM (with associated ID).

Based on the procedure analyzed above, we can observe that:

- For AI-Example1, UE needs to identify the UE-sided models to NW. And, NW still needs to dual with UE-side additional condition for proper controlling UE-sided model.

- It may be too optimistic to simplify the UE-sided model development as ‘one dataset of associated ID#1 trains one or multiple logical model#1(s)’. UE vendor may develop a model by synthesis dataset constructed by sub-datasets collected in thousands of different cells, or from large time scale.

- For functionality-based LCM, UE-side additional condition can be addressed by UE itself naturally. This is delightedly easier to accommodate functionality-based LCM. No model identification from UE to NW. The only add-on is NW to indicate associated ID#2 to UE during inference phase, which is the essential part of MI-Option 1 in Step A.

It is premature to say MI-Option 1 is a MUST-do mechanism. In our view functionality-based LCM with associated ID indication is much simpler. Unless clear additional benefit is found and justified for MI-Option1 compared to functionality-based LCM with associated ID, we should save the efforts (e.g. on model identification procedure, meta information design, model ID design) and do not support MI-Option1.

Proposal 2: Unless clear additional benefit is found and justified for MI-Option1 compared to functionality-based LCM with associated ID, do not support MI-Option1.

Relationship between model ID and associated ID

In general, there are two kinds of relationships:

Relationship#1: Model ID is relevant to associated ID

In this direction, model ID will be connected to one or more associated ID(s) in one way or the other. Some possible examples may include:

- Model ID is the same as associated ID(s).

- Model ID is part of associated ID(s).

- Associated ID(s) is part of model ID.

- Model ID can be uniquely derived from a function that using associated ID(s) as input, i.e. model ID = f (associated ID#1, associated ID#2, …).

Relationship#2: Model ID is independent from associated ID

In this direction, model ID is irrelevant to associated ID. The design of model ID does not impact the design of associated ID, and vice versa.

In AI-Example1, it is already assumed that the UE will report information of its model to NW, including the corresponding associated ID(s), and the model will be assigned or predetermined with a model ID. The whole step D is to facilitate model-ID-based LCM, e.g. allow NW to indicate UE to activate a specific AI/ML model. Therefore, it is unclear what benefit can be brought by mandating a linkage between model ID and associated ID. During Rel-18, RAN1 and RAN2 had some conclusion on model ID, e.g. whether model ID is globally unique or not, whether model ID represents a logical AI/ML model or physical AI/ML model, etc. However, associated ID is a new concept agreed recently, which is full of uncertainty. It is risky to adopt every characteristic of model ID to associated ID.

Besides, an AI/ML model may be developed by a dataset constructed by huge number of sub-datasets that collected with different associated IDs.  It is quite unnecessary to incorporate all related associated IDs, even part of, into a model ID just for the purpose of enabling model level indication.

All in all, it is more reasonable to assume that model ID is independent from associated ID.

Proposal 3: In AI-Example1, model ID is assumed to be independent from associated ID.

Applicable range of associated ID

In RAN1#117, on the applicable range of associated ID, the following working assumption was reached [4].

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Generally, the alternatives of the applicable rage of associated ID can be summarized as follows:

- Alt.1: Per cell, i.e. the same associated ID in one cell can only guarantee the same NW-side additional condition within THIS cell, but can NOT guaranteed the same NW-side additional condition in any other cell;

- Alt.2: Per cell group, i.e. the same associated ID in one cell can only guarantee the same NW-side additional condition in another cell that belonging to the same cell group;

- Alt.3: Per NW vendor, i.e. the same associated ID in one cell can only guarantee the same NW-side additional condition in another cell that belonging to the same NW vendor;

- Alt.4: Per PLMN, i.e. the same value of associated ID in one cell can only guarantee the same NW-side additional condition in another cell that belonging to the same PLMN;

- Alt.5: Global, i.e. the same associated ID always implies the same network additional condition across all networks on earth.

The above Alt.2~Alt.5 can all be categorized as ‘multiple cells’. In the WA, it was already agreed that Alt.1 (i.e. ‘per cell’) is assumed, and FFS whether per ‘multiple cells’ can be further considered. Intuitively, for associated ID, the larger applicable range, the more useful from AI/ML perspective. For example, it may help simplify dataset categorization at UE-side where data is collected from UEs in thousands of different cells. But the difficulty at network vendor or operator will also increase since the consistency of NW indicated ID needs to be maintained across more gNBs/TRPs. The risk of proprietary exposure will also increase – any UE/UE vendor can easily know the same/different NW-side additional condition of more and more cells by one associated ID. It is also questionable whether it is really feasible to extend the applicable range, if offline coordination engineering becomes unacceptable.

As a compromise, Alt.2 ‘per cell group’ may be further considered as the applicable range of associated ID, while how to categorize different cells into a ‘cell group’ is up to NW implementation. In this case, Alt.1 ‘per cell’ can be regarded as a special case of Alt.2, i.e. each cell group only includes one cell. For other alternatives, feasibility issue, proprietary issue and negligible offline coordination engineering effort are observed, and therefore not recommended.

Proposal 4: Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent within:

- One cell (baseline);

- One cell group. Whether/how to categorize cells into a cell group is up to NW implementation;

- Other ranges (e.g. W vendor, per PLMN or global) are not recommended.

How model ID is determined or assigned

For model ID determination, several alternatives are proposed in the study of AI-Example1:

- Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification

The motivation of MI-Option 1 is to enable model-ID-based LCM that visible to network. Furthermore, network can manage the LCM of UE-sided model by indicating the model ID. Hence it is more straightforward to consider that model ID should be under network control as much as possible. And since MI-Option 1 is online model identification, i.e. directly registering model from UE device to a specific network, it is safer to consider that model ID is assigned by network, after UE reporting the information of AI/ML models to the network.

Proposal 5: In AI-Example1, model ID is assigned by network after UE reporting the information of its AI/ML models to the network.

Report of information of models and the corresponding associated ID(s)

The information of AI/ML model reported from UE to network has many names during Rel-18 and Rel-19 study. To name a few, it may be described as metadata of the model, meta information of the model (RAN2 terminology in Rel-18), model description information… whatever the name is, the motivation of sharing such information is clear: to ensure the common understanding between the UE and the network. Specifically, for MI-Option 1, the model identification is from UE to NW. It is therefore UE’s responsibility to provide necessary information to NW, for the sake of NW’s proper indication on LCM of its model.

Let’s continue to use ‘meta information’ for discussion. The content of ‘meta information’ has been discussed in RAN1 and RAN2 in Rel-18 but no clear conclusion was reached. But now we introduce the concept of associated ID, which is preferably irreverent to model ID, it is natural to consider that the associated ID(s) corresponds to the AI/ML model shall be carried in the meta information of the model. UE shall transmit the meta information that carrying all related associated ID(s) to the network.

However, it is up to RAN2 to consider the procedure and signaling of meta information transmission.

Proposal 6: In AI-Example1, meta information (if supported) of an AI/ML model carries all related associated ID(s) of the AI/ML model, and is transmitted from UE to network.

- Meta information may carry other information, which is up to future discussion.

- The procedure and signaling of meta information transmission is out of RAN1.

MI-Option2

The following example for MI-Option2, namely AI-Example2-1, was agreed for further study in RAN1#117 [4]:

Agreement From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset

The procedure of AI-Example2-1 is illustrated in Figure 2.

Figure 2 Procedure of AI-Example2-1.

One-sided model case

AI-Example2-1 was agreed only for two-sided model use case. First of all, a simple question should be answered: do we need to discuss AI-Example2-1 in the context of one-sided model?

In our view, for one-sided model, the training data should be collected by vendor itself. Both UE-side data collection and NW-side data collection are part of the normative work in either beam management or positioning. On top of what we are establishing, supporting dataset transfer seems meaningless. On one hand, only vendor itself exactly knows what data is needed and how much data should be collected. On the other hand, data collection by vendor itself can handle the additional conditions at its side. For example, only UE vendor knows the corresponding UE speed, UE Rx antenna assumption, maximum UE synchronization error… of each data sample collected by a UE, but NW cannot distinguish them, not to mention documenting them and transfer them to UE/UE vendor as metadata of a dataset.

In summary, we think the discussion of MI-Option2 should be limited within two-sided model use case. This is aligned with the progress in study of AI/ML-based CSI compression, where dataset transfer is categorized as Option 4 as one alternative for inter-vendor collaboration reduction.

Proposal 7: MI-Option2 is only discussed under the context of two-sided model use case.

Mapping between dataset and model ID

AI-Example2-1 hides a critical component behind: dataset identifier, e.g. dataset ID. Such identifier is like an anchor for aligning the understanding on a specific dataset. Without a unique dataset identifier which can be recognized across cells, the dataset can only be identified by the originated cell, so as the identified model(s). This leads to an agnostic issue when UE tries to match its UE part of model(s) to the NW part of model(s) in a new cell. This implies localized (cell-specific) model(s) have to be assume, which may not be reasonable. At the same time, if a dataset can be uniquely distinguished across different cells, e.g. by dataset ID, inter-vendor collaboration may be needed, similar to associated ID.

Observation 2: Unless there is a dataset identifier (e.g. dataset ID) acknowledged across different cells, the dataset can only be assumed corresponding to the originated cell that transfers the dataset, so as the model ID(s). This implies localized (cell-specific) model(s) have to be assume, which may not be reasonable.

Proposal 8: In MI-Option2, study dataset ID and its applicable range to clarify whether dataset can be uniquely identified across different cells.

Even if we put aside dataset identifier, the construction of a dataset and the relationship between dataset and model(s) are still unclear. The following questions should be addressed to answer the question on the mapping between dataset and model ID.

- Is a dataset allowed to include sub datasets, which corresponds to different configurations (e.g. ranks, layers, bandwidth or ports in CSI compression)?

- Is a dataset allowed to include data samples with different NW-side additional conditions?

- Is a dataset allowed to include data samples with different UE-side additional conditions?

- Are all this information recognizable to UE, i.e. specified and carried in dataset’s meta data along with dataset?

- Is a UE allowed to train a UE part of model corresponds to multiple datasets?

- …

Different answer to these questions with lead to different design of dataset and model ID. The relationship between dataset and model ID may be very complicated. With such high uncertainty, it is hard to conclude that the dataset and model ID can be one-on-one mapping. One-on-one mapping maybe a very special case, corresponding to a specific combination of answers to a series of questions listed above. It is safer to keep the door open for now. As a baseline, it should be assumed that the mapping relationship is flexible, and the model ID is assigned by network after the UE reports information of its UE part of two-sided model(s).

Proposal 9: In AI-Example2-1, as a starting point, the mapping relationship between dataset and model ID is flexible, i.e. not limited to one-on-one mapping. The model ID is assigned by network after the UE reports information of its UE part of two-sided model(s).

- FFS the prerequisite when dataset and model ID is one-one-one mapping, and how to determine model ID in this case.

Consideration on UE-side additional condition

In AI-Example2-1, data samples are collected by NW side. It is natural to assume that the UE part of two-sided models, when trained by a specific dataset, can keep the consistence of NW-side additional condition at least in the originated cell. However, UE-side additional condition may be an issue since the UEs in data collection phase and UEs to deploy UE part of two-sided model(s) may be different. This issue is illustrated in Figure 3.

Figure 3 Different UEs in data collection and model deployment.

Intuitively, misalignment of UE-side additional condition will cause performance degradation of a two-sided model, similar to the case of NW-side additional condition to UE-sided model. In Rel-18 study, such degradation was usually evaluated in generalization performance investigation, where the simulation may be conducted under different assumption of UE speeds, Rx assumptions, etc., using one model. According to Clause 6.2.2.2 of TR 38.843 [1], the following generalization aspects have been investigated in AI/ML-based CSI compression.

- Generalization over deployment scenarios

- Generalization over UE distributions

- Generalization over carrier frequencies

- Generalization over TxRU mappings

So, unfortunately, in Rel-18 CSI compression SI, none of the evaluated generalization aspect is related to UE-side additional condition. Even though the UE-side additional condition should impact the performance theoretically, RAN1 didn’t (or at least insufficiently) evaluate and never identify a specific UE-side additional condition that has to concern in Rel-18 AI/ML-based SI compression. Worse still, it seems unrealistic to pursue large scale evaluation at this juncture. But we think this issue still exist and worth to study in the future.

Observation 3: Even though the UE-side additional condition should impact the performance theoretically, RAN1 didn’t (or at least insufficiently) evaluate and never identify a specific UE-side additional condition that has to concern in Rel-18 AI/ML-based CSI compression.

Proposal 10: In AI-Example2-1, for AI/ML-based CSI compression, the need and benefit of UE-side additional condition(s) requires further justification.

MI-Option3

MI-Option3 is related to model transfer/delivery from NW to UE. Until now, the situation of model delivery/transfer cases is summarized in the following table. Case z4 is the most promising option to be investigated. Case y means model delivery transparent to 3GPP signaling, so no need to study in RAN1.

Table 1 Situation of model delivery/transfer cases.

| Case | UE-sided | Two-sided |
| --- | --- | --- |
| y | Open | Open |
| z1 | No more discussion in this agenda | No more discussion in this agenda |
| z2 | Deprioritized | Open |
| z3 | Deprioritized | Deprioritized |
| z4 | Open | Open |
| z5 | Deprioritized | Deprioritized |

Model identification is only part of the procedures of model transfer/delivery. The necessity and feasibility of MI-Option 3 is strongly dependent on model transfer/delivery itself. We share our views on Model transfer/delivery in Section 3.

Model transfer/delivery

To further study the necessity, feasibility and benefit of model delivery/transfer Case z4, two alternatives are agreed in RAN1#117 [4]:

Agreement From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion.

In the following sub sections, we share our views on the procedure and necessity/feasibility/benefit of model transfer, respectively.

Procedure of model transfer

Figure 4 illustrates the procedures of Alt.A for model transfer Case z4. To better understand Alt.A, Figure 4 also includes a step of ‘UE capability report’, which is essential for any optional features.

Usually, the reported UE capability is specified and static, which may be stored in core network for a long time. But for a UE, the supported known model structure may or may not be specified, and may be varying. Thus Step A-1 is unlikely to be part of UE capability report. However, it seems strange that UE performs Step A-1 without being asked. In other words, the UE needs to know that NW has interest to transfer an AI/ML model to UE. Therefore, a Step A-0 can be added before Step A-1, in which the NW sends a request to UE on reporting the supported known model structure to NW. This is marked in red in Figure 4.

Proposal 11: For Alt.A of model transfer case z4, a Step A-0 can be added before Step A-1, in which the NW sends a request to UE on reporting the supported known model structure to NW.

- This is assuming Step A-1 is not part of UE capability report.

Figure 4 Procedures of Alt.A.

In Figure 5, two branches of Alt.B are illustrated, i.e. Alt.B-1 and Alt.B-2.

- In Alt.B-1, NW indication in Step B-1 is assumed to be a UE-specific signaling. This implies that NW exactly knows THIS UE supports model transfer Case z4 in Step B-1. Therefore, Step B-0 should happen before Step B-1.

- In Alt.B-2, NW indication in Step B-1 is assumed to be a broadcast signaling, e.g. carried in SIB. This implies that NW does not know which UE supports model transfer Case z4 in Step B-1. Then, Step B-0 (and also UE capability report) should happen after Step B-1.

- In either Alt.B-1 or Alt.B-2, Step B-0 may be part of UE capability report, since it is just a simple indication on supporting model transfer Case z4. It does not include interaction on aligning the supported known model structure(s).

Figure 5 Procedures for Alt. B.

Proposal 12: For Alt.B of model transfer case z4,

- If NW indication in Step B-1 is UE-specific signaling, Step B-0 should happen before Step B-1.

- If NW indication in Step B-1 is broadcast signaling, Step B-0 should happen after Step B-1.

- Step B-0 may be part of the UE capability report.

So far, we think both Alt.A and Alt.B are valid alternatives. Nevertheless, regardless Alt.A or Alt.B, there is no details on ‘What information is exchanged between UE and NW to align the understanding of supported known model structure(s)’.  This is the key of feasibility study and should be considered carefully. In general, we can start with the following directions:

- Direction 1: Exchange model ID.

This direction reuses model ID, assuming the NW and UE knows the specific model corresponding to the model ID before, e.g. by standardized AI/ML model, or by inter-vendor coordination. The only difference is that NW and UE can ignore the parameters in the model, just concern about the structure of the model corresponding to the model ID.

- Direction 2: Exchange model structure ID.

This direction borrows the concept of model ID. The relationship between model structure ID and the corresponding model structure should be known by UE and NW before, e.g. by standardized AI/ML model structure, or by inter-vendor coordination.

- Direction 3: Exchange model structure described by a known model description format.

In this direction, UE and NW need to understand a common model description format. This model description format (or perhaps model structure description format) is used to describe the model structure. Such format may be standardized, or may be understood by inter-vendor coordination.

The above directions are just starting points. Further investigation is required to form a full picture on model transfer Case z4.

Proposal 13: For model transfer Case z4, the following directions can be considered to align the understanding on supported known model structure between UE and NW.

- Direction 1: Exchange model ID;

- Direction 2: Exchange model structure ID;

- Direction 3: Exchange model structure described by a known model description format.

Necessity, feasibility and benefit

The key task for this study is to conclude the necessity, feasibility and benefit of standardized model transfer. We provide our views on these issues below.

Necessity

From any angle, model transfer Case z4 is unlikely to be necessary.

- In one-sided model use case, UE vendors can collect training data, train their AI/ML models, and deploy the models into their UEs by their own. This is widely assumed as baseline. Cell-specific optimization is still possible, as long as a vendor can mark the cell ID when collecting data sample, which is quite simple.

- In two-sided model use case, model transfer Case z4 is one option to address inter-vendor collaboration issue. It is denoted as ‘Option 3’ or ‘Option 5’ in the discussion of Agenda 9.1.3.2 for CSI compression. Indeed, ‘Option 3’ and ‘Option 5’ are possible solutions, but there are other possible options under discussion. So far, only ‘Option 2 (standardized dataset)’ has been deprioritized, so we still have Option 1, 3, 4, 5 on the table. Unless all other options are precluded, Case z4 cannot be treated as necessary.

Feasibility

Feasibility is contradictory. Though not completely infeasible, huge difficulty/effort is foreseen for realizing Case z4.

- Feasibility at NW-side may not be a serious issue. For Case z4, network has to afford the cost of data collection/storage, model training/storage, etc. This is a burden for sure, but may be acceptable for a NW that capable for AI/ML.

- Feasibility at UE-side is complicated. It is important to see whether a UE device can directly applied the transferred parameter into its chipset. If not, additional procedure is needed, i.e. UE transparently delivers the parameter to its OTT server, and gets an executable form of this model (model structure + parameter in a compiled form). This is marked as ‘Option 3a/5a’ in the study on inter-vendor collaboration effort alleviation in Agenda 9.1.3.2 for CSI compression. If so, the feasibility issue should concern on (1) the timing/delay cycle for compiling, and (2) inter-vendor collaboration, which may be mandated to make sure that the OTT server can compile the model from network with certain model structure and model description format.

- From standardization point of view, the potential specification impact may include standardized model structure, standardized model, standardized model description format, etc., with which the standardization effort is non-negligible. To alleviate this issue, one possible way to reduce the standardization work is to select a small set of simple model structures (e.g. CNN) as reference model structure. But this may also reduce the performance gain and made Case z4 less attractive.

Benefit

Some benefit of model transfer Case z4 is identified, but this benefit may be achieved by other alternatives not rely on model transfer.

- Standardized model transfer/delivery may bring benefits on (1) supporting two-sided model while reducing inter-vendor collaboration; (2) cell-specific model optimization at NW-side; (3) avoiding training data leakage that should not be collected/transferred outside the network.

- Some alternatives other than standardized model transfer/delivery can also achieve similar benefits, e.g. collaboration options under discussion in Agenda 9.1.3.2 CSI compression, data or dataset categorization for cell-specific model trained at UE side, standardized or unstandardized data desensitization.

In summary, we think the necessity, feasibility, and benefit of model transfer Case z4 can be concluded as in the following proposal. Note that the proposal is under the circumstance of Rel-19. With deeper understanding on AI/ML in air interface, the evaluation may change in the future.

Proposal 14: In Rel-19, for model transfer/delivery Case z4, conclude that:

- Necessity: Not necessary, since some other alternatives can achieve similar goal;

- Feasibility: Not completely infeasible, but huge difficult/effort is foreseen;

- Benefit: Benefits are identified on inter-vendor collaboration reduction, cell-specific model optimization, and avoiding training data leakage, etc. But some other alternatives can also achieve similar benefits.

Data collection for UE-sided model training

CN/OAM/OTT collection of UE-sided model training data has been roughly discussed in RAN2 in Rel-18, but no consensus or recommendation was reached [1].

- 7.2.1.3.2 Data collection for UE-side model training The following proposals were discussed in RAN2: 1. UE collects and directly transfers training data to the Over-The-Top (OTT) server; 1a) OTT (3GPP transparent) 1b) OTT (non-3GPP transparent) 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the needed data to the OTT server. RAN2 did not study or analyse these proposals and did not agree to requirements or recommendations.

As AI/ML is a data driven approach, it is self-evident that data collection for UE-side model training is the essential feature for UE-sided model. There is a clear division of responsibility between RAN1 and higher layers (including RAN2):

- RAN1 is responsible for data collection in the air interface, e.g. collecting data by measurement into UE device.

- Higher layers (including RAN2) are responsible for data collection from UE device to the OTT server for UE-side model training, i.e. delivery of a set of collected data.

From RAN1 perspective, the normative work on data collection for UE-side model training is already started, for each agreed use case. For example, for beam management, data collection initiated/triggered by configuration from NW or request from UE are under discussion. For another example, for positioning, the ground-truth label, measurement for model input and quality indicator for data collection have been identified. However, how to transfer/deliver the collected data in UE device to OTT server is out of RAN1scope.

Observation 4: RAN1 already starts the normative work of data collection for UE-side model training within RAN1 scope, including the corresponding contents of UE data collection per WI use case.

Obviously, CN/OAM/OTT collection of UE-sided model training data shall be discussed in RAN2, and possibly even higher layers. In RAN2#126, RAN2 already reached consensus on further clarifications for each option, and the results are going to be capture in TR 38.843 [6]. Anyway, it is out of RAN1 scope.

Proposal 15: For data collection for UE-side model training,

- RAN1 focuses on how to collect training data into UE device in air interface, including the corresponding contents of UE data collection per WI use case;

- RAN2 and higher layers focus on whether and how the training data is transferred/delivered from UE device to UE-side server, e.g. via CN/OAM/OTT.

Conclusion

In this contribution, we provide our views on other aspects of AI/ML for NR air interface. The observations and proposals are summarized as follows:

Observation 1: Generalization capability and performance monitoring are considerable alternative solutions to address/alleviate additional condition consistency issue and provide minimum guaranteed performance.

Observation 2: Unless there is a dataset identifier (e.g. dataset ID) acknowledged across different cells, the dataset can only be assumed corresponding to the originated cell that transfers the dataset, so as the model ID(s). This implies localized (cell-specific) model(s) have to be assume, which may not be reasonable.

Observation 3: Even though the UE-side additional condition should impact the performance theoretically, RAN1 didn’t (or at least insufficiently) evaluate and never identify a specific UE-side additional condition that has to concern in Rel-18 AI/ML-based CSI compression.

Observation 4: RAN1 already starts the normative work of data collection for UE-side model training within RAN1 scope, including the corresponding contents of UE data collection per WI use case.

Proposal 1: Offline model identification, i.e. type A, is out of 3GPP and cannot be justified by RAN1.

Proposal 2: Unless clear additional benefit is found and justified for MI-Option1 compared to functionality-based LCM with associated ID, do not support MI-Option1.

Proposal 3: In AI-Example1, model ID is assumed to be independent from associated ID.

Proposal 4: Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent within:

- One cell (baseline);

- One cell group. Whether/how to categorize cells into a cell group is up to NW implementation;

- Other ranges (e.g. W vendor, per PLMN or global) are not recommended.

Proposal 5: In AI-Example1, model ID is assigned by network after UE reporting the information of its AI/ML models to the network.

Proposal 6: In AI-Example1, meta information (if supported) of an AI/ML model carries all related associated ID(s) of the AI/ML model, and is transmitted from UE to network.

- Meta information may carry other information, which is up to future discussion.

- The procedure and signaling of meta information transmission is out of RAN1.

Proposal 7: MI-Option2 is only discussed under the context of two-sided model use case.

Proposal 8: In MI-Option2, study dataset ID and its applicable range to clarify whether dataset can be uniquely identified across different cells.

Proposal 9: In AI-Example2-1, as a starting point, the mapping relationship between dataset and model ID is flexible, i.e. not limited to one-on-one mapping. The model ID is assigned by network after the UE reports information of its UE part of two-sided model(s).

- FFS the prerequisite when dataset and model ID is one-one-one mapping, and how to determine model ID in this case.

Proposal 10: In AI-Example2-1, for AI/ML-based CSI compression, the need and benefit of UE-side additional condition(s) requires further justification.

Proposal 11: For Alt.A of model transfer case z4, a Step A-0 can be added before Step A-1, in which the NW sends a request to UE on reporting the supported known model structure to NW.

- This is assuming Step A-1 is not part of UE capability report.

Proposal 12: For Alt.B of model transfer case z4,

- If NW indication in Step B-1 is UE-specific signaling, Step B-0 should happen before Step B-1.

- If NW indication in Step B-1 is broadcast signaling, Step B-0 should happen after Step B-1.

- Step B-0 may be part of the UE capability report.

Proposal 13: For model transfer Case z4, the following directions can be considered to align the understanding on supported known model structure between UE and NW.

- Direction 1: Exchange model ID;

- Direction 2: Exchange model structure ID;

- Direction 3: Exchange model structure described by a known model description format.

Proposal 14: In Rel-19, for model transfer/delivery Case z4, conclude that:

- Necessity: Not necessary, since some other alternatives can achieve similar goal;

- Feasibility: Not completely infeasible, but huge difficult/effort is foreseen;

- Benefit: Benefits are identified on inter-vendor collaboration reduction, cell-specific model optimization, and avoiding training data leakage, etc. But some other alternatives can also achieve similar benefits.

Proposal 14: In Rel-19, for model transfer/delivery Case z4, conclude that:

- Necessity: Not necessary, since some other alternatives can achieve similar goal;

- Feasibility: Not completely infeasible, but huge difficult/effort is foreseen;

- Benefit: Benefits are identified on inter-vendor collaboration reduction, cell-specific model optimization, and avoiding training data leakage, etc. But some other alternatives can also achieve similar benefits.

Proposal 15: For data collection for UE-side model training,

- RAN1 focuses on how to collect training data into UE device in air interface, including the corresponding contents of UE data collection per WI use case;

- RAN2 and higher layers focus on whether and how the training data is transferred/delivered from UE device to UE-side server, e.g. via CN/OAM/OTT.

References

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, v18.0.0.

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11th – 15th, 2023.

- Chairman’s Notes RAN1#116bis, Changsha, China, April 15th – 19th, 2024.

- Chairman’s Notes RAN1#117, Fukuoka, Japan, May 20th – 24th, 2024.

- R1-2404388, Study on AI/ML for  other aspects of model and data, CATT, CICTCI, RAN1#117, Fukuoka, Japan, May 20th – 24th, 2024.

- Chair’s Note RAN2#126, Fukuoka, Japan, May 20th – 24th, 2024.