3GPP TSG RAN WG1 #118			R1-2406482

Maastricht, The Netherlands,  19 – 24 August 2024

Agenda Item	: 	9.6.3

Source 	:	Sony

Title 	:	LP-WUS operation in CONNECTED mode

Document for 	:	Discussion and decision

# Introduction

A revised work item on Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR) was approved in RANP#103 [1]. The objective for the operation in connected mode is:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In RAN1#117, the following agreements were made [2]:

Agreement For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - FFS: exact value(s) of the minimum time gap - FFS: support of multiple minimum time gaps - FFS whether the reported value includes the duration for time/frequency synchronization of MR Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode. - FFS whether to define a time window for Mos - It is at least supported that a UE can monitor MOs with a periodicity. - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. - Rel-16 DCP - Rel-17 PDCCH skipping - Rel-17 SSSG switching - Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

RAN2 as the topic-led of this topic has also started the discussion on LP-WUS in connected mode. The related agreements made in RAN2#126 meeting are shown below [5].

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In this document, we provide our views on LP-WUS operation in RRC CONNECTED mode.

# Discussion

The legacy UE operating in RRC connected mode and configured with C-DRX should keep the main receiver (MR) fully operated to monitor PDCCH in C-DRX active time. However, there are some occasions when there is nothing scheduled for the UE. Consequently, the UE uses unnecessary power consumption by monitoring PDCCH when the gNB does not transmit anything on the PDCCH. There have been some enhancements proposed in previous NR releases to reduce UE power consumption. These techniques are Rel-16 DCP on the usage of DCI with CRC scrambled by PS-RNTI and Rel-17 PDCCH monitoring adaptation. However, both techniques are based on PDCCH-based signalling. PDCCH decoding is known as a power-hungry and high computation operation as it involved blind detection process. LP-WUS can be used as an alternative  of DCP so that the UE power consumption can be improved. LP-WUS can trigger the PDCCH monitoring. This issue was also discussed in RAN1#116. Several options were identified. Our preference is option 1-1 that the LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. The operation of option 1-1 is illustrated in Figure 1.

In RAN1#117, the FL indicated that RAN2 started the discussion on Procedures for LP-WUS in RRC_CONNECTED from RAN2#126 meeting. Hence, it would be better to wait for RAN2 progress before further down-selection among the options. Furthermore, the FL suggested that in RAN1 #117 meeting, RAN1 to discuss specific aspects in high level, which are commonly applicable to all options, such as the LP-WUS monitoring occasions, timeline between LP-WUS reception/detection and PDCCH monitoring.

During RAN2 #126, most of the agreements are related to the operation LP-WUS in connected mode with Option 1-1. Hence, we consider RAN2 also strongly consider this option. Hence, RAN1 can continue with the down-selection of the options.

Figure 1: Illustration of LP-WUS with Option 1-1 [3]

Proposal 1: Support LP-WUS monitoring to trigger the starting of the drx-onDurationTimer (known as Option 1-1).

In RAN1#117, there was a discussion on support of multiple minimum time gaps (i.e., the time offset in Figure 1). We consider the LP-WUS operation with multiple minimum time-gaps can be beneficial. Some of the UE operation may require a shorter gap (e.g., XR applications). Some others can be more predicted by the gNB, so that the time gap can be longer. The possible options and the actual time-gap for each option can be further studied.

Proposal 2: Support LP-WUS in RRC connected mode with multiple time-gaps

Apart from the DCP, another technique introduced in LTE-MTC and NB-IoT is on the usage of wake-up signal (WUS) so that the PDCCH decoding for paging channel can be avoided in case the UE is not being paged. The WUS in both MTC and NB-IoT is sequence-based WUS. We consider a similar approach can be applied for NR. In our companion technical contribution [6], we described the design of LP-WUS design for NR. One main consideration aspect that is described in the WID is to specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes. Based on our analysis described in [1], the sequence-based LP-WUS operation should also be applicable for the UE operating in RRC connected mode. Another alternative being discussed in RAN1#117 is to use encoded bits. While flexible, the usage of encoded bits, however, is challenging as it may be difficult to meet the required performance resulting in high miss and false-alarm rates and thereby unnecessary high system overhead at gNB and high power consumption at the UE, respectively.  Using bitmap/encoded bits is not our preferred solution as it results in high power consumption and high system overhead. Thus, reducing the UE power saving gain. In RAN1#117, it was suggested to discuss this issue in LP-WUS & LP-SS design agenda item. Hence, we consider there should be a common design of LP-WUS operated in all RRC states.

Proposal 3: RAN1 strives for the LP-WUS common design including supporting sequence-based LP-WUS operation in both RRC CONNECTED and RRC idle/inactive mode.

During study item phase, various traffic models when the UE is in RRC connected mode were evaluated. The traffic models were XR application, FTP application, and IM application. These applications have different behavior and characteristics, where XR application is one of the examples. The received XR traffic to the UE may experience jitter so that the actual received transmission can be delayed or advanced within certain window (e.g, +/- 4 ms). In this case, the gNB can indicate the UE that after reception of LP-WUS the UE shall advancing/delaying the PDCCH monitoring operation. In this example, the gNB indicates the subsequent action from the UE upon the reception of LP-WUS. Furthermore, when the UE performs the above applications, the UE is expected in performing unicast transmission/reception with the gNB. Subsequently, the LP-WUS transmission in RRC connected mode is a unicast transmission between the gNB and the target UE.

Proposal 4: LP-WUS transmission in RRC Connected mode indicates the target UE and the subsequent action from the UE upon the reception of LP-WUS.

The information on the target UE and the subsequent action from the UE (upon the reception of LP-WUS) is carried by the transmission of LP-WUS. We consider the overall equivalent information bits can be associated to the property of the LP-WUS itself and the number of time/frequency resources of that LP-WUS. In RAN1#116, it was discussed the maximum number of LP-WUS information bits. We consider this as the information bits associated to the property of the LP-WUS. The LP-WUS reception for high number of information bits may increase UE power consumption. As we propose sequence-based LP-WUS, we consider maximum of 8 bits for the LP-WUS information bits is sufficient. Note that the total-length of the sequence-based LP-WUS is not affected by the size of the information bits. The total-length of the sequence based LP-WUS may vary, such as with a long transmission in order to obtain high processing gain at the receiver. The total information bits can be more than 8 bits by considering the time/frequency resources to carry LP-WUS. Hence, a gNB can have more flexibility for the LP-WUS operation. For example, if more UEs are required then more LP-WUS resources can be configured. In RAN1#117, it was suggested that the issue on the number of information bits is to be discussed in the other agenda item (LP-WUS & LP-SS design). Hence, we consider RAN1 should aim for the common design for the LP-WUS in RRC idle/inactive and connected mode.

Proposal 5: The maximum number of LP-WUS information bits for RRC connected mode is up to 8 bits

The LP-WUS resource is allocated prior to the PDCCH resource to be monitored by the UE. In case there are multiple UEs  going to monitor the same timing of PDCCH resource, the LP-WUS resource allocation prior to the PDCCH resource can be multiplexed, such as in time and/or frequency. From the UE point of view, the UE is expected to monitor a given LP-WUS resource. The UE is not expected to monitor multiple LP-WUS resources as it will increase the UE power consumption.

Proposal 6: The UE operating in RRC connected mode and configured to receive LP-WUS is expected to monitor a given LP-WUS resource.

During study item phase, two modes of LP-WUS operation were discussed. It includes duty-cycled monitoring and continuous monitoring mode for LP-WUS. The pros and cons of each mode were also summarized in Section 7.3.2.2 in [3]. We consider both can be applied for different applications. The UE is expected to support various applications / traffic types in RRC connected mode. Some applications may be beneficial by using one of the monitoring modes. Furthermore, the monitoring mode can be expressed as a UE capability. The LP-WUS monitoring mode can an optional UE feature.

Proposal 7: Support both duty-cycled monitoring mode and continuous monitoring mode for LP-WUS in RRC-connected.

It was agreed in RAN1#116 that the PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB via RRC signaling. Furthermore, this was further discussed where there is a need to have a similar mechanism but faster than RRC signalling, which is known as LP-WUS activating/deactivating. We consider the LP-WUS enabling/disabling via RRC signalling is sufficient. If LP-WUS activating/deactivating is required then it shall be shown the case / scenario when it will be used. The pros and cons should also be described.

Proposal 8: Enabling/Disabling of LP-WUS monitoring by RRC signalling is sufficient. No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring (known as Option 1).

In RAN1#117, it was discussed on the operation of LP-WUS for the UE operating Carrier Aggregation (CA) in RRC connected mode. We are not sure whether CA for LP-WUS is part of the WI’s objectives. Furthermore, the benefit is still unclear. Hence, we propose to down-prioritize on supporting LP-WUS for a UE operating CA. RAN1 should focus more to the fundamental aspect of the LP-WUS operation in RRC connected mode.

Proposal 9: Down-prioritize on supporting LP-WUS for the UE operating Carrier Aggregation (CA).

# Conclusion

This document has considered the LP-WUS operation in RRC CONNECTED mode. We made the following proposals:

Proposal 1: Support LP-WUS monitoring to trigger the starting of the drx-onDurationTimer (known as Option 1-1).

Proposal 2: Support LP-WUS in RRC connected mode with multiple time-gaps

Proposal 3: RAN1 strives for the LP-WUS common design including supporting sequence-based LP-WUS operation in both RRC CONNECTED and RRC idle/inactive mode.

Proposal 4: LP-WUS transmission in RRC Connected mode indicates the target UE and the subsequent action from the UE upon the reception of LP-WUS.

Proposal 5: The maximum number of LP-WUS information bits for RRC connected mode is up to 8 bits

Proposal 6: The UE operating in RRC connected mode and configured to receive LP-WUS is expected to monitor a given LP-WUS resource.

Proposal 7: Support both duty-cycled monitoring mode and continuous monitoring mode for LP-WUS in RRC-connected.

Proposal 8: Enabling/Disabling of LP-WUS monitoring by RRC signalling is sufficient. No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring (known as Option 1).

Proposal 9: Down-prioritize on supporting LP-WUS for the UE operating Carrier Aggregation (CA).

# References

- RP-240801, “Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, RANP#103, March 2024.

- Chairman’s notes, RAN1#117, Fukuoka – Japan, May 2024.

- 3GPP TR 38.869 v18.0.0, “Study on low-power Wake-up Signal and Receiver for NR”

- R1-2405721, FL summary #4 on LP-WUS operation in CONNECTED mode, NTT Docomo, 1#117, Fukuoka – Japan, May 2024.

- Chairman’s notes, RAN2#126, Fukuoka – Japan, May 2024.

- R1-2406480, “LP-WUS and LP-SS design”, Sony, RAN1#118, Maastricht - Netherlands, August 2024.