3GPP TSG RAN WG1 Meeting #118		R1-2406527

Maastricht, Netherlands, Aug 19th – Aug 23rd, 2024

Source:	Fujitsu

Title:	Discussion on measurement related enhancements for LTM

Agenda Item:	9.9.1

Document for:	Discussion

# Introduction

In RAN plenary meeting #102, WID of mobility enhancement was defined, and the WID was updated in RAN#104 [1], which is excerpted below with RAN1-related.

RAN1-related WID

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

    - As secondary priority, support the case where CU is acting as SN and MN is unchanged

    - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

    - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In this contribution, we discuss three main issues for CSI-RS measurement for LTM, event triggered L1 measurement reporting and other L1 enhancement.

# CSI-RS measurement for LTM procedure

The use of CSI-RS measurement for LTM offers significant advantages. One key benefit is the ability to select narrow beam, that is, a UE can maintain higher data rates during cell switch procedure. Additionally, the CSI-RS measurement allows for the assessment of wide bandwidth. Unlike the measurement using SSB where the UE can measure only the bandwidth of the SSB itself, CSI-RS enables the measurement across whole bandwidth of the band by the configuration. This results in a more accurate representation of the channel conditions, leading to better-informed mobility decisions and overall enhanced network performance. In subsequent sections, we investigate the details of the CSI-RS measurement in Rel-19 LTM.

## Measurement Quantity

L1-RSRP provides the signal strength at the timing of the measurement while L1-SINR offers a more comprehensive evaluation by incorporating interference levels, especially for inter-frequency LTM. Inter-frequency scenarios are characterized by varying levels of interference between inter-frequency cells. If cell switch decision is based solely on L1-RSRP, there is a risk of selecting a candidate cell with high signal strength but poor quality of signal due to the potential interference, resulting in suboptimal performance. L1-SINR ensures a more accurate assessment of the candidate cell’s quality, consequently, it can achieve to improve the overall LTM performance. On the other hand, since only SSB as a reference signal for the LTM is adopted in Rel-18 and the SSB has some restriction to measure the L1-SINR, there was little motivation to introduce the L1-SINR, consequently, the discussion about L1-SINR was dropped in Rel-18.

In Rel-19, the introduction of CSI-RS for measurement and beam management is clearly described in the WID but the L1-SINR based on the CSI-RS measurement is not clearly defined though we support the introduction of L1-SINR. The discussion on this topic might be controversial as we had discussion about this topic in Rel-18. Therefore, we propose to discuss the introduction of L1-SINR based on CSI-RS measurement in RAN1.

      - Proposal 1: Support L1-RSRP measurement based on CSI-RS.

      - Proposal 2: Support L1-SINR measurement based on CSI-RS.

## Intra- and Inter-frequency

In Rel-18 LTM, SSB is supported for intra- and inter-frequency measurement rather than CSI-RS as discussed in the previous subsection. RAN1 and RAN2 had a discussion on the necessity of inter-frequency scenario for LTM, and RAN2 concluded that this is an important scenario. Considering this history, we believe that inter-frequency scenario is still important for Rel-19 LTM, and hence the CSI-RS based L1 measurement should support intra- and inter-frequency measurement. It is noted that the detailed definition of intra-/inter-frequency is up to RAN4, which is the same approach as in Rel-18.

      - Proposal 3: Support intra- and inter-frequency L1 measurement based on CSI-RS.

        - Applicable to L1-RSRP and L1-SINR if supported.

        - The detailed definition of intra- and inter-frequency CSI-RS based L1 measurement is up to RAN4.

## CSI-RS Configuration

For the use of CSI-RS, we need to consider how to configure the CSI-RS in terms of type, i.e. beam management, CSI-RS for tracking, CSI, mobility and CSI-IM. For the consideration that LTM aims to perform seamless cell switch without performance degradation, the CSI-RS for beam management should be supported as it can operate to select the narrow beam which is able to achieve higher performance than SSB.

      - Proposal 4: RAN1 to discuss the type of CSI-RS applied to L1 measurement.

        - At least CSI-RS for beam management is supported in Rel-19 LTM.

Three types of CSI-RS periodicity, i.e. periodic, semi-persistent and aperiodic manner, can be considered for candidate cell L1 measurement. This is also associated with the report types as summarized in Table 1 below:

Table 1: association between report and CSI-RS periodicity

| Report | CSI-RS |
| --- | --- |
| Periodic | Periodic |
| Semi-persistent | Periodic, Semi-persistent |
| Aperiodic | Periodic, Semi-persistent, Aperiodic |

For periodic CSI-RS, based on the subsequent property for LTM, it is straightforward for a gNB including candidate cells that the configured CSI-RS needs to be always transmitted while the interference might be expected since the candidate cells aren’t aware of the presence of LTM UEs but RRC reconfiguration is unavailable. For semi-persistent and aperiodic CSI-RS, it is reasonable from the viewpoint of the interference as the CSI-RS will be transmitted at the time when the UE measures the candidate cell. However, it will have an impact to RAN3 since the serving cell needs to indicate the timing for CSI-RS transmission to candidate cell(s) when the suspension of CSI-RS transmission is allowed. Based on our analysis, it is required further discussion about what kind of CSI-RS is needed.

      - Proposal 5: RAN1 to discuss what kind of CSI-RS periodicity is specified for LTM taking into account the following aspects:

        - Periodic CSI-RS, which may require candidate cells to always transmit CSI-RS for LTM measurement based.

        - Semi-persistent and aperiodic CSI-RS, which allows suspension of CSI-RS transmission at the candidate cells and may impact RAN3 specification to inform transmit/suspend timing of the CSI-RS for LTM measurement.

If L1-SINR is supported, RAN1 also needs to discuss how the interference is measured. Based on the current mechanism for L1-SINR measurement, CSI-IM needs to be introduced for candidate cells.

      - Proposal 6: If L1-SINR measurement is supported for LTM, CSI resource for candidate cells, e.g. CSI-IM, needs to be introduced to measure the interference.

# Reporting framework

In Rel-19 LTM, we will challenge to adopt the CSI-RS framework as an enhancement of Rel-18 LTM. To accommodate the candidate cell measurement report in Rel-19, two schemes for the measurement report need to be discussed, i.e., the gNB scheduled reporting in Rel-18 and the event triggered reporting in Rel-19. In the subsequent sections, we discuss the details about the schemes.

## gNB scheduled reporting with CSI-RS

The gNB scheduled reporting is the mechanism introduced in Rel-18. Meanwhile, this reporting may be enhanced in Rel-19 LTM because of the introduction of CSI-RS based measurement, which is not clearly captured in the latest WID.

### Supporting the report of L1 measurement results based on CSI-RS

As we discussed in Section 2, the L1 measurement based on CSI-RS provides several merits to measure the channel with narrow beam and/or in terms of interference. While event triggered report is useful to reduce the overhead, gNB scheduled reporting is also useful to obtain the comprehensive information of candidate cells around a UE. Therefore, we propose to support conveying CSI-RS based L1 measurement results using Rel-18 gNB scheduled reporting.

      - Proposal 7:  CSI-RS based LTM measurement result can be conveyed by gNB scheduled reporting.

### Report Quantity

Again, the CSI-RS can perform fine measurement the interference, especially for inter-frequency cell measurement. If L1-SINR measurement based on CSI-RS is supported, we believe it is natural to support conveying it using gNB scheduled reporting. Therefore, we propose that the L1-RSRP and L1-SINR based on CSI-RS measurement should be supported in Rel-19 LTM.

      - Proposal 8: L1-RSRP and L1-SINR based on CSI-RS measurement should be supported for gNB scheduled reporting.

### Report Container

In Rel-18 LTM, the UCI is used to transmit the measurement results. It is beneficial to reuse the same report container with Rel-18 due to the limited TU.

      - Proposal 9: Reuse the same report container with Rel-18, i.e. UCI over PUCCH and PUSCH with periodic/aperiodic/semi-persistent reporting, for gNB scheduled reporting carrying CSI-RS based L1 measurement results.

### Report Format

The report format of Rel-18 LTM is well defined in Table 6.3.1.1.2-8C of 38.212. As we can see in the table, the existing report format can include only SSB as the CSI-RS is not adopted in Rel-18. We think, however, the existing table can also be reused for CSI-RS measurement by adding CRI in the table as marked with cyan as shown in below table. Also, we can add L1-SINR results into the table if L1-SINR is supported as marked in magenta as shown in below table.

Table 6.3.1.1.2-8C: Mapping order of CSI fields of one report for SSBRI/RSRP reporting for L1/L2-triggered mobility

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | CRI or SSBRI # L×M as in Table 6.3.1.1.2-6, if reported |
|  | SINR or RSRP #1 as in Table 6.3.1.1.2-6, if reported |
|  | Differential SINR or RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | Differential SINR or RSRP # L×M as in Table 6.3.1.1.2-6, if reported |

      - Proposal 10: Reuse the same report format with Rel-18 by adding CRI and SINR for gNB scheduled reporting carrying CSI-RS based measurement results, if supported.

### Filtering

The necessity of specifying filtering for L1 measurement results has been discussed in Rel-18 for both time domain and cell domain. However, no consensus was achieved at that time. In Rel-19 the situation may be different if L1-SINR based on CSI-RS is introduced. For the case that L1-SINR is adopted as the measurement quantity, its fluctuation is generally more severe than L1-RSRP. Consequently, the one-shot report might be unstable to use in cell switch determination. By applying L1 filtering, the reported measurement result might be more stable.

On the other hand, for the gNB scheduled reporting, multiple measurement results can be obtained and hence the necessary filtering operation can still be performed at gNB side, similarly with L1-RSRP. Therefore, while L1 filtering at UE side can mitigate the ping-pong problem, its necessity and effectiveness in the context of Rel-19 LTM are debatable.

      - Proposal 11: L1 filtering for gNB scheduled measurement report is not needed for CSI-RS based L1 measurement results including L1-RSRP and L1-SINR.

## Event triggered reporting

### Reference signal for the event triggered reporting

Similar to the gNB scheduled reporting described in previous section, the UE can also use SSB and CSI-RS for the measurement, which are important for Rel-19 LTM operations.

      - Proposal 12: Support both SSB and CSI-RS as a CSI resource for event triggered reporting.

### Report Quantity

For L3 event triggered reporting, RSRP, SINR and RSRQ are defined as the quantity, and the gNB can indicate one of them to the UE. Similarly, as we discussed in previous section, L1-RSRP and L1-SINR can be the report quantity in event triggered reporting, too. By acquiring either/both L1-SINR and L1-RSRP as reporting quantity, the gNB can make more informed and precise cell switch decisions.

      - Proposal 13: Support L1-RSRP based on SSB, L1-RSRP based on CSI-RS and L1-SINR based on CSI-RS as a reporting quantity.

### Report Container

In Rel-18, the introduction of event triggered reporting for LTM was discussed, but there is no consensus to adopt it due to lack of time. At that time, the use of MAC CE, rather than UCI, was proposed by the FL to reduce the workload in RAN1. Compared with UCI, MAC CE offers distinct advantages. Firstly, it can carry large amounts of data which is needed to be reported. Also, the reliability is guaranteed since it is using the PUSCH, and thus the retransmission can be achieved if initial transmission is failed. Finally, it can efficiently manage the allocation of resources, reducing the waste of resources that might otherwise occur with UCI. Additionally, MAC CE allows for the transmission of extensive measurement data on the comparably large PUSCH resources. Corresponding to the triggering condition, the UE can send a scheduling request to request PUSCH resources. The flexibility of MAC CE also extends to its suitability for event triggered reports, where the predefined resources used in UCI on PUCCH/PUSCH is not required to be allocated in advance. The adaptability is crucial for the event triggered report where the gNB cannot predict when the UE is triggered to transmit the reports from the viewpoint of the resource usage efficiency. On the other hand, the major advantage of the reporting with UCI is short latency from event triggered to report. In addition, from the discussion and decision in Rel-19 beam management in MIMO session, the mechanism to use UCI for event triggered reporting is almost ready.

Comparing the benefit and drawback for each approach, we think MAC CE based solution would be more suitable for LTM, and we suggest adopting it in Rel-19.

      - Proposal 14: Support MAC CE as a report container for event triggered L1 measurement.

### Report Format

If the report container is decided to MAC CE, then the report format should be defined by RAN2 because the format of MAC CE is RAN2’s task. This means that the discussion should be led by RAN2 but RAN1 can help their discussion when required. On the other hand, for the case that the report container is decided to UCI, RAN1 needs to discuss further such as whether current format can be reused or not. However, to reduce the workload in RAN1, we believe the report format for LTM in Rel-18, i.e. SSBRI + 7bit absolute value + 4bit differential value(s), SpCellInclusion, L cells x M beams, should be the baseline and necessary fields can be added based on the necessity.

      - Proposal 15: For the reporting format for event triggered reporting,

        - If MAC CE is supported as a container, the discussion on report format should be led by RAN2.

        - If UCI is supported as a container, the report format is discussed in RAN1 and the format used for gNB scheduled reporting can be the baseline.

### Filtering

In previous RAN2 #126 meeting, RAN2 decided that it is up to RAN1 whether the specified L1 filtering is needed or not as shown in below.

RAN2 #126 Agreement RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

The purpose of the RAN1 filtering is to resolve frequent cell change, named the ‘ping-pong’ issue, due to the unreliable measurement results. Additionally, Time-to-Trigger (TTT) is also one of the solutions to cope with the ping-pong issue, and the details of event evaluation are still ongoing in RAN2. What we have to discuss in RAN1 is whether the specified L1 filtering is needed or not considering the fact the event triggered reporting can convey only one result for each beam/cell if the same report format as Rel-18 is adopted.

Adopting filtering methods in Rel-19 LTM has following advantages and disadvantages. The primary goal of such filtering is to mitigate the ping-pong issue, where rapid and unstable L1 measurement changes cause frequent and unnecessary cell switches. There are two main filtering methods, time-domain (beam level) and spatial domain (cell level) filtering.

The time domain filtering is viewed as immature, largely because additional latency which is undesirable for the quick cell switching process. This method can be done in gNB or UE side. On the other hand, spatial domain filtering, which involves averaging values from multiple beams within a cell, seems more promising. By reporting these averaged statics to the gNB, this method can effectively reduce the impact of variance among different beams in a candidate cell. However, the drawback is that the configuration might be more complex than time domain filtering.

More importantly, we RAN1 should be considered that the TTT and event evaluation introduced in RAN2 may achieve the same benefit as filtering. The introduction of specified L1 filtering in RAN1 should be decided taking this aspect into consideration as RAN2 is a leading group of this aspect.

      - Proposal 16: RAN1 to discuss the necessity to specify filtering method of spatial domain and/or time domain.

        - RAN1 should start the discussion after checking the details of TTT and event evaluation defined in RAN2.

# Beam management by CSI-RS

## TCI-state CSI-RS

In Rel-18 LTM RRC parameters, the unified and separate TCI state have been defined for LTM in ltm-DL-OrJointTCI-StateToAddModList and/or ltm-UL-TCI-StateToAddModList. However, it is associated with SSB and TRS while the full configuration of CSI-RS has been defined in the Rel-18 ASN.1 definition. As described in the WID, the beam management by CSI-RS is supported in Rel-19, and what we need to support the beam management by CSI-RS in LTM is, to update the restriction between CSI-RS and Rel-18 beam management framework captured in 38.213 below.

Section 21 of 38.213

A UE can be provided by a LTM Cell Switch Command MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI state ID and/or an UL TCI state ID indicating a CandidateTCI-State and/or CandidateTCI-UL-State from ltm-DL-OrJointTCI-StateToAddModList and/or ltm-UL-TCI-StateToAddModList [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells. The UE may assume that DM-RS antenna ports for PDCCH receptions and for PDSCH receptions are quasi co-located with the SS/PBCH block or the TRS in the TCI state with respect to quasi co-location 'typeA' and 'typeD' properties, when applicable. The UE does not expect to be indicated quasi co-location 'typeA' properties when a SS/PBCH block is configured as a source RS of the TCI state.

      - Proposal 17: The beam management using CSI-RS in Rel-19 LTM should be supported by updating the restriction of QCL RS on the candidate TCI states described in TS38.213.

# Other necessary L1 enhancements

In this section, we discuss the interpretation of “other necessary L1 enhancements” described in WID, which intention is not clear, and companies may have different understanding. The potential items are listed in the subsequent sections and our views are also captured there.

## Enhancement on TRS Tracking

The TRS is utilized by LTM UEs for time and frequency tracking, thereby it can achieve improving the overall system performance. One argument in Rel-18 was that TRS tracking is a default behavior and hence should be included in the candidate TCI states for early tracking, rather than using SSB defined from Rel-15 as a fallback behavior. It was claimed that TRS is usually used as UE common signal (even though the configuration is UE specific) and hence the TRS should be available even before LTM cell switch. On the other hand, this means that a candidate cell has no idea whether the TRS transmitted to UEs connected to the candidate cell is monitored by UEs connected to neighbor cells, resulting in the situation that the candidate cell cannot suspend TRS transmission. To solve this issue, UE capability and gNB configurability were introduced in Rel-18, and the two types of UE behavior shown in Figure 1 (i.e., UE Behaviour 1 and UE Behaviour 2) were introduced.

Figure 1. Possible UE behaviours to apply TRS in LTM

UE Behaviour 1 applies when the UE is capable of TRS tracking for candidate cells. After receiving the UE capability, the serving cell can configure TRS included in candidate TCI states. The UE can track the TRS right after the candidate TCI states are activated even before cell switch. In our understanding, this behaviour assumes that TRS is always transmitted at the candidate cell. On the other hand, UE Behaviour 2 applies when the UE is incapable of TRS tracking for candidate cell. In this case, the serving cell will not configure TRS in candidate TCI states. Hence, the UE starts the tracking using SSB until the new TCI state is activated at the target cell after cell switch.

Based on this history, there might be two potential enhancements in Rel-19.

Firstly, in Rel-18, another UE behaviour other than two UE behaviours in the Figure.1 was proposed but not specified, i.e., even when candidate TCI state including TRS is activated before CSC, the TRS is available after receiving CSC as shown in ‘UE Behaviour 3’ in Fig.1. The benefit of this behaviour is that the candidate cell can start the transmission of TRS after the cell switch is informed from the serving cell.

Secondly, in Rel-18, RAN1 assumed that the LTM procedure is based on SSB rather than CSI-RS. Besides, under the MIMO discussion in Rel-19, discussion on the introduction of only TRS TCI state is ongoing, and this might be applicable to Rel-19 LTM, but the essentiality needs to be assessed in RAN1.

      - Proposal 18: RAN1 to discuss the necessity of enhancements for TRS configured with candidate TCI state for Rel-19 LTM.

        - Potential enhancement 1: Introduce a UE behaviour to track SSB before CSC and to track TRS after reeiption of cell switch command, which allows more flexible TRS transmission at candidate cells.

        - Potential enhancement 2: TRS only TCI state for candidate cells.

## CSI Acquisition before/during LTM

Another aspect is to improve the LTM performance by CSI acquisition before/during LTM. The main benefit of the early CSI acquisition is to provide a suitable MCS right after cell switch, resulting in the avoidance of performance degradation. The serving cell can indicate to transmit the CSI report to the UE for a candidate cell ‘before LTM’ or including command in LTM cell switch command for ‘during LTM’. However, there are potential drawbacks of the early CSI acquisition, e.g., the large gap compared to channel variation between the CSI report and first DL scheduling after cell switch (which may cause performance degradation due to MCS mismatch, though), necessity of measurement gap for inter-frequency CSI measurement, and/or introduction of candidate cell CSI report. Based on above analysis, further discussion is required to mitigate such trade-off relationship.

      - Proposal 19: RAN1 to discuss the necessity of CSI acquisition before/during LTM considering throughput gain and/or spec impact.

## UL measurement

UL measurement was discussed in Rel-18 LTM, but there was no consensus. One significant advantage of UL measurement is its potential to reduce measurement delay and decrease the processing complexity at the UE side. More precisely, in contrast to DL measurement, the UL measurements involve the reference signal (i.e., SRS) transmitted by UE. Then, the SRS is measured by the candidate cells, and the measurement results are feedback to serving cell for cell switching decision. Given that the gNB has superior data processing capabilities compared to UE, UL measurement might be efficient from the viewpoint of processing delay.

On the other hand, UL measurement also has several challenges. One key issue is the increased power consumption at the UE side, as the UE is required to transmit SRS, periodically or semi-persistently, which may include intra-frequency and inter-frequency. Also, the candidate cells may not have any timing advance information of the UE. From the reason, the measurement result may have gap from actual channel state. Additionally, the gNB may face considerable challenges in managing UL resources for SRS, especially when there are excessive UEs in its coverage. For the case that the hundreds of SRS are transmitted by UEs, it causes large resource wasting that the resources for PUSCH gets smaller, accordingly, the overall throughput will decrease.

For the consideration that UL resources ratio to DL resources is small portion in TDD system, UL measurement may not be beneficial. Therefore, we propose that the UL measurement should deprioritize in Rel-19 LTM discussion.

      - Proposal 20: the UL measurement should deprioritize in Rel-19 LTM discussion.

## Measurement configuration update

When a large number of candidate cells/beams are configured for the LTM measurements, it becomes challenges for the UE to measure all configured cells/beams in terms of processing burden, which might be more serious when CSI-RS based measurement is introduced in Rel-19 as RRC reconfiguration is not available due to subsequent property for LTM. In Rel-18, we had discussed the gNB can indicate a subset of candidate cells/beams for the UE, saying measurement configuration update, based on e.g., the historical data to measure through MAC CE. This method provides the UE to focus measurement of the indicated candidate cells/beams, accordingly, and improves the efficiency at UE side. Additionally, the UE can reduce the power consumption since the number of candidate cells/beams to measure gets smaller.

On the other hand, in order to select the candidate cells/beams for the UE, it requires additional complexity at the gNB side. And the reliability issue may occur unless the historical data is high precision. As a summary, the use of the measurement configuration update in the LTM can offers benefits in terms of efficiency and power savings while it introduces additional complexity and potential challenges to select suitable candidate cells/beams for measurement.

We think it is needed to discuss further to specify to support measurement configuration update.

      - Proposal 21: RAN1 to discuss further about the measurement RS update in Rel-19 LTM taking into account following pros and cons:

        - Pros: Improving measurement efficiency / reducing power consumption at UE side especially when large number of CSI-RS is configured.

        - Cons: Increasing complexity to select candidate cells/beams.

## CSI-RS operation for intra- and inter-CU

Figure 2 . Scenario for intra- and inter-CU for LTM

Although RAN1 is not involved in the inter-CU scenario described in the WID, we need to further study how to transmit CSI-RS from candidate cells whose CU is different from the serving cell. It is noted that RAN2 has discussed if CSI-RS operation is possible for inter-CU scenario and RAN1 confirmation is necessary. Figure 2 shows the scenario for both intra- and inter-CU for LTM. Generally, the candidate cell CSI-RS measurement is coordinated and managed by the CU, which includes CSI-RS transmission and UE measurement and report. The CSI-RS transmission is managed by candidate CU while UE measurement and reporting is managed by serving CU. So, for the intra-CU case, the UE remains connected to the same CU, even for the LTM cell switch cases as shown in above Fig.2. The CSI-RS resources are managed solely by the CU. This centralized CSI-RS management scheme provides the optimized CSI-RS configuration coordination including CRS-RS resource allocation and CSI-RS transmission/suspension timing at the candidate cells plus UE’s timing to measure the CSI-RS for TRS, L1 measurement etc.

On the other hand, for the inter-CU case, the UE#2 receives the CSI-RS from candidate cells with different CUs. In this case, CSI-RS configuration should be coordinated by both CUs. It might be required signalling between CUs through Xn interface to configure the CSI-RS transmission and to receive the measurement reports based on the CSI-RS measurement from candidate cells under different CU when transmission/suspension of CSI-RS is flexibly controlled. Furthermore, the resource management may be more complex than that of intra-CU case, and therefore it has additional specification impact to define Xn interface related with CSI-RS coordination. From this reason, it is needed to discuss how to support the CSI-RS transmission/suspension for the inter-CU scenario.

      - Proposal 22: RAN1 to discuss whether/how to support CSI-RS transmission/suspension for the inter-CU scenario for the consideration of following aspects:

        - Since the CSI-RS measurement and reporting framework is managed by serving CU and CSI-RS transmission/suspension is managed by candidate CU, inter-CU coordination is required.

        - Unless the always-on CSI-RS is assumed, it is essentially required the standardization efforts across working group (e.g., RAN2 and RAN3) to support inter-CU LTM.

# Conclusion

In this contribution, we provide our views as below.

[CSI-RS measurement for LTM procedure]

      - Proposal 1: Support L1-RSRP measurement based on CSI-RS.

      - Proposal 2: Support L1-SINR measurement based on CSI-RS.

      - Proposal 3: Support intra- and inter-frequency L1 measurement based on CSI-RS.

        - Applicable to L1-RSRP and L1-SINR if supported.

        - The detailed definition of intra- and inter-frequency CSI-RS based L1 measurement is up to RAN4.

      - Proposal 4: RAN1 to discuss the type of CSI-RS applied to L1 measurement.

        - At least CSI-RS for beam management is supported in Rel-19 LTM.

      - Proposal 5: RAN1 to discuss what kind of CSI-RS periodicity is specified for LTM taking into account the following aspects:

        - Periodic CSI-RS, which may require candidate cells to always transmit CSI-RS for LTM measurement based.

        - Semi-persistent and aperiodic CSI-RS, which allows suspension of CSI-RS transmission at the candidate cells and may impact RAN3 specification to inform transmit/suspend timing of the CSI-RS for LTM measurement.

      - Proposal 6: If L1-SINR measurement is supported for LTM, CSI resource for candidate cells, e.g. CSI-IM, needs to be introduced to measure the interference.

[Reporting Framework]

(gNB scheduled reporting with CSI-RS)

      - Proposal 7:  CSI-RS based LTM measurement result can be conveyed by gNB scheduled reporting.

      - Proposal 8: L1-RSRP and L1-SINR based on CSI-RS measurement should be supported for gNB scheduled reporting.

      - Proposal 9: Reuse the same report container with Rel-18, i.e. UCI over PUCCH and PUSCH with periodic/aperiodic/semi-persistent reporting, for gNB scheduled reporting carrying CSI-RS based L1 measurement results.

      - Proposal 10: Reuse the same report format with Rel-18 by adding CRI and SINR for gNB scheduled reporting carrying CSI-RS based measurement results, if supported.

      - Proposal 11: L1 filtering for gNB scheduled measurement report is not needed for CSI-RS based L1 measurement results including L1-RSRP and L1-SINR.

(Event triggered reporting)

      - Proposal 12: Support both SSB and CSI-RS as a CSI resource for event triggered reporting.

      - Proposal 13: Support L1-RSRP based on SSB, L1-RSRP based on CSI-RS and L1-SINR based on CSI-RS as a reporting quantity.

      - Proposal 14: Support MAC CE as a report container for event triggered L1 measurement.

      - Proposal 15: For the reporting format for event triggered reporting,

        - If MAC CE is supported as a container, the discussion on report format should be led by RAN2.

        - If UCI is supported as a container, the report format is discussed in RAN1 and the format used for gNB scheduled reporting can be the baseline.

      - Proposal 16: RAN1 to discuss the necessity to specify filtering method of spatial domain and/or time domain.

        - RAN1 should start the discussion after checking the details of TTT and event evaluation defined in RAN2.

(Beam management by CSI-RS)

      - Proposal 17: The beam management using CSI-RS in Rel-19 LTM should be supported by updating the restriction of QCL RS on the candidate TCI states described in TS38.213.

[Other L1 enhancement]

(Enhancement on TRS tracking)

      - Proposal 18: RAN1 to discuss the necessity of enhancements for TRS configured with candidate TCI state for Rel-19 LTM.

        - Potential enhancement 1: Introduce a UE behaviour to track SSB before CSC and to track TRS after reeiption of cell switch command, which allows more flexible TRS transmission at candidate cells.

        - Potential enhancement 2: TRS only TCI state for candidate cells.

(CSI Acquisition before/during LTM)

      - Proposal 19: RAN1 to discuss the necessity of CSI acquisition before/during LTM considering throughput gain and/or spec impact.

(UL measurement)

      - Proposal 20: the UL measurement should deprioritize in Rel-19 LTM discussion.

(Measurement configuration update)

      - Proposal 21: RAN1 to discuss further about the measurement RS update in Rel-19 LTM taking into account following pros and cons:

        - Pros: Improving measurement efficiency / reducing power consumption at UE side especially when large number of CSI-RS is configured.

        - Cons: Increasing complexity to select candidate cells/beams.

(CSI-RS operation for intra- and inter-CU)

      - Proposal 22: RAN1 to discuss whether/how to support CSI-RS transmission/suspension for the inter-CU scenario for the consideration of following aspects:

        - Since the CSI-RS measurement and reporting framework is managed by serving CU and CSI-RS transmission/suspension is managed by candidate CU, inter-CU coordination is required.

        - Unless the always-on CSI-RS is assumed, it is essentially required the standardization efforts across working group (e.g., RAN2 and RAN3) to support inter-CU LTM.

# References

[1] RP-241515, Revised Work Item: NR mobility enhancements Phase 4 3GPP TSG RAN meeting #104 Shanghai, China, June 17-20, 2024