3GPP TSG RAN WG1 #118                                                              R1- 2406830

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.1.3.3

Source:	Apple Inc.

Title:	Discussion on other aspects of AI/ML model and data

Document for:	Discussion/Decision

# Introduction

General aspects of AI life cycle management are identified for further study in R19 [1].

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24):

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

In this paper, we discuss the related aspects.

# Discussion

## Model identification.

Agreement

- To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases.

- To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases

- MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s)

- MI-Option 2: Model identification with dataset transfer

- MI-Option 3: Model identification in model transfer from NW to UE

- FFS: The boundary of the options

- Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose

- Note: other options are not precluded

In RAN1 116, the discussion was mainly focused on the necessity of model identification.

MI-Option 1:

For MI-option 1, this can be used in one sided model to handle NW side additional condition. In RAN1 #116bis meeting, the following agreement on MI-option 1 was captured.

Agreement

From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity)

- A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s)

  - Associated IDs for each sub use case in relation with NW-sided additional conditions

- B: UE(s) collects the data corresponding to the associated ID(s)

- C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s).

- D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model

  - relationship between model ID(s) and the associated ID(s)

  - How model ID(s) is determined/assigned, e.g.,

    - Alt.1: NW assigns Model ID

    - Alt.2: UE assigns/reports Model ID

    - Alt.3: Associated ID(s) is assumed as model ID(s)

      - “Model ID is determined/assigned for each AI/ML model” in D is not needed

    - Alt.4: Model ID is determined by pre-defined rule(s) in the specification

  - FFS: how to report

  - Note: D is to facilitate AI/ML model inference

- Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

In the example, there are a few FFS to be further clarified.   On the relationship between model ID and the associated ID, Alt 3 should be the most straightforward solution.

For Alt 1, where NW assigns a model ID to the UE side model, the goal is to enable NW control which AI model that UE should use during inferencing. However, since this is UE sided model, how UE implement the AI model is UE’s proprietary information, and NW does not have the information on the detailed model, therefore NW control which physical AI model runs at the UE is not feasible.

For Alt 2, UE assigns/report model ID. Since the main purpose is to align NW side additional condition for UE side model, we do not see any motivation for UE to assign a model ID that is different from the association ID, with additional signaling complexity of letting gNB know how the model ID is mapped to the association ID.

Proposal 1: In MI-option 1, the associated ID(s) is assumed as model ID(s).

MI-Option 2:

In RAN1 117, an example is agreed for MI-option 2, which is mainly used for CSI compression with training collaboration type 3, where the training dataset for the sequential training needs to be delivered from the first training entity to the second training entity. In the example, it is “FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID)”.

Proposal 2: In MI-option 2, the dataset ID(s) is assumed as model ID(s).

MI-Option 3:

MI-option 3 is mainly used for CSI compression with training collaboration type 1, where the model is trained at one entity and delivered to another entity for inferencing. The model ID is assigned by the NW and transferred together with the model parameters. Additional meta information can be attached to the model transfer to provide more description to UE.

Proposal 3: In MI-option 3, the NW assign the model ID and transferred together with model parameters.

## Association ID

Working Assumption

Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell.

- FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study).

In RAN1 117, association ID in MI-option 1 was discussed and the following working assumption are captured.

In order to reduce UE side data collection, model training and management complexity, it is critical to ensure the association ID is applied to multiple cells. Otherwise, UE needs to collect large amount of training data for every cell. Although it is possible to train one model for multiple cells, each with different association ID, the amount of engineering effort is prohibitive in order to categorize data, design model and determine which model cover which cells.

To enable multiple cells, consistency area can be defined by the NW, and NW configure the consistency area with one association ID. The signaling of consistency area can reuse some concept defined in MDT, such as a list of global cell identifies, a list of area IDs etc.

Proposal 4: Association ID can be applicable for multiple cells. A list of global cell IDs can be signaled per association ID.

## Model transfer

Model transfer was extensively discussed throughout the entire R18 study. Different sub-types of model delivery and transfer cases z1 to z5 are defined, and potential advantages and drawbacks from the UE perspective and the NW perspective are captured in section 4.3 of TR 38.843.

In RAN1 116 and RAN1 116bis, model transfer case z5 and z3 are deprioritized. Model transfer case z2 are deprioritized for UE side model.

Conclusion:

From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19.

Conclusion

From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons:

- Risk of proprietary design disclosure

- Burden of offline cross-vendor collaboration

Conclusion

From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y):

- No much benefit compared to Case y

- Risk of proprietary design disclosure

- Large burden of offline cross-vendor collaboration

- Additional burden on model storage within in 3GPP network

For case z1, comparing to case y where the UE side model is stored outside of 3GPP network, the main benefit is lower latency for model delivery. However, the drawback includes the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, and burden on model maintenance/storage on network side.

For case z4, an example of model transfer is agreed in RAN1 117.

Agreement

From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits):

- Alt. A

  - Step A-1: UE reports the supported known model structure(s) to network

  - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1

  - FFS: whether some additional step(s), and/or whether other information is needed

- Alt. B

  - Step B-0: UE reports to NW its support of model transfer/delivery case z4

  - Note: Step B-0 may be before or after Step B-1, or not necessary

  - Step B-1: NW indicates to UE the candidate known model structure(s)

  - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported

  - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2

  - FFS: whether some additional step(s), and/or whether other information is needed

- Note: Other alternative(s) is not precluded

- Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion.

In related CSI compression, Option 3a/3b and 5a/5b are related to model transfer z4 are discussion.

Agreement

- For Option 3, further define the two sub-options:

  - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 5, further define the two sub-options:

  - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

In order to reflect the overall discussion, also indicate whether UE will be able to perform inferencing after receiving the model, the additional steps and information required including:

- UE indicating whether model can be used directly for inference, together with the supported known model structure to NW.

- NW indicate the model ID and related meta information to the UE before model transfer.

- UE confirm whether model transfer is required. UE can send negative indication if the model is already transferred before.

Below figure shows an example of additional steps and information for Alt A. Similar steps are required for Alt. B.

Fig. 1 Model transfer z4 procedure example

Proposal 5: For model transfer z4, additional steps and information are added for Alt A and Alt B, including:

- UE indicating whether model can be used directly for inference, together with the supported known model structure to NW.

- NW indicate the model ID and related meta information to the UE before model transfer.

- UE confirm whether model transfer is required. UE can send negative indication if the model is already transferred before.

## UE side training data collection

7.2.1.3.2	Data collection for UE-side model training

The following proposals were discussed in RAN2:

1.	UE collects and directly transfers training data to the Over-The-Top (OTT) server;

1a)	OTT (TRansparent)

1b)	OTT (non-TRansparent)

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the needed data to the OTT server.

RAN2 did not study or analyse these proposals and did not agree to requirements or recommendations.

Data collection for UE-side model training framework was discussed in RAN2 and the following summary was captured in TR.

Offline training is considered in R18 SI and R19. In [3], RAN1 analyzed the requirement of UE side training data collection for each of the use cases including CSI compression, CSI prediction, BM and positioning. The latency requirement for all use cases is “relaxed”. The data sample size can be up to 1.5Mbits per training sample for CSI prediction, and the UE can aggregate multiple samples before transmitting to the OTT server due to relaxed latency requirement. Therefore, the overall data file size can be quite big.

The training dataset is transmitted to the OTT server for offline training. Currently, option 1-1a is used in industry. From RAN1 perspective, there is no additional requirement identified for other types of enhancement.

Proposal 6: From RAN1 perspective, option 1-1a is sufficient and no additional requirement is identified to enhance data collection for UE side model training.

# Conclusion

In this contribution, we discussed aspects on R19 AI based enhancement SI and WI scope. Based on the discussion, the following proposals have been proposed.

Proposal 1: In MI-option 1, the associated ID(s) is assumed as model ID(s).

Proposal 2: In MI-option 2, the dataset ID(s) is assumed as model ID(s).

Proposal 3: In MI-option 3, the NW assign the model ID and transferred together with model parameters.

Proposal 4: Association ID can be applicable for multiple cells. A list of global cell IDs can be signaled per association ID.

Proposal 5: For model transfer z4, additional steps and information are added for Alt A and Alt B, including:

- UE indicating whether model can be used directly for inference, together with the supported known model structure to NW.

- NW indicate the model ID and related meta information to the UE before model transfer.

- UE confirm whether model transfer is required. UE can send negative indication if the model is already transferred before.

Proposal 6: From RAN1 perspective, option 1-1a is sufficient and no additional requirement is identified to enhance data collection for UE side model training.

# Reference

[1] RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023

[2] TR 38.843, Study on AI/ML for NR air interface, Nov 2023

[3] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, Xiamen, China, Oct 2023