3GPP TSG RAN WG1 #118	                                                                                       R1-2406889

Maastricht, NL, August 19th – 23rd, 2024

Source:	Meta

Title:	Other Aspects of AI/ML Model and Data

Agenda item:	9.1.3.3

Document for:	Discussion and Decision

# Introduction

In Rel-19, RAN1 is studying the details of model identification, model transfer/delivery, as well as training data collection for AI/ML with the following objectives from the AI/ML for NR Air Interface Work Item [1]:

Objective: - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

In this paper, we provide some views on the study item based on agreements made in the last three RAN1 meetings for 5G NR Rel-19.

# Discussion

## Model Identification

Agreement - To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases. - To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) - MI-Option 2: Model identification with dataset transfer - MI-Option 3: Model identification in model transfer from NW to UE - FFS: The boundary of the options - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose - Note: other options are not precluded Observation The other options are proposed for model identification type B by companies during the discussion: - MI-Option 4. Model identification via standardization of reference models. (for CSI compression) - MI-Option 5. Model identification via model monitoring Agreement - Regarding MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)) of model identification type B, RAN1 further study the following aspects: - Relationship between model ID and data collection related configuration(s) and/or indication(s) - Information transmitted from NW to UE (if any) - Information transmitted from UE to NW (if any) - The associated procedure - Usage/Applicable use case(s) of MI-Option 1 Note: whether MI-Option 1 is needed or not is a separate discussion Conclusion - It is clarified that MI-Option 4 refers to the Option 1 of CSI compression o Option 1: Fully standardized reference model (structure + parameters)

Model identification refers to assignment of a model ID either to a logical or physical AI/ML model such that the model life cycle management procedure can leverage the model ID to maintain different AI/ML functions or models in the 3GPP network. In general, either functionality-based LCM (where LCM is performed for a specific functionality and not on physical or logical model) or Model ID based LCM is assumed. However, based previous agreements in RAN1, model ID can also be used within a functionality i.e., model ID based LCM is essentially a sub-class of functionality-based LCM.

Model identification has multiple uses within the 3GPP network, but the most important use cases are to facilitate model transfer, identifying and matching UE and NW-sided parts of two-sided models (currently only limited to CSI compression use case) and finally to ensure consistency of NW and/or UE side additional conditions for training and inference in different AI/ML use cases. While the nature of model transfer delivery may be quite limited for Rel-19 and support of two-sided models is debatable, the last use case is very critical for supporting practical deployment of AI/ML models for NR air interface.

To maintain consistency of additional conditions across a variety of use cases, Model Identification Type A i.e., offline model identification would be quite limiting as it may not be adaptable to dynamic network conditions. Therefore, in addition to Type A, over-the-air model identification Type B should also be supported. Furthermore, for MI-Options 1-3 from the above agreement, at least MI-Option 1 and 2 can be supported since this can aid in consistency of additional conditions. While MI-Option-3 can also help in this regard, it may be much more complicated and depend more on the capabilities of the UE with respect to the types of models that can be supported for model transfer from the NW-side.

- Support model ID based model identification Type B with at least MI-Options 1, 2

In our view, MI-Option-4 may be difficult for general AI/ML use cases and can be specific to only uses cases (like CSI compression) where it is inherently necessary. For MI-Option-5, the details are not very clear and may introduce the burden of increased model monitoring. Therefore, we prefer to prioritize the other MI options.

For MI-Options 1 and 2, the following agreements were made in previous RAN1 meetings:

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification. Agreement From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset

For MI-Option 1, steps A-C are necessary for support of UE side AI/ML model training and inferencing with consistency of additional conditions. The associated ID can be related to data collection configurations of each sub-use case. For example, in beam management, the associated ID can be an ID associated with each set A/B configuration for a UE. In our view, since the model ID is envisioned mainly for LCM purposes, associated ID can be mapped directly to the model ID. If the physical model at the UE side has a many to one mapping or other implementation-based mapping, it may not need to be reported to the network.

- For MI-Option-1 support steps A-C for ensuring consistency of network side additional conditions and use Alt-3 for associating model IDs to associated ID i.e., associated ID is assumed to be model ID

For MI-Option 2 from AI-Example2-1, a dataset ID similar to associated ID for MI-Option 1 can be considered where UE assumes that the dataset ID is mapped directly to the model ID. Step C may not be explicitly needed in this case. This can apply to both UE side model and UE part of two-sided model.

- For MI-Option-2, a dataset ID similar to associated ID can be used as a model ID for the UE side model or UE part of the 2-sided model and only step A-B are used in this case.

## Model Transfer/Delivery

For model transfer/delivery there were 6 different collaboration levels identified during the Rel-18 study phase [2]:

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

Based on discussions in previous meetings, the following is the current status for collaboration level discussions in the Rel-19 study item with respect to UE-sided and two-sided models:

| Case | UE-sided model | Two-sided model |
| --- | --- | --- |
| y |  |  |
| z1 |  |  |
| z2 | Deprioritized |  |
| z3 | Deprioritized | Deprioritized |
| z4 |  |  |
| z5 | Deprioritized | Deprioritized |

It seems feasible that only collaboration level y (outside 3GPP network) may be supported due minimal impact on 3GPP specifications. But in this case, it can set a precedent for a closed eco-system as well. Therefore, some discussion on the collaboration level z should be facilitated to identify the possibility of model transfers over 3GPP network.

Cases z1 and z4 are under discussion. For case z1, the advantages compared to level y are not very clear since the model is in proprietary format and the UE may end up having to support many combinations for different vendors i.e., cross-vendor collaboration may be very complicated. In our view, level z4 can be a potential way to enable 3GPP based model transfer without placing high burden of supporting multiple proprietary model formats at the UE. Therefore, support of case z4 with known model structure based on UE capability may be further discussed. We are also open to further discussing a set of candidate model structures which the UE may indicate support for using UE capability signaling when the UE supports case z4.

- Further discuss model transfer/delivery with collaboration level z4 where UE can indicate support of a subset of known candidate model structures based on UE capability

# Conclusion

The main proposals from this paper are outlined here:

- Support model ID based model identification Type B with at least MI-Options 1, 2

- For MI-Option-1 support steps A-C for ensuring consistency of network side additional conditions and use Alt-3 for associating model IDs to associated ID i.e., associated ID is assumed to be model ID

- For MI-Option-2, a dataset ID similar to associated ID can be used as a model ID for the UE side model or UE part of the 2-sided model and only step A-B are used in this case.

- Further discuss model transfer/delivery with collaboration level z4 where UE can indicate support of a subset of known candidate model structures based on UE capability

# References

- RP-240774, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting#103, Maastricht, Netherlands, March 2024.

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

- Chairman’s Notes, RAN1#117 Meeting, Fukuoka City, Japan, May 2024.