3GPP TSG RAN WG1 #118	                             	 R1-2407507

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.6.2

Source:	Moderator (Apple Inc.)

Title:	Summary #5 (Final) on LP-WUS operation in IDLE/INACTIVE mode

Document for:	Discussion/Decision

# Introduction

After the completion of the Rel-18 study item on low-power wake-up signal and receiver for NR (TR 38.869), the Rel-19 WI on LP-WUS/WUR was approved in RP-234056. One of the objectives is related to LP-WUS operation in idle/inactive mode:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

This contribution summarizes the discussions on LP-WUS operation in IDLE/INACTIVE modes in RAN1#118.

Section 2 provides a summary of the agreements as the outcome of RAN1#118 discussions. Section 3 captures the proposals for online sessions. Section 4 documents the detailed discussions. Section 5 provides guidance for RAN1#119 discussions. Agreements from previous meetings and companies’ proposals from the contributions are captured in Appendix A.

# RAN1#118 Agreements

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

Agreement

At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams.

Agreement

For the wake-up delay, consider the following options:

Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting.

- FFS: X is 2, 3, 4

Above applies for IDLE/INACTIVE mode.

Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Agreement

For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis:

- Option 1: X = 8

- Option 2: X = 16

- Option 3: X = 32

- Option 4: X = 64

- Option 5: X = 128

- Option 6: X = 256

For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option.

Conclusion

There is no consensus in RAN1 on the support of dynamic PO.

Agreement

On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured.

- FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use

- FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported)

- (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle.

- If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives

  - Alt 1: it monitors the PO associated with the offset.

  - Alt 2: it monitors the first PO after its reported wake-up delay.

  - Other alternatives are not precluded

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

Send an LS to RAN2 and RAN4 to convey the following

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Final LS in R1-2407559.

Conclusion

RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4

# Proposals for Online Sessions

## Proposals for Tue (08/20) Online

Proposal 3-1:

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources that carry LP-SS.

Proposal 1-4r1:

A UE is not required to monitor a PO if it does not detect a LP-WUS on the monitored LO or the LP-WUS does not indicate the subgroup the UE belongs to monitor its associated PO a positive wake-up indication in LP-WUS in an LO.

Note: this does not preclude the discussion of potential fallback behavior if the UE does not receive LP-WUS for a long time.

Proposal 1-5r1:

LP-WUS and LP-SS use the same beams as SSB. Define the association between LP-WUS MOs/LP-SS transmissions and SSB beams.

Proposal 1-3:

Do not support dynamic PO.

Proposal 2-2:

Support separate subgrouping configurations for LP-WUS and PEI.

- Details are left to RAN2 to decide.

## Proposals for Wed (08/21) Online

Proposal 1-6r1:

For the wake-up delay, consider the following options:

Option 1: No UE capability report for the wake-up delay. One value is pre-defined in the specifications for all the UEs and used by gNB.

Option 2: UE reports one value from a set of candidate values for the wake-up delay via UE capability reporting.

  - Option 2A: gNB assumes a single value for all the UEs.

    - E.g. gNB configures a single offset value based on the maximum wake-up delay reported by the UEs.

  - Option 2B: gNB can configure LOs differently for different UEs based on the reported wake-up delay.

  - Option 2C: gNB configures LOs separately from PO, and UE monitors the next PO after the wake-up delay according to its capability.

Proposal 2-1r1:

For the maximum number of subgroups per PO (X) and the maximum number of information bits per LP-WUS (Z), down-select from the following options:

Option 1: X = 8, Z = 3

Option 2: X = 16, Z = 4

Option 3: X = 32, Z = 5

Option 4: X = 256, Z = 8

Proposal 1-3:

Do not support dynamic PO.

Proposal 3-2:

The definitions of LP-RSSI for LP-RSRQ is further updated as follows:

LP-RSSI is the linear average of total received power in all the ON and OFF LP-SS OOK symbols over the frequency resources that carry LP-SS.

Proposal 2-2:

Support separate subgrouping configurations for LP-WUS and PEI.

- Details are left to RAN2 to decide.

## Proposals for Thu (08/22) Online

Proposal 2-1r2:

For the maximum number of subgroups per PO (X), down-select from the following options:

Option 1: X = 8

Option 2: X = 16

Option 3: X = 32

Option 4: X = 64

Option 5: X = 128

Option 6: X = 256

Proposal 2-2:

Support separate subgrouping configurations for LP-WUS and PEI.

- Details are left to RAN2 to decide.

Discuss (related to Section 4.4):

Whether there is a need to continue to discuss the LP-WUS entry/exit condition and RRM measurement offloading/relaxation conditions in RAN1, and

Whether there is a need to send an LS to RAN2/RAN4 to state that RRM measurement offloading or sufficient relaxation is needed in order to achieve power saving gain for LP-WUS.

Proposal 1-2r3:

On the LO configuration for iDRX, consider the following options:

Option A: offset value(s) between a LO and a PO/PF is/are configured.

  - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use

  - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LOs corresponding to Option 1, 2, or 3 (if supported)

  - For each UE, the periodicity of LO is the same as its iDRX cycle.

  - If a UE receives a wake-up indication in a LP-WUS,

    - Alt 1: it monitors the PO associated with the offset.

      - Alt 1A: it monitors the PO associated with the offset if its reported wake-up delay is no larger than the time gap between the LO and the PO; otherwise it does not monitor LP-WUS and falls back to the legacy paging monitoring behavior.

    - Alt 2: it monitors the first PO after its reported wake-up delay.

Option B v1 (the version after offline discussion): a periodicity and an offset are configured by the gNB for LO determination, which is separate from the PO configuration.

  - Periodicity of LO is the same as the default iDRX cycle.

  - FFS the reference point for the offset

  - If a UE receives a wake-up indication in a LP-WUS, it monitors the first PO after the wake-up delay.

  - Option 2A: gNB configures the number of LOs with a LO periodicity, UE determines the LO index according to LO index = [(UE_ID mod N) *Ns + i_s] mod L, where L is the number of LOs within a LO periodicity.

  - Option 2B: …

Option B v2 (the description for the previous Option 2A): gNB configures the number of LOs (L) within the default iDRX cycle and an offset.

  - FFS the reference point for the offset

  - UE determines the LO index according to LO index = [(UE_ID mod N) *Ns + i_s] mod L.

    - Note: this can support Option 1/2, but not Option 3

  - If a UE receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

Discuss whether to support LR and MR on different carriers/bands (related to Section 4.5).

## Proposals for Fri (08/23) Online

LR and MR on different carriers

In RAN1, the common understanding is that LR may support a subset of bands that MR supports. There is a concern that the network may not be able to load balance idle/inactive UEs on different carriers any more, if all the LP-WUS capable UEs camp on the band that LR supports.

Companies that want to support LR and MR on different carriers in R19: vivo, CMCC

Concerns on supporting LR and MR on different carriers:

Significant spec impact in RAN2/RAN4 (entry/exit conditions, RRM offloading/relaxation conditions, RRM requirements, etc.)

Whether the UE power saving gain is still there

There may be some other implementation solutions to address the issue?

This is not a R19 issue?

Proposed working assumption 4-1:

When the UE starts LP-WUS monitoring and stops the legacy paging monitoring,

LR performs serving cell measurement.

MR either performs serving cell measurement with relaxation or does not perform serving cell measurement.

MR either performs neighbor cell measurement with relaxation or does not perform neighbor cell measurement.

Note: The details are left to RAN2 and RAN4 to decide. It can be revisited based on RAN2/RAN4 discussions.

# PHY procedures

## LP-WUS configuration and monitoring

This is a summary of companies’ views on the LO configurations:

- [1] Futurewei: Consider the following options, depicted in Figure 1, for LO configuration and association with the Ns POs per PF where K is the number of LP-WUS MO groups per LO

  - Option 1: a reference point, Ns × K offsets to first LP-WUS MOs, and a time gap

  - Option 2: a reference point, Ns offsets to first LP-WUS MOs, a number (K) and periodicity for LP-WUS MO groups, and a time gap

- [2] HW/HiSi

  - The location of LO can be determined based on the location of a PO/PF by an offset.

  - For mapping between LO and PO, support Option 2 that UEs corresponding to different POs monitor the same LO

  - Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam, K can be larger than or equal to 1.

- [4] CMCC

  - Regarding the determination of LO location, the folloiwng three options cannbe considered:

    - Option 1: gNB configure the time domain offset(s) of LO related to PO.

    - Option 2: gNB configure the periodicity of LO and number of LOs, UE determines the LO index based its PO index, e.g., LO index = [(UE_ID mod N) *Ns + i_s] mod L, L is the number of LOs within LO periodicity

    - Option 3: gNB configure the time domain offset(s) of LO relative to LP-SS

- [5] TCL

  - To locate LP-WUS occasion in the time domain, a reference point and an offset from the reference point can be used:

    - For idle/inactive UEs, the incoming PO or SSB can be used as a reference point

    - For connected UEs, SSB or periodic LP-SS can be used as reference point.

- [6] vivo

  - Proposal 9: Clarify that the same subgrouping information will be carried in the MOs of a LO.

- [7] OPPO

  - A UE can monitor one LO in each LO period.

- [8] Xiaomi

  - Proposal 12: Support separate configuration of LP-WUS, with multiple LP WUS monitoring occasions (LOs) within a duty cycle. And UE can determine its corresponding LO index by its UE ID or by its PO index.

- [9] CATT

  - Proposal 3: The offset between LO and PO is configured by the network and UE will determine the sleeping type based on the transition time of different sleep mode and the network configured offset in its UE implementation.

  - Proposal 4: The number of LP-WUS MOs for each beam can be larger than or equal to 1. The information of paging early indication by LP-WUS in multiple MOs within a SSB can be different.

- [10] ZTE

  - Proposal 7: LP-WUS indicates subgroups based on associated PO.

    - Further discuss whether/how LP-WUS indicates subgroups independent from PO

  - Proposal 8: Support multiple subgroups as much as possible by LP-WUS in idle/inactive state.

  - Proposal 9: Unified LP-WUS monitoring scheme is pursued, considering

    - I-DRX, eDRX

    - FDD, TDD

  - Proposal 10: Each LO consists of N * K LP-WUS MOs, option 2 is adopted with K>1

    - K could be larger than repetition number

  - Proposal 11: Consider option 2 to determine the beam index and repetition.

  - Proposal 12: For duty cycle LP-WUS in IDLE/INACTIVE mode, NW can configure different LO periodicity according to different situation.

  - Proposal 16: Support LP-WUS deployed in FDD/TDD bands

  - Proposal 17: The LO and MO definition should be applicable for both FDD and TDD

    - FFS whether MO could be based on flexible symbols or slots in TDD

- [11] Nokia

  - Proposal: Define starting symbol for each the LP-WUS MOs in LO.

  - Proposal: At least for operation with shared spectrum access, support having one or more LP-WUS MOs per LP-WUS beam (K≥1).

  - Proposal: Support LP-WUS monitoring configuration that allows to split the sub-groups of a PO to multiple LP-WUS MOs. Also mapping multiple POs to same LP-WUS could be supported if efficient encoding of sub-group information to LP-WUS is supported.

- [12] Sony

  - Proposal 1 – Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on an always-on or a duty-cycle scheme.

  - Proposal 2 - For duty-cycled scheme where the LP-WUS monitoring is done in connection to existing DRX, the configuration needs to consider at least the following configuration parameters,

  - Proposal 3 – Support time-frequency multiplexing of LP-WUS for different UEs to increase LP-WUS address/identification space.

- [14] Fujitsu

  - Proposal: Introduce a special LO, which is not associated with any PO, to efficiently wake up all UEs for SI change/ETWS/CMAS

- [15] NEC

  - Proposal 1: for RRC-IDLE/INACTIVE, the LP-WUS configuration indicates one or more LP-WUS frequency bands in a carrier, and for each LP-WUS frequency band, a periodicity and an offset of LOs are comprised in the configuration.

  - Observation 1: explicit association between an LO and a PO (e.g., by a time offset) is not necessary, UE can determine the association based on its UE-ID implicitly.

  - Proposal 3: from UE perspective, it is not necessary to be aware of which option is used, UE may firstly determine a LOs based on its UE-ID, if a wake-up indication is received, UE can start monitoring PO from the first PO (also determined based on its UE-ID) after the MR is activated and resynchronized.

- [17] LG

  - Proposal #10: UEs with different wake-up delay should be assigned to different UE subgroups

  - Proposal #11: UEs having longer wake-up delay can be assigned to separated UE subgroups

    - The number of those subgroups can be separately configured.

    - wake-up delays above a certain threshold are considered as longer wake-up delay

  - Proposal #12: Following options can be considered for determination of PO to be monitored

    - Option 1: UE determines PO to monitor based on how the LO is allocated

      - After the reception of LP-WUS within a LO, UE monitors the PO which is used to determine where the LO is allocated.

    - Option 2: UE determines PO to monitor based on the wake-up time

      - After the reception of LP-WUS within a LO, UE monitors the nearest PO after the wake-up from the LO.

  - Proposal #13: For UE triggered by LP-WUS to monitor a PO, introducing a time window based on the reception of LP-WUS. The time window has the following characteristics:

    - UE receives the existing PO only within the time window

    - UE transits to sleep state at the end of the window

    - The length of window is determined based on

      - The number of upcoming paging occasions

      - The number of slot/frame configured by SIB

    - FFS: LO mapped to dynamic PO

- [18] Samsung

  - Proposal 7: The time location of LP-WUS occasion can be determined by a reference point and an offset.

  - Proposal 8: The time offset between the end position of the LP-WUS monitoring and the reference point should be larger than LP-WUS processing time, MR transition time for ramp up and time/frequency synchronization of MR (e.g., at least > 400ms).

- [19] ETRI

  - Proposal 3: Time location of the LO is defined by the time offset from the PO.

  - Proposal 4: The time offset is configured by gNB.

  - Proposal 5: UE monitoring relationship between LO and PO is decided based on the result of discussion on the maximum number of information bits in a LP-WUS and the maximum number of subgroups per PO.

- [26] QC

  - Proposal 1: The LO contains a three-tier structure of LP-WUS MOs for beam sweeping, time domain repetition and transmission of different LP-WUSs in different MOs associated with different UE subgroups.

  - Proposal 2: In the agreement “Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam”, support that K can be larger than or equal to 1 and K = X×L where X is the number of LP-WUS repetitions in same beam and L is the number of LP-WUS occasions for different sets of LP-WUSs.

  - Proposal 3: LP-WUS occasion is a set of N×X×L consecutive LP-WUS monitoring occasions where N is the number of beams, L is the number of LP-WUS occasions associated with different sets of LP-WUSs and X is the number of LP-WUS repetitions. The [x×N×L+l×N+m]th LP-WUS monitoring occasion in the LO corresponds to the mth beam for the lth set of LP-WUSs and the xth repetition, where x=0,1,…,X-1, l=0,1,…,L-1 and m=1,2,…,N. The LP-WUS monitoring occasions which do not overlap with UL symbols determined according to tdd-UL-DL-ConfigCommon are sequentially numbered from zero starting from the first LP-WUS monitoring occasion in the LO. When the UE detects a LP-WUS associated with its UE subgroup, the UE is not required to monitor subsequent LP-WUS monitoring occasions in this LO.

- [27] Ericsson

  - Proposal 5	Type of WUR capability supported in the cell, e.g., support for both OFDM and OOK-based WUR, or OFDM-based WUR only, or OOK-based WUR only, should be configurable by the NW.

  - Proposal 6	It should be possible for NW to configure flexibly the placement of LP-WUS and LP-SS resources in frequency and time to minimize overhead and NW energy efficiency impact.

  - Proposal 7	For LP-WUS monitoring in RRC-IDLE/INACTIVE,

    - a.	LO periodicity is the same as the DRX/paging cycle.

    - b.	Introduce time offset between LP-WUS occasion (LO) and UE paging occasion/frame.

    - c.	The network configures a single time offset value that is applicable to all UEs operating with LP-WUS. The time offset can be selected by the network from a candidate set. Minimum value of the time offset in the candidate set is FFS.

    - d.	LP-WUS monitoring occasion is determined from a starting symbol and duration of possible LP-WUS transmission within the LO.

- [28] Nordic

  - Proposal-4: In Option 1, for UEs belonging to the same paging group/subgroup and monitoring the same LO, the first monitored PO after LR receives LP-WUS can be different, is determined based on UE-reported wake-up time capability.

Regarding the 3 options on the mapping between LO and PO:

Option 1: [3] Spreadtrum, [4] CMCC, [5] TCL, [6] vivo, [9] CATT, [13] InterDigital, [15] NEC, [17] LG, [18] Samsung, [19] MediaTek, [22] Lenovo, [23] Apple, [25] Docomo, [26] QC, [27] Ericsson, [28] Nordic

  - [28] QC: LP-WUS MOs in the LO can be associated with different sets of UE subgroups of the PO.

Option 2: [1] Futurewei, [2] HW, [3] Spreadtrum, [4] CMCC, [5] TCL, [10] ZTE, [11] Nokia (maybe), [15] NEC, [27] Ericsson

Option 3: [3] Spreadtrum, [5] TCL, [9] CATT, [11] Nokia, [13] InterDigital (maybe) , [15] NEC, [17] LG, [18] Samsung, [21] Panasonic, [23] Apple, [25] Docomo (maybe)

1-to-1 mapping at the UE side: [16] KT

Value of K

- K >= 1: [2] HW, [3] Spreadtrum, [5] TCL, [6] vivo, [9] CATT, [10] ZTE, [13] InterDigital, [14] Fujitsu, [17] LG, [24] Sharp, [25] Docomo (maybe), [27] Ericsson

  - Information can be different: [14] Fujitsu, [25] Docomo (maybe)

  - Same information: [17] LG

  - Same or different: [27] Ericsson

- K = 1: [7] OPPO, [18] Samsung, [21] Panasonic, [28] Nordic

Periodicity of LO

- Same as iDRX cycle: [5] TCL, [6] vivo, [7] OPPO, [11] Nokia, [17] LG, [23] Apple, [24] Sharp, [26] QC, [27] Ericsson

- Same or smaller than iDRX cycle: [18] Samsung

Some companies proposed that different sets of MOs are monitored by different subgroups of UEs corresponding to the same PO (in order to increase the total number of subgroups per PO). Such an example is shown in the figure, where set 1 of MOs are monitored by UEs in subgroups 1 to N for the corresponding PO, while set 2 of MOs are monitored by UEs in subgroups N+1 to 2N for the corresponding PO. It was observed that companies used two different approaches to define an LO for such a case:

Approach 1: The LO consists of both set 1 and set 2 of MOs.

Approach 2: One LO consists of set 1 of MOs, and another LO consists of set 2 of MOs

As the specifications define the UE behaviors, it is easier to define an LO from UE perspective, i.e., the LO should include, and only include, all the LP-WUS MOs intended for a UE. Moderator’s recommendation is to go with Approach 2 above.

#### [CLOSED] Proposal 1-1:

Clarification for “LO”: an LO is defined as a set of LP-WUS MOs monitored by a UE.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | OK to clarify. |
| CMCC | Support |
| FW | We are OK with the proposal |
| Xiaomi | This definition seems too general. We understand the original meaning is to refer the multiple MOs across multiple beams by “LO”. We suggest the following: an LO is defined as a set of LP-WUS MOs across multiple beams, and among which one or more MOs are monitored by a UE. |
| InterDigital | Support |
| Nokia1 | Past definition was worded as: - “Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs.” It is not fully clear to me what the change from ‘one or more’ to ‘set of’ LP-WUS MOs changes from UE behaviour perspective? Is the target to enforce that one LO can contain sub-grouping in MO level or that MOs need to be configured as set(s)? |
| ETRI | OK with the proposal. |
| Qualcomm | It is good to clarify whether LO is defined from network perspective or UE perspective. |
| CATT | OK with the proposal. |
| vivo | Ok for the intention. |
| LG | Support. |
| Panasonic | We are okay with the intention as explained above by the Figure. But not sure how the proposal achieves the purpose. |
| DCM | We are OK with FL’s consideration, but we think the meaning of Approach 2: “One LO consists of set 1 of MOs, and another LO consists of set 2 of MOs” also should be added to the proposal. Thus, we support the proposal with also supporting the following sentence: ”More than one LO can be associated with one PO”. |
| OPPO | Ok for proposal. |
| Ericsson1 | Our preference is for LO to follow similar high-level principle as specified for PEI-O in Rel17. i.e., following from 38.304. “The UE monitors one PEI occasion per DRX cycle. A PEI occasion (PEI-O) is a set of PDCCH monitoring occasions (MOs) and can consist of multiple time slots (e.g. subframes or OFDM symbols) where PEI can be sent (TS 38.213 [4]).” … … “A PEI occasion is a set of 'S*X' consecutive PDCCH MOs, where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH MO for PEI in the PEI-O corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH MOs for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH MO for PEI in the PEI-O. When the UE detects a PEI within its PEI-O, the UE is not required to monitor the subsequent MO(s) associated with the same PEI-O. ” |
| MTK | OK |
| Spreadtrum | OK but in our understanding PO/PF is not just from UE perspective, because we always mentioned that POs/PFs in a paging cycle which is from NW perspective |
| Nordic | We support. |
| Moderator | There are different views among companies whether to go with Approach 1 or Approach 2. We will not try to converge on this issue in this meeting. |

#### [CLOSED] Proposal 1-2:

On the LO configuration for iDRX, down-select from the following options:

Option 1: LO is configured using offset value(s) relative to a PO/PF

  - For each UE, the periodicity of LO is the same as its iDRX cycle.

  - If a UE receives a positive wake-up indication in a LP-WUS, it monitors the corresponding PO associated with the offset.

Option 2: LOs are configured with a periodicity and an offset from the network perspective, which is separate from the PO configuration.

  - Periodicity of LO can be the same as or smaller than the default iDRX cycle.

  - If a UE receives a positive wake-up indication in a LP-WUS, it monitors the first PO after the wake-up delay.

  - Option 2A: gNB configure the number of LOs, UE determines the LO index based its PO index, e.g., LO index = [(UE_ID mod N) *Ns + i_s] mod L, L is the number of LOs within LO periodicity.

  - Option 2B: …

Please comment on the above options and add other options if needed. Sufficient details should be provided for a new option, especially for Option 2 in terms of how to achieve subgrouping.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | For option1, we hope it could be changed as ‘the periodicity of LO could be the same as its iDRX cycle’, and further discuss other periodicity value for LO monitoring. Additional, positive wake-up indication is confusing, and we can just use wake-up indication; |
| CMCC | We are fine with both Option 1 and Option 2. BTW, another option under Option 2 can be considered is that LOs can be configured with the time domain offset(s) relative to LP-SS. From our understanding, this has the benefit that it does not rely on any signal/channels of MR. Hence, when the MR is in a long period ultra-deep sleep state, LR may not be required to well synchronize to the MR. Therefore, we suggest to revised the proposal as following: #### Proposal 1-2-rev1: On the LO configuration for iDRX, down-select from the following options: Option 1: LO is configured using offset value(s) relative to a PO/PF - For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a positive wake-up indication in a LP-WUS, it monitors the corresponding PO associated with the offset. Option 2: LOs are configured with a periodicity and an offset from the network perspective, which is separate from the PO configuration. - Periodicity of LO can be the same as or smaller than the default iDRX cycle. - If a UE receives a positive wake-up indication in a LP-WUS, it monitors the first PO after the wake-up delay. - Option 2A: gNB configure the number of LOs, UE determines the LO index based its PO index, e.g., LO index = [(UE_ID mod N) *Ns + i_s] mod L, L is the number of LOs within LO periodicity. - Option 2B: LO is configured with the time domain offset(s) relative to LP-SS. |
| FW | As discussed in our contribution, we still see a possibility for the UE to monitor the first PO after the wake-up delay under Option 1 and not necessarily the PO associated with the offset which can handle the different wake-up delays. For Option 2A, it is still not clear how the LO configuration work. Option 2 proposes one periodicity and one offset, then we have multiple LOs (i.e., associated with different POs). that are within the defined LO periodicity. The UE determines the LO index using the specified formula, but how it defines the time location of that LO, are they equally spaced within the LO periodicity or have different offsets from the network perspective? |
| Xiaomi | Generally OK. For Option 2, we have the following modification suggestion Option 2: LOs are configured with a periodicity and an offset from the network perspective, which is separate from the PO configuration. - Periodicity of LO can be the same as or smaller than the default iDRX cycle. - If a UE receives a positive wake-up indication in a LP-WUS, it monitors the first PO after the wake-up delay. - Option 2A: gNB configure the number of LOs, UE determines the LO index based its PO index, e.g., LO index = [(UE_ID mod N) *Ns + i_s] mod L, L is the number of LOs within LO periodicity. - Option 2B: gNB configure the number of LOs, UE determines the LO index based its UE ID. - Options can be combined. |
| Nokia1 | OK with the proposal (supporting option 1) |
| Qualcomm | Support option 1. Option 1 is consistent with PEI design. |
| CATT | We support Option1. |
| vivo | OK and support option 1, since option 1 can easily achieve the purpose that LP-WUS is to trigger the PO monitoring and LP-WUS subgrouping is on top of PO group. |
| LG | We are fine to have both option 1 and 2 for now. For Option 2A, we also think it is better to remove the example. For remaining options, we think following possibilities. In the existing paging procedure, everything is based on UE-ID. So may be UE-ID would be included in any of options. Option 2A: UE determines the LO index based on its UE-ID and PO index Option 2B: UE determines the LO index based on its UE-ID and subgroup index Option 2C: UE determines the LO index based on its UE-ID only For determining LO index per each UE, gNB may configure a number of LO during a period or UE determine its “offset” differently based on UE-ID, PO index and/or subgroup index. |
| OPPO | Support option 1. It may save complexity. |
| MTK | Ok and we support option 1 for simplicity. |
| Spreadtrum | Option 1 is easy for standardization, but resource inbetween PF is stringent, since PEI/LP-WUS should use these resources. |
| Moderator | Here is the updated proposal. For Option 2, I would prefer to get some detailed description on how the LO is determined exactly, not just a high level description (e.g. based on UE ID, subgroup ID, …). This will help us understand better different ways to achieve Option 2. Would appreciate if proponents can provide more details. #### Proposal 1-2r1: On the LO configuration for iDRX, down-select from the following options: Option 1: LOs are is configured using offset value(s) relative to a PO/PF - For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a positive wake-up indication in a LP-WUS, - Alt 1: it monitors the corresponding PO associated with the offset. - Alt 2: it monitors the first PO after the wake-up delay. Option 2: LOs are configured with a periodicity and an offset from the network perspective, which is separate from the PO configuration. - Periodicity of LO can be the same as or smaller than the default iDRX cycle. - FFS the reference point for the offset - If a UE receives a positive wake-up indication in a LP-WUS, it monitors the first PO after the wake-up delay. - Option 2A: gNB configures the number of LOs, UE determines the LO index based its PO index, e.g., according to LO index = [(UE_ID mod N) *Ns + i_s] mod L, where L is the number of LOs within LO periodicity. - Option 2B: … |
| Moderator | The proposal is updated to the following after the offline discussion. #### Proposal 1-2r2: On the LO configuration for iDRX, down-select from consider the following options: Option 1: LOs are is configured using offset value(s) relative to a POs/PFs - For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a positive wake-up indication in a LP-WUS, - Alt 1: it monitors the corresponding PO associated with the offset. - Alt 2: it monitors the first PO after the wake-up delay. - FFS wake-up delay Option 2: LOs are configured with a periodicity and an offset from the network perspective, which is separate from the PO configuration. - Periodicity of LO can be is the same as or smaller than the default iDRX cycle. - FFS the reference point for the offset - If a UE receives a positive wake-up indication in a LP-WUS, it monitors the first PO after the wake-up delay. - Option 2A: gNB configures the number of LOs with a LO periodicity, UE determines the LO index based its PO index, e.g., according to LO index = [(UE_ID mod N) *Ns + i_s] mod L, where L is the number of LOs within a LO periodicity. - Option 2B: … Note: the PO a UE monitors here refers to a legacy PO configured for the UE. |
| Nordic | Our preference is Option 1 + Alt 2 because one LO once PO (from UE point of view) is fully sufficient. gNB configuration for this option is low overhead and simple. Alt2. allows gNB to configure offset suitable for gNB and UEs can have different capabilities. We understand that Paging message from AMF to gNB needs to contain UE wake-up delay capability. However, this a business as usual in RAN3. |
| OPPO | We are OK with moving forward from the 2 options. Our understanding of the iDRX configuration dependent is the LO can be simply 1 per period. It is unclear that the multiple LOs could be fulfilled by multiple MOs. Thus, may be it is easier to have option 2 without the sub-options 2a 2b... |
| Moderator | The descriptions of the options are updated based on the discussions and based on my understanding of Option 2: #### Proposal 1-2r3: On the LO configuration for iDRX, consider the following options: Option A: offset value(s) between a LO and a PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LOs corresponding to Option 1, 2, or 3 (if supported) - For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, - Alt 1: it monitors the PO associated with the offset. - Alt 1A: it monitors the PO associated with the offset if its reported wake-up delay is no larger than the time gap between the LO and the PO; otherwise it does not monitor LP-WUS and falls back to the legacy paging monitoring behavior. - Alt 2: it monitors the first PO after its reported wake-up delay. Option B v1 (the version after offline discussion): a periodicity and an offset are configured by the gNB for LO determination, which is separate from the PO configuration. - Periodicity of LO is the same as the default iDRX cycle. - FFS the reference point for the offset - If a UE receives a wake-up indication in a LP-WUS, it monitors the first PO after the wake-up delay. - Option 2A: gNB configures the number of LOs with a LO periodicity, UE determines the LO index according to LO index = [(UE_ID mod N) *Ns + i_s] mod L, where L is the number of LOs within a LO periodicity. - Option 2B: … Option B v2 (the description for the previous Option 2A): gNB configures the number of LOs (L) within the default iDRX cycle and an offset. - FFS the reference point for the offset - UE determines the LO index according to LO index = [(UE_ID mod N) *Ns + i_s] mod L. - Note: this can support Option 1/2, but not Option 3 - If a UE receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. |
| Moderator | The following was agreed (Option A in the previous version of the proposal): Agreement On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported) - (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives - Alt 1: it monitors the PO associated with the offset. - Alt 2: it monitors the first PO after its reported wake-up delay. - Other alternatives are not precluded Note: The PO mentioned above refers to legacy PO configured for the UE. |

Dynamic PO for iDRX

- Yes: [1] Futurewei, [10] ZTE, [16] KT, [17] LG

- No: [7] OPPO, [11] Nokia, [23] Apple, [25] Docomo, [26] QC, [27] Ericsson

There seems to be some strong concern on the support of dynamic PO from some companies. Moderator proposes not to support it as it is not an essential feature.

#### [CLOSED] Proposal 1-3:

Do not support dynamic PO.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Benefits is clear in RAN1 for dynamic PO. For RAN1, we do not think we should preclude this. If we can not agree on using dynamic PO, considering the impacts on other working groups, we can accept sending an LS to RAN2/RAN4, and the decision can be made by other groups. |
| FW | As we commented to proposal 1-2 above, dynamic PO can at least be used along with Option 1 to handle the different MR wake-up delay capabilities. |
| Xiaomi | OK |
| Nokia1 | Support the proposal. When UE can move between PO monitoring and LP-WUS monitoring, actual latency benefits cannot be ensured, and hence not accounted. |
| ETRI | Support the proposal. |
| Qualcomm | Paging delay is not an issue for idle/inactive mode. Dynamic PO increases false alarm rate and resource overhead of LP-WUS. |
| CATT | Support the proposal. |
| vivo | OK |
| LG | Most of proponent for dynamic PO are considering it as complementary solution with above option 1 or 2 from proposal 1-2. Thus, the UE procedure for dynamic PO is highly up to the decision on proposal 1-2. We think it is a bit premature to preclude dynamic PO at this point. |
| Panasonic | Support |
| DCM | OK. As stated in our tdoc, the motivation of dynamic PO can be achieved by changing length of I-DRX cycle with smaller NW overhead compared to dynamic PO. |
| OPPO | Support the proposal. |
| Ericsson1 | OK with the proposal. |
| MTK | Support. Paging delay is not the main target. |
| Samsung | Support . |
| Nordic | Agree |
| Moderator | The following was concluded: Conclusion There is no consensus in RAN1 on the support of dynamic PO. Discussion is closed. |

Here is a proposal from [6] vivo, which seems reasonable.

UE is not required to monitor a PO, if UE does not detect LP-WUS at all LP-WUS monitoring occasion(s) for the PO.

#### [CLOSED] Proposal 1-4:

A UE is not required to monitor a PO if it does not detect a positive wake-up indication in LP-WUS in an LO.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | If the UE does not receive LP-WUS for a long time, we hope the UE should monitor PO. |
| CMCC | Support |
| FW | Agree in general with the proposal, but as ZTE suggests, this should not contradict with any potential condition to quit LP-WUS monitoring. |
| Xiaomi | OK. And also suggest to make it a premise that UE has correctly detected the LP WUS. |
| InterDigital | Agree with ZTE that potential solution to monitor PO if the UE does not receive LP-WUS due to potential coverage loss. |
| Nokia1 | While it should be evident(?), I think it might be good to clarify that this behaviour relates to the case when UE is monitoring (and is allowed to monitor) LP-WUS. We think any fall-back timers can be discussed separately (if needed). |
| Vodafone | Similar comment as ZTE, FW |
| ETRI | Agree with ZTE. |
| Qualcomm | Agreed with the proposal. |
| vivo | Support. And for the fall back issue if any, it needs to be discussed separately. |
| LG | Agree in general. It seems a reinforcement of following sentence from previous WA. - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up The proposal is equivalent to remove “may” from the sentence. We think the previous WA should be addressed with this proposal. |
| Panasonic | Although falling back operation should not be part of this proposal, it should be clearly clarified. Current wording is too general. We also agree with ZTE and FW. More conditions should be taken into account |
| DCM | Our understanding is that the proposal intends to basic LP-WUS procedure, in that sense, we are OK. Fall back mechanism as other companies say can be discussed if needed. |
| OPPO | OK for the proposal |
| Ericsson1 | The LO to PO linkage should be clarified in the proposal. i.e., we prefer to follow similar principle as defined for PEI in 38.304. “If the UE detects PEI and the PEI indicates the subgroup the UE belongs to monitor its associated PO, as specified in clause 10.4a in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1. If the UE does not detect PEI on the monitored PEI occasion or the PEI does not indicate the subgroup the UE belongs to monitor its associated PO, as specified in clause 10.4a in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1. ” |
| MTK | Ok |
| Moderator | #### Proposal 1-4r1: A UE is not required to monitor a PO if it does not detect a LP-WUS on the monitored LO or the LP-WUS does not indicate the subgroup the UE belongs to monitor its associated PO a positive wake-up indication in LP-WUS in an LO. Note: this does not preclude the discussion of potential fallback behavior if the UE does not receive LP-WUS for a long time. |
| Spreadtrum | #### OK |
| Nordic | #### OK |
| OPPO | #### OK, the wording would be better as:“does not detect a LP-WUS on the monitored LO indicating the subgroup the UE belongs to monitor its associated PO” |
| Moderator | #### Proposal 1-4r2: When a UE is monitoring LP-WUS, a UE is not required to monitor a PO if it does not detect a LP-WUS on the monitored LO or the LP-WUS does not indicate the subgroup the UE belongs to monitor its associated PO a positive wake-up indication in LP-WUS in an LO. Note: this does not preclude the discussion of potential fallback behavior if the UE does not receive LP-WUS for a long time. |
| Moderator | #### After the offline discussion, companies would like to take some time to think about whether such an agreement is necessary. We will come back to this in future meetings if it is considered necessary. Discussion is closed. |

QCL and beam operation related

[2] HW/HiSi: Proposal 9:	QCL assumptions between SSB/LP-SS and LP-WUS should be configured or implicitly defined and it is supported that LP-WUS/LP-SS can be transmitted via a subset of all SSB beams.

[6] vivo: Proposal 13: For multi-beam operation of LP-SS, each LP-SS MO is associated with each transmitted beam in turn.

[8] Xiaomi: Proposal 5: LP WUS can be QCLed with LP -SS by QCL type A. LP WUS and LP -SS can be QCLed with SSB by QCL type C.

[11] Nokia

  - Proposal: Define association between LP-WUS MOs and LP-SS and SSB beams.

  - Proposal: Define that LP-SS and LP-WUS MO time locations are associated to a beam.

[16] KT

  - Proposal 5: For the LP-WUS monitoring in RRC IDLE/INACTIVE mode, a LP-WUS can be QCLed with existing NR signal/channel/CORESET for the TCI state.

  - Proposal 7: Discuss for the effectiveness of QCL relations between signals in MOs in LO and corresponding PO(s).

[25] Docomo

  - Proposal 5: The same beam as SSB should be used for LP-WUS and LP-SS in IDLE/INACTIVE mode

  - Regarding LP-WUS, if a beam for LP-WUS is transmitted in a MO, each MO should be associated with the beam index of SSB

  - Regarding LP-SS, it should be further discussed how to associate with beam index between LP-SS and SSB

#### [CLOSED] Proposal 1-5:

LP-WUS and LP-SS use the same beams as SSB. Define the association between LP-WUS MOs/LP-SS and SSB beams.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | OK |
| CMCC | Support |
| Xiaomi | OK |
| InterDigital | Support |
| Nokia1 | OK with the proposal. |
| Vodafone | OK |
| ETRI | Support. |
| Qualcomm | Support the proposal. |
| CATT | OK |
| vivo | OK, with a small update that “Define the association between LP-WUS MOs/LP-SS MOs and SSB beams.” Besides, we also want to clarify that LP-WUS/LP-SS may not use all SSB beams, it can be transmitted via a subset of all SSB beams. |
| LG | Support. Maybe some clarifications or details are needed for “define the association”. |
| Panasonic | OK. |
| DCM | OK |
| OPPO | Understand the intention. Just clarification, the LP-SS or LP-WUS would support different beam. Should the proposal mean one of SSB of NR SSBs? |
| Ericsson1 | OK with the proposal |
| MTK | Ok |
| Moderator | #### Proposal 1-5r1: LP-WUS and LP-SS use the same beams as SSB. Define the association between LP-WUS MOs/LP-SS transmissions and SSB beams. |
| Moderator | The following was agreed. Agreement At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams. Discussion is closed. |

UE capability report on wake-up delay and network’s assumption on the wake-up delay

- [2] HW/HiSi

  - Support MR to stay in both deep sleep state and ultra-deep sleep state when LP-WUR is monitoring LP-WUS.

  - Support UEs to report its transition time considering at least different LP-WUR types and sleep states.

  - gNB can configure a single minimum time gap between LO and PO, which can be based on the capability report from UEs.

    - For a UE with LP-WUR, if the guaranteed minimum time gap in the cell is smaller than the reported transition time, the UE will not monitor LP-WUS.

- [6] vivo

  - Proposal 11: Support to report UE capability on wake-up delay which is the time offset between receiving LP-WUS targeted to itself and being ready for PO reception.

  - Proposal 12: Support the network to configure one or multiple offsets between LO and PO.

- [8] Xiaomi

  - Proposal 15: Suggest to specify wake-up delay for activating MR from sleep state.

- [11] Nokia

  - Proposal: RAN1 discusses and agrees for the configuration one common assumption for the minimum offset between LP-WUS and PO.

  - Proposal: RAN1 considers value in range of 400ms for the minimum offset between LP-WUS and the associated PO.

  - Proposal: No maximum time offset between LP-WUS and LP-SS is defined.

  - Proposal: LP-WUS to PO mapping is commonly defined so that all UEs of same sub-group have same LP-WUS to PO mapping. UEs should respect the configured paging period and monitor PO so that service latency requirement can be met.

- [16] KT

  - Proposal 3: Support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PO monitoring in RRC IDLE/INACTIVE mode, and further discuss that the capability is sharable with the minimum time gap to start PDCCH monitoring in RRC CONNECTED mode.

- [18] Samsung

  - Proposal 9: UE report one value per SCS for the minimum time offset, NW can configure one offset value based on the reported UE capability.

- [23] Apple

  - Proposal 1: MR waking up from deep sleep state, in addition to ultra-deep sleep state, is supported, via supporting smaller value(s) for the offset between LP-WUS monitoring and PO.

  - Proposal 2: UE reports one value for the supported minimum wake-up delay.

    - FFS the value set

    - FFS whether a UE can report multiple values (corresponding to different sleep states)

- [25] Docomo

  - Proposal 1:

    - At least 400ms/800ms wake-up delay should be supported by LP-WUS capable UE in IDLE/INACTIVE modes

    - FFS: 400ms and/or 800ms is(are) supported

  - Proposal 2:

    - 30ms/10ms and more than 400ms/800ms wake-up delay should be optionally supported by LP-WUS capable UE in IDLE/INACTIVE modes

#### [CLOSED] Proposal 1-6:

For the wake-up delay, consider the following options:

Option 1: No UE capability report for the wake-up delay. One value is pre-defined in the specifications for all the UEs and used by gNB.

Option 2: UE reports one value for the wake-up delay.

  - Option 2A: gNB assumes a single value for all the UEs.

    - E.g. gNB configures a single offset value based on the maximum wake-up delay reported by the UEs.

  - Option 2B: gNB can configure LOs differently for different UEs based on the reported wake-up delay.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | For option 2A, more clarification may be needed. For example, whether the one value is based on the WUR type or sleep states. Additionally, for the gNB, it mainly considers the minimum wake-up delay, instead of the maximum wake-up delay. For option 2B, it may need more clarification regarding how to configure different offset for different UEs. |
| CMCC | Support Option 2B, for which the gNB can configure the reasonable value for each UE to balance the transmission overhead of LP-WUS (e.g. configure different time gap values for different UEs, so that their corresponding LP-WUS monitor occasion is the same) and the time validity of LP-WUS (PO monitoring). |
| FW | As we discussed in our previous comments, an additional option 2C may be considered where the gNB configures dynamic PO for UEs with short delay and legacy PO is considered for UEs with long delay. |
| Xiaomi | The Option 2 seems not very clear. And we suggest the following modification Option 2: Multiple UE capabilities are defined and UE reports one value for the wake-up delay. - Option 2A: gNB assumes a single value for all the UEs. - E.g. gNB configures a single offset value between LO and PO based on the maximum wake-up delay reported by the UEs. - Option 2B: gNB can configure LOs differently for different UEs based on the reported wake-up delay. - Option 2C: gNB can configure LOs separately from PO, and UE will monitor the nearest PO after wake up according to its capability. |
| Nokia1 | OK with the proposal (preference for option 1, option 2A could be considered). It could be clarified for these options that if the UE wake-up delay does not meet the configured offset value(s), UE does not monitor LP-WUS (or can do so but with lower power saving benefit?). |
| Qualcomm | We prefer option 2. |
| CATT | Network would not be able to configure a valid range of offset to satisfy all the transition time of ultra-deep/deep/light/micro sleep values from different UE implementations. UE determines the possible sleeping type based on the network configured offset and the sleeping mode transition time in the implementation. Thus Option1 should be supported. Regarding Option2, can proponents show that how/when to report the UE capability? |
| vivo | OK and support option 2 and further prefer option 2B for addressing the case that the range of reported wake up delay is wide so that it can be latency efficient. |
| Sharp | Option 2A may not work well in some case. For example, gNB configure a 100ms delay for some UE whose delay is reported as 100ms, then a UE with 200ms capability camp into the cell, then gNB need to refonfig the delay to 200ms? Option 2B may have complexity impact ,so we perfer option 1. |
| LG | Support. We think Option 2A should be baseline. |
| Panasonic | Basically okay. But for Option 2A/2B, depending on the design details, it could be gNB implementation on how to make use of the reported wake-up delay from different UEs. So far, only main bullet of Option 1 and 2 are needed. We support Option 2. |
| DCM | To avoid frequent reconfiguration of resource parameters by NW, time offset should be configured based on the pre-defined value. Additionally, it should also be supported that additional time offset can be configured based on the wake-up delay for achieving variety wake-up delay. Thus, we propose following sentence as option 3: Option 3: One value is pre-defined in the specifications for all the UEs and NW at least configures single time offset between LO and PO based on the pre-defined value. - NW can additionally configure time offset between LO and PO which is based on the UE capability report on the wake-up delay |
| OPPO | We current consider Option 1 would be sufficient, and would fine for the proposal. |
| MTK | For option 2, we have the same question as Sharp. How does a gNB update the scheduling offset when more UEs access the cell? |
| Spreadtrum | OK. gNB needs to know deep-sleep or ultra-deep-sleep is used by UE’s MR |
| Apple | For Option 2B, at least one way to make it work is that gNB configures multiple offsets, and each UE would choose one offset according to its own capability. If it cannot support any of the offsets, it does not monitor LP-WUS. |
| Moderator | Some update below. On DCM’s comment, I am not sure if we really need an Option 3. The same purpose can be achieved by Option 2 already in my view. But we can discuss more. #### Proposal 1-6r1: For the wake-up delay, consider the following options: Option 1: No UE capability report for the wake-up delay. One value is pre-defined in the specifications for all the UEs and used by gNB. Option 2: UE reports one value from a set of candidate values for the wake-up delay via UE capability reporting. - Option 2A: gNB assumes a single value for all the UEs. - E.g. gNB configures a single offset value based on the maximum wake-up delay reported by the UEs. - Option 2B: gNB can configure LOs differently for different UEs based on the reported wake-up delay. - Option 2C: gNB configures LOs separately from PO, and UE monitors the next PO after the wake-up delay according to its capability. |
| Samsung | UE can report one or more value(s) for the wake-up delay, but gNB can configure one value based on those wake-up delay UE reported in our understanding. |
| Nordic | It can, but this will result in gNB configuring/defining separate LOs for UEs with different capability. If this is what networks want, we can accept. |
| Moderator | The following was agreed: Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring Discussion is closed. |

Power offset between SSB/LP-SS and LP-WUS

[2] HW: Proposal 10:	The power offset between SSB and LP-WUS/LP-SS should be specified and can be configured by gNB in system information.

[8] Xiaomi: Proposal 6: The power offset of LP-SS/LP-WUS compared to the power of SSB should be configured.

This issue is deprioritized for now.

## LP-WUS payload size and content

This is a summary of companies’ views on the number of subgroups, the number of information bits and the related subgrouping function:

[1] Futurewei

  - should support at least a subset of Rel-17 PEI subgrouping configuration, i.e., up to 8 subgroups and up to 8 POs per PEI. Larger number of subgroups can be considered to ensure power saving gain.

  - Support at least the alternative to carry up to 16 bits of LP-WUS information using encoded bits with an 8-bit CRC when bitmap based wake-up indication is considered and without CRC when codepoint based wake-up indication is considered.

[2] HW/HiSi: The block of information bits carried in LP-WUS monitoring occasion contains group ID information whose bit length is configurable to support various network deployments, UE densities and traffic statistics.

  - Configuring up to 16-bit information is supported.

[5] TCL: codepoint based subgroup ID

  - Proposal 4: Codepoint based subgroup ID can be included in the LP-WUS, where each codepoint target a separate UEs subgroups.

  - Proposal 5: Consider the maximum number of information bits (excluding CRC) in an LP-WUS as 8 bits.

  - Proposal 6: Consider the total payload size of the LP-WUS (excluding CRC) as 32 bits, which may include the information bits and the codepoints-based subgroup IDs.

[7] OPPO

  - A unified LP-WUS design can be used for IDLE/INACTIVE mode, as well as CONNECTED mode.

  - For wake-up information, A UE group is assigned with single bit in the LP-WUS for indication of wake-up in IDLE/INACTIVE modes.

[8] Xiaomi

  - Proposal 13: At least wake up indication at the granularity of UE subgroup should be carried in LP WUS.

  - Proposal 14: UE can be provided with assistant information about whether PEI provide more subgrouping information besides LP WUS subgrouping indication.

[15] NEC

  - Proposal 4: for the LP-WUS content, support bitmap based indication for multiple UE subgroups.

  - Proposal 5: further study toggle based wake-up indication and flag based wake-up indication, and take the miss-detection issue into account.

[18] Samsung: bitmap-based indication

[25] Docomo

  - Proposal 10: If UEs subgrouping can not be divided based on paging rate UE capabilities, bitmap indication should be supported for indicating subgroup information using LP-WUS

  - Proposal 11: If codepoint-based is supported for indicating subgroup information using LP-WUS, the codepoint value enabling to wake-up all subgroups should be supported

    - E.g., Wake-up indication for all subgroups via single/multiple codepoint value(s)

  - Proposal 12:  For idle/inactive mode, RAN1 does not discuss further carrying additional information other than subgroup-based wake-up indication and ETWS/CMAS notification

    - If entry/exit procedure for LP-WUS monitoring can ensure that paging misdetection performance is not impacted by LP-WUS detection, ETWS/CMAS notification should not be conveyed by LP-WUS payload

  - Note: LP-SS may or may not carry the cell ID information, and LP-SS or other sequence generation may or may not be associated with PCIs

Maximum number of subgroups per PO

- 8: [3] Spreadtrum, [7] OPPO (as the starting point), [9] CATT, [11] Nokia, [26] QC, [27] Ericsson, [28] Nordic, [5] TCL: 8? (8 bits, max 32 bits?)

- 8~16: [6] vivo, [20] MediaTek

- >8: [17] LG, [23] Apple

- [13] InterDigital: 16 or 32 (if implicit indication is supported) (16 bits)

- 256: [2] Huawei (group ID, 8 bits), [10] ZTE, [21] Panasonic (16 bits)

- [12] Sony: 16 bits

Separate/independent subgrouping for LP-WUS and PEI: [4] CMCC, [8] Xiaomi (?), [17] LG, [23] Apple, [25] Docomo, [26] QC

For the mechanism to carry the subgroup-based wake-up indication,

- Bitmap: [3] Futurewei, [16] NEC (flag-based or toggle-based), [9] xiaomi

- Subgroup ID: [2] HW/HiSi, [3] Futurewei, [5] ZTE, [20] MediaTek, [21] Sony, [26] QC

As there are a diverse set of proposals from companies, the intention of Proposal 2-1 is to narrow down to a few options for further down-selection. Proposal 2-1 serves as the starting point of discussion. If an option is not preferred by any company, we can remove it. Additional options can be added as needed. The goal is to keep as few options as possible but still cover the different interest from companies.

#### [CLOSED] Proposal 2-1:

For the maximum number of subgroups per PO (X) and the maximum number of information bits per LP-WUS (Z), down-select from the following options:

Option 1: X = 8, Z = 8

Option 2: X = 8, Z = 3

Option 3: X = 16, Z = 16

Option 4: X = 16, Z = 4

Option 5: X = 32, Z = ?

Option 6: X = 256, Z = 16

…

Please provide your comments on the options (to add/delete/modify) and your preferred option(s).

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | X=256, Z=24. Larger payload size help NW improve the spectrum efficiency, and provide the maximum flexibility for the NW to configure the payload size and number of subgroups for different scenarios. |
| FW | We support either option 5 with Z = 16 or Option 6. |
| Xiaomi | If bitmap is adopted, we support option 1/3; if codepoint is adopted, we think X=256,Z=8 or Z=16 is reasonable |
| InterDigital | Support option 4 or option 5. |
| Nokia1 | In our companion paper Ag. 9.6.1 observed that 8 sub-groups would suffice in most scenarios. I.e. X=8. The payload size range 8 -16 was considered to be sufficient to ensure other metrics like false alarms. Should the number of bits be actually agreed under ag. 9.6.1? |
| Qualcomm | We prefer option 2 and can consider to further study option 4. |
| CATT | Either Option 1 or Option 2. |
| vivo | Support option 3. |
| Sharp | Prefer option 3 |
| LG | We support option 5. |
| OPPO | We prefer Option1. The list is OK. |
| MTK | Option 4 |
| Moderator | Given that we agreed to use codepoint for LP-WUS, I modified the proposal as follows: #### Proposal 2-1r1: For the maximum number of subgroups per PO (X) and the maximum number of information bits per LP-WUS (Z), down-select from the following options: Option 1: X = 8, Z = 3 Option 2: X = 16, Z = 4 Option 3: X = 32, Z = 5 Option 4: X = 256, Z = 8 |
| Samsung | Option 4 will use more MOs to indicate multiple UE subgroups when more than one UE subgroups are waked up by LP-WUS, the resource overhead is much higher. So we do not support option 4. Either option 1 and 2 can be accepted by us. |
| Nordic | Option 1 and Option 2 |
| OPPO | Option 1. We are also OK to consider Z value first as in discussed in the morning session of Wednesday. |
| Moderator | #### Proposal 2-1r2: For the maximum number of subgroups per PO (X), down-select from the following options: Option 1: X = 8 Option 2: X = 16 Option 3: X = 32 Option 4: X = 64 Option 5: X = 128 Option 6: X = 256 |
| Moderator | #### The following was agreed: Proposal 2-1r2: For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis: Option 1: X = 8 Option 2: X = 16 Option 3: X = 32 Option 4: X = 64 Option 5: X = 128 Option 6: X = 256 For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option. |

#### Proposal 2-2:

Support separate subgrouping configurations for LP-WUS and PEI.

- Details are left to RAN2 to decide.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Yes |
| CMCC | Support |
| FW | We are OK with proposal |
| Xiaomi | OK |
| InterDigital | Fine |
| Nokia1 | Do not support. I think the overall decision whether or not to support separate sub-group configuration for LP-WUS and PEI should be left for RAN2. From power saving perspective, due to large MR transition energy, the benefits are marginal, thus does not seem necessary to require RAN2 to support this complexity from this perspective. |
| Qualcomm | Support the proposal. |
| CATT | Depends on RAN2 discussion. |
| vivo | It should be discussed in RAN2. |
| LG | Support |
| Panasonic | Support |
| DCM | OK |
| OPPO | We think this is natural results, should we send LS to RAN2? |
| MTK | Ok |
| Spreadtrum | RAN2 has no knowledge about information bits size per LP-WUS |
| Samsung | If option 1 in proposal 2-1 is supported, we think the subgrouping configuration for PEI can be reused for LP-WUS. If more subgroup is supported in a PO, separate subgrouping configurations for LP-WUS and PEI should be assumed. |
| Nordic | We would like to hear why separate subgrouping is needed. |

## RRM measurement metrics

[2] HW/HiSi

The RRM metrics for OOK-based receiver, including LP-RSRP and LP-RSRQ can be utilized for OFDM-based receiver.

- [6] vivo

  - Proposal 6: For LP-RSSI definition for LP-RSRQ, LP-RSSI is the liner average of total received power in the same number of ON and OFF OOK symbols.

  - Proposal 7: Define the frequency domain granularity of LP-RSRP and LP-RSSI for LP-RSRQ.

  - Proposal 8: The network configures the measurement time resource for LP-SSS-RSSI, which can be different from NR SMTC.

- [8] xiaomi

  - Proposal 17: For OFDM-based LR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LR.

- [10] ZTE

  - Proposal 3: Further clarify definition for OOK ON symbols or OOK OFF symbols.

  - Proposal 4: For OFDM-WUR without FFT block, clarify the SS reference signal received power is based on 11RBs or 127 subcarriers to calculate the RSRP.

- [11] Nokia

  - Proposal: LP-RSRP and LP-RSRQ measurements should be associated to a particular beam.

  - Proposal: For LP-RSRP and LP-RSRQ measurements should be possible to network to configure sub-set of LP-SS’s from which LP-RSRP and LP-RSRQ can be measured.

  - Proposal: For LR that is capable of receiving PSS/SSS, RAN1 confirms the working assumption of re-using the measurement quantity definition but separates the definitions of measurement quantities for LR measurements based on SSB (from the MR measurement quantities).

- [13] InterDigital

  - Proposal 14: SS-RSRP and SS-RSRQ are reused for OFDM based LP-WUR.

  - LP-SSS-RSRP and LP-SSS-RSRQ are not defined in the specifications.

  - Proposal 15: LP-SS based RRM measurement is supported for OFDM based LP-WUR.

  - Proposal 16: Introduce definitions of LP-RSRP and LP-RSRQ for OFDM based LP-WUR to support LP-SS based RRM measurement.

- [25] Docomo

  - Proposal 17: For RRM metric based on OFDM-based LP-WUR,

    - No need to introduce LP-SSS-RSRP and LP-SSS-RSRQ in the spec

    - Enhancement to legacy SS-RSRP/SS-RSRQ of following can be considered

      - The measurement time resource(s) for SS-RSRP/RSSI are confined within a duration, e.g., within the window that the OFDM-based LP-WUR receiver is activated

      - The additional use of PBCH DMRS and CSI-RS for SS-RSRP determination is not applicable for OFDM-based LP-WUR

      - If SS-RSRP or SS-RSRQ is measured by OFDM-based LP-WUR, following is applicable,

        - RRC_IDLE intra-frequency

        - RRC_INACTIVE intra-frequency

For the frequency domain resources for the definition of LP-RSRP/LP-SSS-RSRP and LP-RSSI/LP-SSS-RSRQ, the following clarifications were proposed:

- [6] vivo: Proposal 7: Define the frequency domain granularity of LP-RSRP and LP-RSSI for LP-RSRQ.

- [10] ZTE: Proposal 4: For OFDM-WUR without FFT block, clarify the SS reference signal received power is based on 11RBs or 127 subcarriers to calculate the RSRP.

P4-1 below proposes to use the LP-SS bandwidth as the frequency domain resources for LP-RSRP and LP-RSSI.

On the proposal from [10] ZTE, it is not clear if such clarification is needed, as reusing the definition means it should consider “the resource elements that carry secondary synchronization signals”, not the LP-SS resources.

#### [CLOSED] Proposal 3-1:

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources that carry LP-SS.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | OK to update. For OFDM WUR, clarification also should be similar. |
| Xiaomi | OK |
| Nokia1 | OK with the proposal. |
| Qualcomm | Proposal is fine. |
| CATT | OK |
| vivo | OK |
| LG | Support. |
| Panasonic | OK |
| OPPO | OK |
| MTK | Ok |
| Moderator | The following was agreed: Agreement The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows: LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS. LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS. Discussion is closed. |

The following clarification was proposed:

[6] vivo: Proposal 6: For LP-RSSI definition for LP-RSRQ, LP-RSSI is the liner average of total received power in the same number of ON and OFF OOK symbols.

The proponent thinks the current definition is unclear in terms of how many ON/OFF OOK symbols the UE is required to measure.

#### [CLOSED] Question 3-1:

Please share your views on the need for clarification according to the above Proposal 6 from [6], or any other clarifications. If not, is your understand that the UE should measure all the ON/OFF OOK symbols?

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | We are open to discuss, but no need to specify the implementation for how to measure based on OOK/OFF symbols. |
| FW | We don’t understand the need to specify the same number of ON and OFF symbols, if we have more OFF symbols than ON symbols, then we should get a better estimate for noise plus interference than limiting the number of OFF symbols to be equal to ON symbols. |
| Nokia1 | Now this evidently relates to LP-SS sequence design. Ideally the number of ON and OFF symbols in LP-SS would be equal. Thus we should first attempt to select sequence(s) so that they have equal number of ON and OFFs. If this is not the case, this could results bias (if UE measures all symbols) or reduce the accuracy (max number of measured symbols would be less). |
| Qualcomm | Our understanding is that the UE should measure all the ON/OFF OOK symbols. |
| CATT | The measurement results of LP-RSSI should not depends on what the LP-SS exact is. Different LP-SS will bring various ratios of measured OOK ON/OFF symbols. Thus, we think it is necessary to balance the number of ON and OFF symbols for the LP-RSSI measurement. |
| vivo | Our intention is to clarify the measured ON and OFF symbols for the definition of LP-RSSI agreed in the last meeting. It is still unclear which ON/OFF OOK symbols of the LP-SS can be measured. If it can be up to UE implementation to choose some of OOK ON/OFF symbols to be measured, LP-RSSI would vary with different ratio of measured OOK ON/OFF symbols, which is unknown to the network. Consequently, it would be difficult for network to properly set the threshold for LP-WUS entry/exist. To keep the consistent understanding on the definition of LP-RSSI, it is necessary to specify the ratio of ON and OFF OOK symbols to be measured. Same number of ON and OFF OOK symbols is an example for the specified ratio. It is noted that, it does not necessarily restrict the number of ON and OFF symbols in LP-SS. |
| Panasonic | Our understanding is the measurement is performed on all the OOK symbols/chips. On whether/how to further balance ON/OFF symbols or limit the measurement, it is next level details on the LP-SS sequence design or explicitly putting limitation. We are open to discuss but better to agree on the LP-SS further. |
| MTK | Measure all ON/OFF OOK symbols over the time and frequency resources that carry LP-SS, if balanced 0/1 has been considered. |
| Moderator | In the RAN1#117 agreement, there is a note “Note: Above does not constrain LP-SS sequence design for OOK” based on Ericsson’s comment, as they did not want this definition to restrict the LP-SS sequence design. So at this point, we cannot guarantee that there are same number of 0s and 1s in the sequence. I see two possible ways to address vivo’s concern: - Alt 1: clarify LP-RSSI is measured across all the OOK ON/OFF symbols. In this way, gNB has clear understanding on the number of ON and OFF symbols, and it can interpret the reported value properly. - Alt 2: define LP-RSSI such that UE needs to measure across the same number of OOK ON and OFF symbols. This would leave some flexibility to the UE implementation in terms of how many symbols to measure. I wonder if we can take Alt 1 as it is simpler, also considering that the initial intention of agreeing on this definition is that companies want to have more symbols to measure the noise. |
| Nordic | I suppose UE can do some scaling. However, report should assume that reported value is measured on balanced sequence. |
| OPPO | I have impression all symbols/REs is not strictly required in the existing RSSI. |

Based on the discussions above, the following is proposed:

#### [CLOSED] Proposal 3-2:

The definitions of LP-RSSI for LP-RSRQ is further updated as follows:

LP-RSSI is the linear average of total received power in all the ON and OFF LP-SS OOK symbols over the frequency resources that carry LP-SS.

| Company | Comments |
| --- | --- |
| OPPO | This update may not be needed. |
| Moderator | The proposal is updated to reflect the most recent agreement on the LP-RSSI definition. #### Proposal 3-2r1: The definitions of LP-RSSI for LP-RSRQ is further updated as follows: LP-RSSI is the linear average of total received power in all the ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS. |
| Moderator | After further offline discussion, companies prefer to see if there is a need to revisit the issue after LP-SS sequence design is done, depending on e.g. whether the sequences have balanced 0s and 1s, or whether they have the same ratio of 0s and 1s, etc. So we will not discuss this issue until then. |

There are different views on whether LP-SSS-RSRP/LP-SSS-RSRQ should be separately defined, and/or whether additional changes are needed on top of SS-RSRP/SS-RSRQ definitions.

- [6] vivo: Proposal 8: The network configures the measurement time resource for LP-SSS-RSSI, which can be different from NR SMTC.

- [11] Nokia: Proposal: For LR that is capable of receiving PSS/SSS, RAN1 confirms the working assumption of re-using the measurement quantity definition but separates the definitions of measurement quantities for LR measurements based on SSB (from the MR measurement quantities).

- [13] InterDigital: LP-SSS-RSRP and LP-SSS-RSRQ are not defined in the specifications.

- [25] Docomo: Proposal 17: For RRM metric based on OFDM-based LP-WUR,

  - No need to introduce LP-SSS-RSRP and LP-SSS-RSRQ in the spec

  - Enhancement to legacy SS-RSRP/SS-RSRQ of following can be considered

    - The measurement time resource(s) for SS-RSRP/RSSI are confined within a duration, e.g., within the window that the OFDM-based LP-WUR receiver is activated

    - The additional use of PBCH DMRS and CSI-RS for SS-RSRP determination is not applicable for OFDM-based LP-WUR

    - If SS-RSRP or SS-RSRQ is measured by OFDM-based LP-WUR, following is applicable,

      - RRC_IDLE intra-frequency

      - RRC_INACTIVE intra-frequency

#### Question 3-2:

Please share your views on whether LP-SSS-RSRP/LP-SSS-RSRQ should be separately defined, and whether additional changes are needed on top of SS-RSRP/SS-RSRQ definitions, considering the proposals above.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Why LP-SSS-RSRP/LP-SSS-RSRQ should be separately defined may need clarification. |
| Xiaomi | We confirm the working assumptions in last meeting and see no needs for separate LP-SSS-RSRP/LP-SSS-RSRQ definition. |
| InterDigital | So far, we failed to observe reasons to define LP-SSS-RSRP/RSRQ especially when the same definitions are reused. |
| Nokia1 | We think it could be simpler to separate the definitions. It would keep configuration and UE reporting clear. Also, like noted DoCoMO we would need also separate the applicability. |
| Qualcomm | There is no need to define LP-SSS-RSRP/RSRQ separately. |
| CATT | No need separate definition. |
| vivo | From our view, the network configures the measurement time resource for LP-SSS-RSSI, which can be different from NR SMTC. And separate definition of LP-SSS-RSRP/LP-SSS-RSRQ seems not needed. |
| LG | We think there is no issue to re-use same definitions. |
| Samsung | The legacy definition can be reused in our understanding. |
| Nordic | Reuse |

## LP-WUS entry/exit conditions and RRM measurement offloading/relaxation

Here are the related proposals from RAN1#118 contributions:

[2] HW/HiSi: The same threshold(s) is used for entry/exit conditions for LP-WUS monitoring and RRM relaxation (including RRM offloading)

[4] CMCC

  - Proposal 1. In IDLE/INACTIVE mode,

    - When UE starts LP-WUS monitoring, the MR RRM measurement should also be relaxed or offloaded.

    - When UE monitors the legacy PO (and may monitor PEI) and stops LP-WUS monitoring, UE follows the legacy MR RRM measurement behavior.

  - Proposal 2. The following entry threshold(s) can be considered as the criteria of UE starting monitoring LP-WUS:

    - A MR RSRP threshold;

    - A MR RSRP variation threshold within a period of time;

    - A MR RSRP threshold and a MR RSRP variation threshold within a period of time;

    - A MR RSRP/RSRQ threshold for inter-frequency measurement for the frequency where LP-WUR operates.

  - Proposal 3. The following exit threshold(s) can be considered as the criteria of UE starting monitoring the legacy PO/PEI and stopping monitoring LP-WUS:

    - A LR RSRP threshold;

    - A LR RSRP variation threshold within a period of time;

    - A LR RSRP threshold and a LR RSRP variation threshold within a period of time;

    - A LR RSRP threshold and a MR RSRP threshold.

[5] TCL: For the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, a predefined time window (or X number of paging cycles) can also be considered as an entry condition into the LP-WUS monitoring.

[6] vivo

  - Proposal 4: Support the following UE behavior on RRM measurement when UE starts LP-WUS monitoring,

    - UE performs LR based serving cell measurement, and performs relaxed or does not perform MR based serving cell measurement;

    - UE performs MR based relaxed neighbour cell measurement or UE stops neighbour cell measurement.

    - FFS the conditions for different RRM measurement cases.

  - Proposal 5: Thresholds of for serving/neighbor cell measurement relaxation should be lower than or equal to the threshold of entry condition for LP-WUS monitoring.

[7] OPPO

  - When the UE starts LP-WUS monitoring and stops the legacy paging monitoring, set of combinations of LR/MR measurement periodicities should be introduced.

  - Study the mechanism to switch MR back to sleep when it is wakened up by false alarm.

[8] Xiaomi

  - Proposal 7: Support UE activation and/or deactivation of LR based on preconfigured criteria

  - Proposal 8: Conditions to activate LR for UE with OOK-based LR and UE with OFDM-based LR can be different.

  - Proposal 9: Conditions to deactivate LR for UE with OOK-based LR and UE with OFDM-based LR can be different.

  - Proposal 10: LR is deactivated and MR is activated when (1) UL transmission happens; (2) LP WUS indicating the UE to wake up is received; (3)The reception performance of LP-SS/LP-WUS falls below LR sensitivity; (4)The reception performance of LP-SS/LP-WUS falls below a threshold, which triggers the MR waking up for neighbouring cell RRM measurement.

  - Proposal 11: LR sensitivity requirement for OOK-based LR and OFDM-based LR should be defined by RAN4.

  - Proposal 18: Totally offloading and partially offloading of serving cell measurement from MR to LR can both be considered.

  - Proposal 19: Criteria for whether to apply totally offloading or partially offloading can be based on (1) preconfigured criteria, (2) gNB configuration.

  - Proposal 20: Support neighbouring cell measurement by OFDM-based LR.

  - Proposal 21: Support relaxing of neighbouring cell measurement by MR.

[9] CATT

  - Proposal 9: The LP-WUS detection can up to UE implementation or determined by a predefined threshold.

  - Proposal 10: The UE falls back to legacy mechanism when the LP-WUS is in detection errors.

[10] ZTE

  - Proposal 1: For the activation condition of LP-WUS monitoring, that serving cell measurement performed by the LR is not below exit threshold(s) should be included.

  - Proposal 2: UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

    - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, or

    - The UE has not received LP-WUS for a long time

  - Proposal 5: The entry/exit condition for LP-WUS monitoring and serving cell measurement relaxation should be satisfied at the same time.

    - If UE performs relaxed measurement based on WUR, the UE also should monitor LP-WUS, vice versa.

[11] Nokia: Proposal: RAN1 concludes that the threshold configuration for entry/exit conditions and RRM relaxation/offloading can be independently configuration by network. Design is up to RAN2.

[12] Sony: Proposal 6 – In order to avoid ping-pong behaviours, a time widow can be used. The UE switches on its MR to evaluate the channel condition only if no LP-WUS received/detected by the UE within this time-window or after such evaluation period.

[13] InterDigital

  - Proposal 12: For the entry condition, the other conditions other than serving cell measurement are not supported.

  - Proposal 13: For the exit condition, the other conditions such as timer are supported to ensure proper UE behavior.

[15] NEC: Proposal 6: study the following two options of RRM measurement: (1) MR measurement is triggered by the measurement result of LR; (2) UE performs both MR measurement (with relaxed periodicity) and LR measurement.

[17] LG

  - Proposal #15: The UE may start LP-WUS monitoring if LP-SS-based measurement for serving cell performed by the main radio is above entry threshold(s), if configured.

  - Proposal #16: Whether the UE can receive overlaid OFDM sequence over LP-WUS or not should be considered to specify entry/exit condition of LP-WUS monitoring.

  - Proposal #17: Consider the following aspects about RRM measurement relaxation of the main radio by offloading to LP-WUR.

    - The UE can measure cell quality by LP-WUR while the main radio is in ultra-deep sleep state.

    - The existing RRM measurement procedure by the main radio can be triggered based on the serving cell measurement by LP-WUR.

    - If the UE stops LP-WUS monitoring due to the serving cell measurement which is below exit threshold(s) by LP-WUR, the UE starts to measure either a serving cell or a neighbor cell by the main radio.

  - Proposal #18: If RRM measurement relaxation by using LP-WUR is supported, the UE with OFDM-based LP-WUR can perform RRM measurement by receiving LP-SS and/or PSS/SSS.

[18] Samsung

  - Proposal 10: UE may start LP-WUS monitoring when the entry conditions for LP-WUS monitoring is satisfied:

    - RSRP and the RSRQ of the SSB for serving cell measurement is better than a configured threshold;

    - FFS: RSRP and/or the RSRQ of the LP-SS/SSS on LR.

  - Proposal 11: A UE stops monitoring LP-WUS when the fallback condition is satisfied:

    - RSRP and RSRQ of LP-SS/SSS are less than the threshold configured by system information;

    - Or, the UE haven’t received LP-WUS for a configured time duration;

  - Proposal 12: Further study to determine LP-SS power control parameter based on the EPRE ratio between LP-SS to the ss-PBCH-BlockPower.

  - Proposal 13: The condition to enable RRM measurement by LR based on LP-SS and SSS should be decided by RAN4 and RAN2 according to the WID.

[19] ETRI

  - Proposal 7: RRM relaxation and RRM offloading condition should be based on serving cell measurement results.

  - Proposal 8: The entry/exit conditions for LP-WUS monitoring are used as baselines for determine conditions for RRM relaxation and RRM offloading. Further details can be discussed further e.g., different thresholds.

[21] Panasonic

  - Proposal 4: UE starts LP-WUS monitoring, if one or more following conditions are met, depending on the configuration,

    - the serving cell RSRP measurement performed by the MR is above an entry threshold

    - the serving cell RSRP measurement gap between MR and LR based measurement is below a threshold

    - the serving cell RSRP measurement performed by the LR is above an entry threshold

    - the serving cell RSRQ measurement performed by the LR is above an entry threshold

  - UE falls back to legacy PO monitoring and may stop LP-WUS monitoring, if one or more following conditions are met, depending on the configuration,

    - the serving cell RSRP measurement performed by the LR is below an exit threshold

    - the serving cell RSRP measurement performed by the MR is below an exit threshold, assuming relaxed MR-based measurement

    - the serving cell RSRP measurement gap between MR and LR based measurement is above a threshold, assuming relaxed MR-based measurement

    - the serving cell RSRQ measurement performed by the LR is below an exit threshold

  - Proposal 5: LR performs serving cell measurement, if one or more following conditions are met, depending on the configuration,

    - the serving cell RSRP measurement performed by the MR is above a threshold

    - the serving cell RSRP measurement gap between MR and LR based measurement is below a threshold

    - the serving cell RSRP measurement performed by the LR is above a threshold

    - the serving cell RSRQ measurement performed by the LR is above an entry threshold

  - When LR performs serving cell measurement offloaded from NR, for serving cell measurement relaxation performed by MR, both Option 1 and Option 2 are supported, based on configuration and subject to conditions, e.g., the serving cell measurement performed by MR is above or below a threshold

    - Option 1: MR does not perform serving cell measurement

    - Option 2: MR performs serving cell measurement with relaxation

[23] Apple: Proposal 9: Send an LS to RAN2 including the following statement to facilitate the determination of entry/exit conditions for LP-WUS monitoring in RAN1:

  - “As shown in the UE power saving evaluation results in TR 38.869 Section 8.1.1.5, assuming MR enters ultra-deep sleep state, UE power saving gain can be achieved only if MR RRM measurements (including both serving cell and neighbor cell measurements) is significantly relaxed. This should be taken into consideration when defining the entry/exit conditions for LP-WUS monitoring.”

[24] Sharp: Proposal 7: Consider both MR measurement and LR measurement for entry conditions if LP-WUS and channels for MR are not in the same carrier or band.

[25] Docomo

  - Proposal 14: The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

    - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

    - In addition, if enough power saving gain is achieved, the serving cell measurement performed by the MR with relaxation can be considered for exit condition

  - Proposal 15: The accuracy of serving cell measurement by LR should be further considered (up to RAN2/RAN4)

  - Proposal 16: If SNR of when LP-WUS monitoring is enabled is better than SNR of when serving cell measurement by LR is enabled, entry/exit condition for serving cell measurement by LR should be supported

[26] QC: Proposal 11: RSRP and RSRQ measured by the MR and LP-RSRP and LP-RSRQ measured by the LR are used as the serving cell measurement metrics for LP-WUS monitoring entry/exit conditions.

[27] Ericsson

  - Proposal 13	Entry/exit conditions should be configurable, e.g., as part of the LP-WUS configuration.

  - Proposal 14	It should be possible to configure different entry/exit conditions based on supported WUR types, if any.

  - Proposal 15	The entry condition is based at least on serving cell measurements performed by MR and LR. The exit condition is based at least on serving cell measurements performed by LR.

  - Proposal 16	If serving cell RRM using LP-SS is supported, only scenarios that are suitable for operation with low LP-SS overhead and NW energy impact should be considered for RRM.

The moderator thinks it is good for RAN1 to avoid the detailed discussions on the low-level detailed conditions for entry/exit, e.g. exact what conditions and which measurement metrics to use. Those should be discussed in RAN2.

However, based on the input, a rather important issue is the relationship between the conditions for LP-WUS monitoring and the conditions for RRM measurement offloading/relaxation. As RAN1 performed the power saving evaluations, it is well understood in RAN1 that sufficient RRM measurement offloading/relaxation of MR is essential for UE to achieve power saving, which may not be obvious in RAN2/RAN4. Therefore, it is important that RAN1 agrees on the high level principle and RAN2/RAN4 can continue to work on the details. This can be a working assumption, which serves as a basis for RAN2/RAN4 to continue their work.

#### [CLOSED] Proposed working assumption 4-1:

When the UE starts LP-WUS monitoring and stops the legacy paging monitoring,

LR performs serving cell measurement.

MR either performs serving cell measurement with relaxation or does not perform serving cell measurement.

MR either performs neighbor cell measurement with relaxation or does not perform neighbor cell measurement.

Note: The details are left to RAN2 and RAN4 to decide. It can be revisited based on RAN2/RAN4 discussions.

| Company | Comments |  |
| --- | --- | --- |
| ZTE, Sanechips | Whether the entry/exit condition for LP-WUS monitoring and serving cell measurement relaxation should be satisfied at the same time should be discussed. For this working assumption, it is not emergent to confirm before the above issue is solved. |  |
| CMCC | Support |  |
| FW | We are in general OK |  |
| Xiaomi | support |  |
| Nokia1 | It is fair observation that the UE power saving opportunities with LP-WUS monitoring are more limited, if MR based measurements are performed. However, specification does not strictly specify UE measurement sample rate in IDLE mode, only that evaluation needs to be based e.g. two samples separated by at least half a DRX cycle. Also like discussed, the proposed approach would mandate network to configure entry/exit conditions so that it ensures that MR based mobility measurements. E.g. in sectorised cell deployments. This would negatively affect the overall LP-WUS operation opportunities. In addition, it has been raised that deep sleep should be considered in addition to ultra-deep sleep, as it can offer still attractive power saving benefits. For these the impact of MR based measurements (MR transition energy) is reduced. Hence as different configuration of threshold and resulted (UE) behaviour can be preferable in different scenarios/deployments, we do not believe that RAN1 should make mandates relates to the mobility related configuration (and behaviour). |  |
| Vodafone | 2nd and 3rd bullet don’t really say anything without any details about the conditions. Also, we don’t see the need to have it as a working assumption, as the discussion would still be open, and if the point is to help RAN2/RAN4 it would be preferable to have clear guidance from RAN1. Suggest to change it as a proposal and not working assumption |  |
|  | Support the working assumption. |  |
| Qualcomm | The condition for UE to start RRM measurement seems out of RAN1 WID scope. We prefer to leave it for RAN4 to discuss. We only support full overloading of MR serving cell RRM measurement to LP-WUR. There is also no need to discuss neighbour cell measurement relaxation given it is already specified. |  |
| vivo | Support |  |
| LG | Fine with the conclusion. |  |
| Panasonic | We agree with ZTE that we make discuss whether entry/exit conditions for LP-WUS monitoring and measurement offloading to LR are defined the same, or separately designed. |  |
| DCM | Genially OK. But we can not see the necessity of neighbour cell measurement during LP-WUS monitoring if the entry/exit condition for LP-WUS is properly operated. So, it should be delated from the working assumption. |  |
| OPPO | We generally think this is fine. But we see there is ongoing RAN4 discussion to further narrow it down. |  |
| Ericsson1 | Not support. Do not see need for explicitly specifying the proposed linkage. Also, the conditions for RRM offloading are outside RAN1 discussion scope as commented by other companies. |  |
| Samsung | To understand the logic of the proposal, we divided it into 4 cases shown as following: - Case 1(partially offloading): UE performs LR based serving cell measurement, and performs relaxed MR based serving cell measurement, UE performs MR based relaxed neighbour cell measurement. - Case 2(fully offloading): UE performs LR based serving cell measurement, and does not perform MR based serving cell measurement, UE performs MR based relaxed neighbour cell measurement. - Case 3(partially offloading): UE performs LR based serving cell measurement, and performs relaxed MR based serving cell measurement, UE stops neighbour cell measurement. - Case 4(fully offloading): UE performs LR based serving cell measurement, and does not perform MR based serving cell measurement, UE stops neighbour cell measurement. For case 1, it may aim to the UE located not at cell edge. And for case 4, it may aim to the UE located in the cell centre. However, the case 2 and 3 seems strange for us. We wonder if these 2 cases exist and are reasonable. For case 2, when the RRM measurement is fully offloading, which means the UE located in the cell centre, stopping neighbour cell measurements by UE seems more reasonable than performing MR based relaxed neighbour cell measurements. For case 3, when UE perform relaxed MR based serving cell measurement, it seems the UE located not at the cell edge, in this case, we do not think it’s reasonable to stop neighbour cell measurement since the UE is not in the cell centre. From that understanding, we suggest to split the case to make the proposal more clear. |  |
| Nordic | Proposal is reasonable. |  |
| CATT |  | In RAN4#111, the different states of MR and LR were discussed and the following 5 cases/scenarios were considered: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON. #5 Exit case (legacy case); On （relaxed）; On; Off. It was agreed to discuss the RAN4 requirements first for the case #1 and FFS for other cases. Are we plan to have parallel discussion in RAN1? |
| Moderator | After online discussion, the following conclusion was made: Conclusion RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4 |  |

## LR and MR on different carriers/frequencies

Here are the views presented in the contributions regarding the support of MR and LR operating on different band/carrier:

- Yes: [4] CMCC, [6] vivo, [8] xiaomi

  - [4] CMCC: For LR and MR operating on different bands/frequencies, the entry/exit condition of LP-WUS monitoring should always consider both LR and MR measurement results.

  - [8] xiaomi: Proposal 22: Support offloading RRM measurement to LR when LP-SS and NR channels are located in different carriers/bands.

- No: [2] HW/HiSi

  - [2] HW/HiSi

    - For simplicity, it is assumed that the MR and LR of a UE operate in the same band/carrier

    - Support UE to report which band/frequency/carrier it supports to monitor LP-WUS.

    - A UE can be informed in which band/frequency/carrier LP-WUS feature is enabled

- [22] Lenovo

  - Proposal 5: Continue studying In-band Vs out-of-band deployment options for LPWUR due to:

    - Better coverage at lower frequency.

    - Increased payload size due to less repetition needed to attain MR Msg-3 coverage thereby supporting more functionality at LPWUR.

    - Not supporting RRM measurement offloading from MR.

- [23] Apple

  - Proposal 10: Support LR and MR operating on the same carrier as the baseline. LR and MR operating on contiguous carriers in the same band can be supported if not much additional effort is needed. Other cases FFS.

#### [CLOSED] Question 5-1:

Please share you views on whether to support MR and LR operating on different band/carrier and explain why.

| Company | Comments |  |
| --- | --- | --- |
| ZTE, Sanechips | Open to consider, and the workload may come from RAN4 or RAN4. Additionally, the LP and PO mapping relationship could be discussed |  |
| CMCC | Support MR and LR can operate on different band/carrier. As we discussed in our tdoc: - The first issue here is coverage. Based on the evaluation results provided by companies in AI 9.6.1, without introducing enhancement mechanism (e.g. repetition), the coverage performance of LP-WUS may still have some gap compared with the coverage performance of Msg3. Considering the typical frequency point in current NR deployment (e.g. 2.6GHz, 4.9GHz, 3.5GHz), deploy LR in a lower frequency point (e.g. 700MHz, 900MHz) is beneficial from improving LP-SS/LP-WUS’s coverage performance. - The second issue here is hardware capabilities. During the previous discussion on receiver architecture, the components in LR-WUR (for instance, RF BPF and LO) has lower processing complexity, which can reduce both the power consumption and the cost of LR. Therefore, if LR support all possible operating bands for MR, the requirement for processing complexity on the hardware components shall increase, which can result in higher power consumption and cost for the LR. Therefore, it is recommended that can operate on different band/carrier. |  |
| Xiaomi | OK to consider this case, since LR may not be able to support the bands/carrier where MR operate restricted by its hardware complexity. |  |
| Vodafone | Support, with similar comments as CMCC. |  |
| Qualcomm | We do not support the assumption that LP-WUR and MR are operated in different carriers for Rel-19 in RAN1 understanding. Or we can leave this to RAN4 discussion. |  |
| vivo | Support MR and LR operating on different band/carrier. As mentioned in our contribution, if LR and MR are always confined in the same carrier/band, it will limit the opportunity for LP-WUS operation, considering LR may only support a limited number of candidate carriers/bands to reduce complexity and power consumption. In this case, for the carriers/bands supported by MR but not supported by LR, LP-WUS cannot be used. An alternative way is to ask UEs to camp on only the carriers/bands that supported by LR, however, this forces UEs which support LP-WUS operation to prioritize the carriers/bands supported by LR, which changes current camping priority rules and limits the camping priority from operators. Further, such camping priority will cause camping congestion on LR operated carriers/bands. Therefore, LR and MR operating in different carriers/bands shall be supported. To support this, minor spec effort is observed, e.g., the existing NR mechanism has already supported different timing, SCS and duplex modes across different carriers/bands, which can be reused here. |  |
| DCM | Support MR and LR can operate on different band/carrier with similar reason as CMCC. |  |
| OPPO | It seems the measurement for different carrier can not be shared between LR/MR. The different carrier scenarios should be really justified. |  |
| MTK | Open to discuss. We see the benefit to support a low-band LPWUR lower than MR. But it is unclear how much power saving gain can be achieved in this case. |  |
| Spreadtrum | Can be left to further enhancement if low band has benefit |  |
| Samsung | We have concern to support MR and LR operating on different band/carrier. If different band is supported, the RRM measurement result based on LR cannot be used to judge the channel condition on MR. Even though RRM measurement can be offload to LR, MR need to wake up periodically to check the channel condition in MR. We think this operation will cost a lot of power which are not acceptable. |  |
| Nordic | In our opinion UE can by implementation decide to camp on bands offering LP-WUS. This ensures that MR and LR are camping on the same band. |  |
| Nokia2 | We think that having MR and LR to camp on different carriers that could have very different deployments would be challenging to ensure consistent behaviour e.g. in relation to thresholds. In our view it would be preferable to ensure via other methods (e.g. priorities) that UE camps on only one carrier, with both MR and LR. |
| Moderator | In RAN1, the common understanding is that LR may support a subset of bands that MR supports. There is a concern that the network may not be able to load balance idle/inactive UEs on different carriers any more, if all the LP-WUS capable UEs camp on the band that LR supports. Companies that want to support LR and MR on different carriers in R19: vivo, CMCC Concerns on supporting LR and MR on different carriers: Significant spec impact in RAN2/RAN4 (entry/exit conditions, RRM offloading/relaxation conditions, RRM requirements, etc.) Whether the UE power saving gain is still there There may be some other implementation solutions to address the issue? This is not a R19 issue? After online discussion, the following was agreed: Agreement Send an LS to RAN2 and RAN4 to convey the following In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. Final LS in R1-2407559. |  |

## [INACTIVE] LP-SS

## Other

eDRX: Note that we had decided to postpone eDRX related discussion to a later stage until we have a clearer picture on the design for iDRX.

Topics that are expected to be discussed in AI 9.6.1:

- Whether to have preamble for LP-WUS

- Bandwidth of LP-WUS

- Periodicity of LP-SS

- Overlaid OFDM sequence for LP-SS

#### Question 7-1:

Please provide any other comments/suggestions you may have, e.g., the topics that you think are important to be discussed now.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | LP-WUS/LP-SS power control could be discussed. |
| Xiaomi | Similar view as ZTE. Power offset between LP-SS and SSB should be considered. The power offset is essential for determine the criteria for LR activation/de activation. |

# Guidance for RAN1#118bis Discussions

For RAN1#118bis discussions, it is recommended that companies’ contributions include discussions at least on the following aspects:

Details of LO configurations, support of Option 1/2/3

Support of beam operations

Maximum number of subgroups, and how it is supported by the proposed maximum number of information bits

Remaining issues on RRM measurement metrics

It is recommended to leave the following to RAN2 to avoid duplicated discussion:

Whether to support separate subgrouping configurations for LP-WUS and PEI

# References

- R1-2405807	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2405868	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2405920	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum Communications

- R1-2405997	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2406105	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

- R1-2406194	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2406223	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2406296	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2406380	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2406413	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2406423	LP-WUS operation in IDLE/INACTIVE mode	Nokia

- R1-2406481	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2406499	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2406519	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Fujitsu

- R1-2406538	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2406567	Discussion on LP-WUS operation in IDLE/INACTIVE modes	KT Corp.

- R1-2406612	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2406662	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2406735	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2406763	LP-WUS operation in IDLE INACTIVE modes	MediaTek Inc.

- R1-2406786	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2406815	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

- R1-2406851	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2406882	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2406942	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2407041	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2407060	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2407135	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

# Appendix A: Agreements from previous meetings

## RAN1#116

Agreement

Multi-beam operations are supported for LP-WUS and LP-SS for idle mode

Agreement

LP-WUS occasions (LOs) are defined for LP-WUS monitoring.

- Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs.

  - Different LP-WUS MOs may correspond to different beams in multi-beam operation

  - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs

  - FFS details

- It is at least supported that a UE monitors LOs with a configured periodicity.

- Each UE has a periodicity for LO monitoring, and it is at least supported that a UE monitors one LO per period.

  - FFS: A UE does not expect its LP-WUS monitoring occasions overlapping in time

  - FFS: monitoring of multiple more than one LOs per period e.g. if LP-WUS common to all UEs is supported or in case of eDRX (if supported)

- FFS eDRX, if supported

Agreement

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Agreement

It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

- FFS: support of UE monitoring dynamic PO

Conclusion

For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

## RAN1#116bis

Agreement

For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS

- the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation

Agreement

Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam.

- Option 1: K = 1

- Option 2: K can be larger than or equal to 1

  - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Conclusion

LP-SINR is not considered further as a metric for RRM serving cell measurement for OOK-based receiver.

## RAN1#117

Conclusion

Regarding the “FFS: How to determine the received power of LP-SS in OOK ON symbols” for LP-RSRP, no additional work in RAN1.

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Agreement

For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16].

- FFS the exact value of Z

Agreement

For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256.

- FFS the exact value of X.

# Appendix B: Proposals from contributions

## [1]	R1-2405807	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

LP-WUS Payload Size/Structure Observation 1: LP-WUS support of an adequate number of subgroups in RRC Idle/Inactive is required to limit the impact of false paging and high transition energy on power saving gain. Observation 2: Rel-19 LP-WUS information size should support at least a subset of Rel-17 PEI subgrouping configuration, i.e., up to 8 subgroups and up to 8 POs per PEI. Larger number of subgroups can be considered to ensure power saving gain. Observation 3: Carrying LP-WUS information using encoded bits may require an 8-bit CRC to target a FAR < 1% and the corresponding overhead may be minimized by considering up to 16 bits of information. Proposal 1: Support at least the alternative to carry up to 16 bits of LP-WUS information using encoded bits with an 8-bit CRC when bitmap based wake-up indication is considered and without CRC when codepoint based wake-up indication is considered. Observation 4: The maximum time offset between an LP-SS and an LP-WUS without a preamble can be 74 ms and 174 ms for LP-WUS time error tolerance of 2us and 4us, respectively, at a sampling frequency of 7.68 MHz and assuming no frequency error correction by LR. Observation 5: The maximum time offset between an LP-SS and an LP-WUS without a preamble can be 228 ms and 534 ms for LP-WUS time error tolerance of 2us and 4us, respectively, at a sampling frequency of 7.68 MHz and assuming frequency error correction by LR. Proposal 2: Assuming no frequency error correction by LR, consider a preamble to precede the transmission of an LP-WUS if LP-SS periodicity is >= 320 ms and the time offset between LP-WUS and last LP-SS is, e.g., > 50 ms. LP-WUS Configuration Observation 6: PEI-Os are defined using a reference point, one or more symbol-level offsets, a number of POs per PEI, and a search space. Proposal 3: Consider the following options, depicted in Figure 1, for LO configuration and association with the Ns POs per PF where K is the number of LP-WUS MO groups per LO Option 1: a reference point, Ns × K offsets to first LP-WUS MOs, and a time gap Option 2: a reference point, Ns offsets to first LP-WUS MOs, a number (K) and periodicity for LP-WUS MO groups, and a time gap Observation 7: According to TR38.869, considering LP-WUS and dynamic PO determination for UEs configured with an eDRX cycle can provide significant improvement in paging latency without an increase in power consumption. Proposal 4: Support LP-WUS and dynamic PO determination for UEs configured with an eDRX cycle and consider LO configuration using at least a reference point, an offset, and an LO cycle ≤ the eDRX cycle and ≥ the I-DRX cycle. Observation 8: Configuration of Dynamic POs can be used to handle the different LR’s wake-up delay capabilities without penalizing UEs with short LR’s wake-up delay. Observation 9: Existing paging configuration allows PF presence every {1, 2, 4, 8, 16} radio frames and therefore support of LP-WUS with Dynamic PO under I-DRX configuration may not require the introduction of additional PFs. Observation 10: The SFN for the dynamic PF can be determined by introducing a negative dynamic PO offset (DPO_offset) to the UE_ID in existing PF determination formulation where DPO_offset may be determined by floor ((SFN – SFN_LO – RU_offset)*(N div T)) for an SFN_LO determined based on LP-WUS detection and a ramp-up offset (RU_offset) for the main radio. Proposal 5: Support LP-WUS with dynamic PO determination for UEs configured with an I-DRX cycle, to enable lower paging latency and/or handle different LR’s wake-up delay capabilities, and consider LO configuration using an LO cycle ≤ the I-DRX cycle. Observation 11: An alternative paging RNTI may be considered for the paging DCI in the Dynamic PO when only UEs with Dynamic PO configuration are to be paged to avoid increasing false paging to legacy UEs. Subgroup Indication in LP-WUS Observation 12: Option 1 for indicating subgroup information can support number of subgroups M<=32 with reasonable network resource overhead and no increase in paging latency whereas Option 2 can support number of subgroups 32<M<=256 with marginal increase in network resource overhead but potentially with an impact on paging latency. Proposal 6: Consider the following options for LP-WUS to support large number of paging subgroups with limited impact on coverage and/or resource utilization Option 1: LP-WUS carrying bitmap-based wake-up indication for subsets of the subgroups – Option 1-1: subgroup-subset identification using association with LP-WUS MOs – Option 1-2: subgroup-subset identification using a header – Option 1-3: subgroup-subset identification using a preamble/frame sync Option 2: LP-WUS switching between bitmap and ID-based subgroup wake-up indication – Option 2-1: Number of IDs in LP-WUS determined by a header – Option 2-2: Number of IDs in LP-WUS determined by an end of frame delimiter – Option 2-3: Fixed number of IDs in LP-WUS with padding bits, if needed

## [2]	R1-2405868	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

Observations: - The existing subgrouping mechanisms of R17 PEI in RAN2 specification can easily adapt to larger number of subgroups, i.e. a maximum subgroup number larger than 8 only requires a parameter replacement in existing subgrouping formula. Therefore, supporting larger subgroup number for LP-WUS only requires negligible RAN2 efforts and no CN specification effort. - Due to the extremely high transition energy of ultra-deep sleep, little power saving gain and even negative power saving gain may be introduced by LP-WUS, if the effective paging rate is high. - To get good power saving gain, the effective paging rate of LP-WUS should be < 3%. - The evaluation assumption that only 10 UEs monitoring the same PO may be not reasonable for LP-WUS design, which should be re-discussed. - Within an I-DRX cycle, the number of LOs can be smaller than that of POs, which is beneficial for reducing the signaling overhead, as well as giving gNB more scheduling flexibility. - Based on the requirements/assumptions in TR37.910, to achieve effective paging rate <=3%, up to ~20-bit information to distinguish a group/subgroup may be needed. - One-to-one mapping between LO and PO may lead to resource wasting for the following reasons: - The average effective paging rate per PO is considered low, thus in most of the cases there is no LP-WUS transmission in one LO - The spectrum efficiency is low for LP-WUS, which increases the resources occupied by one LO - Beam sweeping operation increases the resources occupied by one LO - If the burst traffic arrival case is considered for LO resource configuration, it increases the resources occupied by one LO - One-to-multiple mapping between LO and PO is helpful for utilizing resources more efficiently since multiple POs can share the same resource pool in one LO. - Continuous monitoring can reduce paging latency and can provide gNB more flexibility of LP-WUS transmission. - When e-DRX is configured, the power saving gain provided by LP-WUS is not attractive. - Deploying LP-WUS in only some bands/carriers helps to reduce the cost of LP-WUR and the system overhead of the network, thus the number bands/carriers supported by LP-WUR can be less than for the MR. Proposals: - The maximum number of subgroups per PO supported by LP-WUS is X, and X equals to 256. - The block of information bits carried in LP-WUS monitoring occasion contains group ID information whose bit length is configurable to support various network deployments, UE densities and traffic statistics. - Configuring up to 16-bit information is supported. - The location of LO can be determined based on the location of a PO/PF by an offset. - For mapping between LO and PO, support Option 2 that UEs corresponding to different POs monitor the same LO - Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam, K can be larger than or equal to 1. - How to incorporate MOs that are used for multi-beam operation, LP-WUS repetitions and different paged UE sub-groups into a LO is studied. - From network perspective a single LO with periodicity equal to one DRX cycle can be configured to all UEs so that all the potential LP-WUS MO resources available from network perspective in one DRX cycle can be shared and monitored by all UEs irrespective of their PO grouping. - The following issues need to be discussed to support duty-cycle monitoring of LP-WUS - How to balance the tradeoff between collision probability and LP-WUR power consumption. - How to reduce the network resource/energy for common information transmission. - How to provide system level timing information for an LP-WUR to locate its monitoring duration. - QCL assumptions between SSB/LP-SS and LP-WUS should be configured or implicitly defined and it is supported that LP-WUS/LP-SS can be transmitted via a subset of all SSB beams. - The power offset between SSB and LP-WUS/LP-SS should be specified and can be configured by gNB in system information. - Confirm the working assumption for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode - The detailed metrics to be used for serving cell measurement can be left to RAN2/RAN4 discussion. - The same threshold(s) is used for entry/exit conditions for LP-WUS monitoring and RRM relaxation (including RRM offloading) - LP-WUS with e-DRX is deprioritized. - Confirm the working assumption that from RAN1 perspective for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - The RRM metrics for OOK-based receiver, including LP-RSRP and LP-RSRQ can be utilized for OFDM-based receiver. - The transition time required by wake-up processing, i.e. the time from LP-WUS processing in LP-WUR to MR ready for paging monitoring should be known by gNB. - Study whether time and frequency information of LP-WUR can be utilized by MR to reduce the transition time, e.g. reducing the time of re-sync procedure of MR. - Support MR to stay in both deep sleep state and ultra-deep sleep state when LP-WUR is monitoring LP-WUS. - Support UEs to report its transition time considering at least different LP-WUR types and sleep states. - gNB can configure a single minimum time gap between LO and PO, which can be based on the capability report from UEs. - For a UE with LP-WUR, if the guaranteed minimum time gap in the cell is smaller than the reported transition time, the UE will not monitor LP-WUS. - For simplicity, it is assumed that the MR and LR of a UE operate in the same band/carrier - Support UE to report which band/frequency/carrier it supports to monitor LP-WUS. - A UE can be informed in which band/frequency/carrier LP-WUS feature is enabled - Enhancements for adjusting band/frequency priority may be needed.

## [3]	R1-2405920	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum Communications

LP-WUS occasion and monitoring occasion Proposal 1: K>1 can be supported for LP-WUS monitoring occasions. Proposal 2: If LO has large overhead for time resource, LO cannot be placed before PF/PO with association. Proposal 3: Fro relationship between LO subgroup(s) and PO subgroup(s), Option 1/2/3 can be supported by gNB configuration. Proposal 4: The maximum number of subgroups per PO is 8.

## [4]	R1-2405997	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

Proposal 1. In IDLE/INACTIVE mode, - When UE starts LP-WUS monitoring, the MR RRM measurement should also be relaxed or offloaded. - When UE monitors the legacy PO (and may monitor PEI) and stops LP-WUS monitoring, UE follows the legacy MR RRM measurement behavior. Proposal 2. The following entry threshold(s) can be considered as the criteria of UE starting monitoring LP-WUS: - A MR RSRP threshold; - A MR RSRP variation threshold within a period of time; - A MR RSRP threshold and a MR RSRP variation threshold within a period of time; - A MR RSRP/RSRQ threshold for inter-frequency measurement for the frequency where LP-WUR operates. Proposal 3. The following exit threshold(s) can be considered as the criteria of UE starting monitoring the legacy PO/PEI and stopping monitoring LP-WUS: - A LR RSRP threshold; - A LR RSRP variation threshold within a period of time; - A LR RSRP threshold and a LR RSRP variation threshold within a period of time; - A LR RSRP threshold and a MR RSRP threshold. Proposal 4. For LR and MR operating on different bands/frequencies, the entry/exit condition of LP-WUS monitoring should always consider both LR and MR measurement results. Proposal 5. Support further discuss how to avoid the degradation on monitoring performance for MR while exiting LP-WUS monitoring. For specific information in LP-WUS, Proposal 6. The subgrouping between LP-WUS and PEI is independent, e.g., separate subgroup number, separate association with PF/PO. For LP-WUS monitoring occasion, Proposal 7. Support the following options for the association of LO and PO from cell perspective: - Option 1: One LO is associated with one PO. - Option 2: One LO is associated with multiple POs. Proposal 8. Regarding the determination of LO location, the folloiwng three options cannbe considered: - Option 1: gNB configure the time domain offset(s) of LO related to PO. - Option 2: gNB configure the periodicity of LO and number of LOs, UE determines the LO index based its PO index, e.g., LO index = [(UE_ID mod N) *Ns + i_s] mod L, L is the number of LOs within LO periodicity - Option 3: gNB configure the time domain offset(s) of LO relative to LP-SS For relationship between operating bands on MR and LR, Proposal 9. Support LR and MR work on both the same and different operating bands (or carriers).

## [5]	R1-2406105	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

Observation 1: Defining a separate subgrouping for LP-WUS may complicate the higher layer procedure for subgrouping and increase the false paging. Observation 2: Option 1, which includes a bitmap in the LP-WUS with each bit corresponding to one subgroup, does not specify the information that should be used to target specific UEs or subgroups of UEs. Observation 3: Using the existing UE ID, UEs subgroup ID in the content of LP-WUS may result in significant overhead and increased complexity in decoding the LP-WUS. Observation 4: LP-WUS occasion has one or more LP-WUS monitoring occasions (MOs), where the LP-WUR performs detection attempts to decode the LP-WUS Observation 5: LP-WUS occasion can be defined as a set of consecutive monitoring occasions, and during this time, LP-SS is not configured to avoid conflicts. Observation 6: In idle/inactive state configuring LP-WUS monitoring together with PEI monitoring may trigger the MR of UE by two times, one time by LP-WUS to wake up and one time by PEI to indicate paging, which may increase the UE power consumption. Observation 7: In idle/inactive state the MR trigger of a UE via the LP-WUS can be consider as an indication for a UE to monitor the upcoming PO, which can replace the Rel-17 PEI. Proposal 1: Consider the similar subgroups for LP-WUS in idle/inactive state, which are intended for paging purposes. Proposal 2: Support X = 8 as the maximum number of subgroups supported by a LP-WUS in a PO. Proposal 3: For idle/inactive mode, consider including the subgroup ID as part of the LP-WUS payload for subgroup-based wake-up indication. Proposal 4: Codepoint based subgroup ID can be included in the LP-WUS, where each codepoint target a separate UEs subgroups. Proposal 5: Consider the maximum number of information bits (excluding CRC) in an LP-WUS as 8 bits. Proposal 6: Consider the total payload size of the LP-WUS (excluding CRC) as 32 bits, which may include the information bits and the codepoints-based subgroup IDs. Proposal 7: For LO consists of N * K LP-WUS MOs, K can be larger than or equal to 1. Proposal 8: To locate LP-WUS occasion in the time domain, a reference point and an offset from the reference point can be used: - For idle/inactive UEs, the incoming PO or SSB can be used as a reference point - For connected UEs, SSB or periodic LP-SS can be used as reference point. Proposal 9: For the periodicity of LO in iDRX cycle, Support option 1, which states that the periodicity of the LO for a UE in iDRX cycle is the same as the iDRX cycle Proposal 10: For LO association with PO support all options: - Option 1: One-to-one mapping between LO and PO - Option 2: One LO can be associated with multiple POs - Option 3: Multiple LOs can be associated with one PO Proposal 11: For the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, a predefined time window (or X number of paging cycles) can also be considered as an entry condition into the LP-WUS monitoring. Proposal 12: Consider the LP-WUS as an alternative option for paging early indication. Proposal 13: After the MR of UE trigger by LP-WUS, the UE can Perform PO monitoring, and afterwards follow the legacy procedures.

## [6]	R1-2406194	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

Observation 1: With more realistic assumptions e.g., around 20000 ~50000 UEs using LP-WUS per paging area, 8~16 subgroups per PO is sufficient to guarantee significant UE power saving gain e.g., 85% with the system overhead for LP-WUS transmissions to be less than 3%. Observation 2: LP-WUS repetition can be achieved within a LP-WUS MO not among multiple MOs. Observation 3: For operating MR and LR in the same carrier/band by forcing UEs camped on only the carriers/bands supported by LR, it will limit the camping priority from operators and causes UE camping congestion. Observation 4: Supporting LR and MR in different carriers/bands requires minor spec effort. Proposal 1: UE is not required to monitor a PO, if UE does not detect LP-WUS at all LP-WUS monitoring occasion(s) for the PO. Proposal 2: Support the maximum number of subgroups to be 8~16. Proposal 3: Confirm the working assumption for the entry/exit conditions for LP-WUS monitoring agreed in RAN1 #116bis meeting. Proposal 4: Support the following UE behavior on RRM measurement when UE starts LP-WUS monitoring, - UE performs LR based serving cell measurement, and performs relaxed or does not perform MR based serving cell measurement; - UE performs MR based relaxed neighbour cell measurement or UE stops neighbour cell measurement. - FFS the conditions for different RRM measurement cases. Proposal 5: Thresholds of for serving/neighbor cell measurement relaxation should be lower than or equal to the threshold of entry condition for LP-WUS monitoring. Proposal 6: For LP-RSSI definition for LP-RSRQ, LP-RSSI is the liner average of total received power in the same number of ON and OFF OOK symbols. Proposal 7: Define the frequency domain granularity of LP-RSRP and LP-RSSI for LP-RSRQ. Proposal 8: The network configures the measurement time resource for LP-SSS-RSSI, which can be different from NR SMTC. Proposal 9: Clarify that the same subgrouping information will be carried in the MOs of a LO. Proposal 10: For the relationship between LO and PO, support Option 1 i.e., UEs monitoring the same PO monitor the same LO. Proposal 11: Support to report UE capability on wake-up delay which is the time offset between receiving LP-WUS targeted to itself and being ready for PO reception. Proposal 12: Support the network to configure one or multiple offsets between LO and PO. Proposal 13: For multi-beam operation of LP-SS, each LP-SS MO is associated with each transmitted beam in turn. Proposal 14: Support K i.e., the number of LP-WUS MOs for each beam can be larger than or equal to 1 to provide the flexibility for the network. - when the UE detects a LP-WUS within its LO, the UE is not required to monitor the subsequent MO(s) associated with the same LO. Proposal 15: Support duty-cycled LP-WUS monitoring for RRC idle/inactive mode. Proposal 16: The periodicity of LP-WUS occasion (LO) for a UE is the same as its IDRX cycle. Proposal 17: Support LR and MR to work in the same or different Carriers/bands.

## [7]	R1-2406223	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- A unified LP-WUS design can be used for IDLE/INACTIVE mode, as well as CONNECTED mode. For wake-up information, A UE group is assigned with single bit in the LP-WUS for indication of wake-up in IDLE/INACTIVE modes. - The periodicity of LP-SS can use 320ms as baseline. It can be further studied for 80ms, 160ms and 640ms as additional periodicity Observation 1: LP-SS without overlaid OFDM sequences may provide simple design for all types of LP-WUR. But it can only be used for coarse time/frequency synchronization of LP-WUR. Observation 2: LP-SS with overlaid OFDM sequences can provide synchronization for OFDM based LP-WUR. In addition to synchronization, it could be beneficial for other functions, e.g., on LP-WUS monitoring, PCI indication, group/sub-group indication. - LP-SS is overlaid with same type of sequences same as that for LP-WUS. Further consider information carried by overlaid OFDM sequences. - A UE can monitor one LO in each LO period. - The LO periodicity is the same as the iDRX periodicity. - Each LO consists of N * K LP-WUS MOs, where K is the number of LP-WUS MOs for each beam, and K = 1. - When the UE starts LP-WUS monitoring and stops the legacy paging monitoring, set of combinations of LR/MR measurement periodicities should be introduced. - Study the mechanism to switch MR back to sleep when it is wakened up by false alarm. - Dynamic PO is not supported and the power saving gain can be up to gNB’s implementation for scheduling. - LP-WUS should at least contain the information on which user(s) is/are target by the LP-WUS, and UE group/sub-group information should be considered. - Additional information, such as cell information, SI change and ETWS/CMAS information, tracking area information and RAN area information, can be contained by LP-SS or its overlaid OFDM sequence. - For idle/inactive mode, the maximum payload size of LP-WUS with 8 bits can be the starting point.

## [8]	R1-2406296	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

Proposal 1: Restricted by LP WUR capability, LP-WUS/LP-SS and signals/channels used by MR can be on different carriers or different bands. Proposal 2: The detailed LP WUR supporting bands requirement is up to RAN4 to decide. Proposal 3: Support BW of LP-WUS is configurable, and support LP WUS BW being different for RRC idle/inactive and RRC connected states. Proposal 4: Support beam sweeping pattern of LP-WUS. Proposal 5: LP WUS can be QCLed with LP -SS by QCL type A. LP WUS and LP -SS can be QCLed with SSB by QCL type C. Proposal 6: The power offset of LP-SS/LP-WUS compared to the power of SSB should be configured. Proposal 7: Support UE activation and/or deactivation of LR based on preconfigured criteria Proposal 8: Conditions to activate LR for UE with OOK-based LR and UE with OFDM-based LR can be different. Proposal 9: Conditions to deactivate LR for UE with OOK-based LR and UE with OFDM-based LR can be different. Proposal 10: LR is deactivated and MR is activated when (1) UL transmission happens; (2) LP WUS indicating the UE to wake up is received; (3)The reception performance of LP-SS/LP-WUS falls below LR sensitivity; (4)The reception performance of LP-SS/LP-WUS falls below a threshold, which triggers the MR waking up for neighbouring cell RRM measurement. Proposal 11: LR sensitivity requirement for OOK-based LR and OFDM-based LR should be defined by RAN4. Proposal 12: Support separate configuration of LP-WUS, with multiple LP WUS monitoring occasions (LOs) within a duty cycle. And UE can determine its corresponding LO index by its UE ID or by its PO index. Proposal 13: At least wake up indication at the granularity of UE subgroup should be carried in LP WUS. Proposal 14: UE can be provided with assistant information about whether PEI provide more subgrouping information besides LP WUS subgrouping indication. Proposal 15: Suggest to specify wake-up delay for activating MR from sleep state. Proposal 16: Support the following working assumption Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Proposal 17: For OFDM-based LR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LR. Proposal 18: Totally offloading and partially offloading of serving cell measurement from MR to LR can both be considered. Proposal 19: Criteria for whether to apply totally offloading or partially offloading can be based on (1) preconfigured criteria, (2) gNB configuration. Proposal 20: Support neighbouring cell measurement by OFDM-based LR. Proposal 21: Support relaxing of neighbouring cell measurement by MR. Proposal 22: Support offloading RRM measurement to LR when LP-SS and NR channels are located in different carriers/bands. Proposal 23: Support LP WUS applied for eDRX to reduce paging latency.

## [9]	R1-2406380	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

Proposal 1：UE capability of the support of LP-WUS can be indicated during UE registration procedure through NAS procedure. Proposal 2: The parameters of LP-WUS configuration can be indicated by SIB-1/SIB-X and/or NAS signaling. Proposal 3: The offset between LO and PO is configured by the network and UE will determine the sleeping type based on the transition time of different sleep mode and the network configured offset in its UE implementation. Proposal 4: The number of LP-WUS MOs for each beam can be larger than or equal to 1. The information of paging early indication by LP-WUS in multiple MOs within a SSB can be different. Proposal 5: The paging subgroup indication by LP-WUS would depend on the number of information carried by LP-WUS in the LP-WUS design. Proposal 6: Support Option 3(Option1 is the special case of Option 3 when then number of sets is 1). Proposal 7: The parameters for the mapping of the information point carried by the LP-WUS to the paging subgroup can be provided by NAS and/or derived from the parameters broadcasted by the system information. Proposal 8: The maximum number of subgroups per PO is 8. Proposal 9: The LP-WUS detection can up to UE implementation or determined by a predefined threshold. Proposal 10: The UE falls back to legacy mechanism when the LP-WUS is in detection errors. Proposal 11: When eDRX is configured, LP-WUS is monitored within PTW.

## [10]	R1-2406413	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

Observations Observation 1: The serving cell measurement performed by the MR is above entry threshold(s) and the serving cell measurement performed by the LR is below exit threshold(s) may be satisfied at the same time. Observation 2: - If UE only monitor LP-WUS but not perform LR measurement, there is no power saving gain - If UE only perform LR measurement but not monitor LP-WUS, it would waste NW resources of LP-WUS, there is no much power saving gain due to periodical paging monitoring, and it is out of scope. Observation 3: For a spare PO configuration scenario, 8 subgroups via LP-WUS can not provide PSG. For a dense PO configuration scenario, 8 subgroup via LP-WUS can provide PSG and less overhead. Observation 4: 256 subgroups via LP-WUS can provide obvious PSG. The supported UEs number in a cell can be up to 20000 with around 3% system overhead. Observation 5: With the number of subgroups increasing, one subgroup wake-up in a time duration becomes a minor case. Observation 6: Compared with LP-WUS based legacy PO monitoring, LP-WUS based dynamic PO monitoring has benefits of latency reduction and UE power saving. Proposals Proposal 1: For the activation condition of LP-WUS monitoring, that serving cell measurement performed by the LR is not below exit threshold(s) should be included. Proposal 2: UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, or - The UE has not received LP-WUS for a long time Proposal 3: Further clarify definition for OOK ON symbols or OOK OFF symbols. Proposal 4: For OFDM-WUR without FFT block, clarify the SS reference signal received power is based on 11RBs or 127 subcarriers to calculate the RSRP. Proposal 5: The entry/exit condition for LP-WUS monitoring and serving cell measurement relaxation should be satisfied at the same time. - If UE performs relaxed measurement based on WUR, the UE also should monitor LP-WUS, vice versa. Proposal 6: Support maximum number of subgroups per PO via LP-WUS up to 256 Proposal 7: LP-WUS indicates subgroups based on associated PO. - Further discuss whether/how LP-WUS indicates subgroups independent from PO Proposal 8: Support multiple subgroups as much as possible by LP-WUS in idle/inactive state. Proposal 9: Unified LP-WUS monitoring scheme is pursued, considering - I-DRX, eDRX - FDD, TDD Proposal 10: Each LO consists of N * K LP-WUS MOs, option 2 is adopted with K>1 - K could be larger than repetition number Proposal 11: Consider option 2 to determine the beam index and repetition. Proposal 12: For duty cycle LP-WUS in IDLE/INACTIVE mode, NW can configure different LO periodicity according to different situation. Proposal 13: For LP-WUS in IDLE/INACTIVE mode, a LP-WUS occasion (LO) can be mapped one or multiple POs. Proposal 14: eDRX is an important feature to be supported especially for IoT device. - Combined with LP-WUS, more PSG can be achieved. Proposal 15: Adaptive PTW could be considered for latency reduction and PSG. Proposal 16: Support LP-WUS deployed in FDD/TDD bands Proposal 17: The LO and MO definition should be applicable for both FDD and TDD - FFS whether MO could be based on flexible symbols or slots in TDD Proposal 18: For LP-WUS in IDLE/INACTIVE mode, dynamic PO can be considered.

## [11]	R1-2406423	LP-WUS operation in IDLE/INACTIVE mode	Nokia

In Section 2.1 we considered some aspects related to RRM and entry/exit procedures, with following observations:- Observation: Further discussion on entry/exit condition configuration should be done in RAN2 Observation: When UE monitors LP-WUS, and ceases the PO monitoring, there can be still power saving benefit, if MR based measurements are done. The scale of power saving depends on the rate of MR based measurements. Observation: In order to get power saving benefit from MR serving cell measurement relaxation and / or offloading to LR, LP-WUS monitoring and stopping paging monitoring is required as paging is monitored each paging cycle. Observation: Network needs consider different aspects when determining the suitable thresholds for entry/exit and RRM offloading. Forcing these thresholds to be linked can leading to overall reduced power saving opportunities for the UE. Observation: The approach for scenario where both, LR and MR based measurements are available, the approach for threshold determination and evaluation would need to be discussed in RAN2. Proposal: RAN1 concludes that the threshold configuration for entry/exit conditions and RRM relaxation/offloading can be independently configuration by network. Design is up to RAN2. Observation: The approach for scenario where both, LR and MR based measurements are available, the approach for threshold determination and evaluation would need to be discussed in RAN2. In Section 2.2 we discuss the LR measurement quantities, with following observation and proposals:- Proposal: LP-RSRP and LP-RSRQ measurements should be associated to a particular beam. Proposal: For LP-RSRP and LP-RSRQ measurements should be possible to network to configure sub-set of LP-SS’s from which LP-RSRP and LP-RSRQ can be measured. Observation: Definitions of NR SS-RSRP and SS-RSRQ would seem viable to be re-used for LR that can receive SSS. Proposal: For LR that is capable of receiving PSS/SSS, RAN1 confirms the working assumption of re-using the measurement quantity definition but separates the definitions of measurement quantities for LR measurements based on SSB (from the MR measurement quantities). Observation: It is expected that performance requirements will ensure consistent reporting from different UEs. In Section 3.1 we discuss the LP-WUS procedures for IDLE/Inactive mode:- Observation: ‘Dynamic PO’ and ‘closest PO’ approaches would result increased LP-WUS monitoring power consumption due to more frequent monitoring. Observation: For a same LP-WUS content size, ‘dynamic PO’ and ‘closest PO’ would result increased false alarm probability. Observation: ‘Dynamic PO’ and ‘closest PO’ would in minimum double the paging overhead. Observation: Reducing the overhead due to ‘Dynamic PO’ and ‘closest PO’ by UE behaviour would significantly increase the UE IDLE/Inactive mode power consumption. Observation: Latency benefit claimed for ‘dynamic PO’ and ‘closest PO’ cannot be guaranteed from system perspective if LP-WUS monitoring is not always mandated. Proposal: UE monitors the legacy PO after receiving LP-WUS indicating wake-up and no changes to the paging occasion to be monitored are supported. Proposal: If LP-WS operation with eDRX is supported and UE is configured with eDRX and LP-WUS, the LP-WUS monitoring is determined based on paging occasions so that UE can monitor LP-WUS associated to the POs that are within the PTW. In Section 3.2 we consider the aspects related to the LP-WUS monitoring configuration:- Observation: LO periodicity should be at least the same as iDRX periodicity. For eDRX the sparser monitoring would be effectively achieved by limiting the LP-WUS monitoring to POs contained in the PTW. Observation: LO periodicity smaller than iDRX period is not needed for reliability nor scheduling flexibility if number of LP-WUS MOs per beam is can be configured to be larger than one (K=>1). Proposal: Support from UE perspective that LO periodicity to be same as iDRX period. Proposal: LO time location is defined in relation to PF. Proposal: Define starting symbol for each the LP-WUS MOs in LO. Proposal: At least for operation with shared spectrum access, support having one or more LP-WUS MOs per LP-WUS beam (K≥1). Observation: By extending LP-WUS monitoring configuration flexibility, the LP-WUS channel design configurability need can reduced. In Section 3.3 we discuss the minimum offset between LO/LP-WUS and the associated PO:- Observation: RAN1 should agree common assumption for the MR transition time to derive the LP-WUS monitoring configuration. Observation: It would seem viable that LR provides timing assistance information for MR timing so that SSB search can be omitted from MR synchronisation timeline. Observation: MR synchronisation would need to be accounted in minimum timeline for paging monitoring upon LP-WUS triggering. Benefitting the LR timing assistance could be used to reduce the effort needed for MR synchronisation. Proposal: RAN1 discusses and agrees for the configuration one common assumption for the minimum offset between LP-WUS and PO. Proposal: RAN1 considers value in range of 400ms for the minimum offset between LP-WUS and the associated PO. Proposal: No maximum time offset between LP-WUS and LP-SS is defined. Observation: Making the paging delay UE capability dependent, may prohibit the use of LP-WUS in deployments where paging cycle (iDRX) needs to be configured to be shorter than the MR wake-up delay. Observation: Making the paging delay UE capability dependent, would complicate the system paging procedure. Proposal: LP-WUS to PO mapping is commonly defined so that all UEs of same sub-group have same LP-WUS to PO mapping. UEs should respect the configured paging period and monitor PO so that service latency requirement can be met. In Section 3.4 we the association between LO, LP-WUS and PO is considered, focusing on splitting PO sub-groups to different LP-WUS MOs or LOs:- Observation: In more ‘concentrated’ approaches (option1/option2) network could be able to obtain better energy saving, compared to more ‘distributed’ (option3). Observation: In general splitting sub-groups of a PO over multiple LP-WUS MOs can have some impact to the system overhead with higher paging probabilities. Observation: In more ‘concentrated’ approaches (option1/option2) the data scheduling may be complicated due to LP-WUS burst, while ‘distributed’ approach will spread the LP-WUS load more evenly. Observation: From UE perspective, splitting sub-groups of a PO over multiple LP-WUS MOs would not affect the need to monitor one LP-WUS MO. Proposal: Support LP-WUS monitoring configuration that allows to split the sub-groups of a PO to multiple LP-WUS MOs. Also mapping multiple POs to same LP-WUS could be supported if efficient encoding of sub-group information to LP-WUS is supported. Section 3.5 considers the issues related to the determination of beams association:- Observation: To enable selection of LP-WUS beams, there should be association between LP-WUS MOs and LP-SS and SSB beams. Proposal: Define association between LP-WUS MOs and LP-SS and SSB beams. Observation: UE should be able to identify the LP-SS beam association with specific SSB index via time location association. Observation: To close the loop in UE LP-WUS beam selection, LP-WUS MOs association to LP-SS/SSB beam should be identifiable via time location association. Proposal: Define that LP-SS and LP-WUS MO time locations are associated to a beam. In Section 3.6 we discuss LP-WUS content and make two propsosals:- Proposal: Consider configurable number of sub-groups in LP-WUS, up to 8 sub-groups . Proposal: Do not consider shortMessage related information separately in LP-WUS information content. In section 4 we discuss some additional aspects that relate to the UE power consumption in IDLE/Inactive and make an observation:- Observation: To reduce the energy cost of repeated paging monitoring due to MR wake-ups, UE should be provided assistance information whether it should expect further paging in following paging occasion.

## [12]	R1-2406481	LP-WUS operation in IDLE / INACTIVE modes	Sony

Observation 1 – The operation of LP-WUR based on duty-cycling is necessary to reduce the total power consumption. The long transition time to wake-up the main radio from ultra-sleep time together with sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement. Observation 2 – There could be a case where the LP-WUS is defined to have full cell coverage, still the LP-WUR may not be able to detect the LP-WUS. Proposal 1 – Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on an always-on or a duty-cycle scheme. Proposal 2 - For duty-cycled scheme where the LP-WUS monitoring is done in connection to existing DRX, the configuration needs to consider at least the following configuration parameters, Proposal 3 – Support time-frequency multiplexing of LP-WUS for different UEs to increase LP-WUS address/identification space. Proposal 4 – Support LP-WUS structure with two fields, a preamble field for synchronization and cell identification purposes and a data field for indication of sub-group wake-up. Proposal 5 – Support 16 bits as the maximum number of information bits for LP-WUS of UEs operating in Idle/Inactive mode. Proposal 6 – In order to avoid ping-pong behaviours, a time widow can be used. The UE switches on its MR to evaluate the channel condition only if no LP-WUS received/detected by the UE within this time-window or after such evaluation period.

## [13]	R1-2406499	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

Proposal 1: Support 16 bits as the maximum number of information bits excluding CRC in a LP-WUS. Proposal 2: Support up to 16 subgroups for LP-WUS. - Up to 32 subgroups can be considered if implicit indication is supported. Proposal 3: Target UE related information should be included in LP-WUS. Proposal 4: Support update of system Information via LP-WUS indication among preconfigured sets. Proposal 5: Information on neighboring cells should be supported in LP-WUS. Proposal 6: Support Option 1 (one-to-one association between LO and PO). - Option 3 can be additionally considered for implicit indication considering the limited information bits. Proposal 7: Support different LP-WUS monitoring occasions within one LP-WUS occasion considering the following cases. - Case 1. Support transmission of a same information group with multiple beams. - Case 2. Support transmission of different information groups with same beam. Proposal 8: For LP-WUS MOs, support Option 2 (K can be larger than or equal to 1). Proposal 9: For Case 1, support transmission of identical signals with different beams in N LP-WUS monitoring occasions within one LP-WUS occasion. Proposal 10: For Case 2, support delivering different information in K LP-WUS monitoring occasion with an identical beam within one LP-WUS occasion. Proposal 11: The working assumption on entry/exit conditions for LP-WUS monitoring is confirmed. Proposal 12: For the entry condition, the other conditions other than serving cell measurement are not supported. Proposal 13: For the exit condition, the other conditions such as timer are supported to ensure proper UE behavior. Proposal 14: SS-RSRP and SS-RSRQ are reused for OFDM based LP-WUR. - LP-SSS-RSRP and LP-SSS-RSRQ are not defined in the specifications. Proposal 15: LP-SS based RRM measurement is supported for OFDM based LP-WUR. Proposal 16: Introduce definitions of LP-RSRP and LP-RSRQ for OFDM based LP-WUR to support LP-SS based RRM measurement.

## [14]	R1-2406519	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Fujitsu

Observation: Option 1 requires the gNB to schedule the LP-WUS transmission in an intended single LO, and if that is missed, the UE may fail to wake-up, potentially causing paging delays. Observation: If a single LO is associated with multiple MOs for a specific beam (Option 2), and the information within these multiple LP-WUS is always the same, it imposes significant constraints on the scheduler. Proposal: For the association between LO and MO, K can be larger than or equal to 1 (i.e. Option 2) - if more than 1 LP-WUS is transmitted from the same beam, the information in these multiple LP-WUS can be different Proposal: Introduce a special LO, which is not associated with any PO, to efficiently wake up all UEs for SI change/ETWS/CMAS Proposal: Do not introduce UE capability for wake-up delay Proposal: Regarding the entry condition in the working assumption, the following yellow highlighted section should be added: From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - This condition does not apply after the detection of an LP-WUS and until the monitoring of the associated PO - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

## [15]	R1-2406538	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

Proposal 1: for RRC-IDLE/INACTIVE, the LP-WUS configuration indicates one or more LP-WUS frequency bands in a carrier, and for each LP-WUS frequency band, a periodicity and an offset of LOs are comprised in the configuration. Observation 1: explicit association between an LO and a PO (e.g., by a time offset) is not necessary, UE can determine the association based on its UE-ID implicitly. Proposal 2: for the association between LO and PO, support all the three following options in RAN#117, and it is up to gNB implementation, • Option 1: UEs monitoring the same PO monitor the same LO • Option 2: UEs corresponding to different POs monitor the same LO • Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Proposal 3: from UE perspective, it is not necessary to be aware of which option is used, UE may firstly determine a LOs based on its UE-ID, if a wake-up indication is received, UE can start monitoring PO from the first PO (also determined based on its UE-ID) after the MR is activated and resynchronized. Proposal 4: for the LP-WUS content, support bitmap based indication for multiple UE subgroups. Proposal 5: further study toggle based wake-up indication and flag based wake-up indication, and take the miss-detection issue into account. Proposal 6: study the following two options of RRM measurement: (1) MR measurement is triggered by the measurement result of LR; (2) UE performs both MR measurement (with relaxed periodicity) and LR measurement.

## [16]	R1-2406567	Discussion on LP-WUS operation in IDLE/INACTIVE modes	KT Corp.

Proposal 1: Support that the UE monitors the dynamic PO (Paging Occasion) after receiving LP-WUS. Proposal 2: For the duty-cycle based monitoring, at least in the UE side, only one-to-one mapping between LO and PO is supported. Proposal 3: Support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PO monitoring in RRC IDLE/INACTIVE mode, and further discuss that the capability is sharable with the minimum time gap to start PDCCH monitoring in RRC CONNECTED mode. Proposal 4. The payload contents carried by overlaid OFDM sequence of LP-WUS contains only part of the UE/subgroup ID information, with individual parity/CRC if needed. Proposal 5: For the LP-WUS monitoring in RRC IDLE/INACTIVE mode, a LP-WUS can be QCLed with existing NR signal/channel/CORESET for the TCI state. Proposal 6: A feedback is triggered for the non-existence of receivable LP-WUS in RRC IDLE/INACTIVE mode, included to the LR exit conditions. Proposal 7: Discuss for the effectiveness of QCL relations between signals in MOs in LO and corresponding PO(s).

## [17]	R1-2406612	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

Proposal #1: It is desirable that one LO is associated with a single PO. - The periodicity of LO is same as the iDRX cycle. Proposal #2: Considering the association between LO and PO, UE is required to monitor at least the associated PO with the LP-WUS occasion on which UE receives wake-up indication. Proposal #3: For LO allocation from UE perspective, LO is allocated based on the existing paging occasion. Proposal #4: If the separately configured LP-WUS cycle is supported, some LOs (which are other than LOs associated with legacy PO) can be associated with dynamic PO. Proposal #5: From gNB perspective, multiple LOs can be allocated based on the single existing paging occasion and each UE can selectively monitor the LO based on the subgroup configuration of the UE. - Each LO are mapped to one or more UE subgroups - FFS: How to map UE subgroup to LP-WUS MO - FFs: How to indicate one or more UE subgroup in an LP-WUS MO Proposal #6: Multiple LOs can be allocated based on the single existing paging occasion, UE can apply different offset based on UE subgroup index for LP-WUS Proposal #7: When the number of LP-WUS MOs for each beam is larger than 1, the same information can be repeated over those LP-WUS MOs. Proposal #8: Discuss the following options to determine how to allocate N*K LP-WUS MOs where K is larger than 2. - Option 1: K LP-WUS MOs are repeated N times. - Within one LO, N LMO groups are mapped to each beam, and the LMOs within the group are repeated K times. - Option 2: N LP-WUS MOs are repeated K times. - Within one LO, there are K LMO groups, and the LMOs within each group are repeated N times for each beam. Proposal #9: UE subgrouping for LP-WUS can be separately configured from the legacy PEI subgroup - For IDLE/INACTIVE mode UE, the maximum number of subgroups per PO should be larger than 8. Proposal #10: UEs with different wake-up delay should be assigned to different UE subgroups Proposal #11: UEs having longer wake-up delay can be assigned to separated UE subgroups - The number of those subgroups can be separately configured. - wake-up delays above a certain threshold are considered as longer wake-up delay Proposal #12: Following options can be considered for determination of PO to be monitored - Option 1: UE determines PO to monitor based on how the LO is allocated - After the reception of LP-WUS within a LO, UE monitors the PO which is used to determine where the LO is allocated. - Option 2: UE determines PO to monitor based on the wake-up time - After the reception of LP-WUS within a LO, UE monitors the nearest PO after the wake-up from the LO. Proposal #13: For UE triggered by LP-WUS to monitor a PO, introducing a time window based on the reception of LP-WUS. The time window has the following characteristics: - UE receives the existing PO only within the time window - UE transits to sleep state at the end of the window - The length of window is determined based on - The number of upcoming paging occasions - The number of slot/frame configured by SIB - FFS: LO mapped to dynamic PO Proposal #14: It is not necessary to consider eDRX for LP-WUS operation. Proposal #15: The UE may start LP-WUS monitoring if LP-SS-based measurement for serving cell performed by the main radio is above entry threshold(s), if configured. Proposal #16: Whether the UE can receive overlaid OFDM sequence over LP-WUS or not should be considered to specify entry/exit condition of LP-WUS monitoring. Proposal #17: Consider the following aspects about RRM measurement relaxation of the main radio by offloading to LP-WUR. - The UE can measure cell quality by LP-WUR while the main radio is in ultra-deep sleep state. - The existing RRM measurement procedure by the main radio can be triggered based on the serving cell measurement by LP-WUR. - If the UE stops LP-WUS monitoring due to the serving cell measurement which is below exit threshold(s) by LP-WUR, the UE starts to measure either a serving cell or a neighbor cell by the main radio. Proposal #18: If RRM measurement relaxation by using LP-WUR is supported, the UE with OFDM-based LP-WUR can perform RRM measurement by receiving LP-SS and/or PSS/SSS.

## [18]	R1-2406662	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

Proposal 1: For RRC_IDLE/RRC_INACTIVE mode, information carried by LP-WUS can be: - Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24 - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups; - FFS: If the payload size allows, other information to support additional functionality such as SI change and ETWS/CMAS information. Proposal 2: For the relationship between LO and PO, further clarify whether option2 is defined from network perspective or from UE perspective. Proposal 3: For the mapping relationship between LO and PO, consider the following options for further down-selection: - Option 1: UEs monitoring the same PO monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Proposal 4: The periodicity of LP-WUS occasion can be determined based on the mapping relationship between LO and PO. When option 1 is adopted, the periodicity of LP-WUS occasion can be same as iDRX cycle. When option 3 is adopted, the periodicity of LP-WUS occasion can be smaller than iDRX cycle. Proposal 5: The configuration of LO can be provided to the UE by system information for RRC_IDLE and RRC_INACTIVE mode. Proposal 6: For the number of LP-WUS MOs for each beam select Option 1: K=1. Proposal 7: The time location of LP-WUS occasion can be determined by a reference point and an offset. Proposal 8: The time offset between the end position of the LP-WUS monitoring and the reference point should be larger than LP-WUS processing time, MR transition time for ramp up and time/frequency synchronization of MR (e.g., at least > 400ms). Proposal 9: UE report one value per SCS for the minimum time offset, NW can configure one offset value based on the reported UE capability. Proposal 10: UE may start LP-WUS monitoring when the entry conditions for LP-WUS monitoring is satisfied: - RSRP and the RSRQ of the SSB for serving cell measurement is better than a configured threshold; - FFS: RSRP and/or the RSRQ of the LP-SS/SSS on LR. Proposal 11: A UE stops monitoring LP-WUS when the fallback condition is satisfied: - RSRP and RSRQ of LP-SS/SSS are less than the threshold configured by system information; - Or, the UE haven’t received LP-WUS for a configured time duration; Proposal 12: Further study to determine LP-SS power control parameter based on the EPRE ratio between LP-SS to the ss-PBCH-BlockPower. Proposal 13: The condition to enable RRM measurement by LR based on LP-SS and SSS should be decided by RAN4 and RAN2 according to the WID.

## [19]	R1-2406735	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

Proposal 1: LP-WUS monitoring occasion (LO) is required to be associated with PO of the UE. Proposal 2: For RRC IDLE/INACTIVE mode, minimum time gap between LP-WUS reception and MR to start PO monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR Proposal 3: Time location of the LO is defined by the time offset from the PO. Proposal 4: The time offset is configured by gNB. Proposal 5: UE monitoring relationship between LO and PO is decided based on the result of discussion on the maximum number of information bits in a LP-WUS and the maximum number of subgroups per PO. Proposal 6: Confirm the working assumption for the entry/exit conditions for LP-WUS monitoring in IDLE/INACTIVE mode from RAN1#116bis meeting. Proposal 7: RRM relaxation and RRM offloading condition should be based on serving cell measurement results. Proposal 8: The entry/exit conditions for LP-WUS monitoring are used as baselines for determine conditions for RRM relaxation and RRM offloading. Further details can be discussed further e.g., different thresholds.

## [20]	R1-2406763	LP-WUS operation in IDLE INACTIVE modes	MediaTek Inc.

Proposal 1: Support Option 1: UEs monitoring the same PO monitor the same LO for simplicity. Proposal 2: Clarify Option 2 whether NW can page multiple POs for the same paging message. Proposal 3: Support Z = 4 bits for a small payload design. Proposal 4: Support 8<= X <=16 as the maximum number of subgroups per PO.

## [21]	R1-2406786	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

Proposal 1: Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K=1 is the number of LP-WUS MOs for each beam. If repetition is supported, it should be within a MO. Proposal 2: Option 3 should at least be supported that UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Proposal 3: For IDLE/INACTIVE mode, the maximum number of information bits Z = 16 in a LP-WUS, and the maximum number of subgroups per PO X = 256, can be considered. Proposal 4: UE starts LP-WUS monitoring, if one or more following conditions are met, depending on the configuration, - the serving cell RSRP measurement performed by the MR is above an entry threshold - the serving cell RSRP measurement gap between MR and LR based measurement is below a threshold - the serving cell RSRP measurement performed by the LR is above an entry threshold - the serving cell RSRQ measurement performed by the LR is above an entry threshold UE falls back to legacy PO monitoring and may stop LP-WUS monitoring, if one or more following conditions are met, depending on the configuration, - the serving cell RSRP measurement performed by the LR is below an exit threshold - the serving cell RSRP measurement performed by the MR is below an exit threshold, assuming relaxed MR-based measurement - the serving cell RSRP measurement gap between MR and LR based measurement is above a threshold, assuming relaxed MR-based measurement - the serving cell RSRQ measurement performed by the LR is below an exit threshold Proposal 5: LR performs serving cell measurement, if one or more following conditions are met, depending on the configuration, - the serving cell RSRP measurement performed by the MR is above a threshold - the serving cell RSRP measurement gap between MR and LR based measurement is below a threshold - the serving cell RSRP measurement performed by the LR is above a threshold - the serving cell RSRQ measurement performed by the LR is above an entry threshold When LR performs serving cell measurement offloaded from NR, for serving cell measurement relaxation performed by MR, both Option 1 and Option 2 are supported, based on configuration and subject to conditions, e.g., the serving cell measurement performed by MR is above or below a threshold - Option 1: MR does not perform serving cell measurement - Option 2: MR performs serving cell measurement with relaxation

## [22]	R1-2406815	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

Proposal 1: Consider candidate maximum LPWUS size supported for the LPWUS design excluding CRC for the idle mode/inactive mode: - to support PEI functionality - FFS 12, 16, 41 bits and other sizes. Proposal 2: Consider LPWUS content which includes wake up bit and continue to discuss whether to include Y bits for subgroup size, X bits cell id/part of cell id etc., Proposal 3: RAN1 can wait for the RAN2 decision to consider whether to include SI change and ETWS/CMAS information, tracking area information, and RAN area information. Proposal 4: Consider 12 PRBs for 30kHz SCS, 24PRBs for 15 kHz SCS for LPWUS and LPSS. Proposal 5: Continue studying In-band Vs out-of-band deployment options for LPWUR due to: - Better coverage at lower frequency. - Increased payload size due to less repetition needed to attain MR Msg-3 coverage thereby supporting more functionality at LPWUR. - Not supporting RRM measurement offloading from MR. Proposal 6: For the case, subgroup id is transmitted in LPWUS, consider upto [8] bits as subgroup id for evaluation until RAN2 makes progress on the subgrouping. Proposal 7: Continue to evaluate whether multiple FDMed LPWUS blocks. Proposal 8: Consider ramp up time of different MR sleep state to calculate the time gap between LPWUS reception and PO occasion. Proposal 9: Consider one LPWUS occasion associated with atleast one PO.

## [23]	R1-2406851	LP-WUS operation in IDLE/INACTIVE modes	Apple

Observation 1: Supporting MR waking up from deep sleep state has much less stringent requirement on MR RRM measurement offloading/relaxation to achieve UE power saving, and has much shorter wake-up delay compared to ultra-deep sleep state. Proposal 1: MR waking up from deep sleep state, in addition to ultra-deep sleep state, is supported, via supporting smaller value(s) for the offset between LP-WUS monitoring and PO. Proposal 2: UE reports one value for the supported minimum wake-up delay. - FFS the value set - FFS whether a UE can report multiple values (corresponding to different sleep states) Proposal 3: Do not support dynamic PO. Proposal 4: For iDRX mode, the periodicity of LO is the same as the iDRX cycle. Observation 2: For LO-to-PO mapping, Option 2 (one LO mapped to multiple POs) results in higher overhead than Option 1 (one LO mapped to one PO). Proposal 5: For the mapping between LO and PO, support Option 1 and Option 3. - Option 1: UEs monitoring the same PO monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Each UE monitors one LO. Observation 3: Whether to support K>1 for LO partly depends on whether subgroup ID/codepoint is carried in LP-WUS. Proposal 6: For multi-beam support of LP-WUS monitoring, the LP WUS MO(s) are defined in a similar way as for paging occasions and PEI occasions. Observation 4: The maximum number of subgroups per PO to support can be derived based on the group paging rate that is envisioned for typical high paging load scenario and the target subgroup paging rate (1% ~3% preferred). Proposal 7: Support more than 8 subgroups per PO. FFS the exact number of subgroups (e.g. tens of subgroups). Proposal 8: Support independent subgroup ID for LP-WUS and PEI. Proposal 9: Send an LS to RAN2 including the following statement to facilitate the determination of entry/exit conditions for LP-WUS monitoring in RAN1: - “As shown in the UE power saving evaluation results in TR 38.869 Section 8.1.1.5, assuming MR enters ultra-deep sleep state, UE power saving gain can be achieved only if MR RRM measurements (including both serving cell and neighbor cell measurements) is significantly relaxed. This should be taken into consideration when defining the entry/exit conditions for LP-WUS monitoring.” Proposal 10: Support LR and MR operating on the same carrier as the baseline. LR and MR operating on contiguous carriers in the same band can be supported if not much additional effort is needed. Other cases FFS.

## [24]	R1-2406882	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

Proposal 1: The periodicity of LO monitoring cycle can be the same as the PO monitoring cycle. Proposal 2: Support the number of K for MOs in a LO can be larger than 1. Proposal 3: Support time repetition and/or FDM of LP-WUS MO in a LO. Proposal 4: Discuss the construction of LO before further discussion on the association between LO and PO. Proposal 5: Down-select for the LO’s configuration Option 1: Configuration of LO is associated with configuration of PO Option 2: Configuration of LO is NOT associated with configuration of PO. Proposal 6: It is not recommended to include other information in LP-WUS for UE in idle/inactive mode. Proposal 7: Consider both MR measurement and LR measurement for entry conditions if LP-WUS and channels for MR are not in the same carrier or band. Proposal 8: Confirm the working assumption of using the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ for OFDM-based LP-WUR.

## [25]	R1-2406942	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

Proposal 1: - At least 400ms/800ms wake-up delay should be supported by LP-WUS capable UE in IDLE/INACTIVE modes - FFS: 400ms and/or 800ms is(are) supported Proposal 2: - 30ms/10ms and more than 400ms/800ms wake-up delay should be optionally supported by LP-WUS capable UE in IDLE/INACTIVE modes Proposal 3: - At least, option 1 should be supported - Option 1: UEs monitoring the same PO monitor the same LO - Support at least one of the followings: - Different LP-WUS information in a LO with option 1 - Option 1: UEs monitoring the same PO monitor the same LO - Same LP-WUS information in a LO with option 3 - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO Proposal 4: - Whether coverage enhancement is needed or not should be considered together with the signal design in A.I. 9.6.1 Proposal 5: - The same beam as SSB should be used for LP-WUS and LP-SS in IDLE/INACTIVE mode - Regarding LP-WUS, if a beam for LP-WUS is transmitted in a MO, each MO should be associated with the beam index of SSB - Regarding LP-SS, it should be further discussed how to associate with beam index between LP-SS and SSB Proposal 6: - Further consider independent LP-WUS subgrouping from PEI subgrouping Proposal 7: - Do not support dynamic PO Proposal 8: - Support only duty cycle monitoring for LP-WUS Proposal 9: - RAN1 firstly discuss LP-WUS operation with I-DRX before eDRX Proposal 10: - If UEs subgrouping can not be divided based on paging rate UE capabilities, bitmap indication should be supported for indicating subgroup information using LP-WUS Proposal 11: - If codepoint-based is supported for indicating subgroup information using LP-WUS, the codepoint value enabling to wake-up all subgroups should be supported - E.g., Wake-up indication for all subgroups via single/multiple codepoint value(s) Proposal 12: For idle/inactive mode, RAN1 does not discuss further carrying additional information other than subgroup-based wake-up indication and ETWS/CMAS notification - If entry/exit procedure for LP-WUS monitoring can ensure that paging misdetection performance is not impacted by LP-WUS detection, ETWS/CMAS notification should not be conveyed by LP-WUS payload - Note: LP-SS may or may not carry the cell ID information, and LP-SS or other sequence generation may or may not be associated with PCIs Proposal 13: - Confirm the previous working assumption, i.e., - From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB - Note: This may be revisited based on the RAN2/RAN4 discussion. Proposal 14: - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - In addition, if enough power saving gain is achieved, the serving cell measurement performed by the MR with relaxation can be considered for exit condition Proposal 15: - The accuracy of serving cell measurement by LR should be further considered (up to RAN2/RAN4) Proposal 16: - If SNR of when LP-WUS monitoring is enabled is better than SNR of when serving cell measurement by LR is enabled, entry/exit condition for serving cell measurement by LR should be supported Proposal 17: - For RRM metric based on OFDM-based LP-WUR, - No need to introduce LP-SSS-RSRP and LP-SSS-RSRQ in the spec - Enhancement to legacy SS-RSRP/SS-RSRQ of following can be considered - The measurement time resource(s) for SS-RSRP/RSSI are confined within a duration, e.g., within the window that the OFDM-based LP-WUR receiver is activated - The additional use of PBCH DMRS and CSI-RS for SS-RSRP determination is not applicable for OFDM-based LP-WUR - If SS-RSRP or SS-RSRQ is measured by OFDM-based LP-WUR, following is applicable, - RRC_IDLE intra-frequency - RRC_INACTIVE intra-frequency

## [26]	R1-2407041	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

Observation 1: In order to associate multiple POs with LO, a long LP-WUS may be needed which has the following issues: - Network cannot align the LP-WUS beam sweeping pattern with existing SSB beam sweeping pattern - Longer detection time of the LP-WUS consumes higher power. Observation 2: It is beneficial to allow UE to report its minimum supported time offset between LO and MR wakeup. Observation 3: Dynamic PO requires N×Ns×M times higher average data rate of the LP-WUS, where N is the number of paging frames (PF) in the paging cycle, Ns is the number of POs for a PF and M is the number of LOs before each PO. This is challenging for the LP-WUS design given the limited spectral efficiency that can be achieved by OOK. Observation 4: Independent subgrouping between LP-WUS and PEI improves the subgrouping granularity. Observation 5: For the UE to monitor PEI after receiving LP-WUS indicating a wakeup, the time offset between LO and the PEI occasion should be no smaller than the minimum MR wakeup delay supported by the UE. Proposal 1: The LO contains a three-tier structure of LP-WUS MOs for beam sweeping, time domain repetition and transmission of different LP-WUSs in different MOs associated with different UE subgroups. Proposal 2: In the agreement “Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam”, support that K can be larger than or equal to 1 and K = X×L where X is the number of LP-WUS repetitions in same beam and L is the number of LP-WUS occasions for different sets of LP-WUSs. Proposal 3: LP-WUS occasion is a set of N×X×L consecutive LP-WUS monitoring occasions where N is the number of beams, L is the number of LP-WUS occasions associated with different sets of LP-WUSs and X is the number of LP-WUS repetitions. The [x×N×L+l×N+m]th LP-WUS monitoring occasion in the LO corresponds to the mth beam for the lth set of LP-WUSs and the xth repetition, where x=0,1,…,X-1, l=0,1,…,L-1 and m=1,2,…,N. The LP-WUS monitoring occasions which do not overlap with UL symbols determined according to tdd-UL-DL-ConfigCommon are sequentially numbered from zero starting from the first LP-WUS monitoring occasion in the LO. When the UE detects a LP-WUS associated with its UE subgroup, the UE is not required to monitor subsequent LP-WUS monitoring occasions in this LO. Proposal 4: Periodicity of the LO for a UE in idle/inactive mode is the same as the iDRX cycle. Proposal 5: Support one LO is associated with one PO - LP-WUS MOs in the LO can be associated with different sets of UE subgroups of the PO. Proposal 6: Support 8 UE subgroups per PO for idle and inactive mode LP-WUS design. Proposal 7: For idle/inactive modes, the maximum number of UE subgroups associated with a LP-WUS monitoring is 8. Proposal 8: Do not consider dynamic PO in Rel-19 idle/inactive mode LP-WUS design. Proposal 9: Support the UE monitoring PEI after receiving LP-WUS indicating wakeup to enhance the UE subgrouping granularity. Proposal 10: Confirm the RAN1 #116 working assumption for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode. No other condition is considered in RAN1 for LP-WUS monitoring entry/exit condition. Proposal 11: RSRP and RSRQ measured by the MR and LP-RSRP and LP-RSRQ measured by the LR are used as the serving cell measurement metrics for LP-WUS monitoring entry/exit conditions. Proposal 12: Confirm the RAN1 #117 working assumption on the RRM measurement metrics based on SSS for OFDM-based LP-WUR.

## [27]	R1-2407060	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

Observation 1 Dynamic PO does not provide any meaningful latency gain but can lead to large impacts on specifications and implementations. Observation 2 From an eco-system support point of view, it would be beneficial to allow NW to configure the type of WUR capabilities which are supported in a cell. Observation 3 Using LP-WUS together with very long DRX duty cycle may have little power saving gain but negative paging latency impact. Observation 4 Segmentation of MOs in a LO (based on UE subgroup index) may be needed if LP-WUS operation with higher paging load and large number of subgroups per PO is to be supported. Based on the discussion in the previous sections we propose the following: Proposal 1 For LP-WUR operation in RRC-IDLE and INACTIVE, LP-WUS is used to trigger paging monitoring of the UE, i.e., monitoring of DCI format 1_0 with CRC scrambled by P-RNTI. Proposal 2 Do not support UE monitoring dynamic PO after receiving LP-WUS indicating wake-up. Proposal 3 Paging misdetection performance of the UE should not be impacted when LP-WUS is used by the UE for power savings. Proposal 4 LP-WUS and LP-SS configuration is provided to the UE in a cell by system information. Further details of higher layer mechanisms can be discussed in RAN 2. Proposal 5 Type of WUR capability supported in the cell, e.g., support for both OFDM and OOK-based WUR, or OFDM-based WUR only, or OOK-based WUR only, should be configurable by the NW. Proposal 6 It should be possible for NW to configure flexibly the placement of LP-WUS and LP-SS resources in frequency and time to minimize overhead and NW energy efficiency impact. Proposal 7 For LP-WUS monitoring in RRC-IDLE/INACTIVE, a. LO periodicity is the same as the DRX/paging cycle. b. Introduce time offset between LP-WUS occasion (LO) and UE paging occasion/frame. c. The network configures a single time offset value that is applicable to all UEs operating with LP-WUS. The time offset can be selected by the network from a candidate set. Minimum value of the time offset in the candidate set is FFS. d. LP-WUS monitoring occasion is determined from a starting symbol and duration of possible LP-WUS transmission within the LO. Proposal 8 For LP-WUS monitoring in RRC-IDLE/INACTIVE, each LO consists of N*K LP-WUS MOs, where K can be larger than or equal to 1. For K>1, the information in the LP-WUS, if transmitted in these K MOs can be the same or different. Proposal 9 Support only configuration of duty-cycled WUR monitoring in Rel-19. Proposal 10 For idle/inactive mode, one LP-WUS LO corresponds to NPOWUS POs where each PO contains NWUS-SGPO UE subgroups. The values NPOWUS and NWUS-SGPO are configured by the NW and can be larger than or equal to 1. Proposal 11 For idle/inactive mode, the maximum number of LP-WUS information bits (excluding CRC) is 8 bits. Proposal 12 When monitoring for LP-WUS in an MO, the UE looks for codepoint(s) or bitmap carried in the LP-WUS payload and triggers a subsequent paging monitoring if its subgroup is indicated by the codepoint(s) or in the bitmap. Proposal 13 Entry/exit conditions should be configurable, e.g., as part of the LP-WUS configuration. Proposal 14 It should be possible to configure different entry/exit conditions based on supported WUR types, if any. Proposal 15 The entry condition is based at least on serving cell measurements performed by MR and LR. The exit condition is based at least on serving cell measurements performed by LR. Proposal 16 If serving cell RRM using LP-SS is supported, only scenarios that are suitable for operation with low LP-SS overhead and NW energy impact should be considered for RRM. Proposal 17 Confirm the working assumption on reusing the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ.

## [28]	R1-2407135	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

Proposal-1: For LP-WUS monitoring - In iDRX, for LO, K=1 is sufficient. - Periodicity of LO equal to iDRX periodicity is supported. - FFS: LO periodicity different to periodicity of iDRX. - eDRX for MR is supported. eDRX for LR is not supported. Proposal-2: Support “dynamic PTW”, at least for the case where MR is configured with eDRX while LOs are configured with periodicity < eDRX periodicity. Proposal-3: One-to-one mapping between LP-WUS MO and PO (Option 1) is supported. The need for other options should be discussed further. Proposal-4: In Option 1, for UEs belonging to the same paging group/subgroup and monitoring the same LO, the first monitored PO after LR receives LP-WUS can be different, is determined based on UE-reported wake-up time capability. Proposal-5: The maximum number of information bits (excluding CRC) in a LP-WUS is 8.