3GPP TSG-RAN WG1 #64	R1-110930

February 21st – February 25th, 2011

Taipei, Taiwan

Agenda item:	6.2.6

Source: 		        Qualcomm Incorporated

Title: 	Text proposal for resource specific CSI specification in 36.213

Document for:	Discussion/Decision

# 1		Introduction

RAN 1 #63 agreed in [1] on the details of the cell state information (CSI) measurement and feedback restriction to a signalled set of resources. The agreement is given below:

Agreement:

- CSI (CQI,PMI,RI) feedback based on interference measurement in restricted subsets of subframes is enabled through configured subsets of subframes indicated by CSI measurement subframe configuration

  - Subframe subsets are signalled by RRC (e.g. with bitmaps of size matching the size of almost blank subframe pattern)

    - 0 or 2 subframe subsets can be configured per UE

      - Baseline is that the UE only reports CSI for each configured subframe subset

      - If 0 subframe subsets are configured, this whole proposal does not apply.

    - The 2 subframe subsets may or may not be the complement of each other.

    - Signalling details are up to RAN2.

    - No mention of complementary subset in RAN1:  whether to introduce this is subject to feedback from RAN4 after discussion in RAN4.

- For aperiodic CSI reporting, working assumption is:

  - For CSI reporting instance at subframe n, UE shall report CSI feedback based on the subset containing the CQI reference resource

    - As noted above, the case of the CQI reference resource not being contained in either of the configured subsets is FFS subject to feedback from RAN4 after discussion in RAN4

- For periodic CSI reporting

  - Explicit RRC configuration links each CSI feedback to a configured subset

In addition, RAN #63bis [2] agreed that:

Agreement:

eNB can configure either 1 or 2 sets of periodic reporting parameters (periodicity, offset)

Collision handling:

    - No specified collision handling method; avoidance of collisions is left to the eNB; UE behaviour is undefined if collision occurs

In this contribution, we provide the text proposal for TS36.213 [3] that accounts for CSI measurement subframe configuration restriction signalled to the UE. The text proposal also accounts for the CSI collision handling method described in [4], where it is proposed to avoid CSI collisions by configuration.

# 2		Text proposal for TS36.213

Based on the agreement from in [1] and the proposal for CSI collision handling from [3], we propose the following changes to TS 36213 to capture interference averaging restriction for CSI (CQI, PMI, RI) and implicit and explicit linking for a periodic and periodic reporting, respectively.

### 7.2.2	Periodic CQI/PMI/RI Reporting using PUCCH

…

The periodicity  (in subframes) and offset  (in subframes) for CQI/PMI reporting are determined based on the parameter cqi-pmi-ConfigIndex () given in Table 7.2.2-1A for FDD and Table 7.2.2-1C for TDD.  The periodicity  and relative offset   for RI reporting are determined based on the parameter ri-ConfigIndex () given in Table 7.2.2-1B. Both cqi-pmi-ConfigIndex and ri-ConfigIndex are configured by higher layer signalling.  The relative reporting offset for RI  takes values from the set .

If more than one CSI measurement subframe subset for CSI (CQI/PMI/RI) reporting is configured by upper layers,  and are determined for each CSI report, linked to the  corresponding subframe subset based on the parameters cqi-pmi-ConfigIndex and ri-ConfigIndex, respectively.

….

For the calculation of CQI/PMI conditioned on the last reported RI, in the absence of a last reported RI the UE shall conduct the CQI/PMI calculation conditioned on the lowest possible RI as given by the bitmap parameter codebookSubsetRestriction. If more than one subframe subset for CSI (CQI/PMI/RI) reporting is configured by upper layers, CQI/PMI is conditioned on the last reported RI linked to the same subframe subset as CQI/PMI report.

…

### 7.2.3	Channel quality indicator (CQI) definition

The CQI indices and their interpretations are given in Table 7.2.3-1.

Based on an unrestricted observation interval in time and frequency, the UE shall derive for each CQI value reported in uplink subframe n the highest CQI index between 1 and 15 in Table 7.2.3-1 which satisfies the following condition, or CQI index 0 if CQI index 1 does not satisfy the condition:

- A single PDSCH transport block with a combination of modulation scheme and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CQI reference resource, could be received with a transport block error probability not exceeding 0.1.

If CSI measurement subframe subsets are configured by higher layers, the UE shall derive CQI representative of the channel and interference conditions over the configured subset that contains the CQI reference resource. If no CSI measurement subframe subset contains the CQI measurement resource, or if CSI measurement subframe subsets are not configured by higher layers, the UE shall derive CQI representative of the channel and interference conditions over the subframe that contains the CQI reference resource. In the frequency domain, the UE shall derive CQI representative of the channel and interference conditions over the group of downlink physical resource blocks corresponding to the band to which the derived CQI value relates.

For transmission mode 9 and feedback reporting modes the UE shall derive the channel measurements for computing the CQI value reported in uplink subframe n based on only the Channel-State Information (CSI) reference signals defined in [3].  For other transmission modes and their respective reporting modes the UE shall derive the channel measurements for computing CQI based on CRS.

A combination of modulation scheme and transport block size corresponds to a CQI index if:

- the combination could be signalled for transmission on the PDSCH in the CQI reference resource according to the relevant Transport Block Size table, and

- the modulation scheme is indicated by the CQI index, and

- the combination of transport block size and modulation scheme when applied to the reference resource results in the effective channel code rate which is the closest possible to the code rate indicated by the CQI index. If more than one combination of transport block size and modulation scheme results in an effective channel code rate equally close to the code rate indicated by the CQI index, only the combination with the smallest of such transport block sizes is relevant.

The CQI reference resource is defined as follows:

- In the frequency domain, the CQI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CQI value relates.

- In the time domain, the CQI reference resource is defined by a single downlink subframe n-nCQI_ref,

  - where for periodic CQI reporting nCQI_ref  is the smallest value greater than or equal to 4, such that it corresponds to a valid downlink subframe and if CSI measurements subframe subsets are configured by higher layers, the subframe is element of the subset linked to the current CQI reporting instance.

  - where for aperiodic CQI reporting nCQI_ref  is such that the reference resource is in the same valid downlink subframe as the corresponding CQI request in an uplink DCI format.

  - where for aperiodic CQI reporting nCQI_ref  is equal to 4 and downlink subframe n-nCQI_ref corresponds to a valid downlink subframe, where downlink subframe n-nCQI_ref is received after the subframe with the corresponding CQI request in a Random Access Response Grant.

A downlink subframe shall be considered to be valid if:

    - it is configured as a downlink subframe for that UE, and

    - except for transmission mode 9, it is not an MBSFN subframe, and

    - it does not contain a DwPTS field in case the length of DwPTS is  and less, and

    - it does not fall within a configured measurement gap for that UE.

If there is no valid downlink subframe for the CQI reference resource, CQI reporting is omitted in uplink subframe n.

- In the layer domain, the CQI reference resource is defined by any RI and PMI on which the CQI is conditioned.

…

# 3		Conclusions

In this contribution, we give a text proposal for TS36.213 to account for the interference averaging restriction for CSI (CQI, PMI, RI) and implicit and explicit linking for aperiodic and periodic reporting, respectively.

# References

- 3GPP RAN1 #63, Chairman’s notes – final, Jacksonville, Florida, November 2010.

- 3GPP RAN1 #63bis, Chairman’s notes – final, Dublin, Ireland, January 2011.

- 3GPP TS36.213, Physical layer procedures (Release 10), V10.0.1 (2010-12).

- R1-110931 Remaining aspects for resource specific CSI reporting, Qualcomm Incorporated.