3GPP TSG-RAN WG1 #68	R1-120634

Dresden, Germany, 6 – 10 February 2012

Source:	MediaTek Inc.

Title:	Analysis of reduction in transmit power

Agenda Item: 7.7.4

Document for: 	Discussion

# Introduction

Motivated by the goal of avoiding, in the future, the need to maintain a separate GSM/GPRS network just for MTC devices, the study item of provisioning of low-cost MTC UEs based on LTE was proposed and approved [1]. The most important factor for the business success of LTE based MTC is obviously cost if satisfactory coverage & power consumption can be ensured as well.

In the last RAN1 #67 meeting, it was agreed that the reference for the cost comparison for the low cost MTC device will be a single band, single RAT, Cat-1 UE, and operating on a 20 MHz carrier. Also, the following techniques have been identified for further analysis:

- Reduction of maximum bandwidth

- Single receive RF chain

- Reduction of peak rate

- Reduction of transmit power

- Half duplex operation

A tabulated analysis of most of the proposed techniques can be found in [4]. In this contribution, we present more detailed analysis on reduction in transmit power. We follow the structure of the TR section 6 so that most of the text in all the sections can be considered as text proposal.

---------------------------Text Proposal Start (other than text in brackets) ------------------------------------

# Description

Reduction of transmit power is a technique aimed to lower or remove the cost of PA. It is expected that this technique will have much of the impact on UL performance and coverage, with some cost benefit.

# Analysis/evaluation of performance against requirements

An analysis of the technique against system requirements is provided in this section. The analysis is to be used in combination with the cost analysis:

Table 1. Impact of redcued transmit power

| Metric | Impact (Yes/No) |
| --- | --- |
| Coverage same as GSM/EGPRS [and legacy LTE] | Yes |
| Minimum Data rate | No |
| Power consumption | Yes |
| Impact to non-MTC UE | No |
| eNB Hardware impact | Yes |
| Impact on specification | No |
| Cell spectral efficiency | Yes |
| ….. |  |
| …… |  |

[Editor’s Note: Whilst the Low cost MTC UE based on LTE is required to meet all the requirements, a particular requirement may not be applicable to an identified technique. Evaluation/analysis of impact (positive/negative) to be provided below for only for the requirement’s that has an impact (indicated by “Yes” above in the table). Below shown are example placeholders for some analysis/evaluation of some of the requirements]

# Coverage Analysis

Note the coverage requirement in [1],

- Ensure that service coverage is not worse than GSM/GPRS, at least comparable and preferably improved beyond what is possible for providing MTC services over GPRS/GSM today (assuming deployment in the same spectrum bands). The same defined LTE cell coverage footprint as engineered for “normal LTE UEs” should apply for low-cost MTC UEs.

It is most likely that the above requirement will not be satisfied if the maximal transmission power is allowed to be reduced significantly.

# Power Consumption

Requirement for power consumption in [1] states:

- Ensure that overall power consumption is no worse than existing GSM/GPRS based MTC devices.

Power consumption is not proportional to the PA output power because of fixed DC current drain.  Only if a PA can be removed or its DC current drain can be significantly reduced, the power consumption reduction is probably minimal.

# Analysis/evaluation of cost reduction

Reducing PA power may have very limited cost saving, unless we can completely remove the PA, which will be a max of 10% of total saving based on the fact that PA accounts for 25% of RF cost that is 40% of total cost [3]. For the envisioned system operation with direct eNB-to-UE access, it is unlikely that we can remove PA and still meet the coverage requirements.

---------------------------Text Proposal Ends ------------------------------------

# Conclusion

In this contribution, we present more detailed analysis on reduced transit power.

We propose:

- Consider text in all the sections as text proposal

- Conclude that :

  - Reducing PA power may have very limited cost saving, unless we can completely remove the PA, which will be a max of 10% of total saving based on the fact that PA accounts for 25% of RF cost that is 40% of total cost [3]. For the envisioned system operation with direct eNB-to-UE access, it is unlikely that we can remove PA and still meet the coverage requirements.

# References

- R1-111112, Study Item “Provision of low-cost MTC UEs based on LTE”

- R1-113616, “TR36.888 - Study on provision of low-cost MTC UEs based on LTE”

- R1-120636, “Cost Drivers of LTE modem and Text Proposal”, MediaTek, RAN1#68

- R1-113867, “LTE MTC Device Cost Reduction Measures with Cost Analysis”, MediaTek, RAN1#67

- R1-120636, “Analysis of reduced maximum bandwidth”, MediaTek, RAN1#68