3GPP TSG-RAN WG1 #75	R1-135330

November 11 – 15, 2013

San Francisco, CA, USA

Agenda item:	6.2.10.1

Source: 	Qualcomm Incorporated

Title: 	System Level Modeling Methodology for NAIC

Document for:	Discussion

# Introduction

For system evaluation for NAIC, it is important to model the performance of the advanced receiver. This contribution describes a link to system model for evaluation of the system level throughput gains of network assisted data interference cancelation for a symbol level interference cancellation (SLIC) receiver [1]. Although the example is shown with SLIC receiver in mind, the general methodology can be also applied to other receiver types.

Section 2 gives a detailed description of the model.  Section 3 presents the link-level validation of the accuracy of the proposed model. Section 4 concludes the contribution. Finally, in the appendix, we provide the detailed LUT for the parameters for system level simulations.

# Link to System Model Overview

Let the number of simultaneously transmitting cells be N, including the serving cell. The received signal is given by the superposition of all the N transmitted signals:

|  | yk= i=1NβiHi,kPixi,k +nk, for k=0,1,…, K-1, |  |
| --- | --- | --- |

where, βi is the traffic to pilot ratio of the signal transmitted from ith cell, Hi,k is the channel matrix of the ith cell on the kth tone / resource element (RE), xi,k  is the symbol transmitted by the ith cell on the kth tone and Pi is the spatial precoding matrix used by the ith cell and K is the total number of observed tones. The number of cells in this case is N with one serving cell and N-1 interferers.

Without loss of generality, assume that cell 1 is the serving cell and the UE attempts to cancel the data transmission of cell 2. We first focus on the single interferer case and define the quantity zk= i=3N-1βiHi,kPixi,k +nk. Then the above equation can be rewritten as

|  | yk= β1H1,kP1x1,k + β2H2,kP2x2,k +zk, for k=0,1,…, K-1. |  |
| --- | --- | --- |

As a first step in data cancelation, a SLIC receiver obtains estimates of the channel H2,k, the traffic to pilot ratio β2, the precoder/spatial scheme P2 , and the interferer’s symbol x2,k . Let ek denote the residual error after cancelation,

|  | ek=β2H2,kP2x2,k -β2H2,kP2x2,k , for k=0,1,…, K-1. |  |
| --- | --- | --- |

The received symbol post cancelation can be written as

|  | yk=ρ1H1,kP1x1,k +ek+zk , for k=0,1,…, K-1. |  |
| --- | --- | --- |

In a system simulation, our model approximates the residual error by a scaled value of the signal being cancelled as follows:

|  | ek=αβ2H2,kP2x2,k , for k=0,1,…, K-1. |  |
| --- | --- | --- |

After the SLIC modelling, the equivalent received signal as in (4) is further processed by the standard MMSE receiver to produce the decoding statistics.

In the next section we describe how a lookup table for  α is computed using link level simulations.

## Computation of lookup tables for parameters using link level simulations

Let α be a scalar quantity that denotes the quality of cancelation. It is defined as

|  | α= Trace(ek ekH)Traceβ2H2,kP2x2,k β2H2,kP2x2,k H , |  |
| --- | --- | --- |

where the summation is over all tones k in an RB. We call α the interference suppression factor. As an example, a value of α= -10  dB for a given RB denotes that the interference from cell 2 is suppressed by 10dB on average on that RB.

In general, the interference suppression factor is a random variable. The distribution of  α will have a different mean and variance when all detectors are correct than when one or more detectors are in error. Thus alpha will have a multimodal distribution. When the blind detectors are correct, we expect interference cancelation to be accurate, i.e. α should be a small quantity, and when the detectors are incorrect, the interference cancelation will be inaccurate, i.e. α should be a large quantity and may also exceed 1.

For simplicity, we consider α to have a bi-modal distribution. We will say that α is in Mode 0 when the spatial scheme and precoding matrix has been detected accurately and in mode 1 otherwise. In general, a more elaborate multi-mode modelling can be used. Our simulation results show that a bi-modal model is sufficient.

In a single interferer link level simulation, we log the value of the interference suppression factor, computed per RB, and across multiple fading channel realizations as a function of the following five parameters.

- Interferer long term signal to noise ratio I/Noc

- Interferer modulation order

- Interferer rank

- Interferer transmission mode/ spatial scheme

- Serving long term signal to noise ratio Es/Noc

For each choice of the above five parameters, we compute

- the quantity α0 , defined as the mean value of α (in the linear domain) given that α is in Mode 0,  by averaging the α  values across multiple RBs and subframes (note that we log one value of α per RB), and

- the probability p0 that α is in Mode 0, by counting across multiple RBs and subframes. As a special case, if network assistance reveals to the UE the spatial scheme and the precoding matrix used by the interferer on RBs on which cancelation is attempted, then p0=1.

The quantity α1, defined as the mean value of α (in the linear domain) given that α is in Mode 1, is fixed to be 1. In other words, when the detection of the spatial scheme and precoding matrix fails, our model assumes that no cancelation is performed. Using single interferer link level simulations we build a five dimensional lookup table ( I/Noc  and Es/Noc are quantized in 3dB increments) for the pair of quantities(α0, po). Although the description in this paper was for the single interferer case, this method could be extended to multiple interferer scenarios by using a higher dimensional lookup table to incorporate parameters of the additional interferer(s).

# Model validation

In this section, we present link level results to illustrate the accuracy of the proposed model. We consider a closed loop, 50RB simulation with wideband CQI, PMI feedback. We perform symbol level cancelation (SLIC) on a single rank 1, QPSK TM4 interferer at I/Noc=16dB . The serving cell transmission mode is TM4, and rank adaptation is enabled. The channel model is ETU5 for both the serving and interfering cells. The following figure shows the closed loop throughput obtained with (a) SLIC (b) SLIC approximation using the model described above, and (c) with no interference cancelation.

Figure 1: Model validation using a 16dB TM4 rank 1 QPSK interferer

# Conclusion

In this contribution we described a link to system model for data interference cancelation. In this model, link level simulations were used to compute lookup tables for a quantity we call the interference suppression factor. We validated the accuracy of this model using link level simulations. In addition, we provided the LUT in the appendix for further system level simulations.

Based on the discussions, we make the following proposal:

Proposal:

- Adopt the described link to system mapping methodology for NAIC system evaluations.

# References

RP-130404, “Network Assisted Interference Cancellation and Suppression for LTE”, MediaTek, Renesas, Broadcom.

# Appendix

The detailed parameters for link to system mapping are provided in the Table 1 and Table 2 for ETU-5 and EPA-5 respectively. Some additional notes for the LUT below:

- For a UE, compute C/N and I/N, where C is the serving cell RSRP, I is the RSRP of the dominant interferer sending data, and N is the sum of all RSRP of all interferers sending data

- Compute the probability that the detector fails (using the above three dimensional tables in C/N, I/N, and interferer MCS) and using bilinear interpolation. Our simulations show the detector (for spatial scheme) pass fail probability is not sensitive to MCS of the interferer, so we provide failure probabilities only for QPSK.

- Determine probabilistically if the detector (for spatial scheme) has succeeded on the given RB. If it fails, assume no cancelation.

- If the detector succeeds, assume cancelation with \alpha (suppression value) value given by the tables above (use bilinear interpolation if necessary)

- The parameter alpha, models inaccuracies in TPR and modulation order estimation, so the detector failure probability is referring only to the blind detection of the spatial scheme.

Table 1. Parameters for ETU-5

| C/N | alpha[dB] | C/N | alpha[dB] | C/N | alpha[dB] | Pfail |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK, I/N =0dB |  | 16QAM, I/N =0dB |  | 64QAM, I/N =0dB |  |  |
| 0 | -1.829 | 0 | -1.539 | 0 | -1.49 | 0.5502 |
| 4 | -1.342 | 4 | -1.136 | 4 | -1.066 | 0.6418 |
| 8 | -1.026 | 8 | -0.886 | 8 | -0.792 | 0.736 |
| 12 | -0.777 | 12 | -0.666 | 12 | -0.643 | 0.849 |
| 14 | -0.777 | 14 | -0.601 | 14 | -0.68 | 0.8908 |
| 16 | -0.677 | 16 | -0.601 | 16 | -0.574 | 0.9278 |
| 18 | -0.677 | 18 | -0.601 | 18 | -0.591 | 0.9556 |
| 20 | -0.677 | 20 | -0.4 | 20 | -0.46 | 0.9736 |
| 24 | -0.677 | 24 | -0.4 | 24 | -0.46 | 0.9962 |
| QPSK, I/N =3dB |  | 16QAM, I/N =3dB |  | 64QAM, I/N =3dB |  |  |
| 0 | -3.818 | 0 | -3.17 | 0 | -3.104 | 0.3544 |
| 4 | -3.07 | 4 | -2.586 | 4 | -2.527 | 0.4502 |
| 8 | -2.41 | 8 | -2.048 | 8 | -2.005 | 0.5802 |
| 12 | -1.84 | 12 | -1.554 | 12 | -1.527 | 0.7124 |
| 14 | -1.659 | 14 | -1.352 | 14 | -1.376 | 0.7938 |
| 16 | -1.489 | 16 | -1.24 | 16 | -1.237 | 0.8498 |
| 18 | -1.182 | 18 | -1.011 | 18 | -1.028 | 0.9116 |
| 20 | -1.05 | 20 | -0.972 | 20 | -0.899 | 0.9414 |
| 24 | -0.887 | 24 | -0.972 | 24 | -0.899 | 0.9838 |
| QPSK I/N =6dB |  | 16QAM I/N =6dB |  | 64QAM I/N =6dB |  |  |
| 0 | -6.723 | 0 | -5.443 | 0 | -5.282 | 0.2008 |
| 4 | -5.614 | 4 | -4.591 | 4 | -4.486 | 0.2734 |
| 8 | -4.525 | 8 | -3.774 | 8 | -3.652 | 0.385 |
| 12 | -3.497 | 12 | -2.927 | 12 | -2.863 | 0.5364 |
| 14 | -3.06 | 14 | -2.604 | 14 | -2.557 | 0.6276 |
| 16 | -2.661 | 16 | -2.337 | 16 | -2.279 | 0.7266 |
| 18 | -2.377 | 18 | -2.041 | 18 | -2.008 | 0.8134 |
| 20 | -2.241 | 20 | -1.953 | 20 | -1.921 | 0.8644 |
| 24 | -1.448 | 24 | -1.435 | 24 | -1.264 | 0.962 |
| 28 | -1.237 | 28 | -1.287 | 28 | -1.231 | 0.9902 |
| QPSK, I/N =9dB |  | 16QAM I/N =9dB |  | 64QAM I/N =9dB |  |  |
| 0 | -10.292 | 0 | -8.056 | 0 | -7.717 | 0.1038 |
| 4 | -8.823 | 4 | -6.996 | 4 | -6.736 | 0.1448 |
| 8 | -7.28 | 8 | -5.862 | 8 | -5.7 | 0.2158 |
| 12 | -5.835 | 12 | -4.712 | 12 | -4.633 | 0.342 |
| 14 | -4.938 | 14 | -4.111 | 14 | -4.05 | 0.4254 |
| 16 | -4.23 | 16 | -3.607 | 16 | -3.505 | 0.5222 |
| 18 | -3.702 | 18 | -3.152 | 18 | -3.032 | 0.6254 |
| 20 | -3.445 | 20 | -2.847 | 20 | -2.801 | 0.7334 |
| 24 | -2.709 | 24 | -2.361 | 24 | -2.187 | 0.9032 |
| 28 | -1.827 | 28 | -1.446 | 28 | -1.411 | 0.9762 |
| QPSK 12dB |  | 16QAM 12dB |  | 64QAM 12dB |  |  |
| 0 | -14.32 | 0 | -11.085 | 0 | -10.287 | 0.0454 |
| 4 | -12.648 | 4 | -9.771 | 4 | -9.206 | 0.0662 |
| 8 | -10.654 | 8 | -8.346 | 8 | -7.976 | 0.107 |
| 12 | -8.578 | 12 | -6.888 | 12 | -6.603 | 0.1872 |
| 14 | -7.585 | 14 | -6.018 | 14 | -5.895 | 0.2484 |
| 16 | -6.469 | 16 | -5.288 | 16 | -5.068 | 0.326 |
| 18 | -5.416 | 18 | -4.476 | 18 | -4.424 | 0.4306 |
| 20 | -4.703 | 20 | -3.918 | 20 | -3.811 | 0.5372 |
| 24 | -3.622 | 24 | -3.131 | 24 | -2.927 | 0.7826 |
| 28 | -2.839 | 28 | -2.432 | 28 | -2.388 | 0.9304 |
| QPSK 16dB |  | 16QAM 16dB |  | 64QAM 16dB |  |  |
| 0 | -19.537 | 0 | -15.826 | 0 | -14.011 | 0.018 |
| 4 | -18.197 | 4 | -14.199 | 4 | -12.743 | 0.0224 |
| 8 | -15.878 | 8 | -12.358 | 8 | -11.308 | 0.0346 |
| 12 | -13.105 | 12 | -10.311 | 12 | -9.674 | 0.0712 |
| 14 | -11.723 | 14 | -9.283 | 14 | -8.775 | 0.0972 |
| 16 | -10.345 | 16 | -8.166 | 16 | -7.847 | 0.1434 |
| 18 | -8.892 | 18 | -7.053 | 18 | -6.768 | 0.2034 |
| 20 | -7.436 | 20 | -6.032 | 20 | -5.83 | 0.2804 |
| 24 | -5.101 | 24 | -4.325 | 24 | -4.138 | 0.5124 |
| 28 | -3.83 | 28 | -3.266 | 28 | -3.207 | 0.7714 |

Table 2. Parameters for EPA-5

| C/N | alpha[dB] | C/N | alpha[dB] | C/N | alpha[dB] | Pfail |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK, I/N = 0dB |  | 16QAM, I/N=0dB |  | 64QAM, I/N=0dB |  |  |
| 0 | -1.233 | 0 | -1 | 0 | -0.932 | 0.5666 |
| 4 | -0.869 | 4 | -0.74 | 4 | -0.663 | 0.6384 |
| 8 | -0.853 | 8 | -0.753 | 8 | -0.635 | 0.7278 |
| 12 | -0.844 | 12 | -0.768 | 12 | -0.787 | 0.8076 |
| 14 | -1.014 | 14 | -0.795 | 14 | -0.781 | 0.8332 |
| 16 | -0.917 | 16 | -0.78 | 16 | -0.756 | 0.8596 |
| 18 | -0.917 | 18 | -0.78 | 18 | -0.756 | 0.8896 |
| 20 | -0.917 | 20 | -0.78 | 20 | -0.756 | 0.9142 |
| 24 | -0.917 | 24 | -0.78 | 24 | -0.756 | 0.9626 |
| QPSK, I/N=3dB |  | 16QAM, I/N=3dB |  | 64QAM, I/N=3dB |  |  |
| 0 | -3.029 | 0 | -2.562 | 0 | -2.554 | 0.3804 |
| 4 | -2.575 | 4 | -2.185 | 4 | -2.141 | 0.4738 |
| 8 | -2.216 | 8 | -1.948 | 8 | -1.875 | 0.5748 |
| 12 | -2.02 | 12 | -1.824 | 12 | -1.674 | 0.6764 |
| 14 | -1.953 | 14 | -1.695 | 14 | -1.683 | 0.7232 |
| 16 | -1.924 | 16 | -1.662 | 16 | -1.642 | 0.7536 |
| 18 | -1.907 | 18 | -1.607 | 18 | -1.637 | 0.7878 |
| 20 | -1.938 | 20 | -1.67 | 20 | -1.655 | 0.8278 |
| 24 | -1.776 | 24 | -1.701 | 24 | -1.677 | 0.8904 |
| 28 | -1.776 | 28 | -1.701 | 28 | -1.677 | 0.9626 |
| QPSK 6dB |  | 16QAM 6dB |  | 64QAM 6dB |  |  |
| 0 | -5.618 | 0 | -4.758 | 0 | -4.608 | 0.2172 |
| 4 | -4.946 | 4 | -4.164 | 4 | -4.12 | 0.3008 |
| 8 | -4.307 | 8 | -3.721 | 8 | -3.533 | 0.4062 |
| 12 | -3.876 | 12 | -3.267 | 12 | -3.219 | 0.5248 |
| 14 | -3.736 | 14 | -3.198 | 14 | -3.001 | 0.5804 |
| 16 | -3.62 | 16 | -3.091 | 16 | -2.993 | 0.6244 |
| 18 | -3.395 | 18 | -2.981 | 18 | -2.826 | 0.669 |
| 20 | -3.222 | 20 | -2.873 | 20 | -2.763 | 0.7196 |
| 24 | -2.96 | 24 | -2.629 | 24 | -2.615 | 0.7976 |
| 28 | -2.96 | 28 | -2.629 | 28 | -2.615 | 0.886 |
| QPSK, 9dB |  | 16QAM 9dB |  | 64QAM 9dB |  |  |
| 0 | -8.783 | 0 | -7.16 | 0 | -6.966 | 0.1144 |
| 4 | -7.867 | 4 | -6.452 | 4 | -6.269 | 0.1628 |
| 8 | -6.912 | 8 | -5.736 | 8 | -5.658 | 0.2426 |
| 12 | -6.146 | 12 | -5.219 | 12 | -5.105 | 0.3444 |
| 14 | -5.978 | 14 | -5.004 | 14 | -4.917 | 0.4162 |
| 16 | -5.713 | 16 | -4.909 | 16 | -4.819 | 0.4842 |
| 18 | -5.452 | 18 | -4.633 | 18 | -4.574 | 0.5456 |
| 20 | -5.279 | 20 | -4.43 | 20 | -4.344 | 0.5958 |
| 24 | -4.887 | 24 | -4.178 | 24 | -4.008 | 0.6958 |
| 28 | -4.429 | 28 | -3.857 | 28 | -3.803 | 0.7916 |
| QPSK 12dB |  | 16QAM 12dB |  | 64QAM 12dB |  |  |
| 0 | -12.158 | 0 | -9.836 | 0 | -9.302 | 0.0534 |
| 4 | -11.013 | 4 | -8.909 | 4 | -8.502 | 0.0822 |
| 8 | -9.965 | 8 | -8.181 | 8 | -7.899 | 0.1308 |
| 12 | -8.849 | 12 | -7.486 | 12 | -7.212 | 0.2142 |
| 14 | -8.479 | 14 | -7.18 | 14 | -6.935 | 0.2668 |
| 16 | -8.082 | 16 | -6.904 | 16 | -6.764 | 0.3334 |
| 18 | -7.79 | 18 | -6.686 | 18 | -6.492 | 0.3926 |
| 20 | -7.485 | 20 | -6.526 | 20 | -6.214 | 0.459 |
| 24 | -6.915 | 24 | -5.906 | 24 | -5.747 | 0.5822 |
| 28 | -6.374 | 28 | -5.444 | 28 | -5.36 | 0.694 |
| QPSK 16dB |  | 16QAM 16dB |  | 64QAM 16dB |  |  |
| 0 | -16.776 | 0 | -14.123 | 0 | -12.866 | 0.0212 |
| 4 | -15.544 | 4 | -12.837 | 4 | -11.81 | 0.0294 |
| 8 | -13.991 | 8 | -11.683 | 8 | -10.928 | 0.0504 |
| 12 | -12.709 | 12 | -10.749 | 12 | -10.221 | 0.0942 |
| 14 | -11.939 | 14 | -10.286 | 14 | -9.841 | 0.1234 |
| 16 | -11.437 | 16 | -9.851 | 16 | -9.474 | 0.165 |
| 18 | -10.842 | 18 | -9.596 | 18 | -9.183 | 0.2198 |
| 20 | -10.619 | 20 | -9.252 | 20 | -8.884 | 0.2778 |
| 24 | -9.773 | 24 | -8.633 | 24 | -8.318 | 0.4016 |
| 28 | -9.019 | 28 | -7.812 | 28 | -7.619 | 0.543 |