3GPP TSG RAN WG1 Meeting #76bis		R1-141369

Shenzhen, China, 31st March – 4th April 2014

Source:	New Postcom

Title:	Discussion on RRC impact of TDD eIMTA

Agenda Item:	7.2.1.3

Document for:	Discussion/Decision

Introduction

According to email discussion after RAN1#76, the following open issues with potential RRC impact are identified for Rel-12 LTE_TDD_eIMTA.

- Issue 1: whether the beta_offset (r.f. TS36.213 section 8.6.3) values shall be configured separately for the two UL subframe sets?

- Issue 2: whether the PSRS_OFFSET,c(m) (r.f. TS36.213 section 5.1.3.1) values shall be configured separately for the two UL subframe sets, for both periodic SRS and aperiodic SRS?

- Issue 3: whether the cell-specific parameter srs-BandwidthConfig (r.f. TS36.211 section 5.5.3.2) values can be configured separately for the two UL subframe set?

In this contribution, we would like to discuss further these open issues and give our points on which parameters can be configured separately for the two subframe sets.

Discussion

Beta offset value

The offset values are defined for controlling the UCI coding rate which is closely releated to the channel condition. In TDD eIMTA operation, the UL subframes are classified as flexiable subframe set and fixed subframe set, and the two subframe sets can experience different level of DL interference and UL interference. It is flexible for eNB implementation to guarantee the QoS of PUSCH separately if configuring the beta offset value separately. The exiting specification signals the offset values via RRC, it needs to define new signaling mechanism for RAN2 to support such configuration.

Proposal 1: For more accuracy measurement and flexibility implementation, it is suggested to configure beta offset values for the two subframe sets separately.

SRS power control parameters

As defined by current specification, the SRS transmission power is derived from the PUSCH transmission power and the SRS offset value semi-statically configured by higher layers. As agreed in TDD eIMTA operation, both period and a-period SRS transmission are supported in flexible subframes and fixed subframes. Considering that the transmission power and interference dominated level are different between flexible subframes and fixed subframes, in order to get uplink channel quality precisely for the two subframe sets, flexibility of SRS power control parameter can provide correct UL power control operation and better link adaption. And more, as agreed in RAN1#76, the PUSCH power parameter can set separately, so the SRS should consist with the same principle.

Proposal 2: For better link adaption and flexibility implementation, it is suggested to configure SRS power offset for the two subframe sets separately.

SRS transmission parameters

SRS are configured for channel quality estimation. The density of SRS transmission can impact the resource allocation and frequency selection scheduling. Except for the SRS transmission power related parameters, there are other parameters that define the SRS transmission, including cell specific parameters (srs-BandwidthConfig, srs-SubframeConfig) and dedicated parameters ( srs-HoppingBandwidth, duration..). Definitely, flexible configuration for the two subframe sets can provide better channel quality measurement and more proper eNB scheduling. But considering that extending all these parameters to two subframe sets have large impact on current specification and the performance of legacy UEs. So it is suggested to consider further which parameters are necessary to set separately for the two subframe sets.

Proposal 3: it is suggested to consider further if SRS transmission parameters are configured for the two subframe sets separately.

Conclusions

In this contribution, we continue discussion on the open issues with potential RRC impact, from the point view of benefits for scheduling, the impact on specification and legacy UE performance, the following proposals are suggested:

Proposal 1: For more accuracy measurement and flexibility implementation, it is suggested to configure beta offset value for the two subframe sets separately.

Proposal 2: For better link adaption and flexibility implementation, it is suggested to configure SRS power offset for the two subframe sets separately.

Proposal 3: it is suggested to consider further if SRS transmission parameters are configured for the two subframe sets separately.

References

- R1-141055,  “Summary of email discussion [76-10]”

- 3GPP TS 36.213: “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedure”.