3GPP TSG RAN WG1 #80	R1-150134

February 9 – 13, 2015

Athens, Greece

Agenda item:	7.2.3.1

Source: 	Cisco Systems

Title: 	Simulation results for Coexistence of LAA and Wi-Fi

Document for:	Discussion and Decision

# Introduction

According to [1], the design targets of an LAA system are

-	A single global solution framework allowing compliance with any regional regulatory requirements

A single global solution framework for LAA should be defined to ensure that LAA can be operated according to any regional regulatory requirements. Furthermore, LAA design should provide sufficient configurability to enable efficient operation in different geographical regions.

-	Effective and fair coexistence with Wi-Fi.

The LAA design should target fair coexistence with existing Wi-Fi networks to not impact Wi-Fi services more than an additional Wi-Fi network on the same carrier, with respect to throughput and latency.

-	Effective and fair coexistence among LAA networks deployed by different operators

The LAA design should target fair coexistence among LAA networks deployed by different operators so that the LAA networks can achieve comparable performance, with respect to throughput and latency.

It has been agreed [2] that the Listen-before-talk (LBT) courtesy mechanism is an important component for LAA in order to achieve a fair co-existence not only with other Wi-Fi, but also with other LAA devices. Simulations have shown that the presence of LAA without LBT can cause significant degradation to the Wi-Fi system [3]. LBT is defined as [1]

…a mechanism by which an equipment applies a clear channel assessment (CCA) check before using the channel. The CCA utilizes at least energy detection to determine the presence or absence of other signals on a channel in order to determine if a channel is occupied or clear, respectively. European and Japanese regulations mandate the usage of LBT in the unlicensed bands. Apart from regulatory requirements, carrier sensing via LBT is one way for fair sharing of the unlicensed spectrum and hence it is considered to be a vital feature for fair and friendly operation in the unlicensed spectrum in a single global solution framework.

As specified in the European regulations EN 301.893 [4], the LBT mechanism can be divided into two types: Frame Based Equipment (FBE) and Load Based Equipment (LBE). In FBE, the transmit/receive structure is not based on the instantaneous traffic. Instead, a clear channel assessment (CCA) is performed over a channel observation period ≥20μs at the beginning of every fixed frame period. If the channel is clear, the node can transmit over a duration of a given channel occupancy time. If the channel is busy, the node will have to wait until the end of the fixed frame before a new CCA is performed again. However, an IEEE 802.11 Wi-Fi AP accesses the channel continuously without a fixed frame structure. Rather, a random number is defined based on a certain contention window size.  The Wi-Fi AP transmits whenever this number of idle occurrences has passed. As pointed out in [8], if an LAA-LTE eNB were to adopt this approach, the performance of the eNB would suffer, as the eNB can only sense the channel once every frame while the Wi-Fi node does not have such a restriction. In fact, as discussed in [8], this approach is detrimental even in the case of LAA-LTE to LAA-LTE co-existence, as the transmitting node can potentially capture the channel for a long time.

In the case of LBE, the transmit/receive structure is demand-driven. A CCA is performed, and if the channel is clear, the node would transmit over a given channel occupancy time that is no greater than  ms, where is selected by the vendor in the range between 4 and 32  If the channel is not clear, an extended CCA is performed where the channel is observed over a period of , where is selected uniformly between 1 and , and is the CCA observation time of no less than 20μs [4]. Similar to Wi-Fi, CCA is continuously performed without any restriction to frame boundaries. Although the extended CCA mentioned above is similar to the backoff mechanism in Wi-Fi, [4] does not mandate the backoff to be exponential as in the case of Wi-Fi. Thus, if the LAA-LTE backoff mechanism is fixed (i.e. as outlined in [4]) while that of Wi-Fi is exponential, the Wi-Fi performance will be unfairly impacted as numerically confirmed in [5], [6], and [7]. Additionally, as pointed out in contributions such as [9], [10], [11], and [12], the LBE is inherently unfair to Wi-Fi. This contribution attempts to examine the potential improvement of the exponential back-off as outlined in the option A of [14] relative to the fixed back-off in [4].

Having shown the impact of LAA upon Wi-Fi in the absence of LBT in [3], this follow-up contribution presents results for the DL transmissions in unlicensed spectrum with special focuses on

- the impact of LAA upon Wi-Fi in the presence of LBT. In particular, this contribution compares the effects of exponential [14] and fixed back-off for LBE LBT [4], with the modification suggested in [10];

- the comparison of performance between LAA/Wi-Fi vs Wi-Fi/Wi-Fi network;

- the impact of the use of preamble detection (PD) and virtual carrier sensing (VCS) as compared to the use of ED alone during carrier sensing;

- the potential benefit of using a Clear-to-Send to Self (CTS-to-Self) with the NAV mechanism for channel reservation

# Simulation results and discussions

In this contribution, five groups of simulation cases are included. Within each group, 6 different traffic loads are simulated. The q value is set to be 32. The detail description of the simulation groups are shown in Table 1.  Further simulation assumptions and parameters are given in the Appendix.

Table 1 Categories of simulation cases.

number

Victim

Aggressor

Detection

Reservation

Backoff

100s

Wi-Fi

Wi-Fi

-

-

-

200s

Wi-Fi

LAA-LBT

ED+PD+VCS

CTS-to-self

Exponential

300s

Wi-Fi

LAA-LBT

ED+PD+VCS

CTS-to-self

Fixed

400s

Wi-Fi

LAA-LBT

ED only

None

Exponential

500s

Wi-Fi

LAA-LBT

ED+PD+VCS

None

Exponential

LAA Medium Access Design

RAT

In this contribution, three different versions of the back-off mechanisms are used.  The first one is labelled as ‘Scheme A’, which corresponds to the LBE LBT proposed in [4] with the introduction of sub-frame boundary. The second version is labelled as ‘Scheme B (fixed)’, which corresponds to the back-off mechanism shown in Figure 2 with a fixed back-off as in [7]. The third version is labelled as ‘‘Scheme B (exp)’, which corresponds to the back-off mechanism shown in Figure 2 but with an exponential back-off as in Option A proposed in [14]. The label X00s in Table 1 corresponds a set of simulation cases with the poisson packet arrival rates, lambda, of 0.25, 0.5, 1, 1.5, 2.5, 3.5 per sec.

ICCA

Complete ECCA

Tx

Wait for subframe boundary

Wait for medium to become unoccupied

Medium occupied*?

N

Y

Y

CTS-to-self (optional)

Medium occupied?

Y

N

Figure 1 Scheme A back-off mechanism version.

ICCA

Complete ECCA (includes extra ICCAs**)

Set ECCA_completed = True

1.7.1: fixed value of n slots in 1..q

1.7.4: exponential back-off

Tx

Wait for subframe boundary

Wait for medium to become unoccupied

ECCA_completed == True?

Medium occupied*?

N

N

Y

Y

CTS-to-self (optional)

Set ECCA_completed = False

* This test is being continually applied during the whole period of waiting for the subframe boundary

** After every occupied slot during ECCA backoff, an extra 20us ICCA is required prior to recommencing the countdown/countup. It is analogous to the 34us DIFs of Wi-Fi.

Figure 2 Scheme B back-off mechanism version.

Scheme A

Scheme B (fixed)

Scheme B (exp.)

Figure 3 Comparison between fixed and exponential back-off for victim Wi-Fi.

Scheme B (exp.)

Scheme B (fixed)

Scheme A

Figure 4 Comparison between fixed and exponential back-off for aggressor LAA.

Figure 3 and Figure 4 show the downlink network throughput as a function of traffic load for the aggressor and victim networks respectively, with ED+PD+VCS based detection mechanism and CTS-to-Self-based channel reservation. Scheme B (exp) is expected to be more polite than the Scheme B (fixed) due to the extra exponential mechanism in the former. Scheme B (fixed), in turn, is expected to be more polite than the Scheme A due to the extra polite mechanism as described in Figure 2. The results confirm the expectation that Scheme A is the least polite, and the victim network suffers significantly at high load, while the aggressor dominates the channel. As the Scheme B (fixed) includes the extra polite mechanism, the victim network performance significantly improves. However, as LAA is still associated with the fixed back-off, while the Wi-Fi is with the exponential back-off, the results are still unfair towards Wi-Fi. Finally, the results for Scheme B (exp) suggest that LAA is more polite than necessary, and so the victim Wi-Fi network performs better than the aggressor LAA network. These observations are not entirely surprising, as the exponential back-off mechanism in [14] is not exactly the same as that in Wi-Fi. In other words, a better fairness can potentially be achieved through more refined parameter tuning, i.e. the speed of the exponential growth, defer time, etc.

Figure 5 Comparison between LAA/Wi-Fi and Wi-Fi/Wi-Fi performance.

Figure 5 shows the comparison of network throughput between LAA/Wi-Fi and Wi-Fi/Wi-Fi networks based on Scheme B (exp). It can be seen that, in the case of Wi-Fi/Wi-Fi, both aggressor and victim achieve a very similar throughput as expected. This observation is consistent with the expectation due to the symmetry between the aggressor and victim. However, in the case of LAA/Wi-Fi, the aggressor performance is lower due to the fact that LAA is more polite than Wi-Fi under Scheme B (exp).

Figure 6 Comparison between LAA performance with and without channel reservation.

Figure 6 shows the downlink network throughput for the LAA/Wi-Fi scenario using Scheme B (exp), with and without medium reservation, both in the presence of preamble detection and virtual carrier sense. The results show that when medium reservation is absent, both LAA and Wi-Fi performance degrades due to increased chance for collisions.

Figure 7 Comparison between cases with energy detection and with preamble detection and vitrual carrier sensing.

Figure 7 shows the comparison of downlink network throughputs for the LAA/Wi-Fi scenario using Scheme B (exp), with energy detection and with Preamble detection + virtual carrier sensing. In both cases, medium reservation is not available. Comparing Figure 6 and Figure 7, the results show that the performance of both aggressor and victim degrades visibly. This is due to the less accurate channel sensing, and thereby giving rise to an increased level of collisions. With energy detection, the ability for LAA to respect the Wi-Fi channel occupancy is reduced, and thereby causing further degradation to the victim Wi-Fi network.

Figure 8 Comparison of downlink network throughput between ETSI V1.7.1 and V1.7.4

Figure 8 shows the downlink network throughput as a function of traffic load for both aggressor and victim networks based on Scheme A with back-off mechanism based on ETSI 1.7.1 [4] and ETSI 1.7.4 [14] respectively, with ED+PD+VCS based detection mechanism and CTS-to-Self-based channel reservation. The results show that even though [14] corresponds to a truncated exponential back-off, while [4] to a fixed back-off, the respective performances are very similar. The reason is that the Wi-Fi stations usually need to wait at least a 34us DIFS period before transmitting while the LAA network will frequently see the medium unoccupied for its 20us ICCA (see Figure 1) and transmit most of the time. In other words, if the medium is unoccupied for the 20us ICCA the LAA network never enters the random back-off ECCA stage.  Therefore the back-off mechanism within the ECCA phase never gets called upon and hence the results for fixed vs. exp back-off look very similar as below.

# Conclusion

Simulation results show that the use of a fixed back-off mechanism as described in [4] is not enough for LAA-LTE to provide a fair coexistence with Wi-Fi. However, simulation results suggest that the exponential back-off described in Scheme B (exp) may produces unfair results against LAA. Based on the simulation results, further investigations need to be done in order to assess the fairness issue in LAA/Wi-Fi coexistence.

Proposal 1: Using a larger CCA interval from 20 μs to something comparable to Wi-Fi interval of 43 μs (based on EDCA, Best Effort Values) should be considered in future simulations

Proposal 2: Using an adaptive backoff window mechanism (such as exponential backoff) in ECCA as well as a more fine-tuned q value should be considered in future simulations.

Proposal 3: LAA nodes should consider a CTS-Self mechanism to enable Wi-FI nodes detect start of a LAA transmission burst.

Proposal 4: LAA nodes should consider including Wi-Fi Preamble Detection and Virtual Carrier to improve co-existence with WI-Fi.

# References

- TR 36.889, Study on Licensed-Assisted Access to Unlicensed Spectrum, V0.1.1, (2014-11).

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #79, San Francisco, USA, 17th – 21st November 2014.

- R1-144603, “Initial Coexistence Evaluations comparing LAA Versus Wi-Fi”, Cisco Systems.

- ETSI EN 301 893 V1.7.1.

- A. Babaei, J. Padden, J. Andreoli-Fang, “Overview of EU LBT and its Effectiveness for Coexistence of LAA LTE and Wi-Fi”, IEEE 802.19-14/0082r0, Nov., 2014.

- BRAN(14)000098r1, “5 GHz Coexistence Simulations”, Broadcom, BRAN#81, 2014-12-10.

- BRAN(14) 000095a1, “5 GHz Coexistence Simulations LAA-LTE (LTE-U) versus Wi-Fi”, CableLabs, BRAN#81, 2014-12-10.

- R1-144000, “Solutions for required functionalities and design targets”, Qualcomm.

- R1-145132, “Discussion on the comparison of LBE and FBE for LBT”, Coolpad.

- R1-145193, “Details of Listen-Before-Talk for LAA”, Ericsson.

- BRAN(14) 000097, “Let’s make equal sharing in 5GHz a reality”, Cisco, BRAN#81, 2014-12-10.

- BRAN(14) 000102, “WFA proposal for addressing comments on clause 4.8.3.2 on LBT in EN 301 893 v1.7.2”, WiFi Alliance, BRAN#81, 2014-12-10.

- R1-150549, “Proposal for achieving fair coexistence between LAA and Wi-Fi”, Cisco, Feb. 9-13, 2015.

- ETSI EN 301 893 V1.7.4.

# Appendix:

# Detailed Simulation Parameters and Assumptions

The working assumption and agreements from RAN1#79 on detailed coexistence evaluation assumptions in LAA are reproduced in the tables below with the addition of a column to report our implementation against each of these.

## Indoor scenario for LAA

| Indoor scenario for LAA | Licensed cell | Unlicensed cell | Current assumption |
| --- | --- | --- | --- |
| Layout for nodes | For DL-only coexistence evaluations: Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. |  | Only unlicensed cell is modelled The location of the APs of one operator is equi-distance from APs of the second operator (with the exception of the edge APs). |

| Indoor scenario for LAA | Licensed cell | Unlicensed cell | Current assumption |
| --- | --- | --- | --- |
| System bandwidth per carrier | 10MHz | 20MHz | Only a 20MHz unlicensed cell is explicitly modelled |
| Carrier frequency | 3.5 GHz | 5.0GHz | Use 5.3 GHz which is the centre frequency of the 5.0GHz unlicensed band (channel 60) |
| Number of carriers | 2 (one for each operator) | For DL-only LAA coexistence evaluations: 1, 4 (to be shared between two operators) | One 20MHz channel for the unlicensed LAA-LTE cells to be modelled. This is shared between the two operators. |
| Total BS TX power | 24dBm (Ptotal per carrier) | 18 dBm across aggregated carriers Optional: 24 dBm | 18dBm |
| Total UE TX power | Total UE TX power: 23dBm across aggregated cells Max total UE TX power per cell in licensed spectrum: 23dBm Max total UE TX power across aggregated cells in unlicensed spectrum: 18 dBm |  | 18dBm |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [referring to Table B.1.2.1-1 in TR36.814] Indoor UE-to-indoor UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability and break point distance) |  | Same |
| Penetration | 0dB |  | Same |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |  | Same as the ITU InH model with 1dB indoor handover margin (TR 36.814) |
| Antenna pattern | 2D Omni-directional is baseline; directional antenna is not precluded |  | Same |
| Antenna Height: | 6m |  | Same |
| UE antenna Height | 1.5m |  | Same |
| Antenna gain + connector loss | 5dBi |  | Antenna + connector: 5dBi 2dB feeder loss to calibrate against TR36.814 InH model |
| Antenna gain of UE | 0 dBi |  | Same |
| Fast fading channel between eNB and UE | ITU InH |  | Ped A 3kmh assumed for LTE-LAA link level |
| Number of clusters/buildings per macro cell geographical area | N/A |  | N/A |
| Number of small cells per cluster | N/A |  | N/A |
| Number of small cells per Macro cell | N/A |  | N/A |
| Number of UEs | 10 UEs per unlicensed band carrier for DL-only LAA coexistence evaluations |  | Same |
| UE dropping per network | All UEs should be randomly dropped and be within coverage of the small cell Example of a dropping method to achieve this with N=10 UEs: - Drop a large enough number of UEs, so that at least 10 UEs are covered by the small cell. Randomly select 10 UEs from the UEs that have coverage. |  | Same |
| Radius for small cell dropping in a cluster | N/A |  | N/A |
| Radius for UE dropping in a cluster | N/A |  | N/A |
| Minimum distance (2D distance) | 3m |  | Assume 3m is the minimum distance for AP to UE not UE to UE or AP to AP distance. Assume Local Area Base Station type with 45dB MCL as specified in 36.104 V11.9.0 (2014-07) |
| Traffic model | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP Model 1 as in TR 36.814 FTP model file size: 0.5 Mbytes. Optional: Mixed traffic model with each UE carrying only VoIP traffic or only FTP traffic in the Wi-Fi network that is not replaced by LAA. - Two UEs with VoIP traffic in addition to UEs with FTP traffic - The VoIP traffic model is based on G.729A (data rate is 24 kbps) - Packet inter-arrival time: 20 ms - Packet size: 60 bytes (payload plus IP header overhead) - Voice activity is assumed to be 100% statistics are independently reported in each direction No associated control plane traffic is modelled |  | All LAA-LTE UEs will use: - FTP 3 traffic - With 0.5MB file sizes Wi-Fi victim network modelling will consider the scenario of FTP3 in DL traffic alone The aggressor {LAA, Wi-FI} and victim Wi-Fi network will use loading levels of Lambda={0.25, 0.5,1.0,1.5, 2.5, 3.5} Hz |
| UE receiver | MMSE-IRC as baseline |  | Rel 8 UE receiver (as in [3]) |
| UE noise figure | 9dB |  | Same |
| UE speed | 3km/h |  | Same |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |  | Same |
| UE Bandwidth | UE bandwidth for LAA: 10 MHz licensed + 20 MHz unlicensed - CA scheduling assumptions stated when reporting results - Served traffic per small cell per carrier can be reported UE bandwidth for Wi-Fi: 20 MHz unlicensed |  | Licenced portion of LAA-LTE not modelled. 20 MHz bandwidth for unlicensed carrier assumed throughout |
| Network synchronization | For the same operator, the network can be synchronized and the assumed synchronization accuracy in such simulations should be stated. Small cells of different operators are not synchronized. |  | Synchronized |
| Backhaul assumptions | Dropped in R1-145453 |  | Not modelled |
| Performance metrics | Performance metric - User perceived throughput (UPT) UPT CDF File throughput is calculated per file Unfinished files should be incorporated in the UPT calculation. The number of served bits (possibly zero) of an unfinished file by the end of the simulation is divided by the served time (simulation end time – file arrival time). User throughput is the average of all its file throughputs - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) Latency CDF If VoIP users are included, number of VoIP users with 98%ile latency greater than 50 ms should be reported Note: DL and/or UL can be reported when applicable |  | We are using the FTP network throughput for simplicity. |

## Additional LAA assumptions

|  | 3GPP value as per R1-145453 | Current assumption |
| --- | --- | --- |
| PCI planning for each NW | Planned | Not explicitly modelled (as in [3]) |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. Optional: 1Tx2Rx in DL. 1Tx2Rx in UL (should be the same as for Wi-Fi) | 2x2 Cross polarized (as in [3]) TM4 (closed loop spatial multiplexing, which falls back to TM6 for low SINR/scattering) |
| Transmission schemes | Based on TM4 or TM10, QPSK/16QAM/64QAM Optional: include 256QAM (should be the same as for Wi-Fi) | Not explicitly modelled but implied in SINR to Tput mapping to be used. |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) | Not explicitly modelled |
| Scheduling | Proportional fair | Fair (as in [3]) |
| Link adaptation | Realistic | Ideal (as in[3]) |
| CCA-ED | Up to each company; should state assumption when reporting results | Assume an LAA energy detection threshold of -62dBm for CAA-ED and -82dBm for CCA-PD where a recognizable preamble is transmitted (in line with Wi-Fi) |
| Channel selection | Up to each company; should state assumption when reporting results | Not implemented (as in[3]) |
| Cyclic Prefix | Normal | Not explicitly modelled but assumed in SINR to Tput mapping used. |

## Additional Wi-Fi assumptions

| Parameter |  | 3GPP value as per R1-145453 | Current assumption |
| --- | --- | --- | --- |
| MCS |  | 802.11ac MCS table without 256 QAM Optional: include 256QAM (should be the same as for LAA) | Yes, 256 QAM included for Wi-Fi |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized Optional: 1Tx2Rx in DL. UL: 1Tx2Rx (should be the same as for LAA) Baseline: open loop Company should state assumptions if assumed otherwise | As in phase 1: 2Tx2Rx in DL, Cross-polarized UL 1x2 (which is a change from 2x2 in phase 1) |
| Channel coding |  | BCC Optional: LDPC code | BCC (Not LPDC) |
| Frame aggregation |  | A-MPDU | Yes |
| MPDU size |  | Up to each company | Fixed 1500B MPDU size (variable transmission duration) as in [3] |
| Max PPDU duration |  | Baseline:< 4 ms (Asynchronous to LTE timing) Company should state assumptions if assumed otherwise | 4.096ms maximum PPDU applied. |
| MAC | Coordination | DCF If VoIP users are included, EDCA can be used | DCF (no EDCA) |
|  | SIFS, DIFS | SIFS, DIFS | Yes |
|  | Detection | Energy detection & preamble detection | Yes |
|  | RTS/CTS | Optional | Not modelled |
|  | Contention window | Per DCF If VoIP users are included, per EDCA can be used | Yes – EDCA not being used for VoIP users |
| CCA-PD |  | -82dBm and preamble decoding (Note preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) | Yes |
| CCA-ED |  | -62dBm | Yes |
| ACK Modeled (successful reception, resources utilized) |  | Yes | Yes |
| DL/UL Duplexing |  | DL traffic only for DL-only LAA coexistence evaluation | DL and UL for victim network DL only for aggressor network |
| Rate control |  | Up to each company; should state assumption when reporting results | Same rate adaption as in [3] |
| Channel selection |  | Up to each company; should state assumption when reporting results | Congested scenario: Single 20MHz channel for all APs |
| OFDM symbol length |  | 4 micro second | As in [3], short guard interval is adopted therefore OFDM symbol length is 3.6 micro second |