3GPP TSG-RAN WG1 Meeting #81	R1-152932

Fukuoka, Japan, 25th – 29th May 2015

Agenda Item:	6.2.4.1

Source:	InterDigital Communications

Title:	LBT Cat 4 evaluation results for DL-only LAA with WiFi and VoIP

Document for:	Discussion

# Introduction

We presented coexistence evaluation results for DL-only LAA with FTP NFB and VoIP traffic in [3]-[6]. For the 3 coexistence evaluation cases, Wi-Fi/Wi-Fi, LAA/LAA and Wi-Fi/LAA, we showed results for the Indoor Scenario [9], for Y=1 and Y=4 unlicensed carriers in the 5 GHz band, for LBE Cat 3 and Cat 4 schemes, for different assumed TXOP durations and with both FTP NFP and VoIP traffic. In contributions [1]-[2] we present our findings from the evaluation results for the Wi-Fi and LAA DL+UL coexistence case with FTP traffic using 50% DL/50% UL and 80% DL/20% UL traffic ratios in the Indoor scenario with Y=1.

In this contribution we compare and discuss DL-only LAA and WiFi coexistence evaluation including VoIP for the Indoor scenario with Y=1 (a single 5 GHz channel) for different types of LBT Cat 4 implementations. We evaluate and discuss the relative performance of two possible LBT Cat 4 schemes, one using a countdown based, the other one using UE NACK based contention-window re-sizing approach. Furthermore, we compare the relative performance of both of these LBT Cat 4 schemes when subjected to different CCA thresholds levels.

# Evaluation methodology and assumptions

Overview

For evaluations purposes, we assume that the LAA eNB employs LBT/CCA for channel access in the DL. DTX is used to limit the maximum possible transmission duration for an LAA transmitter. The results presented here are for the Y=1 case (a single 5 GHz channel), so no carrier selection mechanism is employed by the LAA eNB as by assumption.

We use FTP non-full buffer traffic model 3 with 0.5 MB file size. More detailed evaluation assumptions for both Wi-Fi and LTE are summarized in Appendix B of this document. VoIP modeling follows the assumption agreed in the TR [9].

Channel Access

Channel Access is modeled according to the agreed WF on Cat 4 Access Schemes presented in April RAN1-80bis [10].

For DL transmissions, the LAA eNB employs LBT Cat 4 with variable window size. The CW size is exponentially variable and initially set to q=16 at the beginning of the simulation. Initial CCA has duration BiCCA = 32 us. In case the channel is not sensed to be idle, the random backoff N is selected from [0, q-1]. DeCCA has a duration of 32 us. For subsequent eCCA steps, 8 us sensing intervals are used. Following the initial CCA period(s), if the channel is deemed to be unoccupied, the backoff counter is decremented by 1. When the channel is observed to be occupied during the extended CCA period by the eNB, the backoff counter is frozen and the eNB defers back. The CW will then be updated according to one of the 2 following evaluated options,

LBT Cat 4 Option 1 – Resetting of CW using the countdown mechanism as by ETSI EN 301 893 v1.8.0, Section 4.8.3.2

LBT Cat 4 Option 2 – Resetting of CW based on the eNB received UE A/N on the licensed band PCell UL

The eNB can start DL scheduling immediately after the backoff counter value reaches zero, i.e., earliest after the initial CCA periods. TXOPs are fixed to 4 ms.

DTX

DTX for the LAA DL is modeled as a maximum allowed time duration to schedule a number of consecutive DL subframes by the LAA eNB. We use maximum TXOP durations of 4 ms. The duration of the busy signal when transmitted by the eNB prior to regular DL scheduling does not impact the allowed maximum number of schedulable DL subframes. Therefore, the LTE LAA DL will transmit up to 4 consecutive DL subframes once it obtains channel access using LBT/CCA. When a DL busy signal is transmitted by the LAA eNB, it is accounted for as overhead until the beginning of the next regularly occurring SCell subframe boundary.

# Coexistence evaluation results for DL-only LAA

For the 2 coexistence evaluation cases, Wi-Fi/Wi-Fi and Wi-Fi/LAA, our DL only results for the Indoor Scenario in presence of VoIP traffic are summarized in Appendix A.

Tables A.1 and A.2 show UPT for the FTP NFB traffic and the VoIP outage results for Wi-Fi/Wi-Fi and Wi-Fi-LAA at low, medium and high traffic load points when using an energy detection threshold of -62dBm or -82dBm respectively. Both tables correspond to LBT Cat 4 Option 1 where the CW is reset using the countdown mechanism.

Tables A.3 and A.4 show the corresponding results for LBT Cat 4 Option 2 where the CW is reset based on the eNB received UE A/N on the licensed band PCell UL.

Figures 1 for the case of an -62 dBm energy detection threshold and Figure 2 for -82 dBm summarize our findings for performance of the Wi-Fi network in coexistence Step 1 and Step 2 with both evaluated LBT Cat 4 window adjustment schemes.

Figure 1: DL UPT and VoIP outage for Indoor Scenario, Y=1, TXOP=4ms;
LBT Cat 4; CCA threshold = -62 dBm

Figure 2: DL UPT and VoIP outage for Indoor Scenario, Y=1, TXOP=4ms;
LBT Cat 4; CCA threshold = -82 dBm

From our evaluation results it can be seen that LAA is a better neighbor to Wi-Fi in Step 2 than the replaced Wi-Fi network from Step 1 when a sensing threshold of -82 dBm for both LBT Cat 4 Options 1 and 2 is employed. FTP NFB throughput improves with Wi-Fi/LAA when compared to the Wi-Fi/WiFi case. Observed VoIP UE outage is no worse with Wi-Fi/LAA than in the Wi-Fi/WiFi case.

When LAA uses a sensing threshold of -82 dBm, there is no discernible difference between the performances of the two LBT Cat 4 CW adjustment schemes, one using a countdown based, the other one using a UE NACK based contention-window re-sizing approach. This implies that the system is less sensitive to the choice of the CW re-sizing mechanism when operating at these lower and more aggressive CCA energy detection thresholds.

Our evaluation results also indicate that there is no discernible difference in the FTP NFB throughput when comparing LBT Cat 4 Option 1 (count down based) and Option 2 (A/N based) when LAA uses a sensing threshold of -62 dBm. However, VoIP UE outage with LBT Cat 4 Option 2 (A/N based) is better than that seen with Option 1 (count down based). This is intuitively explainable as in the A/N based approach, an LAA node that observes a transmission failure would double its CW, therefore allowing APs serving latency sensitive VoIP traffic an increased opportunity to gain access to the channel.

# Conclusion

In this contribution we compare and discuss DL-only LAA and Wi-Fi coexistence evaluation including VoIP for the Indoor scenario with Y=1 (a single 5 GHz channel) for different types of LBT Cat 4 implementations.

We evaluate and discuss the relative performance of two possible LBT Cat 4 contention window adjustment schemes, one using a countdown based, the other one using a UE A/N based contention-window re-sizing approach. Furthermore, we compare the relative performance of both of these LBT Cat 4 schemes when subjected to different CCA thresholds levels.

From our evaluation results it can be seen that LAA is a better neighbor to Wi-Fi in Step 2 when an aggressive sensing threshold of -82 dBm for both LBT Cat 4 Option 1 (count down based) and Option 2 (A/N based) is employed. FTP NFB throughput improves. Observed VoIP UE outage is no worse with Wi-Fi/LAA than in the Wi-Fi/WiFi case. When LAA uses a sensing threshold of -82 dBm, there is no discernible difference between the performances of the two LBT Cat 4 CW adjustment schemes. This implies that the system is less sensitive to the choice of the CW re-sizing mechanism with LBT Cat 4 when operating at these lower CCA energy detection thresholds.

For the case of -62 dBm energy detection thresholds, we don’t observe any discernible difference in the FTP NFB throughput when comparing the LBT Cat 4 Option 1 (count down based) and Option 2 (A/N based). However, VoIP UE outage ratios using LBT Cat 4 Option 2 (A/N based) are significantly better than what is observed with LBT Cat 4 Option 1 (count down based).

# References

R1-152933 Coexistence evaluation results for Wi-Fi and UL+DL LAA; InterDigital

R1-151458 Coexistence evaluation results for DL+UL LAA and WiFi; InterDigital

R1-152161 Evaluation results for DL-only LAA with WiFi and VoIP; InterDigital

R1-151066 Coexistence evaluation results for LTE LAA DL-only and Wi-Fi; InterDigital

R1-151068 Design considerations on LTE LAA DL and UL operation; InterDigital

R1-150540 Coexistence evaluation results for LTE LAA DL-only and WiFi; InterDigital

R1-150541 Design considerations on LTE LAA DL+UL mode; Interdigital

R1-145186 On the need to model RTS/CTS in LAA coexistence scenarios; InterDigital

3GPP TR 36.889 v031 Feasibility Study on License-Assisted Access to Unlicensed Spectrum (R13)

R1-152413 WF on Cat 4 Channel Access Scheme; Intel, LG, Ericsson, et alt.

# Appendix A – DL only LAA evaluation results with Wi-Fi and VoIP

Table A.1: Wi-Fi/LAA; DL only; Indoor Deployment; Y=1; TXOP=4ms; VoIP;
LBT Cat 4 with countdown mechanism; CCA threshold = -62 dBm

| LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Cat. 4 Option 1 (count down) | UPT CDF [Mbps] | 5% | 20.382 | 12.44 | 13.494 | 8.132 | 2.837 | 2.965 | 4.306 | 1.156 | 1.558 |
|  |  | 50% | 82.287 | 67.759 | 69.747 | 34.878 | 19.050 | 19.229 | 19.177 | 10.616 | 11.519 |
|  |  | 95% | 119.852 | 110.549 | 131.344 | 92.197 | 82.237 | 96.817 | 74.300 | 75.243 | 76.290 |
|  |  | Mean | 77.727 | 67.976 | 72.826 | 41.537 | 28.377 | 30.397 | 27.062 | 20.207 | 22.528 |
|  | Delay CDF [s] | 5% | 0.035 | 0.041 | 0.036 | 0.052 | 0.052 | 0.053 | 0.067 | 0.057 | 0.064 |
|  |  | 50% | 0.056 | 0.244 | 0.180 | 0.281 | 0.564 | 0.501 | 0.435 | 0.780 | 0.568 |
|  |  | 95% | 0.235 | 1.309 | 0.942 | 1.400 | 3.323 | 2.962 | 2.267 | 4.694 | 3.439 |
|  |  | Mean | 0.087 | 0.362 | 0.321 | 0.450 | 1.070 | 0.866 | 0.700 | 1.481 | 0.990 |
|  | VoIP outage (%) |  | 0.0 | 12.5 | n/a | 0.0 | 75.00 | n/a | 6.25 | 75.00 | n/a |
|  | 𝜌 |  | 0.914 | 0.881 | 0.947 | 0.918 | 0.777 | 0.891 | 0.836 | 0.638 | 0.821 |
|  | BO |  | 14.47 | 25.44 | 23.73 | 44.49 | 61.70 | 60.87 | 61.46 | 75.05 | 71.35 |
|  | 𝜆 |  | 0.7 |  |  | 1.0 |  |  | 1.2 |  |  |
| Additional comments |  |  | Results without licensed carrier, ED threshold = -62 dBm; TXOP independent of q and set to 4ms. LBT Cat.4 based on R1-152413: eCCA step = 8 us. BiCCA and DeCCA = 32 us. Resetting of CW for LAA based on countdown mechanism as by ETSI EN 301 893 v1.8.0, Section 4.8.3.2 Option A. |  |  |  |  |  |  |  |  |

Table A.2: Wi-Fi/LAA; DL only; Indoor Deployment; Y=1; TXOP=4ms; VoIP;
LBT Cat 4 with countdown mechanism; CCA threshold = -82 dBm

| LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Cat. 4 Option 1 (count down) | UPT CDF [Mbps] | 5% | 20.382 | 25.870 | 13.931 | 8.132 | 10.437 | 3.940 | 4.306 | 6.523 | 2.365 |
|  |  | 50% | 82.287 | 87.712 | 81.915 | 34.878 | 45.488 | 23.150 | 19.177 | 27.505 | 16.183 |
|  |  | 95% | 119.852 | 120.278 | 142.742 | 92.197 | 103.794 | 104.255 | 74.300 | 89.541 | 99.219 |
|  |  | Mean | 77.727 | 83.024 | 82.662 | 41.537 | 50.600 | 35.901 | 27.062 | 36.189 | 29.014 |
|  | Delay CDF [s] | 5% | 0.035 | 0.035 | 0.030 | 0.052 | 0.042 | 0.047 | 0.067 | 0.048 | 0.044 |
|  |  | 50% | 0.056 | 0.050 | 0.056 | 0.281 | 0.136 | 0.710 | 0.435 | 0.286 | 0.532 |
|  |  | 95% | 0.235 | 0.196 | 0.439 | 1.400 | 0.849 | 2.424 | 2.267 | 1.513 | 3.974 |
|  |  | Mean | 0.087 | 0.075 | 0.120 | 0.450 | 0.246 | 0.952 | 0.700 | 0.475 | 1.212 |
|  | VoIP outage (%) |  | 0.0 | 0.0 | n/a | 0.0 | 0.0 | n/a | 6.25 | 6.25 | n/a |
|  | 𝜌 |  | 0.914 | 0.921 | 0.984 | 0.918 | 0.961 | 0.915 | 0.836 | 0.888 | 0.715 |
|  | BO |  | 14.47 | 12.87 | 14.80 | 44.49 | 36.02 | 55.50 | 61.46 | 52.87 | 69.07 |
|  | 𝜆 |  | 0.7 |  |  | 1.0 |  |  | 1.2 |  |  |
| Additional comments |  |  | Results without licensed carrier, ED threshold = -82 dBm; TXOP independent of q and set to 4ms. LBT Cat.4 based on R1-152413: eCCA step = 8 us. BiCCA and DeCCA = 32 us. Resetting of CW for LAA based on countdown mechanism as by ETSI EN 301 893 v1.8.0, Section 4.8.3.2 Option A. |  |  |  |  |  |  |  |  |

Table A.3: Wi-Fi/LAA; DL only; Indoor Deployment; Y=1; TXOP=4ms; VoIP;
LBT Cat 4 with A/N feedback; CCA threshold = -62 dBm

| LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Cat. 4 Option 2 (with A/N) | UPT CDF [Mbps] | 5% | 20.382 | 12.566 | 10.954 | 8.132 | 3.041 | 2.960 | 4.306 | 1.043 | 1.545 |
|  |  | 50% | 82.287 | 72.380 | 68.376 | 34.878 | 19.778 | 18.866 | 19.177 | 10.296 | 10.208 |
|  |  | 95% | 119.852 | 111.885 | 132.346 | 92.197 | 83.039 | 87.370 | 74.300 | 63.028 | 74.368 |
|  |  | Mean | 77.727 | 69.303 | 72.040 | 41.537 | 28.226 | 29.766 | 27.062 | 18.225 | 20.124 |
|  | Delay CDF [s] | 5% | 0.035 | 0.037 | 0.034 | 0.052 | 0.053 | 0.059 | 0.067 | 0.066 | 0.058 |
|  |  | 50% | 0.056 | 0.268 | 0.187 | 0.281 | 0.582 | 0.500 | 0.435 | 0.676 | 0.629 |
|  |  | 95% | 0.235 | 1.184 | 0.839 | 1.400 | 3.347 | 2.704 | 2.267 | 4.815 | 3.433 |
|  |  | Mean | 0.087 | 0.380 | 0.307 | 0.450 | 1.023 | 0.863 | 0.700 | 1.505 | 1.092 |
|  | VoIP outage (%) |  | 0.0 | 6.25 | n/a | 0.0 | 37.5 | n/a | 6.25 | 62.5 | n/a |
|  | 𝜌 |  | 0.914 | 0.889 | 0.946 | 0.918 | 0.792 | 0.889 | 0.836 | 0.626 | 0.802 |
|  | BO |  | 14.47 | 23.97 | 23.10 | 44.49 | 60.88 | 60.06 | 61.46 | 76.04 | 73.22 |
|  | 𝜆 |  | 0.7 |  |  | 1.0 |  |  | 1.2 |  |  |
| Additional comments |  |  | Results without licensed carrier, ED threshold = -62 dBm; TXOP independent of q and set to 4ms. LBT Cat.4 based on R1-152413: eCCA step = 8 us. BiCCA and DeCCA = 32 us. Resetting of CW for LAA based on A/N received from LAA UE on UL PCell. |  |  |  |  |  |  |  |  |

Table A.4: Wi-Fi/LAA; DL only; Indoor Deployment; Y=1; TXOP=4ms; VoIP;
LBT Cat 4 with A/N feedback; CCA threshold = -82 dBm

| LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Cat. 4 Option 2 (with A/N) | UPT CDF [Mbps] | 5% | 20.382 | 26.387 | 12.468 | 8.132 | 10.778 | 4.616 | 4.306 | 6.510 | 1.96 |
|  |  | 50% | 82.287 | 86.986 | 76.070 | 34.878 | 47.123 | 21.977 | 19.177 | 25.809 | 13.429 |
|  |  | 95% | 119.852 | 120.291 | 142.500 | 92.197 | 105.984 | 115.827 | 74.300 | 82.717 | 87.769 |
|  |  | Mean | 77.727 | 82.069 | 81.519 | 41.537 | 52.192 | 37.666 | 27.062 | 33.314 | 25.133 |
|  | Delay CDF [s] | 5% | 0.035 | 0.035 | 0.030 | 0.052 | 0.041 | 0.041 | 0.067 | 0.056 | 0.057 |
|  |  | 50% | 0.056 | 0.051 | 0.059 | 0.281 | 0.126 | 0.500 | 0.435 | 0.293 | 0.563 |
|  |  | 95% | 0.235 | 0.191 | 0.510 | 1.400 | 0.682 | 2.019 | 2.267 | 1.335 | 4.240 |
|  |  | Mean | 0.087 | 0.077 | 0.134 | 0.450 | 0.206 | 0.706 | 0.700 | 0.459 | 1.264 |
|  | VoIP outage (%) |  | 0.0 | 0.0 | n/a | 0.0 | 0.0 | n/a | 6.25 | 6.25 | n/a |
|  | 𝜌 |  | 0.914 | 0.921 | 0.983 | 0.918 | 0.966 | 0.935 | 0.836 | 0.892 | 0.721 |
|  | BO |  | 14.47 | 13.20 | 15.57 | 44.49 | 33.00 | 51.00 | 61.46 | 53.82 | 72.06 |
|  | 𝜆 |  | 0.7 |  |  | 1.0 |  |  | 1.2 |  |  |
| Additional comments |  |  | Results without licensed carrier, ED threshold = -82 dBm; TXOP independent of q and set to 4ms. LBT Cat.4 based on R1-152413: eCCA step = 8 us. BiCCA and DeCCA = 32 us. Resetting of CW for LAA based on A/N received from LAA UE on UL PCell. |  |  |  |  |  |  |  |  |

# Appendix B – Evaluation assumptions

| Parameter | Table B.1: Indoor scenario for LAA coexistence evaluations |
| --- | --- |
| Layout for nodes | Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. |
| System bandwidth per carrier | 20 MHz |
| Carrier frequency | 5.0 GHz |
| Number of carriers | 1 (to be shared between two operators) |
| Total BS TX power | 18 dBm |
| Total UE TX power | 18 dBm |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [Table B.1.2.1-1 in TR36.814] UE-to-UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability.) |
| Penetration | 0dB |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |
| Antenna pattern | 2D Omni-directional |
| Antenna Height: | 6m |
| UE antenna Height | 1.5m |
| Antenna gain + connector loss | 5dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU InH |
| Number of clusters/buildings per macro cell geographical area | N/A |
| Number of small cells per cluster | N/A |
| Number of small cells per Macro cell | N/A |
| Number of UEs | 10 UEs per unlicensed band carrier per operator |
| UE dropping per network | Uniform random drop within indoor coverage |
| Radius for small cell dropping in a cluster | N/A |
| Radius for UE dropping in a cluster | N/A |
| Minimum UE-to-UE distance (2D distance) | 3m |
| Traffic model | FTP3, 0.5 MB file size Mixed traffic model with each UE carrying only VoIP traffic or only FTP traffic in the Wi-Fi network that is not replaced by LAA. - Two UEs with VoIP traffic in addition to UEs with FTP traffic - The VoIP traffic model is based on G.729A (data rate is 24 kbps) - Packet inter-arrival time: 20 ms - Packet size: 60 bytes (payload plus IP header overhead) - Voice activity is assumed to be 100%. Statistics are independently reported in each direction - No associated control plane traffic is modelled |
| UE receiver | MMSE-IRC as baseline |
| UE noise figure | 9dB |
| UE speed | 3km/h |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |
| UE Bandwidth | UE bandwidth for LAA and Wi-Fi: 20 MHz unlicensed |
| Network synchronization | For the same operator, the network is synchronized. Small cells of different operators are not synchronized. |
| Performance metrics | UPT, Delay, Buffer occupancy, Outage |

| Parameter |  | Table B.2: Additional Wi-Fi system evaluation assumptions |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table without 256 QAM |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized Open loop |
| Channel coding |  | BCC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | Fixed 1ms (A)-MPDU transmission duration |
| Max PPDU duration |  | 4 ms |
| MAC | Coordination | EDCA |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | CCA-CS |
|  | RTS/CTS | OFF |
|  | Contention window | FTP Min: 15 slots, Max: 1023 slots VoIP Min: 3 slots, Max: 7 slots |
| CCA-CS |  | -82dBm |
| CCA-ED |  | -62dBm |
| RTS/CTS |  | OFF |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | DL only |
| Rate control |  | Minstrel algorithm with 100 ms reconfiguration period |
| Channel selection |  | Minimum neighbour |
| OFDM symbol length |  | 4 us |

| Parameters | Table B.3: Additional LTE system evaluation assumptions |
| --- | --- |
| PCI planning for each NW | Planned |
| MCS | QPSK/16QAM/64QAM |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. Closed loop |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) |
| Scheduling | Proportional fair |
| Feedback | 10 ms reporting period, 6 ms delay |
| Link adaptation | Realistic |
| LBT | Load based (LBT Cat 4) with Option 1: Resetting of CW for LAA based on countdown mechanism as by ETSI EN 301 893 v1.8.0, Section 4.8.3.2 Option A. Option 2: Resetting of CW for LAA based on A/N received from LAA UE on UL PCell. |
| CCA-ED | -62 dBm or -82 dBm |
| CCA time | LBT Cat 4 based on R1-152413: eCCA step = 8 us BiCCA and DeCCA = 32 us |
| Channel occupancy time | TXOP 4 ms |
| LBT backoff | LBT Cat 4 based on R1-152413 with Initial CCA using BiCCA then Extended CCA using DeCCA and eCCA step. |
| Channel selection | Minimum neighbour |
| Cyclic Prefix | Normal |