3GPP TSG RAN WG1 Meeting #82                                                                             	                    R1-153962

Beijing, China, 24 – 28 August 2015

Source:	Panasonic

Title: 	DCI design for MTC

Agenda Item:		7.2.1.2

Document for:	Discussion and Decision

Introduction

In the last RAN1 meeting, we discussed DCI design and showed some of our views [1]. This document shows our views on detailed design on MTC DCIs.

Discussion

Naming and search space

Reduction of DCI format size is one of the requirements for Rel-13 MTC DCI design especially for large repetition case since it reduces the required repetitions and improve the power consumption of MTC UEs in case of the repetitions.

For the no/small repetition and large repetition cases, the required DCI contents are different. Then, the size is also different.  Although it may be proper to have different names for no/small repetition case and large repetition case, for this document, the same name is assigned. For this document, we name following and discussed respectively.

- DCI format 1X for the scheduling of unicast MPDSCH

- DCI format 1Y for the scheduling of MPDSCH of RAR/Paging

- DCI format 1Z to carry single MAC RAR

- DCI format W for the scheduling of unicast MPUSCH

Search space

For unicast, the search space and what RNTI is used is described as following.

Table 1: MPDCCH and MPDSCH configured by C-RNTI

| DCI format | Search Space | RNTI |
| --- | --- | --- |
| DCI format 1X | UE-specific search space by C-RNTI | C-RNTI |

Table 2: MPDCCH and MPUSCH configured by C-RNTI

| DCI format | Search Space | RNTI |
| --- | --- | --- |
| DCI format W | UE-specific search space by C-RNTI | C-RNTI |

For common channels, the search space and what RNTI is used is described as following.

Table 1: MPDCCH and MPDSCH configured by P-RNTI

| DCI format | Search Space | RNTI |
| --- | --- | --- |
| DCI format 1Y | CE level specific or not are FFS depending on RAN2/3/SA2 reply to [9][10] | P-RNTI |

Table 2: MPDCCH and MPDSCH configured by RA-RNTI

| DCI format | Search Space | RNTI |
| --- | --- | --- |
| DCI format 1Y | search space determined by CE-level-specific RNTI | CE-level-specific RA-RNTI |
| DCI format 1Z | search space determined by RA-preamble-specific RNTI | RA-preamble-specific RA-RNTI |

DCI contents

Our view on the detailed DCI contents is following [2]~[8].

DCI format 1X for the scheduling of unicast MPDSCH (FDD)

Table 3 Contents in DCI format 1X

| Field Name | Field Size in No/Small Repetition case | Field Size in Large Repetition case | Remarks |
| --- | --- | --- | --- |
| Modulation and coding scheme | FFS | FFS | The size depends the MCS table. Field size would be reduced from 5 bits. |
| HARQ process number | 3 | 0 | In order to support maximum throughput without the same scheduling, 8 HARQ process number is supported for no/small repetition case. 1 HARQ process is assumed for large repetition case. |
| New data indicator | 1 | 1 |  |
| Redundancy version | 2 | 0 | In no/small repetition case, RV is indicated in the DCI. In large repetition case, same RV is used during the coherent combining period. Fixed RV switching pattern is used among the coherent combining periods. |
| TPC command for PUCCH | 2 | 0 | Always transmitting maximum power for large repetition case. |
| SRS request | 1 | 0 | Aperiodic SRS is supported for no/small cases was agreed in RAN1 #80bis. |
| HARQ-ACK resource indicator | FFS | FFS | If implicit PUCCH resource determination is used and overlap of PUCCH region among narrowbands is considered, dynamic offset like ARO is needed. Detailed discussion is in [2]. |
| Narrowband indicator | FFS | 0 | In no/small repetition case, the size depends on number of narrowbands in a system bandwidth. In large repetition case, the scheduled MPDSCH is transmitted in the same narrowband of the MPDCCH under the same hopping pattern [3]. |
| Same/cross-subframe scheduling | FFS 0 or 1 | 0 | In large repetition case, it is always cross-subframe scheduling. In no/small repetition case, if the other field can indicate it implicitly, 0 bit. If explicit indication is required, 1 bit. |
| Resource block assignment | FFS 5 or less | 0 | In large repetition case, MPDSCH always fully occupy the narrowband. In no/small repetition case, reduction is proposed in [4]. |
| Valid subframe indicator | FFS | 0 | In large repetition case, valid subframes are semi-statically indicated. In no/small repetition case, we propose to indicate it in DCI. Detailed discussion is in [5]. |
| Repetition/time location of PDSCH | 0 | 3 | In no/small repetition case, valid subframe indicator shows the used subframes. In large repetition case, 3 bits indicate the repetition number and location in time domain. Detailed discussion is in [3] |

DCI format 1Y for the scheduling of MPDSCH of RAR/Paging (FDD)

As proposed in [6], the DCI scheduling PDSCH carrying multiple MAC RARs is transmitted in CE-level-specific way. The DCI format can also be used for scheduling Paging message.

Table 4 Contents in DCI format 1Y

| Field Name | Field Size in No/Small Repetition case | Field Size in Large Repetition case | Remarks |
| --- | --- | --- | --- |
| Modulation and coding scheme | FFS | FFS | The size may be reduced further from DCI format 1X. |
| TPC command for PUCCH | 2 | 0 | Same as DCI format 1X. |
| SRS request | 1 | 0 | Same as DCI format 1X. |
| Narrowband indicator | FFS | 0 | Same as DCI format 1X. |
| Same/cross-subframe scheduling | 0 | 0 | Always cross-subframe scheduling |
| Resource block assignment | FFS 5 or less | 0 | Same as DCI format 1X. |
| Valid subframe indicator | FFS | 0 | Same as DCI format 1X. |
| Repetition/time location of PDSCH | 0 | 3 | Same as DCI format 1X. |

DCI format 1Z to carry single MAC RAR (FDD)

Table 5 Contents in DCI format 1Z

| Field Name | Field Size in No/Small Repetition case | Field Size in Large Repetition case | Remarks |
| --- | --- | --- | --- |
| Timing Advance Command | 11 | 11 | Same as legacy MAC RAR. |
| Temporary C-RNTI | 16 | 16 | Same as legacy MAC RAR. |
| Narrowband indicator of Message 3 | FFS 2 | FFS 0 | In no/small repetition case, subset of narrowbands can be configured by SIB for RAR and DCI indicate one narrowband in the subset to reduce the DCI size. In large repetition case, 0 bit is assumed same as DCI format 1X [3]. |
| Resource block assignment of Message 3 | FFS 2 or 3 | 0 | In large repetition case, 6 PRBs are fully occupied. In no/small repetition case, reduction from DCI format 1X would be possible. |
| Valid subframe indicator | FFS | 0 | Similar as the indicator in DCI format 1X. It indicates the starting subframe, available subframe and repetition number of Message 3. |
| Repetition/time location of PUSCH | 0 | 3 | Same as DCI format 1X. |
| Truncated modulation and coding scheme of Message 3 | FFS | FFS | The size would be further reduced from legacy 4 bits |
| TPC command for scheduled PUSCH of Message 3 | 3 | 0 | In large repetition case, maximum transmitting power is always used. |
| CSI request | 1 | 0 | In large repetition case, we propose CSI feedback is not supported. In no/small repetition case, aperiodic CSI feedback is supported and triggered by DCI [7]. |

DCI format W for the scheduling of unicast MPUSCH (FDD)

Table 6 Contents in DCI format W

| Field Name | Field Size in No/Small Repetition case | Field Size in Large Repetition case | Remarks |
| --- | --- | --- | --- |
| Modulation and coding scheme | FFS | FFS | The size depends the MCS table. Field size would be reduced from 5 bits. |
| New data indicator | 1 | 1 |  |
| TPC command for PUSCH | 2 | 0 | Same as DCI format 1X. |
| SRS request | 1 | 1 | Same as DCI format 1X. |
| CSI request | 1 | 0 | In large repetition case, CSI feedback is not supported. In no/small repetition case, aperiodic CSI feedback is supported [7]. |
| Cyclic shift for DMRS, OCC index and multiple subframe code index | 3~5 | 3~5 | Legacy indicator of cyclic shift for DMRS, OCC index is 3 bits. In [8], multiple subframe code spreading is proposed. If the code index is indicated dynamically, 2 bits are needed. Same field could be used for OCC index for and CDM code. The CDM code is also indicated by semi-static or UE-ID dependent like C-RNTI. |
| Narrowband indicator | FFS | FFS | Same as DCI format 1X. In no/small repetition, if resource block assignment allocates PRBs in system BW, Narrowband indicator is not necessary. |
| Resource block assignment | FFS 5 or variable | FFS 2 or 3 | In large repetition case, one PRB is allocated to PUSCH. 2 or 3bits are used to indicate the PRB location in narrowband. In no/small repetition case, if resource allocations are limited to 6PRBs in a narrowband, 5 bits are used for full flexibility. If resource allocations are system BW, the number of bits is variable depending on system BW [4] |
| Valid subframe indicator | FFS | 0 | Same as DCI format 1X. |
| Repetition/time location of PUSCH | 0 | 3 | Same as DCI format 1X. |

Conclusion

In the contribution, we present our thoughts on detailed DCI design for MTC. This contribution is rather a summary on many of our views on MTC designs.

- Reference

- R1-152907, “DCI design for Rel.13 MTC”, Panasonic

- R1-153970, “Discussion and evaluation on PUCCH for MTC”, Panasonic

- R1-153961, “Search space for MTC”, Panasonic

- R1-153967, “Resource allocation of PDSCH for Rel.13 MTC”, Panasonic

- R1-153965, “Valid subframe indication of PDSCH/PUSCH in MTC”, Panasonic

- R1-153972, “RAR and paging for  MTC”, Panasonic

- R1-153971, “CSI report/measurement in MTC”, Panasonic

- R1-153969, “Multiple subframe code spreading for MTC UEs”, Panasonic

- R1-145454, " LS on Paging for MTC", RAN1

- R1-152442, " LS on Paging Optimization (S2-150698/ R3-150345)", RAN3