3GPP TSG RAN WG1 Meeting #83	R1-156789

Anaheim, USA, 16th - 20th November 2015

Agenda item:	6.2.4.3.1

Source: 	Samsung

Title: 	Finalizing Rel.13 Rank>1 Codebook Design

Document for:	Discussion and Decision

# Summary

In RAN1#82, the following has been agreed for submitting rank >1 codebook proposals over RAN1 email reflector [1]:

- Deadline to submit codebook proposals for evaluation (– Eko (Samsung))

  - October 19th for rank 2

  - October 23rd for rank 3-8

The following proposals were submitted:

| No. | Proposal | Codebook availability |  |  |
| --- | --- | --- | --- | --- |
|  |  | Rank 2 | Rank 3-4 | Rank 5-8 |
| 1 | Samsung/Ericsson/NTT DOCOMO/CATT | x | x | x |
| 2 | Alcatel-Lucent/ASB | x | x | x |
| 3 | Huawei/HiSilicon | x | x | x |
| 4 | CMCC | x | x | x |
| 5 | Intel | x | x |  |
| 6 | LGE | x | x | x |
| 7 | Qualcomm | x | x |  |
| 8 | ZTE | x | x |  |

The following summarizes the status of the codebook proposals. The observations below are high-level. A more detailed outlook can be found in companion contributions [2][3].

## Class A Rank 2

Out of the 8 submitted proposals:

- 7 proposals (proposals 1-4, 6-8) are aligned in Config 1.

- 5 proposals (proposals 1, 2, 6-8) are aligned in Config 4. The remaining 3 proposals are not aligned among themselves.

- Different designs are proposed for Configs 2 and 3. Some reasons for such divergence include:

  - Maintaining or not maintaining the total number of codewords in codebooks tables to 32. For instance, proposals 2, 6, and 7 require at least 36, 34, and 34 codewords, respectively, across all the Configs to maintain nested property  across rank 1-2. The other proposals maintain the total number of codewords to be 32.

- For Config 3, proposals 1 and 8 are identical. In addition, proposals 6 and 7 are identical.

- For Config 2, proposals 2 and 7 are identical.

## Class A Rank 3-4

Out of the 8 submitted proposals:

- 3 proposals (proposals 1, 5, 6) incorporate the choice of orthogonal beam grouping as a part of W1 feedback. 4 proposals (proposals 2, 3, 7, 8) incorporate the choice of orthogonal beams as a part of W2 feedback.

- Proposals 1 and 6 share the same design when the number of orthogonal beam groups (values of k) is 2 and checkerboard pattern are used in proposal 1.

## Class A Rank 5-8

Out of the 5 submitted proposals:

- Proposals 1, 2, and 4 are characterized by W1-only structure (hence require no i2 feedback). In addition, they share many similarities including identical designs in some Configs.

- On the other hand, proposals 3 and 6, although both requiring i2 feedback, are quite different from each other.

## Class B Rank 2-8

Only proposal 1 contains a complete design proposal for class B codebooks.

# Conclusion

Despite a considerably large number of class A proposals, there are some common grounds which can be used as a starting point for achieving consensus. It is recommended that the proposing companies strive to quickly agree on aspects where a clear majority is observed and focus discussion and comparison efforts on diverging views. Some examples where a clear majority is observed are rank-2 Config 1 (and 4) as well as W1-only codebook structure for rank 5-8.

For class B codebooks, since there is only one concrete proposal, it is expected that consensus can be achieved relatively easier compared to class A.

# References

RAN1#82, Chairman Notes

R1-156790, Samsung, “Rank 2 codebook -- summary of RAN1 proposals and performance” 3GPP TSG RAN1#83

R1-156791, Samsung, “Rank 3-8 codebook -- summary of RAN1 proposals and performance” 3GPP TSG RAN1#83