3GPP TSG RAN1 Meeting #83								R1-157390

Anaheim, CA, USA, 16th– 20th November, 2015

Source: 		Ericsson

Title:	NB-IoT - Battery lifetime comparison for standalone operation

Agenda Item:	6.2.6.1.1

Document for:	Discussion and decision

# Introduction

At RAN#69 a new work item on narrowband IoT and tasked RAN1 to evaluate two numerology options for both UL and DL:

- DL: 15 kHz sub-carrier spacing (with normal or extended CP) and 3.75 kHz sub-carrier spacing.

- UL: FDMA with GMSK modulation and SC-FDMA (including single-tone transmission as a special case of SC-FDMA)

One of the performance objectives is for the IoT device to meet the battery lifetime requirement of 10 years. Even if a possible down selection of DL and UL numerology could be seen as unconnected evaluations, in order to estimate the battery lifetime, a system with both DL and UL must be assumed for the evaluation. For this reason, in this contribution we will compare the performance of a NB-IoT system as described in [3], in the following called NB-LTE, to the NB-CIoT system as described in [1].

First, we update the evaluation of NB-LTE in [2], using the methodology given in TR45.820 [1] and aligning with the assumptions used for evaluating NB-CIoT. Secondly, we compare the battery life time for the tow described systems.

# NB-LTE energy consumption evaluation

## Updated Assumptions

This analysis follows the methodology as described in [2] with the following updates:

- The transmission time for the 200 byte IP report on PUSCH for 164 dB MCL at has been reduced to 2760 ms since the previous value was pessimistic resulting in an MCL of 166.1 dB. The new transmission time results in an MCL of 164.4 dB.

- Update of cell search times to latest design [6]

- In order to align with the evaluation in section 7.3.6.4 in[1], the effect of BLER has only been included for PUSCH and for other channels no retransmissions are assumed.

- In order to align with the evaluation in section 7.3.6.4 in[1] and [5], the UE only wakes up three times during the 20 s ready timer. During the DRX, the UE is in standby mode.

## Results

The overall reception, transmission and idle times per report at different coverage based on the new assumptions are shown in Table 1.

Table 1 Transmission, Reception and Idle time per report (ms)

|  | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| Tx (50 bytes) | 48 | 334 | 2090 |
| Tx (200 bytes) | 80 | 1005 | 3700 |
| Rx | 163 | 223 | 804 |
| Idle | 1263 | 1423 | 2085 |

The estimated lifetime in years are presented in Table 2for two different packet sizes, two reporting intervals and at different coverage.

Table 2 estimated battery lifetime in years

| Packet size, reporting interval | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| 50 bytes, 2 hours | 26.8 | 13.1 | 3.1 |
| 200 bytes, 2 hours | 24.1 | 6.0 | 1.9 |
| 50 bytes, 1 day | 36.8 | 32.8 | 19.5 |
| 200 bytes, 1 day | 36.3 | 26.4 | 14.5 |

We see that with the updated assumptions in section 2.1, the estimated battery life for NB-LTE is further improved fulfilling the 10 year target for battery lifetime.

Observations 1:

- For all MCL targets, a 10 year battery life is achievable with a reporting interval of one day for both 50 bytes and 200 bytes application payloads.

- For MCL target 144 dB, a 10 year battery life is achievable with a two hour reporting interval for both 50 bytes and 200 bytes application payloads.

- For a MCL target 154 dB with 200 byte application payload, or 164 dB with 50 or 200 byte application payload, a 10 year battery life is not achievable for a 2 hour reporting interval.

# Comparison of NB-LTE and NB-CIoT

The estimated battery lifetime for NB-CIoT with integrated PA from [1] is repeated for convenience in Table 3.

Table 3 NB-CIoT estimated battery lifetime in years[1]

| Packet size, reporting interval | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| 50 bytes, 2 hours | 22.4 | 11.0 | 2.5 |
| 200 bytes, 2 hours | 18.2 | 5.9 | 1.5 |
| 50 bytes, 1 day | 36.0 | 31.6 | 17.5 |
| 200 bytes, 1 day | 34.9 | 26.2 | 12.8 |

Table 4 shows the increase in battery life time that can be achieved with NB-LTE over NB-CIoT for the studied scenarios.

Table 4 Increase in estimated battery lifetime with NB-LTE

| Packet size, reporting interval | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| 50 bytes, 2 hours | 22% | 13% | 21% |
| 200 bytes, 2 hours | 34% | -1% | 19% |
| 50 bytes, 1 day | 2% | 3% | 11% |
| 200 bytes, 1 day | 4% | 0% | 13% |

As we cans see, for all cases except one, NB-LTE has longer estimated battery lifetime compared to NB-CIoT.

Observation 2:

- For all cases but one, where the difference is negligible, NB-LTE will have longer battery lifetime than NB-CIoT

# Conclusions

In this contribution we evaluated battery lifetime of the NB-LTE proposal further and compared it to the battery lifetime of NB-CIoT and make the following observations.

Observations 1:

- For all MCL targets, a 10 year battery life is achievable with a reporting interval of one day for both 50 bytes and 200 bytes application payloads.

- For MCL target 144 dB, a 10 year battery life is achievable with a two hour reporting interval for both 50 bytes and 200 bytes application payloads.

- For a MCL target 154 dB with 200 byte application payload, or 164 dB with 50 or 200 byte application payload, a 10 year battery life is not achievable for a 2 hour reporting interval.

Observation 2:

- For all cases but one, where the difference is negligible, NB-LTE will have longer battery lifetime than NB-CIoT

# References

TR 45.820 v13.0.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

R1-156006, “NB-IoT - Battery lifetime evaluation”, 	Ericsson, ZTE, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, Samsung, LGE

R1-156013, “NB-LTE – General L1 concept description”, Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung

R1-156009, “Narrowband LTE – Synchronization Channel Design and Performance”, Ericsson

GP-150137, “NB M2M – Battery life analysis”, Neul

R1-157418, “NB-IoT – Improved Synchronization Channel Design”, Ericsson