3GPP TSG RAN WG1 Meeting #83	R1-157466

Anaheim, USA, November 15-22, 2015

Agenda Item:	6.2.6.1.2

Source:	Huawei, HiSilicon

Title:	Exception report latency evaluation in guard-band operation

Document for:	Discussion and decision

# Introduction

At RAN#69 meeting, the “NB-IoT” WI was approved based on outcome of the GERAN SI on cellular IoT [1]. The objective on exception report latency in study item is stated in TR 45.820 as

For devices supporting such applications a delay requirement of 10 seconds is appropriate for the uplink when measured from the application ‘trigger event’ to the packet being ready for transmission from the base station towards the core network.

The evaluation results following methodologies in [2]  and following conclusion on designs with 3.75 kHz sub-carrier spacing downlink and FDMA uplink have been captured in TR 45.820:

This section demonstrates that exception report can be delivered to the base station within 10s for all coverage classes with a reliability of at least 99%.

In this contribution, the evaluation results of exception report delivery latency with an UL numerology supporting 2.5 kHz  sub-channel spacing and 1.875 kHz sample rate is presented for standalone operation (see [8] for more information). Downlink with 3.75 kHz sub-carrier spacing and uplink with FDMA are assumed, and the same methodology for standalone case [5] is used in the analysis except that the time to receive downlink channels is prolonged, due to the limited downlink power boosting in guard-band operation.

# Exception report procedure

The different durations for transmitting an exception report with 90% and 99% confidence of successful delivery are summarized in Table 1 and Table 2, respectively. The assumptions different to standalone case [5] are as following:

- PSD boosting of 6dB is assumed for downlink channels.

- The results on synchronization refer to [3], and the results on broadcast channel refer to [4].

Table 1 Exception report activity duration for 90% confidence of delivery

| Activity |  | Report with no header compression (105 byte payload) |  |  |
| --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 |
| Tsync (ms) |  | 140 | 200 | 760 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 80 | 160 | 640 |
|  | Time to decode PSI Duration (ms) | 80 | 160 | 640 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 |
|  | PSI to RACH gap | 40+4 | 40+4 | 320+4 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 320 | 320 | 320 |
|  | NB-EPDCCH | 10 | 30 | 430 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 |
|  | Processing time | 3 | 3 | 3 |
| Total time (ms) |  | 807 | 1397 | 5397 |

Table 2 Exception report activity duration for 99% confidence of delivery

| Activity |  | Report with no header compression (105 byte payload) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 | 164 |
| Initial BLER |  | 10% | 10% | 10% | 1% |
| Tsync (ms) |  | 140 | 200 | 760 | 760 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 80 | 160 | 640 | 640 |
|  | Time to decode PSI Duration (ms) | 80 | 160 | 640 | 640 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 | 320 |
|  | PSI to RACH gap | 40+4 | 40+4 | 320+4 | 320+4 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 320 | 320 | 320 | 320 |
|  | NB-EPDCCH | 10 | 30 | 430 | 430 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 | 2560 |
|  | Processing time | 3 | 3 | 3 | 3 |
| TUplinkAck (ms) & TUplinkAssignment (ms) | Wait for NB-EPDCCH | 217 | 167 | 167 |  |
|  | NB-EPDCCH | 10 | 30 | 430 |  |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |  |
|  | Transmission time | 50 | 400 | 1920 |  |
|  | Processing time | 3 | 3 | 3 |  |
| Total time (ms) |  | 1127 | 2037 | 7957 | 5713 |

Based on above analysis, the following could be observed:

Observation 1: For design with 3.75 kHz subcarrier spacing downlink and FDMA uplink, the exception report can be delivered to the network with 99% confidence within 10 seconds in guard-band operation, for all the target MCLs, including 164 dB.

The further evaluation results for 15 kHz subcarrier spacing downlink with FDMA uplink are provided in Annex B. For comparison, the results of design with 15 kHz sub-carrier spacing downlink and SC-FDMA uplink are evaluated in Annex C. By comparing the results in Annex B and Annex C., the following could be observed:

# Observation 2: FDMA with GMSK uplink design has lower latency than SC-FDMA based uplink design (for both numerologies) in delivering exception report to the network in guard-band operation.
    - This is without taking into account power back-off for SC-FDMA, see [9] for details. Conclusions

This document provides the latency evaluation for transmitting an exception report to the network in guard-band operation. Based on the analysis, the following can be observed:

Observation 1: For design with 3.75 kHz subcarrier spacing downlink and FDMA uplink, the exception report can be delivered to the network with 99% confidence within 10 seconds in guard-band operation, for all the target MCLs, including 164 dB.

Observation 2: FDMA with GMSK uplink design has lower latency than SC-FDMA based uplink design (for both numerologies) in delivering exception report to the network in guard-band operation.
    - This is without taking into account power back-off for SC-FDMA, see [9] for details.

# References

RP-151621, “New Work Item: Narrow Band IOT (NB-IOT)”, Qualcomm Incorporated, RAN#69, Phoenix, US, September, 2015.

3GPP TR 45.820, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things, Release 13, version 13.0.0.

R1-156465, Evaluations of network synchronization, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Annaheim, Nov., 2015.

R1-156462, NB-IOT - Downlink physical layer concept description, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Annaheim, Nov., 2015.

R1-156467, Exception report latency evaluation in standalone operation, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Anaheim, Nov., 2015.

R1-156027, NB-IoT – Exception report latency evaluation, Ericsson, RAN1#82BIS, Malmo, Oct., 2015.

R1-156013, Coverage analysis in stand-alone operation, Ericsson, RAN1#82BIS, Oct., 2015.

R1-156930, Uplink FDMA physical layer design updates, Neul, Huawei, HiSilicon, RAN1#83, November 2015.

R1-156466, Uplink coverage evaluation, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Annaheim, Nov., 2015.

R1-156019, NB-IoT – Exception report latency evaluation inband, Ericsson, RAN1#82BIS, Malmo, Oct., 2015.

R1-155984, Performance Evaluations for In-band Operation, Nokia Networks, RAN1#82BIS, Malmo, Oct., 2015.

- PUSCH configurations

The PUSCH configurations in guard-band operation are given in Table 3.

Table 3 PUSCH configurations with less than 10% BLER

| Burst type | PHY burst size | Coupling loss | Symbol rate | modulation | repetition | code rate | Duration (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH | 105 bytes | 144dB | 30kHz | GMSK | 1 | 0.77 | 50 |
|  |  | 154dB | 3.75kHz | GMSK | 1 | 0.77 | 400 |
|  |  | 164dB | 1.875 kHz | GMSK | 1 | 0.32 | 1920 |

Table 4 PUSCH configurations with less than 1% BLER

| Burst type | PHY burst size | Coupling loss | Symbol rate | modulation | repetition | code rate | Duration (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH | 105 bytes | 144dB | 30kHz | GMSK | 1 | 0.48 | 80 |
|  |  | 154dB | 3.75kHz | GMSK | 1 | 0.45 | 680 |
|  |  | 164dB | 1.875 kHz | GMSK | 1 | 0.24 | 2560 |

- Evaluation results of 15 kHz subcarrier spacing downlink and FDMA uplink

This section provides the further evaluation results for 15 kHz subcarrier spacing downlink and FDMA uplink. The different durations for transmitting an exception report with 90% and 99% confidence of successful delivery are summarized in Table 5 and Table 6, respectively. The assumptions different to standalone case [5] are as following:

- PSD boosting of 6dB is assumed for downlink channels.

- The results on synchronization refer to [3], and the results on broadcast channel refer to [4].

- The EPDCCH and PDSCH results refer to [10]. However, the EPDCCH and PDSCH transmission time is reduced due to the lower downlink overhead compared to in-band operation.

Table 5 Exception report activity duration for 90% confidence of delivery

| Activity |  | Report with no header compression (105 byte payload) |  |  |
| --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 |
| Tsync (ms) |  | 140 | 200 | 760 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 80 | 160 | 640 |
|  | Time to decode PSI Duration (ms) | 80 | 160 | 640 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 |
|  | PSI to RACH gap | 40+4 | 40+4 | 320+4 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 580 | 580 | 580 |
|  | NB-EPDCCH | 2 | 16 | 95 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 |
|  | Processing time | 3 | 3 | 3 |
| Total time (ms) |  | 1059 | 1643 | 5322 |

Table 6 Exception report activity duration for 99% confidence of delivery

| Activity |  | Report with no header compression (105 byte payload) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 | 164 |
| Initial BLER |  | 10% | 10% | 10% | 1% |
| Tsync (ms) |  | 140 | 200 | 760 | 760 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 80 | 160 | 640 | 640 |
|  | Time to decode PSI Duration (ms) | 80 | 160 | 640 | 640 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 | 320 |
|  | PSI to RACH gap | 40+4 | 40+4 | 320+4 | 320+4 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 580 | 580 | 580 | 580 |
|  | NB-EPDCCH | 2 | 16 | 95 | 95 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 | 2560 |
|  | Processing time | 3 | 3 | 3 | 3 |
| TUplinkAck (ms) & TUplinkAssignment (ms) | Wait for NB-EPDCCH | 0 | 16 | 32 |  |
|  | NB-EPDCCH | 2 | 16 | 95 |  |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |  |
|  | Transmission time | 50 | 400 | 1920 |  |
|  | Processing time | 3 | 3 | 3 |  |
| Total time (ms) |  | 1154 | 2118 | 7412 | 5962 |

- Latency Evaluation in guard-band operation for SC-FDMA uplink

For comparison, the exception report latency of design with 15 kHz sub-carrier spacing downlink and SC-FDMA uplink is re-evaluated with following assumptions different to [6]:

- 6dB PSD boosting is assumed.

- The synchronization, EPDCCH and PDSCH results refer to [10]. However, the EPDCCH and PDSCH transmission time is reduced due to the lower downlink overhead compared to in-band operation.

- Based on results in [11] and our simulation, the time to receive MIB/PSI (primary system information) is 2560 ms with 35dBm transmission power. A 640 gap is added to between synchronization and start reading of MIB/PSI.

- The time to wait for NB-EPDCCH in receiving uplink ACK and uplink assignment is calculated as 95 – (T mod 95), where 95 ms is the NB-EPDCCH periodicity considering the reduced downlink overhead and T is the time required of preceding steps since last NB-EPDCCH transmission.

Table 7 Latency Evaluation in guard-band operation for 15 kHz subcarrier spacing downlink and SC-FDMA uplink

| Activity | Size (bytes) | 164 dB |
| --- | --- | --- |
| Synch |  | 1110 |
| MIB |  | 2560 |
| PRACH |  | 1440 |
| Wait for EPDCCH |  | 580 |
| DL assignment | 8 | 95 |
| Wait for PDSCH |  | 0 |
| RA msg 2 (RAR) | 10 | 118 |
| Wait for PUSCH |  | 9 |
| RA msg 3 (TLLI + Access cause + BSR) | 11 | 236 |
| Wait for EPDCCH |  | 17 |
| DL assignment | 8 | 95 |
| Wait for PDSCH |  | 0 |
| RA msg 4 (cont. res.) | 8 | 95 |
| Wait for EPDCCH |  | 0 |
| UL grant | 8 | 95 |
| Wait for PUSCH |  | 9 |
| Uplink report (50 bytes) | 100 | 2300 |
| eNB processing |  | 3 |
| Total (ms) |  | 8762 |