3GPP TSG RAN WG1 Meeting #83	R1-157613

Anaheim, USA, 16 – 20 November 2015

Agenda Item:	6.2.2

Source: 	NEC

Title:	Outcome of eCA Monday evening offline discussion

Document for:	Discussion/Decision

# Introduction

On Monday evening, eCA topics that may have impact to RRC signalling were discussed, several documents were treated and offline agreements were reached. It is suggested that these offline agreements below are to be formally agreed in RAN1.

# eCA Topics discussed on Monday evening offline session

- Soft Buffer Management

  - OPPO (R1-157189)

Offline agreement:

- eNB rate matching will not have RRC impact

- Wideband A-CSI reporting mode 1-0

  - ZTE (R1-157564)

Offline agreement

- Introduce a new aperiodic CSI reporting mode 1-0 for eCA to contain wideband CQI and RI if applicable

- HARQ-ACK bundling configuration

Offline agreement

- One bit RRC configuration of spatial bundling

  - UE always apply spatial bundling

  - FFS: Independent configuration for PUCCH and PUSCH

Offline agreement

- 3-bit RRC configuration of maximum code rate for HARQ-ACK + P-CSI + SR (if any)

  - Same RRC parameter is for multi P-CSI and P-CSI + HARQ-ACK + SR (if any)

- HARQ-ACK timing for TDD

  - Lenovo (R1-156731)

Possible offline agreement

- For self-scheduling case only, reuse the DL HARQ timing of FDD SCell with TDD PCell as the HARQ timing of a TDD SCell when aggregated with a TDD PCell of different UL/DL configurations.

  - 1 bit RRC configuration to inform UE the new HARQ timing

  - 1 bit UE capability signalling to indicate the support